3GPP TSG RAN WG1 #106-e	R1-2107110

e-Meeting, August 16th – 27th, 2021

Agenda item:		8.2.7

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results

Document for:		Discussion and Decision

# Introduction

This contribution evaluates coexistence between different types of system and presents simulation results in an outdoor scenario with NR-U sharing the channel with a Fixed Wireless Access (FWA) system. This is based on contribution presented to ETSI BRAN in [1]. The evaluated coexistence mechanisms for the NR-U system are beamforming with quasi-omni and directional Listen-Before-Talk (LBT), and no-LBT and with different antenna and beamforming gains.

# Discussion

## Scenario and parameter descriptions

The outdoor scenario definition is based on 3GPP Urban Micro Street Canyon as described in [2] while the FWA system is based on an example scenario as described in [3]. These two are overlaid on the same simulation dropping area.

The overview picture is shown in Figure 1, and the technical 3GPP UMi scenario in Figure 2.

Figure 1. Illustration of the simulation scenario. The baseline fixed wireless access illustration is from [3] with additional elements showing the NR-U system.

Simulation area

Figure 2: 3GPP Urban Micro Street Canyon scenario [2] with the simulation dropping area. The illustration is showing example beam configurations for the base station transmitters.

The simulation area represents a single street with both NR-U and FWA systems deployed (100m x 20m area). The NR-U gNB is mounted in the corner of the area x = y = 0m, at z = 10m height, and is serving 5 UEs on the street level z = 1.5m. The FWA AP is mounted on a light post in the middle of the street y = 10m at z = 5m, and is serving a single housing unit at balcony/window at z = 3, 6, or 9m. The housing units may be at either side of the street (y = 0 or 20m); the FWA AP antenna panel is pointed at the north or south side wall depending on the client position, and uses beam steering at both link ends. The positions of the FWA AP and housing units are randomized along the street (x = 0 to 100m). Otherwise the main simulation parameters are shown in Table 1.

Table 1: Main simulation parameters

| Parameter | Value |
| --- | --- |
| Simulation area | 100 x 20 meters |
| gNB TX power at antenna ports | 20 and 27 dBm |
| gNB antenna panel configuration | - 4x8 elements, 5 dBi element gain 20 dBi total antenna gain - 2x4 elements, 4 dBi element gain 13 dBi total antenna gain Optimized mechanical downtilt and beam pattern |
| gNB TX power EIRP | 40 dBm |
| UE TX power at antenna ports | 14 dBm |
| UE antenna panel configuration | 2x2 elements, 5 dBi element gain Back-to-back panels, best panel selection |
| UE TX power EIRP | 25 dBm |
| FWA AP and housing unit TX power at antenna ports | 20 dBm |
| FWA AP and housing unit antenna panel configuration | 4x8 elements, 5 dBi element gain 20 dBi total antenna gain |
| FWA AP and housing unit TX power EIRP | 40 dBm |
| Traffic model | FTP model 3, packet size 27 MB, varying arrival rate 50:50 DL:UL split |
| LBT energy detection threshold | -48 dBm at antenna ports |
| LBT modes for NR-U system | 1) No LBT 2) Quasi-omni LBT at gNB, 5 ms COT (no UE LBT) 3) Directional LBT at gNB, 5 ms COT (no UE LBT) |

The simulation campaign compares different LBT modes and two beamforming options (13 and 20 dBi gain) as the mechanism to coexist with the FWA system.

## Simulation results

The FTP3 packet arrival rates are set to achieve low, medium, and high load points, which correspond to roughly ~20%, ~45%, and ~70% buffer occupancy. Different arrival rates are used for the NR-U system and the FWA systems. Median and 5th percentile point throughput per finished FTP3 packet are reported.

Figure 3. FWA system DL (top) and UL (bottom) median (left) and 5th percentile (right) throughput 
with different NR-U gNB antenna panel configurations and LBT modes

Figure 4. NR-U system DL (top) and UL (bottom) median (left) and 5th percentile (right) throughput 
with different NR-U gNB antenna panel configurations and LBT modes

The simulation results show that both systems achieve the best performance when the other system is not present at all.

Regarding the FWA system performance, there is almost no difference between the LBT mode and antenna gain that the NR-U system employs. Some more impact can be observed on the 13 dBi antenna gain case, compared to 20 dBi case, but even then it seems the use of LBT or no-LBT is irrelevant.

Regarding the NR-U system performance, using LBT results always in worse performance than no-LBT.

Observation: For outdoor deployments, 13 dBi antenna gain without additional LBT results in reasonable coexistence between systems.

# Conclusion

A mixed 3GPP outdoor / fixed wireless access scenario was simulated with NR-U and FWA systems sharing the same channel. The coexistence mechanism used by NR-U in the simulations was LBT in different forms, and beamforming with different gNB panel configurations (13 and 20 dBi gain).

Based on the results presented in this document, no-LBT would provide at least similar coexistence as typical LBT implementations, if the initiating device has at least 13 dBi antenna and beamforming gain.

Observation: For outdoor deployments, 13 dBi antenna gain without additional LBT results in reasonable coexistence between systems.

# References

- BRAN(21)110044, “60 GHz outdoor coexistence simulations”, Nokia

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 15)

- ECC Report 288, Conditions for the coexistence between Fixed Service and other envisaged outdoor uses/applications in the 57-66 GHz range

- BRAN(21)109d006, Coexistence simulations, Nokia