3GPP TSG RAN WG1 #106-e			R1-2107221

e-Meeting, August 16th – 27th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct a parameter name for PSSCH power control in TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A parameter is wrongly used in PSSCH power control. For PSSCH power control, the following is used to explain the parameter PMAX,CBR used in the power control formula: “PMAX,CBR is determined by a value of sl-MaxTransPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N”. While the pamameter sl-MaxTransPower is not related to CBR and priority. Another parameter sl-MaxTxPower-r16, which is defined in SL-PSSCH-TxParameters-r16 and accordingly in SL-CBR-PSSCH-TxConfig-r16, is determined by CBR and priority. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the parameter name and refer to the right parameter defined in TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The wrong parameter name will cause confusion when determining transmission power for PSSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

16.2.1	PSSCH

A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f of serving cell c as:

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PMAX,CBR is determined by a value of sl-MaxTxPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTxPower is not provided, then PMAX,CBR=PCMAX;

-	if dl-P0-PSSCH-PSCCH is provided

-	PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm]

-	else

-	PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm]

where

-	PO,D is a value of dl-P0-PSSCH-PSCCH if provided

-	αD is a value of dl-Alpha-PSSCH-PSCCH, if provided; else, αD=1

-	PLD=PLb,f,c(qd) as described in clause 7.1.1 except that

-	the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration

-	if sl-P0-PSSCH-PSCCH is provided and if a SCI format scheduling the PSSCH transmission includes a cast type indicator field indicating unicast

-	PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm]

-	else

-	PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm]

where

-	PO,SL is a value of sl-P0-PSSCH-PSCCH, if provided

-	αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1

-	PLSL= referenceSignalPower – higher layer filtered RSRP, where

-	referenceSignalPower is obtained from a PSSCH transmit power per RE summed over the antenna ports of the UE, higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-filterCoefficient, and

-	higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by sl-filterCoefficient

-	MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i and μ is a SCS configuration

The UE splits the power PPSSCH(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power.

A UE determines a power PPSSCH2,b,c(i) for a PSSCH transmission on a resource pool in the symbols where a corresponding PSCCH is transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f of serving cell c as

PPSSCH2(i)=10log10MRBPSSCHi-MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm]

where MRBPSCCH(i) is a number of resource blocks for the corresponding PSCCH transmission in PSCCH-PSSCH transmission occasion i.

The UE splits the power PPSSCH2(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power.

*** Unchanged text is omitted ***