3GPP TSG RAN WG1 #106-e R1-2107263 e-Meeting, August 16th – 27th, 2021 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 16.6.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on PUCCH spatial setting when subslotLength-ForPUCCH is provided. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_L1enh_URLLC; Date:; 2021-8-6. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In last RAN1 meeting, the issue of correction for sub-slot based PUCCH was discussed and the following TP was agreed. ---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9 UE procedure for reporting control information *** Unchanged text is omitted *** In the remaining of this Clause, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission with HARQ-ACK information includes a number of symbols indicated by subslotLength-ForPUCCH. *** Unchanged text is omitted *** ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------. With the clarification, the “slot” in section 9.2.2 in TS 38.213 (PUCCH-SpatialRelationInfo setting part) would be also replaced “sub-slot” since the timing is defined with regard to the slot where the UE would transmit a PUCCH with HARQ-ACK information. However, the introduction of sub-slot based PUCCH transmission is not intended to change the effective time for MAC CE, so the slot in PUCCH-SpatialRelationInfo setting part should not be replaced by sub-slot.. -. Summary of change:; Clarify the “slot” in PUCCH spatial setting part in TS 38.213 is not replaced by “subslot” even if UE is provided subslotLength-ForPUCCH.. Consequences if not approved:; The effective time for MAC CE is unnecessarily mis-interpreted.. Clauses affected:; 9, 9.2.2. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR has isolated impact on PUCCH spatial setting when subslotLength-ForPUCCH is provided: If gNB is implemented according to this CR and UE is not, or, if UE is implemented in accordance to this CR while gNB is not, UE may use incorrect beam for PUCCH transmission leading to PUCCH transmission failure.. This CR's revision history:; This is the first version for this CR..

- 9	UE procedure for reporting control information

*** Unchanged text is omitted ***

In the remaining of this clause, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission with HARQ-ACK information includes a number of symbols indicated by subslotLengthForPUCCH, unless stated otherwise.

*** Unchanged text is omitted ***

- 9.2.2	PUCCH Formats for UCI transmission

*** Unchanged text is omitted ***

A spatial setting for a PUCCH transmission is provided by PUCCH-SpatialRelationInfo if the UE is configured with a single value for pucch-SpatialRelationInfoId; otherwise, if the UE is provided multiple values for PUCCH-SpatialRelationInfo, the UE determines a spatial setting for the PUCCH transmission as described in [11, TS 38.321]. The UE applies corresponding actions in [11, TS 38.321] and a corresponding setting for a spatial domain filter to transmit PUCCH in the first slot that is after slot  where  is the slot consisting of Nsymbslot symbols where the UE would transmit a PUCCH with HARQ-ACK information with ACK value corresponding to a PDSCH reception providing the PUCCH-SpatialRelationInfo and  is the SCS configuration for the PUCCH

*** Unchanged text is omitted ***