| 3GPP TSG-RAN WG1 Meeting #106e e-Meeting, August 16th – 27th, 2021 | R1-2107309 |
| --- | --- |

Title: 		[Draft] Reply to RAN2 LS on Physical Layer Aspects of SDT

Response to:		R2-2106561

Release:           			NR R17

Work Item:       		NR_SmallData_INACTIVE-Core

AI: 		5.2

Source:		Qualcomm [RAN1]

To:		RAN2

Send any reply LS to:	  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		  None

# Overall Description

RAN1 would like to thank RAN2 for the LS on PHY aspects of SDT. In this contribution, we will share our views on PRACH resource configuration for RA-SDT, as well as SS configuration, BWP configuration and feedback information for CG-SDT.

- PRACH Resource Configuration for RA-SDT

To enable SDT based on 4-step RACH and 2-step RACH, PRACH resources can be configured in the initial UL BWP of  UE supporting R17 SDT capabilities. When PRACH resources for RA-SDT are configured on shared RO, at least the following parameters can be configured for 4-step RACH and 2-step RACH, respectively:

- Number of SSB indexes associated with a valid  RO

- Number of contention based preambles dedicated to SDT per SSB index per valid RO

- PRACH mask index

When PRACH resources for RA-SDT are configured on separate RO, at least the following parameters can be configured for 4-step RACH and 2-step RACH, respectively:

- PRACH-ConfigurationIndex

- RO-FrequencyStart, RO-FDM

- Number of SSB indexes associated with a valid RO

- Number of contention based preambles per SSB index per valid RO

- totalNumberofRA-Preambles

In addition,  SS-RSRP threshold for SDT procedure selection,  threshold for SDT payload size, counter of PRACH preamble retransmission and power control parameters for SDT can also be configured for the RACH procedure related to SDT.

- BWP,  Search Space and DL Feedback for CG-SDT

CG-SDT/PUSCH resources should be configured in the initial UL BWP of UE, since SSB is the DL RS for TA validation,  which is QCL’ed with the PDCCH in CORESET0 of initial DL BWP. Besides, the beam correspondence/association for CG-SDT is based on SSB.

If CG-SDT/PUSCH resource is configured in an UL BWP different from initial UL BWP, the DL BWP may not include CORESET0/SSB. As a result, UE needs to monitor PDCCH and measure SSB outside the DL BWP for paging/SI update/TA validation/tracking, which is not desirable for power saving, link maintenance and RRM measurements. Therefore, it is not justified to configure a separate SDT BWP for RRC inactive UE.

On the other hand, DL feedback information for CG-SDT is beneficial, which can include TPC for CG-SDT and UL grant for PUSCH reTX. To this end, UE can monitor DCI format 0_0 and a RNTI associated with CG-SDT in a CSS of initial DL BWP.

To 3GPP RAN2

ACTION: RAN1 respectfully asks RAN2 to take RAN1’s feedback on physical layer aspects of RACH-based and CG-based small data transmission into account in their specification work.

# Dates of Next TSG-RAN1 Meetings:

TSG-RAN WG1 Meeting #106bis-e	        11th – 19th October 2021			E-meeting

TSG-RAN WG1 Meeting #107-e		        11th – 19th November 2021			E-meeting

# References

- RP-210870, “Work Item on NR small data transmissions in INACTIVE state, ” 3GPP TSG RAN#91e.

- R2-2106561, “Reply LS to RAN1 on physical layer aspects of small data transmission” 3GPP TSG RAN2#114e, May 2021.