3GPP TSG RAN WG1 Meeting #106-e 	         R1-2107502

e-Meeting, Aug. 16th – Aug 27th, 2021

Agenda Item: 8.14.3

Source: MediaTek Inc.

Title: Initial Performance and Evaluation Results for XR and CG

Document for: Discussion and decision

Introduction

In the Revised SID of Rel-17 XR Evaluations for NR [1], the objective of this study item are listed as follows:

- Confirm XR and Cloud Gaming applications of interest

- Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc.

- Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios

- Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs

This paper provides the initial system capacity results for CG and AR/VR in both FR1 and FR2 in Dense Urban, UMa and Indoor Hotspot scenario for preliminary observation on the feasibility of the services of interest.

Evaluation Assumption

In this section, the traffic model assumption and the SLS simulation parameters are captured.

Downlink Traffic model

The mean packet size of CG and AR/VR DL traffic is assumed to be of truncated Gaussian distribution and can be derived from the target data rates and the fps. The maximum packet size is assumed to be 150% of the mean packet size, the minimum packet size is 50% of the mean size and the standard deviation is 10.5% of the mean size. Jitter is also modelled through a truncated Gaussian distribution. The corresponding values related to the packet size are given in Table 1, assuming the data rates for cloud gaming and AR/VR are 30/8 Mbps and 45/30 Mbps for the downlink traffic, respectively. In the following sections, we assume that one video frame equals one packet for our simulation.

Table 1: Traffic model parameters for Cloud gaming and AR/VR

|  |  | Cloud Gaming |  | AR/VR |  |
| --- | --- | --- | --- | --- | --- |
| DL traffic | Data rate | 30 Mbps | 8 Mbps | 45 Mbps | 30 Mbps |
|  | Packet size distribution | Truncated Gaussian |  |  |  |
|  | Avg. data packet size (Bytes) | 65536 | 17476 | 98304 | 65536 |
|  | Data packet size STD (Bytes) | 6881 | 1835 | 10322 | 6881 |
|  | Max packet size (Bytes) | 98304 | 26214 | 147456 | 98304 |
|  | Min packet size (Bytes) | 32768 | 8738 | 49152 | 32768 |
|  | Packet format | UDP |  |  |  |
|  | Packet arrival rate (sec) | Periodic (1/FR), FR = 60 |  |  |  |
|  | Jitter value distribution | Truncated Gaussian |  |  |  |
|  | Avg. jitter value (ms) | 0 |  |  |  |
|  | Jitter value STD (ms) | 2 |  |  |  |
|  | Max jitter value (ms) | 4 |  |  |  |
|  | Min jitter value (ms) | -4 |  |  |  |

System capacity definition

It was agreed in RAN1 103-e meeting that the system capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied, where X=90 is the baseline for evaluation. The user being satisfied should meet both requirements for packet latency and reliability. The requirements are captured in Table 2.

Table 2: Packet delay budget and reliability requirement for CG and AR/VR

|  |  | Cloud Gaming |  | AR/VR |  |
| --- | --- | --- | --- | --- | --- |
| DL traffic | Data rate | 30 Mbps | 8 Mbps | 45 Mbps | 30 Mbps |
|  | Packet delay budget | 15ms |  | 10ms |  |
|  | Reliability requirement | 99% |  |  |  |

SLS parameters

In this contribution, both outdoor and indoor deployment are evaluated, including Dense Urban, UMa and Indoor Hotspot. The SLS parameters used for evaluation are listed in Table 3, Table 4, Table 5, Table 6 and Table 7.

Table 3: SLS assumptions for Dense Urban in FR1

Table 4: SLS assumptions for UMa deployment in FR1

Table 5: SLS assumptions for Indoor Hotspot in FR1

Table 6: SLS assumptions for Dense Urban in FR2

Table 7: SLS assumptions for Indoor Hotspot in FR2

CG Simulation Results

FR1

Figure 1 shows the system capacity results for the downlink traffic of CG in FR1. It can be seen that with the data rate equal to 8Mbps, the capacity number is larger than 20 for all evaluated scenarios. The system is capable of accommodating large number of users in this case. However, when the data rate is 30Mbps, the capacity number reduces to 13, 9 and 9 for Dense Urban, Urban Macro and Indoor Hotspot, respectively. Another observation is that the capacity in Dense Urban outnumbers the others due to smaller path loss compared with UMa and more powerful MIMO configuration than the antenna setting used in Indoor Hotspot.

Figure 1: System capacity curves for the downlink traffic of Cloud gaming in FR1

Observation 1: The downlink capacity result for Cloud gaming in FR1 is larger than 20 with 8Mbps data rates for all evaluated scenarios and is 13, 9 and 9 with 30Mbps data rates for Dense Urban, UMa and Indoor Hotspot, respectively.

Observation 2: For the downlink traffic evaluation in FR1, the capacity number in Indoor Hotspot is smaller than the capacity number in Dense Urban and UMa due to inferior antenna setting.

FR2

Figure 2 shows the system capacity results for the downlink traffic of CG in FR2, which is again larger than 20 with 8Mbps data rates and is 11 for both scenarios with 30Mbps data rates. Note that the capacity for CG in Indoor Hotspot in FR2 is comparable to its counterpart in FR1 while the capacity for CG in Dense Urban suffers a reduction from FR1 to FR2. The cause may be the larger path loss at higher frequency band and to support Cloud gaming service, FR2 can be considered for indoor scenarios.

Figure 2: System capacity curves for the downlink traffic of Cloud gaming in FR2

Observation 3: The downlink capacity results for Cloud gaming in FR2 is larger than 20 with 8Mbps data rates and is 11 with 30Mbps data rates for both scenarios.

Observation 4: Since the downlink capacity results for Indoor Hotspot in FR1 and FR2 are comparable for Cloud gaming service, the service can be supported via FR2 spectrum for indoor scenarios.

XR Simulation Results

FR1

Figure 3 shows the system capacity results for the downlink traffic of AR/VR in FR1. Due to higher downlink data rates and more stringent requirement for the packet delay budget than Cloud gaming, the system can only support up to 6, 4 and 4 users with 45Mbps data rate and 10, 8 and 8 users with 30Mbps data rate for Dense Urban, UMa and Indoor Hotspot, respectively. The capacity for such services is relatively small and should be enhanced.

Figure 3: System capacity curves for the downlink traffic of AR/VR in FR1

Observation 5: The downlink capacity result for AR/VR is poorer than the downlink capacity results of CG due to larger data rates and more stringent latency requirement.

Observation 6: The downlink capacity result for AR/VR in FR1 is 10, 8 and 8 with 30Mbps data rates and is 6, 4 and 4 with 45Mbps data rates for Dense Urban, UMa and Indoor Hotspot, respectively.

FR2

Figure 4 shows the system capacity results for the downlink traffic of AR/VR in FR2. For AR/VR with 30Mbps data rates, the system can support up to 10 users for both Dense Urban and Indoor Hotspot. For a higher data rates of 45Mbps, the capacity reduces to 4 for both Dense Urban and Indoor Hotspot. One common observation for both FR1 and FR2 is that the downlink capacity for AR/VR with 45Mbps data rates is quite limited. Enhancements can be towards to increase the throughput or reduce the latency for these latency-sensitive services, for example, carrier aggregation.

Figure 4: System capacity curves for the downlink traffic of AR/VR in FR2

Observation 7: The downlink capacity result for AR/VR in FR2 is 10 with 30Mbps data rates and is 4 with 45Mbps data rates for both Dense Urban and Indoor Hotspot.

Observation 8: With only 100MHz transmission bandwidth, the downlink system capacity for AR/VR with 45Mbps data rates is poor in both FR1 and FR2. Enhancement like carrier aggregation can be utilized to increase the capacity.

XR Simulation Results with Carrier Aggregation

In previous sections we have seen that the downlink capacity for AR/VR is small. A straightforward way to increase the capacity is via larger transmission bandwidth. Due to the limitation of the 100MHz maximum bandwidth in FR1, UE can be configured with larger total bandwidth via carrier aggregation. In this section, the downlink capacity of FR1 inter-band CA of two carriers is evaluated. We also provide the results considering additional enhancements for CA, including PUCCH on both carriers and cross-carrier retransmission. We consider here the frequency bands deployed in real world which are N41 (2.6 GHz) and N79 (4.9 GHz), and the corresponding used TDD patterns are DDDDDDDSUU and DSUDDSUUDD. Note that uplink slots are intentionally separated in two patterns in case of uplink power sharing between two carriers.

It can be observed from Figure 5 that the system capacity with CA is the larger than the sum of the capacity of its component carriers, indicating that CA itself without any enhancements can already provide the capacity gain. With additional enhancements, the capacity can further increase 20% (10 to 12 users per cell) for Dense Urban compared to the CA baseline.

Figure 5: System capacity curves for the downlink traffic of AR/VR in FR1 with inter-band CA

In choosing the TDD patterns that are suitable in supporting latency-sensitive and high downlink data rate services, there is a trade-off between the uplink round trip time and the downlink transmission ability. If there are more uplink slots distributed evenly in the TDD pattern, it can reduce the HARQ delay, which is more critical in FR1 due to longer time duration of a slot. However, more uplink slots mean less downlink opportunity and thus less downlink data rates can be supported.

Observation 9: CA can provide capacity gain and the system capacity with CA is the larger than the sum of the capacity of its component carriers.

Observation 10: With additional enhancements on CA, 20% capacity gain for Dense Urban can be observed compared to the CA without enhancements.

Observation 11: The adopted TDD patterns in supporting latency-sensitive services like AR/VR should consider the trade-off between the uplink and downlink opportunities and the uniformity of uplink slots.

Evaluation for GOP based traffic model with I/P streams

IDR (Instantaneous Decoder Refresh) model has one large Intra-coded I-frame at the beginning of each Group Of Picture (GOP) and small Predicted P-Frame afterwards encoded differentially to the previous I-frame or P-Frame [2]. Compared to GDR (Gradual Decoder Refresh), IDR model has lower encoding latency and low implementation complexity. The IDR model is proposed to be prioritized over GOP model in our companion contribution in this meeting for the evaluation of two-stream DL traffic.

An illustration of the IDR model is shown as Option 1B in Figure 6 and the simulation parameters for the I/P frame modelling is captured in Table 8 where we assume the data rate is 45Mbps.

Figure 6: Illustration of GDR model (Option 1A) v.s. IDR model (Option 1B) courtesy of Huawei

Table 8: Simulation assumption for the I/P frame modelling

| Application | AR 45Mbps |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Two data streams, i.e. M1 = 2 | Option 1A: slice-based |  |  |  | Option 1B: GOP-based |  |  |
|  | I-stream |  | P-stream |  | I-stream |  | P-stream |
| Packet modelling | Slice-level |  |  |  | Frame-level |  |  |
| Traffic pattern | Both streams are periodic at 60 fps with the same jitter model as for single stream. |  |  |  | Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream. |  |  |
| Number of packets per stream at a time | 1 |  | N-1 |  | I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet |  |  |
|  | N = 8: the number of slices per frame. |  |  |  |  |  |  |
| Average data rate per stream | RI=R*αN-1+α |  | RP=R*N-1N-1+α |  | RI=R*αK-1+α |  | RP=R*K-1K-1+α |
|  | - R: average data rate of a single stream video - α: average size ratio between one I-frame/slice and one P-frame/slice, e.g. α = 1.5, 2, 3 |  |  |  |  |  |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |  |  |  |  |
|  | Mean = RIFPS | Mean = RPFPS*(N-1) |  |  | Mean = RI*KFPS | Mean = RP*KFPS*(K-1) |  |
|  | - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video |  |  |  |  |  |  |
| PER, PDB | [PER_I, PER_P] = [A %, B %] [PDB_I, PDB_P] = [C ms, D ms] |  |  | [PER_I, PER_P] = [E %, F %] [PDB_I, PDB_P] = [G ms, H ms] |  |  |  |

In addition to the capacity results under the baseline requirements where the PDB for both I/P frame is 10ms and PER is 1%, we also provide the simulation results w.r.t. to different requirements. For example, longer delay budget for I frame is reasonable since an I frame has larger size than a P frame and we choose 17ms PDB for I frame and 9ms PDB to have an averaged 10ms delay budget. Results with different PER requirements are also provided to compare the results with the effect of different PDB requirements. Moreover, we also take into account the effect of different scheduling algorithm, comparing the legacy proportional-fair scheduler and the so-called “delay-aware” proportional-fair scheduler that is aware of the remaining delay budget of each UE and prioritizes the users who need urgent transmission.

Proposal 1: Adopt the (E, F, G, H) values for the GOP-based model of the I/P frames evaluation as (1, 1, 17, 9)

The FR1 system capacity results of AR/VR traffic with 45Mbps using IDR model are shown in Figure 7. We can see from the curves “PDB (10, 10), PER (1%, 1%)” and “PDB (10, 10), PER (1%, 5%)” that relaxing the PER requirement for P-frame does not help a lot for the capacity. On the contrary, from the curves “PDB (10, 10), PER (0.5%, 1%)” and “PDB (10, 10), PER (1%, 1%)” we can see that the successful rate of I-frame is the key factor in this modelling. From the dimension of PDB requirements we that if we apply PDB (17, 9) rather than PDB (10, 10), a huge capacity increase is observed. Note that when α is smaller, the size difference of an I frame and a P frame will be smaller. In this case, applying different requirements will have smaller impact on the system capacity. The effect of different scheduling algorithm can be observed from the solid curves and the dashed curve which shows a constant boost on the capacity irrespective of the I/P size difference.

Figure 7: System capacity curves for the downlink traffic of AR/VR in FR1 using IDR model

Observation 12: In the IDR model, relaxing the PER budget for P frame provides negligible capacity gain.

Observation 13: In the IDR model, relaxing the PER budget for I frame provides observable capacity gain.

Observation 14: In the IDR model, longer PDB for I frame and short PDB for P frame can increase the capacity.

Observation 15: In the IDR model, the impact on the capacity with different requirements becomes limited when the size difference of an I frame and a P frame is small.

Observation 16: In the IDR model, the capacity gain of delay-aware proportional-fair scheduling algorithm compared to the legacy proportional-fair is invariant of the I/P size difference.

Uplink Simulation Results

Uplink traffic model and requirements

For CG and VR services, the uplink traffic is modelled as a single stream which carries the information of pose/control. For AR services, the uplink traffic could have multiple streams and the detail is still under discussion. In this contribution, we model the AR uplink traffic by video data only, and limit the packet delay budget to be 30 ms. The detailed traffic model assumptions and requirements are captured in Table 9.

Table 9: UL Traffic model parameters and requirements for pose control and video

|  |  | Pose/Control | Video |
| --- | --- | --- | --- |
| UL traffic | Data rate | 25 KBps | 10 Mbps |
|  | Packet size distribution | fixed | Truncated Gaussian |
|  | Avg. data packet size (Bytes) | 100 | 21845 |
|  | Data packet size STD (Bytes) | N/A | 2293 |
|  | Max packet size (Bytes) | N/A | 32768 |
|  | Min packet size (Bytes) | N/A | 10922 |
|  | Packet format | UDP |  |
|  | Packet arrival rate (sec) | 250 | 60 |
|  | Packet delay budget | 10ms | 30ms |
|  | Reliability requirement | 99% | 99% |

CG/VR

Figure 8 shows the system capacity results for the uplink traffic of CG/VR services. We can see that the capacity number is quite adequate in FR1, with the number larger than 30 in all evaluated scenarios. The capacity number reduces in FR2 but the system can still accommodate at least 12 users per cell. Compared with downlink capacity performance with CG/VR traffic, we can see that the capacity bottleneck is likely to be the downlink capacity except for the CG services with 8Mbps data rates in FR2 indoor.

Figure 8: System capacity curves for the uplink traffic of CG/VR

Observation 17: The uplink capacity number for CG/VR with only pose/control traffic is larger than 30 in FR1.

Observation 18: The downlink capacity is the bottleneck of CG/VR services except for CG service with 8Mbps data rates in FR2 indoor.

AR

Figure 9 shows the system capacity results for the uplink traffic of AR services. Compare to CG/VR services, the uplink capacity of AR with 10Mbps video data rates quite limited, which is 9, 2 and 5 users per cell in FR1 for Dense Urban, UMa and Indoor Hotspot, respectively. In FR2, the system capacity for the AR uplink traffic is also very poor, indicating that the uplink traffic could be the bottleneck in AR services for both FR1 and FR2.

Figure 9: System capacity curves for the uplink traffic of AR

Observation 19: The uplink traffic of AR with 10Mbps data rates could be the bottleneck of the system capacity in both FR1 and FR2.

Power Consumption Evaluations

Cloud gaming and AR/VR services are of high data rates, having short data inter-arrival time and stringent data latency requirement. The traffic characteristics make those services much more power-consuming than other existing applications like VoIP or instant messaging. The most difficult part for power saving in Cloud gaming and AR/VR is to save power while maintaining an acceptable user satisfaction rate.

In this section, we provide the preliminary power evaluation results for cDRX and other DCI-based power saving schemes including Rel-15 and Rel-16 bandwidth part framework based power saving schemes and the potential enhancement in Rel-17.

Firstly, we consider the power saving scheme for downlink as following cases:

- Case 1 (baseline): No cDRX

- Case 2: cDRX.

  - Notation: (x, y, z) = (cycle length, on-duration timer, inactivity timer) msec

- Case 3: Rel-16 BWP framework based power saving schemes

  - Apply cross-slot scheduling (k0 = 2) and MIMO layer adaptation after BWP timer expires.

- Case 4: Rel-16 BWP framework based power saving schemes

  - Apply cross-slot scheduling, MIMO layer adaptation, and change of control monitoring periodicity (50% reduction) after BWP timer expires

- Case 5: Rel-17 DCI-based PDCCH adaptation (retransmission-aware) in our previous contribution [3]

  - After receiving DCI indicate to switch to power saving setting, UE changes control-monitoring periodicity (50% reduction)

Figure 10 shows the power evaluation results for case 2-cDRX (10, 5, 5) under different scenario and application. 10ms is chosen as the cycle length to match the frequent data arrival. And the capacity shown in Figure 10 is the maximum number of UEs that are satisfied with both downlink and uplink traffic.

In the aspect of power consumption and outage rate, there are only slight difference between these scenarios/application.  About 5% power saving gain can be obtained by adopting cDRX(10,5,5), while the outage rate increases significantly. The power saving gain of cDRX is limited and the use of cDRX leaves no space in terms of outage rate for other power saving techniques.

Figure 10: Power consumption and outage rate under different cDRX configurations for DL traffic

In our previous contribution for Rel-17 power saving [3] we have proposed a Rel-17 retransmission-aware method to enhance the existing scheduling DCI-based power saving solution. Figure 11 shows the power consumption comparison between the cases mentioned above.

Notice that with Rel-15 and Rel-16 power saving solution, the power saving gain is larger than the use of cDRX with little increment of the outage rate which is about 1%. When we apply the Rel-17 enhancement, the power saving gain of 12.86% compared with the baseline is achieved and with only minor increase of the outage rate.

Figure 11: Power consumption and outage rate between No cDRX, Rel-15 & Rel-16 DCI-based power saving and Rel-17 enhancement for DL traffic

Observation 20: The power saving gain for cDRX used in data-intensive services like AR/VR for DL traffic is limited and it can lead to larger increase of outage rate.

Observation 21: The retransmission-aware DCI-based power saving adaptation can provide considerable power saving gain with acceptable increase of the outage rate for DL traffic.

After that, we consider the uplink power saving schemes. According to our previous observation, cDRX is not applicable for data-intensive services. Thus, we consider the power saving schemes for uplink:

- Case 1 (baseline): No cDRX

- Case 2: Apply cross-slot scheduling (k0 = 2)

- Case 3: Rel-17 DCI-based PDCCH adaptation (retransmission-aware) in our previous contribution [3]

- Case 4: Apply cross-slot scheduling (k0 = 2) and Rel-17 DCI-based PDCCH adaptation (retransmission-aware) in our previous contribution [3]

And the uplink transmit power is modelled by option 2 (linear interpolation in linear domain), and not less than 0 dBm. The power model of 0 dBm for UE with transit power less than 0 dBm.

In Figure 12, we can observe that applying cross-slot scheduling and Rel-17 enhancement can achieve the largest power saving gain. Rel-17 DCI-based retransmission-aware PDCCH adaptation is also well effective for uplink traffic service.

Figure 12: Power consumption and outage rate between No cDRX, cross-slots scheduling and Rel-17 enhancement for UL traffic of VR/CG

The trend of evaluation results for other scenarios/applications is similar. The detailed evaluation results for each scenarios/applications is attached in Appendix A and Appendix B.

Observation 22: The retransmission-aware DCI-based power saving adaptation can provide considerable power saving gain with acceptable increase of the outage rate for UL traffic.

Conclusion

In this contribution, we provided the evaluation results for Cloud gaming and AR/VR in both FR1 and FR2 including Dense Urban, UMa and Indoor Hotspot scenarios. We also provides the capacity results under carrier aggregation for the services with higher data rates. For the discussion of IDR and GDR modelling for two-stream traffic, we prioritize IDR model and also provide the preliminary results for IDR model. The system capacity can be summarized in Table 10 and Table 11 and our observations/proposals are listed as follows.

Observation 1: The downlink capacity result for Cloud gaming in FR1 is larger than 20 with 8Mbps data rates for all evaluated scenarios and is 13, 9 and 9 with 30Mbps data rates for Dense Urban, UMa and Indoor Hotspot, respectively.

Observation 2: For the downlink traffic evaluation in FR1, the capacity number in Indoor Hotspot is smaller than the capacity number in Dense Urban and UMa due to inferior antenna setting.

Observation 3: The downlink capacity results for Cloud gaming in FR2 is larger than 20 with 8Mbps data rates and is 11 with 30Mbps data rates for both scenarios.

Observation 4: Since the downlink capacity results for Indoor Hotspot in FR1 and FR2 are comparable for Cloud gaming service, the service can be supported via FR2 spectrum for indoor scenarios.

Observation 5: The downlink capacity result for AR/VR is poorer than the downlink capacity results of CG due to larger data rates and more stringent latency requirement.

Observation 6: The downlink capacity result for AR/VR in FR1 is 10, 8 and 8 with 30Mbps data rates and is 6, 4 and 4 with 45Mbps data rates for Dense Urban, UMa and Indoor Hotspot, respectively.

Observation 7: The downlink capacity result for AR/VR in FR2 is 10 with 30Mbps data rates and is 4 with 45Mbps data rates for both Dense Urban and Indoor Hotspot.

Observation 8: With only 100MHz transmission bandwidth, the downlink system capacity for AR/VR with 45Mbps data rates is poor in both FR1 and FR2. Enhancement like carrier aggregation can be utilized to increase the capacity.

Observation 9: CA can provide capacity gain and the system capacity with CA is the larger than the sum of the capacity of its component carriers.

Observation 10: With additional enhancements on CA, 20% capacity gain for Dense Urban can be observed compared to the CA without enhancements.

Observation 11: The adopted TDD patterns in supporting latency-sensitive services like AR/VR should consider the trade-off between the uplink and downlink opportunities and the uniformity of uplink slots.

Observation 12: In the IDR model, relaxing the PER budget for P frame provides negligible capacity gain.

Observation 13: In the IDR model, relaxing the PER budget for I frame provides observable capacity gain.

Observation 14: In the IDR model, longer PDB for I frame and short PDB for P frame can increase the capacity.

Observation 15: In the IDR model, the impact on the capacity with different requirements becomes limited when the size difference of an I frame and a P frame is small.

Observation 16: In the IDR model, the capacity gain of delay-aware proportional-fair scheduling algorithm compared to the legacy proportional-fair is invariant of the I/P size difference.

Observation 17: The uplink capacity number for CG/VR with only pose/control traffic is larger than 30 in FR1.

Observation 18: The downlink capacity is the bottleneck of CG/VR services except for CG service with 8Mbps data rates in FR2 indoor.

Observation 19: The uplink traffic of AR with 10Mbps data rates could be the bottleneck of the system capacity in both FR1 and FR2.

Observation 20: The power saving gain for cDRX used in data-intensive services like AR/VR for DL traffic is limited and it can lead to larger increase of outage rate.

Observation 21: The retransmission-aware DCI-based power saving adaptation can provide considerable power saving gain with acceptable increase of the outage rate for DL traffic.

Observation 22: The retransmission-aware DCI-based power saving adaptation can provide considerable power saving gain with acceptable increase of the outage rate for UL traffic.

Proposal 1: Adopt the (E, F, G, H) values for the GOP-based model of the I/P frames evaluation as (1, 1, 17, 9)

Table 10: Summary of the downlink system capacity results for baseline

| Scenarios | Dense Urban | UMa | Indoor Hotspot |
| --- | --- | --- | --- |
| FR1 1CC |  |  |  |
| DL Capacity for CG, 8Mbps | >20 | >20 | >20 |
| DL Capacity for CG, 30Mbps | 13 | 9 | 9 |
| DL Capacity for XR, 30Mbps | 10 | 8 | 8 |
| DL Capacity for XR, 45Mbps | 6 | 4 | 4 |
| FR2 1CC |  |  |  |
| DL Capacity for CG, 8Mbps | >20 | N/A | >20 |
| DL Capacity for CG, 30Mbps | 11 | N/A | 11 |
| DL Capacity for XR, 30Mbps | 10 | N/A | 10 |
| DL Capacity for XR, 45Mbps | 4 | N/A | 4 |

Table 11: Summary of the uplink system capacity results for baseline

| Scenarios | Dense Urban | UMa | Indoor Hotspot |
| --- | --- | --- | --- |
| FR1 1CC |  |  |  |
| UL Capacity for CG/VR | >30 | >30 | >20 |
| UL Capacity for AR, 10Mbps video | 10 | 4 | 5 |
| FR2 1CC |  |  |  |
| UL Capacity for CG/VR | >30 | N/A | 12 |
| UL Capacity for AR, 10Mbps video | 2 | N/A | <2 |

Reference

- RP-201145, “New SID on XR Evaluations for NR”, Qualcomm, RAN #88e

- R1-2105377, “Traffic Model for XR and CG”, MediaTek Inc., RAN1 #105-e

- R1-2100593, “On enhancements to DCI-based UE power saving during DRX active time”, MediaTek Inc., RAN1 #104-e

- Appendix A: Downlink Evaluation Results

Table 12. Evaluation results: DL, FR1 DU, CG-30Mbps

Table 13. Evaluation results: DL, FR1 DU, AR-45Mbps

Table 14. Evaluation results: DL, FR1 InH, AR-45Mbps

- Appendix B: Uplink Evaluation Results

Table 15. Evaluation results: UL, FR1 DU, CG-Pose Control Only

Table 16. Evaluation results: UL, FR1 InH, CG-Pose Control Only

Table 17. Evaluation results: UL, FR1 DU, AR-10Mbps

Table 18. Evaluation results: UL, FR1 InH, AR-10Mbps