3GPP TSG RAN WG1 #106-e	   	 	  	                	      R1-2107817

e-Meeting, Aug 16th – 27th, 2021

Agenda item:	8.1.2.3

Source:	LG Electronics

Title:	Enhancements on beam management for multi-TRP

Document for:	Discussion and Decision

Introduction

The approved WID [1] includes enhancements on MIMO for NR, and main target of the enhancements is to enhance multi-beam operation, multi-TRP transmission and reception, SRS and CSI measurement and reporting. Specifically for multi-TRP enhancements, there are four sub-topics, regarding reliability enhancements on PDCCH, PUSCH and PUCCH using multi-TRP, inter-cell multi-TRP operations, beam management for multi-TRP, and HST-SFN deployment, respectively.

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In this contribution, we discuss the enhancements on beam management for multi-TRP including simultaneous multi-TRP transmission with UE multi-panel reception.

Discussion

Beam measurement and reporting enhancements for per-TRP beam management

In previous meetings, enhancement on the beam measurement and reporting for M-TRP had been discussed and agreed as below:

Agreement Down-select at least one of the following options for beam measurement/reporting enhancement to facilitate inter-TRP beam pairing in RAN1 #104-e - Option 1: In a CSI-report, UE can report N>1 pair/groups and M>=1 beams per pair/group - Different beams in different pairs/groups can be received simultaneously - FFS: whether M is equal or can be different across different pair/group - Option 2: In a CSI-report, UE can report N(N>=1) pairs/groups and M (M>1) beams per pair/group - Different beams within a pair/group can be received simultaneously - Option 3: UE report M(M>=1) beams in N (N>1) CSI-reports corresponding to N report setting - Different beams in different CSI-reports can be received simultaneously - FFS: whether/how to introduce an association between different CSI-reports - FFS: whether/how to differentiate reported measurements for beams that are received simultaneously vs. beams that are not received simultaneously - Whether/how to introduce an indication along with the CSI-reports to indicate whether the beams in different CSI-reports can be received simultaneously - FFS: value of N and M in each option - FFS: Association between different beams in above options and different TRP/UE panels - FFS: Identify new use cases per option compared with R16 (including backhaul) - FFS: whether different beams in different pairs/groups/reports can be received by same spatial filter per option Agreement For beam measurement in support of M-TRP simultaneous transmission - Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously - Support M = 2 - Support extending the maximum value of N > 1, exact value FFS - N=1 and N=2 - FFS: Other values larger than 2 - FFS: Whether the UE could report beams are received with different RX beams - Further study the support of option 1 and option 3 - The above applies at least for L1-RSRP - FFS: L1-SINR Agreement - For beam measurement/reporting option 2, the maximum number of beam groups in a single CSI-report is a UE capability and may take value from Nmax = {1,2,3,4} in Rel.17. - FFS: If UCI payload reduction for Nmax>=2 is needed and if so, how - The number of beam groups (N) reported in a single CSI-report - Alt1: The value of N is configured by RRC signalling

For M-TRP beam measurement and reporting, option 2(group-based beam reporting) had been agreed to be supported. And, in the last meeting, it was agreed to support UE capability reporting for the maximum number of beam groups (Nmax) in a single CSI-report from Nmax = {1, 2, 3, 4}, where the number of beam groups (N) to be reported in a single CSI-report is configured by RRC signaling. For option 2, we prefer to keep the number of beams within a pair/group as 2 since up to 2 TRPs can be cooperated for M-TRP transmission, so M is 2.

Furthermore, more flexible beam reporting can be achieved with option 2, when ‘per-group reporting quantity’ is defined. For example, gNB may configure two beam pairs/groups, where group 1 is to report the best M-TRP beam pair based on option 2 and group 2 is to report any best beam pair to be received simultaneously in a conventional way. Then, it enables UE to report legacy group-based beam report in addition to the M-TRP beam report in a single report. Specifically, gNB can configure group 2 to report S-TRP best beam pair to be received simultaneously, considering fallback mode from M-TRP DL transmission in case of strong cross interference between M-TRP pair or severe blockage from one TRP. With above assessment, option 2 based solution can improve flexibility and expandability of group-based beam reporting.

Proposal #1: For option 2, gNB can configure ‘per-group reporting quantity’ for each N group to indicate reporting hypothesis/criteria per group

- ‘per-group reporting quantity’ can be at least either ‘M-TRP beam pair’ or ‘S-TRP beam pair’

Offline proposal 1.2.1: On reporting of information related to UE Rx panel/antenna group for beam measurement/reporting option 2, further study and decide if any of the following alternatives for additional UE indication in the report is to be supported in Rel.17 in RAN1#106b-e - Alt-1.0: UE reports UE panel ID per CMR within a group/pair - Alt-1.1: if reported beams within a beam group/pair are associated to the same or different RX spatial filters - Alt-1.2: maximum number of supported layers per CMR within a group/pair - Alt-1.3: whether two beams within a group/pair can be used for spatial multiplexing or diversity

Additionally, regarding “FFS: Whether the UE could report beams are received with different RX beams” on the above first agreement, it can be naturally resolved if received panel ID reporting per reported CMR is supported within a group in option 2. It had been discussed for several meetings as above captured offline proposal 1.2.1, we think Alt-1.0 can include the functionality of Alt-1.1 and/or Alt-1.3. When the Rx panel IDs for CMRs within a group are different, then gNB can be aware that the CMRs are received with different Rx beams. Subsequently, the CMRs can be associated to multiple UE panels.

Proposal #2: For option 2, support UE to report Rx panel ID per CMR within a pair/group.

Option 2 based solution is for UE to report the N best beam pairs/groups wherein different beams within a pair/group can be received simultaneously. We think option 2 can simply enable inter-TRP beam pairing in the beam reporting, if UE knows which CMR(s) are from which TRP in a CSI-ResourceConfig. And, if L1-SINR based beam reporting is supported for option 2, cross-beam (cross-TRP) interference can be easily exploited for best beam pair reporting. Specifically, In order to enable best beam pair/group reporting with consideration of cross-beam (cross-TRP) interference using option 2, NZP-CSI-RS resource(s) from a TRP (TRP#1) and NZP-CSI-RS resource(s) from another TRP (TRP#2) can be paired/grouped for both channel and interference measurement which can be configured as group candidate(s) as shown in Figure 1. In each pair/group, UE can simultaneously measure L1-SINR of TRP#1 (channel from NZP-CSI-RS of TRP#1, interference from NZP-CSI-RS of TRP2) and L1-SINR of TRP#2 (channel from NZP-CSI-RS of TRP#2, interference from NZP-CSI-RS of TRP#1) using multiple Rx panels, and then report best N pair(s)/group(s) of NZP-CSI-RS resources. It means that the best pair/group of transmission beams from two different TRPs can be reported with consideration of interference between those 2 TRPs.

Proposal #3: For option 2, support L1-SINR based beam pair/group reporting.

- Cross-beam (cross-TRP) interference can be considered for option 2.

  - UE can report best N beam pair(s), each of which corresponds to (NZP-CSI-RS of TRP# 1, NZP-CSI-RS of TRP #2).

Figure 1. Potential enhancements for group-based based beam reporting

Agreement On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”: - NOTE: the following has been agreed - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset. - FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets Agreement For CMR configuration for option 2, adopt - Alt-1: “set” Agreement The bitwidth of each SSBRI/CRI is determined based on the number of SSB/CSI-RS resources in the associated CMR resource set - FFS: specify the association between SSBRIs/CRIs in a reported group and CMR resource sets

In the last meeting, it was agreed that two CMR resource sets can be configured/associated with CMR resource setting. In this agreement, it is supposed that CMR resource(s) in a CMR resource set is to be transmitted from a specific TRP, and CMR resource(s) in the other CMR resource set is to be transmitted from the other TRP. There is one remaining issue regarding the extension to aperiodic CMR resource setting, that at most single CMR resource set can be selected via RRC for a single reportConfig of CSI-AperiodicTriggerState for aperiodic CSI triggering in the legacy specification. We think it should be enhanced to trigger two CMR resource sets per reportConfig of CSI-AperiodicTriggerState. It can be naturally supported when two different CMR resource sets can be configured/activated with a single reportConfig of CSI-AperiodicTriggerState.

Proposal #4: Two different CMR resource sets can be associated with a single reportConfig of CSI-AperiodicTriggerState to trigger both CMR resource sets for aperiodic CSI/beam reporting.

Regarding beam measurement and reporting enhancement, another aspect is to support BM functionality for a scenario with different TDD DL/UL configuration across different TRPs. For example, it is considerable to allow SRS to be configured in CSI-ResourceConfig for cross-link beam management, e.g., to control UE Tx beam to UE Rx beam interference when TRP#0 is in UL and TRP#1 is in DL, where this scenario can happen frequently in typical NR FR2 deployments.

Proposal #5: Consider beam measurement and reporting enhancement for different TDD DL/UL configuration across multiple TRPs.

Beam failure recovery for multi-TRP

Regarding BFR enhancement for M-TRP, it had been discussed in the last meeting including beam failure recovery request procedure.

Agreement - Support S-DCI and M-DCI in TRP-specific BFR in Rel.17 - S-DCI is low priority, M-DCI is high priority - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization Agreement - Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs. - FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC. Agreement On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 Agreement Adopt the following beam failure detection criteria for each BFD-RS set - The physical layer in the UE assesses the radio link quality per BFD-RS set and indicates the BFD-RS set index to higher layers every X ms, if the hypothetical PDCCH BLER of all BFD-RS in the corresponding set of BFD-RS is higher than a threshold - X is max{minimal periodicity of BFD RS in the set, 2ms} Agreement A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group - NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group Agreement For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs, - Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS - Alt-3: Leave it up to UE implementation - Note: PUCCH-SR resource is PUCCH resource carrying SR FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1) Agreement Select one of the following alternatives with possible modification in RAN1#106-e - Alt 2.5.2 A: - On PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, there is no consensus to adopt alt-1 or alt-2. PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 B: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 2 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 C: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 D: - Revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

In typical M-TRP scenario, one TRP(TRP#1) may transmit system information, paging and RACH message and another TRP(TRP#2) just transmits UE dedicated PDSCH to improve DL throughput without broadcast information. According to the current specification, if hypothetical BLERs of CORESET beams for TRP#1 are above a threshold and not for all of CORESET beams within a BWP, beam failure event may not occur (i.e., BF is claimed only when all CORESET TCIs within a BWP fail), but UE cannot receive broadcast information such as SIB, unfortunately. In that regard, two approaches can be considered for BFD in M-TRP scenario.

Approach 1: BFD is performed on the primary TRP only

Approach 2: BFD is performed per TRP

For Approach 1, BFD needs to be enhanced to when all of CORESET beams for default/primary TRP are in beam failure event (i.e., but not for all of the CORESET beams within a CC/BWP).

For Approach 2, two separated BFR procedures can be defined within a CC/BWP, i.e., BFD is performed per CORESET pool or BFD-RS set. New beam RS candidates can be configured separately for each TRP.

Both Approach 1 and Approach 2 are valid depending on different M-TRP operation scenarios. So far, the focus of discussion was on Approach 2 while Approach 1 needs also to be supported. Regarding TRP-specific BFD-RS set, we think both implicit and explicit BFD-RS configuration should be supported for M-TRP specific BFR as legacy BFR procedure. In Rel-15/16 BFR discussion/decision, both methods were agreed because they have different use cases. Implicit BFD was firstly introduced as it is very natural to use PDCCH beam(s) to chase quality of serving beam(s). Explicit BFD was additionally introduced because gNB/TRP may use UE-specifically beamformed CSI-RS as serving beam RS for the UE, where the beam for the CSI-RS can be updated according to UE mobility by gNB implementation, e.g., via sounding signal from UE. Due to the reason, only RRC based CSI-RS was agreed for explicit BFD. In summary, implicit BFD is for cell-specifically beamformed or UE-group specifically beamformed CSI-RS/TRS and explicit BFD is for UE-specifically beamformed CSI-RS. Thus, we think both implicit and explicit BFD need to be supported to address various ways of network implementation.

In the previous meeting, both S-DCI and M-DCI based M-TRP operation are agreed to be supported for M-TRP specific BFR. Also, there were debates whether each implicit BFD-RS set should be determined by CORESET pool configuration or it can be determined by a separated configuration of CORESET groups. The former approach is straight-forward when two CORESET pools are configured in the BWP. On the other hand, the latter approach can also be supported when single CORESET pool exists in the BWP for S-DCI based M-TRP or for partial BFD within a TRP. For explicit BFD, gNB configures two BFD-RS sets explicitly. In this mode of operation, UE shall perform BFD per configured RS set so there is no need to define any restriction/dependency with respect to CORESET pool configuration or PDSCH TCI configuration.

Proposal #6: Support both implicit and explicit BFD-RS configuration.

- For implicit BFD, each BFD-RS set is determined per CORESET pool when two CORESET pools are configured in the BWP. When single CORESET pool is configured in the BWP, gNB can configure two CORESET groups for TRP-specific BFD.

- For explicit BFD, two BFD-RS sets are explicitly configured by gNB without any dependency on CORESET pool configuration or PDSCH TCI configuration.

Moreover, regarding the number of BFD-RS resources per BFD-RS set and BFD-RS resources across all BFD-RS sets per DL BWP, relevant UE capability reporting needs to be supported in order to address the increased UE complexity for tracking hypothetical BLER of more than two BFD-RSs. Since UE should perform BFD per independent RS set, the number of RSs per set also impact UE complexity as well as the total number of BFD-RSs. Therefore, UE capability report on the maximum number of BFD-RSs per BFD-RS set is needed to avoid some extreme configuration from gNB, e.g. UE reports total BFD-RS number as 6 then gNB configures 5 and 1 BFD-RSs, respectively for each BFD-RS set. For implicit BFD, it should also be clarified whether/how to define a BFD-RS selection rule when the total number of QCL type-D RSs across CORESETs exceeds the reported UE capability. For the selection rule, if needed, Rel-17 M-TRP PDCCH features such as PDCCH repetition and SFNed PDCCH need to be considered from the beginning in order to avoid excessive maintenance issues later.

Proposal #7: Support UE capability report on the number of BFD-RSs for M-TRP BFR.

- The maximum number of BFD-RSs per BFD-RS set can be also a UE capability.

- Clarify whether/how to define BFD-RS selection rule for implicit BFD when the total number of QCL type-D RSs across CORESETs exceeds the reported UE capability.

BFRQ procedure using up to two dedicated PUCCH-SR resources in a cell group was agreed to be supported. Regarding the “FFS: Whether PUCCH-SR resource can be configured with 2 spatial relations”, we think that the solution is useful especially for the case that the single PUCCH-SR resource in cell group is configured for M-TRP BFR. UE can choose one spatial relation among the two depending on TRP failure status, and both TRPs can monitor the single PUCCH-SR resource.

In the last meeting, the selection rule for PUCCH-SR resource for BFRQ was intensively discussed, and four alternatives were agreed. Defining a selection rule may be beneficial since it could let gNB know which TRP is in failure faster compared with the option that does not define any selection rule. If the selection rule should be supported in Rel-17, we think the partial solution like Alt 2.5.2 B or Alt 2.5.2 C should be avoided because gNB don’t know which case will happen to UE. To be specific, if the solution covers partial case (e.g., at most one BFD-RS set fails per CC, only for specific CC) and remaining cases are up to UE implementation, there is no way for gNB to know whether the transmitted PUCCH-SR resource is determined from UE PUCCH selection rule or UE implementation. If the selection rule is to be supported, it should be a complete selection rule which covers whole BF cases, including when different BFD-RS set is failed across different CCs. Otherwise, it could be cleaner to leave PUCCH selection up to UE implementation. One case to be addressed is when multiple CCs are in failure where TRP failure status are different across CCs. In this case, CC(s) in a same band with SpCell could be prioritized for the PUCCH-SR selection as DL-UL channels are quite correlated within a same band.

Proposal #8: PUCCH-SR resource can be configured with up to two spatialRelationInfo.

Proposal #9: In case of multiple dedicated PUCCH-SR resources in a cell group, either completely defining selection rule or not defining any selection rule is preferred.

- Do not support the selection rule which covers partial case, e.g., at most one BFD-RS set fails per CC, only for SpCell, etc.

In the previous meeting, it was agreed to support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs. In this case, it is much efficient to allow a single BFR MAC-CE for carrying both cell-specific BF status and TRP-specific BF status for SCell BFR. Regarding FFS point “FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.”, we think the simultaneous configuration of cell-specific and TRP-specific BFR in the same CC can be supported only for SpCell, meaning that Rel-15 BFR and Rel-17 BFR can be configured in SpCell with separated configuration. For SpCell, if single TRP is failed in the SpCell, Rel-17 TRP-specific BFRQ can be triggered with PUCCH-SR resource dedicated for BFR, and if both TRPs are failed in the SpCell, Rel-15 PRACH-based cell-specific BFRQ can be triggered. These two BFR procedures are independently configured and operated. CBRA based fallback behavior can also be supported without any additional modification of specification. One issue is when more than two CORESETs are configured in the SpCell with different TCIs. UE would select only two of the TCIs for BFD for the cell-specific BFR which may be transmitted from only one of the TRPs if we follow the legacy specification for the cell-specific BFD. Thus, this may need to be enhanced that cell-specific BFD-RS set can well include both BFD-RS sets for M-TRP BFR.

For SCell BFR procedure, Rel-17 TRP-specific BFR can include Rel-16 SCell BFR functionality, i.e., unified operation is possible, if the BFR MAC-CE can indicate that all TRPs are in beam failure. In other words, one or two failed TRP index(es) can be conveyed by BFR MAC-CE.

Proposal #10: Support simultaneous configuration of cell-specific and TRP-specifc BFR for SpCell.

Proposal #11: Support indication of both TRP failure as well as a single TRP failure via a single BFR MAC-CE.

- One or two failed TRP index(es) can be conveyed by BFR MAC-CE in case of SCell M-TPR beam failure.

  - TRP index can be BFD-RS set index from specification perspective

- One failed TRP index can be conveyed by BFR MAC-CE in case of SpCell M-TRP beam failure.

  - If both TRP is failed in SpCell, PRACH-based BFR procedure can be triggered.

Conclusion

In this contribution, we discuss our views for Rel-17 enhancements on beam management for multi-TRP including simultaneous multi-TRP transmission with UE multi-panel reception, and propose the followings based on the discussion.

Proposal #1: For option 2, gNB can configure ‘per-group reporting quantity’ for each N group to indicate reporting hypothesis/criteria per group

- ‘per-group reporting quantity’ can be at least either ‘M-TRP beam pair’ or ‘S-TRP beam pair’

Proposal #2: For option 2, support UE to report Rx panel ID per CMR within a pair/group.

Proposal #3: For option 2, support L1-SINR based beam pair/group reporting.

- Cross-beam (cross-TRP) interference can be considered for option 2.

  - UE can report best N beam pair(s), each of which corresponds to (NZP-CSI-RS of TRP# 1, NZP-CSI-RS of TRP #2).

Proposal #4: Two different CMR resource sets can be associated with a single reportConfig of CSI-AperiodicTriggerState to trigger both CMR resource sets for aperiodic CSI/beam reporting.

Proposal #5: Consider beam measurement and reporting enhancement for different TDD DL/UL configuration across multiple TRPs.

Proposal #6: Support both implicit and explicit BFD-RS configuration.

- For implicit BFD, each BFD-RS set is determined per CORESET pool when two CORESET pools are configured in the BWP. When single CORESET pool is configured in the BWP, gNB can configure two CORESET groups for TRP-specific BFD.

- For explicit BFD, two BFD-RS sets are explicitly configured by gNB without any dependency on CORESET pool configuration or PDSCH TCI configuration.

Proposal #7: Support UE capability report on the number of BFD-RSs for M-TRP BFR.

- The maximum number of BFD-RSs per BFD-RS set can be also a UE capability.

- Clarify whether/how to define BFD-RS selection rule for implicit BFD when the total number of QCL type-D RSs across CORESETs exceeds the reported UE capability.

Proposal #8: PUCCH-SR resource can be configured with up to two spatialRelationInfo.

Proposal #9: In case of multiple dedicated PUCCH-SR resources in a cell group, either completely defining selection rule or not defining any selection rule is preferred.

- Do not support the selection rule which covers partial case, e.g., at most one BFD-RS set fails per CC, only for SpCell, etc.

Proposal #10: Support simultaneous configuration of cell-specific and TRP-specifc BFR for SpCell.

Proposal #11: Support indication of both TRP failure as well as a single TRP failure via a single BFR MAC-CE.

- One or two failed TRP index(es) can be conveyed by BFR MAC-CE in case of SCell M-TPR beam failure.

  - TRP index can be BFD-RS set index from specification perspective

- One failed TRP index can be conveyed by BFR MAC-CE in case of SpCell M-TRP beam failure.

  - If both TRP is failed in SpCell, PRACH-based BFR procedure can be triggered.

References

[1] RP-193133, WID proposal for Rel.17 enhancements on MIMO for NR