3GPP TSG RAN WG1 #106-e                                                                                            R1-21xxxxx

e-Meeting, August 16th – 27th, 2021

Title:	Draft reply LS on physical layer aspects of small data transmission

Response to:	R2-2106561

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	RAN1

To:	RAN2

Cc:

Attachments:

# Contact Person:

# Name:	                Xiaohang CHEN

E-mail Address: chenxiaohang@vivo.com

# 1. Overall Description:

RAN1 would like to thank RAN2 for the LS on physical layer aspects of small data transmission in R2-2106561.

Regarding RA-SDT, RAN1 discussed the configuration parameters for the PRACH resource configuration when PRACH occasions are shared between SDT and non-SDT and when PRACH occasions are separately configured for SDT and non-SDT. RAN1 made the following agreements/conclusion for RA-SDT.

Proposal 1: For RA-SDT, when PRACH occasions are separate between SDT and non-SDT, the existing PRACH resource configurations can be separately configured, including 4-step RACH and/or 2-step RACH based SDT. - E.g. following parameters can be configured - PRACH Configuration Index - Time resources and frequency resources of PRACH resources, - Number of SSBs associated with a valid RO, - Number of contention based preambles per SSB per valid RO, - totalNumberofRA-Preambles - prach-RootSequenceIndex, - zeroCorrelationZoneConfig, - restrictedSetConfig - FFS if separate ROs need to be configured for RA-SDT of both 4-step RACH and 2-step RACH Proposal 2: For RA-SDT, when PRACH occasions are shared between SDT and non-SDT, at least following parameters can be configured, including 4-step RACH and/or 2-step RACH based SDT operation. - Number of contention-based preambles for SDT per SSB per valid RO - Starting position of the preambles for SDT per SSB per valid RO - FFS explicit indication, or implicit determination for preamble for SDT in shared ROs Note: whether starting position of the preambles for SDT per SSB per valid RO needs to be configured for RA-SDT in shared ROs is up to RAN2 discussion. Conclusion: Further discuss on the case when ROs are shared between SDT and non-SDT, but different RACH types have separate ROs after RAN2’s decision - Ask RAN2 about the possible combinations of RACH type for SDT and non−SDT Proposal 3: For RA-SDT, when PRACH occasions are shared between SDT and non-SDT, a PRACH mask can be configured to indicate a subset of ROs for RA-SDT. - The PRACH mask is used to indicate a subset of ROs associated with same SSB index for RA-SDT - The PRACH Mask indexes for 4-step SDT and 2-step SDT should be separately configured. Proposal 4: For RA-SDT in shared ROs and separate ROs with non-SDT, the power control parameters follow those for non-SDT, - i.e. preambleReceivedTargetPower and power ramping setting follow those for non-SDT.

Regarding the questions for CG-SDT on the working assumption and agreements, RAN1 made the following agreements.

Search space is configured for UEs performing CG-SDT Proposal 5: RAN1 confirms the working assumption on search space for CG-SDT. - UE-specific search space is configured for UEs performing CG-SDT. BWP for CG-SDT resource Proposal 6: RAN1 confirms the RAN2 agreement that CG-SDT resource can be configured on either initial BWP or separate SDT BWP. - For TDD if SDT is configured in a separate BWP, the BWP should includes the initial BWP to avoid BWP switching for receiving the SSBs L1 feedback for CG-SDT Proposal 7: RAN1 to further discuss the followings for L1 feedback for CG-SDT - Option 1: Support explicit L1 feedback for CG-SDT. - Option 1A: Reuse DFI based mechanism as introduced in Rel-16 NR-U - Option 1B: Use UL grant scheduling DG-PUSCH with same HARQ process ID as CG-SDT PUSCH - Option 2: Explicit ACK is not supported for CG-SDT. Reuse Rel-15 CG re-transmission for CG-SDT

# 2. Actions:

RAN1 respectfully asks RAN2 to take into account the above information.

# 3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting #106b-e				11th Oct – 19th Oct 2021		                              		e-meeting

TSG-RAN WG1 Meeting #107-e			11th Nov – 19th Nov 2021 	    		     			e-meeting