3GPP TSG RAN WG1 #106-e	                                                                 R1-210XXXX

e-Meeting, August 16th – 27th, 2021

Agenda item:	7.1

Source:	Moderator (CATT)

Title:	Summary of [106-e-NR-7.1CRs-05]

Document for:	Discussion and Decision

Introduction

This document is created to facilitate the email discussion of “[106-e-NR-7.1CRs-05] Issue#10: Discussion on cancellation of semi-static transmission due to dynamic transmission”. This email thread is triggered by the following draft CR.

R1-2106928	Discussion on cancellation of semi-static transmission due to dynamic transmission	CATT

Company views

For operation on a single carrier in unpaired spectrum, for an overlapping case of a DL/UL semi-static transmission and an UL/DL dynamic transmission which collides with semi-static DL/UL symbol(s) or SSB/valid PRACH occasion,

- Understanding 1: both the dynamic transmission colliding with semi-static DL/UL symbol(s) or SSB/valid PRACH occasion and the semi-static transmission overlapping with the dynamic transmission are not transmitted / received;

- Understanding 2: semi-static transmission could be transmitted / received while dynamic transmission colliding with semi-static DL/UL symbol(s) or SSB/valid PRACH occasion is dropped.

More detailed elaboration of the case and the above understandings can be found in R1-2106928.

Q1: Do you agree with understanding 1 or 2 above?

| Company | Understanding 1 or 2 | Comment |
| --- | --- | --- |
| vivo | Understanding 1 | Current spec. reflects understanding 1. |
| Huawei, HiSilicon | Und. 2 |  |

Q2: Do you think it necessary to clarify the intended UE behavior if there are different understandings among companies? If not, why?

| Company | Yes or No | Comment |
| --- | --- | --- |
| vivo | Not necessary for Rel-15 and Rel-16. Open for Rel-17 if there are different understandings among companies. | If there are different understandings, - For Rel-15 and Rel-16, it may not be possible to have a unified UE behavior due to NBC concern. - But we prefer have a clear UE behavior for future release. |
| Huawei, HiSilicon | OK to clarify. | If cannot be converged, our understanding is that for R15 the resulted effect would be up to UE implementation - similar issue as to RACH. For R16, a clarification would be preferred. |

Conclusion

To be added after the discussion.