3GPP TSG RAN WG1 #106-e	R1-210xxxx

# e-Meeting, August 16th – 27th, 2021

Agenda item:	7.1

Source:	Moderator (Samsung)

Title:	Summary of [106-e-NR-7.1CRs-10]

Document for:	Discussion and decision

# Introduction

This considers possible interpretations of the specifications [1] for the search space set overbooking/dropping procedure when a search space set is associated with multiple DCI format sizes, and intends to collect views on this issue and, if necessary, decide on a correction to [1]. The following discussion is based on analysis in [2].

# Round 1: Discussion

In Rel-15, it is possible that a CSS set or a USS set is associated with DCI formats having more than one size.

The following is captured in clause 10.1 of [1]. However, although monitoring of a PDCCH candidate implies decoding according to corresponding DCI formats in the search space set, that is not captured in the counting of PDCCH candidates.

A UE monitors a set of PDCCH candidates in one or more CORESETs on the active DL BWP on each activated serving cell configured with PDCCH monitoring according to corresponding search space sets where monitoring implies decoding each PDCCH candidate according to the monitored DCI formats.

A UE capability for PDCCH monitoring relates to a number of decoding operations the UE can perform within a time period such as a slot or a span. However, a PDCCH candidate can be related with different number of decoding operations depending on the search space set associated with the PDCCH candidate. Therefore, the definition of LMScss(i)(L) as counting PDCCH candidates for CSS set Scss(i), or LMSuss(j)(L) for USS set Suss(j), in the UE procedure for determining/resolving search space set overbooking needs to be clarified.

A first interpretation is that the above summations are for PDCCH candidates per search space set, as is currently defined in [1]. A second interpretation is that the above summations are for the number of decoding operations per PDCCH candidate per search space set.

Based on second interpretation, the number of sizes of DCI formats for a search space set needs to also be considered in the overbooking procedure. Basically, if for USS set Suss(j1) there is one DCI format size and for USS set Suss(j2) there are two DCI format sizes, using LMSuss(j1)(L) is correct but LMSuss(j2)(L) needs to be replaced by 2∙LMSuss(j2)(L) as LMSuss(j2)(L) is the total number of PDCCH candidates for Suss(j2), not the total number of decoding operations. Consequently, the search space set overbooking procedure in clause 10.1 of [1] needs to be amended.

Question 1: Companies are requested to provide views on the two interpretations, and any other input, for the counted variable in the UE procedure for search space set overbooking/dropping.

- Interpretation 1: the PDCCH candidates for each search space set are counted, or

- Interpretation 2: the decoding operations for DCI formats per PDCCH candidate are counted.

| Company | Comments |
| --- | --- |
| Samsung | Interpretation 2 |

If RAN1 concludes on Interpretation 2 above, companies are requested to provide input for why a TP, such as the following from [3], is needed or is not needed.

Question 2: If Interpretation 2 is concluded, please explain why a TP to [1] is or is not needed.

| Company | Comments |
| --- | --- |
| Samsung | A TP is needed because [1] captures “the number of counted PDCCH candidates for monitoring for a CSS/USS set” (the “counted” does not relate to this discussion) while [1] also captures that “monitoring implies decoding each PDCCH candidate according to the monitored DCI formats” (implying Interpretation 2). Therefore, the number of DCI format sizes needs to scale “the number of counted PDCCH candidates for monitoring for the CSS/USS set” but there is currently no association between the number of sizes of DCI formats and “the number of counted PDCCH candidates for monitoring for a CSS/USS set”. |

## 10.1 UE procedure for determining physical downlink control channel assignment < unchanged text is omitted> Denote by , , the number of counted PDCCH candidates for monitoring for CSS set and by , , the number of counted PDCCH candidates for monitoring for USS set , for CCE aggregation level L. For the CSS sets, a UE monitors MPDCCHcss=i=0Icss-1d(i)∙LMScss(i)(L)PDCCH candidates requiring a total of CPDCCHcss non-overlapping CCEs in a slot or in a span, where d(i) is a number of sizes for DCI formats for CSS set i after alignment of DCI format sizes [5, 38.212]. < unchanged text is omitted> Set Set Set d(j) to a number of sizes for DCI formats for USS set j after alignment of DCI format sizes Set while d(j)∙ AND allocate d(j)∙ PDCCH candidates for monitoring to USS set MPDCCHuss=MPDCCHuss-d(j)∙LMSuss(j)(L); ; ; end while

# References:

- TS 38.213, v16.6.0 “NR; Physical layer procedures for control”

- R1-2106856, “Discussion on counting PDCCH candidates in overbooking procedure,” Samsung

- R1-2106857, “Correction on counting PDCCH candidates in overbooking procedure,” Samsung.