3GPP TSG RAN WG1 Meeting #106-e	R1-21xxxx

E-meeting, August 16th – 27th, 2021

Agenda Item:	7.2.10

Source:	Huawei, HiSilicon

Title:	Moderator summary of [106-e-NR-MRDC-CA-03]

Document for:	Discussion and Decision

# Introduction

This contribution is feature lead summary on email discussion on corrections to 38.213 on SCell dormancy for power saving (R1-2106514 and R1-2106515).

# Email discussion

## Change 1: Remove “one or both” in TS38.213 for SCell dormancy indication by DCI format 0_1/1_1

It was proposed in [1][2] that According to 38.331, if configured, DCI format 0_1 and 1_1 are configured simultaneously for a search space set. According to 38.212, if the higher layer parameter(s) dormancyGroupWithinActiveTime are configured, both DCI format 0_1 and 1_1 should contain the field of "SCell dormancy indication" which has at least 1 bit. Therefore, the current description of "if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field" causes misunderstanding that it allows only one of the DCI format 0_1 and DCI format 1_1 includes a SCell dormancy indication field.

### 38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field, - the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by dormancyGroupWithinActiveTime, - each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells - if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0 - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part=======

Please provide your input/views on the proposed change#1:

| Company | Comment |
| --- | --- |

## Change 2:  Remove SCell dormaincy indiction with exception of “indication of SPS PDSCH release” since SCell dormancy indication is only supported for DCI format 1_1 with CRC scrambled by C-RNTI or MCS-RNTI (no CS-RNTI).

It was proposed in [1][2] that considering a SPS PDSCH release indication is carried by a DCI with CRC scrambled by a CS-RNTI, and a PDCCH is considered as a Case 2 PDCCH only if the CRC of DCI format 1_1 is scrambled by a RNTI which includes C-RNTI and MCS-C-RNTI, therefore, the description of “the UE considers the DCI format 1_1 as not indicating a SPS PDSCH release” should be removed to avoid confusion on the understanding.

### 38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1 the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of ======skipped part=======

Please provide your input/views on the proposed change#2:

| Company | Comment |
| --- | --- |

## Change#3: Corrections on configuration of dormant BWP needed for SCell dormancy

It was proposed in [1][2] that for a configured SCell, if the higher layer parameters dormantBWP-Id and/or firstWithinActiveTimeBWP-Id are not configured, when a Case 2 PDCCH is detected, the UE cannot execute the specified procedure, considering the UE does not have dormant BWP and first non-dormant BWP on the SCell. In order to correct this issue, it is proposed in [1][2] to restrict that the specified procedure only applies to the activated SCell(s) configured with the higher layer parameters dormantBWP-Id and firstWithinActiveTimeBWP-Id.

### 38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of - modulation and coding scheme - new data indicator - redundancy version and of - HARQ process number - antenna port(s) - DMRS sequence initialization as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where, for an activated SCell configured with dormantBWP-Id and firstWithinActiveTimeBWP-Id, - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding the activated SCell - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding the activated SCell, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for a corresponding the activated SCell, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part=======

Please provide your input/views on the proposed change#3:

| Company | Comment |
| --- | --- |

# Conclusions

# References

- R1-2106514	Discussion on corrections of Scell dormancy for power saving	Huawei, HiSilicon

- R1-2106515	Corrections of Scell dormancy for power saving	Huawei, HiSilicon