3GPP TSG RAN WG1 Meeting #106-e			R1-210xxxx

e-Meeting, August 16-27, 2021

Agenda Item:	7.2.2

Source:	Moderator (Nokia)

Title:	Feature lead summary#1 on NR-U Channel Access Procedures

Document for:	Discussion and Decision

# Introduction

The corrections proposed for NR-U Channel Access Procedures at RAN1#106-e are summarized in Section 2:

# Preparation phase

The correction proposed in TDocs [1-7] are summarized below.

| TDoc | CR summary | FL recommendation |
| --- | --- | --- |
| R1-2106441 | - Change of COT initialization according to MIIT consultation in 37.213 Add an optional value of duration Tf = 18us in section 4.1.1 and section 4.2.1.1 to meet the minimum idle channel assessment of at least 27us in China for LBE device. Add an optional value of sensing duration Tmc =27us in section 4.1.6.2 and section 4.2.1.0.4 to meet the minimum idle channel assessment of at least 27us in China for LBE device. The channel sensing requirement for channel ci is changed from “Type 2 channel access ” to at least sensing interval Tmc in section 4.2.1.0.4, where Tmc = 25us by default or 27us in China for LBE device. Add an optional vaule of sensing slot duration Tsl = 18us in section 4.3 to meet the minimum idle channel assessment of at least 18us in China for FBE device. “9us” is deleted in Table 7.3.1.1.1-4A in TS 38.212 to align the changes in section 4.3 in TS 37.213 - Alignment of the unit of cg-COT-SharingOffset-r16 in 38.331 and 37.213 In Section 4.1.3, “DL transmission X= cg-COT-SharingOffset-r16 “->“DL transmission X= cg-COT-SharingOffset-r16*14” - Editorial changes to 37.213 In Section 4.2.1.0.3, "DL transmission" -> "DL transmissions" In Section 4.2.1.2.1, " one slot sensing slot" -> "one sensing slot" In Section 4.2.2.2, “not been for”-> “not been used for “ In Section 4.3 “assess”->”access”, “Clause”->”clause” | 1) Discuss in the meeting together with * 2) Straightforward correction 3) Straightforward corrections |
| R1-2106507 | Discussion related to the issue 1) in R1-2106441 | * |
| R1-2106510 | Change of COT initialization according to MIIT consultation in 38.212 “9us” is deleted in Table 7.3.1.1.1-4A to align the changes in section 4.3 in TS37.213 | * |
| R1-2107010 | Editorial correction on the channel access for type-2 random access Remove the duplicated sentence “as defined in Table 7.3.1.1.1-4 in TS 38.212” in Clause 8.2A. | Straightforward correction (also included in R1-2108051) |
| R1-2107236 | Definition of “a channel” in Section 4.0 of 37.213 To make correspondence between the ‘channel’ and the 20MHz band defined in RAN4 by adding “corresponding to the sub-band defined in [TS 38.101-1]. ## 4.0 General Unless otherwise noted, the definitions below are applicable for the following terminologies used in this specification: - A channel, corresponding to a sub-band defined in [TS 38.101-1], refers to a carrier or a part of a carrier consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum. | Discuss in the meeting |
| R1-2108049 | Correction on channel access type and CP extension indication - Removed the if condition so that “otherwise” is correctly against operation with shared spectrum channel access. - Add” if ChannelAccessMode-r16 = "dynamic" is provided” to title of Table 7.3.1.1.1-4. - Correct typo of Table 7.3.1.1.1.4A to Table 7.3.1.1.1-4A. | Straightforward correction |
| R1-2108051 | Correction on ChannelAccess-CPext field in RAR Remove the duplicated sentence “as defined in Table 7.3.1.1.1-4 in TS 38.212” in Clauses 8.2 and 8.2A. | Straightforward correction (partial overlap with (R1-2107010) |

# References

| 1 | R1-2106441 | Changes of channel access procedure in TS 37.213 | Huawei, HiSilicon |
| --- | --- | --- | --- |
| 2 | R1-2106507 | Discussion on the impact of MIIT consultation to channel access procedure | Huawei, HiSilicon |
| 3 | R1-2106510 | Changes of channel access procedure according to MIIT regulation in TS 38.212 | Huawei, HiSilicon |
| 4 | R1-2107010 | Editorial correction on the channel access for type-2 random access | ZTE, Sanechips |
| 5 | R1-2107236 | Draft CR on LBT bandwidth | OPPO |
| 6 | R1-2108049 | Correction on channel access type and CP extension indictaion | ASUSTeK |
| 7 | R1-2108051 | Correction on ChannelAccess-CPext field in RAR | ASUSTeK |