3GPP TSG RAN WG1 Meeting #106-e			R1-210xxxx

e-Meeting, Aug 16-27, 2021

Agenda Item:	7.2.2

Source:	Moderator (Huawei)

Title:	Feature lead summary#1 on NR-U HARQ maintenance

Document for:	Discussion and Decision

# Introduction

Corrections on HARQ and multi-PUSCH scheduling for NRU have been submitted at RAN1#106 e-meeting.

Section 2 provides a summary of the proposed corrections, and the feature lead’s recommendation for discussion in the preparation phase.

# Preparation phase

The corrections proposed in 5 Tdocs to RAN1#106e are summarized in the table below.

| CR | CR summary | FL recommendation |
| --- | --- | --- |
| R1-2106508 | #### Correction on RRC parameter name of HARQ-ACK codebook in TS 38.213 #### 9.1.3.3 Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission If a UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE determines HARQ-ACK information for multiplexing in a PUCCH transmission occasion according to the following procedure. | Straightforward correction |
| R1-2107232 | #### Draft CR on e-type 2 HARQ codebook #### 9.1.3.3 Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission If a HARQ-ACK transmission for a given PDSCH group with a given value of NFI is requested, the UE does not expect to receive a DCI format scheduling the given PDSCH group with a different value of NFI before the ending of the HARQ-ACK transmission. | Discussion is needed for clarifying UE behaviour for error cases or specifying a new behaviour in case of NFI toggling for an unreported PDSCH group |
| R1-2107235 | Draft CR on PDSCH-to-HARQ feedback timing indicator field values ### 9.2.3 UE procedure for reporting HARQ-ACK <Unchanged part omitted> For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For a DCI format, other than DCI format 1_0, scheduling a PDSCH reception or a SPS PDSCH release, or requesting Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception as described in Clause 9.1.4, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, as defined in Table 9.2.3-1. | Straightforward correction |
| R1-2107695 R1-2107976 (not a CR) | Frequency hopping for multi-PUSCH scheduling with single DCI TS38.214 reads for both rel-15 and Rel-16: ### 6.3 UE PUSCH frequency hopping procedure […] One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission. R1-2107695 notes that it is not clear if PUSCH transmission due to multi-PUSCH scheduling by a single DCI is classified as "single-slot" or "multi-slot." R1-2107695 proposes that only intra-slot frequency hopping applies to PUSCH transmissions scheduled with a single DCI: - Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission and multiple PUSCH transmissions scheduled by a DCI. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission. R1-2107976 proposes to clarify that intra-slot frequency hopping is applicable to multi-PUSCH scheduling, while inter-slot frequency hopping is not applicable to multi-PUSCH scheduling. | Discuss to reach RAN1 common understanding on this issue. The CR may need a clearer formulation maybe linking to the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH |

# References

- R1-2106508, Correction on RRC parameter name of HARQ-ACK codebook in TS37.213, Huawei, HiSilicon

- R1-2107232, Draft CR on e-type 2 HARQ codebook, OPPO

- R1-2107235, Draft CR on PDSCH-to-HARQ feedback timing indicator field values, OPPO

- R1-2107695, Correction on frequency hopping for multi-PUSCH scheduling with single DCI, Ericsson Inc.

- R1-2107976, Discussion on frequency hopping for multi-PUSCH scheduling, vivo