3GPP TSG RAN WG1 #106-e	                                                                 R1-210XXXX

# e-Meeting, August 16th – 27th, 2021

Agenda item:	7.2.5

Source:	Moderator (Qualcomm)

Title:	Summary of [106-e-NR-L1enh-URLLC-01]

Document for:	Discussion and Decision

# Introduction

This document is created to facilitate the email discussion of “[106-e-NR-L1enh-URLLC-01] Issue#3: Correction on SRS resource configuration.  This email thread is triggered by the following draft CR.

R1-2107318	Remaining issues on PDCCH enhancements for URLLC	Qualcomm Incorporated

# Explanation of the problem

In the RAN1 #105_e meeting, an agreement was made to restrict the RRCs parameters associated with the SRS resource set in srs-ResourceSetToAddModListDCI-0-2 with usage equal to “codebook” or “noncodebook” to be the same as the RRC parameters associated with the SRS resource set configured in srs-ResourceSetToAddModList with the same usage.  The purpose is to make sure that the SRS resource set configured by srs-ResourceSetToAddModListDCI-0-2 only contain a subset of SRS resources that are configured by the SRS resources set in srs-ResourceSetToAddModList.

However, [1] argues that it is neither necessary nor correct to make all RRC parameters associated with the two SRS resource sets to be the same.  For example, [1] suggests to remove the restrictions of same RRC configuration for the following parameters associated with an SRS resource set configuration.

- srs-ResourceSetId

- srs-ResourceIdList

- aperiodicSRS-ResourceTrigger

- aperiodicSRS-ResourceTriggerList

Furthermore, the following CR on TS 38.212 was proposed.

##### 7.3.1.1.3	Format 0_2

< Unchanged parts are omitted >

-	SRS resource indicator – log2k=1minLmax, NSRS,0_2NSRS,0_2k or log2NSRS, 0_2 bits, where NSRS, 0_2 is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook', where the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook', respectively, except for the higher layer parameters ‘srs-ResourceSetId’, ‘srs-ResourceIdList’, ‘aperiodicSRS-ResourceTrigger’, and ‘aperiodicSRS-ResourceTriggerList’

-	log2k=1minLmax, NSRS,0_2NSRS,0_2k bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where NSRS, 0_2 is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'nonCodeBook', where the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'nonCodeBook', except for the higher layer parameters ‘srs-ResourceSetId’, ‘srs-ResourceIdList’, ‘aperiodicSRS-ResourceTrigger’, and ‘aperiodicSRS-ResourceTriggerList’, and

-	if UE supports operation with maxMIMO-LayersDCI-0-2 and the higher layer parameter maxMIMO-LayersDCI-0-2 of PUSCH-ServingCellConfig of the serving cell is configured, Lmax is given by that parameter

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	log2NSRS, 0_2 bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook, where NSRS, 0_2 is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codeBook', where the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codeBook', except for the higher layer parameters ‘srs-ResourceSetId’, ‘srs-ResourceIdList’, ‘aperiodicSRS-ResourceTrigger’, and ‘aperiodicSRS-ResourceTriggerList’.

< Unchanged parts are omitted >

##### 7.3.1.1.3	Format 0_2

< Unchanged parts are omitted >

-	SRS resource indicator – log2k=1minLmax, NSRS,0_2NSRS,0_2k or log2NSRS, 0_2 bits, where NSRS, 0_2 is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook', where the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook', respectively, except for the higher layer parameters ‘srs-ResourceSetId’, ‘srs-ResourceIdList’, ‘aperiodicSRS-ResourceTrigger’, and ‘aperiodicSRS-ResourceTriggerList’

-	log2k=1minLmax, NSRS,0_2NSRS,0_2k bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where NSRS, 0_2 is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'nonCodeBook', where the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'nonCodeBook', except for the higher layer parameters ‘srs-ResourceSetId’, ‘srs-ResourceIdList’, ‘aperiodicSRS-ResourceTrigger’, and ‘aperiodicSRS-ResourceTriggerList’, and

-	if UE supports operation with maxMIMO-LayersDCI-0-2 and the higher layer parameter maxMIMO-LayersDCI-0-2 of PUSCH-ServingCellConfig of the serving cell is configured, Lmax is given by that parameter

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	log2NSRS, 0_2 bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook, where NSRS, 0_2 is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codeBook', where the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codeBook', except for the higher layer parameters ‘srs-ResourceSetId’, ‘srs-ResourceIdList’, ‘aperiodicSRS-ResourceTrigger’, and ‘aperiodicSRS-ResourceTriggerList’.

< Unchanged parts are omitted >

For your reference, the RRC configuration for SRS resource set in Rel-16 is provided below.

# Company views

Companies are encouraged to provide comments on the following questions.

Q1: Do you agree with the intention of the CR described in Section 2 that the RRC parameters associated with the SRS resource set configured in srs-ResourceSetToAddModListDCI-0-2 and srs-ResourceSetToAddModList with the same usage can not all be configured the same? If not, any justifications?

| Company | Yes or No | Comment |
| --- | --- | --- |
| HW/HiSi | [No] | Tentative no support. In our view it does not seem needed to configure the parameters separately since the SRS resource set of 0_2 is intended to be a sub-set of 0_1. The specification is not broken, and this CR would be an optimization in our view. But we would like to hear the views from others. |
| Apple | [No] | We have a similar understanding as Huawei. |

Q2: If you answer to Q1 is yes, what are the RRC parameters that you think may or must be configured differently?

| Company | Yes or No | Comment |
| --- | --- | --- |

Q2: If you answer to Q1 is yes, any additional comments on the CR in Section 2?

| Company | Yes or No | Comment |
| --- | --- | --- |

# Conclusion

To be added after the discussion.

# References

- R1-2107318, Remaining issues on PDCCH enhancements for URLLC, Qualcomm, 3GPP WG1 106-e, Aug 16-27, 2021.