3GPP TSG-RAN WG1 Meeting #106-e	R1-210xxxx

e-Meeting, August 16th – 27th, 2021

Agenda Item:	7.2.8

Source:	Moderator (Huawei)

Title:	Summary of [106-e-NR-Pos-01] Replacement of cell terminology

Document for:	Discussion and decision

Introduction

This document provides the summary for [106-e-NR-Pos-01] on the replacement of cell terminology.

[106-e-NR-Pos-01] Email discussion/approval on replacement of cell terminology (Aspect #1) until August 20 – Su (Huawei)

The related submission of contributions includes

- R1-2106448	Draft CR on terminology correction to cell for positioning	Huawei, HiSilicon

- R1-2107991	Maintenance on Rel-16 NR positioning	vivo

The email discussion is divided into two rounds, with the intermediate summary at 23:59 UTC, Aug. 18.

General information

In [1], it is proposed to change the terminology “cell” in the descriptions of the higher layer parameters NR-DL-PRS-SFN0-Offset and dl-PRS-QCL-Info as shown below:

- In NR-DL-PRS-SFN0-Offset, the “transmitting cell” is changed to “DL PRS resource set”, and the “reference cell” is changed to “reference indicated by nr-DL-PRS-ReferenceInfo”.

- In dl-PRS-QCL-Info, “a non-serving cell” is changed to “not from any serving cell”.

The corresponding TP is provided below:

TS 38.214 #### 5.1.6.5 PRS reception procedure ========================= Unchanged parts ========================= A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by: ========================= Unchanged parts ========================= - NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the DL PRS resource set with respect to SFN0 slot 0 of reference provided by nr-DL-PRS-ReferenceInfo. ========================= Unchanged parts ========================= A DL PRS resource is defined by: ========================= Unchanged parts ========================= - dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS either from a serving cell or not from any serving cell, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a serving or non-serving cell. ========================= Unchanged parts =========================

In [2], the similar change was proposed, for the description of ‘cell’ in section 5.1.6.5 in TS38.214, it is suggested to use the unified description just like ‘TRP’ in the specification, e.g., ‘dl-PRS-ID’ as provided in TP below:

TS 38.214 (clause 5.1.6.5) < Unchanged parts are omitted > NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the transmitting cell each dl-PRS-ID with respect to SFN0 slot 0 of the reference indicated by dl-PRS-ID in higher layer parameter nr-DL-PRS-ReferenceInforeference cell. < Unchanged parts are omitted >

Discussion

NR-DL-PRS-SFN0-Offset

The reason for the change was believed to be removing “cell” terminology, since from UE reception of PRS perspective, TRP and cell should be differentiated. [1] also argued that PRS could be transmitted from a TRP-only TP where there is no cell associated with it.

- Proposal: Select one alternative to fix the cell terminology in NR-DL-PRS-SFN0-Offset

- Alt.1

- NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the DL PRS resource set with respect to SFN0 slot 0 of reference provided by nr-DL-PRS-ReferenceInfo.

- Alt.2

NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the transmitting cell each dl-PRS-ID with respect to SFN0 slot 0 of the reference indicated by dl-PRS-ID in higher layer parameter nr-DL-PRS-ReferenceInforeference cell.

- Alt.3 Others

| Company | Alternative | Comments |
| --- | --- | --- |

dl-PRS-QCL-Info

The reason for the change was that PRS/PRS QCL could refer to the PRS resource from the same PRS-only TP, which does not necessarily need to be a cell.

- Proposal: Decide whether to adopt the following change.

- dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS either from a serving cell or not from any serving cell, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a serving or non-serving cell.

| Company | Yes/No | Comments |
| --- | --- | --- |

Conclusion

TBD