3GPP TSG RAN WG1 #106e		R1-210zzzz

e-Meeting, August 16th – 27th, 2021

Source:	Moderator (Intel Corporation)

Title:	Summary of the preparation phase for Rel.16 NR positioning maintenance

Agenda item:	7.2.8

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide review of the remaining opens identified for Rel.16 NR positioning framework based on submitted contributions to RAN1#106e meeting.

The identified issues and draft CRs / TPs are summarized in this document. Finally, proposal for RAN WG1 e-mail discussion(s) on Rel.16 NR positioning maintenance is made.

# Remaining Opens

In this section, we summarize submitted TPs / draft CRs for identified open aspects on NR positioning maintenance based on review of contributions [1]-[9].

## Aspect #1: Replacement of Cell Terminology

In [1], it is proposed to change the terminology “cell” in the descriptions of the higher layer parameters NR-DL-PRS-SFN0-Offset and dl-PRS-QCL-Info as shown below:

- In NR-DL-PRS-SFN0-Offset, the “transmitting cell” is changed to “DL PRS resource set”, and the “reference cell” is chagned to “reference indicated by nr-DL-PRS-ReferenceInfo”.

- In dl-PRS-QCL-Info, “a non-serving cell” is changed to “not from any serving cell”.

The corresponding TP is provided below:

TS 38.214 #### 5.1.6.5 PRS reception procedure ========================= Unchanged parts ========================= A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by: ========================= Unchanged parts ========================= - NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the DL PRS resource set with respect to SFN0 slot 0 of reference provided by nr-DL-PRS-ReferenceInfo. ========================= Unchanged parts ========================= A DL PRS resource is defined by: ========================= Unchanged parts ========================= - dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS either from a serving cell or not from any serving cell, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a serving or non-serving cell. ========================= Unchanged parts =========================

In [7], the similar change was proposed, for the description of ‘cell’ in section 5.1.6.5 in TS38.214, it is suggested to use the unified description just like ‘TRP’ in the specification, e.g., ‘dl-PRS-ID’ as provided in TP below:

TS 38.214 (clause 5.1.6.5) < Unchanged parts are omitted > NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the transmitting cell each dl-PRS-ID with respect to SFN0 slot 0 of the reference indicated by dl-PRS-ID in higher layer parameter nr-DL-PRS-ReferenceInforeference cell. < Unchanged parts are omitted >

FL response:

- It is proposed to discuss and decide whether/how to clarify this aspect

## Aspect #2: DL PRS Antenna Ports + Editorial SRS Corrections

In [3], it is noticed that in TS 38.211, the antenna ports description lacks that for DL PRS, particularly for the cases when (slot-level) repetition is configured, whereas the DM-RS have the dedicated restriction for the same port “in the same slot”. Therefore, 1) the description for PRS antenna ports is added and 2) some editorial corrections to the SRS are provided as shown below:

### 4.4.1 Antenna ports ========================= Unchanged parts ========================= For DM-RS associated with a PBCH, the channel over which a PBCH symbol on one antenna port is conveyed can be inferred from the channel over which a DM-RS symbol on the same antenna port is conveyed only if the two symbols are within a SS/PBCH block transmitted within the same slot, and with the same block index according to clause 7.4.3.1. For PRS, the channel over which a PRS symbol on one antenna port is conveyed can be inferred from the channel over which a PRS symbol on the same antenna port is conveyed only if the two symbols are within a DL PRS resource within the same slot. Two antenna ports are said to be quasi co-located if the large-scale properties of the channel over which a symbol on one antenna port is conveyed can be inferred from the channel over which a symbol on the other antenna port is conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters. ========================= Unchanged parts ========================= ##### 6.4.1.4.2 Sequence generation ========================= Unchanged parts ========================= The sequence group u=fghns,fμ,l'+nIDSRS mod 30 and the sequence number in clause 5.2.2 depends on the higher-layer parameter groupOrSequenceHopping in the SRS-Resource IE or the SRS-PosResource IE. The SRS sequence identity is given by the higher layer parameter sequenceId in the SRS-Resource IE, in which case nIDSRS∈0, 1, …, 1023, or the SRS-PosResource IE, in which case nIDSRS∈0, 1, …, 65535. The quantity l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number within the SRS resource. ========================= Unchanged parts ========================= ##### 6.4.1.4.4 Sounding reference signal slot configuration For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity (in slots) and slot offset are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-Resource IE, or in the SRS-PosResource IE. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying SRS is transmitted as described in clause 11.1 of [5, TS 38.213]. ========================= Unchanged parts =========================

FL response:

- It is proposed to discuss and decide whether/how to clarify this aspect

## Aspect #3: Expected RSTD and RSTD Uncertainty

In [4], it is stated that according to the field descriptions in TS 37.355, nr-DL-PRS-ExpectedRSTD indicates the RSTD value that the target device is expected to measure between one TRP and the assistance data reference TRP. Therefore, the proposed correction aims to clarify that nr-DL-PRS-ExpectedRSTD and nr-ExpectedRSTD-Uncertainty are defined per pair of TRPs rather than per pair of DL PRS resource sets.

---- Unchanged texts omitted ---- #### 5.1.6.5 PRS reception procedure <Unchanged parts are omitted> For the case when measurement gap is configured, the UE DL PRS processing capability is defined in [TS 37.355]. For the purpose of DL PRS processing capability, the duration K msec of DL PRS symbols within P msec window corresponding to the maximum PRS periodicity in a positioning frequency layer, is calculated by - Type 1 duration calculation with UE symbol level buffering capability K=s∈SKs Ks=Tsend-Tsstart - Type 2 duration calculation with UE slot level buffering capability K=12μS - S is the set of slots based on the numerology of the DL PRS of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of the associated dl-PRS-ID of the DL PRS and the reference provided by nr-DL-PRS-ReferenceInfo. - For Type 1, Tsstart, Tsend is the smallest interval in msec within slot s corresponding to an integer number of OFDM symbols based on the numerology of the DL PRS of a serving cell that covers the union of the potential PRS symbols and determines the PRS symbol occupancy within slot s, where the interval Tsstart, Tsend considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of the associated dl-PRS-ID of the DL PRS and the reference provided by nr-DL-PRS-ReferenceInfo. - For Type 2, μ is the numerology of the DL PRS, and S is the cardinality of the set S. ---- Unchanged texts omitted ----

FL response:

- It is proposed to discuss and decide whether/how to clarify this aspect

## Aspect #4: DL PRS Reference Resources

In [5], it is noticed that, when network indicates a reference, it may optionally select a DL PRS Resource ID, a subset of DL PRS Resource IDs or a DL PRS Resource set. None of the three items is mandatory. However, the description in section 5.1.6.5 of TS 38.214 seems that the DL PRS Resource set ID should always be indicated. In addition, UE is allowed to use a single different DL PRS Resource to determine the reference. This option is missed in the spec, as only ‘different DL PRS resources’ is mentioned in section 5.1.6.5 of TS 38.214.

The following TP is proposed to address the raised aspect.

#### 5.1.6.5 PRS reception procedure ===================== Unchanged parts ===================== The UE may be indicated by the network that DL PRS resource(s) can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter nr-DL-PRS-ReferenceInfo. The reference indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncerainty. The UE expects the reference to be indicated whenever it is expected to receive the DL PRS. This reference provided by nr-DL-PRS-ReferenceInfo may include a dl-PRS-ID, and optionally a DL PRS resource set ID, a single DL PRS resource ID or a list of DL PRS resource IDs [17, TS 37.355]. The UE may use different DL PRS resource(s) or a different DL PRS resource set to determine the reference for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference than indicated by the network, then it is expected to report the dl-PRS-ID, the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. ===================== Unchanged parts =====================

FL response:

- It is proposed to discuss and decide whether/how to clarify this aspect

## Aspect #5: UE Rx-Tx time difference measurements from different DL PRS resources

In [8], it is noticed that the UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different positioning frequency layers. The UE capability provides additional details which are not present in the 38.214 specs.

The following TP was provided to clarify that the UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements from different DL PRS resources configured with the same dl-PRS-ID, in the same positioning frequency layer:

---- Unchanged texts omitted ---- #### 5.1.6.5 PRS reception procedure <Unchanged parts are omitted> The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of dl-PRS-ID with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those dl-PRS-ID. The up to 4 measurements being performed on the same pair of dl-PRS-ID and all DL RSTD measurements in the same report use a single reference timing. The UE may be configured to measure and report, subject to UE capability, up to 8 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements from different DL PRS resources configured with the same dl-PRS-ID, in the same positioning frequency layer and corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different positioning frequency layers. ---- Unchanged texts omitted ----

FL response

- It is proposed to discuss and decide whether/how to clarify this aspect

## Aspect #6: Alignment with RAN4 on DL PRS Processing

In [7], it is noticed that based on current TS 38.133, in RSTD/RSRP/Rx-Tx time difference measurement period requirements, it is described that if more than one PRS periodicities are configured in PRS frequency layer i, the least common multiple of PRS periodicities among all DL PRS resource sets is used to represent the periodicity TPRS,i of DL PRS resource on frequency layer i and further derive the measurement period of that PRS frequency layer i.

It is proposed to adopt one of the following options and related text proposals into TS38.214 regarding PRS processing capability.

Option 1: change the descriptions related to ‘P msec window’ to align with RAN4 specification.

< Unchanged parts are omitted > For the case when measurement gap is configured, the UE DL PRS processing capability is defined in [TS 37.355]. For the purpose of DL PRS processing capability, the duration K msec of DL PRS symbols within P msec window corresponding to the maximum PRS periodicity in a positioning frequency layer described in Clause 9.9.2.5 [11, TS 38.133], is calculated by… < Unchanged parts are omitted >

Option 2: delete the descriptions related to ‘P msec window’.

< Unchanged parts are omitted > For the case when measurement gap is configured, the UE DL PRS processing capability is defined in [TS 37.355]. For the purpose of DL PRS processing capability, the duration K msec of DL PRS symbols within P msec window corresponding to the maximum PRS periodicity in a positioning frequency layer, is calculated by… < Unchanged parts are omitted >

In [9], the following changes were proposed to align with RAN4 specification on DL PRS processing:

- Evaluation window of P should no longer be corresponding to the maximum PRS periodicity, and the simplest way is to cite the RAN4 terminology TPRS,i.

- Duration calculation equation, the counted PRS slot/symbol should be mapping to those PRS that are neither muted nor outside the MG. From RAN1 perspective, those PRS could be referred to as “PRS to process”, as UE is not expected to process those PRS outside the MG or muted.

- Change the wording “For the purpose of DL PRS processing capability” since it may sometimes be interpreted inaccurately. The suggested wording can be “For the purpose of DL PRS processing”.

- =========================== Unchanged parts =========================== For the case when measurement gap is configured, the UE DL PRS processing capability is defined in [TS 37.355]. For the purpose of DL PRS processing, the duration K msec of DL PRS symbols within P msec window corresponding to TPRS,i as defined in clause 9.9 of [11, TS 38.133] in positioning frequency layer i, is calculated by - Type 1 duration calculation with UE symbol level buffering capability K=s∈SKs Ks=Tsend-Tsstart - Type 2 duration calculation with UE slot level buffering capability K=12μS - S is the set of slots based on the numerology of the DL PRS of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources to process considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets. - For Type 1, Tsstart, Tsend is the smallest interval in msec within slot s corresponding to an integer number of OFDM symbols based on the numerology of the DL PRS of a serving cell that covers the union of the potential PRS symbols to process and determines the PRS symbol occupancy within slot s, where the interval Tsstart, Tsend considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS resource sets (target and reference). - For Type 2, μ is the numerology of the DL PRS, and S is the cardinality of the set S. - =========================== Unchanged parts ===========================

FL response

- It is proposed to discuss and decide whether/how to clarify this aspect

## Aspect #7: Section re-arrangement in TS 38.214

In [2], [6] it is noticed that “current arrangement of DL PRS reception procedure and positioning SRS transmission procedure in the clauses for PDSCH/PUSCH related procedures in TS 38.214 is not proper” due to the following reasons:

- DL PRS reception procedure is NOT related to PDSCH Rx procedure at all.

- SRS for positioning Tx procedure is NOT related to PUSCH Tx procedure at all.

- SRS for MIMO used for positioning is transparent to the UE.

- It is proposed to address the discussed aspect by the following TP implementing proposed changes below:

- A new clause for positioning related procedures separated from PDSCH/PUSCH is created. The content from clauses 5.1.6.5 and 6.2.1.4 is transferred to the new clause, with two new sub-clauses.

- Create sub-sub-clauses for DL PRS related procedures, including assistance data, DL PRS measurement reporting, DL PRS processing capability

- Move three paragraphs in 6.2.1 that are only related to positioning SRS procedures in the new subclause for SRS for positioning related procedures

============================== Unchanged parts ============================== ### 5.1.6 UE procedure for receiving reference signals ============================== Unchanged parts ============================== #### 5.1.6.5 Void TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 μ=0, 1, 2, 3 TperPRS2μ×10240μ=0, 1, 2, 3 TrepPRS∈1,2,4,6,8,16,32 K=s∈SKs Ks=Tsend-Tsstart K=12μS Tsstart, TsendssTsstart, Tsend μSS ============================== Unchanged parts ============================== ### 6.2.1 UE sounding procedure ============================== Unchanged parts ============================== ============================== Unchanged parts ============================== #### 6.2.1.4 Void ============================== Unchanged parts ============================== # X Positioning related procedures ## X.1 DL PRS related procedures ### X.1.1 Assistance data The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource as defined by Clause 6.4.3 [17, TS 37.355]. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS positioning frequency layer configuration(s) as indicated by the higher layer parameter NR-DL-PRS-PositioningFrequencyLayer. A DL PRS positioning frequency layer is defined as a collection of DL PRS resource sets which have common parameters configured by NR-DL-PRS-PositioningFrequencyLayer. The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters NR-DL-PRS-PositioningFrequencyLayer, NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource. A positioning frequency layer is configured by NR-DL-PRS-PositioningFrequencyLayer, consists of one or more DL PRS resource sets and it is defined by: - dl-PRS-SubcarrierSpacing defines the subcarrier spacing for the DL PRS resource. All DL PRS resources and DL PRS resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-SubcarrierSpacing. The supported values of dl-PRS-SubcarrierSpacing are given in Table 4.2-1 of [4, TS38.211], excluding the value of 240kHz. - dl-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-CyclicPrefix. The supported values of dl-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211]. - dl-PRS-PointA defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS resource set have common Point A and all DL PRS resources sets belonging to the same DL PRS positioning frequency layer have a common Point A. The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource. The UE may be configured by the network with nr-PhysCellID, nr-CellGlobalID, and nr-ARFCN [17, TS 37.355] associated with a dl-PRS-ID. - If nr-PhysCellID or nr-CellGlobalID is provided, and if nr-PhysCellID, nr-CellGlobalID and nr-ARFCN associated with the dl-PRS-ID, if provided, are the same as the corresponding information of a serving cell, the UE may assume that the DL PRS is transmitted from the serving cell; - Otherwise, the UE may assume that the DL PRS is not transmitted from a serving cell. If the UE assumes that the DL PRS is transmitted from a serving cell, and if the serving cell is the same as the serving cell defined by the SS/PBCH block, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same serving cell. If the UE assumes that the DL PRS is not transmitted from a serving cell, and if nr-PhysCellID is provided, and is the same as physical cell ID of the SS/PBCH block from a non-serving cell of the same band as the DL PRS, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same non-serving cell. A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by: - nr-DL-PRS-ResourceSetID defines the identity of the DL PRS resource set configuration. - dl-PRS-Periodicity-and-ResourceSetSlotOffset defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity. The UE does not expect that the product of DL PRS resource periodicity TperPRS, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the size of the bitmap of dl-PRS-MutingOption1 exceeds 2μ×10240, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively. - dl-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32. All the DL PRS resources within one resource set have the same resource repetition factor. - dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceSetId within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap. - dl-PRS-MutingOption1 and dl-PRS-MutingOption2 define the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. If dl-PRS-MutingOption1 is configured, each bit in the bitmap of dl-PRS-MutingOption1 corresponds to a configurable number provided by higher layer parameter dl-prs-MutingBitRepetitionFactor of consecutive instances of a DL PRS resource set where all the DL PRS resources within the set are muted for the instance that is indicated to be muted. The length of the bitmap can be {2, 4, 6, 8, 16, 32} bits. If dl-PRS-MutingOption2 is configured each bit in the bitmap of dl-PRS-MutingOption2 corresponds to a single repetition index for each of the DL PRS resources within each instance of a nr-DL-PRS-ResourceSet and the length of the bitmap is equal to the values of dl-PRS-ResourceRepetitionFactor. Both dl-PRS-MutingOption1 and dl-PRS-MutingOption2 may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in Clause 7.4.1.7.4 of [4, TS 38.211]. - NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the transmitting cell with respect to SFN0 slot 0 of reference cell. - dl-PRS-ResourceList determines the DL PRS resources that are contained within one DL PRS resource set. - dl-PRS-CombSizeN defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.3 of [TS38.211]. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of dl-PRS-CombSizeN. - dl-PRS-ResourceBandwidth defines the number of resource blocks configured for DL PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resources sets within a positioning frequency layer have the same value of dl-PRS-ResourceBandwidth. - dl-PRS-StartPRB defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter dl-PRS-PointA. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of dl-PRS-StartPRB. - dl-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.3 of [4, TS38.211]. A DL PRS resource is defined by: - nr-DL-PRS-ResourceID determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set. - dl-PRS-SequenceID is used to initialize cinit value used in pseudo random generator as described in Clause 7.4.1.7.2 of [4, TS 38.211] for generation of DL PRS sequence for a given DL PRS resource. - dl-PRS-CombSizeN-AndReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211]. - dl-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset. - dl-PRS-ResourceSymbolOffset determines the starting symbol of a slot configured with the DL PRS resource. - dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS from a serving cell or a non-serving cell, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a serving or non-serving cell. ### X.1.2 DL PRS measurement reporting The UE assumes constant EPRE is used for all REs of a given DL PRS resource. The UE may be indicated by the network that DL PRS resource(s) can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter nr-DL-PRS-ReferenceInfo. The reference indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncerainty. The UE expects the reference to be indicated whenever it is expected to receive the DL PRS. This reference provided by nr-DL-PRS-ReferenceInfo may include a dl-PRS-ID, a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of DL PRS resource IDs [17, TS 37.355]. The UE may use different DL PRS resources or a different DL PRS resource set to determine the reference for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference than indicated by the network, then it is expected to report the dl-PRS-ID, the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. The UE may be configured to report quality metrics NR-TimingQuality corresponding to the DL RSTD and UE Rx-Tx time difference measurements which include the following fields: - timingQualityValue which provides the best estimate of the uncertainty of the measurement - timingQualityResolution which specifies the resolution levels used in the timingQualityValue field. The UE expects to be configured with higher layer parameter nr-DL-PRS-ExpectedRSTD, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS, and nr-DL-PRS-ExpectedRSTD-Uncertainty, which defines a search window around the nr-DL-PRS-ExpectedRSTD. For DL UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation the UE can be configured to report the DL PRS resource ID(s) or the DL PRS resource set ID(s) associated with the DL PRS resource(s) or the DL PRS resource set(s) which are used in determining the UE measurements DL RSTD, UE Rx-Tx time difference. For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter nr-TimeStamp. The nr-TimeStamp can include the dl-PRS-ID, the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo. The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When the UE is expected to measure the DL PRS resource, it may request a measurement gap via higher layer parameter NR-PRS-MeasurementInfoList [12, TS 38.331]. The UE assumes that the DL PRS from the serving cell is not mapped to any symbol that contains SS/PBCH block from the serving cell. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that the DL PRS from a non-serving cell is not mapped to any symbol that contains the SS/PBCH block of the same non-serving cell. The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of dl-PRS-ID with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those dl-PRS-ID. The up to 4 measurements being performed on the same pair of dl-PRS-ID and all DL RSTD measurements in the same report use a single reference timing. The UE may be configured to measure and report, subject to UE capability, up to 8 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different positioning frequency layers. The UE may be configured to measure and report, subject to UE capability, the timing and the quality metrics of up to 2 additional detected paths that are associated with each RSTD or UE Rx – Tx time difference. The timing of each additional path is reported relative to the path timing used for determining nr-RSTD or nr-UE-RxTxTimeDiff. If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are associated with the same dl-PRS-ID. If DL-PRS-QCL-Info is configured to the UE with qcl-Type set to 'type-D' with a source DL PRS resource then the nr-DL-PRS-ResourceSetId and the nr-DL-PRS-ResourceId of the source DL PRS resource are expected to be indicated to the UE. UE is not expected to process DL PRS without configuration of measurement gap. Within a positioning frequency layer, the DL PRS resources are sorted in the decreasing order of priority for measurement to be performed by the UE, with the reference indicated by nr-DL-PRS-ReferenceInfo being the highest priority for measurement, and the following priority is assumed: - Up to 64 NR-SelectedDL-PRS-IndexPerTRP of the frequency layer are sorted according to priority if nr-SelectedDL-PRS-IndexListPerFreq is provided, or up to 64 NR-DL-PRS-AssistanceDataPerTRP of the frequency layer are sorted according to priority otherwise; - Up to 2 DL-SelectedPRS-ResourceSetIndex per dl-PRS-ID of the frequency layer are sorted according to priority if dl-SelectedPRS-ResourceSetIndexList is provided, or up to 2 NR-DL-PRS-ResourceSet per dl-PRS-ID of the frequency layer are sorted according to priority otherwise. ### X.1.3 DL PRS processing capability For the case when measurement gap is configured, the UE DL PRS processing capability is defined in [TS 37.355]. For the purpose of DL PRS processing capability, the duration K msec of DL PRS symbols within P msec window corresponding to the maximum PRS periodicity in a positioning frequency layer, is calculated by - Type 1 duration calculation with UE symbol level buffering capability K=s∈SKs Ks=Tsend-Tsstart - Type 2 duration calculation with UE slot level buffering capability K=12μS - S is the set of slots based on the numerology of the DL PRS of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets. - For Type 1, Tsstart, Tsend is the smallest interval in msec within slot s corresponding to an integer number of OFDM symbols based on the numerology of the DL PRS of a serving cell that covers the union of the potential PRS symbols and determines the PRS symbol occupancy within slot s, where the interval Tsstart, Tsend considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS resource sets (target and reference). - For Type 2, μ is the numerology of the DL PRS, and S is the cardinality of the set S. ## X.2 SRS for positioning related procedures When the SRS is configured by the higher layer parameter SRS-PosResource and if the higher layer parameter spatialRelationInfoPos is configured, it contains the ID of the configuration fields of a reference RS according to Clause 6.3.2 of [TS 38.331]. The reference RS can be an SRS configured by the higher layer parameter SRS-Resource or SRS-PosResource, CSI-RS, SS/PBCH block, or a DL PRS configured on a serving cell or a SS/PBCH block or a DL PRS configured on a non-serving cell. The UE is not expected to transmit multiple SRS resources with different spatial relations in the same OFDM symbol. If the UE is not configured with the higher layer parameter spatialRelationInfoPos the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources. The UE is only expected to transmit an SRS configured by the higher layer parameter SRS-PosResource within the active UL BWP of the UE. When the configuration of SRS is done by the higher layer parameter SRS-PosResource, the UE can only be provided with a single RS source in spatialRelationInfoPos per SRS resource for positioning. For operation on the same carrier, if an SRS configured by the higher parameter SRS-PosResource collides with a scheduled PUSCH, the SRS is dropped in the symbols where the collision occurs. For operations in the same carrier, the UE is not expected to be configured on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as 'periodic'. For operations in the same carrier, the UE is not expected to be activated or triggered to transmit SRS on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as 'semi-persistent' or 'aperiodic'. For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resource configured by SRS-PosResource on different CCs, subject to UE's capability. The UE does not expect to be configured with SRS-PosResource on a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission. ============================== Unchanged parts ==============================

FL response

- It is proposed to discuss and decide how to proceed on section re-arrangement

# Proposal for E-Mail Discussion

Based on review of the submitted corrections, it is proposed to organize one or two e-mail discussion(s) (number of discussions is up to chair decision) to cover the following aspects:

Aspect #1: Replacement of Cell Terminology

Aspect #2: DL PRS Antenna Ports + Editorial SRS Corrections

Aspect #3: Expected RSTD and RSTD Uncertainty

Aspect #4: DL PRS reference resources

Aspect #5: UE Rx-Tx time difference measurements from different DL PRS resources

Aspect #6: Alignment with RAN4 on DL PRS Processing

Aspect #7: Section re-arrangement in TS 38.214

Companies are invited to provide comments in table below

| Company Name | Comments |
| --- | --- |

Based on expressed views and discussion, the following is observed:

TBD

# Conclusions

As an outcome of preparation phase, it was agreed to organize the following two e-mail discussions:

# References

- R1-2106448	Draft CR on terminology correction to cell for positioning	Huawei, HiSilicon

- R1-2106503	Discussion on clauses for positioning procedures in TS 38.214	Huawei, HiSilicon

- R1-2106504	Draft CR on PRS antenna ports	Huawei, HiSilicon

- R1-2106540	Interpretation of expected RSTD and expected RSTD uncertainty	ZTE

- R1-2106994	Draft CR on PRS reception procedure in NR positioning	CATTI

- R1-2107682	Correction on clauses for positioning procedures in TS 38.214	Huawei, HiSilicon

- R1-2107991	Maintenance on Rel-16 NR positioning	vivo

- R1-2108163	Maintenance on Rel-16 NR positioning	Ericsson

- R1-2108189	Aligning PRS duration calculation with RAN4	Huawei, HiSilicon