3GPP TSG RAN WG1 #106-e	R1-2108253

e-Meeting, August 16th – August 27th, 2021

Agenda item:		8.8.1.2

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)

Document for:		Discussion and Decision

# Introduction

TB processing over multi-slot PUSCH was included as one of the enhancements, for both FR1 and FR2 as well as TDD and FDD, to be specified in the NR coverage enhancement work item approved in RAN1#90-e [1]:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

Section 2 summarizes the key aspects of TB processing over multi-slot PUSCH based on companies’ contributions submitted under AI 8.8.1.2 to RAN1 #106-e [3]-[28].

All related proposals from different contributions, organized per aspect, are listed in Appendix A, for reference.

Previous Rel-17 agreements are listed in Appendix B, for reference.

# Summary of Contributions on TB processing over multi-slot PUSCH

Contributions submitted under AI 8.8.1.2 discussed several aspects of TB processing over multi-slot PUSCH (referred to as TBoMS in this document, for simplicity). A systematic categorization will be used in this document to summarize the content of all contributions. This is done according to both the number of submitted proposals on the different aspects and on the relevance the latter have for designing the feature, from FL’s perspective. Concerning the second criterion, its rationale is given by the natural relationship of consequentiality which exists between different aspects. In the remainder of the document, aspects are thus categorized as follows:

- High priority aspects

  - TOT definition

  - Single TBoMS structure

  - Rate matching (including how RVs are refreshed, if applicable)

  - Whether and how to use the S slots

- Mid priority aspects

  - How to count slots for transmitting TBoMS: available vs. consecutive

  - How to indicate the number of allocated slots for TBoMS

  - UCI multiplexing and collision handling

  - TBS determination: Ninfo calculation

  - TBoMS repetitions

- Other aspects

  - Further design aspects of TBoMS

    - Relationship between TBoMS and PUSCH repetitions

    - FDRA

    - DM-RS

    - Transmission power determination

    - Special TBS values for TBoMS

    - Rank of TBoMS transmission

    - Link adaptation

    - Frequency hopping

    - CB segmentation

    - Retransmissions

    - Interleaved TBoMS transmissions

    - Application of DM-RS bundling to TBoMS

  - Signaling and interaction with other signals/channels

    - Additional indicators and configuration options

    - Application of TBoMS for Msg3 transmission

The categorization above will determine the initial priority order for the discussions to be held for AI 8.8.1.2.  In this context, sections 2.1 and 2.2 will focus on discussions which will (2.1) and may (2.2) be discussed during RAN1 #106-e. Section 2.3 will collect all other aspects.

Tags [OPEN], [CLOSED] and [PAUSED] will be used to identify the status of the discussion at any moment of the meeting. New sections for specific aspects will be open during the meeting, should discussions for the higher priority aspects progress fast.

## High priority aspects

Six high priority aspects are identified at the beginning of the meeting:

- TOT definition

- Single TBoMS structure

- Rate matching (including how RVs are refreshed, if applicable)

- Whether and how to use the S slots

Most companies have discussed at large about such aspects in the submitted contributions. Summary, discussion, and proposals on these aspects are provided in the following different sub-sections. Sub-section numbers follow the list above, for simplicity.

### [OPEN] TOT definition

Most contributions acknowledged the fundamental nature of this aspect and discussed it in detail. High-level summary of companies’ preferences and opinions based on the contributions follows.

Working assumption

Six companies commented on aspects related to the existing working assumption on TOT (RAN1 #105-e), as follows:

- Option 1: WA should be confirmed, i.e., a TOT is constituted of at least one slot or multiple consecutive physical slots for UL transmission [2 companies]: ZTE [5], Lenovo Motorola [27]

- Option 2: WA should be modified by limiting the definition of TOT to one slot [2 companies]: Nokia/NSB [21], Qualcomm [17]

- Option 3: WA should be modified by expanding the definition of TOT to include also sets of multiple consecutive slots [2 companies]: Fujitsu [10], CMCC [12]

Role of TOT in the signal generation

Three companies commented on the role that TOT should have in the signal generation of TBoMS, as follows

- Option 1: The concept of TOT should be used to specify fundamental aspects of signal generation [2 companies]: vivo [6], Lenovo Motorola [27]

- Option 2: The concept of TOT should not be used to specify fundamental aspects of signal generation [1 company]: ZTE [5]

Use of TOT in specification

Three companies commented on whether the concept of TOT should be specified, as follows

- Option 1: The concept of TOT should be specified [2 companies]: vivo [6], Lenovo Motorola [27]

- Option 2: The concept of TOT should not be specified [1 company]: ZTE [5]

FL’s comments on August 16th

Views and proposals related to TOT are rather heterogeneous. The number of companies who expressed an explicit view on this aspect is not very large. However, from FL’s perspective, the implications of taking different directions related to the definition of TOT are large. More precisely, if the notion of TOT is different from the notion of slot, then it would be rather straightforward to expect TOT to be considered as a unit for important aspects of TBoMS such as rate matching, UCI multiplexing, power control, collision handling and so on. However, decisions on such aspects should be taken based on technical elements and not on the fact that an arbitrary unit of time has been taken as a reference. In a way this goes against common sense and logic. Indeed, we have that:

- The concept of TOT has been introduced to simplify the discussion related to the single TBoMS structure. In all generality, considering different units of time helped describing several Options (i.e., 4) for the TBoMS structure. On the other hand, its introduction was never meant to justify the adoption of a TOT-based logic to define other aspects of TBoMS, but for its structure.

- It is reasonable to assume that the goal of RAN1 in this AI is to specify the TBoMS feature according to technically solid rationales, which may or may not need the concept of TOT to be valid. In practice, RAN1 should not decide on aspects such as rate matching, UCI multiplexing, power control, collision handling and so on, depending on the definition of TOT, but rather the converse. Stated differently, decisions on aspects such as rate matching, UCI multiplexing, power control, collision handling and so on should bring RAN1 to decide whether specifying the notion of TOT is necessary or not, and not the converse.

Of course, discussions in RAN1 could lead to deciding to define and specify TOT in a specific way, however it is reasonable to assume that this should be the result of what is decided on all fundamental aspects of TBoMS, more than the starting point of the discussion.

In this context, the following question is formulated:

2.1.1-Q1: Do you agree that RAN1 should first decide on aspects such as rate matching, UCI multiplexing, power control, collision handling and so on, and then decide whether or not the concept of TOT is needed (and revised and specified, if applicable)?

#### First round of discussions

FL’s recommendation is to have a first round of discussion among companies about 2.1.1-Q1. The goal is to identify the preferred direction RAN1 should pursue for handling the design of next aspects. Feel free to elaborate on your answer in the suitable box, if applicable. It is very much appreciated if discussion is kept at technical level, for the sake of an efficient use of the limited time RAN1 has.

| Company | Answer (Yes/No) | Additional comments, if any. |
| --- | --- | --- |

### [OPEN] Single TBoMS structure

Most contributions acknowledged the fundamental nature of this aspect and discussed it in detail. A high-level summary of companies’ preferences based on the contributions is as follows:

| Option 3 [19 companies] | Option 4 [10 companies] |
| --- | --- |
| Huawei/HiSi [3] | Panasonic [18] |
| ZTE [5] | LGE [28] |
| vivo [6] | CMCC [12] |
| CATT [8] | Qualcomm* [17] |
| Ericsson [28] | Apple [16] |
| OPPO [9] | NEC [25] |
| China Telecom [11] | Samsung [19] |
| Interdigital [14] | MediaTek [20] |
| Intel [15] | Sharp* [24] |
| Fujitsu [10] |  |
| NTT Docomo [26] |  |
| Lenovo/Motorola [27] |  |
| WILUS [29] |  |
| Sierra Wireless [23] |  |
| Nokia/NSB [21] |  |
| Qualcomm* [17] |  |
| Sharp* [24] |  |
| Xiaomi [13] |  |
| WILUS [7] |  |

FL’s comments on August 16th

Option 3, based on the use of single RV is preferred by 17 companies, whereas Option 4, which is based on RV cycling, is preferred by 10 companies (“starred” companies expressed views which seem to accommodate both Options, depending on further choices in terms of rate-matching).

Several arguments are used by companies to substantiate their preference. In summary:

- Companies supporting Option 3 state that it provides larger robustness against systematic bit loss, yielding better performance overall, regardless of the TBS value. It should be noted that this problem can never occur in PUSCH repetition Type A, where TBS is calculated using the resources of one slot. Additionally, Options 3 also allows to puncture a lower number of parity bits as well, if any, in turn yielding a lower effective coding rate for the TBoMS. These advantages are observable regardless of the chosen rate-matching time unit (per slot/TOT/TBoMS).

- Companies supporting Option 4 state that it arguably allows to support efficient UCI multiplexing and collision handling approaches, given how different RVs can be decoded by gNB (0 and 3 assumed to be self-decodable as in case of PUSCH Type A repetitions). Solutions to avoid puncturing of systematic bits are proposed, to ensure that coded bits are continuously selected from the circular buffer during the rate matching. Such solutions should yield same result as single RV utilization.

From FL’s perspective, several technical observations can be made from companies’ Tdocs:

- Considerations made for Option 3 are valid independently of the assumptions on the code rate the number of allocated slots for TBoMS [28].

- Option 3 does not ensure self-decodability per slot of a sub-set of slots. Self-decodability of the first slot may depend on the actual code rate.

- Option 4 may not offer self-decodability per slot of a sub-set of slots for the following three reasons:

  - When the equivalent coding rate of the TBoMS transmission is larger than one (i.e., R×M>1, where R denotes ideal coding rate, M denotes the number of available slots allocated for a single TBoMS transmission, and R×M is the equivalent coding rate) self-decodability per slot of a sub-set of slots is not guaranteed [3].

  - PUSCH repetitions type A offer self-decodability of RV0 and RV3 since TBS is calculated using the resources of one slot. This guarantees that a sufficiently large number of systematic bits is present in RV0 and RV3, together with an adequate number of parity bits, for the decoding to be effectively possible. Conversely, TBS is calculated using the resources of more than one slot in TBoMS. In this case, the number of systematic bits per slot may or may not be sufficient to guarantee self-decodability of RV0 and RV3, depending on the scaling factor K used to calculate Ninfo, which cannot be arbitrarily larger than the number of RVs used to transmit the TB [22].

  - In case coded bits were continuously selected from the circular buffer, as per proposed solutions to address the systematic bit puncturing issue, self-decodability per slot of a sub-set of slots would not be guaranteed. In this case, in fact, self-decodability per slot would be the same as for Option 3.

- Both Option 3 and Option 4 are compatible with “on the fly” determination of the coded bits to be transmitted on a given slot and with predetermined approach to identify the starting bit location for each slot be prior to the start of the TBoMS transmission [17]. In this sense, RAN1 would have complete flexibility to pick one approach or the other, for the determination of the coded bits to be transmitted on a given slot, regardless of which option is retained for the single TBoMS structure. It is worth reminding that coded bit selection is one of the two components of rate-matching, the other being the interleaver. Further discussion on this aspect are carried out in Section 2.1.3.

- By definition, all rate-matching options are compatible with Option 3, whereas Option 4 is compatible only with rate-matching per slot and per TOT.

Given all the considerations above, the following 5 questions are formulated.

2.1.2-Q1: Option 3 and Option 4 differ only as to which coded bits are to be transmitted on a given slot, i.e., how starting bit location for each slot is determined. Do you agree with this statement?

2.1.2-Q2: The coded bits transmitted on any given a slot in Option 3 and Option 4 are exactly the same if a suitable offset is applied to the coded bit selection in Option 4, such that the first bit selected from the circular buffer for any given slot is right after the last bit selected from the circular buffer for the previous slot. Do you agree with this statement?

2.1.2-Q3: Do you agree with the following statements?

- Option 3 is compatible with all considered rate-matching options for TBoMS (per slot/TOT/TBoMS).

- Option 4 is compatible only with rate-matching per slot and per TOT.

2.1.2-Q4: Following limitation is necessary to ensure both self-decodability per slot of a sub-set of slots and decodability of the whole TB at gNB, if Option 4 is retained:

- A limit in terms of target maximum code rate supported by Option 4 for any given number of slots allocated for TBoMS.

- The scaling factor used to calculate TBS cannot be arbitrarily larger than the number of RVs used to transmit the TB.

Is this acceptable or should RAN1 rather aim at guaranteeing that the same link adaptation and scheduling flexibility exist for PUSCH type A repetition and TBoMS configuration?

2.1.2-Q5: If self-decodability per slot of a sub-set of slots cannot be guaranteed by either Option 3 or Options 4, then advantage of one option over the other for what concerns UCI multiplexing and collision handling may depend on how MCS, FDRA and TDRA are configured. Do you agree with statement?

#### First round of discussions

FL’s recommendation is to have a first round of discussion among companies about 2.1.2-Q1, 2.1.2-Q2, 2.1.2-Q3, 2.1.2-Q4 and 2.1.2-Q5. The goal is to identify the preferred directions RAN1 should pursue for handling the designof the single TBoMS structure. Feel free to elaborate on your answer in the suitable box, if applicable. It is very much appreciated if discussion is kept at technical level, for the sake of an efficient use of the limited time RAN1 has.

2.1.2-Q1

| Company | Answer (Yes/No) | Additional comments, if any. |
| --- | --- | --- |

2.1.2-Q2

| Company | Comments |
| --- | --- |

2.1.2-Q3

| Company | Answer (Yes/No) | Additional comments, if any. |
| --- | --- | --- |

2.1.2-Q4

| Company | Comments |
| --- | --- |

2.1.2-Q5

| Company | Comments |
| --- | --- |

### [OPEN] Rate matching (including how coded bits are selected)

Most contributions acknowledged the fundamental nature of this aspect and discussed it in detail. A high-level summary of companies’ preferences and views based on the contributions is as follows.

| Per slot [11 companies] | Per TOT [7 companies] | Across all allocated slots for TBoMS [7 companies] |
| --- | --- | --- |
| Panasonic [18] | Huawei/HiSi [3] | vivo [6] |
| Qualcomm* [17] | LGE [28] | Ericsson [28] |
| NEC [25] | CMCC [12] | ZTE [5] |
| Samsung [19] | Apple [16] | China Telecom [11] |
| MediaTek [20] | Sharp* [24] | Intel [15] |
| Sharp* [24] | Fujitsu [10] | CATT [8] |
| Nokia/NSB [21] | WILUS [7] | Xiaomi [13] |
| Interdigital [14] |  |  |
| NTT Docomo [26] |  |  |
| Lenovo/Motorola [27] |  |  |
| NEC [25] |  |  |

Additionally, the following comments on how coded bits are selected have been made:

- Starting points of bit selections other than the first bit selection are right after encoded bits taken from circular buffer in the previous bit selection [4 companies]: NTT DOCOMO [26], Ericsson [28], Lenovo/Motorola [27], Panasonic [18].

  - In this context, one company (NTT DOCOMO) proposed that the starting point of bit selections should be calculated based on available slots for PUSCH transmission

- An offset factor for bit selection may be introduced [2 companies]: OPPO [9], Huawei/HiSilicon [3]

Finally, one company proposed that the index of the starting coded bit for each transmission occasion is predetermined prior to the start of the TBoMS transmission (Qualcomm [17]).

FL’s comments on August 16th

A majority exists in favor of one option (i.e., rate-matching per slot) but support for other options is non-negligible. On the other hand, almost all companies commenting on rate-matching aspects highlighted that selecting coded bits such starting points of bit selections other than the first bit selection are right after encoded bits taken from circular buffer in the previous bit selection should be supported. At times, this seems to contradict the preference some companies expressed for Option 4 in Section 2.1.2.

This brings me to say that differences may exist on how different companies model and think of rate-matching. This may cause confusion and possible misunderstandings. Like what I have done for Section 2.1.1 and 2.1.2, I would then formulate some questions to clarify these aspects, aiming at simplifying any further discussion.

According to TS 38.212, the rate matching for LDPC code is defined per coded block and consists of selecting and interleaving a sequence of E bits selected from the circular buffer, where the N-sized bit sequence after encoding, i.e., d0, …, dN-1, is stored. This is done in two steps, i.e., bit selection and bit interleaving. In this context:

- The result of Bit selection is impacted by decisions taken in Section 2.1.2, i.e., whether TBoMS transmission makes use of single or multiple RVs. Depending on that decision, discussions on possible optimized bit selections could take place.

- The result of the Interleaver does not depend on decisions taken in Section 2.1.2, but rather on decisions which impact the value of E, that are the decisions on the time unit to be used for the rate matching. Of course, other parameters will also impact the value of E, but those would be related to code rate, modulation order, FDRA and so on, i.e., aspects who impact which bits are rate matched but not the time unit over which they will be rate matched.

Therefore, it seems more appropriate to decouple the two steps of the rate matching to ensure we focus on what will actually impact the size of the output sequence of the rate matching function, i.e., E, and not its content (which will depend on the bit selection, in turn depending on decisions related to the single TBoMS structure).

Given the above, the focus of the discussion is switched to the interleaver part of the rate matching for the time being.

#### First round of discussions

FL’s recommendation is to have a first round of discussion among companies about pros and cons of different interleaver options for TBoMS.

Four tables are added to this end. Companies are invited to list pros and cons of each solution, according to their understanding, in the first three tables below, the last column of each table is added for each company to provide analysis of implementation and specification impact. The fourth table is added for company to express an initial preference on one of the options and also indicate an alternative solution which could be acceptable, although not preferred, if applicable. Please do not hesitate to add your company’s name in the fourth table as well. Constructive attitude in this regard is greatly appreciated.

Interleaver per slot

(E is calculated using the resources of one slot)

| Company | Pros | Cons | Analysis of implementation and specification impact |
| --- | --- | --- | --- |

Interleaver per TOT

(E is calculated using the resources of all allocated slots in a TOT)

| Company | Pros | Cons | Analysis of implementation and specification impact |
| --- | --- | --- | --- |

Interleaver over all allocated slots for TBoMS

(E is calculated using the resources in all allocated slots for TBoMS)

| Company | Pros | Cons | Analysis of implementation and specification impact |
| --- | --- | --- | --- |

Time unit for the interleaver

(E is calculated using the resources in the time unit)

|  | First preference | Can live with |
| --- | --- | --- |
| Per slot |  |  |
| Per TOT |  |  |
| Over all allocated slots for TBoMS |  |  |

### [OPEN] Whether and how to use the S slot

Observations on how S slots should be handled in the context of TBoMS are provided in different forms in several contributions. A high-level summary of companies’ preferences and views based on the contributions is as follows.

- Three companies (MediaTek [20], China Telecom [11], CMCC [12]) proposed that UL symbols in the special slots should be used for TBoMS and the indication of these symbols should be supported.

- One company (ZTE [5]) proposed that no optimization specific for the use of special slot in TDD is pursued.

- One company (Panasonic [18]) proposed that if the special slot, where one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the semi-static symbol not intended for PUSCH transmission, needs to be supported, simple modification of PUSCH repetition Type A framework should be supported. Following options should be considered.

  - Option 1: SLIV for special slot is additionally configured for TDRA entry. In normal slot, current SLIV is used and in special slot, SLIV for special slot is used.

  - Option 2: Current SLIV is used even in special slot, while PUSCH resource for special slot is obtained from the symbols indicated by TDRA but not collided with non-UL symbols in the slot.

- One company (Ericsson [22]) proposed that the net gains and use cases of TBoMS support for special slot with different number of UL symbols than that in UL slot for the TB should be carefully studied prior to specifying it.

  - Such study should address how SRS and PUCCH can be transmitted as well as the performance of interference suppression when DMRS in a special or normal uplink slot is used for interference suppression in the other type of slot.

  - If specified, and performance gains are targeted for this case, a TB over consecutive UL symbols in special slot and the following UL slot can be based on PUSCH repetition type-B like TDRA.

FL’s comments on August 16th

From FL’s perspective, and as argued during RAN1 #104-b-e and RAN1 #105-e, the use of S slot for TBoMS is not precluded by current agreements.

No company has argued against this understanding. At the same time, there is no clear consensus on whether the use of S slots can bring non-negligible performance gains, and whether use cases for it are relevant. Indeed, some additional resources for TBoMS could be found in the S slot. From FL’s perspective, no company claims the opposite in the submitted contributions. However, the extent of the actual performance gain one could expect from the S slots, if optimizations targeting its use are considered, seems to depend on the slots structure, on how many slots one can use for TBoMS and which starting slot is used, as evident from the plots in [3]. In some cases, and as argued in [22], using the S slot could lead to a loss of resources, e.g., if DDDSU slot structure is used and 3 slots are allocated to TBoMS one S slot and one U slot (DDDSUDDSU) could be used instead of 2 U slot (DDDSUDDDSU). Further observations found in [3] and [22], and other contributions, highlight that optimizations targeting the use of the S slot would impact aspects such as DMRS mapping type, DMRS positioning, rate matching, TBS determination, UCI multiplexing, power control, coexistence with other channels/signals and so on. This would bring additional and likely non-negligible specification and implementation impact which many companies find unjustified by the arguable, but not deterministic, coverage gain brought by using S slots together with the U slots.

From FL’s perspective, this issue has been open and discussed for way too long. This is unfortunate, since it is clearly not a fundamental issue which can determine the success of TBoMS as a feature or not. It is an optimization over which consensus cannot be reached. Furthermore, and as discussed in other sections, this issue is blocking several other discussions for which progress would be much faster if we closed it. Given the very limited time left before the end of the WI, it must then be resolved during this meeting, to allow the group to move forward with more fundamental aspects of the design.

#### First round of discussions

Given the above analysis of what companies have provided so far, I would ask companies to focus on the technical aspects of the matter and provide technical comments in these regards. Specific focus should be put on the following three items (guidelines explaining how to discuss about them are given):

- Performance increase/reduction. Please note that any statement without supporting evidence cannot be expected to be retained by FL. At this stage of the WI, it is expected that companies against or in favor of this optimization can provide such evidence, e.g., simulation results, constructive examples, or counterexamples, and so on.

- Specification impact. The list above stems from observations companies made in the submitted contributions. Other aspects can be added, if needed. Similar to the performance increase, supporting evidence should be given as well. This may not come in the form of precise reference to specification, but to how the impact can be isolated and characterized.

- Implementation impact. Any relevant observation related to implementation impacts expected at both UE and gNB, given how current operations are performed, can be added. Example of description of implementation impact have been provided in the previous meeting, e.g., how the device handles slot boundary event. This is considered sufficient by FL for the observation to be retained.

All companies are invited to respond and comment on what is stated by other companies in the three tables below (one per analyzed item). Direct questions can be asked. If your company receives a question, please ensure you provide an answer. This would help the group converging faster. The goal is to have a technical discussion such that the most reasonable and sensible direction to solve this use can be identified.

Constructive attitude in this regard is greatly appreciated.

Performance increase/reduction brought by supporting optimizations targeting the use of S slots for TBoMS

| Company | Analysis of performance increase/reduction |
| --- | --- |

Specification impact of supporting optimizations targeting the use of S slots for TBoMS

| Company | Analysis of specification impact (if any) |
| --- | --- |

Implementation impact of supporting optimizations targeting the use of S slots for TBoMS

| Company | Analysis of implementation impact (if any) |
| --- | --- |

## Mid priority aspects

Five mid priority aspects are identified at the beginning of the meeting:

- How to count slots for transmitting TBoMS: available vs. consecutive

- How to indicate the number of allocated slots for TBoMS

- UCI multiplexing & collision handling

- TBS determination: Ninfo calculation

- TBoMS repetitions

Significant attention has been given by several companies to such aspects in the submitted contributions. Although arguably less paramount at this stage of the discussion, they have been included here and will be discussed when need arises, regardless of how many high priority aspects are still being discussed. Summary, discussion, and FL’s comments/proposals on these aspects are provided in the following different sub-sections, whose numbers are given in the list above.

### [OPEN] How to count slots for transmitting TBoMS: available vs. consecutive

Most contributions acknowledged the fundamental nature of this aspect and proposed that available slots should be used for counting the number of slots allocated for TBoMS. A high-level summary of companies’ preferences based on the contributions, is as follows:

- The number of slots allocated for TBoMS is counted based on the available UL slots [7 companies]:

  - Nokia/NSB [21], Panasonic [18], Ericsson [22] (if TBoMS with more than 2 slots is to be supported), Intel [15], Apple [16], Sharp [24], NTT DOCOMO [26]

FL’s comments on August 16th

Situation seems rather clear from FL’s perspective. The following proposal is then formulated.

FL’s proposal 1

The number of slots allocated for TBoMS is counted based on the available slots for UL transmission.

FFS: details of available slot determination

#### First round of discussions

FL’s recommendation is to have a first round of discussion about FL’s proposal 1. Companies are invited to input their views in the corresponding table below. Constructive attitude in this regard is greatly appreciated. In this sense, if you cannot support the proposal, please propose an alternative formulation which takes into account the current spirit.

Views on FL’s proposal 1

| Company | Views |
| --- | --- |

### [OPEN] How to indicate the number of allocated slots for TBoMS

Most contributions acknowledged the fundamental nature of this aspect and discussed it in detail. A high-level summary of companies’ preferences based on the contributions, is as follows:

- Indication of the number of allocated slots for TBoMS:

  - A new column is configured in TDRA table [7 companies]:

    - Huawei/HiSi [3], ZTE [5], Samsung [19], CATT [8], Sharp [24]

    - Vivo [6] (to indicate only the number of slots per TOT, the number of TOTs is separately configured)

    - LGE (indication could be for number of slots or TOTs)

  - Reuse the number of repetitions indicated by TDRA for PUSCH repetition type A [4 companies]:

    - Lenovo/Motorola [27] (if PUSCH repetition is not allowed when TBoMS feature is enabled), OPPO [9], Qualcomm [17], LGE [28] (If repetition is not applied for TBoMS)

  - Configure a separate TDRA table for TBoMS:

    - TCL communications [4]

- Candidate values for the number of allocated slots for TBoMS:

  - Nokia/NSB [21]: {[1], 2, 3, 4, 7}

  - ZTE [5]: {1, 2, 3, 4, 7, 8, 12, 16}

  - Apple [16]: maximum number is 8

The following was also additionally proposed:

- One company (CATT [8]) proposed further studying the configurable set of values for the number of slots.

- Three companies (Fujitsu [10], Qualcomm [17], Sharp [24]) proposed supporting TBoMS for both DG and CG.

FL’s comments on August 16th

Views on this aspect are rather heterogenous. From FL’s perspective, this discussion seems to depend on the decisions which will be taken on at least four other aspects:

- Whether and how to use the S slot.

- Single TBoMS structure (concerning the maximum number of configurable slots).

- How to count slots for transmitting TBoMS.

- Whether TBoMS repetitions are supported,

Where decision on whether TBoMS repetitions are supported depends on the four structural aspects of TBoMS above. This shows that deciding on such structural aspects during #106-e is paramount. We cannot afford leaving the four aspects open after this meeting. Several other aspects are blocked by them. Conversely, decision on how to count slots for transmitting TBoMS seems less controversial at this stage (please see Section 2.2.1).

Having said this, using one TDRA table column to indicate the number of allocated slots for TBoMS is already agreed on. For this reason, the following two proposals are formulated, as first further step to progress on this topic, one based on some conditions related to TBoMS repetitions. Further steps can be taken when decisions on the other discussions are finalized (hopefully during RAN1 #106-e).

FL’s proposal 2 Indication of the number of allocated slots for TBoMS is performed based on the existing TDRA table configured via PUSCH-TimeDomainAllocationList-r16 as follows: - If TBoMS repetitions are not supported: reuse the existing column for configuring the number of repetitions in the TDRA for PUSCH repetition type A, i.e., numberOfRepetitions-r16. - If TBoMS repetitions are supported: a new column is configured in TDRA table FFS: which and how many values for the number of allocated also for TBoMS can be configured FFS: whether TBoMS are supported.

FL’s proposal 3 TBoMS is supported for both configured grant and dynamic grant. Note: Indication of the number of allocated slots for TBoMS is performed based on the existing TDRA table configured via PUSCH-TimeDomainAllocationList-r16.

#### First round of discussions

FL’s recommendation is to have a first round of discussion about FL’s proposal 2 and FL’s proposal 3. Companies are invited to input their views in the corresponding tables below. Please remember that the goal is to advance as much as we can, given current agreements in other discussions, without hindering possible further refinements, e.g., range of configurable values for the number of allocated slots and so on. Therefore, constructive attitude in this regard is greatly appreciated. In this sense, if you cannot support the proposal, please propose an alternative formulation which takes into account the current spirit.

Views on FL’s proposal 2

| Company | Views |
| --- | --- |

Views on FL’s proposal 3

| Company | Views |
| --- | --- |

### [OPEN] UCI multiplexing & collision handling

Details of collision handling for TBoMS were discussed in several contributions and can be summarized as follows.

- Twelve companies discussed about UCI multiplexing on TBoMS

  - One company (Huawei/HiSi [3]) proposed that in case of overlapped PUCCH and TBoMS transmissions, UCI multiplexing should be performed per TOT by rate matching. For latency-sensitive UCI, per-slot UCI multiplexing by puncturing should be allowed.

  - One company (vivo [6]) proposed that the number of modulated symbols in the TBoMS for UCI should be same/close to that multiplexed in a single slot PUSCH.

  - One company (Samsung [19]) proposed that parallel transmission of PUCCH and TBoMS PUSCH is not preferred due to power splitting during CE situation. UCI multiplexing on TBoMS is supported and the timeline requirement is applied for the actual overlapped slot in the TBoMS.

  - One company (OPPO [9]) proposed that UCI is equally multiplexed into all slots of TBoMS transmission.

  - One company (Qualcomm [17]) proposed reusing Rel-15/16 framework for UCI multiplexing.

  - One company (Ericsson [22]) proposed that, if UCI multiplexing in TBoMS is supported, HARQ-ACK can be multiplexed in any overlapping slot by puncturing, and CSI or HARQ-ACK can be repeated in all slots of a TBoMS.

  - One company (Interdigital [14]) proposed further studying whether UCI is repeated on the multiple slots of TBoMS.

  - One company (Sharp [24]) proposed that UCI is multiplexed in a slot or a TOT overlapping with a PUCCH for reporting the UCI.

  - Four companies (ZTE [5], CATT [8], Intel [15], WILUS [29]) proposed further discussing UCI multiplexing rules for TBoMS.

- Two companies discussed overlap between different UL transmission and TBoMS and, more in general, collision handling aspects for TBoMS:

  - One company (ZTE [5]) proposed reusing repetition-like behaviour for collision handling between TBoMS and PUCCH.

  - One company (Qualcomm [17]) proposed reusing Rel-15/16 framework for collision handling.

FL’s comments on August 16th

From FL’s perspective, albeit very relevant in general, discussions on this aspect for TBoMS may not be as paramount as discussions on the high priority aspects in Section 2.1 and strongly depend on other aspects e.g., rate-matching, usage of S slots.  On the other hand, and like what has been done for sections 2.2.1 and 2.2.2, first steps forward can be taken both in terms of UCI multiplexing and collision handling, nonetheless. The idea would be to agree on basic concepts which can then be revised, or not, depending on the outcome of the discussions on other high priority aspects, e.g., rate matching, S slots and so on. The advantage of this approach is to ensure some basic agreement about these aspects exists, should further aspect prioritization be needed in the discussion (i.e., the more time we have for this in the future the better, however this depends on much time companies are willing to spend discussing on other more structural aspects).

The following two proposals are thus formulated.

FL’s proposal 4 UCI multiplexing with PUSCH is supported in case TBoMS transmission is scheduled. Legacy R15/R16 framework for UCI multiplexing with PUSCH is reused as much as possible in case of TBoMS transmission. New rules can be defined if needed and agreed on, otherwise legacy framework applies as is. FFS: details of the new rules, if any.

FL’s proposal 5 For collision handling for TBoMS, at least legacy Rel-15/16 rules for PUSCH repetition type A could be reused by replacing a repetition to a slot of the multiple slots for TB processing. FFS: Whether new collision handling rules are defined.

#### First round of discussions

FL’s recommendation is to have a first round of discussion about FL’s proposal 4 and FL’s proposal 5. Companies are invited to input their views in the corresponding tables below. Please remember that the goal is to advance as much as we can, given current agreements in other discussions, without hindering possible further refinements, e.g., range of configurable values for the number of allocated slots and so on. Therefore, constructive attitude in this regard is greatly appreciated. In this sense, if you cannot support the proposal, please propose an alternative formulation which takes into account the current spirit.

Views on FL’s proposal 4

| Company | Views |
| --- | --- |

Views on FL’s proposal 5

| Company | Views |
| --- | --- |

### [OPEN] TBS determination: Ninfo calculation

Most contributions acknowledged the fundamental nature of this aspect and discussed it in detail. The discussions focused on the scaling factor K in the agreement made during RAN1#105-e for Ninfo calculation. A high-level summary of companies’ preferences based on the contributions, is as follows:

- Definition of the scaling factor K:

  - K equals the number of slots allocated for TBoMS [8 companies]:

    - Wherein the number of slots allocated for TBoMS are available slots:

      - Nokia/NSB [21], CATT [8], Ericsson [22], Huawei/HiSi [3] (if the number of symbols in each slot allocated for TBoMS is the same)

    - ZTE [5], Samsung [19], NTT DOCOMO [26], WILUS [7]

  - K equals the number of slots in a TOT [3 companies]:

    - Fujitsu [10], LGE [28], vivo [6] (if rate-matching is performed per TOT)

  - K equals the number of slots in multiple TOTs which construct a TBoMS [1 company]:

    - Vivo [6] (if rate-matching is performed across TOTs)

  - K equals the ratio of the number of all the symbols allocated for TBoMS transmission excluding DMRS symbols and the one in an uplink slot allocated for TBoMS transmission excluding DMRS symbols [1 company]:

    - Huawei/HiSi [3] (if the number of symbols in an uplink slot allocated for TBoMS transmission and the one in a special slot allocated for TBoMS transmission are different).

  - K is indicated independently from the slots/symbols allocated for TBoMS (e.g., from a set of integer values) [3 companies]:

    - K ∈ {2, 4, 8} OPPO [9]

    - K ∈ {2, 4, 8, 16} Qualcomm [17]

    - LGE [28]

- Indication of the scaling factor K:

  - K is indicated via DCI [2 companies]:

    - Sharp [24], Panasonic [18] (separate field or TDRA)

  - Further study the signaling aspects for the indication of K [1 company]:

    - Qualcomm [17]

The following was also additionally proposed:

- One company (OPPO [9]) proposed that a multi-slot TB size factor is introduced for TB size determination in case when PUSCH repetition is configured. The multi-slot TB size factor is not larger than configured number of slots for repetition.

FL’s comments on August 16th

This discussion seems to depend on the decisions which will be taken on at least two other aspects:

- Whether and how to use the S slot.

- How rate matching is going to be performed, i.e., the time unit of the interleaver.

This shows, once again, that deciding on the two aspects above during #106-e is paramount. We cannot afford leaving the two aspects open after this meeting. Several other aspects are blocked by them.

Having said this, most companies seem to agree on the fact that the case K=total number of allocated slots for TBoMS should be configurable. Other values may still be subject to discussion, depending on the outcome of the discussions above. For this reason, the following proposal is formulated, as first further step to progress on this topic, until further steps can be taken (hopefully during RAN1 #106-e).

FL’s proposal 5

At least the scaling factor value K=total number of allocated slots for TBoMS should be configurable to calculate Ninfo  for TBS determination.

FFS: details related to the indication of K.

FFS: whether and how further values can be indicated.

#### First round of discussions

FL’s recommendation is to have a first round of discussion about FL’s proposal 5. Companies are invited to input their views in the table below. Please remember that the goal is to advance as much as we can, given current agreements in other discussions, without hindering possible further refinements, i.e., indication of K, further supported configurable values and so on. Therefore, constructive attitude in this regard is greatly appreciated. In this sense, if you cannot support the proposal, please propose an alternative formulation which takes into account the current spirit.

| Company | Views |
| --- | --- |

### [CLOSED] TBoMS repetitions

Observations on the support of the repetition of a single TBoMS are provided in different forms in several contributions. A high-level summary of companies’ preferences based on the contributions is as follows:

- Option 1. Support the repetition of a single TBoMS [5 companies]

    - vivo [6], Samsung [19], Intel [15], Apple [16], Xiaomi [13]

- Option 2. Do not support the repetition of a single TBoMS [1 company]

    - Sierra Wireless [23]

- Option 3. Further discuss on whether to support the repetition of a single TBoMS (e.g., based on the outcome of the definition of a single TBoMS) [2 companies]

    - Lenovo/Motorola [27], Ericsson [22]

The following was also additionally proposed:

- One company (vivo [6]) proposed that the repetition factor is indicated in TDRA table.

- One company (China Telecom [11]) proposed down-selecting between two options: (i) the maximum number of aggregated slots for TBoMS is the same as the maximum number of repetition for PUSCH repetition type A in Rel-17 or (ii) repetition on top of TBoMS is supported.

- One company (Lenovo/Motorola [27]) proposed that if repetition of TBoMS is supported, then only PUSCH repetition type A should be considered and two methods can be considered to indicate the number of slots for TBoMS and repetition factor for TBoMS repetition: (i) introduce indication for number of slots for TBoMS in addition to repetition factor via TDRA row index or (ii) only support dynamic indication for number of slots for TBoMS via TDRA, but the repetition factor for TBoMS repetition is indicated only via RRC configuration.

- One company (Sharp [24]) proposed that TBoMS is viewed as repetition in unit of a slot or a TOT.

FL’s comments on August 16th

Most companies who commented on this aspect prefer supporting repetitions of TBoMS. One company prefer not supporting PUSCH repetitions for TBoMS. Two companies propose to further discuss this aspect when the definition of a single TBoMS is finalized.

Current situation seems rather in favour of supporting repetitions of TBoMS. On the other hand, it is acknowledged by FL that the technical need of repetitions of TBoMS may depend on agreements taken for the discussions in Section 2.1, where the structure of a single TBoMS is discussed. It is very likely that a decision on whether supporting repetitions of TBoMS or not will be an incremental effort once details related to single TBoMS transmission are worked out. Indeed, time-domain constraints, if any, and more precise characterization/estimation of the minimum effective coding rate achievable by TBoMS would be available by then.

For all these reasons, and also given that several companies would like to study this aspect further, FL’s suggestion is to focus on the most foundational aspects of TBoMS and to postpone discussion on repetitions of TBoMS to a later time (during #106-e or later).

## Others

As discussed at the beginning of Section 2, discussions on different aspects of TBoMS have been prioritized to ensure that constructive discussions and effective progress can be achieved during RAN1 #106-e. Priority has been given to the aspects and topics discussed in sections 2.1 and 2.2, which mostly focus on resource allocation for TBoMS and structure of single TBoMS in general. All other aspects are listed in this section, i.e, 2.3., where proposals made by companies in their contributions are reported and described in detail.

These aspects may not be handled during RAN1 #106-e unless technical need arises during the discussion on other aspects. For this reason, no specific FL’s proposal or recommendation is formulated at this stage. Should discussions for 2.1 and 2.2 progress fast and converge to agreements, sections for specific aspects, currently in 2.3, may be open for discussions and corresponding FL’s proposals and recommendations may be made.

### [CLOSED] FDRA

Three companies (ZTE [5], Xiaomi [13], and Samsung [19]) proposed that the maximum number of PRBs allocated for TBoMS should be limited.

From FL’s perspective, albeit very relevant in general, discussions on this aspect for TBoMS may not be as paramount as discussions on the higher priority aspects in Sections 2.1-2.2. FL suggests postponing discussions on this topic until need arises (during #106-e or later).

### [CLOSED] DM-RS

One company (Nokia/NSB [21]) proposed that DM-RS optimization for TBoMS is deprioritized in Rel-17.

One company (Ericsson [22]) proposed that DM-RS optimization is discussed after agreements of time unit for rate matching are reached.

### [CLOSED] Transmission power determination

The transmission power determination was discussed in several contributions and can be summarized as follows:

- One company (Huawei/HiSi [3]) proposed that the transmission power determination of TBoMS should be based on the TOT.

- One company (ZTE [5]) proposed that the transmission power determination should be based on the total number of Res within all slots for TBoMS, excluding the overhead of reference signals.

- One company (Ericsson [22]) proposed that the power control aspect is discussed after agreements of time unit for rate matching are agreed.

- One company (CATT [8]) proposed that the transmitted power of a TBoMS remains unchanged during the transmission.

- One company (WILUS [7]) proposed further discussing how to determine the number of REs for UCI multiplexing and UL transmission power in case of TBoMS.

### [CLOSED] Special TBS values for TBoMS

Special TBS values for TBoMS were discussed in several submitted contributions, including maximum supported TBS for TBoMS.  Content of such discussions, and related proposals, can be summarized as follows.

- One company (ZTE [5]) proposed that the maximum TBS can be limited by the conditions of data rate limitations DataRate and DataRateCC.

- One company (CATT [8]) proposed that no restriction is specified except for the maximum TBS.

- One company (NEC [25]) proposed that the maximum supported TBS should not exceed legacy maximum supported TBS in Rel-15/16 for TBoMS.

- One company (Qualcomm [17]) proposed to restrict TBoMS transmissions to TB sizes that permit single codeblock transmission.

- One company (Qualcomm [17]) proposed that no new TBSs are introduced.

### [CLOSED] Rank of TBoMS transmission

The rank of a TBoMS transmission (number of layers) was discussed in several contributions and can be summarized as follows.

- One company (Ericsson [22]) proposed that the number of layers is discussed after agreements of time unit for rate matching are reached.

- Two companies (vivo [6], Qualcomm [17]) proposed that TBoMS should be limited to single-layer transmission.

### [CLOSED] Link adaptation

One company (Ericsson [22]) proposed RAN1 to discuss issues of MCS after agreements of time unit for rate matching are reached.

From FL’s perspective, discussions on this aspect for TBoMS may not be as paramount as discussions on the higher priority aspects in Sections 2.1-2.2. FL suggests postponing discussions on this topic until need arises (during #106-e or later).

### [CLOSED] Frequency hopping

Frequency hopping (FH) aspects were discussed in several contributions and can be summarized as follows:

- Two companies (China Telecom [11] and TCL Communications [4]) proposed that inter-slot FH should be supported for TBoMS.

- Two companies (China Telecom [11] and Intel [15]) proposed that inter-slot FH with inter-slot bundling should be supported for TBoMS.

- One company (TCL Communications [4]) proposed that intra-slot FH should be supported for TBoMS.

- One company (Intel [15]) proposed further studying the support of intra-slot FH for TBoMS.

- One company (Xiaomi [13]) proposed supporting intra-TB FH for TBoMS.

### [CLOSED] CB segmentation

One company (Ericsson [22]) proposed that CB segmentation is needed for TBoMS in order to reuse Rel-15/16 LDPC coding.

One company (Samsung [19]) proposed that RAN1 should confirm whether one or multiple CBs are supported for TBoMS.

### [CLOSED] Retransmissions

Details of retransmission of a TBoMS were discussed in several contributions and can be summarized as follows.

- Three companies (CMCC [12]), InterDigital [14], Lenovo/Motorola [27]) proposed supporting enhanced retransmission mechanisms to avoid the retransmission of the entire TBoMS.

- One company (Ericsson [22]) proposed that the unit of retransmission is discussed after agreements of time unit for rate matching are reached.

- One company (Lenovo/Motorola [27]) proposed that if retransmission for duration shorter than the overall duration of TBoMS is supported, then implicit/explicit configuration of the portion (duration) should be supported with portion indication in the retransmission DCI.

From FL’s perspective, discussions on this aspect for TBoMS may not be as paramount as discussions on the higher priority aspects in Sections 2.1-2.2 and may depend on other aspects e.g., rate-matching.  FL suggests postponing discussions on this topic until need arises (during #106-e or later).

### [CLOSED] Interleaved TBoMS transmission

One company (Qualcomm [17]) proposed that interleaved TBoMS transmissions (carrying different TBs) are not permitted. A UE does not expect a TBoMS transmission in a component carrier to begin before the completion of an ongoing TBoMS transmission in the same component carrier.

### [CLOSED] Application of DM-RS bundling to TBoMS

One company (TCL Communications [4]) proposed that the inter-slot bundling with inter-slot frequency hopping should be supported for TBoMS.

### [CLOSED] Additional indicators and configuration options

Activation indication of TBoMS feature, i.e., indication on whether a PUSCH transmission should follow TBoMS or legacy PUSCH transmission, was discussed in several contributions. Corresponding proposals are summarized as follows

- One company (Nokia/NSB) proposed to specify an indication method for enabling TBoMS per PUSCH scheduling/configuration.

- One company (Lenovo/Motorola [27]) proposed that semi-static and/or dynamic configuration of TBoMS feature for PUSCH should be supported, and independent from PUSCH repetition.

- One company (Interdigital [14]) proposed to support dynamic enabling/disabling of TBoMS transmission using TDRA list configuration.

- One company (Xiaomi [13]) proposed considering the configuration and indication signalling design when a single UE supports both repetition and TBoMS.

From FL’s perspective, albeit very relevant in general, discussions on this aspect for TBoMS may not be as paramount as discussions on the higher priority aspects in Sections 2.1-2.2. In addition, some of these indicators may depend on other aspects e.g., TDRA.  FL suggests postponing discussions on this topic until need arises (during #106-e or later).

### [CLOSED] Application of TBoMS for Msg3 transmission

One company (TCL Communications [4]) proposed studying whether Msg3 transmission also supports TBoMS.

# 3	[CLOSED] Proposals for GTW

# 4	[CLOSED] Agreements during Ran1 #106-e

# References

- RP-202928 		New WID on NR coverage enhancements, China Telecom, RAN#90e, Dec. 2020

- TR 38.830		Study on NR coverage enhancements,	3GPP RAN1 Technical Report,	Dec. 2020

- R1-2106496		Discussion on TB processing over multi-slot PUSCH, Huawei, HiSilicon

- R1-2107198		Discussion on TBoMS, TCL Communication Ltd.

- R1-2106740		Discussion on TB processing over multi-slot PUSCH, ZTE

- R1-2106612		Discussion on PUSCH TB processing over multiple slots, vivo

- R1-2108158		Discussion on TB processing over multi-slot PUSCH, WILUS Inc.

- R1-2106989		Discussion on TB processing over multi-slot PUSCH, CATT

- R1-2107257		Issues for TB over multi-slot PUSCH, OPPO

- R1-2107035		Views on TB processing over multi-slot PUSCH, Fujitsu

- R1-2107124		Discussion on TB processing over multi-slot PUSCH, China Telecom

- R1-2107418		Discussion on TB processing over multi-slot PUSCH, CMCC

- R1-2107936		TB processing over multi-slot PUSCH, Xiaomi

- R1-2107651		TB processing over multi-slot PUSCH, InterDigital, Inc.

- R1-2107603		Discussion on TB processing over multi-slot PUSCH, Intel Corporation

- R1-2107754		Discussion on TB processing over multi-slot PUSCH, Apple

- R1-2107360		TB processing over multi-slot PUSCH, Qualcomm Incorporated

- R1-2107117		Discussion on TB processing over multi-slot PUSCH, Panasonic Corporation

- R1-2106903		TB processing over multi-slot PUSCH, Samsung

- R1-2107523		Discussion on TB Processing over multi-slot PUSCH, MediaTek Inc.

- R1-2106656		Transport block processing for PUSCH coverage enhancements, Nokia, NSB

- R1-2107560		TB Processing over Multi-Slot PUSCH, Ericsson

- R1-2107635		Design Considerations for TB Processing over Multi-Slot PUSCH,	Sierra Wireless

- R1-2107800		TB processing over multi-slot PUSCH, Sharp

- R1-2107141		Discussion on TB processing over multi-slot PUSCH, NEC

- R1-2107873		TB processing over multi-slot PUSCH, NTT DOCOMO, INC.

- R1-2107191		Enhancements for TB processing over multi-slot PUSCH, Lenovo, Motorola Mobility

- R1-2107549		Discussions on TB processing over multi-slot PUSCH, LG Electronics

# Appendix A: Proposals from contributions aggregated by topic

## A.1 TDRA [S slots, number of allocated slots, how allocated slots are counted]

The use of the S slot

R1-2106496 Huawei/Hisi Proposal 1: Support different time domain resource allocations between special slots and uplink slots for TBoMS to fully use the available uplink symbols in special slots. Proposal 2: Introduce an additional SLIV field in RRC configured TDRA table to indicate time domain resource allocation for special slots for TBoMS. R1-2106656 Nokia/NSB Proposal 4. Optimizations on time domain resource determination for allocating resource in the S slots is deprioritized. - DMRS optimization for TBoMS is deprioritized in Rel-17. R1-2106740 ZTE Proposal 1: Confirming the working assumption of allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS. - No optimization specific for the use of special slot in TDD is pursued. R1-2106903 Samsung Proposal 1: the usage of UL symbols (unequal to L in SLIV) in special slot should be supported. R1-2107117 Panasonic Proposal 1: Time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA without optimization for allocating resource in the S slots. R1-2107124 China Telecom Proposal 1: Time domain resource determination for TBoMS can be performed via separate PUSCH repetition Type A like TDRA for UL slots and special slots. R1-2107198 TCL Communications Proposal 3: The special slot in TDD should be a conditional available slot for TBoMS. R1-2107560 Ericsson Proposals: - The net gains and use cases of TBoMS supporting special slot with different number of UL symbols than that in UL slot for the TB should be carefully studied prior to specifying it. - Such study should address how SRS and PUCCH can be transmitted as well as the performance of interference suppression when DMRS in a special or normal uplink slot is used for interference suppression in the other type of slot. - If specified, and performance gains are targeted for this case, a TB over consecutive UL symbols in special slot and the following UL slot can be based on PUSCH repetition type-B like TDRA. - UL symbols in special slot in TDD are not included for time domain resource determination for TBoMS according to the agreed Type A like time domain resource determination for TBoMS. R1-2107651 InterDigital Proposal 7: For PUSCH repetition Type A like TDRA, support the number of symbols in PUSCH larger than 14 when uplink symbols are allocated in the a special slot, prior to an uplink slot. R1-2107754 Apple Proposal 3: Confirm the following working assumption: - Allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS. R1-2107936 Xiaomi Proposal 3: Support optimizing time domain resource allocation for making use of S slots in unpaired spectrum - The reference point of the start symbol can be the first available symbol in special slot - The actual symbol length for TBoMS transmission in special slot can be less than the allocated symbol length

The use of non-consecutive slots

R1-2107560 Ericsson Proposals: - Non-consecutive physical slots can be supported for TBoMS for paired spectrum.

Indication of the number of slots allocated for TBoMS

R1-2106496 Huawei/Hisi Proposal 3: If repetition of TBoMS is supported, existing repetition number field in RRC configured TDRA table should indicate the repetition number of TBoMS, and a new field should be introduced in RRC configured TDRA table to indicate the number of available slots allocated for one TBoMS transmission. R1-2106612 vivo Proposal 3: Number of slots in a TOT is associated with entries in the TDRA table, and a single value for number of TOTs for a TBoMS is separately configured. R1-2106656 Nokia/NSB Proposal 6. RAN 1 to consider the following candidate values of the number of slots allocated for TBoMS as a starting point: - [1], 2, 3, 4, or 7 slots Note: value 1 may or may not be introduced depending on how TBoMS is enabled/disabled. R1-2106740 ZTE Proposal 2: For TBoMS, add an new column in TDRA table to indicate the number of slots. - Support {1, 2, 3, 4, 7, 8, 12, 16} as the candidate values. R1-2106903 Samsung Proposal 2: Indicating number of slot for one TB with an extra parameter in a TDRA row. R1-2106989 CATT Proposal 1: For time domain resource allocation of TBoMS, a new IE is introduced in the TDRA entry to indicate the number of allocated slots for TBoMS. - FFS the configurable set of values for the number of slots. R1-2107035 Fujitsu Proposal 4: The row index of a TDRA list for determining the number of slots allocated for TBoMS is indicated either by the configured grant configuration or by TDRA field in a DCI. R1-2107191 Lenovo/Motorola Proposal 11: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, if PUSCH repetition is not allowed when TBoMS feature is enabled, then the repetition factor indicated by TDRA can utilized to indicate the number of slots for TBoMS PUSCH transmission. R1-2107198 TCL Communications Proposal 1: Configure a separate TDRA table for TBoMS PUSCH. R1-2107257 OPPO Proposal 2: The existing PUSCH repetition type A TRRA and its configuration can be the reused for the TBoMS. R1-2107360 Qualcomm Proposal 2: Reuse TDRA for Type A PUSCH repetition for TBoMS. Proposal 13: Support TBoMS for both dynamic grants and configured grants. R1-2107549 LGE Proposal 4: Discuss following options for slot number determination of TBoMS. - Option 1. The number of slots for TBoMS is indicated by TDRA field. - Option 2. The number of TOTs for TBoMS is indicated by TDRA field. Proposal 5: If repetition is not applied to TBoMS, repetition number in TDRA field can be used to indicate the number of slots or TOTs for TBoMS. R1-2107754 Apple Proposal 1: Considering the maximum number of usable slots for TB transmission is 8. R1-2107800 Sharp Proposal 6: The number of slots can be indicated through a TDRA field in the DCI format for dynamic scheduling of a TBoMS. Proposal 7: The number of slots can be indicated through a value provided by RRC for configured scheduling of a TBoMS. Proposal 8: The number of slots can be indicated through a TDRA field in the DCI format for retransmission of the TBoMS.

How slots allocated for TBoMS are counted

R1-2106656 Nokia/NSB Proposal 5. The number of slots allocated for TBoMS is counted based on the available slots. R1-2107117 Panasonic Proposal 2: For the time domain resource determination for TBoMS, unified solution of determination of available slot is supported. R1-2107560 Ericsson Proposals: - If TBoMS with more than 2 slots is to be supported, TBoMS configuration uses the number of available slots, otherwise physical slots are used. R1-2107603 Intel Proposal 3 - TBoMS can be transmitted on the basis of available UL slots. R1-2107754 Apple Proposal 2: The number of slots for TBoMS transmission is counted based on available slot. R1-2107800 Sharp Proposal 1: Repetition type A-like TDRA employs counting on the basis of available slots. R1-2107873 NTT DOCOMO Proposal 5: The number of slots allocated for TBoMS should be counted on the basis of available slots.

Others

R1-2107257 OPPO Proposal 1: In TBoMS, TB size determination over multiple slots is configured with PUSCH repetition operation. The TB can be transmitted in the multi-slot configured in the PUSCH repetition. The enhanced Type A PUSCH repetition is included. R1-2107360 Qualcomm Proposal 1: Prioritize a modular approach to TBoMS transmission, i.e., when resources for TBoMS span across multiple contiguous/noncontiguous slots, view resources in each slot as one self-contained segment of a longer transmission.

## A.2 TOT definition

R1-2106612 vivo Proposal 5: Concept of TOT should be specified at least for the following purposes - RV refreshing; - UCI multiplexing; - TB size determination. R1-2106656 Nokia/NSB Proposal 1. The following definition of transmission occasion for TBoMS (TOT) and approach for rate-matching for TBoMS is supported: - A TOT is one slot and rate-matching is performed per slot. R1-2106740 ZTE Proposal 7: Confirming the WA of a transmission occasion for TBoMS (TOT) is constituted of at least one slot or multiple consecutive physical slots for UL transmission. - No need to specify the concept of TOT. - The concept of TOT will be not used for designing aspects related to signal generation. R1-2107418 CMCC Proposal 1: The un-consecutive slots, such as multiple sets of consecutive slots, carrying a single TB should be discussed. R1-2107035 Fujitsu Proposal 1: Both consecutive slots and non-consecutive slots can be contained in a TOT. R1-2107191 Lenovo/Motorola Proposal 1: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, the concept of a transmission occasion for TBoMS (TOT) should be specified, where a TOT constitutes of at least on slot or multiple consecutive physical slots for UL transmission. Proposal 2: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, if TOT is specified, then it could be used for further design aspect including redundancy version, rate-matching, power control, partial retransmissions of TBoMS and others (if any). R1-2107360 Qualcomm Proposal 3: A transmission occasion for TBoMS (TOT) constitutes one slot of transmission. R1-2107549 LGE Proposal 1: Define the maximum number of slots constituting a TOT.

## A.3 Single TBoMS structure

- R1-2106496 Huawei/HiSi Proposal 6: The TB is transmitted on the multiple TOTs using a single RV. R1-2106612 vivo Proposal 2: TBoMS definition Option-3 is supported together with rate-matching method Option-c, and TBoMS definition Option-4 is supported together with rate-matching method Option-c. R1-2106656 Nokia/NSB Proposal 2. For definition of a single TBoMS, Option 3 should be adopted, i.e., the TB is transmitted using a single RV. R1-2106740 ZTE Proposal 5: Option 3 should be supported for TBoMS. R1-2106903 Samsung Proposal 7: Option 4(different RV) is slightly preferred for the definition of a single TBoMS. R1-2106989 CATT Proposal 3: For the structure of TBoMS, at least Option 3 with single RV and continuous rate-matching across all the allocated slots/TOTs for TBoMS is supported. - FFS whether/how to additionally support Option 4 with multiple RVs and continuous rate-matching across all the allocated slot(s) per TOT. R1-2107117 Panasonic Proposal 3: TB is transmitted on the multiple TOTs using different RVs. The (maximum) length of TOT is 4 slots. R1-2107124 China Telecom Proposal 2: Option 3 is supported. Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using a single RV. R1-2107141 NEC Proposal 1: Select Option 4, i.e. if a design based on different RVs is adopted. Proposal 2: If a design based on different RVs is adopted and resource in TBoMS is not transmitted due to collision with other resources, the RV should be counted. R1-2107191 Lenovo/Motorola Proposal 3: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, option 3 is adopted where a single RV is applied across entire TBoMS. R1-2107257 OPPO Proposal 3: TBoMS support multiple TOTs to enable non-consecutive/consecutive physical slots for UL transmission. Proposal 6: Single RV scheme can be used across all the TOTs in one TBoMS transmission over multi-slot. Reducing the complexity of TB and RE processing in each slot, e.g., restricting TB size. Consider an offset factor for bit selection. R1-2107360 Qualcomm Proposal 5: For TBoMS, refresh RV indices once every S transmission occasions. - FFS: Value of S. R1-2107418 CMCC Proposal 2: The option 4, a design based on different RVs is preferred. R1-2107523 MediaTek Proposal 1: Option-4 to be down selected because it allows each PUSCH transmission (or one slot) to be independent with different RV. R1-2107549 LGE Proposal 3: RV values applied for TBoMS are cycled for each TOT. R1-2107560 Ericsson Proposals: - TBoMS is transmitted using a single RV. R1-2107603 Intel Proposal 1 - For the definition of a single TBoMS, Option 3 is supported. - For the rate-matching of TBoMS, Option C is supported. R1-2107635 Sierra Wireless Proposal 1: TBoMS encoding follows option 3: Option 3: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using a single RV. Repetition is not supported with TBoMS. FFS: Maximum number of slots FFS: If and how to support early termination R1-2107651 InterDigital Proposal 2: Single RV is supported for TBoMS transmission. Proposal 3: For the structure of TBoMS, Option 3 is supported. R1-2107754 Apple Proposal 5: A transmission block for TBoMBS is transmitted on multiple TOTs using different RVs, and the rate matching is performed across all slots per TOT. R1-2107873 NTT DOCOMO Proposal 2: A single RV should be transmitted over one TOT for consecutive slots or multiple TOTs for non-consecutive slots in a single TBoMS, where starting points of bit selections other than the first bit selection are right after encoded bits taken from circular buffer in the previous bit selection (Opt.3-2). R1-2107936 Xiaomi Proposal 1: Support transmitting a single RV on multiple slots for TBoMS. R1-2108158 WILUS Proposal 1: For TBoMS, the TB is transmitted on multiple TOTs using a single RV (Option-3), and the single RV is continuously rate-matched across all the allocated slot(s) per TOT (Option-b). - FFS: Handling for issues on rate-matching, such as UCI multiplexing.

Relationship between TBoMS and PUSCH repetitions

R1-2106656 Nokia/NSB Proposal 3. RAN1 should specify TBoMS as an independent feature according to WID. It should not be considered as an enhancement of PUSCH repetition type A, regardless of how time domain resource determination is indicated. R1-2107560 Ericsson Proposals: - From an interleaving and RV perspective, TBoMS is designed as a new feature, rather than a Type A PUSCH repetitions enhancement. R1-2107873 NTT DOCOMO Proposal 1: Performance gain of TBoMS compared to PUSCH repetition type A should be taken into consideration, when designing TBoMS.

## A.4 Rate-matching

R1-2106496 Huawei/HiSi Proposal 7: RM is performed per TOT, where the start position of bit selection in the circular buffer on TOT i is defined as ki=0,i=0li-1ZcZc,i=1,2,… where li-1 denotes the end position of bit selection in the circular buffer on TOT i-1, li=0,1,…,Ncb, Ncb denotes the length of coded bits in the circular buffer, Zc is the LDPC lifting size, and i denotes the TOT number, i=0,1,… R1-2106612 vivo Proposal 1: Both rate-matching per TOT (Option-b) and rate matching across TOTs (Option-c) can be supported based on UE capability reporting. - Rate matching per TOT (Option-b) can be considered as baseline capability for TBoMS. Proposal 2: TBoMS definition Option-3 is supported together with rate-matching method Option-c, and TBoMS definition Option-4 is supported together with rate-matching method Option-c. R1-2106656 Nokia/NSB Proposal 1. The following definition of transmission occasion for TBoMS (TOT) and approach for rate-matching for TBoMS is supported: - A TOT is one slot and rate-matching is performed per slot. R1-2106740 ZTE Proposal 6: Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS. R1-2106903 Samsung Proposal 8: option a (Rate-matching is performed per slot) shall be supported for TBoMS. R1-2106989 CATT Proposal 2: For rate-matching for TBoMS, at least Option c is supported, i.e. rate matching is performed continuously across all the allocated slots/TOTs for TBoMS. - FFS whether/how to additionally support Option b, i.e. rate matching is performed continuously across all the allocated slot(s) per TOT. R1-2107035 Fujitsu Proposal 2: Option b for rate-matching for TBoMS is supported. In other words, rate matching is performed continuously across all the allocated slot(s) per TOT. R1-2107117 Panasonic Proposal 4: Rate matching is performed per slot. Starting point (bit position in circular buffer) for rate matching in the subsequent slots in a TOT is based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated. - For example, the start position of rate matching in the circular buffer on TOT i can be given by ki=0i=0i∙Nref+1i>0, where Nref is the reference number of bits based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated. R1-2107124 China Telecom Proposal 3: Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS. R1-2107141 NEC Proposal 3: For rate-matching for TBoMS, support option a, i.e. Rate-matching is performed per slot. R1-2107191 Lenovo/Motorola Proposal 5: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, option a should be adopted for rate-matching i.e., the rate-matching is performed per slot basis. R1-2107360 Qualcomm Proposal 4: Adopt per-slot rate matching for TBoMS. R1-2107418 CMCC Proposal 3: For the rate matching for TBoMS, the option b with all the allocated slot(s) per TOT is preferred. Proposal 4: An updated version of option b should be considered for the further discussion. - Option b’: Rate matching is performed continuously across all the allocated slots over X TOTs; R1-2107523 MediaTek Proposal 2: Rate-matching has to be done for every PUSCH transmission (i.e per slot approach). Option-a is preferred as it allows UE to transmit each PUSCH as a fresh transmission. R1-2107549 LGE Proposal 2: Select one option among TOT based rate-matching and slot based rate-matching for TBoMS. R1-2107560 Ericsson Proposals: - Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS, if CB segmentation doesn't happen. Otherwise every CB is rate matched once R1-2107603 Intel Proposal 1 - For the definition of a single TBoMS, Option 3 is supported. - For the rate-matching of TBoMS, Option C is supported. R1-2107651 InterDigital Proposal 4: Rate matching is performed per slot (Option a). R1-2107754 Apple Proposal 5: A transmission block for TBoMBS is transmitted on multiple TOTs using different RVs, and the rate matching is performed across all slots per TOT. R1-2107800 Sharp Proposal 3: Bit-selection should be defined as a slot or a TOT. The size G should be defined by REs available for transmission of UL-SCH in a slot or a TOT. Proposal 4: RE mapping should be performed per a slot or a TOT. R1-2107873 NTT DOCOMO Proposal 3: Support rate matching per slot for TBoMS, unless the CovEnh performance gap is large between rate matching per slot and rate matching per TOT. R1-2107936 Xiaomi Proposal 2: Support continuous rate-matching for TBoMS transmission. R1-2108158 WILUS Proposal 1: For TBoMS, the TB is transmitted on multiple TOTs using a single RV (Option-3), and the single RV is continuously rate-matched across all the allocated slot(s) per TOT (Option-b). - FFS: Handling for issues on rate-matching, such as UCI multiplexing.

How coded bits are selected

R1-2106496 Huawei/HiSi Proposal 7: RM is performed per TOT, where the start position of bit selection in the circular buffer on TOT i is defined as ki=0,i=0li-1ZcZc,i=1,2,… where li-1 denotes the end position of bit selection in the circular buffer on TOT i-1, li=0,1,…,Ncb, Ncb denotes the length of coded bits in the circular buffer, Zc is the LDPC lifting size, and i denotes the TOT number, i=0,1,… Proposal 8: The start position of bit selection in the circular buffer on the first TOT for each repetition is denoted by RV index and the RV index is cycled for each repetition in a configured sequence. R1-2107191 Lenovo/Motorola Proposal 4: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, if option 3 with single RV is adopted, then different starting points (to apply coded bits) from a single RV should be considered for different slots or TOTs. R1-2107257 OPPO Proposal 6: Single RV scheme can be used across all the TOTs in one TBoMS transmission over multi-slot. Reducing the complexity of TB and RE processing in each slot, e.g., restricting TB size. Consider an offset factor for bit selection. R1-2107360 Qualcomm Proposal 6: Defining a transmission occasion of TBoMS to span a single slot, the index of the starting coded bit for each transmission occasion is predetermined prior to the start of the TBoMS transmission. R1-2107560 Ericsson Proposals: - For TBoMS based on single RV and Option a, b and c, continuous coded bits are selected for all slots/TOTs of TBoMS. R1-2107873 NTT DOCOMO Proposal 2: A single RV should be transmitted over one TOT for consecutive slots or multiple TOTs for non-consecutive slots in a single TBoMS, where starting points of bit selections other than the first bit selection are right after encoded bits taken from circular buffer in the previous bit selection (Opt.3-2). Proposal 6: The starting point of bit selections should be calculated based on available slots for PUSCH transmission

## A.5 TBS determination

NInfo calculation

R1-2106496 Huawei/Hisi Proposal 4: If the number of symbols in each slot allocated for TBoMS transmission is the same, K should be defined as the number of available slots allocated for TBoMS transmission. Proposal 5: If the number of symbols in an uplink slot allocated for TBoMS transmission and the one in a special slot allocated for TBoMS transmission are different, K can be defined as the ratio of the number of all the symbols allocated for TBoMS transmission excluding DMRS symbols and the one in an uplink slot allocated for TBoMS transmission excluding DMRS symbols. R1-2106612 vivo Proposal 6: For definition of scaling factor K, it can be - Number of slots in a TOT, if rate matching is performed per TOT; - Number of slots of multiple TOTs which construct a TBoMS, if rate-matching is performed across the multiple TOTs. R1-2106656 Nokia/NSB Proposal 7. For Ninfo calculation, NRE is scaled by K, where K equals the number of slots allocated for TBoMS counted based on the available UL slots. R1-2106740 ZTE Proposal 8: The number of slots for TBoMS could be reused as scaling factor K for NInfo calculation. R1-2106903 Samsung Proposal 5: K is the number of slots these are allocated to UE for one TBoMS transmission. R1-2106989 CATT Proposal 4: TBS of TBoMS is calculated by the following steps: - Step 1: A UE first determines the number of REs allocated for TBoMS within a PRB (NRE') by NRE'=K∙(NscRB∙Nsymbsh-NDMRSPRB-NohPRB). - Step 2: A UE determines the total number of REs allocated for TBoMS (NRE) by NRE=min(NRE'∙nPRB , 156∙NPRB). - Step 3: Obtain unquantized intermediate variable (NInfo) by NInfo=NRE∙R∙Qm∙v. Where K is the total number of the allocated available slots for TBoMS, and NPRB is the maximum bandwidth of the active UL BWP. R1-2107035 Fujitsu Proposal 3: Scaling factor K is equal to the number of slots per TOT assuming that a TOT can be configured to contain one or more consecutive and/or non-consecutive physical slots for UL transmission. R1-2107117 Panasonic Proposal 5: For TBS determination, scaling factor K is indicated via DCI (separate field or TDRA). R1-2107257 OPPO Proposal 4: For coverage enhancement, TB size of PUSCH can be derived by a larger than 1 factor in case when PUSCH repetition is configured. Ninfo can be multiplied by factor of 2, 4, 8 for determining TBS. Proposal 5: A multi-slot TB size factor is introduced for TB size determination in case when PUSCH repetition is configured. The multi-slot TB size factor is not larger than configured number of slots for repetition. R1-2107360 Qualcomm Proposal 7: When determining Ninfo for TBoMS, NRE is the number of resource elements available in one slot of a TBoMS transmission as indicated by the SLIV in the TDRA and the FDRA. Further, Ninfo is computed as K* NRE*R*Qm*ν, where R denotes the code rate, Qm denotes the modulation order and ν denotes the number of layers. Proposal 8: The scale factor K used to determine the TBS of TBoMS is determined independently of the number of slots over which TBoMS transmission is scheduled. The scale factor may take at least the following values: 2, 4, 8, 16. FFS: signaling aspects of the scale factor. R1-2107549 LGE Proposal 6: Discuss following alternatives for the scaling factor K for TB size determination. - Alternative 1: K is the number of slots consisting a TOT. - Alternative 2: K is indicated independently of the number of slots consisting the TOT/TBoMS. R1-2107560 Ericsson Proposals: - NInfo for TBoMS should be based on the number of REs across all slots of the TBoMS, no matter if the TBoMS is based on single RV or multiple RVs. Namely, K= the number of slots for the TBoMS. - When the number of symbols in each slot is the same for TBoMS, - If the number of physical slots is configured, use TDD UL/DL configuration for TBS determination - If the number of available slots is configured, TBS determination is according to the number of available slots. R1-2107800 Sharp Proposal 5: K is dynamically adapted or signalled by the scheduling DCI for TBoMS. R1-2107873 NTT DOCOMO Proposal 4: Scaling factor K for the number of REs in TBS determination should be the number of slots allocated for one TB, considering the overhead and the issue of code rate in PUSCH repetition type A. R1-2108158 WILUS Proposal 2: The definition of K in Approach 2 is the number of slots allocated for TBoMS determined by using a row index of a TDRA list, configured via RRC. Proposal 3: Ninfo is calculated based on the symbols over which TBoMS transmission is allocated.

Specific TBS values for TBoMS [To be included to ask companies if they envision new TBS values to be introduced, without touching max TBS value – Mid priority]

R1-2106740 ZTE Proposal 9: The maximum TBS can be limited by the conditions of date rate limitations DataRate and DataRateCC. R1-2106989 CATT Proposal 6: For TBoMS, no restriction is specified except for the maximum TBS. R1-2107141 NEC Proposal 4: Limit Ninfo upper bound to make sure that the maximum supported TBS not exceeds legacy maximum supported TBS in Rel-15/16 for TBoMS. R1-2107360 Qualcomm Proposal 9: For TBoMS, no new TB sizes are introduced. Proposal 10: Restrict TBoMS transmissions to TB sizes that permit single codeblock transmissions (i.e., entire TB can be encoded as a single codeblock). Furthermore, restrict TBoMS transmission to single layer transmissions.

## A.6 FDRA

R1-2106740 ZTE Proposal 4: The maximum number of PRBs can be limited when TBoMS is enabled. - FFS how to determine the maximum number of PRBs. R1-2106903 Samsung Proposal 4: The maximal number of PRB allocated in time domain is reduced for TB over multi-slot. R1-2107936 Xiaomi Proposal 5: Limit the number of RBs allocated for TB processing over multi-slot PUSCH by gNB scheduling.

## A.7 TBoMS repetitions [mid priority – comment on the fact that this depends on the TBoMS structure decisions and several companies would like to study this further]

R1-2106612 vivo Proposal 4: Repetition on top of TBoMS is supported, and the repetition number M is indicated in TDRA table. - Where M is the repetition times of X TOTs which composes the TBoMS. R1-2106903 Samsung Proposal 3: Repetition is supported for TB over multi-slot. R1-2107124 China Telecom Proposal 4: Down selection on the following options for TBoMS: - Option 1: The maximum number of aggregated slots for TBoMS is the same as the maximum number of repetition for PUSCH repetition type A in Rel-17. - Option 2: Repetition on top of TBoMS is supported. R1-2107191 Lenovo/Motorola Proposal 8: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, repetitions of TBoMS should be further discussed. Proposal 9: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, if repetition of TBoMS is supported, then only PUSCH repetition type A should be considered Proposal 12: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, if PUSCH repetition is allowed when TBoMS feature is enabled, then following two methods can be considered to indicate the number of slots for TBoMS and repetition factor for TBoMS repetition: - Introduce indication for number of slots for TBoMS in addition to repetition factor via TDRA row index - Only support dynamic indication for number of slots for TBoMS via TDRA, but the repetition factor for TBoMS repetition is indicated only via RRC configuration R1-2107560 Ericsson Proposals: - The need for repetition of TBoMS is further considered. R1-2107603 Intel Proposal 2 - Repetition is supported for the transmission of TBoMS. R1-2107635 Sierra Wireless Proposal 1: TBoMS encoding follows option 3: Option 3: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using a single RV. Repetition is not supported with TBoMS. FFS: Maximum number of slots FFS: If and how to support early termination R1-2107754 Apple Proposal 4: For TB transmission over consecutive UL slots, repetition can be supported on top of TBoMS. R1-2107800 Sharp Proposal 10: TBoMS is viewed as repetition in unit of a slot or a TOT. R1-2107936 Xiaomi Proposal 8: TB processing over multi-slot can be transmitted in conjunction with repetitions.

## A.8 DM-RS

R1-2106656 Nokia/NSB Proposal 4. Optimizations on time domain resource determination for allocating resource in the S slots is deprioritized. - DMRS optimization for TBoMS is deprioritized in Rel-17. R1-2107560 Ericsson Proposals: - RAN1 is to discuss issues of DMRS, MCS, number of layers, unit of retransmission and power control after agreements of time unit for rate matching are reached.

## A.9 Transmission power determination

R1-2106496 Huawei/HiSi Proposal 9: The transmission power determination of TBoMS should be based on the TOT. R1-2106740 ZTE Proposal 11: For TBoMS, the transmission power determination should be based on the total number of REs within all slots for TB processing with excluding the overhead of reference signals. R1-2106989 CATT Proposal 7: The transmitted power of a TBoMS remains unchanged during the transmission. R1-2107560 Ericsson Proposals: - RAN1 is to discuss issues of DMRS, MCS, number of layers, unit of retransmission and power control after agreements of time unit for rate matching are reached. R1-2108158 WILUS Proposal 4: It should be further discussed how to determine the number of REs for UCI multiplexing and UL transmission power in case of TBoMS.

## A.10 Rank of TBoMS transmission

R1-2106612 vivo Proposal 9: PUSCH with TB processing over multiple slots should be limited to single transmission layer. R1-2107360 Qualcomm Proposal 10: Restrict TBoMS transmissions to TB sizes that permit single codeblock transmissions (i.e., entire TB can be encoded as a single codeblock). Furthermore, restrict TBoMS transmission to single layer transmissions. R1-2107560 Ericsson Proposals: - RAN1 is to discuss issues of DMRS, MCS, number of layers, unit of retransmission and power control after agreements of time unit for rate matching are reached.

## A.11 Link adaptation

MCS index

R1-2107560 Ericsson Proposals: - RAN1 is to discuss issues of DMRS, MCS, number of layers, unit of retransmission and power control after agreements of time unit for rate matching are reached.

## A.12 Frequency hopping

R1-2107124 China Telecom Proposal 5: Both inter-slot frequency hopping and inter-slot frequency hopping with inter-slot bundling should be supported for TBoMS. R1-2107198 TCL Communications Proposal 4: Intra-slot and inter-slot frequency hopping should be supported for TBoMS. R1-2107603 Intel Proposal 4 - Inter-slot frequency hopping and inter-slot frequency hopping with inter-slot bundling are supported for TBoMS. - FFS: intra-slot frequency hopping for TBoMS R1-2107936 Xiaomi Proposal 6: Support intra-TB frequency hopping for TB processing over multi-slot PUSCH.

## A.13 CB segmentation

R1-2107560 Ericsson Proposals: - CB segmentation is needed for TBoMS in order to reuse Rel-15/16 LDPC coding. R1-2106903 Samsung Proposal 6: RAN1 to confirm whether one or multiple CBs are supported for TBoMS.

## A.14 Retransmissions

R1-2107191 Lenovo/Motorola Proposal 6: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, retransmission procedure and signaling should be enhanced to support retransmission of only partial slots from the TBoMS. Proposal 7: For PUSCH coverage enhancements in NR Rel-18 with TBoMS, if retransmission for duration shorter than the overall duration of TBoMS is supported, then implicit/explicit configuration of the portion (duration) should be supported with portion indication in the retransmission DCI. Exact duration of the portion can be as follows: - Explicitly configured to the UE - Implicitly determined by UE depending on the duration of TBoMS, number of TOTs, duration of TOTs R1-2107418 CMCC Proposal 5: Per slot/TOTs retransmission could be considered for the retransmission of TBoMS. R1-2107560 Ericsson Proposals: - RAN1 is to discuss issues of DMRS, MCS, number of layers, unit of retransmission and power control after agreements of time unit for rate matching are reached. R1-2107651 InterDigital Proposal 6: Support enhanced retransmission mechanisms to avoid the retransmission of the entire TBoMS.

## A.15 UCI multiplexing, SRS/DL collisions/cancellations [mid-priority – this is well discussed in contributions but strongly depends on how rate matching is performed, and S slot discussion. It should stay close at the beginning of the meeting]

UCI multiplexing

R1-2106496 Huawei/HiSi Proposal 10: In case of overlapped PUCCH and TBoMS transmissions, perform UCI multiplexing per TOT. Proposal 11: For latency-sensitive UCI, allow performing per-slot UCI puncturing. R1-2106612 vivo Proposal 7: For UCI multiplexing on PUSCH with TB processing over multiple slots, the starting symbol for TBoMS used for determining S0 is the starting symbol of a TOT or a TBoMS. Proposal 8: For UCI multiplexing on TBoMS, the number of modulated symbols in the TBoMS for UCI should be same/close to that multiplexed in a single slot PUSCH, following options can be considered - Opt-1: Re-define the parameter Nsymb,allPUSCH as number of symbols per slot allocated for TBoMS; - Opt-2: BetaOffset and scaling (α) is scaled by 1/K, where K is the number of slots for a TOT or TBoMS. R1-2106740 ZTE Proposal 10: Further discuss UCI multiplexing rules for TBoMS with aiming for reusing existing UCI multiplexing rules for PUSCH repetition type A as much as possible. R1-2106903 Samsung Proposal 9: Parallel transmission of PUCCH and TBoMS PUSCH is not preferred due to power splitting during CE situation. Proposal 10: UCI multiplexing in TBoMS PUSCH is supported in Rel-17 CE, Proposal 11: The timeline requirement is applied for the actual overlapped slot in the TBoMS. R1-2106989 CATT Proposal 5: Consider the following options for UCI handling in TBoMS. - Option 1: UCI multiplexing is not supported by TBoMS. - Option 2: Reuse the UCI multiplexing of PUSCH repetition type A in TBoMS, i.e. the UCI is multiplexed into each overlapped slot of the TBoMS. - Option 3: UCI multiplexing is supported in a unit of TOT. - Option 4: UCI multiplexing is supported in a unit of TBoMS. - FFS details, e.g. determination of the number of REs for UCI multiplexing. R1-2107257 OPPO Proposal 7: UCI is equally multiplexed into all slots of TBoMS transmission. R1-2107360 Qualcomm Proposal 11: Defining a transmission occasion of TBoMS to span a single slot and restricting rate matching to occur on a per-slot basis, reuse R15/R16 framework for UCI multiplexing on PUSCH for TBoMS as well. R1-2107560 Ericsson Proposals: - If UCI multiplexing in TBoMS is supported, HARQ-ACK can be multiplexed in any overlapping slot by puncturing, and CSI or HARQ-ACK can be repeated in all slots of a TBoMS. R1-2107603 Intel Proposal 5 - FFS how to handle overlaps between TBoMS and other uplink transmission. R1-2107651 InterDigital Proposal 5: Support UCI multiplexing with TBoMS. FFS whether UCI is repeated on the multiple slots of TBoMS. R1-2107800 Sharp Proposal 2: UCI is multiplexed in a slot or a TOT overlapping with a PUCCH for reporting the UCI. R1-2108158 WILUS Proposal 4: It should be further discussed how to determine the number of REs for UCI multiplexing and UL transmission power in case of TBoMS.

Collision handling

R1-2106740 ZTE Proposal 3: For collision handling of TBoMS, legacy Rel-15/16 collision handling rules for PUSCH repetition type A could be reused by replacing a repetition to a slot of the multiple slots for TB processing. R1-2107360 Qualcomm Proposal 12: Defining a transmission occasion of TBoMS to span a single slot and restricting rate matching to occur on a per-slot basis, reuse R15/R16 framework for collision handling between PUSCH and other channels/signals for collision handling between TBoMS and other channels/signals.

## A.16 Additional indicators and configuration options

R1-2106656 Nokia/NSB Proposal 8. RAN1 to specify an indication method for enabling TBoMS transmission per PUSCH scheduling/configuration. - FFS: Details of the indication method. R1-2107191 Lenovo/Motorola Proposal 10: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, semi-static and/or dynamic configuration of TBoMS feature for PUSCH should be supported, and independent from PUSCH repetition, R1-2107651 InterDigital Proposal 1: Support dynamic enabling/disabling of TBoMS transmission using TDRA list configuration. R1-2107936 Xiaomi Proposal 7: Consider the configuration and indication signalling design when a single UE supports both repetition and TBoMS.

## A.17 Interleaved TBoMS transmissions

R1-2107360 Qualcomm Proposal 14: Interleaved TBoMS transmissions (carrying different TBs) are not permitted. A UE does not expect a TBoMS transmission in a component carrier to begin before the completion of an ongoing TBoMS transmission in the same component carrier.

## A.18 Application of TBoMS to Msg3 transmission

R1-2107198 TCL Communications Proposal 2: Study whether MSG3 support TBoMS.

## A.19 Application of DM-RS bundling to TBoMS

R1-2107198 TCL Communications Proposal 5: The inter-slot bundling with inter-slot frequency hopping should be supported for TBoMS.

# Appendix B: Previous agreements on TB processing over multi-slot PUSCH

Working assumption:  Agreement:

For TBS determination of TBoMS:

- NohPRB is configured by xOverhead and represents the overhead per slot.

- NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated.

Note: xOverhead configuration is as per Rel-15/16.

Agreement:

The following 2 options for time domain resource determination for TBoMS are considered for down-selection during RAN1 #105-e:

- Option 1: Time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA.

- Option 2: Time domain resource determination for TBoMS can be performed via PUSCH repetition Type A like TDRA or via PUSCH repetition Type B like TDRA.

  - The use of PUSCH repetition Type B like TDRA for time domain resource determination is according to an additional UE capability for a TBoMS capable UE.

  - FFS DMRS pattern for PUSCH repetition Type B like TDRA

Working assumption

A transmission occasion for TBoMS (TOT) is constituted of at least one slot or multiple consecutive physical slots for UL transmission

- FFS: whether the concept of TOT will be used for designing aspects related to signal generation, e.g., rate-matching, power control, etc.

- FFS: whether such concept will be specified or not.

Agreement:

- The structure of TBoMS will be according to only one of these two options (to be down-selected in RAN1#106-e)

  - Option 3, if a design based on single RV is adopted.

  - Option 4, if a design based on different RVs is adopted.

- FFS: other details, e.g., rate-matching, TBS determination, collision handling, etc.

- The single RV is not constrained to have only the same coded bits in each slot or in each TOT

- The concept of TOT as per the corresponding Working assumption is used to define Option 3 and Option 4 and may or may not be used to design other details, e.g., rate-matching, TBS determination, collision handling and so on.

Agreement:

Time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA.

- FFS: details

- FFS: whether or not optimizations for time domain resource determination are necessary for allocating resource in the S slots (for the unpaired spectrum case)

Working assumption

Allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS.

Agreement:

The following three options for rate-matching for TBoMS are considered for down-selection during RAN1 #106-e, where only one option will be selected:

- Option a: Rate-matching is performed per slot;

- Option b: Rate matching is performed continuously across all the allocated slot(s) per TOT;

- Option c: Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS

Note: “rate-matching is performed per X” means that the time unit for the bit selection and bit interleaving is X.

Note2: the above 3 options imply that the UL resource in the time unit may or may not be consecutive (depending on the given option)

Agreement:

Number of slots allocated for TBoMS is determined by using a row index of a TDRA list, configured via RRC.

- FFS: details.

Agreement:

The following approach is used to calculate NInfo for TBoMS:

- Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1.

  - FFS: the definition of K.

L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed, and details on how to handle such scenarios.

Agreement:

Non-consecutive physical slots for UL transmission can be used to transmit TBoMS at least for unpaired spectrum.

- How TBoMS is transmitted over non-consecutive physical slots for UL transmission for unpaired spectrum is to be discussed further.

- Whether and how non-consecutive physical slots for UL transmission can be used to transmit TBoMS for paired spectrum and SUL band as well, is to be discussed further.

Working Assumption

The concept of transmission occasion for TBoMS (TOT) is utilized for the purpose of discussion, where a TOT is constituted of time domain resources which may or may not span multiple slots

- FFS: details, whether multiple slots which constitute a TOT are consecutive or non-consecutive physical slots for UL transmissions

- FFS: other details.

- FFS: whether such concept will be specified or not.

Agreements:

For the definition of a single TBoMS, down select among the following options:

- Option 1: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using a single RV.

  - FFS: whether and how the single RV is rate matched across the TOT, e.g., continuous rate-matching across the TOT, rate matched for each slot and so on.

- Option 2: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using different RVs.

  - FFS: how RV index is refreshed within the TOT, e.g. after each slot boundary, at every jump between two non-contiguous resources, if any, and so on.

- Option 3: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using a single RV.

  - FFS: how the single RV is rate matched across single or multiple TOTs, e.g., rate matched for each TOT, rate matched for all the TOTs, rate matched for each slot and so on.

- Option 4: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using different RVs.

  - FFS: whether and how RV index is refreshed within one TOT, e.g. after each slot boundary, at every jump between two non-contiguous resources, if any, and so on.

- FFS: the exact TBS determination procedure.

- FFS: whether a single TBoMS can be repeated or not.

- FFS: other implications, e.g., power control, collision handling and so on.

Agreement:

- Consider one or two of the following options as starting points to design time domain resource determination of TBoMS

  - PUSCH repetition type A like TDRA, i.e., the number of allocated symbols is the same in each slot.

  - PUSCH repetition type B like TDRA, i.e., the number of allocated symbols in each slot are different.

Agreement:

- Consecutive physical slots for UL transmission can be used for TBoMS for unpaired spectrum.

  - To resolve in RAN1#104b-e whether to support non-consecutive physical slots for UL transmission for TBoMS for unpaired spectrum.

- Consecutive physical slots for UL transmission can be used for TBoMS for paired spectrum and the SUL band.

  - FFS if non-consecutive physical slots for UL transmission are also supported for paired spectrum and the SUL band.

Agreement:

- The same number of PRBs per symbol is allocated across slots for TBoMS transmission.

Agreement:

For TBoMS, the maximum supported TBS should not exceed legacy maximum supported TBS in Rel-15/16, for the same number of layers.

- FFS: Details and further constraints on the applicability of TBoMS.

Agreement:

One or two of the following approaches will be considered as a starting point to decide how NInfo for TBoMS is calculated (aiming for down selection in RAN1 #104-bis-e):

- Approach 1: Based on all REs determined across the symbols or slots (FFS whether symbols or slots are used) over which the TBoMS transmission is allocated.

- Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1.

  - FFS: the definition of K.

Note: L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed, and details on how to handle such scenarios.

Agreement:

One or two of the following options will be considered (aiming for down-selection in RAN1#104b-e) to calculate NohPRB for TBoMS:

- Option 1: NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated and can be configured by xOverhead as in Rel-15/16.

- Option 2: NohPRB is calculated depending on both xOverhead and the number of symbols or slots (FFS whether symbol or slot are used) over which the TBoMS transmission is allocated.

  - FFS: if either the number of symbols or the number of slots is used.

  - FFS: if xOverhead is separately configured from the one in Rel-15/16.

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed.