## NR Multicast and Broadcast Services

Please refer to RP-201038 for detailed scope of the WI.

Please also refer to section 5.1 of RP-212559 for additional guidance for this e-meeting.

[106bis-e-R17-RRC-MBS] Email discussion on Rel-17 RRC parameters for NR MBS – Fei (CMCC)

- 1st check point: October 14

- Final check point: October 19

### Mechanisms to support group scheduling for RRC_CONNECTED UEs

[106bis-e-NR-MBS-01] Email discussion/approval on mechanisms to support group scheduling for RRC_CONNECTED UEs with checkpoints for agreements on October 14 and 19 – Fei (CMCC)

R1-2108723	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2108804	Group Scheduling Aspects for Connected UEs	FUTUREWEI

R1-2108851	Discussion on Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

R1-2108926	Discussion on MBS group scheduling for RRC_CONNETED UEs	Spreadtrum Communications

R1-2109001	Discussion on mechanisms to support group scheduling for RRC_CONNECTED Ues	vivo

R1-2109067	Discussion on group Scheduling mechanism for RRC_CONNECTED UEs	OPPO

R1-2109135	Discussion on Group Scheduling Mechanisms for RRC_CONNECTED Ues	NEC

R1-2109194	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2109302	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

R1-2109303	Discussion on group scheduling mechanisms	CMCC

R1-2109316	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

R1-2109387	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UE	Xiaomi

R1-2109515	Support of group scheduling for RRC_CONNECTED UEs	Samsung

R1-2109538	On group scheduling mechanism for RRC_CONNECTED UEs	Lenovo, Motorola Mobility

R1-2109567	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

R1-2109633	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

R1-2109701	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2109767	Group scheduling related questions for RRC_CONNECTED UEs	TD Tech

R1-2109823	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	FGI, Asia Pacific Telecom

R1-2109983	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

R1-2110056	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Apple

R1-2110074	Discussion on common frequency resource configuration for multicast of RRC_CONNECTED UEs	ETRI

R1-2110118	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Convida Wireless

R1-2110210	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2110249	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Google Inc.

R1-2110255	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

R1-2110355	Mechanisms to support MBS group scheduling for RRC_CONNECTED Ues	Ericsson

R1-2110465

Agreement:

The starting PRB and the length of PRBs of CFR are jointly indicated reusing the RIV indication mechanism in the same way as locationAndBandwidth of a BWP.

Agreement:

RBG and PRG for multicast GC-PDSCH in CFR are defined using the same procedure as for unicast PDSCH in DL BWP.

      - For RBG, the size is defined based on the starting PRB of the CFR, size of the CFR and the higher layer parameter rbg-Size configured by PDSCH-Config for multicast in the CFR.

      - For PRG, the size is defined based on the starting PRB of the CFR, size of the CFR and precoding granularity for multicast which can be equal to one of the values among {2, 4, wideband}.

      - Note: Whether the RBG and PRG size for multicast (configured directly or indirectly) is the same as for unicast can be discussed separately.

Proposal:

For LBRM and TBS determination for GC-PDSCH, the default value of the maximum number of layers is 1 if maxMIMO-Layers in PDSCH-Config for MBS in CFR is not configured.

### Mechanisms to improve reliability for RRC_CONNECTED UEs

[106bis-e-NR-MBS-02] Email discussion/approval on mechanisms to improve reliability for RRC_CONNECTED UEs with checkpoints for agreements on October 14 and 19 – Jinhuan (Huawei)

R1-2108724	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2108805	Further Discussions on Reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2108852	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2108927	Mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2109002	Discussion on mechanisms to improve reliability for RRC_CONNECTED Ues	vivo

R1-2109068	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2109136	Discussion on Reliability Improvements for RRC_CONNECTED UEs	NEC

R1-2109195	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2109304	Discussion on reliability improvement	CMCC

R1-2109317	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2109516	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2109539	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2109568	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2109634	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2109702	Discussion on mechanisms to improve reliability for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2109768	PUCCH formats for NACK-ONLY based HARQ-ACK feedback	TD Tech

R1-2109984	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2110057	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2110075	Discussion on HARQ-ACK feedback mechanism for multicast of RRC_CONNECTED UEs	ETRI

R1-2110119	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

R1-2110211	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2110250	Discussion on MBS reliability for RRC_CONNECTED UEs	Google Inc.

R1-2110356	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

R1-2110408

Agreement:

The group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

Agreement:

If the group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is not configured, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

### Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

[106bis-e-NR-MBS-03] Email discussion/approval on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs with checkpoints for agreements on October 14 and 19 – David (BBC)

R1-2108725	Discussion on UE receiving broadcast in RRC IDLE/INACTIVE state	Huawei, HiSilicon, CBN

R1-2108806	MBS Support for RRC IDLE/INACTIVE UEs	FUTUREWEI

R1-2108853	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	ZTE

R1-2108928	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	Spreadtrum Communications

R1-2109003	Remaining issues on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE Ues	vivo

R1-2109069	Discussion on basic functions for RRC_IDLE/RRC_INACTIVE UEs	OPPO

R1-2109196	Discussion on basic functions for broadcast multicast for RRC_IDLE RRC_INACTIVE UEs	CATT

R1-2109305	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states	CMCC

R1-2109318	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

R1-2109517	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

R1-2109540	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

R1-2109569	Discussion on MBS for RRC_IDLE/INACTIVE UEs	MediaTek Inc.

R1-2109635	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

R1-2109703	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	NTT DOCOMO, INC.

R1-2109769	Further discussion on basic functions for RRC_IDLE/RRC_INACTIVE UEs	TD Tech, Chengdu TD Tech

R1-2109802	Considerations on MBS functions for RRC_IDLE/INACTIVE UEs	Sony

R1-2109985	Basic function for broadcast/multicast	LG Electronics

R1-2110058	Discussion on MBS for RRC_IDLE and RRC_INACTIVE UEs	Apple

R1-2110120	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

R1-2110212	Views on group scheduling for Broadcast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

R1-2110251	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Google Inc.

R1-2110258	Discussion on basic functions for broadcast or multicast for RRC_IDLE and RRC_INACTIVE UEs	ASUSTeK

R1-2110357	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

R1-2109388	Discussion on basic functions for broadcastmulticast for RRC_IDLERRC_INACTIVE UEs	Xiaomi

### Others

R1-2108854	Consideration on potential further enhancement for MBS	ZTE

R1-2109004	Other issues for Rel-17 MBS	vivo

R1-2109197	Discussion on other issues in Rel-17 MBS	CATT

R1-2109319	Other Aspects for Rel-17 MBS	Nokia, Nokia Shanghai Bell

R1-2109389	Discussion on remaining issues for idle and inactive UE	Xiaomi

R1-2109742	Impact from MCCH and MTCH on broadcast reception	Huawei, HiSilicon

R1-2109770	Discussion on typical configuration for NR MBS	CHENGDU TD TECH LTD.

R1-2109986	Other aspects for MBS	LG Electronics

R1-2110358	Assumptions for performance evaluations of NR-MBS	Ericsson