3GPP TSG RAN WG1 #106bis-e		                                   					       												                 R1-2109912

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.1

Source:	Moderator (AT&T)

Title:	Summary of UE features for further enhancements on NR-MIMO

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [106bis-e-R17-UE-features-MIMO-01] during RAN1 #106bis-e. According to the Chairman’s Notes:

[106bis-e-R17-UE-features-MIMO-01] Email discussion UE features for further enhancements on NR-MIMO – Ralf (AT&T) - 1st check point: October 14 - Final check point: October 19

The following was discussed and/or agreed during RAN1 #106bis-e within the scope of [106bis-e-R17-UE-features-MIMO-01]. All proposals are based on the latest RAN1 UE features list for Rel-17 NR in [1].

# Summary of Contributions Submitted to RAN1 #106bis-e

The following is the moderator’s summary of contributions submitted to RAN1 #106bis-e in this agenda item.

| 23. NR_FeMIMO | 23-1-1 | Unified TCI for intra- and inter-cell beam management | For both intra- and inter-cell beam management: 1. Support of joint TCI and separate DL/UL TCI with their components (configuration mechanism, QCL rules, applicable source and target signals) 2. Support of association between TCI state and UL PC settings (PLRS and other, including handling of beam misalignment for PLRS) 3. Support of MAC-CE+DCI-based beam indication (including TCI state activation, use of DCI formats 1_1/1_2 with and wthout DL assignment) |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] | On Feature 23-1-1, since a UE supporting unified TCI for intra-cell beam management might not support inter-cell beam management, we suggest separating this feature group into two different feature groups: “Unified TCI for intra-cell beam management” and “Unified TCI for inter-cell beam management”. Furthermore, a UE supporting unified TCI for inter-cell beam management shall support intra-cell beam management. Therefore, “Unified TCI for intra-cell beam management” should be the prerequisite feature group for “Unified TCI for inter-cell beam management”. Proposal: On Feature 23-1-1, separate this feature group into two different feature groups: “Unified TCI for intra-cell beam management” and “Unified TCI for inter-cell beam management”. The “Unified TCI for intra-cell beam management” should be the prerequisite feature group for “Unified TCI for inter-cell beam management”. |
| ZTE [3] | Firstly, in the main feature, we need to explicit that the following functionalities are supported as part of the basic feature in 23-1-1: - The common TCI state ID update and activation for CA - Beam alignment for PL-RS - MAC-CE-only based TCI state indication Then, a new UE feature of beam misalignment has been agreed to be introduced as a sub-feature for 23-1-1, and by default the UE should mandatorily support the case of beam alignment. Also, TCI application timing for DCI based TCI state indication should be introduced as well. Proposal: For unified TCI for intra- and inter-cell beam management, the following modification in red is proposed. 23-1-1; Unified TCI for intra- and inter-cell beam management; For both intra- and inter-cell beam management: 1. Support of joint TCI and separate DL/UL TCI with their components (configuration mechanism, QCL rules, applicable source and target signals) and Common cross-CC TCI update and activation (involving RRC common TCI state pool) 2. Support of association between TCI state and UL PC settings (PLRS and other, including handling of beam misalignment for PLRS) 3. Support of MAC-CE/MAC-CE+DCI-based beam TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment). 23-1-1a; Support of beam misalignment; Support of beam misalignment for PLRS. 23-1-1b; TCI application time; X symbols after transmitting HARQ-ACK, the indicated TCI state is applied to a downlink or uplink signal, in the case of DCI based TCI state indication. 23-1-1c; Number of activated TCI state; 1. For joint TCI, support of reporting up to Nmax_joint activated joint TCI states --- Candidate value of {1,2,3,4,5,6,7,8}; 2. For separate TCI, support of reporting up to Nmax_DL activated DL TCI states and up to Nmax_UL activated UL TCI states --- Candidate value of {1,2,3,4,5,6,7} for Nmax_DL, and candidate value of {1,2,3,4,5,6,7} for Nmax_UL;. |
| Vivo [4] | It has already been agreed as following in RAN1#104-e that support of DCI indicated beam switch is separate UE capability. Thus the third component of 23-1-1 should be a separate UE feature. Agreement For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment: - Support UE to report whether or not to support TCI update by DCI format 1_1/1_2. - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional - FFS: How to handle the case when there is only UL data - FFS: The case for UE being indicated with separate UL TCI in DCI format 1_1/1_2 with DL assignment According to the agreement, following revisions are proposed. 23. NR_FeMIMO; 23-1-1; Unified TCI for intra- and inter-cell beam management; For both intra- and inter-cell beam management: 1. Support of joint TCI and separate DL/UL TCI with their components (configuration mechanism, QCL rules, applicable source and target signals) 2. Support of association between TCI state and UL PC settings (PLRS and other, including handling of beam misalignment for PLRS) 3. Support of MAC-CE beam indication; {Support, Not support}; Optional. 23-1-1a; DCI-based beam indication with DCI formats 1_1/1_2; Support of DCI-based beam indication with DCI formats 1_1/1_2 with DL assignment; {Support, Not support}; Optional. 23-1-1b; DCI-based beam indication with DCI formats 1_1/1_2 without DL assignment; Support of DCI-based beam indication with DCI formats 1_1/1_2 without DL assignment; {Support, Not support}; Optional. |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | It was agreed in RAN#106-e that the maximum number of configured TCI state pools across BWPs and CCs in a band is reported as UE capability. Agreement (RAN1#106-e) On Rel.17 unified TCI framework, confirm the following working assumption as an agreement with a minor refinement highlighted in red For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs: - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP/CC as in Rel-15/16 - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC - When the BWP/CC ID (i.e. bwp-Id or cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies - Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1 - FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band - FFS: How to define reference BWP/CC In addition, to reduce memory consumption at UE, similar to legacy UE feature 2-4/2-59 in Rel-15/16, it should be supported to report maximum number of configured unified TCI states across BWPs/CCs in a band and per CC in a band. With above analysis, we propose the following: Proposal: Support the following UE features regarding configured TCI state/pool(s) - The maximum number of configured TCI state pools across all BWPs and all CCs in a band - The maximum number of configured TCI states across all BWPs and all CCs in a band - The maximum number of configured TCI states per CC in a band It was agreed in RAN#105-e that the maximum number of activated UL TCI states or joint TCI state per band per cell is reported as UE capability. The formulation of “per band per cell” may be misleading, and we suggest changing it to “across all BWPs and all CCs in a band”. Agreement (RAN1#105-e) On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state. - Whether a UE supports “beam misalignment or not” (detailed definition FFS) between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability - Note: The term “beam misalignment” is for discussion purpose only - Whether it is ‘included in’ or ‘associated with’ (including the manner it is performed and the signaling) is up to RAN2 - The UE maintains the PL-RS of the activated UL TCI state or (if applicable) joint TCI state - The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell is a UE capability - FFS: detailed aspects of PL-RS, e.g. CSI-RS type(s), restriction on configuration - FFS: For the definition of “beam misalignment or not”, at least consider the case where the periodic DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication is configured/associated as the PL-RS - Note: PL-RS is assumed to be periodic With above analysis, we propose the following: Proposal: Support the following UE features regarding activated TCI states - The maximum number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band - The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band 23. NR_FeMIMO; 23-1-1; Unified TCI for intra- and inter-cell beam management; For both intra- and inter-cell beam management: 1. Support of joint TCI and separate DL/UL TCI with their components (configuration mechanism, QCL rules, applicable source and target signals) 1-1. The maximum number of configured TCI state pools across all BWPs and all CCs in a band 1-2. The maximum number of configured TCI states across all BWPs and all CCs in a band 1-3. The maximum number of configured TCI states per CC in a band 2. Support of association between TCI state and UL PC settings (PLRS and other, including handling of beam misalignment for PLRS) 3. Support of MAC-CE+DCI-based beam indication (including TCI state activation, use of DCI formats 1_1/1_2 with and wthout DL assignment) 4. Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band 5. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band; {Support, Not support} 1-1. candidate values {1, …, 32} 1-2. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 1-3. candidate values {8, 16, 32, 64, 128, 256} 3. candidate values {no, with DA, w/o DA, both} 4. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 5. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 4 & 5, if two TCI states are in one codepoint, it should be counted as 2.; Optional. |
| CATT [7] | For Rel-17 UE, joint TCI may be applied when DL/UL beam correspondence are fulfilled. Otherwise, separate DL/UL TCI should be used. Both joint TCI and separate DL/UL TCI should be supported by any Rel-17 UE to cover different scenarios (beam correspondence, or not). The configuration of joint TCI or separate DL/UL TCI is based on RRC signalling. In the case of CA, RRC-configured TCI state pool(s) can be configured in the PDSCH configuration for each BWP /CC as in Rel-15/16. In addition, the TCI state pool can be absent in the PDSCH configuration for a BWP/CC. If so, a reference to RRC-configured TCI state pool(s) in a reference BWP/CC is configured instead. How many TCI state pools a UE can support across BWPs and CCs in a band depends on UE capability. This UE capability has to be reported to gNB. For PUCCH/PUSCH/SRS, the UL PC parameters except for PL-RS (P0, alpha, closed loop index) could be associated with UL TCI state or joint TCI state. For SRS, the PC parameters could also be configured as independent of the TCI state. With respect to PL-RS, whether it is included in the TCI state or associated with the TCI state hasn’t been decided yet. But for either options, there are cases that the configured PL-RS is not QCLed with the spatial relation RS in the TCI state, i.e. beam misalignment. Note that the definition of beam misalignment is still FFS. For some UEs, beam misalignment is not support due to its additional UE complexity. In this way, a UE capability is introduced to inform supportive of beam misalignment or not to gNB. Therefore, we have the following proposal as the UE feature: Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-1-1; Unified TCI for intra- and inter-cell beam management; For both intra- and inter-cell beam management: 1. Support of joint TCI and separate DL/UL TCI with their components (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of TCI state pools across BWPs and CCs in a band 3. Support of association between TCI state and UL PC settings (PLRS and other, including handling of beam misalignment for PLRS) 4. Support of MAC-CE+DCI-based beam indication (including TCI state activation, use of DCI formats 1_1/1_2 with and wthout DL assignment); Per band; Optional. In Rel-17, MAC-CE-based beam indication is supported for all the UEs. Whether to support the DCI-based beam indication (i.e. using DCI format 1_1/1_2) depends on UE capability. In addition, for a UE supporting beam indication by DCI format 1_1/1_2, it must support beam indication by using DCI 1_1/1_2 with DL assignment. But the supportive of beam indication by DCI format 1_1/1_2 without DL assignment is UE optional. Therefore two separate UE features regarding the DCI-based beam indication (i.e. use of DCI formats 1_1/1_2 with DL assignment and use of DCI formats 1_1/1_2 with and without DL assignment) are required and should be reported to gNB. If DCI-based beam indication is supported, there are two UE capabilities related. One is used to determine the beam application time, which is the first slot that is at least Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. Another UE capability is to indicate the maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell. These UE capabilities should both be known by gNB. Proposal: For DCI-based beam indication, additional UE feaures are added. Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-1-1a; DCI-based beam indication for intra- and inter-cell beam management; For both intra- and inter-cell beam management: 1. Support of DCI-based beam indication, use of DCI formats 1_1/1_2 with DL assignment 2. The minimum value (Y symbols) of beam application time 3. The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell; 23-1-1; Per band; Optional. 23-1-1b; DCI-based beam indication for intra- and inter-cell beam management; For both intra- and inter-cell beam management: 1. Support of DCI-based beam indication, use of DCI formats 1_1/1_2 with and wthout DL assignment 2. The minimum value (Y symbols) of beam application time 3. The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell; 23-1-1a; Per band; Optional. |
| Samsung [8] | Regarding FG 23-1-1, this is the feature that enables the unified TCI framework for intra-cell and inter-cell beam management. We agree with the components of this feature, namely: - Support of joint TCI beam indication and separate TCI state beam indication. In relation to this, we propose the following: - Add a UE capability for the maximum number of activated TCI states. Note that the number of activated TCI states is configurable by the NW. Whether the maximum number is defined separately for different TCI types (joint, DL-only, and UL-only) or as a total number of TCI states (across all configured TCI types) can be discussed further. This also depends on whether, e.g. UL TCI shares the same pool as joint (hence DL) TCI. - To avoid unnecessary complication/fragmentation, all Rel-17 UEs support both joint TCI and separate DL/UL TCI (i.e. there shouldn’t be separate UE capabilities for the two). - Support of association between TCI state and UL PC settings including PL-RS. Whether a UE capability on the maximum number of path-loss RS per component carrier (CC) a UE can support is needed or not can be discussed further. - Support of MAC CE/DCI based beam indication (with MAC CE based beam indication as a special case when only one TCI state is activated). To avoid unnecessary complication/fragmentation, all valid DCI formats (1_1/1_2 with and without DL assignment) should be supported by all Rel-17 UEs. In addition, the following should also be included: - Support of common beam indication across a group of carriers, including support of configuration of TCI state pool in a reference BWP/CC. - A UE capability for minimum beam activation latency (Y symbols). The candidate values for the minimum Y can be discussed further. Note that Y is configurable by the NW. Proposal: Add a UE capability for the maximum number of activated TCI states. Proposal: Whether a UE capability on the maximum number of path-loss RS a UE can support per CC is needed or not can be discussed further. Proposal: FG 23-1-1 to include support of common beam indication across a group of carriers including support of configuration of a TCI state pool in a reference BWP/CC. Proposal: FG 23-1-1 to include a UE capability for the minimum beam activation latency. |
| MediaTek Inc. [9] | In our view, beam indication for intra-cell and inter-cell cases need to be separated as two FGs, instead of one. Some additional components need to be supported for inter-cell beam indication, but not for intra-cell case. For example, inter-cell beam indication needs to support SSB(s) with PCI(s) different from serving cell PCI as direct or indirect source RS, but intra-cell case doesn't. Proposal: Introduce separate FGs for “Unified TCI framework for intra-cell beam management” and “unified TCI framework for inter-cell beam management ", e.g., - FG-23-1-1 of “unified TCI framework for intra-cell beam management” - FG-23-1-5 of “unified TCI framework for inter-cell beam management” Regarding the basic components of “unified TCI framework for intra-cell beam management”, at least the following components need to be included: - Support of joint DL/UL TCI and separate DL/UL TCI with their components (configuration mechanism, QCL rules, applicable source and target signals) - Support of association between a TCI state and UL PC settings for PUCCH, PUSCH, and SRS - Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE - Support of MAC-CE+DCI-based beam indication (including TCI state activation, use of DCI formats 1_1/1_2 with DL assignment) - Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) - The minimum beam application time in Y symbols - The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell Regarding the 2nd and 3rd components, we prefer to make UL PC settings and PL-RS as two separate components to make the components more clear. Regarding the PL-RS, according to the following agreement from RAN1#105, whether a UE can support “beam misalignment or not” is an optional capability, thus we see “beam alignment” should be supported as the baseline, and the support of “beam misalignment” should be a separate FG. Agreement from RAN1#105 On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state. - Whether a UE supports “beam misalignment or not” (detailed definition FFS) between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability - Note: The term “beam misalignment” is for discussion purpose only - …. Regarding the 4th component, according to the following agreement from RAN1#104bis, TCI update by DCI format 1_1/1_2 without DL assignment should be a UE optional capability, thus not the baseline. The support of TCI update by DCI format 1_1/1_2 without DL assignment should be a separate FG. Agreement from RAN1#104bis For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment: - … - Support UE to report whether or not to support TCI update by DCI format 1_1/1_2. - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional. Regarding the 5th component, it is added according to the following agreements from RAN1#105. Agreement from RAN1#105 On Rel.17 unified TCI framework, - Any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) - Note: This does not imply that all such DL RSs necessarily share a same TCI state - The DL RS includes CSI-RS and DMRS for PDSCH or PDCCH - FFS: Whether some SRS resources or resource sets for BM can be configured as a target signal/channel of a Rel-17 UL TCI (hence the Rel-17 UL TCI state pool) - Note: This does not imply that DL and UL TCI state pools are separate or shared for separate DL/UL TCI (this issue is still TBD). Regarding the 6th component, it is added according to the following agreements from RAN1#104 and RAN1#106. Agreement from RAN1#104 On the beam application time for Rel.17 DCI-based beam indication, the beam application time can be configured by the gNB based on UE capability - Support a UE capability for the minimum value of beam application time - … Agreement from RAN1#106 On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the first slot to apply the indicated TCI is at least Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. - Note: The Y symbols are configured by the gNB based on UE capability, which is also reported in units of symbols.. Regarding the 7th component, it is added according to the following agreement from RAN1#105. Agreement from RAN1#105 On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state. - … - The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell is a UE capability - …. Proposal: For FG-23-1-1 of “Unified TCI framework for intra-cell beam management”, at least the following components should be included in: - Support of joint DL/UL TCI update and separate DL/UL TCI update with their components (configuration mechanism, QCL rules, applicable source and target signals) - Support of association between a TCI state and UL PC settings for PUCCH, PUSCH, and SRS - Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE - Support of MAC-CE+DCI-based beam indication (including TCI state activation, use of DCI formats 1_1/1_2 with DL assignment) - Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) - The minimum beam application time in Y symbols - The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell Considering the FG of “unified TCI framework for intra-cell beam management” as prerequisite, we see the followings need to be added as separate FGs: - FG-23-1-1a: TCI update based on DCI formats 1_1/1_2 without DL assignment This FG is added according to the following agreement from RAN1#104bis. Agreement from RAN1#104bis For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment: - … - Support UE to report whether or not to support TCI update by DCI format 1_1/1_2. - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional. - FG-23-1-1b: Support of “beam misalignment or not” for PL-RS This FG is added according to the following agreement from RAN1#105. Agreement from RAN1#105 On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state. - Whether a UE supports “beam misalignment or not” (detailed definition FFS) between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability - Note: The term “beam misalignment” is for discussion purpose only - …. - FG-23-1-1c: Common TCI state ID update and activation across a set of configured CCs This FG is added according to the following agreement from RAN1#103. Agreement from RAN1#103 On Rel-17 unified TCI framework, support common TCI state ID update and activation to provide common QCL information and/or common UL TX spatial filter(s) across a set of configured CCs: - The above applies to intra-band CA - The above applies to joint DL/UL and separate DL/UL beam indications - Just as Rel.16, the RS in the TCI state that provides QCL-TypeA [or QCL-TypeB] shall be in the same CC as the target channel or RS - The common TCI state ID implies that the same/single RS determined according to the TCI state(s) indicated by a common TCI state ID is used to provide QCL Type-D indication and to determine UL TX spatial filter across the set of configured CCs - FFS: The above also applies to inter-band CA - FFS: TCI state pool for CA - Opt-1: sharing a single RRC TCI state pool for the set of configured CCs, e.g., cell-group TCI state pool, or reuse TCI state pool for PDSCH in a reference cell; A CC ID for QCL-Type A RS is absent in a TCI state, and the CC ID for QCL-Type A RS is determined according to a target CC of the TCI state. - FFS: Whether it is possible that a single TCI state in the pool includes all source RSs from different CCs - Opt-2: configuring RRC TCI state pool per individual CC - FFS: Whether the Rel-17 common beam update across multiple CCs applies to beam indication for single channel (e.g. PDSCH only, single CORESET), a subset of channels, or all channels. - FG-23-1-1d: The maximum number of TCI state pools {1,2, …} can be supported across BWPs and CCs in a band This FG is added according to the following agreement from RAN1#106. Agreement from RAN1#106 On Rel.17 unified TCI framework, confirm the following working assumption as an agreement with a minor refinement highlighted in red For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs: - … - Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1 - FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band - FFS: How to define reference BWP/CC. Proposal: Considering FG-23-1-1 of “Unified TCI framework for intra-cell beam management” as prerequisite, support the following additional FGs: - FG-23-1-1a: TCI update based on DCI formats 1_1/1_2 without DL assignment - FG-23-1-1b: Support of “beam misalignment or not” for PL-RS - FG-23-1-1c: Common TCI state ID update and activation across a set of configured CCs - FG-23-1-1d: The maximum number of TCI state pools {1,2, …} can be supported across BWPs and CCs in a band Regarding the FG of “Unified TCI framework for inter-cell beam management”, according to the following agreements, at least the following components need to be included. - Support Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) for the applicable channels and signals as for intra-cell beam management except for non-UE dedicated channels/signals - Support one Rel-17 active DL TCI state/QCL per band - Support of SSB associated with a physical cell ID different from that of the serving cell is used as an indirect QCL reference for DL TCI (in case of separate DL/UL TCI) or joint TCI, or an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI) - The maximum of physical cell IDs (including that of the serving cell) can be associated with the activated TCI states Agreement from RAN1#106 Confirm the following working assumption with revision in RED On Rel.17 beam indication enhancements for L1/L2-centric inter-cell beam management mobility, support the following: - Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) - FFS (to be decided in RAN1#106-e): Whether this also applies to PDSCH/PUSCH associated with UE-dedicated CORESETs only or additional target channels (e.g. UE-dedicated PDCCH/PUCCH) - FFS: Whether the above is supported only for joint TCI, or both joint TCI and separate DL/UL TCI (including that, if separate DL/UL TCI is supported, the DL TCI and UL TCI associated with a same cell) - FFS: Whether to support activation of TCI states for more than one cells simultaneously - FFS: Whether down-selection between MAC-CE only based and MAC-CE+DCI-based beam indication scheme is necessary - The DL QCL and UL spatial relation rules already agreed for intra-cell scenario - Already agreed up to RAN1#106-e day2 - FFS: The use of SSB associated with a physical cell ID different from that of the serving cell as an indirect QCL reference for UE-dedicated PDSCH - FFS (to be decided in RAN1#106-e): Whether this also applies to UE-dedicated PDCCH - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel - FFS (to be decided in RAN1#106-e): Whether SSB associated with a physical cell ID different from that of the serving cell can also be used as a direct QCL reference (source RS) for UE-dedicated PDCCH/PDSCH Agreement from RAN1#106 On Rel.17 beam indication enhancements for inter-cell beam management, the supported Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) applies to: - The channels and signals as for intra-cell beam management except for non-UE dedicated channels/signals - For the aforementioned applicable channels and signals, SSB associated with a physical cell ID different from that of the serving cell is used as an indirect QCL reference for DL TCI (in case of separate DL/UL TCI) or joint TCI, or an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI) - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel. Here, Rel-15/16 QCL rule is reused by replacing SSB with SSB associated with a physical cell ID different from that of the serving cell - For inter-cell beam management, the support of more than one Rel-17 active DL TCI state / QCL per band is a UE capability - If UE does not support such capability, MAC-CE based beam indication (activation of one TCI state) can be used to switch between two different DL receptions along two different beams - Note: The serving cell does not change when beam selection is done - Note: This does not preclude the possibility for TA update on non-serving cell - FFS: For a UE supporting Rel.17 beam indication feature for inter-cell beam management, up to 5 CORESETs can be configured per BWP Agreement from RAN1#106 On Rel.17 beam indication enhancements for inter-cell beam management, for the supported Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation): - Support a UE feature on how many physical cell IDs (including that of the serving cell) can be associated with the activated TCI states - FFS: If UE is configured for only one physical cell ID, decide between the following two options: - Opt1: the NW can activate TCI states associated with either the same physical cell ID as that of the serving cell or a different physical cell ID from that of the serving cell - Opt2: the NW can only activate TCI states associated with the same physical cell ID as that of the serving cell Note: The above does not necessarily mean that more than 1 physical cell ID that is not serving cell in RRC. Proposal: For FG-23-1-5 of “unified TCI framework for inter-cell beam management”, at least the following components should be included in: - Support Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) for the applicable channels and signals as for intra-cell beam management except for non-UE dedicated channels/signals - Support one Rel-17 active DL TCI state/QCL per band - Support of SSB associated with a physical cell ID different from that of the serving cell is used as an indirect QCL reference for DL TCI (in case of separate DL/UL TCI) or joint TCI, or an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI) - The maximum of physical cell IDs (including that of the serving cell) can be associated with the activated TCI states Considering the FG of “unified TCI framework for inter-cell beam management” as prerequisite, we see the followings need to be added as separate FGs: - FG-23-1-5a: Support of more than one Rel-17 active DL TCI state/QCL per band This FG is added according to the following agreement from RAN1#106. Agreement from RAN1#106 On Rel.17 beam indication enhancements for inter-cell beam management, the supported Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) applies to: - The channels and signals as for intra-cell beam management except for non-UE dedicated channels/signals - For the aforementioned applicable channels and signals, SSB associated with a physical cell ID different from that of the serving cell is used as an indirect QCL reference for DL TCI (in case of separate DL/UL TCI) or joint TCI, or an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI) - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel. Here, Rel-15/16 QCL rule is reused by replacing SSB with SSB associated with a physical cell ID different from that of the serving cell - For inter-cell beam management, the support of more than one Rel-17 active DL TCI state / QCL per band is a UE capability - If UE does not support such capability, MAC-CE based beam indication (activation of one TCI state) can be used to switch between two different DL receptions along two different beams - Note: The serving cell does not change when beam selection is done - Note: This does not preclude the possibility for TA update on non-serving cell - FFS: For a UE supporting Rel.17 beam indication feature for inter-cell beam management, up to 5 CORESETs can be configured per BWP. Proposal: Considering FG-23-1-5 of “unified TCI framework for inter-cell beam management” as prerequisite, support the following additional FGs: - FG-23-1-5a: Support of more than one Rel-17 active DL TCI state/QCL per band 23. NR_FeMIMO; 23-1-1; Unified TCI framework for intra-cell beam management; 1. Support of joint TCI update and separate DL/UL TCI update with their components (configuration mechanism, QCL rules, applicable source and target signals) 2. Support of association between a TCI state and UL PC settings for PUCCH, PUSCH, and SRS 3. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE 4. Support of MAC-CE+DCI-based beam indication (including TCI state activation, use of DCI formats 1_1/1_2 with DL assignment) 5. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) 6. The minimum beam application time of Y symbols 7. The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell; {Support, Not support} Component 6: {x, y, z, …} symbols Component 7 {a, b, c, ...}; Optional. 23. NR_FeMIMO; 23-1-1-1a; TCI update based on DCI formats 1_1/1_2 without DL assignment; Support TCI update based on DCI formats 1_1/1_2 without DL assignment; 23-1-1; {Support, Not support}. 23. NR_FeMIMO; 23-1-1-1b; “Beam misalignment or not” for PL-RS; Support of “beam misalignment or not” for PL-RS; 23-1-1; {Support, Not support}. 23. NR_FeMIMO; 23-1-1-1c; Common TCI state ID update and activation across a set of configured CCs; Support common TCI state ID update and activation across a set of configured CCs; 23-1-1; {Support, Not support}. 23. NR_FeMIMO; 23-1-1-1d; The maximum number of TCI state pools can be supported across BWPs and CCs in a band; The maximum number of TCI state pools can be supported across BWPs and CCs in a band; 23-1-1; {1, 2, 3, ...}. 23. NR_FeMIMO; 23-1-5; Unified TCI framework for inter-cell beam management; 1. Support Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) for the applicable channels and signals as for intra-cell beam management except for non-UE dedicated channels/signals 2. Support one Rel-17 active DL TCI state/QCL per band 3. Support of SSB associated with a physical cell ID different from that of the serving cell is used as an indirect QCL reference for DL TCI (in case of separate DL/UL TCI) or joint TCI, or an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI) 4. The maximum of physical cell IDs (including that of the serving cell) can be associated with the activated TCI states; 23-1-1, 23-1-2; {Support, Not support} Component 4: {1, 2, …}; Optional. 23. NR_FeMIMO; 23-1-5a; Support of more than one Rel-17 active DL TCI state/QCL per band; - Support of more than one Rel-17 active DL TCI state/QCL per band for inter-cell beam management - The maximum number of Rel-17 active DL TCI state/QCL per band for inter-cell beam management; 23-1-5; {Support, Not support} Component 2: {2,3,….}; Optional. |
| Intel Corporation [10] | The feature groups for multi-beam operation are proposed as part of FG 23-1-x family. We note that that FG 23-1-1 indicates support of beam management for both intra- and inter-cell. At the same time separate UE capability FG 23-1-2 is defined for inter-cell measurement and reporting. Considering that UE not capable of FG 23-1-2 is not expected to support FG 23-1-1 for inter-cell, it seems natural to decouple support of unified TCI framework for intra-cell and inter-cell beam management in different components. In addition, joint TCI state and sperate DL/UL TCI state should be also considered as different components to allow UE with common DL / UL beam to support only common TCI state framework. Finally, it should be noted that similar to Rel-15 TCI framework it seems necessary to define UE capability for the total number of configured TCI states per CC and the number of active TCI states to limit UE memory requirements associated with storage of the RRC parameters and UE complexity associated with tracking of the TCI states respectively. 23-1-1; Unified TCI for intra- and inter-cell beam management; For both intra- and inter-cell beam management 1. Support of joint TCI and separate DL/UL TCI with their components (configuration mechanism, QCL rules, applicable source and target signals) 1a. Support of separate DL/UL TCI with their components (configuration mechanism, QCL rules, applicable source and target signals) 2. Support of association between TCI state and UL PC settings (PLRS and other, including handling of beam misalignment for PLRS) 3. Support of MAC-CE+DCI-based beam indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment) 4. Support of intra-cell only or both intra-cell and inter-cell; Components 1-4: {support, not support}; Per band. 23-1-5; TCI states; 1) Maximum number of active TCI states (joint or separate DL/UL) per BWP per CC 2) Maximum number of configured TCI states (joint or separate DL/UL) per CC; Component 1: {1, 2, 4, 8} Component-2: {4, 8, 16, 32, 64, 128}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | We think we need to separate intra- and inter-cell beam management enhancement and the main FG should cover the basic functionality of unified TCI indication. - We propose to introduce FG23-1-1a for the action delay (Y symbols) based on the previous agreement - We propose to introduce FG23-1-1b for the number of TCI states related UE capability for separate TCI indication, addressing both UE memory and complexity. - We propose to introduce FG23-1-1c for the number of TCI states related UE capability for joint TCI indication, addressing both UE memory and complexity. - We propose to introduce FG23-1-1d for beam misalignment for power control based on the previous agreement - We propose to introduce FG23-1-1e for the maximum number of CCs configured with BFR with unified TCI 23. NR_FeMIMO; 23-1-1; Unified TCI indication; - 1. - Supported unified TCI indication mechanism; Y; N/A; UE does not support unified TCI framework; Per band; N; N; Component 1: {MAC CE only, MAC CE+DCI with data, MAC CE+ DCI with and without data}; Optional. 23. NR_FeMIMO; 23-1-1a; Action delay for unified TCI; 1. Minimum action delay (Y symbols) for unified TCI indication in DCI; 23-1-1; Y; N/A; UE does not support DCI based unified TCI indication; Per band; N; N; Component 1: 15kHz SCS: {14, 28, 42} symbols 30kHz SCS: {14, 28, 42} symbols 60kHz SCS: {14, 28, 42} symbols 120kHz SCS: {14, 28, 42} symbols; Optional. 23. NR_FeMIMO; 23-1-1b; Maximum number of unified TCI states for separate TCI indication; 1. Maximum number of active DL TCI states 2. Maximum number of active UL TCI states 3. Maximum number of active DL and UL TCI states 4. Maximum number of configured DL TCI states per TCI states pool 5. Maximum number of configured UL TCI states per TCI states pool 6. Maximum number of configured DL and UL TCI states per TCI states pool 7. Maximum number of DL/UL TCI states pools; 23-1-1; Y; N/A; UE does not support separate TCI indication; Per band; N; N; Component 1: {1, 2, 4, 8} Component 2: {1, 2, 4, 8} Component 3: {1, 2, 4, 8}, if a UL TCI and a DL TCI share the same QCL property as defined in FG 2-62, the UL and DL TCI states are counted as 1. Component 4: {8, 16, 32, 64, 128, X} Component 5: {8, 16, 32, 64, X} Component 6: {8, 16, 32, 64, 128, X} Component 7: {1, 2, 3, 4}; Optional. 23. NR_FeMIMO; 23-1-1c; Maximum number of TCI states for joint TCI indication; 1. Maximum number of active joint TCI states 2. Maximum number of configured joint TCI states per TCI states pool 3. Maximum number of joint TCI states pools; 23-1-1; Y; N/A; UE does not support joint TCI indication; Per band; N; N; Component 1: {1, 2, 4, 8} Component 2: {8, 16, 32, 64, 128, X} Component 3: {1, 2, 3, 4}; Optional. 23. NR_FeMIMO; 23-1-1d; Uplink power control for unified TCI; 1. Support beam misalignment between PL-RS and spatial relation RS; 23-1-1; Y; N/A; UE does not support power control parameters to be associated with unified TCI; Per band; N; N; FR2 only; Component 1: {Support, not support}; Optional. 23. NR_FeMIMO; 23-1-1e; BFR with unified TCI; 1. Maximum number of CCs configured with BFR; 23-1-1; N; N/A; Per band; N; N; Component 1: {1, 2, 3, 4}; Optional. |
| Qualcomm Incorporated [13] | In the preliminary UE feature list (R1-2108679), a single capability is defined for unified TCI for both intra-BM and inter-cell BM. Ideally, they belong to two different features and should have separate capabilitites, e.g. UE supporting inter-cell BM should additionally support QCL source RS from non-serving SSB as well as L1-RSRP measurement on non-serving SSB. Proposal: The unified TCI for intra-cell BM and inter-cell BM should have separate capabilities, e.g. FG 23-1-1 and FG 23-1-2. For intra-cell BM, we also have the following proposals Proposal: As in FG 23-1-1, supporting intra-cell BM implies supporting the following basic sub-features - Support of joint DL/UL TCI state - Support of TCI-independent UL PC settings except for PL RS - Support individual BWP/CC based TCI pool configuration as in R15/16 - Support of MAC-CE based beam indication - Support of MAC-CE+DCI based beam indication with DL assignment - Only for UE supporting more than 1 active TCI state Proposal: UE supporting intra-cell BM may optionally support the following sub-features - Support of separate DL/UL TCI state (FG 23-1-1a) - Support of TCI-specific UL PC settings except for PL RS (FG 23-1-1b) - Support of beam misalignment for PL RS and associated UL beam (FG 23-1-1c) - Support of MAC-CE + DCI based beam indication without DL assignment (FG 23-1-1d) - Support of a common TCI state ID activated/selected for a set of BWPs/CCs (FG 23-1-1f) - Support a TCT state pool configured on a reference BWP/CC shared by a set of BWPs/CCs (FG 23-1-1h) Proposal: UE supporting intra-cell BM should clarify the following capabilities - Minimum time duration between beam indication DCI and TCI application time (FG 23-1-1e) - Maximum number of configured TCI state pools across BWPs/CCs (FG 23-1-1h) - Maximum number of activated and configured TCI states (FG 23-1-1i) 23. NR_FeMIMO; 23-1-1; Unified TCI for intra-cell beam management; - Support of joint DL/UL TCI state - Including QCL rules, applicable source and target signals - Support of TCI-independent UL PC settings except for PL RS - Support individual BWP/CC based TCI pool configuration as in R15/16 - Support of MAC-CE based beam indication - Support of MAC-CE + DCI based beam indication with DL assignment - Including DCI format 1_1 and 1_2; Per band; Note: Component 4 is only supported by UE supporting more than 1 active TCI state; Optional. 23. NR_FeMIMO; 23-1-1a; Unified TCI for intra-cell beam management with separate DL/UL TCI; - Support of separate DL/UL TCI state - Including QCL rules, applicable source and target signals; FG 23-1-1; Per band; Optional. 23. NR_FeMIMO; 23-1-1b; Unified TCI for intra-cell beam management with TCI-specific UL PC settings except for PL RS; - Support of TCI-specific UL PC settings except for PL RS; FG 23-1-1; Per band; Optional. 23. NR_FeMIMO; 23-1-1c; Unified TCI for intra-cell beam management with beam misalignment for PL RS; - Support of beam misalignment for PL RS and associated UL beam; FG 23-1-1; Per band; Note:; Optional. 23. NR_FeMIMO; 23-1-1d; Unified TCI for intra-cell beam management with DCI based beam indication without DL assignment; - Support of MAC-CE + DCI based beam indication without DL assignment - Including DCI format 1_1 and 1_2; FG 23-1-1; Per band; Note:; Optional. 23. NR_FeMIMO; 23-1-1e; Time duration between DCI and TCI application time; - Minimum time duration, Xi, between the last symbol of the beam indication DCI and the 1st slot to apply the TCI indicated by the DCI - Xi is the number of OFDM symbols, i is the index of SCS, l=1,2,3,4 corresponding to 60,120,480, 960 kHz SCS.; FG 23-1-1; Per band; Note: Applicable only to FR2; Candidate value set for Xi is {…},. 23. NR_FeMIMO; 23-1-1f; Unified TCI for intra-cell beam management with common TCI ID selected for a group of CCs; - Support of a common TCI state ID activated or selected for a set of BWPs/CCs; FG 23-1-1; Per band; Note: Supported TCI type is indicated in FG 23-1-1 and FG 23-1-1a; Optional. 23. NR_FeMIMO; 23-1-1g; Unified TCI for intra-cell beam management with configured TCI pool shared by a group of CCs; - Support a TCI state pool configured on a reference BWP/CC shared by a set of BWPs/CCs; FG 23-1-1; Per band; Note: Supported TCI type is indicated in FG 23-1-1 and FG 23-1-1a; Optional. 23. NR_FeMIMO; 23-1-1h; Maximum number of configured TCI pool across CCs; - Maximum number of configured TCI state pools across BWPs/CCs; FG 23-1-1; Per band; Note: Supported TCI type is indicated in FG 23-1-1 and FG 23-1-1a; Optional. 23. NR_FeMIMO; 23-1-1i; Maximum number of activated and configuredTCI states; - Maximum number of activated TCI states per BWP/CC - Maximum number of configured TCI states per BWP/CC - Maximum number of activated TCI states across BWPs/CCs in a band - Maximum number of configured TCI states across BWPs/CCs in a band; FG 23-1-1; Per band; Note 1: Supported TCI type is indicated in FG 23-1-1 and FG 23-1-1a Note 2: Maximum number in each component is across all supported TCI types; Component -1: Candidate value set: {1, 2, 4, 8} Component-2: candidate value set: {4, 8, 16, 32, 64, 128} Component -3: Candidate value set: {…} Component-4: candidate value set: {…}. As in FG 23-1-5, the support of multiple SRS resource sets with different SRS antenna port number includes the following components - Maximum number of SRS resource sets with different number of SRS antenna ports (Component 1 in FG 23-1-5) - Maximum number of SRS resources per set with different number of SRS antenna ports (Component 2 in FG 23-1-5) As listed in FG 23-1-5a, UE supporting FG 23-1-5 can further indicate whether the reported SRS antenna port number can be applied to a set of BWPs/CCs when the associated reported DL RS indicates a common UL beam for the set of BWPs/CCs 23. NR_FeMIMO; 23-1-5; Multiple SRS resource sets with different number of SRS antenna ports; 1. Maximum number of SRS resource sets with different number of SRS antenna ports 2. Maximum number of SRS resources per set with different number of SRS antenna ports; Per band; Component -1: Candidate value set: {0,1,2} Component-2: candidate value set: {1,2,3,4}. 23. NR_FeMIMO; 23-1-5a; Reported SRS antenna port number applicable to multiple BWPs/CCs; 1. Support of reported SRS antenna port number applicable to a set of BWPs/CCs when the associated reported DL RS indicates a common UL beam for the set of BWPs/CCs; FG 23-1-5; Per band; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-1-2 | Inter-cell measurement and reporting (for inter-cell BM and mTRP) | 1. Support of beam measurement on SSB(s) with PCI(s) different from serving cell PCI 2. Support of beam reporting up to Kmax=4 SSBRI-RSRP pairs, with at least one pair associated with a PCI different serving cell PCI |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] | On Feature 23-1-2, in RAN1#106-e meeting, the following was agreed. Therefore, depending on agreements in this meeting, the maximum number of RRC configured PCIs different from the serving cell for measurement/reporting might also be a UE capability and be added to this feature. Agreement On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for inter-cell beam management and inter-cell mTRP, select NMAX (the maximum number of RRC configured PCIs different from the serving cell for measurement/reporting) from the following alternatives (to be decided in RAN1#106bis-e): - Alt1: NMAX is up to UE capability with candidate values of 1 and X. - Note: X as agreed in AI 8.1.2.2 - When NMAX is configured to be X, the UE measures up to X PCIs different from the serving cell PCI - Additional restriction may be added by RAN4 - Alt2. NMAX=1. Proposal: On Feature 23-1-2, depending on agreement in RAN1#106-bis-e, the maximum number of RRC configured PCIs different from the serving cell for measurement/reporting might also be a UE capability and be added to this feature. |
| ZTE [3] | Firstly, regarding PCI(s) different from serving cell PCI, we prefer to reuse the legacy UE feature of [2-24] SSB/CSI-RS for beam measurement as a starting point. Then, based on already agreement, the maximum value of supported K is a UE capability, and then the corresponding candidate value should be provided, e.g., {1, 2, 4} as legacy beam reporting. Proposal: For inter-cell measurement and reporting (for inter-cell BM and mTRP), the following modification is proposed in red 23-1-2; Inter-cell measurement and reporting (for inter-cell BM and mTRP); 1. Support of beam measurement on SSB(s) with PCI(s) different from serving cell PCI 2. Support of beam reporting up to Kmax= 4 SSBRI-RSRP pairs, with at least one pair associated with a PCI different serving cell PCI --- Candidate value of {1,2,4}. |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | As discussed in RAN1#106-e, it is necessary to have UE capability reporting on max number of RRC-configured PCI(s) different from serving cell PCI for beam measurement. More importantly, given that the number of SSBs are different in FR1 and FR2, we think it is necessary to have separate UE features on this for FR1 and FR2. The recommended candidate values for FR1 and FR2 are {1, 2, 4, 6} and {1, 2, 4}, respectively. Proposal 2-3: Support the following UE features on RRC-configured PCI(s) different from serving cell PCI - The number of configured PCI(s) different from serving cell PCI for beam measurement in FR1 - The number of configured PCI(s) different from serving cell PCI for beam measurement in FR2 Regarding the number of SSBs with PCI(s) different from serving cell PCI for L1 beam measurement, to keep the memory consumption and computing complexity at UE under control, we propose the following: Proposal 2-4: Support the following UE features on configured SSBs with PCI(s) different from serving cell PCI for beam measurement - The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI - The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI In addition, to mitigate UE complexity increase from L1 inter-cell beam measurement on top of L3 mobility measurement, it would be beneficial to allow UE to report not supporting measurement on SSBs with PCIs that are not included in L3 mobility measurement. We propose the following: Proposal 2-5: Support the following UE features on L1 measurement on SSBs with PCI(s) different from serving cell PCI - Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement 23. NR_FeMIMO; 23-1-2; Inter-cell measurement and reporting (for inter-cell BM and mTRP); 1. Support of beam measurement on SSB(s) with PCI(s) different from serving cell PCI 2. Support of beam reporting up to Kmax=4 SSBRI-RSRP pairs, with at least one pair associated with a PCI different from serving cell PCI in one report instance 3. The number of RRC-configured PCI(s) different from serving cell PCI for beam measurement in FR1 4. The number of RRC- configured PCI(s) different from serving cell PCI for beam measurement in FR2 5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI 6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI 7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement; {Support, Not support} 3. candidate values {1, 2, 4, 6} 4. candidate values {1, 2, 4} 5. candidate values: {2, 4, 8, FFS} 6. candidate values: {4, 8, 16, 32, 64, FFS} 7. candidate values {not support}; Optional. |
| CATT [7] | For inter-cell beam management, UE would measure SSBs of different cells and report corresponding L1-RSRP values. Besides the serving cell, up to NMAX non-serving cells could be configured for measurement. And K beams are reported in a single CSI reporting instance, where at least the beams of non-serving cells are included. The value of NMAX and the maximum value of K are both UE capabilities, which should be reported to gNB. As there is a separate UE feature for inter-cell mTRP, we prefer to remove mTRP in the feature group. In addition, the number of non-serving cells allowed to be configuration should be added in the component part. Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-1-2; Inter-cell measurement and reporting (for inter-cell BM and mTRP); 1. Support of beam measurement on SSB(s) with PCI(s) different from serving cell PCI 2. The maximum number of RRC configured PCIs different from the serving cell for measurement/reporting 3. Support of beam reporting up to Kmax=4 SSBRI-RSRP pairs, with at least one pair associated with a PCI different serving cell PCI; Per band; {Support, Not support}; Optional. |
| Samsung [8] | Regarding FG 23-1-2, this feature should cover both inter-cell beam measurement and reporting and inter-cell beam indication. We should rename this feature to support of inter-cell beam management. Hence, in addition to the two components listed for FG 23-1-2, namely: - Support of beam measurement and reporting on SSBs of cells with PCI different from that of the serving cell. Include a UE capability on maximum number of cells with PCI(s) different from that of the serving cell the UE can measure. - Support beam reporting for up to beams (SSBRI-RSRP pairs) including beams with cells with PCI(s) different from that of the serving cell. In addition, the following should be cover by FG 23-1-2: - Activation of TCI states associated, through indirect QCL Type-D relation, with SSBs of cells with PCI different from that of the serving cell. Include a UE capability on maximum number of cells with different PCIs (including the serving cell) the activated TCI states can be associated with. Proposal: Rename FG 23-1-2 to “support of inter-cell beam management”. Proposal: Add a UE capability on maximum number of cells with PCI(s) different from that of the serving cell the UE can measure. Proposal: FG 23-1-2, to include support of activation of TCI states associated, through indirect QCL Type-D relation, with SSBs of cells with PCI different from that of the serving cell. Proposal: Add a UE capability on maximum number of cells with different PCIs the activated TCI state can be associated with. |
| MediaTek Inc. [9] | Regarding the FG of inter-cell measurement and reporting (for inter-cell BM and mTRP), according to the following agreements, at least the following components need to be included: - Support of L1-RSRP measurement on SSB(s) with PCI(s) different from serving cell PCI - Support of L1-RSRP reporting of K SSBRI-RSRP pairs, with at least one pair associated with a PCI different serving cell PCI - Support of SSBRI-RSRP pair(s) associated with a non-serving cell can be mixed with that associated with serving-cell in one reporting instance - The maximum number of K SSBRI-RSRP pairs {1,2,3,4} can be supported in one reporting instance Agreement from RAN1#104 On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP: - Rel.15 L1-RSRP is used as reporting quantity for measurement and reporting of non-serving-cell(s) - Support SSB as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP, and Rel.15 SS-RSRP calculated from SSB of non-serving cell(s) - FFS: Whether the measurement for SS-RSRP is limited within SMTC - FFS: Detailed reporting method, e.g. via including existing L1-RSRP report, UE-initiated report etc. - FFS: Whether or not to support CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP. If the support of CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP is confirmed, Rel.15 CSI-RSRP is also supported - Whether the support applies to CSI-RS with or without QCL source, or both - FFS: The number of non-serving cell(s) for measurement/reporting - FFS: time behavior of the reporting, i.e. periodic, semi-persistent, aperiodic, or UE-initiated - FFS: If other reporting quantities are supported, e.g. L3-RSRP, hybrid L1/L3-RSRP - FFS: Dynamic activation/deactivation/selection of the beam measurement on the RS(s) associated with non-serving cell(s) via MAC CE - FFS: Timing assumption (e.g. time of arrival and time of the measurement) for measurement of non-serving cell RS measurement Agreement from RAN1#104 On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP: - A quality of up to K beams associated at least with non-serving cell(s) can be reported in a single CSI reporting instance - For each beam, the UE can report at least: (1) a Measured RS Indicator, and (2) a Beam Metric associated with the Measured RS Indicator - FFS: Maximum value of K - FFS: If K is fixed, configured, reported by UE capability, or dynamically selected - FFS: The type of beam metric (e.g. L1-RSRP, L3-RSRP, or hybrid L1/L3-RSRP) and related measurement behavior - FFS: Whether or not beam reporting associated with non-serving cell(s) can be mixed with that with serving-cell in one reporting instance Agreement from RAN1#104bis On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, - In one reporting instance, depending on NW configuration, beam(s) associated with a non-serving cell can be mixed with that associated with serving-cell - FFS: whether this applies to periodic, semi-persistent, and/or aperiodic - FFS: How to report the K beams and corresponding qualities if the Tx power among the non-serving cell and with serving-cell is not the same - Note: The supported numbers of non-serving cells (in terms of measurement/reporting) have not yet been decided. The above description doesn’t imply only one non-serving cell is allowed to be configured for measurement. Nor does this imply that only one non-serving cell is allowed in one reporting instance. Agreement from RAN1#105 On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, - Support at least K=4, where K is defined as the number of beams associated at least with non-serving cell(s) reported in a single CSI reporting instance - The maximum value of supported K is a UE capability - K is configured by NW based on the UE capability - FFS: The support of K=8 and 16 - For K>4, the maximum number of beams associated with one cell is 4 - FFS: Support L1-based event-driven reporting based on Rel-16 SCell BFR framework or analogous to L3-based event-driven reporting, including the definition of L1-based event, if needed Note: If another beam metric other than L1-RSRP is supported (e.g. L3-RSRP is still FFS), the above also applies Conclusion from RAN1#106 On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for inter-cell beam management and inter-cell mTRP, there is no consensus in supporting additional value(s) of KMAX other than 4. Proposal: For FG-23-1-2 of “inter-cell measurement and reporting (for inter-cell BM and mTRP)”, at least the following components should be included in: - Support of L1-RSRP measurement on SSB(s) with PCI(s) different from serving cell PCI - Support of L1-RSRP reporting of K SSBRI-RSRP pairs, with at least one pair associated with a PCI different serving cell PCI - Support of SSBRI-RSRP pair(s) associated with a non-serving cell can be mixed with that associated with serving-cell in one reporting instance - The maximum number of K SSBRI-RSRP pairs {1,2,3,4} can be supported in one reporting instance 23. NR_FeMIMO; 23-1-2; Inter-cell measurement and reporting (for inter-cell BM and mTRP); 1. Support of L1-RSRP measurement on SSB(s) with PCI(s) different from serving cell PCI 2. Support of L1-RSRP reporting of K SSBRI-RSRP pairs, with at least one pair associated with a PCI different serving cell PCI 3. Support of SSBRI-RSRP pair(s) associated with a non-serving cell can be mixed with that associated with serving-cell in one reporting instance 4. The maximum number of K SSBRI-RSRP pairs can be supported in one reporting instance; {Support, Not support} Component 4: {1, 2, 3, 4}; Optional. |
| Intel Corporation [10] | 23-1-2; Inter-cell measurement and reporting (for inter-cell BM and mTRP); 1. Support of beam measurement on SSB(s) with PCI(s) different from serving cell PCI 2. Support of beam reporting up to Kmax=4 SSBRI, SS-RSRP pairs, with at least one pair associated with a PCI different serving cell PCI; Components 1-2 {support, not support}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | In components, we propose to separate beam measurement from beam indication and add two more components - We propose to introduce FG23-1-2a related to the maximum number of PCIs associated with TCI - We propose to introduce FG23-1-2b related to inter-cell L1-RSRP measurement with overlapping resources/channels 23. NR_FeMIMO; 23-1-2; Inter-cell beam management; 1. Support of L1-RSRP measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of SSB(s) with a PCI different from serving cell PCI as the indirect QCL source for PDSCH/PDCCH 3. Support of the maximum value of K, i.e., the number of beams associated with at least non-serving cell(s) that can be reported in a single CSI reporting instance 4. Support of maximum number of non-serving cells, i.e., NMAX, configured for inter-cell beam measurement; 23-1-1; Y; N/A; UE does not support inter-cell BM; Per band; N; N; Component 1: {Support, Not support} Component 2: {Support, Not support} Component 3: {1, 2, 3, 4} Component 4: {1, 2, 3, 4}; Optional. 23. NR_FeMIMO; 23-1-2a; Maximum number of PCIs associated with TCI states; 1. Maximum number of PCIs associated with the configured TCI states 2. Maximum number of PCIs associated with the active TCI states; 23-1-2; Y; N/A; UE does not support separate TCI indication; Per band; N; N; Component 1: {1, X} Component 2: {1, X}; Optional. 23. NR_FeMIMO; 23-1-2b; L1-RSRP measurement with overlapping resources/channels/signals; 1. Support of L1-RSRP measurement for SSBs with overlapped resources 2. Support of SSBs for L1-RSRP measurement overlapped with PDSCH/PDCCH/CSI-RS QCLed indirectly or directly with SSBs with a different PCI; 23-1-2; N; N/A; N/A.; Per band; N; N; Component 1: {Support, Not support} Component 2: {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | As in FG 23-1-2, supporting inter-cell BM implies supporting SSB from non-serving cell as direct/indirect beam indication RS in unified TCI state - This should be a separate capability from intra-cell BM 23. NR_FeMIMO; 23-1-2; Unified TCI for intra-cell beam management; 1. Support of SSB from non-serving cell as direct/indirect QCL source RS and/or spatial relation RS in unified TCI state; FG 23-1-1; Per band; Note:; Optional. |
| Nokia/Nokia Shanghai Bell [14] | Add 2-24 and 2-29 as pre-requisites |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and associated P-MPR in PHR reporting) |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | Based on the following agreement, the N P-MPR values and corresponding SSBRI(s)/CRI(s) can be reported. In our views, M should be fixed to 1, and if sharing the same MPE impacts, the UE can provide same P-MPR values for different SSBRI(s)/CRI(s), respectively, in a report instance. Therefore, a new sub-feature of maximum number of P-MPR and SSBRI/CRI pairs can be introduced, and reusing legacy candidate values for normal beam report can be considered as a starting point, i.e., {1, 2, 4}. Then, based on the following highlighted part, we need to further consider another new UE feature of maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation. As a candidate value, we think that at least the value of ‘64’ should be supported. Agreement On Rel.17 enhancements to facilitate MPE mitigation, support the following enhancement on the Rel-16 event-triggered P-MPR-based reporting (included in the PHR report when a threshold is reached, reported via MAC-CE): - In addition to the existing field in the PHR MAC-CE, N≥1 P-MPR values can be reported - The N P-MPR values are reported together with the following: - (Working Assumption) For each P-MPR value, up to M SSBRI(s)/CRI(s), where the SSBRI(s)/CRI(s) is selected by the UE from a candidate SSB/CSI-RS resource pool (FFS: how to perform the selection) - FFS: The supported value(s) of M. Proposal: For MPE mitigation, the following modification is proposed in red 23-1-3; MPE mitigation; 1. Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and associated P-MPR in PHR reporting) 2. Support of reporting up to Nmax P-MPR and SSBRI/CRI pairs --- Candidate value of {1,2,4} 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation. --- Candidate value at least includes {64}. |
| Vivo [4] | In the following feature, we would like to further indicate maximum number of different PMPRs that UE can report in a single report. 23. NR_FeMIMO; 23-1-3; MPE mitigation; 1.Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and associated P-MPR in PHR reporting) 2. Number of different PMPRs that UE can report in a single event report.; {Support, Not support}; Optional. |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] |  |
| Samsung [8] |  |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] | 23-1-3; MPE mitigation; Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and associated P-MPR in PHR reporting); {support, not support}; Per UE. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | We propose to add component for the maximum number of P-MPR in a MAC-CE report 23. NR_FeMIMO; 23-1-3; MPE mitigation; 1. Maximum number of P-MPR in a MAC CE report; Y; N/A; UE does not support beam-specific P-MPR report; Per band; N; N; FR2 only; Component 1: {1, 2, 3, 4}; Optional. |
| Qualcomm Incorporated [13] | As in FG 23-1-3, the support of inter-cell measurement/reporting comprises the following 3 components - Maximum number of configured non-serving cells for L1-RSRP measurement (Component 1 in FG 23-1-3) - Maximum number of reported DL RSs in a beam report (Component 2 in FG 23-1-3) - Maximum number of time overlapped SSBs for L1-RSRP measurement (Component 3 in FG 23-1-3) 23. NR_FeMIMO; 23-1-3; Inter-cell measurement and reporting (for inter-cell BM and mTRP); 1. Maximum number of configured non-serving cells for L1-RSRP measurement 2. Maximum number of reported DL RSs in a beam report with measured DL RSs from both serving and non-serving cells 3. Maximum number of time overlapped SSBs from different cells for L1-RSRP measurement; FG 23-1-2, FG 23-4; Per band; Component -1: Candidate value set: {0,1} Component-2: candidate value set: {0,1,2,3,4} Component -3: Candidate value set: {0,1}. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] |  |
| Samsung [8] |  |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] |  |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] |  |
| Qualcomm Incorporated [13] | As in FG 23-1-4, the support of MPE mitigation can be indicated by the maximum number of additionally reported P-MPR values per CC in the enhanced P-MPR report. 23. NR_FeMIMO; 23-1-4; MPE mitigation; 1. Maximum number of additionally reported P-MPR value(s) per CC in enhanced P-MPR report; FG 23-1-1a; Per band; Candidate value set: {0,1,2,3}. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-2-1 | Multi-TRP PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Maximum number of such overlaps in 3 |  |  |  |  |  |  |  |  | Component1: {Support, Not support} Component2: {2, 3, [{2,3}]} Component3: {monitored, not monitored} Component4: details FFS | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | For the third bullet in FG 23-2-1, based on previous RAN1 discussion, when 2 BDs are counted for the two linked PDCCH candidates, UE can do ether individual detection or soft combining. And when 3 BDs are counted for the linked candidates, it is assumed that UE has to do both individual detection and soft combining. That’s why the total number of BDs is 3 which is larger than 2. Therefore, it is clear that the component 3 is only needed when 2 BDs are counted. Here is our suggested change. 23-2-1; Multi-TRP PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET - This is only applicable if the number of BDs is 2 4. Maximum number of such overlaps in 3. Proposal: For FG 23-2-1, add a note to clarify that component 3 is only for the case of BD = 2. |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | Regarding the multi-TRP PDCCH repetition enhancement up to RAN1#106, three schemes have been agreed such as SFN, TDM and FDM non-SFN in Rel-17. Therefore, the combinations of supported schemes should be a UE capability. For capability on SFN PDCCH, this can be a UE capability captured in HST-SFN feature. Proposal: To support the following UE capability with regard to the selection of multi-TRP PDCCH repetition enhancement schemes, - Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) with non-SFN scheme TDM and FDM, candidate values: {none, TDM, FDM, both} - Support of PDCCH repetition with SFN scheme - Note: this can be the capability captured for HST-SFN For the UE capability reporting for enhancements on multi-TRP for PDCCH, the following agreement is achieved: Agreement When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET - Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate). - Whether the individual candidate is monitored or not is determined by a UE capability - FFS (In UE feature session): The details including reusing the reported number of BDs for this purpose, or relation to reported number of BDs - In both cases, the individual candidate is not counted toward the BD limit. - UE capability for max number of such overlaps is introduced - FFS: Value of 0 is included as a candidate value for the UE capability - The details to be discussed as part of UE capability discussions - FFS: When the individual candidate is monitored, the scenario where the other linked candidate is also “overlapping” (same CORESET, DCI size, CCEs, scrambling) with a second individual candidate The details on capability reporting of whether the individual candidate is monitored or not is TBD. In our view, a typical implementation of 2 BDs is that UE decodes the linked candidates individually. And for soft-combining that requires further processing, UE can report a capability of 3 BDs to relax the requirement. And if selective decoding is assumed, the individual candidate can naturally be monitored, while for soft combining, whether the individual candidate can be monitored or not is reported. Proposal: When an individual candidate uses the same set of CCEs as one of a linked PDCCH candidates, and they both are associated with the same DCI size, scrambling, and CORESET, - If 2 BDs are reported for the linked two PDCCH candidates, the individual candidate is assumed to be monitored; - If 3 BDs are reported for the linked two PDCCH candidates, whether the individual candidate is assumed to be monitored or not should be reported by the UE. 23. NR_FeMIMO; 23-2-1; Multi-TRP PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) with non-SFN scheme TDM and FDM 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs 3.1: if 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs. 4. Maximum number of such overlaps in 3; Component1: {none, TDM, FDM, both}{Support, Not support} Component2: {2, 3, [{2,3}]} Component3: {monitored, not monitored} Component 3: {support, not support} Component4: details FFS; Optional. |
| CATT [7] | When PDCCH repetition is performed, it is necessary for UE to report one [or more] number(s) as required number of BDs for the two PDCCH candidates. Therefore, reporting number(s) as required number of BDs is basic UE capability for PDCCH repetition. However, when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET, it was agreed whether the individual candidate is monitored or not depends on additional optional UE capability. Therefore, individual candidate monitoring shall be separated as optional UE capability, and its prerequisite feature group is 23-2-1 multi-TRP PDCCH repetition. Based on the above analysis, we have the following proposal: Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-2-1; Multi-TRP PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Maximum number of such overlaps in 1; Component1: {Support, Not support} Component2: {2, 3, [{2,3}]} Component3: {monitored, not monitored} Component4: details FFS; Optional. 23-2-1a; Individual candidate monitoring when overlaps with linked PDCCH candidates; 1. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 2. Maximum number of such overlaps in 1; 23-2-1; Component1: {monitored, not monitored} Component2: details FFS; Optional. |
| Samsung [8] | Regarding FG 23-2-1 for supporting PDCCH repetition, the name of the FG would be PDCCH repetition (i.e., removing “Multi-TRP”) since single-TRP based PDCCH repetition is also agreed on this agenda item based on the definition of PDCCH repetition (two linked SS sets associated with corresponding CORESET(s) and the corresponding CORESET(s) can be both same and different CORESETs). Also, in order to have fine granularity of UE capability reporting, the reporting level can be per FSPC. Proposal: Modify the name of FG 23-2-1 as PDCCH repetition (i.e., removing “Multi-TRP”). Proposal: Support FG 23-2-1 per FSPC. |
| MediaTek Inc. [9] | For component 2, the UE doesn’t have to report multiple numbers because reporting value 3 means basically UE can also support 2 BDs. Proposal: Remove [or more] in the description of component 2. Remove {2,3} in the candidate value of component 2. UE doesn’t have to report component 4 of FG 23-2-1 if UE reports ‘not monitored’ in component 3 of FG 23-2-1. Therefore, we propose to make a separate FG for component 4. We also want to propose candidate values for this new FG. Proposal: Make a separate FG for component 4 of FG 23-2-1. The candidate values can be {4, 8, 16, 32}. 23. NR_FeMIMO; 23-2-1; Multi-TRP PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one number as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET; Component1: {Support, Not support} Component2: {2, 3} Component3: {monitored, not monitored}; Optional. 23. NR_FeMIMO; 23-2-1a; Overlap of PDCCH candidates; Maximum number of overlaps between one of linked PDCCH candidates and an individual PDCCH candidate when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET; {4, 8, 16, 32}; Optional. |
| Intel Corporation [10] | Reagrding UE capabilities for multi-TRP PDCCH. For component 2 of FG 23-2-1, candidate values for the number of required BD for two PDCCH candidates should be {2}, {3} and there is no need for UE to report both 2 and 3, i.e., candidate values of {2,3} should be removed. In addition, component 3 of FG 23-2-1 should not be applicable for BD=3 or in other words, if BD=3 is reported in component 2, individual candidate monitoring should be always supported (i.e. component 2 should be set ‘monitored’). Component 4 should have no dependency on component 3. In addition, further discussion is needed on necessity of sperate FG 23-2-3, i.e., whether FG 23-2-3 should be assumed as basic functionality and captured as part of component 1 in FG 23-2-1. 23-2-1; Multi-TRP PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) including PDCCH repetition for Type 3 CSS 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET. 4. Maximum number of overlaps in a slot of individual candidate and one of the linked PDCCH candidates and they both are associated with the same DCI size, scrambling, and CORESET; Component 1: {Support, Not support} Component 2: {2, 3, [{2,3}]} Component 3: {monitored, not monitored}, Note: UE indicating ‘3’ for component 2 should report ‘monitored’ for component 3. Component 4: details FFS; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-2-1; Multi-TRP PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduling component carrier; Y; N/A; UE does not support mTRP PDCCH; Per band; N; N; Component1: {Support, Not support} Component2: {2, 3} Component3: {monitored, not monitored} Component4: FFS candidate values; Optional. |
| Qualcomm Incorporated [13] | In Rel-15, PDCCH monitoring capability is defined for different cases including Case 1-1 for basic capability (FG 3-1), Case 1-1 for single occasion monitoring (FG 3-2), and Case 2 for PDCCH monitoring with span gap (FG 3-5b). Furthermore, Rel-16 introduced additional capabilities (FG 11-2 family). The support of Rel-17 mTRP PDCCH repetition should be separately indicated for each of these PDCCH monitoring capabilities. In other words, the basic FG for Rel-17 mTRP PDCCH repetition should only include the basic PDCCH monitoring capability. This is because PDCCH repetition can significantly increase the UE complexity and some of the advanced PDCCH monitoring features that the UE is able to support in the absence of PDCCH repetition need a separate capability when combined with PDCCH repetition. Proposal: The basic FG for Rel-17 mTRP PDCCH repetition (FG 23-2-1) should only include the basic PDCCH monitoring capability. PDCCH repetition for more advanced PDCCH monitoring capabilities need a separate capability. Furthermore, for span-based PDCCH monitoring (either Rel-15 version or Rel-16 version), the UE processing complexity and memory requirements are different depending on whether PDCCH repetitions are within the same span or in different spans. Hence, UE capability for such indication is needed. Proposal: For span-based PDCCH monitoring (either Rel-15 version or Rel-16 version), UE can indicate if PDCCH repetitions can be intra-span or inter-span. 23. NR_FeMIMO; 23-2-1; Multi-TRP PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Maximum number of such overlaps in 3 per scheduled cell per slot; Per FS; Note: For PDCCH monitoring capability, the following is assumed: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot. Component2: {2, 3, [{2,3}]} Component3: {monitored, not monitored} Component4: {2,4,6,10,20,40}; Optional. 23. NR_FeMIMO; 23-2-1a; Multi-TRP PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot; Support of PDCCH repetition for PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot; FG 3-2 FG 23-2-1; Per Band; FR1 only; Optional. 23. NR_FeMIMO; 23-2-1b; Multi-TRP PDCCH repetition for Case 2 PDCCH monitoring with a span gap; 1. Support of PDCCH repetition for PDCCH monitoring of any occasions with span gap as defined in FG 3-5b. 2. Supported mode of PDCCH repetition; FG 3-5b FG 23-2-1; Per FS; This capability is necessary for each SCS. Component2: {intra-span, inter-span, both}; Optional. 23. NR_FeMIMO; 23-2-1c; Multi-TRP PDCCH repetition for Rel-16 PDCCH monitoring; 1. Support of PDCCH repetition with Rel-16 PDCCH monitoring capability as defined in FG 11-2 family. 2. Supported mode of PDCCH repetition 3. Maximum number of overlaps between individual candidates and linked candidates per scheduled cell per span; FG 11-2 FG 23-2-1; Per FS; This capability is signalled for SCS 15 kHz and 30 kHz. Component2: {intra-span, inter-span, both} Component3: {1,2,3,5,10,20}; Optional. |
| Nokia/Nokia Shanghai Bell [14] | No independent signaling for component 1, as it is not allowed to individually disable components of an FG. No need to separate the FG though, support to component 1 is given by support to the FG itself. |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for PDCCH repetition | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition |  |  |  |  |  |  |  | FR2 only | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | For 23-2-2, support of two QCL type D for PDCCH implies that UE is able to use two different receive beams for PDCCH reception regardless of PDCCH repetition schemes including SFN mannered PDCCH or PDCCH repetition based on two linked SSs (search space). Hence to have just one capability report for this two-QCL-TypeD reception is sufficient to satisfy UE’s implementation need. For the sake of simplicity and low signaling overhead, we propose to generalize this UE FG to make it applicable for both SFN mannered PDCCH and PDCCH repetition based on two linked SSs. The wording change is suggested as the following table. The prerequisite of this FG can be 23-2-1, 23-6-1 and 23-6-2 which is the support of PDCCH repetition based on two linked SSs, SFN scheme A and SFN scheme B (TRP based pre-compensation), respectively. 23-2-2; Two QCL TypeD for PDCCH; Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA; …; {Support, Not support}; Optional. Proposal: Generalize FG 23-2-2 to make it applicable for both SFNed PDCCH and PDCCH repetition based on two linked SSs. - The prerequisite includes 23-2-1, 23-6-1 and 23-6-2 |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] | For CORESET monitoring to support time-overlapping PDCCH repetitions in FR2, the following agreement was achieved in RAN1#104-e meeting. Agreement For a UE supporting reception with two different beams, support identifying two QCL-TypeD properties for multiple overlapping CORESETs - FFS: How to enhance existing QCL-TypeD priority rules for overlapping CORESETs - Note: The primary goal of this enhancement for the purpose of this sub-AI is to support time-overlapping PDCCH repetitions in FR2.. It can be seen that the precondition of this feature is that UE supports reception with two different beams. Therefore, according to Rel-16 feature groups defined in [2], the prerequisite feature group of 23-2-2 is 16-2c (Simultaneous reception with different Type-D). Besides, the name of feature group 23-2-2 is not accurate, which shall be only focused on CORESET monitoring. Thus, the following modification is proposed. Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-2-2; Two QCL TypeD for CORESET monitoring in PDCCH repetition; Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition; 16-2c; FR2 only; {Support, Not support}; Optional. |
| Samsung [8] | Regarding FG 23-2-2 in [2] which is related to identify two QCL-TypeD for PDCCH repetition, an extra condition is necessary to acknowledge the fact that no UE would be able to do simultaneous reception of ‘any’ two QCL-TypeD. Such pre-requisite capability is needed, e.g., Rel-16 16-2c Simultaneous reception with different Type-D, and the existing ‘groupBasedBeamReporting’ can be utilized for this purpose. Proposal: Support FG 16-2c (Simultaneous reception with different Type-D) as a pre-requisite UE capability for FG 23-2-2 per band level and FR2 only. |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] | 23-2-2; Two QCL TypeD for PDCCH repetition; Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition; {Support, Not support}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-2-2; Two QCL TypeD for PDCCH repetition; Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition; 23-2-1; Y; N/A; UE does not support two QCL-TypeD; Per band; N; N; FR2 only; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] |  |
| Nokia/Nokia Shanghai Bell [14] | Clarify if FR2-only support is supposed to include FR2-2 as well. |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | For 23-2-3 on support of PDCCH repetition for Type 3 CSS, we think this extra FG is unnecessary. The main FG 23-2-1 is sufficient from our view as there is no much difference between USS and CSS to process PDCCH repetition operation from UE side. No matter two PDCCH is from USS or CSS, the decoding of two PDCCH repetition is the same, i.e. separate decoding or soft combining. Proposal: Remove FG 23-2-3 |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] |  |
| Samsung [8] |  |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] | 23-2-3; FFS PDCCH repetition for Type3 CSS; FFS Support of PDCCH repetition for Type3 CSS; {Support, Not support}. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-2-3; PDCCH repetition for Type3 CSS; Support of PDCCH repetition for Type3 CSS; 23-2-1; Y; N/A; Per band; N; N; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] |  |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B, 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] | In the current list of features multi-TRP PUSCH repetition for type A and B are bundled together, however PUSCH type A and B are separate UE capabilities in Rel-16. Hence, we prefer to separate FGs for multi-TRP PUSCH repetition for type A and B. Another point is that sequential mapping must be supported while cyclic mapping optionally supported, hence sequential mapping should be included as a basic component as well. Similar situation also happens for support of second TPC field, support of PHR reporting Option 4, etc. Thus, we propose to use separate UE features for them. Agreement Confirm the following working assumption (with removing the last bullet): For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - FFS: Support of half-half mapping. - FFS: Additional considerations on mapping patterns (including required beam switching gaps) Agreement For PHR reporting related to M-TRP PUSCH repetition, support Option 4 as UE optional capability for a UE that supports mTRP PUSCH, - Option 4: Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs. Below is proposed revisions. 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition -typeA; 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A 2. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A 3. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A , 4. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 5. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 6. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.); {Support, Not support}; Optional. 23-3-1A; Multi-TRP PUSCH repetition -typeB; 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition B) 2. for single DCI based M-TRP PUSCH repetition Type B 3. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition B, 4. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 5. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 6. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.); {Support, Not support}; Optional. |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | In order to reduce the implementation complexity for a UE for the PHR calculation and report in MTRP across all CCs in a band, the max number of PHR report across CCs should be a UE capability. Proposal: The maximum number of supported PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) is a UE capability, FFS on candidate value, which is no more than 2*the number of supported CCs. 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition; 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B, 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 4.1 The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.); {Support, Not support} Component 4.1: FFS on Candidate values, Note: at most 2* the number of supported CCs; Optional. |
| CATT [7] | Preliminary RAN1 UE features for Rel-17 NR Multi-TRP PUSCH repetition was provided from WI rapporterurs as feature group 23-3-1. Our suggestion is to split the feature group to multiple UE features. Although both single DCI based M-TRP PUSCH repetition and CG M-TRP PUSCH repetition using a single CG configuration are supported in Rel-17, the CG M-TRP PUSCH repetition is an optional UE feature per agreement. It is our view that single DCI based M-TRP PUSCH repetition should be a mandatory feature for M-TRP PUSCH reeptiton as 23-3-1, and M-TRP CG PUSCH repetition should be an optional feature in a separate feature group with prerequisite feature group of 23-3-1. Since it was agreed that single DCI based M-TRP PUSCH repetition schemes are based on Rel-16 PUSCH repetition Type A and Type B, the prerequisite feature groups for M-TRP PUSCH repetition should be at least one of 11-5(pusch-RepetitionTypeB-r16) and 11-6( pusch-RepetitionTypeA-r16). For single DCI based M-TRP PUSCH repetition Type A and B, a UE is possible to be be configured with either cyclic mapping or sequential mapping, where the support of cyclic mapping is optional for the cases when the number of repetitions is larger than 2. It implies that sequential mapping for the cases when the number of repetitions is larger than 2 is a mandatory feature for M-TRP repetition schemes. Besides, the agreed beam mapping principle for the cases when the number of repetitions is equal to 2 also should be included in 23-3-1 as a mandatory feature for single DCI based M-TRP PUSCH repetition Type A and B. The following features are optional UE features, and therefore they should be separate UE feature groups with prerequisite feature group of 23-3-1: - Cyclic mapping for the cases when the number of repetitions is larger than 2; - Additaonl TPC filed for per TRP closed loop power control; - Multiple-PHR reporting. M-TRP CG PUSCH transmission shuld be based on the feature of Type 1 or Type 2 CG PUSCH repetition among multiple slots, therefore other prerequisite feature groups for M-TRP PUSCH transmission for CG PUSCH is at least one of UE feature groups for Type 1 or Type 2 CG PUSCH repetition among multiple slots. It was agreed that same beam mapping principles as dynamic grant M-TRP PUSCH repetition scheme are used for CG PUSCH transmission towards M-TRPs using a single CG configuration, therefore the components on sequential beam mapping and beam mapping principle for the cases when the number of repetitions is equal to 2 should be included in the basic feature group for M-TRP CG PUSCH transmission, and the optional feature on cyclic mapping should be applied for both DCI based M-TRP PUSCH transmission nd CG M-TRP PUSCH transmission. Similarly as TDM schemes for M-TRP PDSCH transmission in Rel-16, most M-TRP PUSCH repetition schemes in Rel-17 can be UE features per band. Since whether DCI format 0_2 is supported or not and the number of bits in single PHR MAC CE are per UE features in Rel-16, UE features for multi-TPC indication or multi-PHR reporting can be per UE features similarly. Different UE features for TDD/FDD or FR1/FR2 are not needed for M-TRP PUSCH repetition. Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-3-1; Multi-TRP PUSCH repetition; 1. Support of sequential mapping for single DCI based multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) when the number of repetitions is larger than 2 2. Support of the first and second transmission occasion be associated with two TRPs, respectively for single DCI based multi-TRP PUSCH repetition when the number of repetitions is equal to 2 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B, 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.); At least one of 11-5 and 11-6; Per band; N/A; N/A; Candidate values: {Support, Not support}.; Optional. 23-3-1a; CG based Multi-TRP PUSCH repetition; 1. Support of sequential mapping for CG PUSCH transmission towards M-TRPs using a single CG configuration when the number of repetitions is larger than 2 2. Support of the first and second transmission occasion be associated with two TRPs, respectively for CG PUSCH transmission towards M-TRPs using a single CG configuration when the number of repetitions is equals to 2; 23-3-1 and at least one of features of {5-14, 5-16, 10-38, 10-39}; Per band; N/A; N/A; Candidate values: {Support, Not support}; Optional. 23-3-1b; Multi-TRP PUSCH repetition with cyclical mapping; 1. Support of cyclic mapping when the number of repetitions is larger than 2; 23-3-1; Per band; N/A; N/A; Candidate values: {Support, Not support}; Optional. 23-3-1c; Multi-TPC indication for PUSCH; 1. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2; 23-3-1; Per UE; N/A; N/A; Candidate values: {Support, Not support}; Optional. 23-3-1d; Multi-PHR reporting; 1. Support of PHR reporting related to M-TRP PUSCH repetition; 23-3-1; Per UE; N/A; N/A; Candidate values: {Support, Not support}; Optional. |
| Samsung [8] | In RAN1#106-e, PHR per TRP is UE optional capability in Rel-17. Therefore, we can split component#4 in FG 23-3-1 as optional capability and add new UE capability signalling for that. And also prerequisite feature group of new UE capability signalling for enhanced PHR can be FG 23-3-1. Proposal: Split component#4 in FG 23-3-1 and support UE capability signaling of PHR reporting related to multi-TRP PUSCH repetition per UE For both mTRP PUCCH scheme(s) and mTRP PUSCH scheme, dynamic switching with new DCI field is supported. So, new components for dynamic switching are required respectively. So, dynamic switching should be defined in FG 23-3-1 For multi-TRP PUCCH/PUSCH repetition, support separate power control per TRP for PUCCH and PUSCH. So, the separate power control per TRP can be captured in new UE capability signalling. In addition, the enhancement on TPC can be moved into the new UE capability signalling for separate power control. Additionally, for multi-TRP PUSCH repetition, enhancement on OLPC with the second p0-PUSCH-SetList-r16 can be component of new UE capability signalling. So, define new UE capability signalling as FG 23-3-1b and FG 23-3-2b to support separate power control per TRP for PUSCH and PUSCH respectively. Components for enhancement on TPC and enhancement on OLPC can be parts of new UE capability signalling. Proposal: Define new UE capability signalings, FG 23-3-1b and FG 23-3-2b, to support separate power control per TRP for PUCCH and PUSCH respectively. Proposal: Move component#3 in FG 23-3-1 to FG 23-3-1b and move component#4 in FG 23-3-2 to FG 23-3-2b. Proposal: Define indicating per-TRP OLPC set in FG 23-3-1b. |
| MediaTek Inc. [9] | Since the components other than the first ones in FG 23-3-1 and FG 23-3-2 are agreed as UE optional features of multi-TRP PUCCH/PUSCH repetitions, these features should be listed as separate feature groups. Proposal: Move Components 2, 3, 4, 5 in FG 23-3-1 into distinct feature groups FG 23-3-1a, FG 23-3-1b, FG 23-3-1c, FG 23-3-1d. 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition; Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B, 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.); {Support, Not support}; Optional. 23. NR_FeMIMO; 23-3-1a; Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B; 23-3-1; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-3-1b; Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2; 23-3-1; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-3-1c; Support of PHR reporting related to M-TRP PUSCH repetition; Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.; 23-3-1; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-3-1d; Support of CG PUSCH transmission towards M-TRPs using a single CG configuration; Use same beam mapping principals as dynamic grant PUSCH repetition scheme.; 23-3-1; {Support, Not support}; Optional. |
| Intel Corporation [10] | 23-3-1; Multi-TRP PUSCH repetition; 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B, 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.); {Support, Not support}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | We propose to add 4 more components covering the AP-CSI/SP-CSI reporting, dynamic switching and PUSCH operation related UE capability 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition; 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B, 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) 6. Support of A-CSI report on two PUSCH repetitions 7. Support of SP-CSI report on two PUSCH repetitions 8: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 9: Support PUSCH operations; Y; N/A; UE does not support mTRP PUSCH; Per band; N; N; Component 1: bitmap {Type A, Type B} Component 2: {Support, Not support} Component 3: {Support, Not support} Component 4: {Support, Not support} Component 5: {Support, Not support} Component 6: bitmap {AP-CSI on Type A, AP-CSI on Type B} Component 7: bitmap {SP-CSI on Type A, SP-CSI on Type B} Component 8: {Support, Not support} Component 9: {codebook, codebook+non-codebook}; Optional. |
| Qualcomm Incorporated [13] | The following aspects need to be considered when designing UE capability for mTRP PUSCH repetitions: - Separate capabilities are required for codebook-based and non-codebook based mTRP PUSCH. - For codebook-based mTRP PUSCH, the structure of Rel-15 capability (FG 2-14) including indication on max number of SRS resources need to be taken in to account, and should be extended for the case that two SRS resource sets are configured. - For non-codebook based mTRP PUSCH, the structure of Rel-15 capability (FGs 2-15, 2-15a, 2-15b) including indication on max number of SRS resources and various capabilities for associated CSI-RS (for precoding calculation) need to be taken in to account, and should be extended for the case that two SRS resource sets / two associated CSI-RS resources are configured. - Separate capabilities for mTRP PUSCH repetition are required for repetition Type A and repetition Type B. - Capabilities are needed for transmitting AP/SP-CSI on two PUSCH repetitions. - Per TRP default power control pramaters or per TRP OLPC set indication features require UE capability. 23. NR_FeMIMO; 23-3-1a; Multi-TRP PUSCH repetition for codebook based; 1. Support of codebook-based multi-TRP PUSCH repetition for repetition Type A 2. Support of two SRS resource sets with usage set to codebook 3. Supported number of SRS resources in one SRS resource set 4. Supported number of SRS resources across two SRS resource sets; FG 2-14; Per FSPC; Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: {1,2,4} Component 4: {2,3,4,5,6,8}; Optional. 23. NR_FeMIMO; 23-3-1b; Multi-TRP PUSCH repetition for non-codebook based; 1. Support of non-codebook-based multi-TRP PUSCH repetition for repetition Type A 2. Support of two SRS resource sets with usage set to noncodebook 3. Supported number of SRS resources in one SRS resource set 4. Supported number of SRS resources across two SRS resource sets; FG 2-15; Per FSPC; Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: {1,2,3,4} Component 4: {2,3,4,5,6,7,8}; Optional. 23. NR_FeMIMO; 23-3-1b-1; Two associated CSI-RS resources; Support of up to two CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH; FG 2-15a FG 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1b-2; CSI-RS processing framework for SRS with two associated CSI-RS resources; 1. Maximum number of periodic SRS resources associated with first or second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first or second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first or second CSI-RS per BWP 4. UE can process Y SRS resources associated with first or second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS.; FG 2-15a FG 23-3-1b; Per Band; Component 1: {1 to 8} Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {1 to 16}; Optional. 23. NR_FeMIMO; 23-3-1c; Multi-TRP PUSCH repetition for repetition Type B; Support of multi-TRP PUSCH repetition for repetition Type B; FG 11-5 FG 23-3-1a or 23-3-1b; Per FS; Optional. 23. NR_FeMIMO; 23-3-1d; Two TPC fields in UL DCI; Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2; FG 8-8 FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1e; Cyclic beam mapping; Support of cyclic mapping when the number of repetitions is larger than 2; FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1f; Two PHRs per CC; Support of reporting two PHRs per CC (for a CC configured with mTRP PUSCH repetition); FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1g; Multi-TRP PUSCH repetition for configured grant; 1. Support of multi-TRP PUSCH repetition for configured grant 2. Supported type of configured grant for multi-TRP PUSCH repetition; FG 23-3-1a or 23-3-1b; Per Band; Component 2: {Type1, Type2, both}; Optional. 23. NR_FeMIMO; 23-3-1h; AP-CSI on two PUSCH repetitions; Support of transmitting AP-CSI on two PUSCH repetitions; FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1i; SP-CSI on two PUSCH repetitions; Support of transmitting SP-CSI on two PUSCH repetitions; FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1j; Per TRP OLPC set configurations; Support configurations of two (per TRP) p0-PUSCH-SetList; FG 11-8 FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1k; Per TRP default power control parameters; Support of two (per TRP) sets of default power control parameters when SRI fields are absent from the DCI; FG 23-3-1a or 23-3-1b; Per Band; Optional. |
| Nokia/Nokia Shanghai Bell [14] | Type A and B repetition are different features, and might be supported independently by UEs. Consider separate FGs, corresponding to each repetition type, and add corresponding pre-requisites. Pre-requisite on CG-PUSCH is missing as well. |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | For the third component in 23-3-2, in order to keep alignment with previous agreements, one restricted condition should be added. That is, cyclic mapping can be optional UE feature for the case when the number of repetitions is larger than 2. Hence we have the following proposal. Proposal: Clarify that cyclic mapping is only an optional UE feature for the case when the number of repetitions is larger than 2 in the third component of FG 23-3-2. For the fourth component in 23-3-2, it has been agreed in RAN1 #106-e meeting that a second TPC field can be configured to support per TRP closed-loop power control when DCI format 1_0 indicates PUCCH transmission is associated with two different closed-loop indexes. Accordingly, this component should include the case of PUCCH scheduled by DCI format 1_0. Hence we have the following proposal. Proposal: The case of PUCCH scheduled by DCI format 1_0 should be added in the fourth component in FG 23-3-2. 23-3-2; Multi-TRP PUCCH repetition; 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_0/1_1 / 1_2. |
| Vivo [4] | Similarly, in the feature group for multi-TRP PUCCH repetition, scheme1 and scheme 3 should be separate features and sequential mapping shall be added as a basic component. Similar situation also happens for support of second TPC field. Thus, we propose to use separate UE features for them. Agreement - To support per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2, a second TPC field can be configured via RRC. - When the second field is configured by RRC, a second TPC field (similar to the existing TPC field) is added in DCI formats 1_1 / 1_2 (option 3). - Each TPC field is for each closed-loop index value respectively - FFS: Whether or not the mapping between the TPC field and the PUCCH transmissions is needed - When the second field is not configured by RRC, a single TPC field (the existing TPC field) is used in DCI formats 1_1 / 1_2, and the TPC value applied for the closed loop index(es) for the scheduled PUCCH - To support per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2, adopt the same solution as with M-TRP PUCCH schemes. - FFS: any additional considerations - Support UE to report the capability on whether it supports the second TPC field - Note1: Per TRP closed-loop power control is only applicable when the “closedLoopIndex” values are not the same for TRPs. Below is the proposed revision. 23. NR_FeMIMO; 23-3-2A; Multi-TRP PUCCH repetition – scheme1; 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. support of sequential mapping for beam mapping/power control parameter set mapping for PUCCH repetition scheme 1 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2; {Support, Not support}; Optional. 23-3-2B; Multi-TRP PUCCH repetition – scheme3; 1. Support of PUCCH repetition scheme 3 (intra-slot repetition) 2. support of sequential mapping for beam mapping/power control parameter set mapping for PUCCH repetition scheme 3 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 3 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2; {Support, Not support}; Optional. |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] | Preliminary RAN1 UE features for Rel-17 NR Multi-TRP PUCCH repetition was provided from WI rapporterurs as feature group 23-3-2. Our suggestion is to split the feature group to multiple UE features. Repetition for PUCCH format 0 or 2 are not supported in Rel-16 and M-TRP PUCCH repetition are supported for all PUCCH format in Rel-17 , therefore we prefer the UE feature group of 23-3-2 to be a UE feature indenpendent of Rel-16 UE features for PUCCH repetition. A UE support 23-3-2 should support PUCCH format 1 or 3 or 4 over multiple slots with the repetition factor 2, 4 or 8 and PUCCH format 0 or 2 over multiple slots with the repetition factor 2. Although PUCCH repetition scheme 1 and PUCCH repetition scheme 3 are supported in Rel-17, PUCCH repetition scheme 3 is an optional UE feature. It is our view that PUCCH repetition scheme 1 should be a mandatory feature for M-TRP PUCCH reeptiton in 23-3-2, and M-TRP PUCCH repetition scheme 3 should be an optional feature in a separate feature group with prerequisite feature group of 23-3-2. A UE is possible to be be configured with either cyclic mapping or sequential mapping, where the support of cyclic mapping is optional for the cases when the number of repetitions is larger than 2. It implies that sequential mapping for the cases when the number of repetitions is larger than 2 is a mandatory feature for M-TRP repetition schemes. Besides sequential mapping for the cases when the number of repetitions is larger than 2, the agreed beam mapping principle for the cases when the number of repetitions is equal to 2 also should be included in 23-3-2. According to the meeting agreements, the following features are optional UE features, therefore they should be separate UE features with prerequisite feature group of 23-3-2: - Cyclic mapping for the cases when the number of repetitions is larger than 2; - Additaonl TPC filed for per TRP closed loop power control. Since same beam mapping methods as PUCCH repetition scheme 1 are used for PUCCH repetition scheme 3, the components on sequential beam mapping and beam mapping principle for the cases when the number of repetitions is equal to 2 also should be included in the feature group for PUCCH repetition scheme 3, and the optional feature on cyclic mapping should be applied for both PUCCH repetition scheme 1 and scheme 3. In RAN1 #106-e meeting, grouping of PUCCH resources in Rel-17 multi-TRP PUCCH repetition schemes was supported, with the following details: - Support MAC-CE activating two spatial relation info’s (for FR2) for a group of PUCCH resources in a CC. - Support MAC-CE activating two sets of power control parameters (for FR1) for a group of PUCCH resources in a CC. - When the PUCCH resource is indicated with two spatial relation info’s or two sets of power control parameters (via a MAC-CE that activating two spatial relation info’s or a MAC-CE that activating two sets of power control parameters for a group of PUCCH resources, respectively), the other PUCCH resources in the group also get updated to have the same two spatial relation info’s or two sets of power control parameters. - When the PUCCH resource is indicated with one spatial relation info or one set of power control parameters (via a MAC-CE that activating single spatial relation info or a MAC-CE that activating single set of power control parameters for a group of PUCCH resources, respectively), then the other PUCCH resources in the group also get updated to have the same spatial relation info or the same set of power control parameters. PUCCH grouping in Rel-16 is an optional feature, so it is natural to support grouping PUCCH resources with multi-TRP PUCCH repetition as an optional feature in Rel-17, with prerequisite feature group of Rel-16 (16-1b-3 , simul-SpatialRelationUpdatePUCCHResGroup-r16). Similarly as the analysis for M-TRP PUSCH repetition, most UE features for M-TRP PUCCH repetition in Rel-17 can be UE features per band except that the UE feature for multi-TPC indication is a per UE features. Different UE features for TDD/FDD or FR1/FR2 are not needed for M-TRP PUCCH repetition. Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-3-2; Multi-TRP PUCCH repetition; 1. Support of equential mapping for PUCCH repetition scheme 1 (inter-slot repetition) when the number of repetitions is larger than 2 2. Support of the first and second transmission occasion be associated with two TRPs, respectively for PUCCH repetition scheme 1 when the number of repetitions is equal to 2 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2; -; Per band; N/A; N/A; Candidate values: {Support, Not support}. If this feature is supported, the supported PUCCH formats and repetition factors are as follows: PUCCH format 1 or 3 or 4 over multiple slots with the repetition factor 2, 4 or 8 and PUCCH format 0 or 2 over multiple slots with the repetition factor 2; Optional. 23-3-2a; Multi-TRP PUCCH repetition based on miniSlot; 1. Support of sequential mapping for PUCCH repetition scheme 3 when the number of repetitions is larger than 2 2. Support of the first and second transmission occasion be associated with two TRPs, respectively for PUCCH repetition scheme 3 when the number of repetitions is equal to 2; 23-3-2; Per band; N/A; N/A; Candidate values: {Support, Not support}; Optional. 23-3-2b; Multi-TRP PUCCH repetition with cyclical mapping; 1. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3; 23-3-2; Per band; N/A; N/A; Candidate values: {Support, Not support}; Optional. 23-3-2c; Multi-TPC indication for PUCCH; 1. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2; 23-3-2; Per UE; N/A; N/A; Candidate values: {Support, Not support}; Optional. 23-3-2d; Spatial relation update for PUCCH group with Multi-TRP repetition; 1. Support of PUCCH resource groups per BWP for simultaneous spatial relation/power control parameter update for Multi-TRP PUCCH repetition.; 23-3-2, 16-1b-3; Per band; N/A; N/A; Candidate values: {Support, Not support}; Optional. |
| Samsung [8] | For both mTRP PUCCH scheme(s) and mTRP PUSCH scheme, dynamic switching with new DCI field is supported. So, new components for dynamic switching are required respectively. So, dynamic switching should be defined in FG 23-3-2 |
| MediaTek Inc. [9] | Since the components other than the first ones in FG 23-3-1 and FG 23-3-2 are agreed as UE optional features of multi-TRP PUCCH/PUSCH repetitions, these features should be listed as separate feature groups. Proposal: Move Components 2, 3, 4 in FG 23-3-2 into distinct feature groups FG 23-3-2a, FG 23-3-2b, FG 23-3-2c. 23. NR_FeMIMO; 23-3-2; Multi-TRP PUCCH repetition; Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-3-2a; Support of PUCCH repetition scheme 3 (intra-slot repetition); 23-3-2; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-3-2b; Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions; 23-3-2; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-3-2c; Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2; 23-3-2; {Support, Not support}; Optional. |
| Intel Corporation [10] | 23-3-2; Multi-TRP PUCCH repetition; 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2; {Support, Not support}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | We propose to add 2 more components covering the dynamic switching and PUCCH format related UE capability - We propose to introduce FG23-3-2a for the group based PUCCH spatial relation/power control parameter activation following the similar approach from Rel-16 23. NR_FeMIMO; 23-3-2; Multi-TRP PUCCH repetition; 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 5: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 6: Support PUCCH format for Multi-TRP PUCCH repetition; Y; N/A; UE does not support mTRP PUCCH; Per band; N; N; Component 1: {Support, Not support} Component 2: {Support, Not support} Component 3: {Support, Not support} Component 4: {Support, Not support} Component 5: {Support, Not support} Component 6: bitmap {PF0, PF1, PF2, PF3, PF4}; Optional. 23. NR_FeMIMO; 23-3-2a; PUCCH spatial relation info and power control parameters activation across a group of PUCCH in a CC; • Support MAC-CE activating two spatial relation info’s (for FR2) for a group of PUCCH resources in a CC. • Support MAC-CE activating two sets of power control parameters (for FR1) for a group of PUCCH resources in a CC.; Y; N/A; Per UE; N; Y; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | The following aspects need to be considered when designing UE capability for mTRP PUCCH repetitions: - Separate capabilities are needed for mTRP PUCCH Scheme 1 versus Scheme 3 - Supported PUCCH format(s) need to be indicated for each of the Scheme 1 and Scheme 3 - For the details of the capability for Scheme 3, we need to consider how the capability of sub-slot based PUCCH repetition with sTRP in Rel-17 IIoT AI is indicated. - Updating two beams / power control parameters for a group of PUCCH resources requires a UE capability 23. NR_FeMIMO; 23-3-2; Multi-TRP PUCCH repetition; 1. Support of multi-TRP PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of updating two Spatial Relation Info’s / two sets of power control parameters for a PUCCH resource by MAC-CE 3. Supported PUCCH formats for this scheme; Per FS; Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1.; Optional. 23. NR_FeMIMO; 23-3-2a; Multi-TRP PUCCH repetition with sub-slots; 1. Support of multi-TRP PUCCH repetition scheme 3 (sub-slot based repetition) 2. Supported PUCCH formats for this scheme; FG 23-3-2; Per FS; Note1: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Note2: The details of this FG should be decided after FG(s) related to sTRP sub-slot based PUCCH repetition in Rel-17 IIoT AI. Component 3: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1.; Optional. 23. NR_FeMIMO; 23-3-2b; Two TPC fields in DL DCI; Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2; FG 8-9 FG 23-3-2; Per Band; Optional. 23. NR_FeMIMO; 23-3-2c; Cyclic beam mapping; Support of cyclic mapping when the number of repetitions is larger than 2; FG 23-3-2; Per Band; Optional. 23. NR_FeMIMO; 23-3-2d; Updating two Spatial relation or two sets of power control parameters for PUCCH group; Support of updating two Spatial Relation Info’s / two sets of power control parameters for a group of PUCCH resources in a CC by MAC-CE; FG 16-1b-3 FG 23-3-2; Per Band; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of including other cell info in RRC TCI state config 2. Support of X>1 (max number of PCIs different from serving cell) |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | For the first component in 23-4, we think it deviates from the previous agreements. RAN1 discussed for many meetings to agree on a detailed description for the configuration of this feature due to its controversy. Hence we suggest to directly use the wording of the RAN1 agreement for the description of the first component to avoid unnecessarily repeating the RAN1 discussion. 23-4; IntCell-mTRP; 1. Support of RRC configuration of non-serving cell information which is associated with the TCI state and/or QCL-info 2. Support of X>1 (max number of PCIs different from serving cell). |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | Similar to the discussions on UE feature for <Enhancements on Multi-beam Operation>, we propose to add UE features on max number of RRC-configured PCIs different from serving cell PCI for TCI indication for inter-cell multi-TRP operation, with candidate value sets of {1, 2, 4, 6} and {1, 2, 4} for FR1 and FR2, respectively. Proposal: Support the following UE features on TCI indication for inter-cell multi-TRP operation - Supported max number of RRC-configured PCIs different from serving cell PCI for FR1 (X1) - Supported max number of RRC-configured PCIs different from serving cell PCI for FR2 (X2) 23. NR_FeMIMO; 23-4; IntCell-mTRP; 1. Support of including PCI(s) different from serving cell PCIother cell info in RRC TCI state config 2. Support of X>1 (max number of PCIs different from serving cell) 2. Supported max number of RRC-configured PCIs different from serving cell PCI for FR1 (X1) 3. Supported max number of RRC-configured PCIs different from serving cell PCI for FR2 (X2); {Support, Not support} Component 1: candidate values X1: [1,2,4,6] Component 2: candidate values X2: [1,2,4]; Optional. |
| CATT [7] |  |
| Samsung [8] | Regarding FG 23-4, for (1), it is not agreed in RAN1 to use RRC TCI state configuration or other (new) RRC signaling(s) to indicate other cell(s) information. Furthermore, we do not think that the RRC signaling should be included in the FG. For (2), we support X>1, though detailed signaling of X is still under discussions in RAN1 (i.e., reporting of a single X value or more than one X values). Proposal: Support X>1 (the maximum number of RRC configured PCIs different from the serving cell PCI) in FG 23-4 as a UE optional feature. |
| MediaTek Inc. [9] | We have the following agreement in RAN1#106-e meeting: Agreement Rel. 17 inter-cell MTRP , the maximum number of additional RRC -configured PCIs per CC is denoted X and can be reported as a UE capability - For the report value of X, multiple candidate values including 1 is supported. - FFS : Which values to support other than 1. - Values larger than 7 are precluded - RAN1 needs to agree on value(s) of X other than 1 - Down-select one of the following alternatives: - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB - The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States. First of all, component 2 should be a separate FG because UE doesn’t have to report this component if it only support 1 non-serving cell PCI. In addition, from the highlighted part, candidate values larger than 7 are precluded in the UE capability signalling. However, the detailed design should be finalized after down-selecting one of alternatives in the above agreement. Therefore, we have the following proposals. Proposal: Change original FG 23-4 to FG 23-4-1. Component 2 should be a separate FG (FG 23-4-2). The candidate values for FG 234-2 are {2, 3, 4, 5, 6, 7}. Inter-cell measurement and reporting (FG 23-1-2) is a prerequisite feature group of these feature groups. Proposal: Prerequisite feature group for FG 23-4-1 and FG 24-3-2 should be FG 23-1-2. 23. NR_FeMIMO; 23-4-1; IntCell-mTRP; Support of including other cell info in RRC TCI state config; 23-1-2; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-4-2; Non-serving cell PCIs; . Support of X>1 (max number of PCIs different from serving cell); 23-1-2; Component {2,3,4,5,6,7}; Optional. |
| Intel Corporation [10] | Reagrding UE capabilities for inter-cell mTRP. We propose to merge component 1 and 2, define three candidate values of X supporting maximum of 7 PCIs different from serving cell in FG 23-4. 23-4; Inter cell-mTRP; 1. Support of including other cell info in RRC TCI state config with X>1 (X is max number of PCIs different from serving cell) 2. Support of X>1 (max number of PCIs different from serving cell); Component 1: {Support, Not support} X = {1,3,7}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | We prefer UE to report the maximum number of X (number of PCIs different from the serving cell) that UE can support 23. NR_FeMIMO; 23-4; IntCell-mTRP; 1. Support of including other cell info in RRC TCI state config 2. Maximum number of X (max number of PCIs different from serving cell); Y; N/A; UE does not support inter-cell mTRP; Per band; N; N; Component 1: {Support, Not support} Component 2: {1, X}; Optional. |
| Qualcomm Incorporated [13] | The following aspects need to be considered when designing UE capability for Inter-cell mTRP - Aligned and unaligned SSBs need to have separate capabilities due to different UE complexity - The max number of additional RRC-configured PCIs should be separately indicated for each of the two scenarios above 23. NR_FeMIMO; 23-4; IntCell-mTRP; Support of additional RRC-configured PCI(s) that a TCI state/QCL information is associated with; FG 16-2a; Per FS; Note1: Only 1 additional PCI can be associated with the active TCI States, Note2: One PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with one CORESETPoolIndex , another PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with another CORESETPoolIndex; Optional. 23. NR_FeMIMO; 23-4b; Aligned SSBs; 1. Support the case where SSBs of the serving PCI and SSBs of additional RRC-configured PCI(s) have the same time domain position and periodicity 2. The maximum number of additional RRC-configured PCI(s); FG 23-4; Per Band; Component 2: {1 … TBD}; Optional. 23. NR_FeMIMO; 23-4c; Unaligned SSBs; 1. Support the case where SSBs of the serving PCI and SSBs of additional RRC-configured PCI(s) do not have the same time domain position and periodicity 2. The maximum number of additional RRC-configured PCI(s); FG 23-4; Per Band; Component 2: {1 … TBD}; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-5-1 | Beam reporting enhancements | Support of the maximum number of beam groups |  |  |  |  |  |  |  |  | Candidate values: {1,2,3,4} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | In our views, the explicit sub-features of supporting Rel-17 group-based reporting and supporting Rel-17 mTRP BFR should be introduced for [23-5-1] and [23-5-2], respectively. 23-5-1; Beam reporting enhancements; 1. Support of enhanced group-based reporting for Rel-17 M-TRP --- Candidate value of {Support, Not support} 2. Support of the maximum number of beam groups. |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | Similar to legacy Rel-15/16 UE features on beam measurement/reporting/failure-recovery for single-TRP case (e.g., 2-24, 2-30, 2-31), it is necessary to define UE features to report the supported number of RSs for beam measurement/reporting/failure-recovery in multi-TRP scenario. We propose the following: Proposal: Support the following UE features on the number of RSs for beam measurement/reporting/failure-recovery in multi-TRP scenario - Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs - Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs 23. NR_FeMIMO; 23-5-1; Beam reporting enhancements; - Support of the maximum number of beam groups in one report instance - Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs - Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs; Per band; Component 1: Candidate values: {1,2,3,4} Component 2: Candidate values: {4, FFS} Component 3: Candidate values: {4, FFS}; Optional. |
| CATT [7] |  |
| Samsung [8] | Proposal: Support 23-5-1, i.e., the maximum number of reported beam groups, as a UE optional feature. |
| MediaTek Inc. [9] | Regarding FG 23-5-1, we prefer to add “measurement” since this feature not only enhance the beam reporting but also the beam measurement. Also, we suggest to add “for MTRP” to clarify the intension of the enhancements. For FG of “Beam measurement and reporting enhancements for MTRP”, at least the following components need to be included according to the following agreements: - Support a single L1-RSRP reporting instance consisting of N beams pairs/groups and M=2 beams per pair/group - Support beam measurement on two CMR resource sets - Support of the maximum number of beam groups {1, 2, 3, 4} in one reporting instance Agreement from RAN1#104 For beam measurement in support of M-TRP simultaneous transmission - Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously - Support M = 2 - Support extending the maximum value of N > 1, exact value FFS - N=1 and N=2 - FFS: Other values larger than 2 - FFS: Whether the UE could report beams are received with different RX beams - Further study the support of option 1 and option 3 - The above applies at least for L1-RSRP - FFS: L1-SINR Agreement from RAN1#105 - For beam measurement/reporting option 2, the maximum number of beam groups in a single CSI-report is a UE capability and may take value from Nmax = {1,2,3,4} in Rel.17. - FFS: If UCI payload reduction for Nmax>=2 is needed and if so, how - The number of beam groups (N) reported in a single CSI-report - Alt1: The value of N is configured by RRC signalling Conclusion from RAN1#106 There is no consensus to support M>2 beams per group for beam reporting option 2 in Rel.17. Agreement from RAN1#104bis On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”: - NOTE: the following has been agreed - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset. - FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets Agreement from RAN1#105 For CMR configuration for option 2, adopt - Alt-1: “set” Agreement from RAN1#106 For aperiodic report of beam reporting option 2, - When associated with aperiodic resource setting, extend the existing RRC parameter CSI-AssociatedReportConfigInfo to be configured with two CMR resource sets where each may be configured with their corresponding QCL information. - FFS: Detailed association scheme - When associated with periodic/semi-persist resource setting, the resource setting comprises two CMR resource sets.. Proposal: For FG-23-5-1 of “Beam measurement and reporting enhancements for MTRP”, at least the following components should be included in: - Support a single L1-RSRP reporting instance consisting of N beams pairs/groups and M=2 beams per pair/group - Support beam measurement on two CMR resource sets - Support of the maximum number of beam groups {1, 2, 3, 4} in one reporting instance 23. NR_FeMIMO; 23-5-1; Beam measurement and reporting enhancements for MTRP; 1. Support a single L1-RSRP reporting instance consisting of N beams groups and M=2 beams per pair/group 2. Support beam measurement on two CMR resource sets 3. Support of the maximum number of beam groups in one reporting instance; {Support, Not support} Component: {1,2,3,4}; Optional. |
| Intel Corporation [10] | 23-5-1; Beam reporting enhancements; Support of the maximum number of beam groups; Candidate values: {1,2,3,4}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-5-1; Group based L1-RSRP reporting enhancements; Support of the maximum number of beam groups; Y; N/A; UE does not support group based L1-RSRP enhancement; Per band; N; N; Candidate values: {1,2,3,4}; Optional. |
| Qualcomm Incorporated [13] | The support of enhanced group-based beam report can be indicated by the maximum number of supported beam groups. UE supporting FG 23-5-1 can indicate whether supporting the 2 reported beams in a report from different cells (This requires UE to support inter-cell mTRP in FG 23-4) 23. NR_FeMIMO; 23-5-1; Enhanced group-based beam report; 1. Maximum number of supported beam groups; Per band; Candidate value set: {0,1,2,3,4}. 23. NR_FeMIMO; 23-5-1a; Support 2 reported beams per group from different cells; 1. Support of 2 reported beams in a reported beam group from different cells; FG 23-4; Per band; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-5-2 | Maximum number of BFD-RS resources per set for MTRP BFR | Support of the maximum number of BFD-RS resources per set |  |  |  |  |  |  |  |  | Candidate values: {1, [2,…]} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | In our views, the explicit sub-features of supporting Rel-17 group-based reporting and supporting Rel-17 mTRP BFR should be introduced for [23-5-1] and [23-5-2], respectively. 23-5-2; Maximum number of BFD-RS resources per set for MTRP BFR enhancements; 1. Support of Rel-17 M-TRP BFR --- Candidate value of {Support, Not support} 2. Support of the maximum number of BFD-RS resources per set. |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | Similar to legacy Rel-15/16 UE features on beam measurement/reporting/failure-recovery for single-TRP case (e.g., 2-24, 2-30, 2-31), it is necessary to define UE features to report the supported number of RSs for beam measurement/reporting/failure-recovery in multi-TRP scenario. We propose the following: Proposal: Support the following UE features on the number of RSs for beam measurement/reporting/failure-recovery in multi-TRP scenario - Maximum number of CSI-RS resources of both BFR-RS sets across all CCs - Maximum number of different SSBs of both BFD-RS sets across all CCs - Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs. 23. NR_FeMIMO; 23-5-2; Maximum number of BFD-RS resources per set for MTRP BFR; 1.Support of the maximum number of BFD-RS resources per set 2. Maximum number of CSI-RS resources of both BFR-RS sets across all CCs 3. Maximal number of different SSBs of both BFD-RS sets across all CCs 4. Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs; Candidate values: {1, [2,…]}; Optional. |
| CATT [7] | For MTRP BFR, the following agreements related to the number of BFD-RS sets and the number of BFD RS resources were achieved in RAN1#104-e, RAN1 #104b-e and RAN1#106-e meeting, respectively. Agreement For M-TRP BFR - Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set - FFS: value of N (e.g. fixed in specification, or UE capability) - FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability) Agreement On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 Agreement The maximum number of BFD-RS resources per set is a UE capability, including a possible candidate value of 1 in Rel.17.. It was agreed that the maximum numbers of BFD-RS resources per set and per BWP are UE capabilities, but the latter (per BWP feature) is missing in UE features list. Besides, the name of the feature group can be revised to show the feature clearly. Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-5-2; MTRP BFRMaximum number of BFD-RS resources per set for; 1. Support of MTRP BFR (based on two configured or determined BFD-RS sets) 2.Support of the maximum number of BFD-RS resources per set 3. Support of the maximum number of RSs in two BFR-RS sets per DL BWP; Component1: {Support, Not support} Component2: Candidate values: {1, [2,…]} Component3: Candidate values: {[2,…]}; Optional. |
| Samsung [8] | Proposal: Support 23-5-2, i.e., the maximum number of BFD RS resources in a BFD RS set, as a UE optional feature. |
| MediaTek Inc. [9] | We think “maximum number of BFD-RS resources per set” should be a component of FG for MTRP BFR, instead of a standalone FG. Thus, we suggest to change FG 23-5-2 to “MTRP BFR”, and at least the following components need to be included according to the following agreements: - Support two BFD-RS sets - Support 1-to-1 association between each BFD-RS set and an NBI-RS set - Support up to two PUCCH-SR resources for MTRP BFRQ - Support MTRP BFRQ MAC-CE - Support the same gNB response as in Rel.16 SCell BFR - Support beam update for PDCCH after receiving gNB response on MTRP BFR MAC-CE - The maximum number of BFD-RS resources per set - The maximum number of BFD-RS resources in two BFR-RS sets per DL BWP Agreement from RAN1#104 For M-TRP BFR - Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set - FFS: value of N (e.g. fixed in specification, or UE capability) - FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability) Agreement from RAN1#104bis On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 Agreement from RAN1#106 The maximum number of BFD-RS resources per set is a UE capability, including a possible candidate value of 1 in Rel.17. Agreement from RAN1#104 For M-TRP BFR Support 1-to-1 association between each BFD-RS set and an NBI-RS set - FFS: Association details Agreement from RAN1#104 For BFRQ of M-TRP BFR - Option 3: Up to two dedicated PUCCH-SR resources in a cell group - FFS: Whether PUCCH-SR for SCell can be reused for M-TRP - Support BFRQ MAC-CE that can convey information of failed CC indices, one new candidate beam for the failed TRP/CC (if found), and whether new candidate beam is found - Support at least indication of a single TRP failure - FFS: whether/what information of failed TRP(s) is conveyed in the MAC-CE - FFS: whether/how to support indication of more than one TRP failure, corresponding BFR procedure, and applicable cell type (SCell vs. SpCell) - FFS: UE behavior when TRP failure status is different across cells - FFS: Whether PUCCH SR resource can be configured with 2 spatial relations Agreement from RAN1#104 BFRQ response - Support at least the same gNB response as in Rel.16 SCell BFR (i.e. DCI with toggled NDI scheduling a same HARQ process ID as the PUSCH carrying BFRQ MAC-CE) Agreement from RAN1#106 - For the case of all CORESETs with 1 activated TCI state per CORESET, after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with the failed BFD-RS set reported in the MAC-CE for TRP-specific BFR is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD-RS set - FFS: How to associate CORESET(s) with failed BFD-RS set - FFS: SCS configuration of 28 symbols - FFS: Update of QCL assumption for other DL channels/RSs, UL spatial filter/power control assumption for PUCCH, and other UL channels/RSs - FFS: the case of CORESETs with 2 activated TCI states per CORESET. - The above applies to SCell and SpCell - The above applies at least for the multi-DCI case. Proposal: For FG-23-5-2 of “MTRP BFR”, at least the following components should be included in: - Support two BFD-RS sets - Support 1-to-1 association between each BFD-RS set and an NBI-RS set - Support up to two PUCCH-SR resources for MTRP BFRQ - Support MTRP BFRQ MAC-CE - Support the same gNB response as in Rel.16 SCell BFR - Support beam update for PDCCH after receiving gNB response on MTRP BFR MAC-CE - The maximum number of BFD-RS resources per set - The maximum number of BFD-RS resources in two BFR-RS sets per DL BWP 23. NR_FeMIMO; 23-5-2; MTRP BFR; - Support two BFD-RS sets - Support 1-to-1 association between each BFD-RS set and an NBI-RS set - Support up to two PUCCH-SR resources for MTRP BFRQ - Support MTRP BFRQ MAC-CE - Support the same gNB response as in Rel.16 SCell BFR - Support beam update for PDCCH after receiving gNB response on MTRP BFR MAC-CE - The maximum number of BFD-RS resources per set - The maximum number of BFD-RS resources in two BFR-RS sets per DL BWP; {Support, Not support} Component 7: {1, [2,…]} Component 8: {2, …}; Optional. |
| Intel Corporation [10] | 23-5-2; Maximum number of BFD-RS resources per set for MTRP BFR; Support of the maximum number of BFD-RS resources per set; Candidate values: {1, [2,…]}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | More components are needed to cover the resource related UE capability for BFR including both BFD and CBD (NBI) - We propose to introduce FG23-5-2a for the maximum number of PUCCH-SR resources for BFR - We propose to introduce FG23-5-2b for the support of simultaneous configuration of cell-specific BFR and TRP-specific BFR 23. NR_FeMIMO; 23-5-2; TRP-specific BFR; 1. Maximum number of BFD-RS resources per set 2. Maximum number of BFD-RS resources across sets across all CCs 3. Maximum number of CBD-RS resources per set 4. Maximum number of CBD-RS resources across sets across all CCs 5. Maximum number of CCs configured with TRP-specific BFR 6. Supported mTRP mode for TRP-specific BFR; Y; N/A; UE does not support TRP-specific BFR; Per band; N; N; Component 1: {1, 2} Component 2: {1, 2, 3, 4} Component 3: {8, 16, 32, 64} Component 4: {8, 16, 32, 64} Component 5: {1, 2, 3, 4} Component 6: {sTRP, mTRP, both sTRP and mTRP}; Optional. 23. NR_FeMIMO; 23-5-2a; Maximum number of PUCCH-SR resources; 1. Maximum number of PUCCH-SR resources in a cell group; 23-5-2; Y; N/A; UE does not support PUCCH-SR; Per UE; N; N; Component 1: {1, 2}; Optional. 23. NR_FeMIMO; 23-5-2b; Simultaneous configuration of cell-specific BFR and TRP-specific BFR; Support of simultaneous configuration of cell-specific BFR and TRP-specific BFR; 23-5-2; Y; N/A; Per BC; N; N; Component 1: bitmap {in the same CC, in different CC}; Optional. |
| Qualcomm Incorporated [13] | The support of mTRP BFR can be indicated by the maximum number of supported BFD-RS resources per set. 23. NR_FeMIMO; 23-5-2; Maximum number of BFD-RS resources per set for MTRP BFR; 1. Maximum number of supported BFD-RS resources per set; Per band; Candidate value set: {0,1,2}. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. Support of scheme A for PDCCH 2. Support of scheme A for PDSCH 3. Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] | In our view first 2 components should be combined as one feature, it doesn’t make sense for a UE to support scheme A for PDCCH and not support for PDSCH, and vice versa. We propose following revision. 23. NR_FeMIMO; 23-6-1; SFN scheme A (scheme 1); 1. Support of scheme A for PDCCH and PDSCH 2. Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2; {Support, Not support}; Optional. |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | 23. NR_FeMIMO; 23-6-1; SFN scheme A (scheme 1); 1. Support of scheme A for PDCCH 2. Support of scheme A for PDSCH 3. Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-1a; SFN scheme A (scheme 1) for PDCCH; 1. Support of scheme A for PDCCH; {Support, Not support}; Optional. |
| CATT [7] | Based on current agreement, dynamic switching between single-TRP and SFN PDSCH is an UE optional feature for both SFN scheme A and B. Therefore, dynamic switching between single-TRP and SFN PDSCH scheme A/B can be listed separately as optional UE capability, and its prerequisite feature group is 23-6-1 SFN scheme A or 23-6-2 for scheme B. Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-6-1; SFN scheme A (scheme 1); 1. Support of scheme A for PDSCH 2. Support of scheme A for PDCCH 3. Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2; Per band; N/A; N/A; Candidate values: {Support, Not support}; Optional. 23-6-1a; Dynamic switching between single-TRP and SFN PDSCH scheme A; Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2; 23-6-1; Per band; N/A; N/A; Candidate values: {Support, Not support}; Optional. |
| Samsung [8] | Regarding FG 23-6-1 (for scheme 1), the TCI indication method which was agreed as Variant E meaning that both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) should be included as a component. Further, FG 23-6-1 should have same structure for support of PDCCH and PDSCH (separate components as in current FG 23-6-1 or combined component as in current FG 23-6-2). Proposal: Add a component supporting Variant E (Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)) in FG 23-6-1. Proposal: Align a structure of component for supporting of scheme 1 and TRP based pre-compensation for PDCCH and PDSCH (defining separate components as in current FG 23-6-1 or combined component as in current FG 23-6-2). |
| MediaTek Inc. [9] | UE feature for dynamic switching should be separated from FG 23-6-1 because some UEs can only support SFN scheme A without supporting dynamic switching. Proposal: Component 3 for dynamic switching should be a separate FG. 23. NR_FeMIMO; 23-6-1; SFN scheme A (scheme 1); 1. Support of scheme A for PDCCH 2. Support of scheme A for PDSCH; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-1a; Dynamic switching for SFN scheme A; Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2; {Support, Not support}; Optional. |
| Intel Corporation [10] | The feature groups for HST-SFN deployments are proposed as part of FG 23-6-x family. We note that Rel-17 HST would support BFR enhancements for SFN PDCCH. In particular, BLER calculation for hypothetical PCCCH transmission are based on two reference signals, comparing to single reference signal supported in Rel-16. It is therefore proposed to introduce new FG 23-6-5 for this functionality as part of Rel-17 UE capability. In RAN1#106e meeting, it was also agreed to support PDCCH monitoring with for overlapping CORESETs configured with different QCL-Type D. Although details of the FG are not finalized, the corresponding enhancement is already agreed as UE optional feature. Therefore, it is proposed to capture the corresponding functionality as part of new FG 23-6-6. 23-6-1; SFN scheme A (scheme 1); 1. Support of scheme A for PDCCH 2. Support of scheme A for PDSCH 3. Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2; {Support, Not support}; Per band. 23-6-5; BFR for SFN PDCCH; Support of BFR using CORESET configured with two TCI states; {Support, Not support}; Per band. 23-6-6; PDCCH monitoring of with different QCL-TypeD; Support of PDCCH monitoring of PDCCH candidates in overlapping monitoring occasions with different QCL-TypeD; {Support, Not support}; Per band. |
| Lenovo/Motorola Mobility [11] | For Feature group 23-6-1, our preference is to merge the first and second components: “Support of scheme A for PDCCH” and “Support of scheme B for PDSCH” into one component, similar to scheme B Feature Group 23-6-2. Proposal 1: For Feature group 23-6-1, merge the first and second components: “Support of scheme A for PDCCH” and “Support of scheme B for PDSCH” into one component |
| Apple [12] | 23. NR_FeMIMO; 23-6-1; SFN scheme A (scheme 1); 1. Support of scheme A for PDCCH 2. Support of scheme A for PDSCH 3. Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2; Y; N/A; Per band; N; N; Component 1: {Support, Not support} Component 2: {Support, Not support} Component 3: {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | In Error: Reference source not found, four FGs (FG 23-6-1 to FG 23-6-4) have been listed for HST-SFN enhancement. Regarding FG 23-6-1 of the support SFN scheme A (scheme 1), we believe it can be split into two sub features. The main features should describe UE capability for supporting SFN scheme 1 for both PDDCH and PDSCH or only PDSCH. This is based on the RAN1 agreement of the supported trannissmision schemes between PDCCH and PDSCH where either same SFN mode is used for both PDCCH and PDSCH in addition to single TRP PDCCH and SFN scheme 1 PDSCH. Agreement Support the following combination of the transmission schemes - Single-TRP PDCCH + Rel-17 Scheme 1 PDSCH - Single-TRP PDCCH + Rel-17 TRP-based pre-compensation PDSCH - FFS: Other combinations of the transmission scheme Note: The PDSCH corresponds to the PDSCH scheduled by DCI formats 1_1 and 1_2. Agreement - UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH.. Proposal: Split FG 23-6-1 into main FG for the support of scheme A for PDDCH/PDSCH and FG 23-6-1A for the support of dynamic switching between single TRP PDSCH and SFN PDSCH scheme A. Proposal: FG 23-6-1 should have two componeonets: - Support of Scheme A for both PDSCH and PDCCH - Support of Scheme A for PDSCH only, where PDCCH is single TRP. 23. NR_FeMIMO; 23-6-1; SFN scheme A (scheme 1); 1. Support of scheme A for both PDCCH and PDSCH. 2. Support of scheme A for PDSCH only, where PDCCH is single TRP.; Per band; For each component: {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-1A; SFNschem A PDSCH dynamic switching; Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2; FG 23-6-1; Per band; {Support, Not support}; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. Support of scheme B for PDCCH and PDSCH 2. Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | 23. NR_FeMIMO; 23-6-2; SFN scheme B (TRP based pre-compensation); 1. Support of scheme B for PDCCH and PDSCH 2. Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-2a; SFN scheme B (TRP based pre-compensation) for PDCCH; 1. Support of scheme B for PDCCH; {Support, Not support}; Optional. |
| CATT [7] | Based on current agreement, dynamic switching between single-TRP and SFN PDSCH is an UE optional feature for both SFN scheme A and B. Therefore, dynamic switching between single-TRP and SFN PDSCH scheme A/B can be listed separately as optional UE capability, and its prerequisite feature group is 23-6-1 SFN scheme A or 23-6-2 for scheme B. Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-6-2; SFN scheme B (TRP based pre-compensation); 1. Support of scheme B for PDCCH and PDSCH 2. Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2; Per band; N/A; N/A; Candidate values: {Support, Not support}; Optional. 23-6-2a; Dynamic switching between single-TRP and SFN PDSCH scheme B; Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2; 23-6-2; Per band; N/A; N/A; Candidate values: {Support, Not support}; Optional. |
| Samsung [8] | Regarding FG 23-6-2 (for TRP based pre-compensation), it also needs additional component identifying the TCI indication method at least for Variant A meaning that one of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA). Further, 23-6-2 should have same structure for support of PDCCH and PDSCH (separate components as in current FG 23-6-1 or combined component as in current FG 23-6-2). Proposal: Add at least a component supporting Variant A (one of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)) in FG 23-6-2. Proposal: Align a structure of component for supporting of scheme 1 and TRP based pre-compensation for PDCCH and PDSCH (defining separate components as in current FG 23-6-1 or combined component as in current FG 23-6-2). |
| MediaTek Inc. [9] | For the same reason, we have the similar proposal. Proposal: Component 2 for dynamic switching should be a separate FG. 23. NR_FeMIMO; 23-6-2; SFN scheme B (TRP based pre-compensation); 1. Support of scheme B for PDCCH and PDSCH; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-2a; Dynamic switching for SFN scheme B; Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2; {Support, Not support}; Optional. |
| Intel Corporation [10] | 23-6-2; SFN scheme B (TRP based pre-compensation); 1. Support of scheme B for PDCCH and PDSCH 2. Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2; {Support, Not support}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-6-2; SFN scheme B (TRP based pre-compensation); 1. Support of scheme B for PDSCH 2. Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2; Y; N/A; Per band; N; N; Component 1: {Support, Not support} Component 2: {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | A similar discussion is applicable to SFN scheme B, where based on RAN1 agreements SFN scheme B is applicable on to HST deployment. FG 23-6-2 can be split into main FG for the support of SFN scheme B for both PDCCH and PDSCH or PDSCH only where PDCCH is single TRP. Proposal: Split FG 23-6-2 into main FG for the support of scheme B for PDDCH/PDSCH and FG 23-6-2A for the support of dynamic switching between single TRP PDSCH and SFN PDSCH scheme B. Proposal: FG 23-6-2 should have two componeonets: 1. Support of Scheme B for both PDSCH and PDCCH 2. Support of Scheme B for PDSCH only, where PDCCH is single TRP. 23. NR_FeMIMO; 23-6-2; SFN scheme B (TRP based pre-compensation); 1. Support of scheme B for both PDCCH and PDSCH 2. Support of scheme B for PDSCH only, where PDCCH is single TRP.; Per band; For each component: {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-2A; SFNschem B PDSCH dynamic switching; Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2; FG 23-6-2; Per band; {Support, Not support}; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST) | Support of simultaneous activation of two TCI states in all BWPs across set of configured component carriers |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | For SFN based HST enhancements, three FGs (23-6-1 ~ 23-6-3) are listed in [1]. We generally agree with the first two. However, the third one, i.e., 23-6-3 on support of simultaneous activation of two TCI states for PDCCH across multiple CCs, is not needed from our view. That’s because whether UE supports such feature can completely be inferred by the existing Rel-16 UE FG 16-1b-1 combined with the Rel-17 FG 23-6-1 or 23-6-2. 16-1b-1; TCI state activation across multiple CCs; - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH. As we know, for a UE support FG 16-1b-1, some configured CCs in which TCI states can be simultaneously updated for CORESETs with the same ID should share the same RF chain or similar channel conditions for analog beamforming. This is regardless of the number of activated TCI states for each CORESET. So, as long as UE supports FG 16-1b-1, and also supports FG 23-6-1 or 23-6-2, it is natural for this UE to support simultaneous activation of two TCI states across set of configured component carriers. Proposal 11: Remove FG 23-6-3 |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] |  |
| Samsung [8] |  |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] | We note that simultaneous activation of the two TCI states across multiple CCs defined by FGs 23-6-3 and FG 23-6-4 defining UE capabilities for default beam in DL and UL can be applicable not only to HST, but also to URLLC scenarios. Therefore, it is proposed to update definition of the corresponding FGs by removing ‘HST’. 23-6-3; Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST); Support of simultaneous activation of two TCI states in all BWPs across set of configured component carriers; {Support, Not support}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-6-3; Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST); Support of simultaneous activation of two TCI states in all BWPs across set of configured component carriers; Y; N/A; Per band; N; Y; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | Regarding FG 23-6-3, it should be prerequist on the support of SFN PDCCH scheme A or scheme B and should be a separate UE feature than Rel-16 FG 16-1b-1. Proposal: The support of SFN PDDCH (FG 23-6-1 or FG 23-6-1) should be prerequist to support FG 23-6-3 of simmultenous activation of two TCI states for PDCCH across multiple CCs 23. NR_FeMIMO; 23-6-3; Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST); Support of simultaneous activation of two TCI states in all BWPs across set of configured component carriers; FG 23-6-1 (component 1 PDCCH), or FG 23-6-2 (component 1 PDCCH); Per UE; {Support, Not support}; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-6-4 | Default beam setup for HST | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] |  |
| Samsung [8] | Regarding FG 23-6-4, since all new default beam behaviors are included in a single FG so far, we would like to suggest to split this FG as three separate feature groups and each feature group can have their own purpose to support how many numbers of default beams and which direction of default beam. Proposal: Split FG 23-6-4 as three separate feature groups and all three feature groups are FR2 only: - Components 1 and 3 as a single feature group to require a new default beam behavior with a single default DL beam - Components 2 to require a new default beam behavior with two default DL beams - Components 4, 5, and 6 to require a new default beam behavior with a single default UL beam. |
| MediaTek Inc. [9] | UE can only support default DL beam not default UL beam. Also, UE can only support default UL beam not default DL beam. Thus, we would like to propose the following. Proposal: FG 23-6-4 should be separated to two FG 23-6-4 (Default DL beam setup for HST) and FG 23-6-4a (Default UL beam setup for HST). Components 1, 2, 3 are included in original FG 23-6-4 and components 4, 5, 6 are included in FG 23-6-4a. 23. NR_FeMIMO; 23-6-4; Default DL beam setup for HST; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-4a; Default UL beam setup for HST; 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured; {Support, Not support}; Optional. |
| Intel Corporation [10] | We note that simultaneous activation of the two TCI states across multiple CCs defined by FGs 23-6-3 and FG 23-6-4 defining UE capabilities for default beam in DL and UL can be applicable not only to HST, but also to URLLC scenarios. Therefore, it is proposed to update definition of the corresponding FGs by removing ‘HST’. 23-6-4; Default beam setup for HST; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured; {Support, Not support}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | We propose to split the default beam related UE capability into two FGs, one for DL and one for UL, following the same approach in Rel-16 23. NR_FeMIMO; 23-6-4-1; DL Default beam setup for HST; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI when PDSCH is scheduled with offset equal or larger than the threshold 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold; Y; N/A; Per UE; N; FR2 only; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-4-2; UL Default beam setup for HST; 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured; 2-53, 2-59; Y; N/A; Per UE; N; FR2 only; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | Regarding FG 23-6-4, it could be split into multiple FGs, the main one on the default beam for PDSCH, add FG FG 23-6-4A for UE capality of two QCL TypeD properories for multiple overlapping CORESETs when a CORESET is activated with two TCI as per RAN1-agreement below. Also, add component 23-6-4b for default beam for AP-CSIRS and compoenent 23-6-4c for default spatial relation and pathloss RS. Agreement When a CORESET is activated with two TCI states which overlaps with another CORESET, support extension of Rel-15 prioritization rule for PDCCH monitoring of PDCCH candidates in overlapping monitoring occasions with different QCL-TypeD - FFS: Prioritization rule considers CORESETs indicated with 1 and/or 2 TCI states - Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs - UE capability is introduced - FFS other details - FFS: Strive to have same / similar solution as discussed under AI 8.1.2.1. Proposal: Split FG 23-6-4 into main FG for the support of default beam for SFN PDSCH, and - FG 23-6-4A: Two QCL TypeD properties for overlapping CORESETs - FG 23-6-4B: Default beam for AP-CSI in SFN mode - FG 23-6-4C: Default spatial relation and pathloss RS Regarding componenet 2 of supporting PDSCH reception when tci filed not present, we believe that this component should be either removed as it has been agreed only for DCI format 1_0 which already doesn’t have tci-field or explcilty clarified in the description that DCI refers to DCI format 1_0. Proposal: Remove compoenent 2 of FG 23-6-4 or clarify the component description that DCI refer for DCI format 1_0. 23. NR_FeMIMO; 23-6-4; Default beam setup for HST; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI for DCI format 1_0.; Per band; FR2 only; For each component {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-4a; Two QCL TypeD properties for SFN PDCCH; Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs when a CORESET is activated with two TCI; FG 23-6-1 (PDCCH), or FG 23-6-2 (PDCCH); Per Band; FR2 only; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-4b; Default QCL assumption for A-CSI-RS in SFN mode; Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold; FG 23-6-1 (PDCCH), or FG 23-6-2 (PDCCH); Per Band; FR2 only; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-4c; Default spatial relation and pathloss RS; 1. Support of single-TRP PUCCH transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured; FG 23-6-1 (PDCCH), or FG 23-6-2 (PDCCH); Per Band; FR2 only; For each component: {Support, Not support}; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of Nmax=1 - FFS others |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | In FG 23-7-1, a default value of Ks,max should be added as the basic component of this feature. We suggest Ks,max =4 as a compromised value between 2 and 8. |
| Vivo [4] | Following was agreed in previous meetings Agreement With regarding to the maximal values of Nmax for N, Ks,max for Ks: - Support of Nmax=2 is a UE optional feature - Support of Ks,max=X is a UE optional feature - X can be up to 8 and other candidate values can be discussed as part of UE features - FFS: Default value of Nmax, Ks,max - FFS: Which combinations of N<=Nmax, Ks<=Ks,max are supported Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support following two options: - Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis - X = 0, 1, 2 - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups - Support of X=1,2 is UE optional for the UE supporting option 1 - FFS omission of CSI associated with NCJT measurement hypothesis - Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses - FFS how to report recommended measurement hypothesis associated with that CSI report Below is revision of the feature to reflect the agreements. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Support of Nmax=1 - Number of CSI to report with single-TRP measurement hypothese X=0 - Support of reporting one CSI associated with the best one among NCJT and single-TRP measurement hypotheses - FFS others; {Support, Not support}; Optional. Number of CSI to report with single-TRP measurement hypothese X; - Number of CSI reports associated with single-TRP hypotheses, Value range {1, 2}; {Support, Not support}; Optional. |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] | For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis. And UE can be configured with two CMR groups with Ks= K1 + K2 CMRs, K1 and K2 are the number of CMRs in two groups respectively. Since both the values of Ks, K1, K2, N are basic configuration in CSI framework for M-TRP, our suggestion is to combine all basic configurtions to one UE feature group Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Support of Nmax for Multi-TRP CSI - Support of Ks,max for Multi-TRP CSI - Support of max # of Tx ports per source in a resource set for Multi-TRP CSI; Per band; N/A; N/A; Candidate values: {1, 2} Candidate values: {FFS} Candidate values: {24 ports, 32 ports}; Optional. |
| Samsung [8] | Support 23-7-1, i.e., basic features of CSI enhancement for Multi-TRP. - Support Nmax=1 as a component of UE basic feature for Multi-TRP CSI - Include CSI report modes, i.e., Mode 1 (Option 1) with X=0 and Mode 2 (Option 2), as a component of UE basic feature for mTRP CSI - For FR1 include CMR sharing between NCJT and sTRP measurement hypotheses as a component of UE basic feature - Include supporting two CMR groups with Ks=K1+K2 CMRs as a component of UE basic feature. - Include reporting 2 PMIs, 2 RIs, and 2 LIs for NCJT measurement hypothesis as a component of UE basic feature |
| MediaTek Inc. [9] | As agreed in RAN1#104e, Option 1 with X = 1, 2 is UE optional. In addition, since Option 2 requires additional processing of single-TRP measurement hypotheses, we prefer Option 2 not considered as a basic feature for NCJT CSI. Thus, we have the following proposals: Proposal: Only Option 1 is considered as a basic feature of CSI enhancement for multi-TRP. In FG 23-7-1, include “Support of reporting one NCJT measurement hypothesis and maximum X single-TRP measurement hypotheses” in the component. Proposal: Add a new FG “Support of reporting one among NCJT and single-TRP measurement hypotheses”. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Support of Nmax=1 - Support of reporting one NCJT measurement hypothesis and maximum X single-TRP measurement hypotheses - Support of max # of Tx ports per resource in a resource set for Multi-TRP CSI - Support of max # resources in a resource set for Multi-TRP CSI - FFS others; {Support, Not support} Component 2: {0, 1, 2} Component 3: {4, 8, 12, 16, 24, 32} Component 4: {2, 3, 4, 5, 6, 7, 8}; Optional. 23. NR_FeMIMO; 23-7-4; Support of reporting one among NCJT and single-TRP measurement hypotheses; 23-7-1; {Support, Not support}; Optional. |
| Intel Corporation [10] | At the RAN1#104b-e meeting it was agreed to support two options/modes for Rel-17 NCJT measurements: • Mode 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis. X = 0, 1, 2; • Mode 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses. For both modes presented above it can be assumed that a UE shall calculate CSI for all the configured CSI measurement hypothesis. For mode 1 the UE encodes and reports CSI for multiple CSI measurement hypothesis while for mode 2 the UE selects one CSI measurement hypothesis for reporting. The difference in UE processing and implementation complexity is very small for different modes; for mode 1 with X = 0, 1, 2 the UE complexity is also similar. The maximum number of processed CSIs can be controlled by the number of CSI processing units (CPUs). Thus, we propose to support mode 1 with X = 0, 1, 2 and mode 2 as basic features for Rel-17 mTRP CSI enhancements. Regarding the reporting granularity for the mTRP CSI features, in our view at least differentiation between FR1 and FR2 is needed since there is significant difference for calculation time requirements (Z, Z’) in milliseconds for FR1 and FR2. In order to achieve good balance between UE capability signalling overhead and UE flexibility, per band reporting can be considered which also solves the issue of FR1/FR2 differentiation. 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Support of Nmax=1 - Support of mode 1 with X = 1, 2, 3 and mode 2 - FFS others; {Support, Not support}; Per band. |
| Lenovo/Motorola Mobility [11] | For Feature group 23-7-1 that discusses the basic UE features for CSI enhancements for multi-TRP transmission, one component that needs to be included is Ks,max, corresponding to the total number of CMRs configured across the two CMR groups for multi-TRP CSI framework. In our opinion, Ks,max should depend on the operating frequency range. For instance, since CMR sharing (Feature group 23-7-5) is allowed without restriction in FR1, Ks,max =2 should be supported within the basic UE feature group. On the other hand, since CMR sharing is an optional feature for FR2, Ks,max =4 seems to be a minimum value for UEs that do not support CMR sharing so it can be configured with all CSI reporting modes, in which 2 CMRs would correspond to NCJT from two TRPs, in addition to 2 CMRs corresponding to single-TRP transmission from the two TRPs. One resolution to this issue would be supporting Ks,max =2 for FR1, and Ks,max =4 for FR2 in the basic UE Feature group 23-7-1. Other details are FFS. Proposal: For the basic UE Feature group 23-7-1 for CSI enhancements under multi-TRP, support Ks,max =2 as a basic feature for FR1, and Ks,max =4 as a basic feature for FR2 |
| Apple [12] | We propose to add two components for the maximum total number of ports and the X related to the joint Single-TRP and Multi-TRP measurement reporting 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Support of Nmax=1 - Support of maximum total number of ports across two CMRs in a pair of CMRs for Multi-TRP CSI measurement - Support of X CSI associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis - FFS others; 2-35; Y; N/A; Per band; N; N; Component 2: {4,8,12,16,24,32,48,64} Component 3: bitmap {0, 1, 2}; Optional. |
| Qualcomm Incorporated [13] | The basic FG need to only include the minimum that is needed for NCJT CSI. o The FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework. o Due to complexity for UE to calculate two PMIs jointly for NCJT, a list of vectors for NCJT capabilities and sTRP capabilities are needed. For NCJT CSI reporting, UE should be able to indicate whether option 1, option 2, or both are supported. o For Option 1, UE should be able to indicate which values of X (X=0,1,2) are supported. o Default capability for Option 1 is X=0. 23. NR_FeMIMO; 23-7-1; CSI Enhancement for Multi-TRP; 1. Support of two CMR groups in a CSI-RS resource set for Multi-TRP CSI 2. Support of one pair of CMRs from the two CMR groups (Nmax=1) 3. Supported option(s) for CSI reporting 4. A list of vectors {max # of ports per resource for NCJT, max # of total NCJT hypotheses, max # of total ports of CMRs for NCJT, max # of ports per resource for sTRP, max # of resources for sTRP, max # of total ports for sTRP} 5. Supported codebook modes 6. A list of (Y1,Y2): UE can process Y1 NCJT CSI report(s) and Y2 sTRP CSI report(s) simultaneously in a CC 7. A list of (X1,X2): UE can process X1 NCJT CSI report(s) and X2 sTRP CSI report(s) simultaneously across all CCs; Per Band Per Band Combination; Note: This feature is applicable to ‘typeI-SinglePanel’ codebook Component 3: {Option 1, Option 2, both} If UE supports ‘Option 1’ or ‘both’, X=0 CSIs associated with single-TRP measurement hypotheses is assumed Compenent 4: The list can have maximum of 16 vectors - Max # of Tx ports in one resource for NCJT: {2, 4, 8, 12, 16, 24, 32} - Max # of total NCJT hypotheses: {1 to 32} - Max # total ports of CMRs for NCJT: {4 to 256} - Max # of Tx ports in one resource for sTRP: {4,8,12,16,24,32} - Max # resources for sTRP: {1 to 64} - Max # total ports for sTRP: {4 to 256} Note: Component 4 is also reported per band combination (in addition to per band). Compenent 5:{ mode 1, both mode 1 and mode 2} Compoentn 6: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 6} Compoentn 7: The list can have maximum of 16 pairs. - X1: {2 to 16} - X2: {1 to 28} Note: When no NCJT CSI is configured/active, sTRP CSI capabilities are based on Rel-15/16 capabilities.; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports per source in a resource set for Multi-TRP CSI | {24 ports, 32 ports} |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | 24 ports of max # of Tx ports per source in a resource set in FG 23-7-2 should be moved into FG 23-7-1 as a basic component. For FG 23-7-2, only ‘32 ports’ is needed to indicate gNB whether UE supports it or not. |
| Vivo [4] | Following was agreed in previous meeting, where the number of ports across two CMRs. Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level - CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs. - At least ‘typeI-SinglePanel’ codebook is supported - FFS: Other codebook types - Note that RAN1 shall strive to finalize NCJT CSI enhancement with single reporting setting firstly. - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI Below is revision of the feature to reflect the agreement. 23. NR_FeMIMO; 23-7-2; Support of max # of Tx ports across two CMRs in a resource set for Multi-TRP CSI; [{24 ports, 32 ports}] Note: same number of ports among CMRs; {Support, Not support}; Optional. |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | In RAN1 #103, to relax the implementation of UE, the support of larger than 32 ports across two CMRs is optional for CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, as following: Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level resource association configuration and the corresponding CRI codepoint design - CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs. - At least ‘typeI-SinglePanel’ codebook is supported - FFS: Other codebook types - Note that RAN1 shall strive to finalize NCJT CSI enhancement with single reporting setting firstly. - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI Accordingly, FG23-7-2, i.e., support of max # of Tx ports per resource in a resource set for multi-TRP CSI is included in the preliminary UE features in [1]. UE can indicate specific Tx port number of CMR within the same CMR pair configured for NCJT larger than 16, e.g., 24 or 32, to be supported. It means that multi-TRP CSI measurement with 32 ports across two CMRs is the basic capability for Rel-17 multi-TRP CSI. However for multi-TRP CSI measurement, the CSI (PMI/CQI/RI) should be jointly calculated across two CMRs for the best NCJT performance and requires additional UE memory and complexity. The basic requirement of 32 CSI-RS ports across two CMRs has reduced the flexibility of UE implementation supporting advanced features. Hence current FG23-7-2 shall support more candidate values with smaller # of CSI-RS ports per resource in a resource set as following: Proposal: Support more candidate values with {2, 4, 8, 12, 16, 24, 32} ports in FG 23-7-2 for better flexibility of UE implementation. 23. NR_FeMIMO; 23-7-2; Support of max # of Tx ports per source in a resource set for Multi-TRP CSI Support of max # Tx ports per source and max # resources; - A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}; 23-7-1; {Support, Not support} Maximum size of the list is 4. The candidate values for max # of Tx ports per source in a resource set for Multi-TRP CSI is {2, 4, 8, 12, 16, 24, 32} The candidate values for max # resources in a resource set for Multi-TRP CSI is {2, 3, 4, 5, 6, 7, 8}; Optional. |
| CATT [7] |  |
| Samsung [8] | Support 23-7-2, i.e., the maximum number of Tx ports per resource in a resource set for Multi-TRP CSI, as UE optional feature. - Support {24, 32} as UE optional feature |
| MediaTek Inc. [9] | In addition to 24, 32, we prefer to add additional number of ports to support more versatile UE implementations. Proposal: For FG 23-7-2 Support of max # of Tx ports per resource in a resource set for multi-TRP CSI, add candidate values 4, 8, 12, 16. Finally, FG 23-7-2 can be merged into FG 23-7-1 as UE can report its capability by reporting the maximum supportable values. Proposal: FG 23-7-2 is merged into FG 23-7-1. 23. NR_FeMIMO; 23-7-2; Support of max # of Tx ports per resource in a resource set for Multi-TRP CSI; {24 ports, 32 ports}; {Support, Not support}; Optional. |
| Intel Corporation [10] | For FG 23-7-2, in our view lower number of CSI-RS ports can be considered since NCJT is beneficial mainly for indoor deployments and small cell deployments with smaller number of Tx antennas at the gNB (e.g. 2 or 4). Regarding the reporting granularity for the mTRP CSI features, in our view at least differentiation between FR1 and FR2 is needed since there is significant difference for calculation time requirements (Z, Z’) in milliseconds for FR1 and FR2. In order to achieve good balance between UE capability signalling overhead and UE flexibility, per band reporting can be considered which also solves the issue of FR1/FR2 differentiation. 23-7-2; Support of max # of Tx ports per source in a resource set for Multi-TRP CSI; {4, 8, 16, 24 ports, 32 ports}; {4, 8, 16, 24, 32}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-7-2; Support of max # of Tx ports per source in a resource set for Multi-TRP CSI; Support of max # of Tx ports per source in a resource set for Multi-TRP CSI; 23-7-1; Y; N/A; Per band; N; N; Component 1: {2, 4,8,12,16,24,32}; Optional. |
| Qualcomm Incorporated [13] | 23. NR_FeMIMO; 23-7-2; Supported value of X for Option 1; Supported values of X (reported CSIs associated with single-TRP measurement hypotheses) for Option 1; FG 23-7-1; Per Band; Note: UE supporting this feature indicates ‘Option 1’ or ‘both’ in component 4 of FG 23-7-1. Candidate values: {1, 2, 1and2}; Optional. |
| Nokia/Nokia Shanghai Bell [14] | Description is missing |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-7-3 | Support of max # resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | In [1], max # of Tx ports per resource in a resource set and max # resources in a resource set for Multi-TRP CSI are reported in FG23-7-2 and FG23-7-3 separately. Actually, above two FGs are coupling from UE implementation perspective. The larger value of max # of Tx ports per resource, the smaller value of max # of resources in a resource set for Multi-TRP joint CSI measurement, and vice versa. To simplify signaling and implementation for both gNB/UE, we prefer to combine FG23-7-2 and FG23-7-3 as one new FG and the size of such a parameter list is up to 4, as an example. Proposal: Combine FG23-7-2 and FG23-7-3 as one FG with pairs of UE capability parameters to reduce fragmentation and simplify gNB/UE implementation. 23. NR_FeMIMO; 23-7-3; Support of max # resources in a resource set for Multi-TRP CSI; FFS exact candidate values, Ks,max is up to 8; {Support, Not support}; Optional. |
| CATT [7] |  |
| Samsung [8] | Support 23-7-3, i.e., the maximum number of resources in a resource set for Multi-TRP CSI (Ks,max), as UE optional feature. - Default Ks,max can be set as: - Ks,max=8 for FR2 when CMR sharing between sTRP and NCJT measurement hypotheses is disabled (not enabled). - Ks,max=4 for FR2 when CMR sharing is enabled and for FR1. |
| MediaTek Inc. [9] | The basic feature of NCJT CSI is to configure one NCJT measurement hypothesis for CSI measurement and reporting. Thus, it is natural that UE can report to support maximum two CMRs in a CSI-RS resource set. To support full flexibility, we prefer all integers from 2 to 8 can be reported by UE. Proposal: For FG 23-7-3 Support of max # resources in a resource set for multi-TRP CSI, include 2, 3, 4, 5, 6, 7, 8 as candidate values. Finally, FG 23-7-3 can be merged into FG 23-7-1 as UE can report its capability by reporting the maximum supportable values. Proposal: FG 23-7-3 are merged into FG 23-7-1. 23. NR_FeMIMO; 23-7-3; Support of max # resources in a resource set for Multi-TRP CSI; FFS exact candidate values, Ks,max is up to 8; {Support, Not support}; Optional. |
| Intel Corporation [10] | Regarding the reporting granularity for the mTRP CSI features, in our view at least differentiation between FR1 and FR2 is needed since there is significant difference for calculation time requirements (Z, Z’) in milliseconds for FR1 and FR2. In order to achieve good balance between UE capability signalling overhead and UE flexibility, per band reporting can be considered which also solves the issue of FR1/FR2 differentiation. 23-7-3; Support of max # resources in a resource set for Multi-TRP CSI; FFS exact candidate values, Ks,max is up to 8; {1,2,3,4,5,6,7,8}; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-7-3; Support of max # resources in a resource set for Single-TRP CSI for CSI Enhancement for Multi-TRP; Support of maximum number of CMR resources configured in a CSI-ReportConfig for Single-TRP CSI measurement for CSI Enhancement for Multi-TRP, i.e., Ks,max; 23-7-1; Y; N/A; Per band; N; N; Component 1: {2, 4 ,6, 8}; Optional. |
| Qualcomm Incorporated [13] | 23. NR_FeMIMO; 23-7-3; More than two resources in a resource set for Multi-TRP CSI; Supported maximum # of CSI-RS resources in a resource set for Multi-TRP CSI (value Ks,max ); FG 23-7-1; Per Band; Candidate values: {3…8}; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI |  |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | FG 23-7-4 on support of Nmax=2 can be moved into FG 23-7-1 as an optional component. Alternatively, its prerequisite should be FG 23-7-1. |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | 23. NR_FeMIMO; 23-7-3; Support of Nmax=2 for Multi-TRP CSI Support of Nmax=2; - Support of Nmax=2 for Multi-TRP CSI; 23-7-1; {Support, Not support}; Optional. |
| CATT [7] |  |
| Samsung [8] | Support 23-7-4, i.e., the maximum number of CMR pairs (Nmax), as UE optional feature - Support Nmax=2 as UE optional feature |
| MediaTek Inc. [9] | 23. NR_FeMIMO; 23-7-2; Support of Nmax=2 for Multi-TRP CSI; 23-7-1; {Support, Not support}; Optional. |
| Intel Corporation [10] | Regarding the reporting granularity for the mTRP CSI features, in our view at least differentiation between FR1 and FR2 is needed since there is significant difference for calculation time requirements (Z, Z’) in milliseconds for FR1 and FR2. In order to achieve good balance between UE capability signalling overhead and UE flexibility, per band reporting can be considered which also solves the issue of FR1/FR2 differentiation. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-7-4; Support of Nmax=2 for Multi-TRP CSI for CSI Enhancement for Multi-TRP; Support of maximum 2 pairs of CMR resources configured in a CSI-ReportConfig for Multi-TRP CSI measurement for CSI Enhancement for Multi-TRP; 23-7-1; Y; N/A; Per band; N; N; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | 23. NR_FeMIMO; 23-7-4; Support two pairs of CMRs; Support of two pairs of CMRs from the two CMR groups (Nmax=2); FG 23-7-1; Per Band; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-7-5 | Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses | FR2 only |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | 23. NR_FeMIMO; 23-7-4; Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses Reuse two CMRs of NCJT for single-TRP measurement; FR2 only - 1. Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses; 23-7-1; FR2 only; {Support, Not support}; Optional. |
| CATT [7] |  |
| Samsung [8] | Support 23-7-5, i.e., whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, as UE optional feature. |
| MediaTek Inc. [9] | 23. NR_FeMIMO; 23-7-3; Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses; FR2 only; 23-7-1; {Support, Not support}; Optional. |
| Intel Corporation [10] | Regarding the reporting granularity for the mTRP CSI features, in our view at least differentiation between FR1 and FR2 is needed since there is significant difference for calculation time requirements (Z, Z’) in milliseconds for FR1 and FR2. In order to achieve good balance between UE capability signalling overhead and UE flexibility, per band reporting can be considered which also solves the issue of FR1/FR2 differentiation. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-7-5; Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses; Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis; 23-7-1; Y; N/A; Per band; N; FR2 only; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | 23. NR_FeMIMO; 23-7-5; CMR sharing; Support a CMR from a pair of CMRs configured for a Multi-TRP CSI measurement hypothesis to be used for Single-TRP measurement hypotheses; FG 23-7-1; Per Band; Optional. |
| Nokia/Nokia Shanghai Bell [14] | Description is missing. Clarify if FR-2-2 is in scope |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] | Rel-17 flexible AP-SRS triggering is an enhancement for more triggering occasions for AP-SRS, it should be a UE feature for TDD only. Compared to lagecy AP-SRS triggering scheme, the only difference of Rel-17 flexible AP-SRS triggering scheme is that when determining the transmit slot of AP-SRS, a number of “available slot(s)” is to be counted. It would not increase much UE’s complexity. So the feature of Rel-17 flexible AP-SRS triggering (23-8-1) can be a feature per UE. To support triggering SRS in DCI 0_1/0_2 without data and without CSI requires would not increase UE’s complexity for a UE supports DCI forma 0_1/0_2, therefore triggering SRS only in DCI 0_1/0_2 can be a feature per UE. For this feature (23-8-2), differential for TDD and FDD is not needed. Proposal: For UE features for flexible AP-SRS triggering, feature group 23-8-1 is a per UE feature for TDD only; |
| Samsung [8] |  |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] |  |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-8-1; SRS triggering offset enhancement; Support of determining aperiodic SRS location based on available slot and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2; 2-52; Y; N/A; Per band; N; N; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | Add FG 2-52 (basic SRS) as prerequisite for FG 23-8-1 |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] | Rel-17 flexible AP-SRS triggering is an enhancement for more triggering occasions for AP-SRS, it should be a UE feature for TDD only. Compared to lagecy AP-SRS triggering scheme, the only difference of Rel-17 flexible AP-SRS triggering scheme is that when determining the transmit slot of AP-SRS, a number of “available slot(s)” is to be counted. It would not increase much UE’s complexity. So the feature of Rel-17 flexible AP-SRS triggering (23-8-1) can be a feature per UE. To support triggering SRS in DCI 0_1/0_2 without data and without CSI requires would not increase UE’s complexity for a UE supports DCI forma 0_1/0_2, therefore triggering SRS only in DCI 0_1/0_2 can be a feature per UE. For this feature (23-8-2), differential for TDD and FDD is not needed. Proposal: For UE features for fleaxible AP-SRS triggering, feature group 23-8-2 is a per UE feature for both TDD and FDD. |
| Samsung [8] |  |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] |  |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | The components are modified to match the current antenna switching related UE capability. We also need to further discuss the allowed combination of fall back configuration or allow UE to report it freely as proposed bitmap 23. NR_FeMIMO; 23-8-2; Triggering SRS only in DCI 0_1/0_2; Support of triggering SRS in DCI 0_1/0_2 without data and without CSI; 2-52; Y; N/A; Per band; N; N; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | Add FG 2-52 (basic SRS) as prerequisite for FG 23-8-2 |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | Support of SRS antenna switching xTyR with y>4 |  |  |  |  |  |  |  |  | {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | SRS antenna switching down gradation was agreed in Rel-16. For the >4Rx antenna switching cases, the down gradation can also be supported for the same reason. Following similar principle, the following UE capabilities can be considered: • {t1r1, t1r2, t2r2, t2r4, t4r4, t4r8} • {t1r1, t1r2, t2r2, t2r4, t4r4, t4r6} • {t1r1, t1r2, t2r2, t2r4, t2r6} • {t1r1, t1r2, t1r4, t2r2, t2r4, t2r6, t2r8} • {t1r1, t1r2, t1r4, t1r6, t1r8} • {t1r1, t1r2, t1r4, t1r6} |
| CATT [7] | Similar design as that for SRS antenna switching for no more than 4Rx can be used for SRS antenna switching for >4Rx, i.e. it is a per band combination feature group with three components, where component#1 indicates whether SRS antenna switching xTyR with y>4 is supported, component #2 reports whether the uplink TX switching impact to downlink receiving in a band, and component #3 reports whether the UL Tx is switched together with UL Tx in another band. The prerequisite feature group is 2-53(“supportedSRS-Resources”). Per agreements in previous meetings, the candidate values for component #1 can be {"Not supported", "1T6R", "1T8R", "2T6R", "2T8R", "4T6R", "4T8R"}. Different UE features for TDD/FDD or FR1/FR2 are not needed for the enhancement for SRS for antenna switching in Rel-17. Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-8-3; SRS Antenna switching for >4Rx; 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the uplink TX switching impact to downlink receiving in a band, 3. Report whether the UL Tx is switched together with UL Tx in another band; 2-53; Per BC; N/A; N/A; {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R}Component-1 is a list of TRx pairs, candidates are {"Not supported", "1T6R", "1T8R", "2T6R", "2T8R", "4T6R", "4T8R"} Component-2: Candidate value set: {yes, no} Component-3: Candidate value set: {yes, no}; Optional. |
| Samsung [8] |  |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] | It can be seen that the candidate value includes {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R}. However, in Rel-16 UE capability, the downgraded antenna switching configuration can also be reported by the UE. For example, the UE of 1T4R can also support 1T2R and 1T1R. Following the same logic, the UE should also be able to report the supported downgrade configuration in Rel-17. xTyR; Supported configuration. 1T8R; 1T8R, 1T6R, 1T4R, 1T2R, 1T1R. 1T6R; 1T6R, 1T4R, 1T2R, 1T1R. 2T8R; 2T8R, 2T6R, 2T4R, 2T2R, 1T8R, 1T6R, 1T4R, 1T2R, 1T1R. 2T6R; 2T6R, 2T4R, 2T2R, 1T6R, 1T4R, 1T2R, 1T1R. 4T8R; 4T8R, 4T6R, 4T4R, 2T8R, 2T6R, 2T4R, 2T2R, 1T8R, 1T6R, 1T4R, 1T2R, 1T1R. 4T6R; 4T6R, 4T4R, 2T6R, 2T4R, 2T2R, 1T6R, 1T4R, 1T2R, 1T1R. In Rel-16 UE feature, the feature group on antenna switching includes another two components: txSwitchImpactToRx and txSwitchWithAnotherBand, which indicates the band that could have impact on the antenna switching operation. In Rel-17, given that the antenna switching is extended to up to 8 Rx, different xTyR may have different impact on multi CC operation. Therefore, it should be further discussed whether additional value of txSwitchImpactToRx and txSwitchWithAnotherBand should be reported by the UE in addition to legacy values. The proposed modifications of the corresponding FGs are summarized below. 23-8-3; SRS Antenna switching for >4Rx; Support of SRS antenna switching xTyR with y>4; {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R} {1T8R-1T6R-1T4R-1T2R-1T1R, 1T6R-1T4R-1T2R-1T1R, 2T8R-2T6R-2T4R-2T2R-1T8R-1T6R-1T4R-1T2R-1T1R, 2T6R-2T4R-2T2R-1T6R-1T4R-1T2R-1T1R, 4T8R-4T6R-4T4R-2T8R-2T6R-2T4R-2T2R-1T8R-1T6R-1T4R-1T2R-1T1R, 4T6R-4T4R-2T6R-2T4R-2T2R-1T6R-1T4R-1T2R-1T1R} FFS Additional report of whether the uplink TX switching impact to downlink receiving in a band and additional report whether the UL Tx is switched together with UL Tx in another band for new xTyR combinations; Per band. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-8-3; SRS Antenna switching for >4Rx; - Support SRS Tx port switch, - Report whether the uplink TX switching impact to downlink receiving in a band, - Report whether the UL Tx is switched together with UL Tx in another band; 2-53; Y; N/A; Per BC; N; N; Component 1: a list of TRx pairs, candidates are bitmap {"Not supported", “1T1R”, “1T2R”,”1T4R”, “1T6R”, “1T8R”, “2T2R”, “2T4R”, “2T6R”, “2T8R”, “4T4R”, “4T6R”, “4T8R”} Component 2: Candidate value set: {yes, no} Component 3: Candidate value set: {yes, no}; Optional. |
| Qualcomm Incorporated [13] | Add FG 2-55 (SRS Tx switch) as prerequisite for FG 23-8-3 and further discuss extension of Rel-16 of combo SRS Tx-switch. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets for antenna switching |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | For UE feature 23-8-4, since the agreement support maximum one periodic SRS resource and maximum 2 SP SRS, as follows: Agreement For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS. - Note: the two SP-SRS resource sets are not activated at the same time - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Applies for all supported xTyR where y<=8 - For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources. This feature is UE optional: For UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS So, the UE feature in the reporting should be change to “maximum 3 SP SRS resource sets for antenna switching”, where the details is “Support of maximum 2 SP SRS resource sets and 1 periodic SRS resource set for antenna switching”. Proposal 4-2: Following the agreement, the UE capability 23-8-24 should updated as: “maximum 3 SP SRS resource sets for antenna switching”, and “Support of maximum 2 SP SRS resource sets and 1 periodic SRS resource set for antenna switching” 23. NR_FeMIMO; 23-8-4; Maximum 3 2 SP SRS sets for antenna switching; Support of maximum 2 SP SRS resource sets and 1 periodic SRS resource set for antenna switching; {Support, Not support}; Optional. |
| CATT [7] | Maximum 2 SP SRS sets for antenna switching was agreed as an optional feature for SRS antenna switching with no more than 4Rx or more than 4Rx, therefore it should be a feature with prerequisite feature group of 2-53 or the new introduced feature group 23-8-3. It can be a UE feature per band combination. Different UE features for TDD/FDD or FR1/FR2 are not needed for the enhancement for SRS for antenna switching in Rel-17. Index; Feature group; Components; Prerequisite feature groups; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 23-8-4; Maximum 2 SP SRS sets for antenna switching; Support of maximum 2 SP SRS resource sets for antenna switching; One of 2-55 and 23-8-3; Per BC; N/A; N/A; {Support, Not support}; Optional. |
| Samsung [8] | Regarding FG 23-8-4 supporting maximum 2 semi-persistent SRS resource sets for antenna switching, some notes are needed for necessary conditions included in the corresponding agreement. Proposal: Add the following notes in FG 23-8-4. - Note: the two SP-SRS resource sets are not activated at the same time - For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources. |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] |  |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-8-4; Maximum 2 SP SRS sets for antenna switching; Support of maximum 2 SP SRS resource sets for antenna switching; 2-53; Y; N/A; Per band; N; N; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | Add two Notes for clarifying the number of sets for periodic and semi-persistent SRS when the UE doesn’t support FG 23-8-4 Add FG 2-53 (SRS resources) as prerequisite for FG 23-8-4 |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] |  |
| Samsung [8] |  |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] |  |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-8-5; Increased repetition for SRS; Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS; 2-53; Y; N/A; Per band; N; N; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | Add FG10-11 (SRS transmission at any symbol) as prerequisite for FG 23-8-5 |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] |  |
| Samsung [8] | Regarding FG 23-8-6 supporting partial frequency sounding of SRS, since FG 23-8-8 defines Comb-8 SRS, the word “RB-level” would be added in the name of FG 23-8-6. Similar modification can be applied to FG 23-8-7 as well. Proposal: Modify the name of FG 23-8-6 as RB-level partial frequency sounding of SRS amd FG 23-8-7 as Start RB location hopping for RB-level partial frequency SRS (i.e., adding “RB-level”). |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] |  |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-8-6; Partial frequency sounding of SRS; Support of partial frequency sounding for SRS; 2-53; Y; N/A; Per band; N; N; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | Clarify FG 23-8-6 with regards to SRS frequency hopping configuration 23. NR_FeMIMO; 23-8-6; Partial frequency sounding of SRS; Support of partial frequency sounding for periodic and semi-persisent SRS, only when freq. hopping enabled.; Per Band; {Support, Not support}; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency sounding |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] |  |
| Samsung [8] |  |
| MediaTek Inc. [9] | FG 23-8-7 (Start RB location hopping for partial frequency SRS) requires FG 23-8-6 (Partial frequency sounding of SRS). 23. NR_FeMIMO; 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial frequency sounding; 23-8-6; {Support, Not support}; Optional. |
| Intel Corporation [10] |  |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial frequency sounding; 23-8-6; Y; N/A; Per band; N; N; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] |  |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for MIMO SRS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] |  |
| CATT [7] |  |
| Samsung [8] |  |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] |  |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-8-8; Comb-8 SRS; Support of comb-8 for SRS; 2-53; Y; N/A; Per band; N; N; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] |  |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=1 - FFS others |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | 23. NR_FeMIMO; 23-9-1; Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II); - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=1 - FFS others; {Support, Not support} Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} Max # total ports: {2 to 256}; Optional. |
| CATT [7] |  |
| Samsung [8] | Similar to Rel.16 FG 16-3b, we prefer to add the following components into FG 23-9-1: - support for rank 1-2, and - parameter combinations with Mv=1 - add the following components - support for rank 1-2 - parameter combinations with Mv=1 |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] | For Rel-16 and Rel-15 PMI codebooks UE capability for mixed codebook types (UE feature group 16-8) is supported which allows to indicate which PMI codebooks can be configured for a UE simultaneously. This allows to improve the efficiency of UE computational capabilities utilization since different codebooks correspond to different PMI search complexity. New Rel-17 FeType-II PMI codebook shall be added to the codebook combinations. The following codebook combinations can be considered. • (Codebook 2, Codebook 3) = {(FeType II, NULL), (Type II, FeType II), (Type II PS, FeType II), (eType II R=1, FeType II), (eType II R=2, FeType II), (eType II PS R=1, FeType II), (eType II PS R=2, FeType II)} Regarding reporting granularity for UE features corresponding to FeType II PMI codebook, it should follow the reporting granularities agreed for Rel. 15 and Rel. 16 Type II codebooks. Thus, FG 23-9-1 should be reported per band and per band combination. 23-9-1; Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II); - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=1 - FFS others; component 1: • Maximum 16 triplets • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: {1 to 64} • Max # total ports: {4 to 256}; per band and per BC. 23-9-4; Active CSI-RS resources and ports for mixed codebook types in any slot; 1. Report a list of codebook combinations as {codebook 1, codebook 2, codebook 3}, where one of the codebook is FeType-II 2. For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports}; Component 1: TBD Component 2: • Maximum 16 triplets for each codebook combination • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: {1 to 64} • Max # total ports: {4 to 256}; per band and per BC. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-9-1; Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II); - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=1 - Support rank 1,2 - Support Mv=1 - FFS others; 2-35; Y; N/A; Per band and Per BC; N; N; Component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional. |
| Qualcomm Incorporated [13] | For Rel-17 port-selection Type II codebook, it is preferred to reuse Rel-16 UE capability framework/mechanism as much as possible. For codebook concurrency, since Type I codebook is typically used for basic periodic CSI reporting, it is reasonable to introduce codebook currency of Type I and Rel-17 port-selection Type II codebook. Hence, we propose Supporting Rel-17 port-selection Type II codebook with reported CSI-RS capability in terms of {max # ports per resource, max # resource, max # total ports} - Per band capability and per band-combination capability 23. NR_FeMIMO; 23-9-1; Port-selection FeType II; 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 2. Parameter combinations related to M = 1 3. Support of rank 1,2; FG 2-35; Per band Per BC; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional. |
| Nokia/Nokia Shanghai Bell [14] | Clarify what is the intended signaling |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II |  |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] | The complexity and UE hardware cost to support Mv=2 can be different for different triplets {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}. In the basic feature 23-9-1, a list of such triplets are reported for Mv=1. Hence it is needed to report another list of triplets for Mv=2. Otherwise, UE has to consider the worst case when determining whether it can supports Mv=2. For example, if UE can support Mv=2 for small number of Tx ports, but it cannot support Mv=2 for large number of Tx ports, it has to report no support of Mv=2 if the report of Mv=2 does not include the list of triplets. This limits NW to realize the better performance from Mv=2 for small number of Tx ports. Hence we have the following proposal. 23. NR_FeMIMO; 23-9-2; Support of Mv=2 for FeType-II; {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=2. |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | 23. NR_FeMIMO; 23-9-2; Support of Mv=2 for FeType-II Mv=2; - Support of Mv=2 for FeType-II; 23-9-1; {Support, Not support}; Optional. |
| CATT [7] |  |
| Samsung [8] | Regarding FG 23-9-2: - similar to Rel. 16 FG 16-3a-2, we propose to add a component: parameter combinations with Mv=2 - as discussed [3], the support of Mv=2 for large number of CSI-RS ports should be based on additional UE capability signaling - also, Mv=2 should be supported only for rank 1-2 - should be supported only for parameter combinations with Mv=1 - add the following components - parameter combinations with Mv=2 - maximum number of CSI-RS ports - supported for rank 1-2 |
| MediaTek Inc. [9] | For FeType-II codebook enhancement with Mv=2 feature, a window with size N is configured to UE. Whether to support FD bases reporting for N>Mv should be a separated UE capability. Proposal: For FG 23-9-2 of “Support of Mv=2 for FeType-II”, add one component “Support of window size N>Mv”. 23. NR_FeMIMO; 23-9-2; Support of Mv=2 for FeType-II; - Support of window size N>Mv - FFS others; {Support, Not support}; Optional. |
| Intel Corporation [10] | For 23-9-2 per ban reporting granularity can be considered since the difference in complexity for FeType II PMI codebook with M = 1 and M = 2 is not significant. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | The triplet related UE capability is added following the similar approach in Rel-16 for R. We need to further discuss the Mv and R depending on the outcome of the discussion related to R 23. NR_FeMIMO; 23-9-2; Support of Mv=2 for FeType-II; - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=1 - Support rank 1,2 - Support Mv=2; 23-9-1; Y; N/A; Per band and Per BC; N; N; Component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional. |
| Qualcomm Incorporated [13] | Supporting M > 1 FD bases with additional UE capability signaling (previous agreement) o Report {support, not support} o Support per band capability 23. NR_FeMIMO; 23-9-2; More than 1 FD basis; Support parameter combinations related to M > 1; FG 23-9-1; Per band; {support, not support}; Optional. |
| Nokia/Nokia Shanghai Bell [14] | Description is missing. Pre-requisites are missing. |
| Ericsson [15] |  |

| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II |  |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] |  |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] |  |
| Huawei/HiSilicon [6] | 23. NR_FeMIMO; 23-9-3; Support of rank 3, 4 for FeType-II Rank 3 and 4; - Support of rank 3, 4 for FeType-II; 23-9-1; {Support, Not support}; Optional. |
| CATT [7] |  |
| Samsung [8] | add the following component: supported for parameter combinations with Mv=1 |
| MediaTek Inc. [9] |  |
| Intel Corporation [10] | For 23-9-3 per band reporting granularity can be supported similar to UE feature corresponding to support of rank 3-4 for eType II codebook. |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-9-3; Support of rank 3, 4 for FeType-II; Support of rank 3,4; 23-9-1; Y; N/A; Per band; N; N; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | Supporting rank 3 and rank 4 as additional UE capability signaling (previous agreement) o Report {support, not support} o Support per band capability 23. NR_FeMIMO; 23-9-3; Rank 3 and 4; Support rank 3 and 4; FG 23-9-1; Per band; {support, not support}; Optional. |
| Nokia/Nokia Shanghai Bell [14] | Description is missing. Pre-requisites are missing. |
| Ericsson [15] |  |

General Comments

| Company | Summary |
| --- | --- |
| FUTUREWEI [2] | In Column “Mandatory/Optional”, the term “Optional” is used when applicable. However, “Optional” is not the same as “Optional with capability signaling”. We suggest changing the term “Optional” in that column to “Optional with capability signaling” - For several high-level features, such as M-TRP PUSCH, M-TRP PUCCH, M-TRP PDCCH, and HST, it may need to be first clarified how they are related to each other, and when one feature is supported, whether another or all other features should also be supported. For example, when a UE / gNB supports M-TRP PUCCH, it may also support M-TRP PUSCH. It is understood that if it is allowed to support the features separately, then more flexibility can be achieved, such as a UE may choose to implement only M-TRP PUSCH but not other features. However, the flexibility may bring some complexity to UE design and gNB design, especially to gNB design, since the gNB may need to handle many different feature combinations. On the other hand, if these features are to be supported (or not supported) as one entire package, there is less flexibility in implementation but the gNB does not need to handle too many cases. This tradeoff should be discussed in 3GPP. For another example, the HST SFN features may not directly rely on the support of M-TRP PDSCH/PUSCH/PUCCH/PDCCH, but there are some ongoing discussions that may affect how the features are grouped, such as whether a UE can support M-TRP non-SFN PDCCH reception and SFN PDSCH reception. Further study may be required. - Clarification of how the Rel-17 M-TRP features are related to Rel-16 M-TRP feature is needed. If a UE supports a Rel-17 M-TRP feature such as M-TRP PUCCH, does it imply the UE also supports Rel-16 M-TRP PDSCH feature? Generally speaking, the Rel-17 M-TRP features are built upon the Rel-16 M-TRP feature, but whether the new enhancements have to rely on Rel-16 M-TRP feature seems to be a different issue and worth further discussion. - “Basic M-TRP features” may be considered Some of the M-TRP features may require much higher complexity and more advanced processing capability, which may not be suitable for some UE / gNB implementations. However, a few basic M-TRP features may be feasible for a larger selection of UEs / gNBs, and it would be appealing to separate these basic features from those more advanced ones such that more UEs / gNBs can benefit from the M-TRP support. This is similar to the discussion of basic feature groups for some WIs in [5]. An initial step towards this goal is to identify which rows from 23-2-1 to 23-6-4 may be considered as part of a basic feature group for M-TRP, if it is agreeable by 3GPP to support such a basic feature group. Proposal: On M-TRP Features 23-2-1 to 23-6-4, discuss and decided on: - Whether the features can be supported separately from each other or jointly as one feature; - Whether the Rel-17 features rely on the support of Rel-16 M-TRP feature; - Potential support for a basic feature group for M-TRP. |
| ZTE [3] |  |
| Vivo [4] |  |
| OPPO [5] | UE feature for multi-beam Operation Rel-17 introduce DCI-based TCI state indication. The DCI format 1_1 or 1_2 can indicate on TCI state and the UE applies the indicated TCI state starting from the first slot that is Y symbols after the ACK to the beam indication DCI. As in previous agreements, the time gap between the beam indication DCI and the first slot must satisfy the UE capability. And the value of Y is configured by the gNB according to UE capability. Therefore, for rel-17 DCI-based TCI state indication, the UE shall report minimum value for two different time gaps: - The UE can report the minimum time gap between the last symbol of the beam indication DCI and the first slot where the indicated TCI state is applied. - The UE can report the minimum gap between the ack and the first slot where the indicated TCI state is applied. Proposal: For rel-17 DCI-based unified TCI state indication, the UE reports the minimum time gap between the last symbol of beam indication DCI and the first slot where the indicated TCI state is applied: - The reported minimum time gap can be X ms. It was agreed that the UE capability of Y symbols is reported in unit of symbols. For the same time gap length, different SCS would have different number of symbols. In the UE capability, the UE can report the minimum number of symbols for a reference SCS, say 15KHz. And the minimum number of symbols for other SCS can be calculated accordingly based on the SCS scale µ. Proposal: For rel-17 DCI-based unified TCI state indication, the UE reports Y the minimum number of symbols between the last symbol of acknowledge to the DCI and the first slot: - The UE reports the candidate value of Y0 for a reference SCS 15KHz - The value of Y for any other SCS is calculated as Y0× It was agreed that UE can report whether the UE supports beam misalignment for PL-RS. If the UE does not support beam alignment, the UE shall expect that the PL-RS and RS for uplink spatial Tx filer configured in the same TCI state are QCLed w.r.t the TypeD. Proposal: The UE can report whether the UE supports beam misalignment or not for PL-RS in Rel-17 TCI state. - If the UE does not support the beam misalignment, the UE shall expect that in one TCI state, the PL-RS and RS for uplink spatial Tx filter are QCLed w.r.t QCL TypeD. - UE feature for enhancements on multi-TRP for PDCCH, PUCCH and PUSCH It was agreed that an individual candidate is monitored or not is determined by a UE capability when the individual candidate and one of the linked PDCCH candidates uses same set of CCEs, DCI size and scrambling. When the individual candidate is monitored and the BD count of such individual candidate is not counted for overbooking, UE may need to process a large number of overlapping candidates even though without exceeding BD limit. To avoid excessive processing of overlapping candidates, it was agreed to introduce max number of such overlaps for UE capability reporting. For 3 BDs, it is possible that two separate decoding and one soft combining for the linked PDCCH candidate are performed which means duplicate identifications are still needed for individual candidate sharing the same parameters as one of the linked PDCCH candidate. Hence, max number of such overlaps reported by UE shall be also applicable for 3 BDs. Proposal: For 2 and 3 BDs, support to report 0 as the max number of the overlapping between individual and one of linked PDCCH candidate. - UE feature for enhancements on multi-TRP inter-cell operation It was agreed that one additional PCI can be associated with the activated TCI states for downlink. The maximal number of non-serving cell PCIs can be configured by RRC within one CC other than 1 needs further discussion. Two candidate values, 3 and 7, was discussed in RAN1#106e meeting. Between the two values, 3 is preferred considering the typical deployment of multi-TRP is coordination between no more than 3 TRPs. Furthermore, with more additional PCIs, the number of TCI states that can be configured by RRC for each PCI will be reduced, which will also impact the configuration flexibility. If the maximal values of X is agreed to be 3, X={1,2,3} is proposed to be the candidate values for UE capability reporting. For UE capability reporting, two independent X values can be reported, for two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB, e.g. whether the SSB positions of neighboring cells are the same as that of serving cell, considering there may be additional complexity for rate-matching. If no value is reported, the default value for maximal value of X is 1. Proposal: For maximal value of X, X={1,2,3} can be reported via UE capability. If no value is reported, the default value for maximal value of X is 1. Proposal: Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB. - UE feature for enhancements on beam management for multi-TRP It was agreed that the maximum number of BFD-RS resources per set is UE capability. Each set contains the BFD-RS for one TRP. For multi-DCI based mTRP system, the maximal number of CORESETs is 5. Practially, the number of CORESETs per TRP can be 1, 2 or 3. Thus the candidate value for the maximum number of BFD-RS per set can be {1,2,3}. Proposal: For the maximum number of BFD-RS resources per set, the candidate value is {1,2,3} in UE capability. If no value is reported, the default value is 1. - UE feature for enhancements on HST-SFN deployment It was discussed in RAN1#106 e-meeting whether to support TRP based pre-compensation in FR2 without conclusion. On one hand, supporting this feature in FR2 requires UE to be equipped with two panels. Furthermore, RAN4 has not make a decision on FR2 HST deployment, which is not a typical application scenario. Hence, even if this feature is supported in FR2, separate UE capability for FR1 and FR2 is needed, e.g. the capability can be per band. Most UE doesn’t need to implement this feature for FR2 considering the unusual application. Furthermore, the feature to use a single MAC CE to activate two TCI states for CORESETs with the same CORESET ID in all the BWPs in a CC list should also be a optional UE feature, similar to the feature with single TCI state in Rel-16. Proposal: If TRP-based pre-compensation is supported in FR2, support separate UE capability for FR1 and FR2 for TRP based pre-compensation. Proposal: Introduce an optional UE capability to report the support of a single MAC CE to activate two TCI states of CORESETs with the same CORESET ID for all the BWPs in a CC list. - UE feature for enhancements on SRS flexibility, coverage and capacity In Rel-17 SRS discussion, some enhancements are introduced for various areas, e.g., the flexible SRS triggering, antenna switching, extended SRS repetition, RB-level partial frequency sounding, Comb-8 SRS. Based on the new features agreed so far, we suggest to consider the UE features listed in the following proposal: Proposal: For Rel-17 SRS enhancement, at least consider the UE features listed in the following table. Categories; UE features. Flexible SRS triggering; Support of flexible SRS triggering. Antenna switching; Support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS - Note: the two SP-SRS resource sets are not activated at the same time - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Applies for all xTyR supported by this UE. - Supported antennas switching, a list of TRx pairs, candidates are - t1r1-t1r2-t1r4-t1r6 (1T6R) - t1r1-t1r2-t1r4-t1r8 (1T8R) - t1r1-t1r2-t2r2-t2r4-t2r6 (2T6R) - t1r1-t1r2-t2r2-t2r4-t2r8 (2T8R) - t1r1-t2r2-t4r4-t4r6 (4T6R) - t1r1-t2r2-t4r4-t4r8 (4T8R) - Report whether the uplink TX switching impact to downlink receiving in a band, - Report whether the UL Tx is switched together with UL Tx in another band. SRS repetition; Supported extended repetitions - FFS whether to report each supported configuration (N_symbol, R). RPFS; Support of RB-level partial frequency Sounding - FFS whether to report the supported PF value(s). Support of the start RB location hopping. Comb-8 SRS; Support of Comb-8 SRS. - UE feature for CSI enhancements on MTRP and FR1 FDD reciprocity It was agreed that UE can report the maximal supported number of CMRs within a set as Ks,max. In FR1, the same CMRs can be reused for S-TRP and NC-JT, and Ks=2 can satisfy most use cases for M-TRP. In FR2, if UE supports CMR sharing between S-TRP and NC-JT or if UE only supports option 1 with X=0, it can also report Ks,max=2 as the minimal capability. Hence, for UE capability, the minimal supported value of Ks,max should be 2. Proposal: The minimal supported value of Ks,max for UE capability reporting is 2. |
| Huawei/HiSilicon [6] | When the UE supports multiple codebook types, this UE may report conservative values assuming that multiple CSI reports associated with different codebook types are triggered simultaneously, to avoid above issue, UE shall report capabilities of CSI-RS per codebook per codebook-combination additionally. Therefore, FG “Active CSI-RS resources and ports for mixed codebook types in any slot” was introduced in Rel-16. The capability signalling can be a list of supported combination for one codebook in one codebook combination. For example, consider one UE supports Rel-15 Type I single panel codebook and Rel-16 eType II regular codebook with R=1. Because more codebook type is introduced, we propose to define mixed codebook types in any slot when the UE supports multiple codebook types. Considering Rel-17 is focusing on port selection codebook enhancement, the following codebook-combination is suggested in Rel-17. • Codebook 1 = {Type I SP, Type I MP} • {Codebook 2, Codebook 3} = {{FeType II PS, NULL}, {Type II, FeType II PS },{eType II R=1, FeType II PS }} Based on the above analysis, we have the following proposal on codebook-combination for Rel-17. Proposal: Add a new UE capability FG for supporting mixed codebook types including Rel-17 PS codebook. 23. NR_FeMIMO; 23-9-x; Active CSI-RS resources and ports for mixed codebook types in any slot; - Report a list of codebook combinations as {codebook 1, codebook 2, codebook 3} - For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports}; 23-9-1, Rel-15 2-35, Rel-16 16-3a and 16-3b; Component-1 candidate values: Codebook 1 = {Type I SP, Type I MP} {codebook 2, codebook 3} = {{FeType II PS, NULL}, {Type II, FeType II PS },{eType II R=1, FeType II PS }}; Optional. The UE is not mandated to support simultaneous reception with different QCL-TypeD for Rel-16 NCJT operation. Therefore UE FG 16-2c simultaneousReceptionDiffTypeD-r16 is used to indicate whether UE supports simultaneous reception with different QCL-TypeD reference signal, as 38.306. However it is unclear whether simultaneousReceptionDiffTypeD-r16 can be applied CSI-IM resources, since one may argue that CSI-IM is not reference signal. Therefore if simultaneousReceptionDiffTypeD-r16 reports “not support” (i.e. such an IE is not reported by UE), NW may be able to configure two CMRs with different QCL-TypeD with a TDM manner, e.g., over different symbols in a slot. Then according to following agreement in RAN1 #104bis-e: Agreement The UE may assume that QCL-Type D of CMRs associated with a NCJT measurement hypothesis are applied to the corresponding CSI-IM resource. the UE still have to apply two different QCL-Type D of CMRs associated with a NCJT measurement hypothesis over the corresponding CSI-IM resource simultaneously. Therefore the UE needs to support two receive beams simultaneously which is contradictive to the spirit of FG 16-2c to report “not support”. So we have the following proposal: Proposal: Clarify whether FG 16-2c in Rel-16 can apply to both CS-RS and CSI-IM at least. If FG 16-2c cannot cover, e.g. CSI-IM configured for NCJT measurement hypothesis, a new/dedicated FG is needed to indicate whether UE supports simultaneous reception with different QCL-TypeD for CSI-IM resource associated with NCJT measurement hypothesis. |
| CATT [7] |  |
| Samsung [8] |  |
| MediaTek Inc. [9] | Add a new FG 23-9-4 of “Support of PMI subbands with R>1 for FeType-II” as a separated UE capability. 23. NR_FeMIMO; 23-9-4; Support of PMI subbands with R>1 for FeType-II; {Support, Not support}; Optional. |
| Intel Corporation [10] |  |
| Lenovo/Motorola Mobility [11] |  |
| Apple [12] | We propose to introduce FG23-6-5 for support of implicit/explicit configuration of RS(s) with two TCI states for beam failure detection 23. NR_FeMIMO; 23-6-5; Support implicit/explicit configuration of RS(s) with two TCI states for beam failure detection; Support RS(s) with two TCI states configured, either implicitly or explicitly, for beam failure detection enhancement for SFN scheme A for HST; 23-6-1; Y; N/A; Per band; N; N; {Support, Not support}; Optional. We propose to introduce FG23-6-6 for HST-SFN scheme 1 PDCCH scheduling Single-TRP PDSCH 23. NR_FeMIMO; 23-6-6; Support of SFN scheme A PDCCH scheduling Single-TRP PDSCH; Support of SFN scheme A PDCCH scheduling Single-TRP PDSCH; 23-6-1; Y; N/A; Per band; N; N; {Support, Not support}; Optional. We propose to introduce FG23-7-6 for the support of CSI-IMR 23. NR_FeMIMO; 23-7-6; Support of CSI-IM for CSI enhancement for Multi-TRP; Support CSI-IM for CSI enhancement for Multi-TRP; 23-7-1; Y; N/A; Per UE; N; Y; {Support, Not support}; Optional. We propose to introduce FG23-7-7 for CMR and IMR configuration restriction for UE complexity and power consumption 23. NR_FeMIMO; 23-7-7; Support of CMR and IMR processing in different CDRX active time; Support to process CMR and IMR for the same CSI report in different CDRX active time; 2-35, 23-7-1; Y; N/A; Per UE; N; Y; {Support, Not support}; Optional. |
| Qualcomm Incorporated [13] | Supporting R > 1 with additional UE capability signaling (previous agreement, but whether having R > 1 is TBD) o Supporting R > 1 with reported CSI-RS capailbity in terms of {max # ports per resource, max # resource, max # total ports} o Per band capability and per band combination capability Introduce codebook combination {Type I, Rel-17 port-selection Type II} and report CSI-RS capability in terms of {max # ports per resource, max # resource, max # total ports} 23. NR_FeMIMO; 23-9-4; Support PMIs in one subband with R > 1 (if agreed in MIMO session); 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R > 1; FG 23-9-1; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}. 23. NR_FeMIMO; 23-9-5; Active CSI-RS resources and ports for mixed codebook types in any slot for R17; 1. Report a list of codebook combinations as {codebook 1, codebook 2} 2. For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports}; 2-36/40 in Rel-15, and 23-9-1/4 in Rel-17; Per band Per BC; Component-1 candidate values: Codebook 1 = {Type I SP, Type I MP} Codebook 2 = R17 port-slection FeType II Note 3：if a UE reports one or more codebook combinations in 23-9-5, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 4: For coexisting of mixed codebooks in any slot, gNB need to honor 23-9-5 and per-codebook capability 2-36/40 and 23-9-1/23-9-4 Component-2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional. |
| Nokia/Nokia Shanghai Bell [14] |  |
| Ericsson [15] | UE feature name; Class; Prerequisite; Content; FDD TDD diff; FR1 FR2 diff. Joint TCI intra-cell and inter-cell; Basic functionality; - Support of joint TCI with its components (configuration mechanism, QCL rules, applicable source and target signals) - Support of MAC-CE+DCI-based beam indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment) - Support of association between joint TCI state and UL PC settings (PLRS and other, including handling of beam misalignment for PLRS) - RRC configuration of additional cell SSB information - Rate matching of PDSCH/PDCCH from non-serving cell (PCI) associated with joint TCI state and/or QCL-info around non-serving cell SSB with the same PCI - Additional PCI different from the serving cell PCI per CC, associated with one or more joint TCI states - Support of beam measurement on SSB(s) with PCI(s) different from serving cell PCI - Support of beam reporting up to Kmax=4 SSBRI-RSRP pairs, with at least one pair associated with a PCI different serving cell PCI - Minimum number of configured TCI states: 32 for FR1 and 128 for FR2; No; Yes. UL TCI; Joint TCI intra-cell; - Support of separate UL TCI with their components (configuration mechanism, applicable source and target signals) - Support of MAC-CE+DCI-based beam indication (including TCI state activation, use of DCI for-mats 1_1/1_2 with and without DL assignment) - Support of association between UL TCI state and UL PC settings (PLRS and other, including handling of beam misalignment for PLRS); No; No. P-MPR reporting; Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and associated P-MPR in PHR reporting); No; Yes. TCI state application time; Basic functionality; Minimum time-duration between beam indication ACK and application of new TCI state; No; No. Additional PCIs; Number of additional PCIs for TCI indication and beam measurements; No; No. Some comments: - Inter-cell beam measurements and reporting cannot be separated from inter-cell beam management – they must belong to the same group. - The minimum number of configured TCI states has been increased from the Rel-15 value, to better support inter-cell beam management. The current minimum values captured in Error: Reference source not found are 64 for FR2, whereas for FR1, the UE has to support as many TCI states as the maximum number of SSBs in the band. The FR1 value is quite restrictive. UE feature name; Class; Prerequisite; Content; FDD TDD diff; FR1 FR2 diff. PDCCH-repetition-r17; Basic functionality; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Maximum number of such overlaps in 3. PDCCH-repetition-TypeD; Enhanced; PDCCH-repetition-r17; Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition; FR2 only. PDCCH repetition-Type3 CSS; Enhanced; PDCCH-repetition-r17; Support of PDCCH repetition for Type3 CSS. PUSCH-repetition-r17; Basic func-tionality; 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B, 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.). PUCCH-repetition-r17; Basic func-tionality; 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2. UE feature name; Class; Prerequisite; Content; FDD TDD diff; FR1 FR2 diff. Inter-Cell-Multi-TRP-r17; Basic functionality; Support of multi-DCI based multi-TRP operation (multiDCI-MultiTRP-r16); - RRC configuration of one additional cell SSB information - Rate matching of PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info around non-serving cell SSB with the same PCI - 1 additional PCI different from the serving cell PCI per CC, associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH; No; No. InterCell-MTRP-multiplecell; Enhanced; Inter-Cell-Multi-TRP-r17; Support for up to X RRC configured additional cells, X>1 is TBD. UE feature name; Class; Prerequisite; Content; FDD TDD diff; FR1 FR2 diff. Beam reporting-r17; Basic functionality; - Support of the maximum number of beam groups; FR2 only. MTRP-BFR-r17; Basic functionality; Support of multi-DCI based multi-TRP operation (multiDCI-MultiTRP-r16) or Support of one of the following single-DCI based multi-TRP schemes -> singleDCI-SDM-scheme-r16, -> supportFDM-SchemeA-r16, -> supportFDM-SchemeB-r16, -> supportTDM-SchemeA-r16 -> supportInter-slotTDM-r16; - Support of the maximum number of BFD-RS resources per set; FR2 only. UE feature name; Class; Prerequisite; Content; FDD TDD diff; FR1 FR2 diff. SFN-PDSCH-r17; Basic functionality; - SFN Scheme A PDSCH - SFN Scheme B PDSCH (only for FR1) - Dynamic switch between SFN and S-TRP PDSCH for DCI formats 1_1, 1_2; No; No. SFN-PDCCH-r17; SFN-PDSCH-r17; - SFN Scheme A PDCCH - SFN Scheme B PDCCH (only for FR1); No; No. In a real HST network, legacy UEs would coexist with Rel-17 SFN capable UEs in the same network. The CORESET#0 configuration for Rel-17 UE need to be the same as Rel-15, with QCL assumption associated with one corresponding SSB configuration. Monitoring and detection of PDCCH in CORESET#0 is the basic function that all UE in the network must support. A Rel-17 SFN capable UE shall support the combination “Single TRP PDCCH + SFN PDSCH” as basic SFN UE capability, which is also a prerequisite capability to any other optional SFN UE capabilities. Proposal: “Single TRP PDCCH + SFN PDSCH” is the basic UE capability for supporting of SFN Schemes. The SFN scheme can be either UE-based or TRP-based. The TRP-based SFN scheme, i.e. TRP pre-compensation, requires gNB implementation rather than UE implementation. In our view, only the UE-based SFN scheme(sfnSchemeA) is a true indication for supporting of HST SFN.TRP based Scheme(sfnSchemeB) is more like a UE incapability than capability. The sfnSchemeB also has fundamental performance issue for low SNR case as shown in [3][4]. UE needs to support the sfnSchemeA as the basic UE capability to ensure the performance of HST deployment. A UE supports sfnSchemeB shall support first sfnSchemeA as prerequisite capability. Proposal: sfnSchemeA is the basic UE capability for supporting of SFN Schemes. Proposal: A UE supports sfnSchemeB shall support sfnSchemeA as prerequisite capability if a separate capability is defined for sfnSchemeB. UE feature name; Class; Prerequisite; Content; FDD TDD diff; FR1 FR2 diff. Ap-SRS-trigger-r_17; Basic; - Configuration of up to 2 bits in DCI 0_1 and DCI 0_2 to indicate slot offset “t” - Dynamic indication of slot offset “t” for SRS transmission (from reference slot) - For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request; No; No. 6RX-8RX-AntennaSwitching_r17; Basic; 1. Antenna switching SRS resource configurations for {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R} 2. Support of all the non-zero integer values N<=N_max except N=1 for 1T8R. Partial_SRS_r17; Basic; - Support for RB level PFS - khopping is fixed to be 0 for all SRS symbols. Partial_SRS_enh_r17; Enhanced; Partial_SRS_r17; - Support start RB location (Noffset) hopping in different SRS frequency hopping periods for RPFS and. SRS_comb8_r17; Basic; - Support for comb 8 SRS - Support for [TBD] cyclic shifts. SRS_repetition_r17; Basic; - Support for (N_symbol, R) = {(8, 1), (8, 2), (8, 4), (8, 8), (12, 1), (12, 2), (12, 3), (12, 4), (12, 6), (12, 12), (10, 1), (10, 2), (10, 5), (10,10), (14, 1), (14, 2), (14, 7), (14, 14)}. SP-SRS_r17; Basic; - For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS, where the two SP-SRS resource sets are not activated at the same time - Applies for all supported xTyR where y<=8. UE feature name; Class; Prerequisite; Content; FDD TDD diff; FR1 FR2 diff. Codebook_feTypeII_r17; Basic functionality; - Support rank 1 and 2 codebook; No. Codebook_feTypeII_enhanced_r17; Enhanced; Codebook_feTypeII_r17; - Support for Mv=2 - Support for R>1 (TBD). Codebook_feTypeII_enhanced_Rank34_r17; Enhanced; Codebook_feTypeII_r17; - Support for rank 3 and 4. UE feature name; Class; Prerequisite; Content; FDD TDD diff; FR1 FR2 diff. Multi-TRP CSI_r17; Basic functionality; Support of singleDCI-SDM-scheme-r16,; - Value of Nmax (value range 1 or 2) - Max # of Tx ports per resource in a resource set (value range to be FFS) - Max number of resources per resource set for multi-TRP CSI (value range to be FFS). Some comments: - On Multi-TRP CSI, we do not see the need to split this feature into 5 different features as proposed. That is, rows 23-7-1, 23-7-2, 23-7-3, and 23-7-4 can be different components in the basic UE feature. - Note that the UE can indicate the supported value for each of these components. There is no need to split them into different features. |

# Discussion/Approval Items during RAN1 #106bis-e

After review of contributions submitted to RAN1 #106bis-e in this agenda item, the following topics were identified by the moderator for discussion/approval during RAN1 #106bis-e. Companies submitted the following views on the moderator’s proposals.

# FG 23-1-1: Unified TCI for intra- and inter-cell beam management

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e.

There seems to be consensus that this FG is needed.

Companies added many new components to FG 23-1-1 of the preliminary NR UE feature list in [1] or proposed new feature groups in addition to FG 23-1-1 such that RAN1 needs to discuss how to spilt this FG into components and FGs, respectively.

| 23. NR_FeMIMO | 23-1-1 | Unified TCI for [intra- and inter-cell] beam management | [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [(PLRS and other, including handling of beam [alignment /misalignment] for PLRS)] 3. Support of [MAC-CE/MAC-CE+DCI]-based beam TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band] [8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE] |  |  |  |  | [ber band] |  |  |  | {Support, Not support} [4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 6. candidate values {no, with DA, w/o DA, both} 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Resolving the additionally proposed components of FG 23-1-1 may result, for instance, in the following new FGs:

| 23. NR_FeMIMO |  | Unified TCI for inter-cell beam management |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1a | Support of beam misalignment | Support of beam misalignment for PLRS |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1b | TCI application time | X symbols after transmitting HARQ-ACK, the indicated TCI state is applied to a downlink or uplink signal, in the case of DCI based TCI state indication |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1c | Number of activated TCI state | 1. For joint TCI, support of reporting up to Nmax_joint activated joint TCI states --- Candidate value of {1,2,3,4,5,6,7,8}; 2. For separate TCI, support of reporting up to Nmax_DL activated DL TCI states and up to Nmax_UL activated UL TCI states --- Candidate value of {1,2,3,4,5,6,7} for Nmax_DL, and candidate value of {1,2,3,4,5,6,7} for Nmax_UL; |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1a | DCI-based beam indication for intra- and inter-cell beam management | For both intra- and inter-cell beam management: 1. Support of DCI-based beam indication, use of DCI formats 1_1/1_2 with DL assignment 2. The minimum value (Y symbols) of beam application time 3. The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1b | DCI-based beam indication for intra- and inter-cell beam management | For both intra- and inter-cell beam management: 1. Support of DCI-based beam indication, use of DCI formats 1_1/1_2 with and wthout DL assignment 2. The minimum value (Y symbols) of beam application time 3. The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-5a | Support of more than one Rel-17 active DL TCI state/QCL per band | - Support of more than one Rel-17 active DL TCI state/QCL per band for inter-cell beam management - The maximum number of Rel-17 active DL TCI state/QCL per band for inter-cell beam management |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1a | Action delay for unified TCI | 1. Minimum action delay (Y symbols) for unified TCI indication in DCI |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1b | Maximum number of unified TCI states for separate TCI indication | 1. Maximum number of active DL TCI states 2. Maximum number of active UL TCI states 3. Maximum number of active DL and UL TCI states 4. Maximum number of configured DL TCI states per TCI states pool 5. Maximum number of configured UL TCI states per TCI states pool 6. Maximum number of configured DL and UL TCI states per TCI states pool 7. Maximum number of DL/UL TCI states pools |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1c | Maximum number of TCI states for joint TCI indication | 1. Maximum number of active joint TCI states 2. Maximum number of configured joint TCI states per TCI states pool 3. Maximum number of joint TCI states pools |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1d | Uplink power control for unified TCI | 1. Support beam misalignment between PL-RS and spatial relation RS |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1e | BFR with unified TCI | 1. Maximum number of CCs configured with BFR |  |  |  |  |  |  |  |  |  |  |

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-1-1 | Unified TCI for [intra- and inter-cell] beam management | [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [(PLRS and other, including handling of beam [alignment /misalignment] for PLRS)] 3. Support of [MAC-CE/MAC-CE+DCI]-based beam TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band] [8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE] |  |  |  |  | [ber band] |  |  |  | {Support, Not support} [4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 6. candidate values {no, with DA, w/o DA, both} 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | At very high level, the potential issue is that based on Rel-16 RAN2 LS, in each FG, only one component can be {support, not support}. The current proposal may in the end still leads to many additional FGs due to that restrictions from RAN2. From our side, following are important things we need to address for unified TCI indication 1. Separate the support of joint DL/UL TCI indication and separate DL/UL TCI indication 2. Separate the support of following 3 possible TCI indication {MAC-CE, DCI with data, DCI without data} 3. Minimum action delays (Y symbols) 4. Beam misalignment for UL OLPC 5. UE memory related 5.1 maximum number of configured unified DL/UL/DL+UL TCIs 5.2 maximum number of configured unified joint DL and UL TCIs 6. UE processing complexity related 6.1 maximum number of active unified DL/UL/DL+UL TCIs 6.2 maximum number of active unified joint DL and UL TCIs 6.3 maximum number of DL/UL/Joint TCI pools 7. Maximum number of CCs configured with BFR |
| Qualcomm | For 23-1-1 - For the head line, suggest to have inter-cell BM as optional for UE supporting unified TCI. It requires UE to measure and track RS from neighbor cells. It can be in a new FG 23-1-x - For component 1, Suggest to have separate TCI as optional for UE supporting unified TCI. Separate DL/UL TCI is not needed for UE not supporting MPE mitigation. It can be a new sub FG, e.g. FG 23-1-1a - For component 2, suggest to have association between TCI and UL PC setting as optional. The baseline should be TCI-independent PC setting. It can be a new sub FG, e.g. FG 23-1-1b - For component 3, suggest to have MAC-CE+DCI as optional for UE only supporting 1 active TCI. Because there is no need for DCI to further down select. It can be a new sub FG, e.g. FG 23-1-1c - Support component 4, 5, 6 with corresponding candidate numbers - For component 7, it should be optional for UE supporting unified TCI. It can be a new sub FG, e.g. FG 23-1-1d - For component 8, suggest to revise as “The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band”, since the TCI can be joint TCI - Support component 9, 10, 11 - For component 12, what is the difference from component 9? For sub FGs - For FG 23-1-1c, suggest to add the following 2 new components to have a joint constraint on activated beams for both R17 and R15/16 frameworks, which may be enabled simultaneously - 3 For total activated joint, separate DL TCI states, and R15/16 TCI states --- Candidate value of {1,2,3,4,5,6,7,8}; - 4 For total activated joint, separate UL TCI states, and R15/16 spatial relations --- Candidate value of {1,2,3,4,5,6,7,8}; - For FG 23-1-1a containing the row of “Support of DCI-based beam indication, use of DCI formats 1_1/1_2 with DL assignment”, this whole row may be the baseline of unified TCI capability and can be moved to FG 23-1-1 - For FG 23-1-1b containing the row of “1. Support of DCI-based beam indication, use of DCI formats 1_1/1_2 with and wthout DL assignment”, suggest to delete the part in red, which can be moved to FG 23-1-1 as baseline - For FG 23-1-5a, the 2nd component should be sufficient, and the 1st component can be deleted - Suggest to further add the following sub FGs as mentioned in our contribution - Support of separate DL/UL TCI state (FG 23-1-1w) - Support of TCI-specific UL PC settings except for PL RS (FG 23-1-1x) - Support of a common TCI state ID activated/selected for a set of BWPs/CCs (FG 23-1-1y) - Support a TCT state pool configured on a reference BWP/CC shared by a set of BWPs/CCs (FG 23-1-1z) |

# FG 23-1-2: Inter-cell measurement and reporting (for inter-cell BM [and mTRP])

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e.

There seems to be consensus that this FG is needed.

Companies added many new components to FG 23-1-2 of the preliminary NR UE feature list in [1] or proposed new feature groups in addition to FG 23-1-2 such that RAN1 needs to discuss how to spilt this FG into components and FGs, respectively.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-1-2 | Inter-cell measurement and reporting (for inter-cell BM [and mTRP]) | 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of up to Kmax=4 SSBRI-RSRP[,SS-RSRP] pairs, with at least one pair associated with a PCI different from serving cell PCI [in one report instance] [3. The number of RRC-configured PCI(s) different from serving cell PCI for beam measurement in FR1] [4. The number of RRC- configured PCI(s) different from serving cell PCI for beam measurement in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] | [2-24, 2-29] |  |  |  |  |  |  |  | {Support, Not support} [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Again, depends on whether the component is {support, not support}, we may need to introduce more FGs to cover inter-cell measurement 1. For component 2: Kmax should be a reported UE capability, the candidate value is {1, 2, 3, 4} 2. We also need to introduce the following UE capability 2.1. The Nmax: i.e., maximum number of non-serving cells, i.e., NMAX, configured for inter-cell beam measurement 2.2. For maximum number of PCI, we need to separate the configured TCI states and the active TCI states 2.3. Addressing the UE processing complexity for (1) Support of L1-RSRP measurement for SSBs with overlapped resources (2) Support of SSBs for L1-RSRP measurement overlapped with PDSCH/PDCCH/CSI-RS QCLed indirectly or directly with SSBs with a different PCI |
| Qualcomm | - Support component 1, 2, 3, 4 - Suggest to add component 8 - Maximum number of time overlapped SSBs from different cells for L1-RSRP measurement |

# FG 23-1-3: MPE mitigation

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and associated P-MPR in PHR reporting) [2. Support of reporting up to Nmax P-MPR and SSBRI/CRI pairs] [3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation] |  |  |  |  |  |  |  |  | {Support, Not support} [2. Candidate value of {1,2,4}] [3. Candidate value at least includes {64}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| Qualcomm | Support component 1, 2, 3 |

# FG 23-2-1: Multi-TRP PDCCH repetition

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-2-1 | [Multi-TRP] PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2,] support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET [for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Maximum number of such overlaps in 3 Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduling component carrier |  |  |  |  |  |  |  |  | Component1: details FFS {Support, Not support} Component2: details FFS {2, 3, [{2,3}]} Component3: details FFS {monitored, not monitored} Component4: details FFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine. For component 4, the FFS point should be removed since we had agreement that this is reported as UE capability |
| QC | - Component 1: “[with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS]” is not needed. The name “non-SFN” and “TDM and FDM” is confusing and unnecessary and not used anymore in the WI. Whether this FG means Type3 CSS repetition is supported should be discussed separately. We think a separate FG is needed. - Component 3: The added text is not needed. The number of BDs should not be tied to this component. If UE indicates 2 BDs, UE may or may not implement soft-combining. - Component 4: This is already agreed, and should not be FFS. It should be “scheduled cell” and not “scheduling cell”. Also, this number should be per slot in this FG. In addition, the description is not accurate. We suggest: “Maximum number of overlaps (same CORESET, same CCEs, same DCI size, same scrambling) between individual candidates and linked candidates per scheduled cell per slot” - For this FG (baseline), the following note should be added in the note column “Note: For PDCCH monitoring capability, the following is assumed: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot.” Since this is baseline, it is important that this note is included as this FG should only include the basic PDCCH monitoring capability. PDCCH repetition for more advanced PDCCH monitoring capabilities need a separate capability. |

# FG 23-2-2: Two QCL TypeD for PDCCH repetition

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] |  |  |  |  |  |  |  | FR2 only | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| QC | This feature (support of two QCL TypeD) is agreed in the WI for PDCCH repetition. Hence, the changes above are not needed. |

# FG 23-3-1: Multi-TRP PUSCH repetition

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e.

There seems to be consensus that this FG is needed.

Companies added many new components to FG 23-3-1 of the preliminary NR UE feature list in [1] or proposed new feature groups in addition to FG 23-3-1 such that RAN1 needs to discuss how to spilt this FG into components and FGs, respectively.

Several companies proposed to split FG 23-3-1 into type A and type B.

Proposal: Replace the FG 23-3-1 in [1] with two new feature groups incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B,] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [11: Support PUSCH operations] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable_ |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine Again, some of the components may need to be separated as individual FGs since it is {support, not support} BTW, we miss the FG23-3-2 family, i.e., the FGs related to Multi-TRP PUCCH repetition |
| QC | Do not support this proposal. The most important aspect for mTRP PUSCH is that separate capabilities are required for codebook-based and non-codebook based mTRP PUSCH. The FGs should be consistent with Rel.15 design (2-14 and 2-15). We suggest the following as a starting point. 23. NR_FeMIMO; 23-3-1a; Multi-TRP PUSCH repetition for codebook based; 1. Support of codebook-based multi-TRP PUSCH repetition for repetition Type A 2. Support of two SRS resource sets with usage set to codebook 3. Supported number of SRS resources in one SRS resource set 4. Supported number of SRS resources across two SRS resource sets; Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: {1,2,4} Component 4: {2,3,4,5,6,8}; Optional with capability signalling. 23. NR_FeMIMO; 23-3-1b; Multi-TRP PUSCH repetition for non-codebook based; 1. Support of non-codebook-based multi-TRP PUSCH repetition for repetition Type A 2. Support of two SRS resource sets with usage set to noncodebook 3. Supported number of SRS resources in one SRS resource set 4. Supported number of SRS resources across two SRS resource sets; Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: {1,2,3,4} Component 4: {2,3,4,5,6,7,8}; Optional with capability signalling. 23. NR_FeMIMO; 23-3-1c; Multi-TRP PUSCH repetition for repetition Type B; Support of multi-TRP PUSCH repetition for repetition Type B; Optional with capability signalling. In addition, it seems that mTRP PUCCH (FG 23-3-2) is missing from the FL proposals. We suggest the following as a starting point: 23. NR_FeMIMO; 23-3-2; Multi-TRP PUCCH repetition; 1. Support of multi-TRP PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of updating two Spatial Relation Info’s / two sets of power control parameters for a PUCCH resource by MAC-CE 3. Supported PUCCH formats for this scheme; Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1.; Optional with capability signalling. 23. NR_FeMIMO; 23-3-2a; Multi-TRP PUCCH repetition with sub-slots; 1. Support of multi-TRP PUCCH repetition scheme 3 (sub-slot based repetition) 2. Supported PUCCH formats for this scheme; Note1: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Note2: The details of this FG should be decided after FG(s) related to sTRP sub-slot based PUCCH repetition in Rel-17 IIoT AI. Component 3: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1.; Optional with capability signalling. |

# FG 23-4: IntCell-mTRP

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of non-serving cell information which is associated with the TCI state and/or QCL-info including other cell info in RRC TCI state config 2. FFS: Support of X>1 (max number of PCIs different from serving cell) [3. Supported max number of RRC-configured PCIs different from serving cell PCI for FR1 (X1)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for FR2 (X2)] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| QC | For component 3 and 4, it is not clear why X1 and X2 are for FR1/FR2. In the WI discussions, these (X1,X2) are for aligned/non-aligned cases |

# FG 23-5-1: Beam reporting enhancements

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-5-1 | Beam reporting enhancements | - Support of the maximum number of beam groups [in one report instance] - [Support of enhanced group-based reporting for Rel-17 M-TRP] - [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] - [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] - [Support a single L1-RSRP reporting instance consisting of N beams groups and M=2 beams per pair/group] - [Support beam measurement on two CMR resource sets] |  |  |  |  |  |  |  |  | Candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine. The FG name can be clarified as “Group based L1-RSRP Beam reporting enhancements” |
| Qualcomm | Suggest to remove component 2, 3, 4, 5, 6. They either must to have to make it work, or already captured in existing FGs, e.g. R16 FGs |

# FG 23-5-2: Maximum number of BFD-RS resources per set for MTRP BFR

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-5-2 | Maximum number of BFD-RS resources per set for MTRP BFR enhancements | - Support of the maximum number of BFD-RS resources per set - [Support of Rel-17 M-TRP BFR] [(based on two configured or determined BFD-RS sets)] - [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] - [Maximal number of different SSBs of both BFD-RS sets across all CCs] - [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] - [Support up to two PUCCH-SR resources for MTRP BFRQ] - [Support MTRP BFRQ MAC-CE] - [Support the same gNB response as in Rel.16 SCell BFR] - [Support beam update for PDCCH after receiving gNB response on MTRP BFR MAC-CE] |  |  |  |  |  |  |  |  | Candidate values: {1, [2,…]} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We need to cover the following UE related capabilities 1. Maximum number of CCs configured with enhanced TRP specific BFR 2. Supported mTRP mode for TRP-specific BFR, i.e., {sTRP, mTRP, both sTRP and mTRP} 3. Support of simultaneous configuration of cell-specific BFR and TRP-specific BFR The following may need further discussion on details 1. Maximum number of BFD-RS resources 2. Maximum number of CBD-RS resources |
| Qualcomm | Suggest to remove component 2, 3, 4, 5, 6, 7, 8, 9. They either must to have to make it work, or already captured in existing FGs, e.g. R16 FGs |

# FG 23-6-1: SFN scheme A (scheme 1)

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. [Support of scheme A for PDCCH] 2. Support of scheme A for PDSCH 3. [Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We can agree on this assuming each component is independent {support, not support} subject to pre-requisite discussion. In other words, it is more appropriate to separate it into three FGs |
| QC | The description of each component should be updated to further clarify the supported combination of the transmission schemes of PDCCH and PDSCH. Based on RAN1 agreements, the basic component should be support of scheme 1 for PDCCH and PDSCH. - Component 1 should be removed. There is no RAN1 agreement to support only SFN PDCCH. Instead, component 1 should be ‘support of scheme A for PDCCH and scheme A PDSCH’ - Component 2: update wording to ‘Support of scheme A for PDSCH scheduled by single TRP PDCCH’ - Component 3: It should be split into a separate FG. |

# FG 23-6-2: SFN scheme B (TRP based pre-compensation)

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. Support of scheme B [for PDCCH] and PDSCH 2. [Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We can agree on this assuming each component is independent {support, not support} subject to pre-requisite discussion. In other words, it is more appropriate to separate it into three FGs |
| QC | The description of each component should be updated to further clarify the supported combination of the transmission schemes of PDCCH and PDSCH. Based on RAN1 agreements, the basic component should be support of scheme 1 for PDCCH and PDSCH. In addition, similar component structure for scheme A should be adopted for scheme B. - Component 1: should be support of scheme B for both PDCCH and PDSCH. - Component 2: Add component 2 of ‘Support of scheme B for PDSCH scheduled by single TRP PDCCH’’. - The dynamic switching component should be a separate FG. |

# FG 23-6-3: Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST)

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs [(HST)] | Support of simultaneous activation of two TCI states in all BWPs across set of configured component carriers |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle,we are fine |
| QC | We are okay to remove the wording HST. The description should clarify to include activation of CORESET. “Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE” |

# FG 23-6-4: Default beam setup for HST

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-6-4 | Default beam setup [for HST] | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold [4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured] [5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured] [6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We prefer to follow Rel-16 design, i.e., at least have two separate UE feature groups, one for DL default beam and one for UL default beam We also need the following two UE capabilities 1. Support RS(s) with two TCI states configured, either implicitly or explicitly, for beam failure detection enhancement for HST-SFN 2. Support of SFN scheme A PDCCH scheduling Single-TRP PDSCH |
| QC | - Remove component 2 as it was only agreed to support tci-filed not present only for DCI format 1_0 which already doesn’t have TCI field. - Add FG 23-6-4A for support of ‘Two QCL TypeD properties for overlapping CORESETs’ which was agreed as separate UE capability in RAN1 #106e. - Split component 4,5 and 6 into a separate FG (FG 23-6-4B) for UL default spatial relation and pathloss RS. 23. NR_FeMIMO; 23-6-4; Default DL beam setup; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI for DCI format 1_0. 2. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold. 23. NR_FeMIMO; 23-6-4a; Two QCL TypeD properties for SFN PDCCH; Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs when a CORESET is activated with two TCI. 23. NR_FeMIMO; 23-6-4B; Default UL spatial relation and pathloss RS; 1. Support of single-TRP PUCCH transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured. |

# FG 23-6-1: Basic Features of CSI Enhancement for Multi-TRP

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of Nmax=1 - FFS others |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine. For others, we prefer to add the following 1. Support of maximum total number of ports across two CMRs in a pair of CMRs for Multi-TRP CSI measurement 2. Support of X CSI associated with single-TRP measurement hypotheses |
| QC | The FG (baseline FG) needs to be consistent with the Rel-15 and Rel-16 CSI capability framework. Also, max number of ports (23-7-2) is needed as part of the baseline FG, and this should come with how these types of capabilities are reported in Rel-15/16. In addition, supported options (Option 1 with X=0,1,2 or Option 2) should be reported as part of baseline FG. Otherwise, a default assumption is needed. Hence, we suggest the following: 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Support of Nmax=1 - Supported option(s) for CSI reporting - A list of vectors {max # of ports per resource for NCJT, max # of total NCJT hypotheses, max # of total ports of CMRs for NCJT, max # of ports per resource for sTRP, max # of resources for sTRP, max # of total ports for sTRP} - Supported codebook modes - A list of (Y1,Y2): UE can process Y1 NCJT CSI report(s) and Y2 sTRP CSI report(s) simultaneously in a CC - A list of (X1,X2): UE can process X1 NCJT CSI report(s) and X2 sTRP CSI report(s) simultaneously across all CCs; {Support, Not support}; Optional with capability signalling. |

# FG 23-6-2: Support of max # of Tx ports per sourcein a resource set for Multi-TRP CSI

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] {24 ports, 32 ports} [Note: same number of ports among CMRs] |  |  |  |  |  |  |  |  | {Support, Not support} [{4, 8, 12, 16, 24, 32}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine with this FG as place hold for the #ports related UE capability |
| QC | This FG should be included as part of baseline FG as mentioned above. Otherwise, it is not clear what the assumption should be when UE reports the support of 23-7-1. |

# FG 23-6-3: Support of max # resources in a resource set for Multi-TRP CSI

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-7-3 | Support of max # resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Depends on the discussion of FG23-7-2, it can be merged with FG23-7-2 with similar design as lists of triplets |
| QC | Ok, but suggest to change the name to “More than two resources in a resource set for Multi-TRP CSI” since two CMRs is assumed as part of baseline FG (23-7-1) |

# FG 23-6-4: Support of Nmax=2 for Multi-TRP CSI

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | [Support of maximum 2 pairs of CMR resources configured in a CSI-ReportConfig for Multi-TRP CSI measurement for CSI Enhancement for Multi-TRP] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| QC | Ok. |

# FG 23-6-5: Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-7-5 | Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses | [Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis] [FR2 only] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine. We also need to add the following two UE capabilities 1. Support CSI-IM for CSI enhancement for Multi-TRP 2. Support to process CMR and IMR for the same CSI report in different CDRX active time |
| QC | Ok, but suggest to change the name to “CMR sharing” instead of describing the FG. |

# FG 23-8-1: SRS triggering offset enhancement

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| QC | Support updated description of the UE feature. |

# FG 23-8-2: Triggering SRS only in DCI 0_1/0_2

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| QC | Support. |

# FG 23-8-3: SRS Antenna switching for >4Rx

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 [2. Report whether the uplink TX switching impact to downlink receiving in a band] [3. Report whether the UL Tx is switched together with UL Tx in another band] |  |  |  |  |  |  |  |  | {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine. The candidate value needs further discussion especially considering full back operation |
| QC | Support adding components 2 and 3. |

# FG 23-8-4: Maximum 2 SP SRS sets for antenna switching

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-8-4 | Maximum [2] [SP] SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets [and 1 periodic SRS resource set] for antenna switching |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| QC | We are okay to update the description and include “[1 periodicd SRS resource set”, however, we would like to add the notes from the agreement that clarify number of SP/P sets when this feature is not supported. Note1: For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for semi-persistent SRS. Note2: For xTyR where y<=4, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS. |

# FG 23-8-5: Increased repetition for SRS

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| QC | Minor edit to add the word resource at the end. Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource. |

# FG 23-8-6: Partial frequency sounding of SRS

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-8-6 | Partial [(RB level)] frequency sounding of SRS | Support of partial frequency sounding for [periodic and semi-persisent] SRS[,only when freq. hopping enabled] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| QC | Support adding the word RB level into FG name. There is still some discussion whether to additionally support this feature for AP-SRS and whether to extend for non-frequency hopping. It is okay to have square brackets till further agreements made in RAN1. |

# FG 23-8-7: Start RB location hopping for partial frequency SRS

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial [(RB level)] frequency SRS | Support of start RB location hopping in partial frequency sounding |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| QC | Support adding the wording RB level. |

# FG 23-8-8: Comb-8 SRS

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for MIMO SRS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| QC | Word MIMO shouldn’t be removed, it distinguishes between SRS Position and SRS for UL/DL/BM usages. The same wording has been used for Rel-16 UE features. |

# FG 23-9-1: Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II)

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=1 - Support rank 1,2 - Support Mv=2 - FFS others |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine. But it should be Mv=1 |
| Qualcomm | First, suggest to remove M=2 as it is not basic features per previous agreement that M=2 requires separate capability signaling. Second, reporting content is the CSI-RS triplet {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} rather than {support, not support} Third, we think it is essential to add a separate row, e.g., FG 23-10, for codebook concurrency of Type I codebook and Rel-17 FeType II codebook, where UE will report CSI-RS triplet for mixed codebook types in any slot (analogous to FG 16-8 in Rel-16). |

# FG 23-9-2: Support of Mv=2 for FeType-II

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II | 1. Support rank 1,2 2. Support Mv=2 [3. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=1] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| Qualcomm | Support with this FG, but seems only component 2 is needed. This FG should be analogous to FG 16-3a-2 in Rel-16. It just reports whether the UE support M=2 or not. |

# FG 23-9-3: Support of rank 3, 4 for FeType-II

The preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. It is important that during RAN1 #106bis-e, we agree a formal baseline for RAN1 #107-e. The focus of this meeting is thus to agree on the number of rows/FGs whereas the details of each row/FG can further be discussed at RAN1 #107-e. There seems to be consensus that this FG is needed. Regarding the FFS points, only those should be discussed during RAN1 #106bis-e whose resolution may result in the introduction of new feature groups.

Proposal: Agree the following feature group incl. any changes highlighted in red as baseline for further discussions during RAN1 #107-e

- FFS points whose resolution may result in the introduction of new feature groups can be discussed during RAN1 #106bis-e

| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In principle, we are fine |
| Qualcomm | ok |

# Conclusion

After further discussion on the RAN1 email reflector the following was agreed as part of this email discussion:

# References

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2108798, UE features for further enhancements on NR-MIMO, FUTUREWEI

- R1-2108884, UE features for feMIMO, ZTE

- R1-2109013, Discussion on UE features for further enhancements on NR-MIMO, vivo

- R1-2109046, UE features for further enhancements on NR-MIMO, OPPO

- R1-2109143, Rel-17 UE features for further NR MIMO enhancements, Huawei/HiSilicon

- R1-2109203, On Rel-17 FeMIMO UE features, CATT

- R1-2109524, UE features for Rel-17 NR FeMIMO, Samsung

- R1-2109549, UE features for further enhancements on NR MIMO, MediaTek Inc.

- R1-2109642, UE features for feMIMO, Intel Corporation

- R1-2109939, Discussion on UE features for FeMIMO, Lenovo/Motorola Mobility

- R1-2110065, Views on Rel-17 FeMIMO UE features, Apple

- R1-2110219, Discussion on FeMIMO UE features, Qualcomm Incorporated

- R1-2110264, On UE features for further enhancements on NR-MIMO, Nokia/Nokia Shanghai Bell

- R1-2110293, UE features for feMIMO, Ericsson