3GPP TSG RAN WG1#107-e		R1-2111049

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Remaining issues on timing relationship enhancements for NB-IoT/eMTC over NTN

Agenda Item:	8.15.2

Document for:	Discussion and Decision

- Introduction & Background

In RAN1#106bis-e meeting, the following agreements were achieved on timing relationship enhancements for NB-IoT/eMTC over NTN [1].

Agreement: In IoT NTN, for a random access procedure initiated by a N/MPDCCH order, the UE shall delay the transmission of the random access preamble by Koffset as compared to the current specification. Agreement: For IoT NTN, with respect to the granularity, configuration, indication and update of K_Offset, the mechanisms concluded in NR-NTN shall be taken as baseline. Agreement: For eMTC in IoT NTN, if the UE determines that a preamble retransmission is necessary, the choice of a suitable preamble retransmission subframe shall be delayed by Koffset as compared to current specifications. Agreement: For NB-IoT, if the UE has initiated an NPUSCH transmission using pre-configured uplink resources ending in subframe n, the UE shall start or restart to monitor the NPDCCH from DL subframe n+4+K_mac (where K_mac is defined as in NR-NTN). Agreement: For eMTC, if the UE has initiated an PUSCH transmission using pre-configured uplink resources ending in subframe n, the UE shall start or restart to monitor the MPDCCH from DL subframe n+4+K_mac (where K_mac is defined as in NR-NTN). Agreement: Support PUR at least for GEO-based IoT NTN in Rel-17 FFS: for NGSO-based IoT NTN. Agreement: NPDCCH monitoring restrictions have been identified for further checking to see if changes for NB-IoT need to be made for the following cases: - case 1: MTBG NPUSCH - case 2: 2 NPUSCH HARQ processes scheduled - case 3: long single NPUSCH when MTBG or 2HARQ configured - case 4: single NPUSCH scheduled by DCI format N0 or RAR - case 5: NPUSCH format 2 in response to DCI format N1 - case 6: NPRACH in response to PDCCH order - case 7: NPUSCH with same HARQ process when 2 HARQ configured - case 8: subframes after NPUSCH processing - case 9: subframes after NPUSCH carrying Msg3 - case 10: NPRACH for SR for long NPRACH transmissions - case 11: NPRACH for SR for short NPRACH transmissions - FFS: the changes in each case - FFS: additional cases

In this contribution, we discuss the remaining issues on the timing relationship enhancements for NB-IoT/eMTC over NTN.

- Discussion

The propagation delay in NTN ranges from several milliseconds to hundreds of milliseconds depending on the altitudes of the spaceborne or airborne platforms and payload type, which is much longer than that in terrestrial mobile systems. Therefore, some issues about timing relationship need to be discussed and further enhanced.

- 2.1. M/NPDCCH order to PRACH

In last meeting, it was agreed that K_offset is introduced to enhance the timing relationship of M/NPDCCH ordered PRACH in IoT NTN. It can be used to lighten the blind detection burden at eNB side. In the previous meetings, it was agreed that the UE-specific K_offset can be used in connected mode. Compared to the cell-specific K_offset, UE-specific K_offset has finer granularity, which can effectively reduce the delay of PRACH transmission, especially for UEs located in beam center. Even if the UE-specific K_offset is inaccurate already, the blind detection burden will also be greatly reduced at the gNB side by applying the UE-specific K_offset to enhance the M/NPDCCH ordered PRACH. Hence, if UE-specific K_offset is configured, the UE-specific K_offset can be used to enhance the timing relationship for M/NPDCCH ordered PRACH. Otherwise, the cell-specific K_offset is applied.

Proposal 1: Support to apply UE-specific K_offset to M/NPDCCH ordered PRACH if configured; otherwise, cell-specific K_offset is applied.

- 2.2. PDCCH monitoring restrictions

In IoT, UE in FDD-HD mode and cannot transmit and receive simultaneously. In current specifications, there are regulations about the subframes in half-duplex transmissions during which the UE is not expected to monitor NPDCCH either because the UE is preparing to receive scheduled DL transmissions such as NPDSCH or the UE is preparing to execute UL transmissions such as NPUSCH. In last meeting, it is agreed that NPDCCH monitoring restrictions should be further checking to see if changes for NB-IoT need to be made for some cases. Considering the large TA applied in the UE side, the specification after modification is shown below, the changes are marked in red.

For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npusch-MultiTB-Config and if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k, - [case 1: MTBG NPUSCH] if the corresponding NPDCCH with DCI format N0 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1-nTAUE, otherwise [case 2: 2 NPUSCH HARQ processes scheduled] the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2-nTAUE to subframe n+k-1-nTAUE; and - [case 3: long single NPUSCH when MTBG or 2HARQ configured] the UE does not expect to receive a DCI Format N0 before subframe n+k-2-nTAUE for which the corresponding NPUSCH format 1 transmission ends later than subframe n+k+255 if the corresponding NPDCCH with DCI format N0 schedules one transport block. - for TDD, and if the corresponding NPUSCH format1 transmission ends in subframe n+m, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1. otherwise - [case 4: single NPUSCH scheduled by DCI format N0 or RAR]if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1-nTAUE. - for TDD, if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission ends in n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k. For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npdsch-MultiTB-Config - and if the NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if a NPDSCH transmission starts from n+k, - if the corresponding NPDCCH with DCI format N1 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1; - otherwise, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1; otherwise - if the NB-IoT UE detects NPDCCH with DCI Format N1 or N2 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1. If a NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, and - [case 5: NPUSCH format 2 in response to DCI format N1] for FDD, if the corresponding NPUSCH format 2 transmission starts from subframe n+m the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1-nTAUE. - for TDD, if the corresponding NPUSCH format 2 transmission ends in subframe n+m the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1. If a NB-IoT UE detects NPDCCH with DCI Format N1 for "PDCCH order" ending in subframe n, and - [case 6: NPRACH in response to PDCCH order] for FDD, if the corresponding NPRACH transmission starts from subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1-nTAUE. - for TDD, if the corresponding NPRACH transmission ends in subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1. If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig - and if the UE has a NPUSCH transmission ending in subframe n, - the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and - [case 7: NPUSCH with same HARQ process when 2 HARQ configured] the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1-nTAUE to subframe n+3-nTAUE; else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and - [case 8: subframes after NPUSCH processing] if the NB-IoT UE has a NPUSCH transmission ending in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1-nTAUE to subframe n+3-nTAUE. otherwise, - [case 9: subframes after NPUSCH carrying Msg3] If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1-nTAUE to subframe n+3-nTAUE. If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12. If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig - the UE is not required to monitor an NPDCCH candidate of an NPDCCH search space if the candidate ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5 otherwise - the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5. An NB-IoT UE is not required to monitor NPDCCH candidates of an NPDCCH search space during an NPUSCH UL gap. An NB-IoT UE is not required to monitor NPDCCH candidates of a Type2A-NPDCCH common search space during the scheduling gap or the processing gap. For an NB-IoT UE configured with higher layer parameter sr-WithoutHARQ-ACK-Config, if the transmission of a narrowband random access preamble for SR ends on subframe n, - [case 10: NPRACH for SR for long NPRACH transmissions] in case of frame structure type 1 with NPRACH format 0 and 1 when the number of NPRACH repetitions is greater than or equal to 64, or NPRACH format 2 when the number of NPRACH repetitions is greater than or equal to 16, the UE is not required to monitor NPDCCH UE-specific search space from subframe n-nTAUE to subframe n+40-nTAUE, - otherwise, [case 11: NPRACH for SR for short NPRACH transmissions] the UE is not required to monitor NPDCCH UE-specific search space from subframe n-nTAUE to subframe n+3-nTAUE.

According to the description above, nTAUE should be introduced to modify the restrictions on NPDCCH monitoring in those case, and the unit of nTAUE is subframe and the value of nTAUE is derived from TA in UE side.

Observation 1: NPDCCH monitoring restrictions need to be changed in NB-IoT for the following cases:

- case 1: MTBG NPUSCH

- case 2: 2 NPUSCH HARQ processes scheduled

- case 3: long single NPUSCH when MTBG or 2HARQ configured

- case 4: single NPUSCH scheduled by DCI format N0 or RAR

- case 5: NPUSCH format 2 in response to DCI format N1

- case 6: NPRACH in response to PDCCH order

- case 7: NPUSCH with same HARQ process when 2 HARQ configured

- case 8: subframes after NPUSCH processing

- case 9: subframes after NPUSCH carrying Msg3

- case 10: NPRACH for SR for long NPRACH transmissions

- case 11: NPRACH for SR for short NPRACH transmissions

Proposal 2: Support to introduce nTAUE for current restrictions on NPDCCH monitoring in those cases, where the unit of nTAUE is subframe and the value of nTAUE is derived from TA in UE side.

- Conclusion

In this contribution, some remaining issues on timing relationship enhancements for NB-IoT/eMTC over NTN were discussed with the following proposals:

Observation 1: NPDCCH monitoring restrictions need to be changed in NB-IoT for the following cases:

- case 1: MTBG NPUSCH

- case 2: 2 NPUSCH HARQ processes scheduled

- case 3: long single NPUSCH when MTBG or 2HARQ configured

- case 4: single NPUSCH scheduled by DCI format N0 or RAR

- case 5: NPUSCH format 2 in response to DCI format N1

- case 6: NPRACH in response to PDCCH order

- case 7: NPUSCH with same HARQ process when 2 HARQ configured

- case 8: subframes after NPUSCH processing

- case 9: subframes after NPUSCH carrying Msg3

- case 10: NPRACH for SR for long NPRACH transmissions

- case 11: NPRACH for SR for short NPRACH transmissions

Proposal 1: Support to apply UE-specific K_offset to M/NPDCCH ordered PRACH if configured; otherwise, cell-specific K_offset is applied.

Proposal 2: Support to introduce nTAUE for current restrictions on NPDCCH monitoring in those cases, where the unit of nTAUE is subframe and the value of nTAUE is derived from TA in UE side.

- References

- 3GPP RAN1 106bis-e meeting, chairman’s notes.