3GPP TSG RAN WG1 Meeting #107-e  								            R1-2111122

e-Meeting, November 11th  – 19th, 2021

Agenda Item:	8.4.2

Source:	Thales

Title:	Considerations on UL timing and frequency synchronization in NTN

Document for:	Discussion and decision

# Introduction

The agreements related to UL time and frequency synchronization in NTN achieved during RAN1 Meeting #106-bis-e are captured in FL Summary on enhancements on UL time and frequency synchronization for NR NTN [R1-2110602].

In this contribution, we make observations and proposals to address remaining issues under AI 8.4.2 identified and discussed during RAN1#106-bis e-meeting.

# UL timing synchronization in NTN

## NTN UE Timing Advance

The Timing Advance to be applied by an NR NTN UE in the different RRC sates was discussed in RAN1 Meeting #104-bis-e and the following agreement was made:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

The following sections will address remaining issues on Common TA parameters, TA maintenance in RRC Connected state and NTN validity timer.

## Indication of Common TA drift parameters

When the reference point for uplink time synchronization is not at the satellite the UE needs to compensate the feeder link’s RTD and thereby the timing drift on the feeder link shall be compensated by the UE with sufficient accuracy to ensure aligned DL and UL symbol level processing (FFT and IFFT window) at Radio Unit(RU)/gNB.

In RAN1#106-bis-e, the following two Agreements were made:

Agreement:

Confirm the working assumption:

Common TA may include parameter(s) indicating timing drift.

- The UE will apply common TA according to the parameters provided by the network (if any). No offset between the common TA according to the parameters provided by the network and the actual feeder link RTT is considered when defining UE UL timing error requirements.

Agreement:

In NTN, the Network may optionally indicate one or more of the following parameters:

- Common TA , Common TA drift rate and Common TA drift rate variation.

- FFS: Common TA third order derivative.

- FFS: Details of combination of Common TA parameters

A remaining open question from last RAN1 meeting: when Common TA includes parameters indicating timing drift (i.e. Common TA drift rate and Common TA drift rate variation) which of these parameters are mandatory? To answer this question, we evaluate the Common TA estimation error at the UE:

To assist the UE to calculate the NTAcommon at time t, the gNB shall regularly broadcast (e.g. every 160ms) the Common TA related parameters, these may include (but not limited to): TACommon , TA CommonDrift and TA CommonDriftVariation.

TA CommonDrift and TA CommonDriftVariationindicate respectively the drift rate of the common TA and the drift rate variation of the common TA (second order derivative).

Then, the UE can perform a second-order approximation (Equation 1, expressed at unit of Tc) to derive the  NTAcommon  from the higher-layer parameters TACommon, TACommonDrift and TACommonDriftVariation, if configured, otherwise NTA,common=0.

Equation 1:

NTAcommont= NTA, t0common+   TA CommonDrift, t0×t-t0+12 TA CommonDriftVariation,t0×t-t02

Where :  NTA, t0common= TACommon, t0 * 64/2μ

And TACommon t0 , TA CommonDrift, t0  and TA CommonDriftVariation,t0are network indicated Common TA parameters at epoch time (or reference time) t0.

The common TA parameters reference time (or epoch time) might be or not the same as the Satellite ephemeris epoch time. But if Common TA related parameters and serving Satellite ephemeris data broadcast on the same SIB, it would be reasonable to use the same reference epoch time. The discussion on epoch time is captured in sections 2.2.3 and 4.5.

This is an example of UE self-estimation of Common TA based on a second-order approximation:

Figure 1 Second-order approximation of Common TA

For better accuracy of self-estimated Common TA a second-order approximation is needed, thereby, both Common TA drift rate and Common TA drift variation rate need to be jointly broadcast by the Network.

### Common TA estimation error

Having feeder link timing drift compensated by UE using Common TA parameters will introduce additional timing inaccuracy.

Common TA parameters  (Common TA, Common TA drift rate and drift variation rate) acquired in the SIB will be valid only during a validity time period which depends on:

- the maximum tolerable error on common TA estimation (which also depends on the CP and the delay spread) and

- the order of the Common TA approximation to be carried out by the UE.

After this validity period, a new acquisition of Common TA estimation assistance information is required.

Based on simulation results, as shown within Table 1 , when common delay drift and common delay drift variation are jointly indicated, the UE can estimate the Common one delay on the feeder link with a maximum error less than 0,084 μs  (= 2.6 ×64 × Tc) assuming the UE acquires the Common TA related parameters once every 10 microseconds.

Note that the following assumptions were considered:  LEO @600 km scenario and the satellite flies by the GW (worst case).

The following sources of errors were taken into account:

- It is assumed that the gNB knows the satellite trajectory with a maximum error on satellite position of 3.87 m (orbit prediction error will be discussed in section 4.3).

- Also, the values provided in Table 1 take into account the error due to the quantization of common TA parameters broadcast in SIB.

Table 1 Maximum one-way Common Delay error estimation

| Prediction time [s] | maximum one way Common Delay estimation error [microseconds] |  |
| --- | --- | --- |
|  | Without quantization error | With quantization : 23 bits for Common Delay 12 bits for Common Delay drift 5 bits for Common Delay drift variation |
| 3 | 0.005 | 0.022 = 0.7 ×64 × Tc |
| 5 | 0.008 | 0.028 = 0,9 ×64 × Tc |
| 10 | 0.028 | 0,084 = 2.6 ×64 × Tc |
| 15 | 0.086 | 0.175 = 5.4 ×64 × Tc |
| 20 | 0.197 | 0.320 = 9.8 ×64 × Tc |
| 25 | 0.381 | 0.590 = 18 ×64 × Tc |
| 30 | 0.652 | 0.880 = 27 ×64 × Tc |

Maximum Common Delay estimation error shall take into account the error on satellite position and the error of  quantization of common TA parameters broadcast in SIB.

The UE can estimate the Common one delay on the feeder link with a maximum error less than 0,084 μs  (= 2.6 ×Ts) assuming the UE acquires the Common TA related parameters once every 10 microseconds..

Further, the residual timing inaccuracy due to common TA approximation can be managed by closed loop TA commands:

For example in case of RRC connected state and 120 kHz SCS, as depicted in Table 1, the UE shall acquire new common TA parameters at least every ~9 seconds. But, by applying the closed loop TA commands, the gNB can gradually adjust the Uplink timing so that the error on UE self-estimated Common TA remains below an acceptable threshold for longer duration; that is without the need to acquire updated common TA parameters in SIB. As shown in Figure 2 and Figure 3, by applying every 200ms a TAC =  TA-31.16.642μ = 16*64/(2^3)*0.509*1e-9, the maximum time during which the UE can keep track of the Common delay without having acquired new assistance information is extended to 15 seconds (instead of 9 seconds if only open loop is used):  As can be seen on the left diagram in Figure 3, all the Common delay estimation errors values are below ± 0.075 μs (Common TA error of 25% of CP in case of 120 kHz SCS).

Figure 2 Gradual Common TA adjustment using Control loop TA Command

Figure 3 Effect of gradual timing adjustment using closed loop TA commands

The residual timing inaccuracy due to common TA approximation can be managed by gradual timing adjustment using closed loop TA commands if necessary

Based on the above analysis, we made the following proposal:

Proposal 1:

Higher-layer parameters TACommon, TACommonDrift and TACommonDriftVariation are jointly indicated if the Common TA include parameters indicating timing drift.

### Granularity and signaling of Common TA

The following Agreement was made at RAN1#106-b.

Agreement

- The granularity of Common TA is set to be 642μ∙Tc

- μ is the highest allowed numerology supported for data, for the given Frequency Range

Therefore, TACommon values shall be indicated in SIB by index values of  TACommon = 0, 1, 2, ..., p. where an amount of the common TA to be applied by the UE at epoch time t0, is given by: 
NTA common= TACommon * 64/2μ, with subcarrier spacing of kHz.

p being the maximum range of TACommon : p=NTA, max common642μ= maximum RTD on the feeder (ms)1000.Tc.642μ

The maximum RTD on the feeder link that shall be supported in NTN is 270.73 ms in case of GEO based non-terrestrial access network, 12.89 ms (600km), 20.89 ms (1200km) in case of LEO based non-terrestrial access network.

With the above considerations in mind, the maximum range of TA-Common as well as the number of bits to be used for TA-Common in SIB are given in the following table:

Table 2 Value range and payload size of common TA

| Frequency Range |  | GEO based NTN | LEO based NTN |  |
| --- | --- | --- | --- | --- |
|  |  |  | 600km | 1200km |
|  | maximum RTD on the feeder (ms) | 270.73 | 12.89 | 20.89 |
| FR1 | maximum range of TACommon | 16621439 | 791380 | 1282539 |
|  | payload (#bits) | 24 bits | 20 | 21 |
| FR2 | maximum range of TACommon | 66485757 | 3165521 | 5130157 |
|  | payload (#bits) | 26 bits | 22 bits | 23 bits |

Further, based on [2]:

For LEO@600 km: The value range of RTD on the feeder = 2 ms…12,89 ms The value range of TACommonDrift = -24 microseconds/second … 24 microseconds/second The value range of TACommonDriftVariation = -0,27 microseconds/second² … 0 microseconds/second²

For LEO@1200 km: The value range of RTD on the feeder = 4 ms … 20,89 ms The value range of TACommonDrift = -21 microseconds/second … 21 microseconds/second The value range of TACommonDriftVariation = - 0,13 microseconds/second² … 0 microseconds/second²

For GEO The value range of RTD on the feeder = 119,28 ms … 270.73 ms The value range of TACommonDrift = -0,93 microseconds/second … 0,93 microseconds/second Indication of TACommonDriftVariation may not be needed

Proposal 2:

- In case of GEO based non-terrestrial access network:

  - In FR1:

    - Value range of TACommon : 0 - 16621439

    - TACommon bit allocation: 24 bits

  - In FR2:

    - Value range of TACommon : 0 - 66485757

    - TACommon bit allocation: 26 bits

- in case of LEO based non-terrestrial access network:

  - In FR1:

    - Value range of TACommon : 0 - 1282539

    - TACommon bit allocation: 21 bits

  - In FR2:

    - Value range of TACommon : 0 - 5130157

    - TACommon bit allocation: 23 bits

Moreover, it was observed through the simulations that in case of LEO-600, a field of 14 bits will be needed for Common TA drift rate indication and a field of 10 bits for Common TA drift variation rate.

Proposal 3:

Higher-layer parameters TACommonDrift and TACommonDriftVariation are indicated:

In 14 bits and with value range : -24 microseconds/second … 24 microseconds/second, for TACommonDrift.

In 10 bits and with value range: -0,27 microseconds/second² … 0 microseconds/second², for TACommonDriftVariation.

### The Epoch time of common TA parameters

The following agreement was made at RAN1#106-e meeting:

Agreement:

Common TA Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame.

- FFS: Whether this starting time is given by predefined rule or it is indicated by the Network

  - Note: “implicitly known” means that UTC is not provided to define the Common TA epoch time.

Whether the starting time defining the Common TA Epoch time is given by predefined rule or it is indicated by the Network is still FFS.

During previous meeting, RAN1 discussed whether the same reference time option for both Comment TA reference time and serving satellite ephemeris epoch time can be used. We share the view that Common TA parameters and Satellite ephemeris data are transmitted within the same SIB and are associated to the  same epoch time.

Proposal 4:

Satellite Ephemeris data and Common TA related parameters are sent within the same SIB/SI.

The epoch time of serving satellite ephemeris implicitly known as a reference time defined by the starting time of a DL slot and/or frame is used as reference time for Common TA parameters.

w.r.t serving satellite ephemeris Epoch, the following two options were discussed in last RAN1 meeting:

Option 1: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame by indication of the Nth slot after start of SI window of SI message carrying Serving satellite ephemeris.

N is optionally signaled with the ephemeris (otherwise 0).

Option 2: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL sub-frame and/or frame by indication with SFN and the sub-frame number that the ephemeris data is valid for.

In our view, if the SFN and slot or sub-frame number (preferably use sub-frame as it provides an SCS agnostic time reference) are given to indicate the time assistance data (ephemeris and common TA) is valid for, the assistance data can be repeated as shown in the figure 4. With option 1, the broadcast data cannot be repeated.

Figure 4 Epoch time of common TA parameters and serving satellite ephemeris

Proposal 5:

The Epoch time of common TA parameters is implicitly known as a reference time defined by the starting time of a DL sub-frame and/or frame by indication with SFN and the sub-frame number that the common TA parameters are valid for.

It is worth noting that in NTN serving satellite ephemeris and common TA related parameters are essential information required for initial access, thereby, such information should be encompassed within  Minimum SI (MSI): Either SIB1 or a specific NTN MSI.

Proposal 6:

Send LS to RAN2 asking RAN2 to include serving satellite ephemeris and common TA related parameters within the Minimum SI: either SIB1 or a specific NTN MSI.

### Reference point for epoch time of the Common TA parameters

The issue on reference point for epoch time of the Common TA parameters and for epoch time for serving Satellite ephemeris was discussed during RAN1#106b but not agreement was made. Definition of such reference point is needed. Clearly, the UE and gNB shall share the same understanding about when the slot or sub-frame start that defines the epoch time is transmitted from the reference point. Knowing the reference point and the indicated epoch time the UE can derive the aging of the indicated assistance information.

Two options may be considered: Either the GW/gNB or the Satellite. Our preference is to have the reference point in the satellite transmitter.

Proposal 7:

The reference point for epoch time of Common TA parameters and the serving satellite ephemeris should be known by UE. This reference point is the satellite transmitter.

## TA update in connected mode

### TA update based on closed loop

In  RAN1#106-b-e meeting, RAN1 concluded to not define a TA margin. But still the issue related to a possible initial TA overcompensation is still to be solved. Indeed: if the UE does not delay its initial transmission (at least by + Te_NTN) there might be two issues; issue (i) and/or Issue(ii) discussed hereafter:

- Issue (i): Performance impact (unwanted interference with previous PRACH occasion or UL PUSCH/PUCCH)

w.r.t (i) as highlighted by some companies (.e.g. ZTE- R1-2008851) negligible impacts due to the pre-compensation error (+/-Te_NTN) can be observed: we may conclude that any delay in initial transmission might not be needed as long as the initial transmission requirement is satisfied (within  +/-Te_NTN).

- Issue (ii): The value of TAC in RAR: would need to support negative value. Indeed, as per RAN4 requirement: When the transmission timing error between the UE and the reference timing exceeds ±Te_NTN then the UE is required to adjust its timing to within ±Te_NTN. If the UE advances its initial transmission by + Te_NTN, RAN1 needs to clarify the behavior of gNB. And define/clarify the adjustment to be sent in TAC/RAR.

Proposal 8:

TAcommon may include a timing offset (.e.g. = + Te_NTN ) if considered necessary to avoid that the UE over pre-compensates its TA during RACH procedure.

### TA update based on open loop

In RRC Connected state, by updating NTA,UE-specific+NTA,common the UE could autonomously track the RTD variation on both service link and feeder link to keep the residual timing error within a maximum tolerable range.

How the UE calculates/updates NTA,UE-specific and NTA,commonis not agreed. Such updated maybe left to UE implementation.

Proposal 9:

-       NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay, which is calculated using the UE position and the serving satellite ephemeris. How the UE calculates/updates NTA,UE-specific is left to UE implementation.

-        NTA,commonis updated autonomously by the UE based on the Common TA parameters indicated by the Network as follows:

NTAcommont= NTA, t0common+   TA CommonDrift, t0×t-t0+12 TA CommonDriftVariation,t0×t-t02

### NTN UE Time Alignment Timers

The following agreement was made in RAN1#106:

Agreement:

•	A validity duration configured by the network for satellite ephemeris data indicates the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris.

FFS: Associated UE behaviour if the UE does not read the ephemeris within the validity duration.

•	FFS: Whether the same validity duration can be applied for Common TA.

Further, the following agreements were made at RAN1#106-bis-e:

Agreement:

The UE assumes that it has lost uplink synchronization if new or additional assistance information (i.e. serving satellite ephemeris data or Common TA parameters) is not available within the associated validity duration.

FFS: details on how to acquire new or additional assistance information

W.r.t Validity timer start/restart:

Agreement:

NTN ephemeris validity timer should be started/restarted with configured timer validity duration at the epoch time of the assistance information (i.e. serving satellite ephemeris data)

W.r.t Validity timer(s) for ephemeris / Common TA:

Agreement:

A single validity duration for both serving satellite ephemeris and common TA related parameters is defined at least if serving satellite ephemeris and common TA related parameters are signaled in the same SIB message.

The Validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB and can be indicated using dedicated signaling when UE is RRC connected, e.g. if UE is not configured with common search space.

It is reasonable to use a single validity timer for both serving satellite ephemeris and common TA related parameters. Thus the value range to be defined for the Validity duration/timer should be the minimum validity duration  for the common TA and satellite ephemeris.

The determination of range/values for this single validity timer should take into account the timing error due to:

- Serving-satellite position estimation error due to orbit propagation at NCC/gNB

- Serving-satellite position estimation error due to orbit propagation at UE: RAN1 made a conclusion that Orbit propagation model is left to implementation, thus  it is reasonable for the Network to assume a simplest orbit propagator model at the UE to determine the validity timer value.

- Common TA estimation error at the UE.

Proposal 10:

A single validity timer for both serving satellite ephemeris and common TA related parameters is indicated using higher-layer parameter ntnUlSyncValidityDuration.

ntnUlSyncValidityDuration is indicated in 4 bits with 16 values: 5s, 10s, 15s, 20s, 25s, 30s, 35s, 40s, 45s, 50s, 55s, 60s, 120s, 180s, 240s, 300s.

# UL frequency synchronization in NTN

Common Doppler shift pre/post compensation on the feeder links

The following Conclusion was made at RAN1#105e:

Conclusion:

The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated by the GW and satellite-payload without any specification impacts in Release 17.

Common Doppler shift pre compensation on the DL service link.

It was observed via simulations [2] that for DL initial synchronization, robust performance can be provided by the SSB design in Rel-15 in case of LEO with beam specific pre-compensation of common frequency shift.

In RAN#104-e, RAN1 made the following conclusion:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary.

- FFS: support of DL frequency compensation for the service link Doppler.

In case of Earth fixed cell, if DL frequency compensation for the service link Doppler is applied, gNB needs to periodically update and broadcast the DL pre-compensated frequency offset (FO). This pre-compensated  FO is time-varying as show in Figure 4 below:

Figure 5 Pre-compensated Frequency Offset w.r.t to the center of an Earth-fixed beam- LEO@600 km

The pre-compensated  FO will be used by the UE to perform its own estimation of the residual Doppler experienced on the DL. The UE needs to know this residual Doppler to be able to generate  UL frequency carrier for its uplink transmission. Therefore, the UE shall acquire the broadcast pre-compensated  FO and performs a first order approximation to self-calculate the pre-compensated  FO. Hence, the network needs to also indicate the drift rate of common pre-compensated FO. The error on self-calculated pre-compensated FO should be within a specified threshold .e.g 10% 0,1 ppm, that is 0.01 ppm.

Based on simulation results, in case of LEO @600 km, by using the indicated amount of frequency compensation and the associated FO drift rate the UE can self-calculate and predict pre-compensated FO 2.5 seconds ahead if quantization error is not taken into account. UE self-calculated pre-compensated FO is shown in Figure 6.

Figure 6 Self-calculated pre-compensated FO

By considering the quantization error, as shown in Figure 7 the UE will be able to predict the pre-compensated FO 2.3 seconds ahead when the  amount of frequency compensation is indicated in a field of 14 bits and the drift rate of common pre-compensated FO in a field of 5 bits.

Figure 7 Maximum pre-compensated FO error

If DL frequency compensation for the service link Doppler is supported, in case of Earth-fixed cell, the UE needs to frequently acquire the SIB to retrieve common pre-compensated FO parameters.

At least 19 bits are needed to indicate the amount of frequency compensation and associated drift rate.

Proposal 11   DL frequency compensation for the service link Doppler is not supported in Release 17.

# Satellite orbit determination and prediction

The aim of this section is to provide some insights on how and with which accuracy satellite position and velocity can be determined and predicted.

Satellite orbit determination

The satellite orbit determination (OD) consists in estimating the most likely track or orbit which have been taken by a satellite based on past and noisy measurements of its position and velocity. In general, the satellite orbit determination is performed as follows:

Step 1: Measurements of the satellite position and velocity are made and dated. Most of the time, these measurements are GNSS-based measurements performed on-board. The accuracy of GNSS measurements depends on several factors such as:

- The GNSS receiver characteristics (mono-frequency vs bi-frequency, single vs multi-constellation, quality of the GNSS receiver)

- The satellite orbit, and the number of GNSS satellites in view

Step 2: The measurements are collected in the NTN Control Center (NCC) where the satellite OD will be performed. In general, the satellite GNSS measurements are delivered to the ground via dedicated telemetry channels between the satellite and the NTN GW network. The reporting period of new GNSS measurements is implementation specific but it is directly related to the accuracy target associated to the instantaneous knowledge of the satellite position and velocity in the system.

Step 3: The satellite OD is performed in the NCC. This operation can be more or less complex depending on the models considered, the quantity of measurements available and the algorithms used. Traditionally, two filtering techniques are used on input measurements: Kalman filtering or least square filtering. Least square filtering usually provides more accurate results but requires more computing power.

An overview of a typical satellite system architecture is given on 4 where the locations of each OD step are explicitly represented.

The satellite OD is useful for several reasons. It helps estimating what was the satellite past trajectory so one can identify whether the satellite is diverging from a reference orbit and if needed schedule satellite station keeping manoeuvres to get it back on track.

Figure 8 : System Architecture Overview

Hereafter, we provide several examples of orders of magnitude of precision which can be expected for orbit determination. This performance shall not be confused with orbit prediction performances which concern the future satellite trajectory and are discussed in the next section.

One shall distinguish between orbit determination performance based on past measurements of the satellite trajectory and orbit prediction performance which concern the future satellite trajectory.

Case 1 : “Best” Precision Orbit Determination

For satellite systems designed for navigation purposes delivering very accurate and abundant GNSS measurements, it is possible to achieve the following performance:

- 3D Position RMS Error = 0.05 m

- 3D Velocity RMS Error = 0.05 mm/s

However, in commercial telecommunication satellites such performance cannot be met because the cost would be prohibitive and such OD precision is not required for the system to work.

Case 2 : “Typical” Precision Orbit Determination

For telecommunication satellites, it is more typical to consider the following values:

- 3D Position RMS Error = 0.5 m

- 3D Velocity RMS Error = 0.5 mm/s

Case 3 : “Low Quality” Precision Orbit Determination

Finally, for low cost system the following performance can be considered:

- 3D Position RMS Error = 5 m

- 3D Velocity RMS Error =5 mm/s

Note: As a rule of thumb; it can be assumed that there is a factor of 1000 between the position error (in m) and the velocity error (in m/s). This is important to keep in mind when defining accuracy targets for satellite position and velocity.

As a rule of thumb; it can be assumed that there is a factor of 1000 between the position error (in m) and the velocity error (in m/s). This is important to keep in mind when allocating an error budget for satellite position and velocity estimations.

Satellite orbit prediction

The satellite orbit determination can also be used to derive updated satellite ephemeris. The satellite ephemeris are useful to predict future satellite positions and velocity vectors. This can be useful for various reasons such as:

- Determine when the satellite is going to fly by over a specific zone of interest

- Anticipate collision avoidance operations

- Anticipate future feeder link switch over operations within the NTN gateways network.

- Perform Doppler compensation (for instance on the feeder links between the satellite and the serving NTN GW)

Of course, the accuracy of the orbit prediction required to perform such operations can be significant different from an application to another. Finally, the NCC can share the satellite ephemeris (under various formats) with other entities in the system responsible for performing such operations (e.g. with the NTN GW).

Then, one can wonder with which accuracy one can predict the future satellite position and velocity vectors and over what time horizon.

The orbit prediction accuracy may depend on several factors:

- The accuracy of the orbit determination used to derive the satellite ephemeris: this will drive the initial errors of the propagation model.

- The accuracy of the orbit propagation model: there are many propagation models which takes into account more or less precisely the physical effects on the satellite movement (J2/4 perturbations, aero-dynamical drag, anticipated station keeping manoeuvres)

- The time horizon over which the prediction is made: The initial errors combined with the propagation model errors tend to increase the prediction errors when the prediction period increases.

The orbit prediction accuracy depends on:

  - -	The accuracy of the orbit determination used to derive the satellite ephemeris

  - -	The accuracy of the orbit propagation model

  - -	The time horizon over which the prediction is made

## Orbit prediction error at NTN Control Center

In this study, we tried to quantify the propagation error evolution depending on the initial PV errors. The main assumptions and results of this study are summarized in Table 3.

A simplified relative position-velocity model for circular orbits (Clohessy-Wiltshire) has been considered. The CW equations are given in Figure 8 where the z-axis is along the radius vector of the target spacecraft, the y-axis is along the angular momentum vector of the target spacecraft, and the x-axis completes the right handed system.	These equations are usually used for rendez-vous or docking relative position evaluations.

However, they are used here to evaluate the propagation error evolution between the reference orbit and the predicted orbit which progressively moves away from the reference orbit due to propagation errors.

Figure 9: Clohessy-Wiltshire equations of motion

This study characterizes only the impact of the initial PV error on the prediction errors. The additional errors related to the orbit propagation model limits are not simulated here. However, when considering short prediction period (i.e. inferior to the orbital period), the propagation errors induced by the propagator models limitations (J2/J4 perturbation, air drag, …) may be considered negligible w.r.t. to the prediction errors induced by the initial PV errors.

Effect of initial errors on tangential (X), normal (Y), radial (Z) axes in position and velocity gives very different results. The worst case effects are observed when the initial error is applied on Z axis for position and X axis for velocity.

For illustrative purpose, the propagation errors over 2 revolution orbits for each worst case initial errors on position (dz0 = 1 m) and on velocity (dvx0 = 1mm/s) have been illustrated on Figure 10 and Figure 11 respectively. It can be observed that the velocity error is not an increasing function of time.

Figure 10 Propagation errors – dZ0 = 1 m

Figure 11 Propagation errors – dvX0 = 1 mm/s

For initial PV errors, the POD assumptions presented in the previous section have been considered.

Finally, one shall define a PV accuracy target to be guaranteed. When the propagation errors exceed the PV accuracy target then it means that a new orbit determination shall be performed and updated ephemeris shall be provided. The PV accuracy target is going to be overshoot after a more or less long prediction period depending on the initial PV error assumption.

In this study, the target PV accuracy has been driven by the accuracy expected to perform Doppler pre/post-compensation on feeder links. The Doppler compensation residual errors experienced on the feeder link shall be almost transparent to the gNB/UE. Then, it shall be an order of magnitude lower than the frequency error budget allocated at UE side (~0.1 ppm). Moreover, the carrier frequencies used on the feeder links may be significantly higher than the one used on the service link so additional margin shall be considered to counter shift this effect. As a consequence, it is proposed to assume that the satellite radial velocity w.r.t the NTN GW should be known with an accuracy better than +/- 0.3 m/s when performing Doppler compensation on the feeder link. It has been demonstrated by simulation (see Figure 11 and Table 3) that when the following assumptions on the PV errors are met the previous requirement is guaranteed :

- Gaussian position error distribution with 3D Position RMS Error = 10 m => 3sigma = 30 m

- Gaussian velocity error distribution with 3D Velocity RMS Error = 10 mm/s => 3sigma = 30 mm/s

Therefore, it is proposed to consider the following PV accuracy target:

- Position error < 30 m

- Velocity error < 30 mm/s

The simulation results are summarized in Table 4. It can be observed that the maximal propagation time before overshooting the PV accuracy target is significantly driven by the initial error. More precisely, depending on the quality of the orbit determination, the maximal propagation time can go from 8 hours to less than 10 minutes. The less accurate is the orbit determination, the more often orbit determination shall be performed based on more recent measurements and updated satellite ephemeris shall be distributed.

It can be also observed that the velocity error degradation is negligible w.r.t the position error. As a consequence, the radial velocity (or Doppler) error is mainly driven by the position error degradation.

In conclusion, it has been demonstrated that even for a satellite system with “low quality” orbit determination Algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably. The limiting condition is that the system should be designed such that new orbit determination and satellite ephemeris update can be performed frequently enough.

Even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably.

Figure 12 Monte Carlo simulation to evaluate radial velocity error [m/s] distribution assuming a PV Gaussian errors with (3D Position RMS Error = 10 m / 3D Velocity RMS Error = 10 mm/s)

Table 3 : Monte Carlo simulation parameters

| Satellite orbit | Polar orbit (i = 90°) |
| --- | --- |
| GW position | Equatorial ground station Min elevation = 30° |
| Error on position/velocity | Gaussian distribution on norm - 3D Position RMS Error = 10 m - 3D Velocity RMS Error = 10 mm/s Uniform distribution on direction |

Table 4 : Results of propagation error study

| Model | Kepler Model Clohessy-Wiltshire equations |
| --- | --- |
| Orbit | Circular @ 600 km Period = 5801 s |
| Initial PV Errors | Case 1 : “Best” Precision Orbit Determination - 3D Position RMS Error = 0.05 m - 3D Velocity RMS Error = 0.05 mm/s Case 2 : “Typical” Precision Orbit Determination - 3D Position RMS Error = 0.5 m - 3D Velocity RMS Error = 0.5 mm/s Case 3 : “Low Quality” Precision Orbit Determination - 3D Position RMS Error = 5 m - 3D Velocity RMS Error =5 mm/s |
| PV accuracy target | Position error < 30 m Velocity error < 30 mm/s |
| Prediction period before overshooting PV accuracy target | Case 1 : “Best” Precision Orbit Determination Propagation to 5 orbital periods before overshooting PV accuracy target ~ 8 h - Statistics on max position error - Mean = 4.35 m - Std dev = 4.10 m - Max = 35.9 m - Statistics on max velocity error - Mean = 0.3 mm/s - Std dev = 0.3 mm/s - Max = 2.6 mm/s Case 2 : “Typical” Precision Orbit Determination Propagation to 0.5 period before overshooting PV accuracy target ~ 48 mn - Statistics on max position error - Mean = 4.63 m - Std dev = 4.23 m - Max = 36.7 m - Statistics on max velocity error - Mean = 0.3 mm/s - Std dev = 0.3 mm/s - Max = 2.6 mm/s Case 3 : “Low Quality” Precision Orbit Determination Propagation to 0.1 period before overshooting PV accuracy target ~ 10 mn - Statistics on max position error - Mean = 5.65 m - Std dev = 3.61 m - Max = 28.9 m - Statistics on max velocity error - Mean = 7.5 mm/s - Std dev = 5.1 mm/s - Max = 46 mm/s |

In above results the PV accuracy targets are Position error < 30 m and Velocity error < 30 mm/s. Thereby, in case of “Low Quality” Precision Orbit Determination,  propagation to 0.1 period before overshooting PV accuracy target is needed. That is, in this case an OD shall be performed every 10 minutes to achieve such requirement on PV accuracy. In case of “Typical” Precision Orbit Determination, propagation to 0.5 period before overshooting PV accuracy target is enough and then an OD shall be performed every 48 minutes.

Therefore, higher performance can be achieved when the Orbit Determination (OD) is performed frequently and  propagation period is shortened to a fraction of orbit period (up to few of minutes).

As can be seen in the following table: By taking into account propagation model errors, satellite position can be predicted 60 seconds ahead with only max error of 1.47m and 5 minutes ahead with max error of 3.87m in case of  initial 3D Position RMS Error = 0.5 m and 3D Velocity RMS Error = 0.5 mm/s (Typical” Precision Orbit Determination used for telecommunication satellites).

Table 5 Satellite PV errors - propagation model errors not taken into account

Table 6 Satellite PV errors - propagation model errors taken into account

Typical Precision Orbit Determination (initial 3D Position RMS Error = 0.5 m and 3D Velocity RMS Error = 0.5 mm/s) allows Satellite position prediction 60 seconds ahead with max error of 1.47m and 5 minutes ahead with max error of 3.87m

## Satellite orbit format

Serving satellite ephemeris broadcast to UEs is required to enable autonomous adjustments of UL TA and frequency at UE side based on the satellite predicted position and velocity vectors. The satellite ephemeris format to be used was discussed in RAN1#104-bis-e and the following agreement was made:

Support serving-satellite ephemeris broadcast based on one or more of the following:

- Set 1: Satellite position and velocity state vectors:

  - position X,Y,Z in ECEF (m)

  - velocity VX,VY,VZ in ECEF (m/s)

- Set 2: At least the following parameters in orbital parameter ephemeris format:

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

    - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

- FFS: The field size for each parameter

- FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris

- FFS: Whether down-selection is needed or both sets are supported

## Serving satellite ephemeris Epoch time

The Following agreement was made in RAN1#106-e:

Agreement

Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame.

•	FFS: Whether this starting time is given by predefined rule or it is indicated by the Network

Further, the following agreement was made at RAN1#106-b:

Agreement:

Common TA Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame.

- FFS: Whether this starting time is given by predefined rule or it is indicated by the Network

  - Note: “implicitly known” means that UTC is not provided to define the Common TA epoch time.

As already mentioned our preference is to use the same epoch time for both serving satellite ephemeris and common TA parameters.

Therefore, similar to Proposal 5 given in section 2.2.3, we propose the following:

Proposal 12:

Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL sub-frame and/or frame by indication with SFN and the sub-frame number that the Serving satellite ephemeris data is valid for.

## Reference point for Serving satellite ephemeris Epoch time

We propose to use the same reference point for both serving satellite ephemeris epoch time and common TA epoch time. Our proposal w.r.t this issue is given in section 2.2.4.

## Serving satellite ephemeris format bit allocations

The required accuracy for the satellite ephemeris format shall be driven based on the prediction budget error on satellite position and velocity vectors and the foreseen  prediction time horizon. It can be further discussed once the timing and frequency synchronization requirements are agreed.

The signalling overhead introduced by satellite ephemeris broadcast shall be carefully quantified.

The bit allocation for each ephemeris parameter will be determined based on the time and frequency accuracy requirements yet-to-be-defined by RAN4 and their implication on satellite position and velocity accuracy.

The following Working assumption was made at RAN1#106-bis-e meeting:

Working assumption:

- Support serving satellite ephemeris format bit allocations for LEO/MEO/GEO based non-terrestrial access network.:

  - Position and velocity state vector ephemeris format [17 bytes payload].

    - The field size for position [m]  is [78 bits]

      - Position range is driven by GEO : +/- 42 200 km

      - The quantization step is [1.3m] for position

    - The field size for velocity [m/s] is [54 bits]

      - Velocity range is driven by LEO@600 km: +/- 8000 m/s

      - The quantization step is [0.06 m/s] for Velocity

  - Orbital parameter ephemeris format [18 byte payload]

    - Semi-major axis α [m] is [33 bits]

      - Range: [6500, 43000]km

    - Eccentricity e is [19 bits]

      - Range: ≤ 0.015

    - Argument of periapsis ω [rad] is [24 bits]

      - Range: [0, 2π]

    - Longitude of ascending node Ω [rad] is [21 bits]

      - Range: [-180o , +180o]

    - Inclination i [rad] is [20 bits]

      - Range: [-90o  , +90o ]

    - Mean anomaly M [rad] at epoch time to is [24 bits]

      - Range: [0, 2π]

- FFS: Additional enhancement to optimize the signalling overhead.

- FFS: Ephemeris format bit allocations for HAPS

Proposal 13:

Confirm the working assumption made at RAN1#106-bis-e on serving satellite ephemeris bit allocations for LEO/MEO/GEO based non-terrestrial access network.

## Orbit prediction error at UE

Depending on adopted ephemeris format; set 1 or set 2, different propagator models can be considered for implementation at UE side. Some have been developed for orbital elements and others for PV state vectors. To enhance the propagator model additional elements may be included in the ephemeris data.

RAN1made the following conclusion in RAN1#104-bis-e:

Conclusion

The orbital propagator model to be used at UE side can be left to implementation.

The adopted ephemeris format can strongly orientate the choice of model (e.g. TLE format and TLE propagation models).

It was observed based on simulation that the prediction time period before overshooting the delay and Doppler pre-compensation target is linked to the broadcast satellite format and to the propagation model used in the UE. As shown in the table below  prediction 45 s ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 45.93 Hz for Doppler error (at FR2) when Kepler model is used. This is sufficient to accommodate UE pre-compensation requirement when using 120 kHz SCS. Prediction 180 s ahead for UE pre-compensation is accurate within 1.74 µs for round trip delay error and within 17.60 Hz (in S band) for Doppler error when Kepler model is in use. This is sufficient to accommodate UE pre-compensation requirement when using 15 kHz SCS.

Using other propagation models in the UE will significantly enhance UE pre-compensation accuracy during a larger prediction time period. For example, by using analytical Eckstein-Hechler model, prediction 5 minutes ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 6.20 Hz (in FR2) for Doppler error.

It is worth noting that adequate fitting can be performed based on the reference orbit and the prediction time horizon considered to further enhance UE pre-compensation accuracy. Of course, this is possible only if the propagator model to be used at UE side is known by the network. But the orbital propagator model to be used at UE side can be left to implementation as per the conclusion made in RAN1#104-bis-e.

Table 7 Orbit propagation model comparison

|  |  |  | Maximum errors |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Without fitting | Model | Duration of prediction (minute) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
|  | Kepler | 3 | 261 | 1.74 | 2.64 | 17.60 |
|  | Eckstein Hechler (J6) | 21 | 303 | 2.02 | 0.48 | 3.20 |
|  | 2x2 num | 22,5 | 301.4 | 2.01 | 0.42 | 2.80 |
|  | 6x6 num | 27 | 302.5 | 2.02 | 0.44 | 2.93 |
|  |  |  | Maximum errors |  |  |  |
|  | Model | Duration of prediction (second) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
| Without fitting | Kepler | 45 | 43.4 | 0.29 | 0.689 | 45.93 |
|  | Eckstein Hechler (J6) | 300 | 43.4 | 0.29 | 0.093 | 6.20 |
|  | 2x2 num | 300 | 44.3 | 0.30 | 0.1 | 6.67 |
|  | 6x6 num | 350 | 44.2 | 0.29 | 0.084 | 5.60 |
| With fitting | TLE | 370 | 44,1 | 0.29 | 0.1 | 6.67 |
|  | Eckstein Hechler (J6) | 390 | 44,2 | 0.29 | 0.083 | 5.53 |
|  | 2x2 num | 375 | 44 | 0.29 | 0.092 | 6.13 |
|  | 6x6 num | 395 | 44,2 | 0.29 | 0.079 | 5.27 |

Based on simulation results, prediction 45 s ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 45.93 Hz for Doppler error (at FR2) when Kepler model is used. This is sufficient to accommodate UE pre-compensation requirement when using 120 kHz SCS

# Conclusion

In this contribution, we made the following observations and proposals:

- For better accuracy of self-estimated Common TA a second-order approximation is needed, thereby, both Common TA drift rate and Common TA drift variation rate need to be jointly broadcast by the Network.

Maximum Common Delay estimation error shall take into account the error on satellite position and the error of  quantization of common TA parameters broadcast in SIB.

The UE can estimate the Common one delay on the feeder link with a maximum error less than 0,084 μs  (= 2.6 ×Ts) assuming the UE acquires the Common TA related parameters once every 10 microseconds..

The residual timing inaccuracy due to common TA approximation can be managed by gradual timing adjustment using closed loop TA commands if necessary

If DL frequency compensation for the service link Doppler is supported, in case of Earth-fixed cell, the UE needs to frequently acquire the SIB to retrieve common pre-compensated FO parameters.

At least 19 bits are needed to indicate the amount of frequency compensation and associated drift rate.

One shall distinguish between orbit determination performance based on past measurements of the satellite trajectory and orbit prediction performance which concern the future satellite trajectory.

As a rule of thumb; it can be assumed that there is a factor of 1000 between the position error (in m) and the velocity error (in m/s). This is important to keep in mind when allocating an error budget for satellite position and velocity estimations.

The orbit prediction accuracy depends on:

  - -	The accuracy of the orbit determination used to derive the satellite ephemeris

  - -	The accuracy of the orbit propagation model

  - -	The time horizon over which the prediction is made

Even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably.

Typical Precision Orbit Determination (initial 3D Position RMS Error = 0.5 m and 3D Velocity RMS Error = 0.5 mm/s) allows Satellite position prediction 60 seconds ahead with max error of 1.47m and 5 minutes ahead with max error of 3.87m

Based on simulation results, prediction 45 s ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 45.93 Hz for Doppler error (at FR2) when Kepler model is used. This is sufficient to accommodate UE pre-compensation requirement when using 120 kHz SCS.

Proposal 1:

Higher-layer parameters TACommon, TACommonDrift and TACommonDriftVariation are jointly indicated if the Common TA include parameters indicating timing drift.

Proposal 2:

- In case of GEO based non-terrestrial access network:

  - In FR1:

    - Value range of TACommon  is : 0 - 16621439

    - TACommon bit allocation is: 24 bits

  - In FR2:

    - Value range of TACommon  is: 0 - 66485757

    - TACommon bit allocation is: 26 bits

- in case of LEO based non-terrestrial access network:

  - In FR1:

    - Value range of TACommon  is: 0 - 1282539

    - TACommon bit allocation is : 21 bits

  - In FR2:

    - Value range of TACommon  is: 0 - 5130157

    - TACommon bit allocation is: 23 bits

Proposal 3:

Higher-layer parameters TACommonDrift and TACommonDriftVariation are indicated:

In 14 bits and with value range : -24 microseconds/second … 24 microseconds/second, for TACommonDrift.

In 10 bits and with value range: -0,27 microseconds/second² … 0 microseconds/second², for TACommonDriftVariation.

Proposal 4:

Satellite Ephemeris data and Common TA related parameters are sent within the same SIB/SI.

The epoch time of serving satellite ephemeris implicitly known as a reference time defined by the starting time of a DL slot and/or frame is used as reference time for Common TA parameters.

Proposal 5:

The Epoch time of common TA parameters is implicitly known as a reference time defined by the starting time of a DL sub-frame and/or frame by indication with SFN and the sub-frame number that the common TA parameters are valid for.

Proposal 6:

Send LS to RAN2 asking RAN2 to include serving satellite ephemeris and common TA related parameters within the Minimum SI: either SIB1 or a specific NTN MSI.

Proposal 7:

The reference point for epoch time of Common TA parameters and the serving satellite ephemeris should be known by UE. This reference point is the satellite transmitter.

Proposal 8:

TAcommon may include a timing offset (.e.g. = + Te_NTN ) if considered necessary to avoid that the UE over pre-compensates its TA during RACH procedure.

Proposal 9:

-       NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay, which is calculated using the UE position and the serving satellite ephemeris. How the UE calculates/updates NTA,UE-specific is left to UE implementation.

-        NTA,commonis updated autonomously by the UE based on the Common TA parameters indicated by the Network as follows:

NTAcommont= NTA, t0common+   TA CommonDrift, t0×t-t0+12 TA CommonDriftVariation,t0×t-t02

Proposal 10:

A single validity timer for both serving satellite ephemeris and common TA related parameters is indicated using higher-layer parameter ntnUlSyncValidityDuration.

ntnUlSyncValidityDuration is indicated in 4 bits with 16 values: 5s, 10s, 15s, 20s, 25s, 30s, 35s, 40s, 45s, 50s, 55s, 60s, 120s, 180s, 240s, 300s.

Proposal 11   DL frequency compensation for the service link Doppler is not supported in Release 17.

Proposal 12:

Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL sub-frame and/or frame by indication with SFN and the sub-frame number that the Serving satellite ephemeris data is valid for.

Proposal 13:

Confirm the working assumption made at RAN1#106-bis-e on serving satellite ephemeris bit allocations for LEO/MEO/GEO based non-terrestrial access network.

# Bibliography

- Thales, FL Summary #6 on enhancements on UL time and frequency synchronization for NR NTN, R1-2110602, October  2021

- TR38.821 Solutions for NR to support non-terrestrial networks.

- TR38.213 Physical layer procedures for control.

- Thales, "RP-201256 Solutions for NR to support non-terrestrial networks (NTN)," RAN#88e, June 2020.

- R4-2115347, Reply LS on NTN UL time and frequency synchronization requirements