3GPP TSG RAN WG1 #107	R1-2111168

e-Meeting, 11 – 19 November, 2021

Agenda item:		7.2.11

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for PDCCH Blind Detection in CA

Document for:		Discussion and Decision

# Introduction

RAN1 received an LS from RAN2 [1] requesting clarifications on URLLC features regarding PDCCH Blind Detection in CA. In this contribution we present our views on the issue.

# Discussion

To support more than one combination in RAN2 signalling for FG 11-2c and FG 11-2g, RAN2 respectfully asks RAN1 to provide the feedback for the following questions:

1. How many combinations for FG 11-2c and FG 11-2g can be reported at most from RAN1 perspective?

2. Whether the “supported span arrangement for CA” should be reported for each of the combinations or reported only once for FG 11-2c?

Besides, RAN2 wonders whether more than one combination should be supported for FG 11-2e as well? If the answer is yes, how many combinations for FG 11-2e can be reported at most from RAN1 perspective?

In [1] RAN2 asked the following questions to RAN1 (detailed description of related FGs can be found in the Annex):

Indeed, it is clear from the descriptions of FG 11-2c, 11-2g and 11-2e that it must be possible to report combinations of Rel-15 and Rel-16 blind detection capabilities. However, it is not defined in UE feature group list or in specifications how many such combinations are possible, though the following restriction needs to be followed in 11-2c and 11-2g:

The minimum of the summation of capability on the number of CCs with Rel-15 PDCCH monitoring capability and the capability on the number of CCs with Rel-16 PDCCH monitoring capability is 3

In any case, it is clear that the Supported span arrangement for CA should be the same regardless of the combinations reported for the first component of FG 11-2c.

Hence, we propose the following:

Proposal 1: Confirm that it must be possible to report different combinations of Rel-15 and Rel-16 blind detection capabilities in FG 11-2c, 11-2g and 11-2e. The exact number of combinations is FFS.

Proposal 2: Confirm that “supported span arrangement for CA” is reported only once for FG 11-2c.

# Conclusion

In this contribution we have presented our views regarding the issues raised in RAN2 LS [1]. We make the following proposals:

Proposal 1: Confirm that it must be possible to report different combinations of Rel-15 and Rel-16 blind detection capabilities in FG 11-2c, 11-2g and 11-2e. The exact number of combinations is FFS.

Proposal 2: Confirm that “supported span arrangement for CA” is reported only once for FG 11-2c.

# References

- R2-2109168, ”LS on PDCCH Blind Detection in CA,” RAN2, Aug. 2021

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. NR_L1enh_URLLC | 11-2c | Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers | - Supported combination(s) of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) - Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15 - Supported span arrangement for CA - Candidate value for the component: {aligned spans only, aligned spans and non-aligned spans} | 11-2b | Yes | N/A |  | Per BC | N/A | N/A | N/A | The minimum of the summation of capability on the number of CCs with Rel-15 PDCCH monitoring capability and the capability on the number of CCs with Rel-16 PDCCH monitoring capability is 3 | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-2e | Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers | - Supported combination(s) of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) | 11-2b | Yes | N/A |  | Per BC | N/A | N/A | N/A | One combination of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) corresponds to one combination of (pdcch-BlindDetectionCA-r15, pdcch-BlindDetectionCA-r16) If the UE reports pdcch-BlindDetectionCA-r15, - Candidate values for pdcch-BlindDetectionMCG-UE-r15 is 0 to pdcch-BlindDetectionCA-r15 - Candidate values for pdcch-BlindDetectionSCG-UE-r15 is 0 to pdcch-BlindDetectionCA-r15 - pdcch-BlindDetectionMCG-UE-r15 + pdcch-BlindDetectionSCG-UE-r15>= pdcch-BlindDetectionCA-r15 Otherwise, if N_(NR-DC,max,r15)^(DL,cells) is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig-r16 = r15monitoringcapability - Candidate values for pdcch-BlindDetectionMCG-UE-r15 is [0, 1, 2] - Candidate values for pdcch-BlindDetectionSCG-UE-r15 is [0, 1, 2] - pdcch-BlindDetectionMCG-UE-r15 + pdcch-BlindDetectionSCG-UE-r15 >= N_(NR-DC,max,r15)^(DL,cells) If the UE reports pdcch-BlindDetectionCA-r16, - Candidate values for pdcch-BlindDetectionMCG-UE-r16 is 0 to pdcch-BlindDetectionCA-r16 - Candidate values for pdcch-BlindDetectionSCG-UE-r16 is 0 to pdcch-BlindDetectionCA-r16 - pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16>= pdcch-BlindDetectionCA-r16 Otherwise, if N_(NR-DC,max,r16)^(DL,cells) is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig-r16 = r16monitoringcapability - Candidate values for pdcch-BlindDetectionMCG-UE-r16 is [0, 1] - Candidate values for pdcch-BlindDetectionSCG-UE-r16 is [0, 1] - pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= N_(NR-DC,max,r16)^(DL,cells) Note: If a UE supports FG 11-2c or FG 11-2g, then the capability defined by FG 11-2c or FG 11-2g is applied to FG 11-2e. | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-2g | Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers with restriction for non-aligned span case | - Supported combination(s) of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) - Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15 - UE supports aligned span and non-aligned span In case of non-aligned span when the configured number of cells with Rel-16 PDCCH monitoring is larger than the UE reported value, PDCCH monitoring occasion(s) should be configured only on same symbol(s) every slot | 11-2b | Yes | N/A |  | Per BC | N/A | N/A | N/A | The minimum of the summation of capability on the number of CCs with Rel-15 PDCCH monitoring capability and the capability on the number of CCs with Rel-16 PDCCH monitoring capability is 3 | Optional with capability signalling |