3GPP TSG RAN WG1 #107-e																			R1-2111532

e-Meeting, November 11th – 19th, 2021

Source: 	Intel Corporation

Title:	Discussion on UE features for NR coverage enhancement

Agenda item:	8.16.8

Document for:	Discussion and Decision

# Introduction

At the RAN1#106b-e meeting, the following agreements were made regarding UE features for NR coverage enhancement [1].

Agreement

FG 30-5 is kept as “Slot based dynamic PUCCH repetition indication” as follows.

| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support dynamic PUCCH repetition indication | [4-23] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 30-5 is supported as optional with capability signalling.

In the contribution, we discuss UE features for NR coverage enhancement. The updated UE features for NR coverage enhancement were captured in [2] and are listed in the Appendix.

# Discussion on UE features for NR coverage enhancement

## UE features for PUSCH repetition type A enhancement

As listed in the Appendix, UE feature groups 30-1 and 30-1a are included for increased maximum number of repetitions for PUSCH repetition type A enhancement. Note that as agreed in RAN1#104-e meeting, the maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH [3]. Following this agreement, it may be more appropriate to further divide the UE feature group for CG-PUSCH for the increased maximum number of repetitions. The same principle can also apply for the PUSCH repetition type A based on available slots.

Based on the discussions above, Table 1 illustrates suggested update for UE feature groups for PUSCH repetition type A enhancement.

Table 1. UE feature groups for PUSCH repetition type A enhancement

| Index | Feature group | Components |
| --- | --- | --- |
| 30-1 | Increased maximum number of PUSCH Type A repetitions for DG-PUSCH | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. |
| 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions for CG-PUSCH | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration. |
| 30-2 | PUSCH Type A repetitions based on available slots for DG-PUSCH | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. |
| 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots for CG-PUSCH | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. |

Proposal 1

- For PUSCH repetition type A enhancement, divide UE feature groups into DG-PUSCH and CG-PUSCH.

- Consider Table 1 as a starting point for discussion of feature groups for PUSCH repetition type A enhancement.

## UE features for TBoMS

As agreed in the RAN1#106-e meeting, TB processing over multiple slots (TBoMS) is supported for both configured grant and dynamic grant [4]. In this case, it is more appropriate to differentiate DG-PUSCH and CG-PUSCH for the UE feature group. Table 2 illustrates suggested update for UE feature groups for TBoMS.

Table 2. UE feature groups for TBoMS

| Index | Feature group | Components |
| --- | --- | --- |
| 30-3 | TB processing over multi-slot PUSCH for DG-PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode for DG-PUSCH. |
| 30-3a | TB processing over multi-slot PUSCH for CG-PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode for CG-PUSCH. |

Proposal 2

- For TBoMS, divide UE feature groups into DG-PUSCH and CG-PUSCH.

- Consider Table 2 as a starting point for discussion of feature groups for TBoMS.

## UE features for DMRS bundling for PUSCH and PUCCH

In the previous RAN1 meetings [3][4][5], the following use cases were agreed to be supported for joint channel estimation of PUSCH:

- back-to-back PUSCH transmissions (of the same TB) for repetition type A and B scheduled by dynamic grant or configured grant

- back-to-back PUSCH transmissions for TB processing over multiple slots

- non-back-to-back PUSCH transmissions across consecutive slots (of the same TB) for repetition type A and type B scheduled by dynamic grant or configured grant.

Considering different use cases as mentioned above, it is more appropriate to divide the UE feature groups for back to back PUSCH transmission and non-back to back PUSCH transmission.

At the RAN1#106-e meeting, it was agreed that for DMRS bundling for PUCCH repetitions, RAN1 at least prioritize use cases 3 and 4a in R1-2104119 [4]. Hence, the same principle can also apply for DMRS bundling for PUCCH repetitions, i.e., UE feature groups can be divided into back to back PUCCH transmission and non-back to back PUCCH transmission.

Note that in RAN4 reply LS [6], certain conditions need to be met in order to maintain phase continuity in case of other UL signals/channels in the gap between repetitions as follows:

RAN4 has agreed for the case of other signals/channels in the gap between repetitions, it is not considered for UE to transmit other channels in the gap with different settings. For the case of other UL signals/channels in the gap between repetitions with same settings, as communicated in R4-2105417, RAN4 has further refined the conditions when phase continuity can be met as follows: - Signals/channels with repetitions and other UL signals/channels in the gap have the same: - PAPR and average power, e.g., PUSCH/PUCCH part of repetitions and SRS has same PAPR and average power. - Allocated number and locations of PRBs transmitted - Antenna port settings

Hence, addition UE feature group may need to be added for the case when other UL signals/channels are inserted in the gap between repetitions with same setting.

Proposal 3

- For UE features for DMRS bundling, consider back to back PUSCH/PUCCH transmission and non-back to back PUSCH/PUCCH transmission.

- For non-back to back PUSCH/PUCCH transmissions, add UE feature group for the case when other UL signals/channels are inserted in the gap between repetitions with same setting.

## UE features for Msg3 PUSCH repetition

For Msg3 PUSCH repetition, it is supported for RRC IDLE/INACTIVE mode UEs and can also be supported for RRC CONNECTED mode UEs. Table 3 illustrates suggested update for UE feature groups for Msg3 PUSCH repetition.

Table 3. UE feature group for Msg3 PUSCH repetition

| Index | Feature group | Components |
| --- | --- | --- |
| 30-6 | Msg3 repetition | Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. |

Proposal 4

- For UE features for Msg3 PUSCH repetition, support Msg3 repetition for initial transmission and re-transmission in RRC idle/inactive and connected mode.

# Conclusions

In this contribution, we discussed UE features for NR coverage enhancement. Further, we summarize the proposals as follows:

Proposal 1

- For PUSCH repetition type A enhancement, divide UE feature groups into DG-PUSCH and CG-PUSCH.

- Consider Table 1 as a starting point for discussion of feature groups for PUSCH repetition type A enhancement.

Proposal 2

- For TBoMS, divide UE feature groups into DG-PUSCH and CG-PUSCH.

- Consider Table 2 as a starting point for discussion of feature groups for TBoMS.

Proposal 3

- For UE features for DMRS bundling, consider back to back PUSCH/PUCCH transmission and non-back to back PUSCH/PUCCH transmission.

- For non-back to back PUSCH/PUCCH transmissions, add UE feature group for the case when other UL signals/channels are inserted in the gap between repetitions with same setting.

Proposal 4

- For UE features for Msg3 PUSCH repetition, support Msg3 repetition for initial transmission and re-transmission in RRC idle/inactive and connected mode.

# References

- Chairman’s notes, RAN1 #106b e-Meeting, October 2021

- R1-2110587, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e”, October 2021

- Chairman’s notes, RAN1 #104 e-Meeting, January 2021

- Chairman’s notes, RAN1 #106 e-Meeting, August 2021

- Chairman’s notes, RAN1 #105 e-Meeting, May 2021

- R4-2114991, “Reply LS on PUCCH and PUSCH transmissions”, e-Meeting, August 2021

# Appendix:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration. | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-17] | Yes | N/A | UE does not support PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4e | [Inter-slot frequency hopping with inter-slot bundling for PUSCH] | Support inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support dynamic PUCCH repetition indication | [4-23] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Msg3 repetition | Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. |  | Yes | N/A | UE does not support Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |