3GPP TSG RAN WG1 #107-e	R1-2111533

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.10

Source: 	Intel Corporation

Title: 	UE features for IAB

Document for:	Discussion and decision

# 1   Introduction

In this contribution, based on RAN1#106bis-e discussion on UE features for IAB [1], we further provide our views on the updated RAN1 IAB UE features list.

# 2   FG 31-2: IAB-DU Beam Restriction Indication

In this FG 31-2, the description and consequence columns are not consistent, the FG column is based on “IAB-node <-> Child IAB-node” link and the consequence column is based on “parent-node <-> IAB-node” link as below.

| Index | Feature group | Components | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- |
| 31-2 | [Child IAB-DU beam restriction indication] | Support [DU Beam Indication] reception | Parent-node cannot indicate restricted beams at the IAB-DU. |

The following changes are suggested.

| Index | Feature group | Components | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- |
| 31-2 | [IAB-DU beam restriction indication] | Support [restricted DU Beam Indication] reception | Parent-node cannot indicate restricted beams at the IAB-DU. |

Proposal 1: Revise FG 31-2 as suggested.

# 3   FG 31-3: IAB-MT Beam Recommendation Indication

In this FG 31-3, the description and consequence columns are not consistent, the FG column is based on “IAB-node <-> Child IAB-node” link and the consequence column is based on “parent-node <-> IAB-node” link as below.

| Index | Feature group | Components | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- |
| 31-3 | [Child IAB-MT beam recommendation indication] | Support [MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node |

The following changes are suggested.

| Index | Feature group | Components | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- |
| 31-3 | [IAB-MT beam recommendation indication] | Support [recommended MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node |

Proposal 2: Revise FG 31-3 as suggested.

# 4   FG 31-4: Case 6 Timing Alignment Indication

This FG 31-4 should be related to RAN1#106e agreement:

- An IAB-node is explicitly indicated by the parent node when Case 6 timing is performed at the IAB-node at least for specific time resources.

However, current FG description does not clearly reflect the signaling reception from parent-node to indicate Case#6 timing operation. The following modifications are suggested.

| Index | Feature group | Components | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- |
| 31-4 | [Case 6 timing alignment indication] | Support [Case 6 timing alignment indication reception ] | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. |

Proposal 3: Revise FG 31-4 as suggested.

# 5   FG 31-5: IAB-MT Beam Restriction Indication

This FG 31-5 should be related to RAN1#106e-RAN1#106bis-e agreements:

- For Case 7 timing at a parent node, the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node.

- Case 7 UL timing offset is indicated by the parent-node via MAC-CE.

However, current FG 31-5 description does not clearly reflect the Case#7 offset reception from parent-node. The following modifications are suggested.

| Index | Feature group | Components | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- |
| 31-5 | Case 7 timing offset | Support [Case7 timing offset reception | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. |

Proposal 4: Revise FG 31-5 as suggested.

# 3   Conclusion

In this contribution, we discussed feature groups regarding Rel-17 NR UE features for IAB.  It is summarized by the following proposals.

Proposal 1: Revise FG 31-2 as suggested.

Proposal 2: Revise FG 31-3 as suggested.

Proposal 3: Revise FG 31-4 as suggested.

Proposal 4: Revise FG 31-5 as suggested.

# 4   References

[1] R1-2110587, “Updated RAN1 UE features list for Rel-17 NR after RAN1#106bis-e”, RAN1#106bis-e, October 2021.

# NR_IAB_enh in [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support [Rel-17 DesiredGuardSymbols] reporting 2) Support [Rel-17 ProvidedGuardSymbols] reception |  | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact Note: If an IAB node does not support a certain timing mode, the reported/provided values shall be ignored | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-2 | [Child IAB-DU beam restriction indication] | Support [DU Beam Indication] reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-3 | [Child IAB-MT beam recommendation indication] | Support [MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam |  | yes | N/A | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-4 | [Case 6 timing alignment] | Support [Case 6 transmission] |  | Yes | N/A | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-5 | Case 7 timing [alignment] | Support [Case7 transmission] |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | 1.) Support [Desired DL TX Power Adjustment] reporting 2.) Support [DL TX Power Adjustment] reception |  | Yes | N/A | Parent-node’s DL TX power adjustment reporting and reception is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-7 | [Desired] UL TX power adjustment | Support [Desired IAB-MT PSD range] reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | [31-8] | [Dynamic indication of Rel-17 or FDM soft resource availability] | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | The IAB-node is unable to receive explicit availability indication for Soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-9 | Simultaneous transmission and reception from multiple parent nodes | Support simultaneous transmission and reception from multiple parent nodes |  | Yes | N/A | Simultaneous transmission and reception is not supported in DC scenario | per BC | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |