3GPP TSG RAN WG1 #107-e                                                       R1-2111560

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.16.1

Source:	Xiaomi

Title:	Discussion on Rel-17 FeMIMO UE features

Document for:	Discussion

- Introduction

- In this contribution, we discuss the UE features for feMIMO uplink related.

2	Discussion on UE feature

2.1    UE features related to enhancements on Multi-TRP PUSCH&PUCCH

The following is proposed as baseline for RAN1 #107-e as the outcome of RAN1 #106bis-e for this agenda item/work item [1].

Table 1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [6. support of sequential mapping for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] |  | Yes |  |  | Per Band | No | No |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable |  | Yes |  |  | Per Band | No | No |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 |  | Yes |  |  | Per Band | No | No |  |  | Optional with capability signalling |

We have some comments on the UE features for enhancement on multi-TRP PUSCH&PUCCH.

PUSCH:

- Components 2-5 in FG23-3-1should split into separate FGs.

- Components 6-11 can be removed or declared without component-level signalling

- Component 7 is not according to any agreement so far

PUCCH

- Components 1-4 in FG23-3-2 should split into separate FGs.

Proposal 1: Suggest to consider the following for the updated Table 1.

PUSCH:

- Components 2-5 in FG23-3-1should split into separate FGs.

- Components 6-11 can be removed or declared without component-level signalling

- Component 7 is not according to any agreement so far

PUCCH

- Components 1-4 in FG23-3-2 should split into separate FGs.

2.2 UE features related to enhancements on SRS

The following is proposed as baseline for RAN1 #107-e as the outcome of RAN1 #106bis-e for this agenda item/work item [1].

Table 2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] |  | Yes |  |  | Per FS | TDD only | No |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI |  | Yes |  |  | Per FS | No | No |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 [2. Report whether the uplink TX switching impact to downlink receiving in a band] [3. Report whether the UL Tx is switched together with UL Tx in another band] |  | Yes |  |  | Per BC | No | No |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP [and 1 periodic] SRS sets for antenna switching | Support of maximum [2] [SP] SRS resource sets [and maximum 1 periodic SRS resource set] for antenna switching |  | Yes |  |  | Per BC | No | No |  | [Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource |  | Yes |  |  | Per FS | No | No |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial [(RB level)] frequency sounding for SRS |  | Yes |  |  | Per Band | No | No |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial [(RB level)] frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS] | [23-8-6] | Yes |  |  | Per Band | No | No |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  | Yes |  |  | Per Band | No | No |  |  | Optional with capability signalling |

Proposal 2：Agree in principle with the updated functionalities in Table 2.

Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: Suggest to consider the following for the updated Table 1.

PUSCH:

- Components 2-5 in FG23-3-1should split into separate FGs.

- Components 6-11 can be removed or declared without component-level signalling

- Component 7 is not according to any agreement so far

PUCCH

- Components 1-4 in FG23-3-2 should split into separate FGs.

Proposal 2：Agree in principle with the updated functionalities in Table 2.

References

- R1-2110587 Updated  RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, RAN1#106biis-e, October 2021

- 3GPP TS 38.306 V16.6.0, NR;User Equipment (UE) radio access capabilities, RAN#93e, September 2021