3GPP TSG RAN WG1 Meeting #107-e	R1-2111928

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.2.7

Source:	Huawei, HiSilicon

Title:	Additional SLS results on channel access

Document for:	Discussion and decision

- Introduction

In the SI of the channel access mechanism for operation in frequency from 52.6GHz to 71GHz, many companies provided the SLS results based on the assumption in TR38.808 [1] under indoor and outdoor scenario. In this document, some new simulation results on the use of quasi-omni, directional LBT, and without LBT using different antenna size to achieve different antenna gain are provided. Simulation results of coexistence with WiGig are also provided.

- Discussion

LBT with a sensing beam covering all subsequent transmission beams is known as quasi-omni-directional LBT. This mechanism is used in IEEE 802.11ad DMG systems and can be introduced in the NR-U system for operation in the same 60GHz band. From an implementation point of view, quasi-omni-directional sensing is easy to implement and can simplify the system design especially when gNB serves multiple UEs in different directions. Omni-directional LBT is also the typical channel access mechanism adopted by the technologies in sub-7 GHz such as 802.11ac/ax/LAA/NR-U whereby the energy collected by each antenna element is averaged out. It has the merit of improving the probability of successful channel access and enhancing the spatial reuse.

Previous results with high antenna gain

In the earlier contribution [2], we found that the performance gain between (omni)-directional LBT and no-LBT in high traffic load is about 2~3% in median and more than 10% in cell-edge for DL. For UL, the gain is very small. From analysing the RSSI CDF curve (Fig. 2 showed in the appendix), we could also find that very little interference energy was above the EDT (-47dBm), and it thus had no impact on CCA measurement. The results from the previous contributions are repeated here for convenience.

Table 1. Performance of various channel access type at high traffic load (64 antenna elements for gNB, 8 antenna elements for UE) [2]

| Channel access type/User throughput(Mbps) | DL mean UPT | UL mean UPT | DL 5% UPT | UL 5% UPT | Gain of DL mean UPT w.r.t No-LBT | Gain of UL mean UPT w.r.t No-LBT | Gain of 5% DL UPT w.r.t No-LBT | Gain of 5% UL UPT w.r.t No-LBT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No-LBT | 2388.7 | 1521.8 | 85.7 | 86.6 |  |  |  |  |
| Directional LBT | 2446.7 | 1521.1 | 101.3 | 82.3 | 2% | 0% | 18% | -5% |
| Omni-dir LBT | 2452.9 | 1538 | 95.9 | 89.8 | 3% | 1% | 12% | 4% |

New results with lower antenna gain

In this section, we provide results with lower antenna gain. Beamforming gain of 14 dBi is adopted in the simulations and it is normalised in the EDT.

Table 2. Parameter settings

| Buffer occupancy | ≈20%， 45%， 70% |
| --- | --- |
| gNB TX power at antenna ports | 26dBm |
| gNB antenna panel configuration | 2x4 elements, 5 dBi element gain |
| gNB TX power EIRP | 40dBm |
| UE TX power at antenna ports | 14dBm |
| UE antenna panel configuration | 2x2 elements, 5 dBi element gain |
| UE TX power EIRP | 25dBm |
| Traffic model | 50:50 DL:UL split; 100: 0 DL:UL split |

Low, medium and high load cases were analysed for no-LBT, omni-directional LBT and directional LBT. In most cases the use of LBT provides better performance than No-LBT, although in low load cases the difference is small. In medium load cases the performance gain is about 10% for mixed traffic whereas a 40% gain can be seen in cell-edge performance for mixed traffic load.

Table 3. DL median UPT (Mbps) simulation results，DL：UL=50:50

|  | Directional LBT | No LBT | Omni LBT | Directional LBT (low EDT) | Omni LBT (low EDT) |
| --- | --- | --- | --- | --- | --- |
| Low load | 4453(0.76%) | 4420(0) | 4437(0.39%) | 4422(0.05%) | 4454(0.78%) |
| Medium load | 2483(7.28%) | 2314(0) | 2493(7.72%) | 2505(8.24%) | 2474(6.92%) |
| High load | 1205(8.55%) | 1110(0) | 1215(9.41%) | 1198(7.92%) | 1227(10.49%) |

Table 4. DL 10 percentile UPT (Mbps) simulation results，DL：UL=50:50

|  | Directional LBT | No LBT | Omni LBT | Directional LBT (low EDT) | Omni LBT (low EDT) |
| --- | --- | --- | --- | --- | --- |
| Low load | 1487(4.32%) | 1426(0) | 1488(4.39%) | 1481(3.91%) | 1493(4.8%) |
| Medium load | 642(12.63%) | 570(0) | 638(11.93%) | 635(11.51%) | 650(14.18%) |
| High load | 139(32.46%) | 105(0) | 144(37.35%) | 155(48.37%) | 154(46.97%) |

Table 5. UL median UPT (Mbps) simulation results，DL：UL=50:50

|  | Directional LBT | No LBT | Omni LBT | Directional LBT (low EDT) | Omni LBT (low EDT) |
| --- | --- | --- | --- | --- | --- |
| Low load | 1441(-1.36%) | 1461(0) | 1441(-1.39%) | 1448(-0.88%) | 1442(-1.26%) |
| Medium load | 1030(4.34%) | 987(0) | 1040(5.33%) | 1034(4.7%) | 1035(4.86%) |
| High load | 524(13.99%) | 460(0) | 523(13.76%) | 512(11.45%) | 526(14.39%) |

Table 6. UL 10 percentile UPT (Mbps) simulation results，DL：UL=50:50

|  | Directional LBT | No LBT | Omni LBT | Directional LBT (low EDT) | Omni LBT (low EDT) |
| --- | --- | --- | --- | --- | --- |
| Low load | 506(1%) | 501(0) | 505(0.83%) | 511(1.99%) | 513(2.42%) |
| Medium load | 212(15.96%) | 183(0) | 218(18.75%) | 211(15.34%) | 213(16.22%) |
| High load | 0 | 0 | 0 | 0 | 0 |

For DL only traffic model, the cell-edge gain becomes smaller, about 20%. Lowering the EDT for directional LBT has an impact on the LBT performance.

Table 7. DL median UPT (Mbps) simulation results，DL：UL=100：0

|  | Directional LBT | No LBT | Omni LBT | Directional LBT (low EDT) | Omni LBT (low EDT) |
| --- | --- | --- | --- | --- | --- |
| Low load | 4350(0.44%) | 4331(0) | 4341(0.23%) | 4322(-0.19%) | 4332(0.02%) |
| Medium load | 3099(1.47%) | 3054(0) | 3084(1%) | 2997(-1.85%) | 3037(-0.54%) |
| High load | 1799(4.54%) | 1721(0) | 1769(2.78%) | 1669(-3.05%) | 1720(-0.07%) |

Table 8. DL 10 percentile UPT (Mbps) simulation results，DL：UL=100: 0

|  | Directional LBT | No LBT | Omni LBT | Directional LBT (low EDT) | Omni LBT (low EDT) |
| --- | --- | --- | --- | --- | --- |
| Low load | 1438(0.47%) | 1138(0) | 1105(-2.99%) | 1090(-4.26%) | 1104(-3.04%) |
| Medium load | 147(15.82%) | 127(0) | 145(14.08%) | 122(-4.09%) | 133(4.65%) |
| High load | 0 | 0 | 0 | 0 | 0 |

Observation 1: LBT is benefit to better performance gain with lower antenna gain in 60GHz band

Coexistence between NR-U and WiGig networks

The simulation setup is the same as before, the only difference is that one network is replaced by a WiFi system.

Three cases are evaluated:

Case 1: 	WiFi (network 1) co WiFi (network 2);

Case 2: 	NRU (Omni LBT, network 1) co WiFi (network 2);

Case 3: 	NRU (No LBT, network 1) co WiFi (network 2)

Table 9. Parameter settings

| Buffer occupancy | ≈20%， 45%， 70% |
| --- | --- |
| gNB TX power at antenna ports | 26dBm |
| gNB antenna panel configuration | 2x4 elements, 5 dBi element gain |
| gNB TX power EIRP | 40dBm |
| UE TX power at antenna ports | 14dBm |
| UE antenna panel configuration | 2x2 elements, 5 dBi element gain |
| UE TX power EIRP | 25dBm |
| Traffic model | 50:50 DL:UL split |
| Network coexistence model | - WiFi (Network1) & WiFi (Network2) - NR-U with LBT (Network1) & WiFi (Network2) - NR-U without LBT (Network1) & WiFi (Network2) |
|  | WiFi:（CCA_threshold：-48dBm，PD_threshold：-68 dBm） NR-U with LBT:（CCA_threshold：-47dBm, Omni LBT） |

WiFi-to-WiFi coexistence case is the baseline on which comparison are being made. From the results, we can find that when one network is replaced by NR-U with LBT enabled, both NR-U and WiFi systems’ performance is better than before. For WiFi network, we observe 5%~15% gain in DL and 7%~50% in UL for median UPT. If LBT is disabled for NR-U, the NR-U performance is a little better (less than 1% ) than LBT case in low traffic load but worse ( about -5%) in medium and high traffic load. Meanwhile, the WiFi system’s performance becomes worse (-5~-25%) than before. Based on these results, we can draw a conclusion that LBT in NR-U is improving the performance of both networks in WiFi coexistence scenarios.

Figure 1. UPT performance with different network coexistence mode in indoor scenario-A，DL：UL=50:50

Table 10. DL median UPT，DL：UL=50:50

|  | Network1 (WiFi & WiFi) | Network1 (NR-U with LBT & WiFi) | Network1 (NR-U w/o LBT & WiFi) | Network2 (WiFi & WiFi) | Network2 (NR-U with LBT & WiFi) | Network2 (NR-U w/o LBT & WiFi) |
| --- | --- | --- | --- | --- | --- | --- |
| Low load | 4352 | 4542(0) | 4577(0.78%) | 4021(0) | 4235(5.32%) | 4216(4.84%) |
| Medium load | 2026 | 2554(0) | 2408(-5.72%) | 2024(0) | 2309(14.05%) | 2179(7.62%) |
| High load | 1083 | 1281(0) | 1220(-4.76%) | 1002(0) | 1123(12.09%) | 1043(4.13%) |

Table 11. DL 10 percentile UPT，DL：UL=50:50

|  | Network1 (WiFi & WiFi) | Network1 (NR-U with LBT & WiFi) | Network1 (NR-U w/o LBT & WiFi) | Network2 (WiFi & WiFi) | Network2 (NR-U with LBT & WiFi) | Network2 (NR-U w/o LBT & WiFi) |
| --- | --- | --- | --- | --- | --- | --- |
| Low load | 1392 | 1491(0) | 1456(-2.35%) | 1326(0) | 1391(4.90%) | 1378(3.96%) |
| Medium load | 575 | 663(0) | 645(-2.71%) | 538(0) | 581(7.98%) | 552(2.52%) |
| High load | 144 | 169(0) | 152(-10.06%) | 111(0) | 131(17.86%) | 102(-8.52%) |

Table 12. UL median UPT，DL：UL=50:50

|  | Network1 (WiFi & WiFi) | Network1 (NR-U with LBT & WiFi) | Network1 (NR-U w/o LBT & WiFi) | Network2 (WiFi & WiFi) | Network2 (NR-U with LBT & WiFi) | Network2 (NR-U w/o LBT & WiFi) |
| --- | --- | --- | --- | --- | --- | --- |
| Low load | 1326 | 1496(0) | 1511(1%) | 1286(0) | 1378(7.11%) | 1354(5.27%) |
| Medium load | 872 | 1099(0) | 1114(1.36%) | 765(0) | 904(18.17%) | 873(14.13%) |
| High load | 327 | 624(0) | 612(-1.92%) | 238(0) | 354(49.03%) | 321(34.93%) |

Table 13. UL 10 percentile UPT，DL：UL=50:50

|  | Network1 (WiFi & WiFi) | Network1 (NR-U with LBT & WiFi) | Network1 (NR-U w/o LBT & WiFi) | Network2 (WiFi & WiFi) | Network2 (NR-U with LBT & WiFi) | Network2 (NR-U w/o LBT & WiFi) |
| --- | --- | --- | --- | --- | --- | --- |
| Low load | 459 | 512(0) | 530(3.52%) | 451(0) | 495(9.75%) | 488(8.15%) |
| Medium load | 139 | 225(0) | 226(0.44%) | 108(0) | 173(59.7%) | 148(36.94%) |
| High load | 0 | 0 | 0 | 0 | 0 | 0 |

Simulations show that LBT has clear benefits compared with no LBT. Details below:

- Table 3 and 5 show around 10% gain in median cell throughput in both UL and DL

- Table 4 shows 30-48% DL gain under high load at 10th percentile throughput or near cell edge

- Table 6 shows 15-18% UL gain under medium load at 10th percentile throughput or near cell edge

- Tables 10- 13 show the clear benefit LBT brings to NR-U - WiFi coexistence

Observation 2: LBT is necessary for NR-U-60 in coexistence with WiGig system in 60GHz band

- Conclusions

In this document, we provided the simulation results with lower antenna gain showing that the importance of LBT in achieving better performance for NR-U coexist NRU/WiGig system than No-LBT with the observations as follows:

Observation 1: LBT is benefit to better performance gain with lower antenna gain in 60GHz band

Observation 2: LBT is necessary for NR-U-60 in coexistence with WiGig system in 60GHz band

References

TR38.808 v17.0.0 Study on supporting 52.6 GHz to 71GHz.

[2] R1-2009610 "Link level and System level evaluation for NR system operating in 52.6GHz to 71GHz" Huawei, HiSilicon.

Appendix

Figure 2. RSSI in LBT simulations for indoor scenario