3GPP TSG RAN WG1 Meeting #107-e	R1-2112063

Online, 11 – 19 November, 2021

Agenda Item:	8.12.1

Source: 	LG Electronics Inc.

Title: 	Support of group scheduling for RRC_CONNECTED UEs

Document for:	Discussion and decision

# Introduction

The WID on NR Multicast and Broadcast Services was revised in RAN#88e [1].

One of the objectives led by RAN1 is to specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state:

- Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

  - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

- Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

In addition, RAN1 is involved in specification of RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states [RAN2, RAN1]:

- Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception. [RAN2, RAN1].

  - Note: the possibility of receiving Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, without the need for those UEs to get the configuration of the PTM bearer carrying the Broadcast/Multicast service while in RRC CONNECTED state beforehand, is subject to verification of service subscription and authorization assumptions during the WI.

In this contribution, we discuss how to support a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service.

# CFR

As discussed in RAN1#105e, RAN1 assumes that Option 2B for CFR associated with UE active BWP other than initial BWP is supported at least for multicast of RRC-CONNECTED UEs.

- FFS: CFR associated with initial BWP

- FFS: CFR larger than initial BWP

Considering that UE in RRC_CONNECTED can use the initial BWP in some cases since Rel-15 NR, we think that CFR associated with the initial BWP can be supported. However, we concern that CFR equal to or smaller than the initial DL BWP would lead to low multicast capacity in CFR considering the initial DL BWP is consistently serving system information, paging, initial access from several UEs and additionally new broadcast support for MBS, and furthermore potentially cause overload in initial DL BWP together with broadcast. Thus, it would be beneficial to support possibility of configuring a wider CFR than initial DL BWP for a connected UE receiving multicast. Wider CFR would be also beneficial for idle and inactive UEs receiving broadcast.

Proposal 1: For a connected UE receiving multicast (as well as idle/inactive UEs receiving broadcast), CFR associated to initial DL BWP can be configured with a wider bandwidth than the initial DL BWP or a bandwidth equal to or smaller than the initial DL BWP.

Meanwhile, a connected UE can change unicast BWP switching e.g. from UE’s active BWP#1 to UE’s active BWP#2. Assuming that separate CFRs are associated to different unicast BWPs, unicast BWP switching should trigger change of CFR from CFR#1 associated to UE’s active BWP#1 to CFR#2 associated to UE’s active BWP#2 (whichever CFR option is agreed).

Proposal 2: At least for multicast, unicast BWP switching between UE’s active BWPs may immediately triggers CFR change between different CFRs associated to different UE’s active BWPs.

In addition, a certain CFR could be confined within more than one UE active BWP with a same numerology. If it is the case, upon unicast BWP switching, UE would not need to change CFR because CFR is associated with both unicast BWPs. UE could maintain CFR configuration and continue to receive PTM/PTP (re-)transmissions on the CFR during/after unicast BWP switching.

Proposal 3: If a CFR is confined within more than one UE active BWP with a same numerology, the CFR can be associated to more than one BWP.

- Upon unicast BWP switching between UE’s active BWPs associated to the same CFR, UE does not change CFR and continues to receive PTM/PTP (re-)transmissions on the CFR during/after unicast BWP switching.

For a connected UE receiving broadcast, one CFR of a cell should be associated at least to initial DL BWP of the cell. When connected UE receiving multicast switches to the initial DL BWP, UE would be expected to receive multicast on CFR of the initial DL BWP. If this UE is also receiving broadcast, it would be beneficial that this UE could receive both multicast and broadcast on the same CFR associated to the initial DL BWP.

Meanwhile, it is not clear how a connected UE activating UE’s active BWP different than the initial DL BWP can receive broadcast. One option is for gNB to transmit broadcast as well as multicast on a CFR associated to UE’s active BWP.

Proposal 4: For broadcast, CFR of a cell is associated at least to initial DL BWP of the cell for any RRC state.

- FFS whether broadcast CFR is associated to UE’s active DL BWP for UE in RRC_CONNECTED.

# CORESET/SS

In RAN1#104bis-e, RAN1 discussed the following proposal without any agreement:

For RRC_CONNECTED multicast UEs supporting CA capability, support the following principles for determining MPDCCHtotal,slot,μ / CPDCCHtotal,slot,μ  and the maximum numbers of BD/CCE UE is required to monitor per slot for a serving cell supporting multicast reception:

- When determining  MPDCCHtotal,slot,μ / CPDCCHtotal,slot,μ  defined in 38.213, the number of DL serving cell(s) supporting multicast reception is increased as R times.

- The maximum BD/CCE numbers are increased as R times MPDCCHmx,slot,μ and R times CPDCCHmax,slot,μ for a serving cell supporting multicast reception, where MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are defined in Table 10.1-2 and Table 10.1-3 in 38.213

- R is a value reported by the UE

To our understanding, the maximum BD/CCE numbers can be increased as R times MPDCCHmx,slot,μ and R times CPDCCHmax,slot,μ for a serving cell supporting multicast reception. However, we think that R is not related to CA capability. For RRC_CONNECTED UE receiving multicast, R can be a value reported by the UE as part of MBS related UE capability, regardless of whether UE supports CA capability

Proposal 5: The maximum BD/CCE numbers are increased as R times MPDCCHmx,slot,μ and R times CPDCCHmax,slot,μ for a serving cell supporting multicast reception, where MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are defined in Table 10.1-2 and Table 10.1-3 in 38.213

  - R is a value reported by the UE as part of MBS related UE capability, regardless of whether UE supports CA capability.

In RAN1#104-e, RAN1 agreed that for broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs when UE-specific active BWP of RRC_CONNECTED UE contains the common frequency resource of RRC_IDLE/INACTIVE UEs and the SCS and CP are the same. Meanwhile, RAN1 previously agreed that for RRC_IDLE/RRC_INACTIVE UEs, beam sweeping is supported for group-common PDCCH/PDSCH. Thus, in our view, assuming that gNB would broadcast multiple group common PDSCH transmissions associated to different SSBs for a same TB for broadcast for idle/inactive UEs, connected UEs interested to receive the broadcast service are expected to selectively receive one or more of the group-common PDSCH transmissions associated to a selected SSB for broadcast.

On the other hand, even for multicast, gNB may need to transmit multiple TDMed group-common PDSCHs of a same TB to connected UEs in a same group with selectively different RSs. In this case, we think that for both broadcast and multicast, different UE in a same group do not need to receive all TDMed group common PDSCHs of the same TB. Instead, the UEs should be allowed to selectively receive same or different PDSCHs among TDMed group common PDSCHs carrying the same TB.

Proposal 6: support transmission of multiple TDMed group-common PDSCHs carrying a same TB with selectively different RSs for both broadcast and multicast.

  - Different UE in the group selectively receive same or different PDSCHs among TDMed PDSCHs carrying the TB.

For unicast PDSCH, PDSCH configuration in 38.331 can currently provide tci-StatesToAddModList in PDSCH-config. Similarly, we propose that multiple TCI states can be configured in PDSCH-config for group common PDSCH for the CFR.

Proposal 7: Multiple TCI states can be configured in PDSCH-config for group common PDSCH for the CFR.

As specified in clause 5.1.5 of 38.214, if the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, where the threshold is based on reported UE capability, for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell. In addition, If the PDSCH is scheduled by a DCI format having the TCI field present, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state.

Assuming that the above text in clause 5.1.5 of 38.214 can be also applied to GC-PDCCH/GC-PDSCH, we would need to clarify how the threshold timeDurationForQCL can be determined based on reported UE capability, because multiple UEs receiving the same GC-PDCCH/GC-PDSCH in the same group may report different UE capability for the threshold timeDurationForQCL. One simple approach seems that UEs always go for the worst threshold timeDurationForQCL for GC-PDCCH/GC-PDSCH. Alternatively, gNB could configure threshold timeDurationForQCL per G-RNTI or per CFR based on the worst reported UE capability among UEs in the same group or in the same CFR. If configured, we prefer the threshold timeDurationForQCL per G-RNTI. If not configured, the threshold could be set to the worst value in the current specification.

Proposal 8: For reception of GC-PDSCH scheduled by GC-PDCCH, UE determines whether the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, as specified in clause 5.1.5 of 38.214, where the threshold is configured per G-RNTI by gNB (based on the worst reported UE capability).

  - If the threshold is not configured, the worst value of the threshold in the current specification is used.

# HARQ New TX and ReTX

RAN1 agreed the same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

In RAN1#106bis, RAN1 discussed the following FL’s proposal, but RAN1 did not reach any agreement.

Initial Question 4-1a: Regarding the NDI conflict issue that different UEs in a group may have different NDI values for a certain HPID before performing an initial PTM transmission, which option should be adopted? - Option 1: Rely on gNB implementation to avoid such issue. - Option 2: Resolve this issue with potential specification enhancement.

In our view, as specified in Rel-15/16, UE should manage NDIs in unicast DCIs only across unicast PDCCHs. Meanwhile, UE should manage NDIs in multicast DCIs only within same G-RNTI. That is, for multiple GC-DCIs scheduling multiple G-RNTIs, NDI values for different G-RNTIs are separately toggled and managed.

Accordingly, NDIs for multicast DCIs and NDIs for unicast DCIs should be independently and separately handled. A NDI value for unicast DCI and a NDI value for subsequent group common DCI can be same or different for a same HPN. Besides, a NDI value for group common DCI and a NDI value for subsequent unicast DCI can be same or different for a same HPN.

In the meantime, RAN1 previously agreed the same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB. We think that the NDI value should be same for a same TB for group common DCIs and PTP retransmissions for a same G-RNTI.

To avoid potential collision between PTP retransmission and unicast transmission for a same HARQ process number in DCIs, UE specific DCI scheduling PTP retransmission can indicate PTP retransmission. Thus, NDIs in UE specific DCIs scheduling PTP retransmission can be managed across PTM and PTP retransmission for a same HARQ process number, separately with NDIs for unicast DCI indicating the same HARQ process number.

Proposal 9: For multiple GC-DCIs scheduling multiple G-RNTIs, NDI values for different G-RNTIs are separately toggled and managed.

Proposal 10: For multiple GC-DCIs scheduling multiple G-RNTIs with different HPNs, PTP retransmission is associated to PTM initial transmission for one of the G-RNTIs based on the same HARQ process ID and NDI.

Proposal 11: UE specific DCI indicates PTP retransmission.

Proposal 12: For PTP ReTX and unicast with a same HPN, NDI value for PTP ReTX and NDI value for unicast are separately toggled and managed.

Meanwhile, we think that if UE receives PTP retransmission, it is desirable to continue on PTP retransmission, rather PTM retransmission. DRX active time may not be well aligned between PTP retransmission and PTM retransmission because PTP/PTM retransmissions will be likely scheduled after NACKs on PUCCHs separately allocated for PTP/PTM. For saving UE power, it seems beneficial for UE to expect PTP retransmission of a TB after sending NACK to the TB once PTP retransmission of a TB has been received. Otherwise, while monitoring PDCCH for PTP retransmission, UE would need to continuously monitor PDCCH for PTM retransmission. We think that it is up to UE whether to additionally receive retransmission of the same TB on group common PDSCH with the same HPN and non-toggled NDI.

Proposal 13: Upon receiving PTP retransmission of a TB with a HPN, UE expects PTP retransmission of the TB after sending NACK to the TB.

  - It is up to UE whether to additionally receive retransmission of the same TB on group common PDSCH with the same HPN and non-toggled NDI.

# Group Common SPS

NOTE: The proposals in this section may be also shown in LG’s other contribution to AI 8.12.2.

Since multiple UEs receive group common SPS, it seems essential for gNB to ensure whether a particular UE in the group is receiving group common SPS activation/(re-)transmission/release. We think that HARQ-ACK for group common SPS will be helpful for gNB to ensure that all UEs in the group are following group common SPS. Especially we propose that all UEs in the group individually send UE specific confirmation to DCI indicating SPS activation or deactivation by using HARQ-ACK on UE specific PUCCH resource. The UE specific PUCCH resource can be allocated by DCI indicating SPS activation or deactivation.

Proposal 14: For group common SPS, UE specific confirmation to group common SPS (de-)activation can be supported by PUCCH A/N.

- UE specific PUCCH resource is allocated by DCI indicating SPS (de-)activation.

When gNB activates group common SPS, a UE in the group may miss the activation DCI. If gNB did not receive confirmation from the UE or gNB did not receive HARQ-ACK to SPS PDSCH of a TB, gNB could schedule PTP transmission of the TB. In this case, initial HARQ transmission of the TB should be supported based on PTP transmission.

Proposal 15: For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

When gNB releases group common SPS, a UE in the group may miss the release DCI. If gNB did not receive confirmation from the UE, it is not clear how UE could finally release the group common SPS. Considering such erroneous case, it would be beneficial to introduce UE autonomous SPS release. It seems possible to use a UE’s individual timer based SPS release. However, considering misalignment among different UEs in the group, we should make sure that all UEs autonomously release group common SPS release at a same slot which could be pre-determined e.g. by RRC and/or DCI.

Proposal 16: After group common SPS activation, all UEs autonomously release the group common SPS right after a pre-determined slot

- The pre-determined time is determined by RRC and/or DCI.

# Multicast scheduling with carrier aggregation

It seems unclear whether carrier aggregation can be configured for a connected UE receiving multicast. In LTE, it is understood that UE configured with CA can receive MBMS depending on UE capability. Obviously, it would be beneficial that UE configured with CA can receive multicast transmission depending on UE capability considering that multicast transmission can work with other useful features. Thus, we propose that UE configured with CA support reception of multicast transmission depending on UE capability.

If UE receiving multicast can be configured with CA, we could further discuss whether group common PDCCH/PDSCH can be configured on a secondary cell for UE receiving multicast. We could also discuss whether CFR can be configured at a scheduled cell.

Proposal 17: UE configured with CA can support reception of multicast transmission depending on UE capability.

# Conclusion

In this contribution, we suggest the following proposals for support of group scheduling in NR MBS:

CFR

Proposal 1: For a connected UE receiving multicast (as well as idle/inactive UEs receiving broadcast), CFR associated to initial DL BWP can be configured with a wider bandwidth than the initial DL BWP or a bandwidth equal to or smaller than the initial DL BWP.

Proposal 2: At least for multicast, unicast BWP switching between UE’s active BWPs may immediately triggers CFR change between different CFRs associated to different UE’s active BWPs.

Proposal 3: If a CFR is confined within more than one UE active BWP with a same numerology, the CFR can be associated to more than one BWP.

- Upon unicast BWP switching between UE’s active BWPs associated to the same CFR, UE does not change CFR and continues to receive PTM/PTP (re-)transmissions on the CFR during/after unicast BWP switching.

Proposal 4: For broadcast, CFR of a cell is associated at least to initial DL BWP of the cell for any RRC state.

- FFS whether broadcast CFR is associated to UE’s active DL BWP for UE in RRC_CONNECTED.

CORESET/SS

Proposal 5: The maximum BD/CCE numbers are increased as R times MPDCCHmx,slot,μ and R times CPDCCHmax,slot,μ for a serving cell supporting multicast reception, where MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are defined in Table 10.1-2 and Table 10.1-3 in 38.213

  - R is a value reported by the UE as part of MBS related UE capability, regardless of whether UE supports CA capability.

Proposal 6: support transmission of multiple TDMed group-common PDSCHs carrying a same TB with selectively different RSs for both broadcast and multicast.

  - Different UE in the group selectively receive same or different PDSCHs among TDMed PDSCHs carrying the TB.

Proposal 7: Multiple TCI states can be configured in PDSCH-config for group common PDSCH for the CFR.

Proposal 8: For reception of GC-PDSCH scheduled by GC-PDCCH, UE determines whether the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, as specified in clause 5.1.5 of 38.214, where the threshold is configured per G-RNTI by gNB (based on the worst reported UE capability).

  - If the threshold is not configured, the worst value of the threshold in the current specification is used.

HARQ New TX and ReTX

Proposal 9: For multiple GC-DCIs scheduling multiple G-RNTIs, NDI values for different G-RNTIs are separately toggled and managed.

Proposal 10: For multiple GC-DCIs scheduling multiple G-RNTIs with different HPNs, PTP retransmission is associated to PTM initial transmission for one of the G-RNTIs based on the same HARQ process ID and NDI.

Proposal 11: UE specific DCI indicates PTP retransmission.

Proposal 12: For PTP ReTX and unicast with a same HPN, NDI value for PTP ReTX and NDI value for unicast are separately toggled and managed.

Proposal 13: Upon receiving PTP retransmission of a TB with a HPN, UE expects PTP retransmission of the TB after sending NACK to the TB.

  - It is up to UE whether to additionally receive retransmission of the same TB on group common PDSCH with the same HPN and non-toggled NDI.

Group common SPS

Proposal 14: For group common SPS, UE specific confirmation to group common SPS (de-)activation can be supported by PUCCH A/N.

- UE specific PUCCH resource is allocated by DCI indicating SPS (de-)activation.

Proposal 15: For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

Proposal 16: After group common SPS activation, all UEs autonomously release the group common SPS right after a pre-determined slot

- The pre-determined time is determined by RRC and/or DCI.

CA

Proposal 17: UE configured with CA can support reception of multicast transmission depending on UE capability.

# Reference

[1] RP-201038	WID revision: NR Multicast and Broadcast Services,	Huawei, HiSilicon