3GPP TSG RAN WG1 #107-e			R1-2112131

e-Meeting, November 11th – 19th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on cross-carrier scheduling enhancements for NR DSS

Agenda Item:	8.13.1

Document for: 	Discussion and Decision

- Introduction

At the previous meetings, cross-carrier scheduling enhancements from sSCell to P(S)Cell were discussed, and RAN1 made following agreements and conclusion [1].

Agreement Option A is supported in Rel-17 - At least for Type B UE, when the UE is configured for CCS from sSCell to P(S)Cell and when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1),[and at least when UE is not provided monitoringCapabilityConfig for any cell] - Option A - On P(S)Cell (for self-scheduling) - UE is not required to monitor more than α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - On sSCell (for cross-carrier scheduling to P(S)Cell) - UE is not required to monitor more than [minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 or MPDCCHmax,slot,μ1] PDCCH BD candidates per sSCell slot - UE is additionally not required to monitor more than (1-α)*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - 0≤α≤1 is based on RRC configuration - α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ is used for P(S)Cell overbooking procedure - When determining MPDCCHtotal,slot,μ and MPDCCHtotal,slot,μ1 - P(S)Cell self-scheduling is counted by applying scaling factor s1 - sSCell to P(S)Cell scheduling is counted additionally (assuming SCS of sSCell) by applying scaling factor s2 - s1=1 and s2=0, FFS other s1 and s2 - 0≤s1≤1 and 0≤s2≤1 are based on RRC configuration - FFS: additional constraints on s1 and s2 e.g., 1 ≤ s1+s2 ≤ 2 or s1 + s2 ≤ 1 - Note: Ncellscap is as in Rel16 - UE capability/incapability indication for below to be discussed as part of UE features discussion - All search space configurations monitored on sSCell for cross-carrier scheduling to P(S)Cell are within a single span of [3] consecutive OFDM symbols within a duration spanning P(S)Cell slot - Same approach as above is used for CCE limits - FFS: Separate vs. same RRC configured scaling factors (corresponding to α) for BD and CCE limits. - When P(S)Cell SCS (μ) is larger than sSCell SCS (μ1), for CCS from sSCell to P(S)Cell and, it is not supported Rel-17 DSS. Conclusion - When sSCell to PCell cross-carrier scheduling is configured, DCI format 2_6 (if configured) is monitored only on P(S)Cell Working Assumption - When CIF for sSCell to PCell cross-carrier scheduling is configured, non-fallback DCI formats on P(S)Cell include same number of CIF bits as the corresponding non-fallback DCI formats on sSCell that are used for sSCell to P(S)Cell scheduling Conclusion - A UE configured for cross-carrier scheduling from SCell to P(S)Cell can also be configured with unaligned CA (i.e., using ca-SlotOffset ), and a non-zero value for ca-SlotOffset can be configured at least for SCells other than the sSCell - FFS: Whether case when sSCell is configured with non-zero ca-SlotOffset is supported and any associated capability signalling - Note: No additional L1 spec impact related to ca-SlotOffset had been identified Conclusion - When CCS from sSCell to P(S)Cell is configured for a UE monitoringSlotPeriodicityAndOffset, monitoringSymbolsWithinSlot, duration for the PDCCH monitoring candidates monitored on sSCell as determined per Rel16 SS linking approach

Here, we discuss the remaining issues on cross-carrier scheduling enhancements from sSCell to P(S)Cell.

- DCI size budget

At the previous meeting, RAN1 made the working assumption on CIF field for DCI size alignment. The working assumption should be confirmed so that the DCI size can be aligned as much as possible. If the DCI sizes may be different in some cases, the existing specification for DCI size budget can cover such cases.

Proposal 1:

- Confirm the following working assumption:

  - When CIF for sSCell to PCell cross-carrier scheduling is configured, non-fallback DCI formats on P(S)Cell include same number of CIF bits as the corresponding non-fallback DCI formats on sSCell that are used for sSCell to P(S)Cell scheduling.

- PDCCH BD limit handling

At the previous meeting, Option A for PDCCH BD limit handling was agreed. The remaining issue on Option A is which of [minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 or MPDCCHmax,slot,μ1] is selected for the number of PDCCH BD candidates per sSCell slot. In our understanding, for CCS from sSCell to P(S)Cell, as MPDCCHtotal,slot,μ was already taken into account, it seems duplicate if MPDCCHtotal,slot,μ1 is taken into account for CCS from sSCell to P(S)Cell. Thus, the choice of MPDCCHmax,slot,μ1 is reasonable for better flexibility.

Proposal 2:

- For option A on PDCCH BD limit handling, UE is not required to monitor more than MPDCCHmax,slot,μ1 PDCCH BD candidates per sSCell slot.

- Deactivation

It was agreed to support dormant BWP operation on sSCell in addition to SCell deactivation on sCell. When sCell is deactive or with dormant BWP, non-fallback DCI formats cannot be used for P(S)Cell scheduling. Thus, when sCell is deactive or with dormant BWP, the mechanism for monitoring of non-fallback DCI formats on P(S)Cell should be supported.

Proposal 3:

- When sCell is deactive or with dormant BWP, the mechanism for monitoring of non-fallback DCI formats on P(S)Cell should be supported.

- Conclusion

In this contribution, we discussed how to support PDCCH of sSCell scheduling PDSCH or PUSCH on P(S)Cell for Rel-17 NR DSS. Based on the discussion, we made following proposals.

Proposal 1:

- Confirm the following working assumption:

  - When CIF for sSCell to PCell cross-carrier scheduling is configured, non-fallback DCI formats on P(S)Cell include same number of CIF bits as the corresponding non-fallback DCI formats on sSCell that are used for sSCell to P(S)Cell scheduling.

Proposal 2:

- For option A on PDCCH BD limit handling, UE is not required to monitor more than MPDCCHmax,slot,μ1 PDCCH BD candidates per sSCell slot.

Proposal 3:

- When sCell is deactive or with dormant BWP, the mechanism for monitoring of non-fallback DCI formats on P(S)Cell should be supported.

References

- 3GPP RAN1#106bis-e, Chairman’s note, October 2021.