# 3GPP TSG RAN WG1 #107-e							R1-2112259

# e-Meeting, November 11th – November 19th, 2021

Agenda item:	8.16.14

Source: 	Qualcomm Incorporated

Title: 	UE features for IoT over NTN

Document for:	Discussion and Decision

# Introduction

In RAN1#106-bis-e, a table for the RAN1 UE feature list for IoT-NTN was agreed as a baseline (Section 6 in R1-2109918), with unresolved items marked in the table with yellow highlights. In this contribution, we provide our proposals on these unresolved aspects, with a goal towards further refining the agreed table.

# Support of GEO vs NGSO NTNs

There will be differences in the minimum set of components that a UE needs to support to facilitate communication with an NTN cell, depending on the orbit of the satellites comprising the NTN cell—specifically, whether the satellites are geostationary (GEO) or non-geostationary (e.g., LEO, MEO). For example, all the time/frequency pre-compensation-related aspects are typically more challenging for LEO satellites than for GEO—these may potentially lead to simplified requirements to support GEO, as compared to supporting non-GEO (NGSO) NTNs. Also, GEO NTNs may not require “segment-based pre-compensation”, since the pre-compensation values for uplink transmissions may remain valid for a long period of time.

Proposal 1: Introduce separate feature groups—such as 2-1-GEO, 2-1-LEO and 2-1-MEO—corresponding to different satellite orbits, each with its own set of minimum required components.

# Capability type and need for base station to know

Proposal 2: A UE’s support of NTN should be a “per band” capability.

Proposal 3: The base station needs to know if the UE supports the IoT-NTN feature.

# Separate capabilities for NB-IoT and eMTC

NB-IoT and eMTC capabilities are captured separately by RAN2. While for the feature list discussion, it may be OK to discuss these together, eventually, these need to be captured separately for eMTC and NB-IoT.

Proposal 4: Capture IoT-NTN capabilities separately for eMTC and NB-IoT

# Text pertaining to the “Mandatory/Optional” column

For the three capabilities (2-1, 2-2 and 2-3), the “Mandatory/Optional” column contains the following text:

“For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported”

For 2-1, this text is anyway true, and is not adding anything new. We therefore propose that this text be removed from 2-1.

For 2-2 and 2-3, however, given that these are features dependent on the mother feature (2-1), a UE supporting 2-1 and NOT supporting 2-2 and/or 2-3 can also (albeit with a restricted realm of operation) communicate with an NTN cell. As a result, we propose to remove the quoted text from 2-2 and 2-3.

Proposal 5: Remove the text “For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” from the “Mandatory/Optional” column for FG 2-1, 2-2 and 2-3.

# Components of FG 2-1

Proposal 6: In FG 2-1, merge current components 4 and 5 (pertaining to UE-specific TA calculation) into a single component

Proposal 7: In FG 2-1, component 11 regarding updating time and frequency pre-compensation between segments may be a LEO/GEO/MEO-specific component

- This feature may not be essential for GEO, while it may be essential for LEO and MEO.

# Applicability of Terrestrial Network (TN) features to NTN

RAN1 should discuss how to determine the applicability of all terrestrial network (TN) features up to Rel16 for the case of NTN. As such, the legacy capabilities may be interpreted as being exclusively applicable for TNs, while a separate set of capabilities corresponding to those features may be introduced for NTN. This may prevent a “feature by feature” determination of applicability to NTNs from the (legacy) feature list for TNs up to Rel16.

Proposal 8: RAN1 to discuss the determination of applicability of Terrestrial Network features up to Release 16 to IoT-NTN in Release 17.

# Conclusion

We summarize our proposals below.

Proposal 1: Introduce separate feature groups—such as 2-1-GEO, 2-1-LEO and 2-1-MEO—corresponding to different satellite orbits, each with its own set of minimum required components.

Proposal 2: A UE’s support of NTN should be a “per band” capability.

Proposal 3: The base station needs to know if the UE supports the IoT-NTN feature.

Proposal 4: Capture IoT-NTN capabilities separately for eMTC and NB-IoT

Proposal 5: Remove the text “For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” from the “Mandatory/Optional” column for FG 2-1, 2-2 and 2-3.

Proposal 6: In FG 2-1, merge current components 4 and 5 (pertaining to UE-specific TA calculation) into a single component

Proposal 7: In FG 2-1, component 11 regarding updating time and frequency pre-compensation between segments may be a LEO/GEO/MEO-specific component

- This feature may not be essential for GEO, while it may be essential for LEO and MEO.

Proposal 8: RAN1 to discuss the determination of applicability of Terrestrial Network features up to Release 16 to IoT-NTN in Release 17.