3GPP TSG RAN WG1 Meeting #107-e	R1-2112365

# E-meeting,  November 11th - 19th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1403 | rev | - | Current version: | 15.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NPDSCH and NPDCCH starting position for TDD NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-15, the following agreements (RAN1# 93) were reached for both NPDSCH and NPDCCH in TDD NB-IoT. - Transmission in DwPTS is supported for in-band when the number of OFDM symbols in DwPTS is greater than 3. - Transmission in DwPTS is supported for guard-band and standalone for all DwPTS configurations. However, the above was not captured into the Technical specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NPDSCH and NPDCCH starting position for TDD NB-IoT in the anchor and non-nachor carrier case have been incorporated in clauses 16.4.1.4 and 16.6.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NPDSCH and NPDCCH starting position for TDD NB-IoT will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.4.1.4 and 16.6.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Text Start ------------------------------------------------------------------------

The starting OFDM symbol for NPDSCH is given by index  in the first slot in a subframe  and is determined as follows

-	if subframe  is a subframe used for receiving SIB1-NB

-	if the value of the higher layer parameter operationModeInfo is set to '00' or '01'

-	 if the value of the higher layer parameter operationModeInfo is set to '10' and the value of the higher layer parameter sib-GuardbandInfo is set to '10' or '11' for TDD

-	otherwise

-	elseif subframe  is a special subframe for NPDSCH without repetition

-	 where is given by the higher layer parameter eutraControlRegionSize if the value of the higher layer parameter eutraControlRegionSize is present

-	otherwise

-	else

-	is given by the higher layer parameter eutraControlRegionSize if the value of the higher layer parameter eutraControlRegionSize is present

-	for TDD if the carrier on which NPSS/NSSS/NPBCH are detected the value of the higher layer parameter operationModeInfo is set to '00' or '01', or if the value of the higher layer parameter inbandCarrierInfo-r13 is configured for a higher layer configured carrier if any.

-	otherwise

-------------------------------------------------------- Text Omitted ----------------------------------------------------------------------

### 16.6.1	NPDCCH starting position

The starting OFDM symbol for NPDCCH given by index  in the first slot in a subframe  and is determined as follows

-	if higher layer parameter eutraControlRegionSize is present

-	if subframe  is a special subframe for NPDCCH without repetition

-	 where is given by the higher layer parameter eutraControlRegionSize

-	= 3 for TDD if the carrier on which NPSS/NSSS/NPBCH are detected the value of the higher layer parameter operationModeInfo is set to '00' or '01', or if the value of the higher layer parameter inbandCarrierInfo-r13 is configured for a higher layer configured carrier if any.

-	= 0 otherwise.

-	else  is given by the higher layer parameter eutraControlRegionSize

-	otherwise

-

---------------------------------------------------------- Text End ------------------------------------------------------------------------