3GPP TSG RAN WG1 #107-e			R1-2112388

e-Meeting, November 11th – 19th, 2021

Source:	WILUS Inc.

Title:	Remining issue on duplex operation for RedCap UE

Agenda item:	8.6.1.2

Document for:	Discussion/Decision

- Introduction

From the RAN1#104b-e meeting and RAN1#106-e meeting [1]-[3], RAN1 discussed issues such as HD-FDD switching time, collision handling for each case of various UL/DL collision cases and whether to introduce semi-static UL/DL configuration or dynamic SFI for HD-FDD. In this contribution, we discuss the remaining issues on UL/DL collision handling related to half-duplex operation for RedCap UEs.

- Remaining issue on half-duplex FDD operations

Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

Working assumption at RAN1#104b-e [1] - If a dynamically scheduled UL transmission overlaps with an SSB, down-select one of the following options: - Option 1: Follow the handling of case 2 that dynamic UL is prioritized over SSB - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamic UL - Option 3: Leave to UE implementation (e.g. UE can receive the SSB if UE needs to receive the SSB; otherwise, UE can transmit the UL transmission) whether to receive the SSB or transmit the UL transmission - Other options are not precluded - If a semi-static configured UL transmission overlaps with an SSB, down-select one of from the following options - Option 1: Up to gNB configuration to avoid such collision and if it happens it is an error case - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over semi-static UL - Option 3: Leave to UE implementation (e.g. UE can receive the SSB if UE needs to receive the SSB; otherwise, UE can transmit the UL transmission) whether to receive the SSB or transmit the UL transmission - Other options are not precluded - FFS: whether/how to account for Tx/Rx switching time before and after the set of SSB symbols - FFS: whether or not the semi-static configured UL transmission includes a valid RO Agreement at RAN1#106-e meeting [3]: - For Case 5 of SSB overlaps with in configured UL transmission, re-use the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over configured UL transmission - The configured UL transmission includes CG-PUSCH, or SRS - FFS: Confirm that PUCCH is included Agreement at RAN1#106-e meeting [3]: - For Case 5 of SSB overlaps with configured UL transmission, the configured UL transmission includes PUCCH transmission configured by higher layers - Note: The UL transmission indicated by DCI is supposed to be dynamic UL transmission. Agreement at RAN1#106-e meeting [3]: - For Case 5 of dynamically scheduled UL transmission vs. SSB, one or both of the following options to be determined till next meeting: - Option 1: Dynamically scheduled UL transmission is prioritized over SSB - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

For Case 5, it was agreed at the previous RAN1#106-e meeting that SSB is prioritized over configured UL transmission for (Case 5-2) collisions between semi-statically configured UL transmission and SSB. RAN1 discussed two options (Option 1 and Option 2) for (Case 5-1) collisions between dynamically scheduled UL transmission and SSB, however it was not concluded at the previous meeting. For Case 5-1, there are no interference from a UL transmission to SSB reception even if the UL transmission overlaps the SSB symbols because it is paired spectrum. So, a gNB can schedule UL transmission dynamically by a DCI format. Therefore, we prefer Option 1 which follows the handling of Case 2 that dynamically scheduled UL transmission is prioritized over SSB.

- Proposal: If a dynamically scheduled UL transmission overlaps with an SSB, we prefer Option 1 that the dynamically scheduled UL transmission is prioritized over SSB.

- Conclusion

In this contribution, we discussed aspects on duplex operation for RedCap UEs. The following is proposed:

- Proposal: If a dynamically scheduled UL transmission overlaps with an SSB, we prefer Option 1 that the dynamically scheduled UL transmission is prioritized over SSB.

References

- Final Report of 3GPP TSG RAN WG1 #104b-e v1.0.0

- Final Report of 3GPP TSG RAN WG1 #105-e v1.0.0

- Draft Report of 3GPP TSG RAN WG1 #106-e v0.1.0

- TS38.213 v15.13.0

- TS38.213 v16.5.0