3GPP TSG-WG1 Meeting # 107-e	R1-2112909

# e-Meeting, November 11th - 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1410 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NB-IoT/eMTC support for Non-Terrestrial Networks in 36.213 s00-s05 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2021-11-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of NB-IoT/eMTC support for Non-Terrestrial Networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for NB-IoT/eMTC support for Non-Terrestrial Networks features according to RAN1 agreements: - timing relationship enhancements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of NB-IoT/eMTC support for Non-Terrestrial Networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, 5.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.211 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

### 4.2.3	Transmission timing adjustments

Upon reception of a timing advance command or a timing adjustment indication for a TAG containing the primary cell or PSCell, the UE shall adjust uplink transmission timing for PUCCH/PUSCH/SRS of the primary cell or PSCell based on the received timing advance command or a timing adjustment indication.

The UL transmission timing for PUSCH/SRS of a secondary cell is the same as the primary cell if the secondary cell and the primary cell belong to the same TAG. If the primary cell in a TAG has a frame structure type 1 and a secondary cell in the same TAG has a frame structure type 2 or frame structure 3, UE may assume that NTA ≥ 624.

If the UE is configured with a SCG, the UL transmission timing for PUSCH/SRS of a secondary cell other than the PSCell is the same as the PSCell if the secondary cell and the PSCell belong to the same TAG.

Upon reception of a timing advance command or a timing adjustment indication for a TAG not containing the primary cell or PSCell, if all the serving cells in the TAG have the same frame structure type, the UE shall adjust uplink transmission timing for PUSCH/SRS of all the secondary cells in the TAG based on the received timing advance command or a timing adjustment indication where the UL transmission timing for PUSCH /SRS is the same for all the secondary cells in the TAG.

Upon reception of a timing advance command or a timing adjustment indication for a TAG not containing the primary cell or PSCell, if a serving cell in the TAG has a different frame structure type compared to the frame structure type of another serving cell in the same TAG, the UE shall adjust uplink transmission timing for PUSCH/SRS of all the secondary cells in the TAG by using NTAoffset = 624 regardless of the frame structure type of the serving cells and based on the received timing advance command or a timing adjustment indication where the UL transmission timing for PUSCH /SRS is the same for all the secondary cells in the TAG. NTAoffset is described in [3].

The timing adjustment indication specified in [11] indicates the initial NTA used for a TAG.The timing advance command for a TAG indicates the change of the uplink timing relative to the current uplink timing for the TAG as multiples of 16. The start timing of the random access preamble is specified in [3].

In case of random access response, an 11-bit timing advance command [8], TA, for a TAG indicates NTA values by index values of TA = 0, 1, 2, ..., 256 if the UE is configured with a SCG, and TA = 0, 1, 2, ..., 1282 otherwise, where an amount of the time alignment for the TAG is given by NTA = TA 16. NTA is defined in [3].

In other cases, a 6-bit timing advance command [8] or the Timing advance adjustment field in DCI format 6-0A/B if present [4], TA, for a TAG indicates adjustment of the current NTA value, NTA,old, to the new NTA value, NTA,new, by index values of TA = 0, 1, 2,..., 63, where NTA,new = NTA,old + (TA 31)16. Here, adjustment of NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing for the TAG by a given amount respectively.

For a non-BL/CE UE, for a timing advance command received on

-	subframe n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of subframe n+5 if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, n+6 otherwise.

-	slot n, the corresponding adjustment of the uplink transmission timing shall apply from the first subframe boundary no earlier than slot [n+8].

-	subslot n, the corresponding adjustment of the uplink transmission timing shall apply from the first subframe boundary no earlier than

-	subslot [n+16] if higher layer parameter proc-TimeAdv-r15= 'nplus4set1'.

-	subslot [n+18] if higher layer parameter proc-TimeAdv-r15= 'nplus6set1'or 'nplus6set2'.

-	subslot [n+20] if higher layer parameter proc-TimeAdv-r15= 'nplus8set2'.

For serving cells in the same TAG, when the UE's uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of subframe n and not transmit the overlapped part of subframe n+1.

For a BL/CE UE, for a timing advance command received on subframe n, the corresponding adjustment of the uplink transmission timing shall apply for the uplink PUCCH/PUSCH/SRS transmissions in subframe n+6+Koffset. When the BL/CE UE's uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are on the same narrowband and are overlapped due to the timing adjustment, the UE shall complete transmission of subframe n and is not required to transmit in subframe n+1 until the first available symbol that has no overlapping portion with subframe n. When the BL/CE UE's uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are on different narrowbands, and the timing adjustment occurs in the guard period for narrowband retuning, the UE is not required to transmit in subframe n+1 until the first available symbol that has no overlapping portion with subframe n and which does not reduce the guard period. The value of Koffset is given by,

-	if the UE is configured with the higher layer parameter CellSpecificKoffset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter CellSpecificKoffset provided by higher layers, and

KUE_offset is the parameter UESpecificKoffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

If the received downlink timing changes and is not compensated or is only partly compensated by the uplink timing adjustment without timing advance command as specified in [10], the UE changes NTA accordingly.

<Unchanged parts are omitted>

### 5.1.1	Physical uplink shared channel

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group

-	When the procedures are applied for primary PUCCH group, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the primary PUCCH group respectively.

-	When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the secondary PUCCH group respectively.

For PUSCH (re)transmissions corresponding to ul-ConfigInfo-r14, the UE shall apply the procedures corresponding to PUSCH (re)transmission corresponding to the random access response grant.

#### 5.1.1.1	UE behaviour

The setting of the UE Transmit power for a Physical Uplink Shared Channel (PUSCH) transmission is defined as follows.

If the UE transmits PUSCH without a simultaneous PUCCH for the serving cell , then the UE transmit power  for PUSCH transmission in subframe/slot/subslot i for the serving cell is given by

[dBm]

If the UE transmits PUSCH simultaneous with PUCCH for the serving cell , then the UE transmit power  for the PUSCH transmission in subframe/slot/subslot i for the serving cell  is given by

[dBm]

If the UE is not transmitting PUSCH for the serving cell c, for the accumulation of TPC command received with DCI format 3/3A for PUSCH, the UE shall assume that the UE transmit power  for the PUSCH transmission in subframe i for the serving cell  is computed by

[dBm]

where,

-	is the configured UE transmit power defined in [6] in subframe/slot/subslot i for serving cell  and  is the linear value of . If the UE transmits PUCCH without PUSCH in subframe for the serving cell c, for the accumulation of TPC command received with DCI format 3/3A for PUSCH, the UE shall assume  as given by Clause 5.1.2.1. If the UE does not transmit PUCCH and PUSCH in subframe  for the serving cell c, for the accumulation of TPC command received with DCI format 3/3A for PUSCH, the UE shall compute  assuming MPR=0dB, A-MPR=0dB, P-MPR=0dB and TC =0dB, where MPR, A-MPR, P-MPR and TC are defined in [6].

-	 is the linear value of defined in Clause 5.1.2.1

-	If the UE is a BL/CE UE configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment valid for subframe i and serving cell is using uplink resource allocation type 5, is the bandwidth of the PUSCH resource assignment expressed in fraction of a resource block and is given by  where  are defined in [3] and is defined in Clause 8.6.1 for subframe i, is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks valid for subframe/slot/subslot i and serving cell  otherwise.

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell  and if subframe  belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12,

-	when j=0, , where j=0 is used for PUSCH (re)transmissions corresponding to a semi-persistent grant. and  are the parameters p0-UE-PUSCH-Persistent-SubframeSet2-r12 and p0-NominalPUSCH-Persistent -SubframeSet2-r12 respectively provided by higher layers, for each serving cell .

-	when j=1,, where j=1 is used for PUSCH (re)transmissions corresponding to a dynamic scheduled grant. and are the parameters p0-UE-PUSCH-SubframeSet2-r12 and p0-NominalPUSCH-SubframeSet2-r12 respectively, provided by higher layers for serving cell .

-	when j=2, where  and , where the parameter preambleInitialReceivedTargetPower [8] () and  are signalled from higher layers for serving cell , where j=2 is used for PUSCH (re)transmissions corresponding to the random access response grant.

Otherwise

-	is a parameter composed of the sum of a component  provided from higher layers for j=0, 1 and 3 and a component  provided by higher layers for j=0, 1 and 3 for serving cell . For PUSCH (re)transmissions corresponding to a semi-persistent grant then j=0 , for PUSCH (re)transmissions corresponding to a dynamic scheduled grant then j=1, for PUSCH (re)transmissions corresponding to the random access response grant then j=2 and for BL/CE UE PUSCH (re)transmission using preconfigured uplink resource then j=3.  and , where the parameter preambleInitialReceivedTargetPower [8] () and  are signalled from higher layers for serving cell .

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell  and if subframe  belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12,

-	For j=0 or 1, . is the parameter alpha-SubframeSet2-r12 provided by higher layers for each serving cell .

-	For j=2, .

-	Else if the UE is configured with higher layer parameter UplinkPowerControlDedicated-v15x0 for serving cell c,

-	For j=0 or 1, . is the parameter alpha-UE-r15 provided by higher layers for each serving cell .

-	For j=2, .

Otherwise

-	For j =0 or 1,  is a 3-bit parameter provided by higher layers for serving cell . For j=2,  For j=3,  is the parameter pur-PUSCH-power-control-alpha provided by higher layers for serving cell .

-	 is the downlink path loss estimate calculated in the UE for serving cell  in dB and  = referenceSignalPower – higher layer filtered RSRP, where referenceSignalPower is provided by higher layers and RSRP is defined in [5] for the reference serving cell and the higher layer filter configuration is defined in [11] for the reference serving cell.

-	If serving cell  belongs to a TAG containing the primary cell then, for the uplink of the primary cell, the primary cell is used as the reference serving cell for determining referenceSignalPower and higher layer filtered RSRP. For the uplink of the secondary cell, the serving cell configured by the higher layer parameter pathlossReferenceLinking defined in [11] is used as the reference serving cell for determining referenceSignalPower and higher layer filtered RSRP.

-	If serving cell  belongs to a TAG containing the PSCell then, for the uplink of the PSCell, the PSCell is used as the reference serving cell for determining referenceSignalPower and higher layer filtered RSRP; for the uplink of the secondary cell other than PSCell, the serving cell configured by the higher layer parameter pathlossReferenceLinking defined in [11] is used as the reference serving cell for determining referenceSignalPower and higher layer filtered RSRP.

-	If serving cell  belongs to a TAG not containing the primary cell or PSCell then serving cell  is used as the reference serving cell for determining referenceSignalPower and higher layer filtered RSRP.

-	for and 0 for where  is given by the parameter deltaMCS-Enabled provided by higher layers for each serving cell .  and , for each serving cell , are computed as below.  for transmission mode 2.

-	for control data sent via subframe-PUSCH without UL-SCH data or slot/sublot-PUSCH without UL-SCH data if the UE is configured with a higher layer parameter uplinkPower-CSIPayload,  and withdefined as the number of CQI/PMI bits including CRC for a given RI value for slot/subslot-PUSCH without UL-SCH data if the UE is not configured with a higher layer parameter uplinkPower-CSIPayload, and BPRE=  for other cases.

-	where  is the number of code blocks,  is the size for code block ,  is the number of CQI/PMI bits including CRC bits and  is the number of resource elements determined as , where , ,  and  are defined in [4].

-	 for control data sent via PUSCH without UL-SCH data and  for other cases.

-	 is a correction value, also referred to as a TPC command and is included in PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or in PDCCH/SPDCCH with DCI format 7-0A/7-0B or in MPDCCH with DCI format 6-0A for serving cell or jointly coded with other TPC commands in PDCCH/MPDCCH with DCI format 3/3A whose CRC parity bits are scrambled with TPC-PUSCH-RNTI. If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell  and if subframe  belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12, the current PUSCH power control adjustment state for serving cell is given by, and the UE shall use  instead of to determine . Otherwise, the current PUSCH power control adjustment state for serving cell is given by. If the UE is configured with multiple UL SPS configurations,  is a correction value, also referred to as a TPC command and is jointly coded with other TPC commands in PDCCH with DCI format 3/3A whose CRC parity bits are scrambled with TPC-PUSCH-RNTI, where x is SPS-ConfigIndex-r14, and  and  are replaced by  and , respectively.

-	 and  if accumulation is enabled based on the parameter Accumulation-enabled or accumulationEnabledsTTI provided by higher layers or if the TPC command  is included in a PDCCH/EPDCCH with DCI format 0 or in a MPDCCH with DCI format 6-0A for serving cell  where the CRC is scrambled by the Temporary C-RNTI or PUR-RNTI

-	 and  if accumulation is enabled based on the parameter Accumulation-enabled or accumulationEnabledsTTI provided by higher layers and if the TPC command  is included in a PDCCH with DCI format 3/3A whose CRC parity bits are scrambled by TPC-PUSCH-RNTI and if the UE is configured with multiple UL SPS configurations.

-	where was signalled on PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or PDCCH/SPDCCH with DCI format 7-0A/7-0B or MPDCCH with DCI format 6-0A or PDCCH/MPDCCH with DCI format 3/3A on subframe/slot/subslot , and where  is the first value after reset of accumulation. For a BL/CE UE configured with CEModeA, subframe i- KPUSCH-Koffset is the last subframe in which the MPDCCH with DCI format 6-0A or MPDCCH with DCI format 3/3A is transmitted.

-	The value of Koffset for a BL/CE UE is

-	if the UE is configured with the higher layer parameter CellSpecificKoffset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter CellSpecificKoffset provided by higher layers, and

KUE_offset is the parameter UESpecificKoffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

-	The value of  is

-	For FDD or FDD-TDD and serving cell frame structure type 1

-	if the UE is configured with higher layer parameter shortTTI and the TPC command  is included in a PDCCH/ SPDCCH with DCI format 7-0A/7-0B and for PUSCH transmissions in a subslot,

-	the UE is configured with higher layer parameters dl-STTI-Length='subslot' and ul-STTI-Length='slot' and the TPC command  is included in a PDCCH/ SPDCCH with DCI format 7-0A/7-0B and for PUSCH transmissions in a slot, corresponds to:

-	a subslot among subslot 4 or 5 of subframe N-3 or subslot 0 of subframe N-2 in which the UE has received the TPC command if the slot PUSCH is to be transmitted in slot 0 of subframe N. A UE is not expected to receive TPC command in more than one subslot among subslot 4 or 5 of subframe N-3 or subslot 0 of subframe N-2 corresponding to slot-PUSCH transmission in slot 0 of subframe N.

-	a subslot among subslot 1 or 2 or 3 of subframe N-2 in which the UE has received the TPC command if the slot-PUSCH is to be transmitted in slot 1 of subframe N. A UE is not expected to receive TPC command in more than one subslot among subslot 1 or 2 or 3 of subframe N-2 corresponding to slot-PUSCH transmission in slot 1of subframe N.

-	if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	otherwise,  = 4 (in unit of slots for slot-PUSCH and the TPC command is included in a PDCCH/ SPDCCH with DCI format 7-0A/7-0B, and in units of subframe for subframe-PUSCH and for slot/subslot-PUSCH with a TPC commandprovided in the PDCCH with DCI format 3/3A).

-	For TDD, if the UE is configured with more than one serving cell and the TDD UL/DL configuration of at least two configured serving cells is not the same, or if the UE is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell, or for FDD-TDD and serving cell frame structure type 2, the "TDD UL/DL configuration" refers to the UL-reference UL/DL configuration (defined in Clause 8.0) for serving cell .

-	For TDD UL/DL configurations 1-6 and UE not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell ,  is given

-	in Table 5.1.1.1-1A if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space for subframe-PUSCH transmissions,

-	in Table 5.1.1.1-1B for special subframe configuration 1, 2, 3, 4, 6, 7, 8 and Table 5.1.1.1-1C for special subframe configuration 0, 5, 9 if the UE is configured with higher layer parameter shortTTI and the TPC command  is included in a PDCCH/ SPDCCH with DCI format 7-0A/7-0B, and for uplink transmissions in a slot,

-	by  if the slot-PUSCH transmission in slot 15 or 16 is scheduled with a PDCCH/SPDCCH with DCI format 7-0A/7-0B in which the LSB of the UL index is set to 1 for TDD UL/DL configuration 6 and special subframe configuration 0, 5, 9,

-	in Table 5.1.1.1-1 otherwise.

-	For TDD UL/DL configuration 0 and UE not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell .

-	If the subframe-PUSCH transmission in subframe 2 or 7 is scheduled with a PDCCH/EPDCCH of DCI format 0/4 or a MPDCCH of DCI format 6-0A in which the LSB of the UL index is set to 1, = 7

-	For slot-PUSCH transmissions, is given by Table 5.1.1.1-1B for special subframe configuration 1, 2, 3, 4, 6, 7, 8 and Table 5.1.1.1-1C for special subframe configuration 0, 5, 9 if the TPC command  is included in a PDCCH/ SPDCCH with DCI format 7-0A/7-0B. If the TPC command  is provided in the PDCCH with DCI format 3/3A,  is given in Table 5.1.1.1-1.

-	For all other subframe-PUSCH transmissions,  is given in Table 5.1.1.1-1A if the UE is configured with shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, otherwise Table 5.1.1.1-1.

-	For TDD UL/DL configurations 0-5 and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell ,  is given in

-	Table 5.1.1.1-4A if the UE is configured with shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	Table 5.1.1.1-4B for slot-PUSCH transmissions if the TPC command  is included in a PDCCH/ SPDCCH with DCI format 7-0A/7-0B,

-	Table 5.1.1.1-4 otherwise.

-	For TDD UL/DL configuration 6 and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell

- If the subframe-PUSCH transmission in subframe 2 or 7 is scheduled with a PDCCH/EPDCCH of DCI format 0/4 if the UE is not configured with higher layer parameter shortProcessingTime or if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the common search space in which the LSB of the UL index is set to 1, = 6

-	For all other PUSCH transmissions,  is given in

- Table 5.1.1.1-4A if the UE is configured with shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

- Table 5.1.1.1-4B for slot-PUSCH transmissions if the TPC command  is included in a PDCCH/ SPDCCH with DCI format 7-0A/7-0B,

-	Table 5.1.1.1-4 otherwise.

-	For a serving cell with frame structure type 3,

-	For an uplink DCI format 0A/4A carrying AUL-DFI according to clause 8.3A, =4.

-	For an uplink DCI format 0A/0B/0C/4A/4B with CRC scrambled by C-RNTI with PUSCH trigger A set to 0, is equal to k+l, where k and l are defined in in Clause 8.0.

-	For an uplink DCI format 0A/0B/0C/4A/4B with CRC scrambled by C-RNTI with PUSCH trigger A set to 1 and upon the detection of PDCCH with CRC scrambled by CC-RNTI and with 'PUSCH trigger B' field set to '1' described in Clause 8.0, is equal to p+k+l, where p, k and l are defined in Clause 8.0.

-	If a UE detected multiple TPC commands in subframe , the UE shall use the TPC command in the PDCCH/EPDCCH with DCI format 0A/0B/0C/4A/4B with CRC scrambled by C-RNTI which schedules PUSCH transmission in subframe i.

-	For serving cell  and a non-BL/CE UE, the UE attempts to decode a PDCCH/EPDCCH of DCI format 0/0A/0B/0C/4/4A/4B or a PDCCH/SPDCCH of DCI format 7-0A/7-0B with the UE's C-RNTI or a PDCCH/EPDCCH of DCI format 0 for SPS C-RNTI or a PDCCH/SPDCCH of DCI format 7-0A/7-0B for SPS C-RNTI or a PDCCH/EPDCCH of DCI format 0 for UL-SPS-V-RNTI and a PDCCH of DCI format 3/3A with this UE's TPC-PUSCH-RNTI in every subframe except when in DRX or where serving cell  is deactivated.

-	For serving cell  and a BL/CE UE configured with CEModeA, the UE attempts to decode a MPDCCH of DCI format 6-0A with the UE's C-RNTI or SPS C-RNTI or PUR-RNTI and a MPDCCH of DCI format 3/3A with this UE's TPC-PUSCH-RNTI in every BL/CE downlink subframe except when in DRX

-	For a non-BL/CE UE and for subframe PUSCH transmissions, if DCI format 0/0A/0B/0C/4/4A/4B for serving cell and DCI format 3/3A are both detected in the same subframe, then the UE shall use the  provided in DCI format 0/0A/0B/0C/4/4A/4B.

-	For slot/subslot-PUSCH transmissions corresponding to a PDCCH/SPDCCH with DCI format 7-0A/7-0B with the UE's C-RNTI for serving cell c, the UE shall use the provided in DCI format 7-0A/7-0B

-	For serving cell c and slot/subslot-PUSCH transmissions without a corresponding PDCCH/SPDCCH in slot/subslot i of subframe I, the UE shall use the TPC command provided in the PDCCH with DCI format 3/3A received in a subframe not later than subframe  when configured by higher layer parameter tpc-PDCCH-ConfigPUSCH-SPS for the serving cell c and for the corresponding SPS-ConfigIndex-r14 if the UE is configured with multiple UL SPS configurations.

-	For a BL/CE UE configured with CEModeA, if DCI format 6-0A for serving cell and DCI format 3/3A are both detected in the same subframe, then the UE shall use the  provided in DCI format 6-0A.

-	dB for a subframe/slot/subslot where no TPC command is decoded for serving cell or where DRX occurs or i is not an uplink subframe/slot/subslot in TDD or FDD-TDD and serving cell c frame structure type 2.

-	dB for subframe-PUSCH transmissions if the subframe i is not the first subframe scheduled by a PDCCH/EPDCCH of DCI format 0B/4B.

-	The  dB accumulated values signalled on PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or PDCCH/SPDCCH with DCI format 7-0A/7-0B or MPDCCH with DCI format 6-0A are given in Table 5.1.1.1-2. If the PDCCH/EPDCCH with DCI format 0 or PDCCH/SPDCCH with DCI format 7-0A/7-0B or MPDCCH with DCI format 6-0A or PDCCH/SPDCCH with DCI format 7-0A/7-0B  is validated as a SPS activation or release PDCCH/EPDCCH/MPDCCH/SPDCCH, then  is 0dB.

-	The  dB accumulated values signalled on PDCCH/MPDCCH with DCI format 3/3A are one of SET1 given in Table 5.1.1.1-2 or SET2 given in Table 5.1.1.1-3 as determined by the parameter TPC-Index provided by higher layers.

-	If UE has reached  for serving cell , positive TPC commands for serving cell  shall not be accumulated

-	If UE has reached minimum power, negative TPC commands shall not be accumulated

-	For serving cell , when the UE is configured with higher layer parameter shortTTI or when there is a change in configuration corresponding to the higher layer parameter shortTTI,  for the first following PUSCH transmission in a slot or subslot in a given subframe is set to the value of   associated with PUSCH of the previous uplink subframe.

-	If the UE is not configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell , the UE shall reset accumulation

-	For serving cell , when  value is changed by higher layers

-	For serving cell , when the UE receives random access response message for serving cell

-	For serving cell , and BL/CE UE configured with CEModeA, when the UE performs PUSCH transmission using preconfigured uplink resource

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell ,

-	the UE shall reset accumulation corresponding to  for serving cell

-	when  value is changed by higher layers

-	when the UE receives random access response message for serving cell

-	the UE shall reset accumulation corresponding to  for serving cell

-	when  value is changed by higher layers

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v15x0 for serving cell , the UE shall reset accumulation corresponding to  for serving cell

-	when  value is changed by higher layers

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell  and

-	if subframe  belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12

-	if subframe  does not belong to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12

-	 and  if accumulation is not enabled for serving cell  based on the parameter Accumulation-enabled or accumulationEnabledsTTI provided by higher layers

-	where was signalled on PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or PDCCH/SPDCCH with DCI format 7-0A/7-0B or MPDCCH with DCI format 6-0A for serving cell  on subframe  . For a BL/CE UE configured with CEModeA, subframe i- KPUSCH-Koffset is the last subframe in which the MPDCCH with DCI format 6-0A or MPDCCH with DCI format 3/3A is transmitted.

-	The value of  is

-	For FDD or FDD-TDD and serving cell frame structure type 1

-	if the UE is configured with higher layer parameter shortTTI and for PUSCH transmissions in a subslot,

-	the UE is configured with higher layer parameters dl-STTI-Length='subslot' and ul-STTI-Length='slot' and for PUSCH transmissions in a slot, corresponds to:

-	a subslot among subslot 4 or 5 of subframe N-3 or subslot 0 of subframe N-2 in which the UE has received the TPC command if the slot- PUSCH is to be transmitted in slot 0 of subframe N. A UE is not expected to receive TPC command in more than one subslot among subslot 4 or 5 of subframe N-3 or subslot 0 of subframe N-2 corresponding to slot-PUSCH transmission in slot 0 of subframe N.

-	a subslot among subslot 1 or 2 or 3 of subframe N-2 in which the UE has received the TPC command if the slot-PUSCH is to be transmitted in slot 1 of subframe N. A UE is not expected to receive TPC command in more than one subslot among subslot 1 or 2 or 3 of subframe N-2 corresponding to slot-PUSCH transmission in slot 1of subframe N.

-	if configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	otherwise, = 4

-	For TDD, if the UE is configured with more than one serving cell and the TDD UL/DL configuration of at least two configured serving cells is not the same, or if the UE is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell, or FDD-TDD and serving cell frame structure type 2, the "TDD UL/DL configuration" refers to the UL-reference UL/DL configuration (defined in Clause 8.0) for serving cell .

-	For TDD UL/DL configurations 1-6 and UE not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell ,  is given

-	in Table 5.1.1.1-1A if the UE is configured with shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space for subframe-PUSCH transmissions,

-	in Table 5.1.1.1-1B for special subframe configuration 1, 2, 3, 4, 6, 7, 8 and Table 5.1.1.1-1C for special subframe configuration 0, 5, 9 if the UE is configured with higher layer parameter sTTI, and for uplink transmissions in a slot,

-	by  if the slot-PUSCH transmission in slot 15 or 16 is scheduled with a PDCCH/SPDCCH with DCI format 7-0A/7-0B in which the LSB of the UL index is set to 1 for TDD UL/DL configuration 6 and special subframe configuration 0, 5, 9,

-	in Table 5.1.1.1-1 otherwise.

-	For TDD UL/DL configuration 0 and UE not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell .

-	If the subframe-PUSCH transmission in subframe 2 or 7 is scheduled with a PDCCH/EPDCCH of DCI format 0/4 or a MPDCCH with DCI format 6-0A in which the LSB of the UL index is set to 1, = 7

-	For slot-PUSCH transmissions, is given by Table 5.1.1.1-1B for special subframe configuration 1, 2, 3, 4, 6, 7, 8 and Table 5.1.1.1-1C for special subframe configuration 0, 5, 9.

-	For all other PUSCH transmissions,  is given in Table 5.1.1.1-1A if the UE is configured with shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, Table 5.1.1.1-1 otherwise.

-	For TDD UL/DL configurations 0-5 and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell ,  is given in

-	Table 5.1.1.1-4A if the UE is configured with shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	Table 5.1.1.1-4B for slot-PUSCH transmissions,

-	Table 5.1.1.1-4 otherwise.

-	For TDD UL/DL configuration 6 and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell

-	If the subframe-PUSCH transmission in subframe 2 or 7 is scheduled with a PDCCH/EPDCCH of DCI format 0/4 if the UE is not configured with higher layer parameter shortProcessingTime or if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the common search space in which the LSB of the UL index is set to 1, = 6.

- If the subframe-PUSCH transmission in subframe s is scheduled with a PDCCH of DCI format 0/4, if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space in which the LSB of the UL index is set to 1,

- for s= 1 or 2

- for s= 3

-	If the slot-PUSCH transmission in slot 4, 15 or 16 is scheduled with a PDCCH/SPDCCH with DCI format 7-0A/7-0B in which the LSB of the UL index is set to 1, = 5.

-	For all other PUSCH transmissions,  is given in

-	Table 5.1.1.1-4A if the UE is configured with shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	Table 5.1.1.1-4B for slot- PUSCH transmissions,

-	Table 5.1.1.1-4 otherwise.

-	For a serving cell with frame structure type 3,

-	For an uplink DCI format 0A/4A carrying AUL-DFI according to clause 8.3A, =4.

-	For an uplink DCI format 0A/4A with CRC scrambled by C-RNTI with PUSCH trigger A set to 0, is equal to k+l, where k and l are defined in in Clause 8.0.

-	For an uplink DCI format 0B/4B with CRC scrambled by C-RNTI with PUSCH trigger A set to 0, is equal to k+l+i' with , where niHARQ_ID is HARQ process number in subframe i, and k, l, nHARQ_ID and NHARQ are defined in Clause 8.0.

-	For an uplink DCI format 0A/4A with CRC scrambled by C-RNTI with PUSCH trigger A set to 1 and upon the detection of PDCCH with CRC scrambled by CC-RNTI and with 'PUSCH trigger B' field set to '1' described in Clause 8.0, is equal to p+k+l, where p, k and l are defined in Clause 8.0.

-	For an uplink DCI format 0B/4B with CRC scrambled by C-RNTI with PUSCH trigger A set to 1 and upon the detection of PDCCH with CRC scrambled by CC-RNTI and with 'PUSCH trigger B' field set to '1' described in Clause 8.0, is equal to p+k+l+i' with , where niHARQ_ID is HARQ process number in subframe i, and p, k, l, nHARQ_ID and NHARQ are defined in Clause 8.0.

- If a UE detected multiple TPC commands in subframe , the UE shall use the TPC command in the PDCCH/EPDCCH with DCI format 0A/0B/0C/4A/4B with CRC scrambled by C-RNTI which schedules PUSCH transmission in subframe i.

-	The  dB absolute values signalled on PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or PDCCH/SPDCCH with DCI format 7-0A/7-0B or a MPDCCH with DCI format 6-0A are given in Table 5.1.1.1-2. If the PDCCH/EPDCCH with DCI format 0 or PDCCH/SPDCCH with DCI format 7-0A/7-0B or a MPDCCH with DCI format 6-0A is validated as a SPS activation or release PDCCH/EPDCCH/MPDCCH/SPDCCH, then  is 0dB.

-	for a non-BL/CE UE,  and  for a subframe where no PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B is decoded for serving cell  or where DRX occurs or i is not an uplink subframe in TDD or FDD-TDD and serving cell c frame structure type 2.

-	for a BL/CE UE configured with CEModeA, and  for a subframe where no MPDCCH with DCI format 6-0A is decoded for serving cell  or where DRX occurs or i is not an uplink subframe in TDD.

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell  and

-	if subframe  belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12

-	if subframe  does not belong to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12

-	For both types of  (accumulation or current absolute) the first value is set as follows:

-	If  value is changed by higher layers and serving cell  is the primary cell or, if  value is received by higher layers and serving cell  is a Secondary cell or, if a BL/CE UE transmits using preconfigured uplink resources,

-

-	Note for a BL/CE UE, fci = 0 for i = 0,1,… up to the last subframe of the PUSCH transmission using preconfigured uplink resource.

-	For serving cell , when the UE is configured with higher layer parameter shortTTI or when there is a change in configuration corresponding to the higher layer parameter shortTTI,  for the first following PUSCH transmission in a slot or subslot in a given subframe is set to the value of  associated with PUSCH of the previous uplink subframe.

-	Else

-	If the UE receives the random access response message for a serving cell

-	, where

-	 is the TPC command indicated in the random access response corresponding to the random access preamble transmitted in the serving cell , see Clause 6.2, and

and  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble in the serving cell , is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks valid for the subframe of first PUSCH transmission in the serving cell , and TF,c(0) is the power adjustment of first PUSCH transmission in the serving cell c. If a UE is performing non-contention based random access procedure and is configured with higher layer parameter pusch-EnhancementsConfig, δmsg2=0. For a PUSCH transmission corresponding to ul-ConfigInfo-r14, the UE shall assume ΔPrampuprequested,c=δmsg2,c=0

-	If value is received by higher layers for a serving cell c.

-

Table 5.1.1.1-1:  for TDD configuration 0-6

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6 | 7 | 4 | - | - | 6 | 7 | 4 |
| 1 | - | - | 6 | 4 | - | - | - | 6 | 4 | - |
| 2 | - | - | 4 | - | - | - | - | 4 | - | - |
| 3 | - | - | 4 | 4 | 4 | - | - | - | - | - |
| 4 | - | - | 4 | 4 | - | - | - | - | - | - |
| 5 | - | - | 4 | - | - | - | - | - | - | - |
| 6 | - | - | 7 | 7 | 5 | - | - | 7 | 7 | - |

Table 5.1.1.1-1A:  for TDD configuration 0-6, special subframe configuration 0, 1, 2, 3, 4, 5, 6, 7, 8, 9 and UE configured with shortProcessingTime

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6 | 3 | 3 | - | - | 6 | 3 | 3 |
| 1 | - | - | 3 | 3 | - | - | - | 3 | 3 | - |
| 2 | - | - | 3 | - | - | - | - | 3 | - | - |
| 3 | - | - | 3 | 3 | 3 | - | - | - | - | - |
| 4 | - | - | 3 | 3 | - | - | - | - | - | - |
| 5 | - | - | 3 | - | - | - | - | - | - | - |
| 6 | - | - | 6 | 4 | 4 | - | - | 6 | 3 | - |

Table 5.1.1.1-1B:  for TDD configurations 0-6, special subframe configuration 1, 2, 3, 4, 6, 7, 8 and UE configured with ul-STTI-Length

| TDD UL/DL Configuration | slot number i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 |  |  |  |  | 4 | 4 | 4 | 5 | 5 | 6 |  |  |  |  | 4 | 4 | 4 | 5 | 5 | 6 |
| 1 |  |  |  |  | 4 | 4 | 4 | 4 |  |  |  |  |  |  | 4 | 4 | 4 | 4 |  |  |
| 2 |  |  |  |  | 4 | 4 |  |  |  |  |  |  |  |  | 4 | 4 |  |  |  |  |
| 3 |  |  |  |  | 6 | 6 | 6 | 6 | 6 | 6 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  | 4 | 4 | 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  | 6 | 6 | 6 | 6 | 6 | 6 |  |  |  |  | 4 | 4 | 4 | 4 |  |  |

Table 5.1.1.1-1C:  for TDD configurations 0-6, special subframe configuration 0, 5, 9 and UE configured with ul-STTI-Length

| TDD UL/DL Configuration | slot number i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 |  |  |  |  | 4 | 5 | 5 | 6 | 6 | 7 |  |  |  |  | 4 | 5 | 5 | 6 | 6 | 7 |
| 1 |  |  |  |  | 5 | 5 | 5 | 5 |  |  |  |  |  |  | 5 | 5 | 5 | 5 |  |  |
| 2 |  |  |  |  | 4 | 4 |  |  |  |  |  |  |  |  | 4 | 4 |  |  |  |  |
| 3 |  |  |  |  | 7 | 7 | 7 | 7 | 7 | 7 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  | 5 | 5 | 5 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  | 4 | 5 | 5 | 6 | 6 | 7 |  |  |  |  | 4 | 4 | 4 | 5 |  |  |

Table 5.1.1.1-2: Mapping of TPC Command Field in DCI format 0/0A/0B/0C/3/4/4A/4B/6-0A/3B/7-0A/7-0B to absolute and accumulated  values

| TPC Command Field in DCI format 0/0A/0B/0C/3/4/4A/4B/6-0A/3B/7-0A/7-0B | Accumulated [dB] | Absolute [dB] only DCI format 0/0A/0B/0C/4/4A/4B/6-0A/7-0A/7-0B |
| --- | --- | --- |
| 0 | -1 | -4 |
| 1 | 0 | -1 |
| 2 | 1 | 1 |
| 3 | 3 | 4 |

Table 5.1.1.1-3: Mapping of TPC Command Field in DCI format 3A/3B to accumulated  values

| TPC Command Field in DCI format 3A/3B | Accumulated [dB] |
| --- | --- |
| 0 | -1 |
| 1 | 1 |

Table 5.1.1.1-4:  for TDD configuration 0-6 and UE configured with symPUSCH-UpPts-r14

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | 5 | 7 | 7 | 4 | - | 5 | 7 | 7 | 4 |
| 1 | - | 6 | 6 | 4 | - | - | 6 | 6 | 4 | - |
| 2 | - | 5 | 4 | - | - | - | 5 | 4 | - | - |
| 3 | - | 4 | 4 | 4 | 4 | - | - | - | - | - |
| 4 | - | 4 | 4 | 4 | - | - | - | - | - | - |
| 5 | - | 4 | 4 | - | - | - | - | - | - | - |
| 6 | - | 6 | 7 | 7 | 5 | - | 6 | 7 | 7 | - |

Table 5.1.1.1-4A:  for TDD configuration 0-6 and UE configured with symPUSCH-UpPts-r14 and UE configured with shortProcessingTime

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | 6 | 6 | 3 | 3 | - | 6 | 6 | 3 | 3 |
| 1 | - | 5 | 3 | 3 | - | - | 5 | 3 | 3 | - |
| 2 | - | 3 | 3 | - | - | - | 3 | 3 | - | - |
| 3 | - | 3 | 3 | 3 | 3 | - | - | - | - | - |
| 4 | - | 3 | 3 | 3 | - | - | - | - | - | - |
| 5 | - | 3 | 3 | - | - | - | - | - | - | - |
| 6 | - | 5 | 3 | 3 | 4 | - | 5 | 6 | 3 | - |

Table 5.1.1.1-4B:  for TDD configurations 0-6 and UE configured with symPUSCH-UpPts-r14, and UE configured with ul-STTI-Length

| TDD UL/DL Configuration | slot number i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 |  |  |  | 11 | 4 | 5 | 5 | 6 | 6 | 7 |  |  |  | 11 | 4 | 5 | 5 | 6 | 6 | 7 |
| 1 |  |  |  | 5 | 5 | 5 | 5 | 5 |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 |  |  |
| 2 |  |  |  | 4 | 4 | 4 |  |  |  |  |  |  |  | 4 | 4 | 4 |  |  |  |  |
| 3 |  |  |  | 7 | 7 | 7 | 7 | 7 | 7 | 7 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  | 5 | 5 | 5 | 5 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  | 4 | 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  | 4 | 4 | 5 | 5 | 6 | 6 | 7 |  |  |  | 11 | 4 | 4 | 4 | 5 |  |  |

For a given serving cell, if the UE is configured with higher layer parameter shortProcessingTime , the UE shall use the TPC command received with DCI format 0/4 mapped onto the UE-specific search space for subframe i.

For a UE capable of simultaneous transmission of different uplink signal durations to different serving cells as indicated by UE capability simultaneousTx-differentTx-duration, if PUSCH/PUCCH transmissions of different duration occur in different serving cells and in the same subframe and if the total transmit power of the UE would exceed , the UE shall follow the dropping rules described in Clause 5.1.5 until the total transmit power of the UE would not exceed or until there are only PUSCH/PUCCH transmissions of the same duration remaining in which case the following rules apply.

If the UE is not configured with an SCG or a PUCCH-SCell, and if the total transmit power of the UE would exceed , the UE scales for the serving cell  in subframe/slot/subslot i such that the condition

is satisfied where  is the linear value of ,  is the linear value of ,  is the linear value of the UE total configured maximum output power  defined in [6] in subframe/slot/subslot i and is a scaling factor of for serving cell  where . In case there is no PUCCH transmission in subframe/slot/subslot i .

If the UE is not configured with an SCG or a PUCCH-SCell, and if the UE has PUSCH transmission with UCI on serving cell j and PUSCH without UCI in any of the remaining serving cells, and the total transmit power of the UE would exceed , the UE scales for the serving cells without UCI in subframe/slot/subslot i such that the condition

is satisfied where  is the PUSCH transmit power for the cell with UCI andis a scaling factor of for serving cell  without UCI. In this case, no power scaling is applied to  unless and the total transmit power of the UE still would exceed .

For a UE not configured with a SCG or a PUCCH-SCell, note thatvalues are the same across serving cells whenbut for certain serving cells may be zero.

If the UE is not configured with an SCG or a PUCCH-SCell, and if the UE has simultaneous PUCCH and PUSCH transmission with UCI on serving cell j and PUSCH transmission without UCI in any of the remaining serving cells, and the total transmit power of the UE would exceed , the UE obtains according to

and

If the UE is not configured with a SCG or a PUCCH-SCell, and

-	If the UE is configured with multiple TAGs, and if the PUCCH/PUSCH transmission of the UE on subframe/slot/subslot  for a given serving cell in a TAG overlaps some portion of the first symbol of the PUSCH transmission on subframe/slot/subslot  for a different serving cell in another TAG the UE shall adjust its total transmission power to not exceed on any overlapped portion.

-	If the UE is configured with multiple TAGs, and if the PUSCH transmission of the UE on subframe/slot/subslot  for a given serving cell in a TAG overlaps some portion of the first symbol of the PUCCH transmission on subframe/slot/subslot  for a different serving cell in another TAG the UE shall adjust its total transmission power to not exceed on any overlapped portion.

-	If the UE is configured with multiple TAGs, and if the trigger type 0/1 SRS transmission of the UE in a symbol on subframe/slot/subslot  for a given serving cell in a TAG overlaps with the PUCCH/PUSCH transmission on subframe/slot/subslot or subframe/slot/subslot  for a different serving cell in the same or another TAG the UE shall drop trigger type 0/1 SRS if its total transmission power exceeds on any overlapped portion of the symbol.

-	If the UE is configured with multiple TAGs, and if the trigger type 2 SRS transmission of the UE in a symbol on subframe/slot/subslot i for a given serving cell in a TAG overlaps with the PUCCH/PUSCH transmission on subframe/slot/subslot i, i-1 or i+1 for a different serving cell in the same or another TAG, the UE shall drop the trigger type 2 SRS in the overlapped symbol if its total transmission power exceeds PCMAX on any overlapped portion of the symbol.

-	If the UE is configured with multiple TAGs and more than 2 serving cells, and if the trigger type 0/1 SRS transmission of the UE in a symbol on subframe/slot/subslot  for a given serving cell overlaps with the SRS transmission on subframe/slot/subslot  for a different serving cell(s) and with PUSCH/PUCCH transmission on subframe/slot/subslot or subframe/slot/subslot  for another serving cell(s) the UE shall drop the trigger type 0/1 SRS transmissions if the total transmission power exceeds on any overlapped portion of the symbol.

-	If the UE is configured with multiple TAGs and more than 2 serving cells, and if the trigger type 2 SRS transmission of the UE in a symbol on subframe/slot/subslot i for a given serving cell overlaps with the SRS transmission on subframe/slot/subslot i-1, i or i+1 for a different serving cell(s) and with PUSCH/PUCCH transmission on subframe/slot/subslot i-1, i or i+1 for another serving cell(s), the UE shall drop the trigger type 2 SRS transmission in the overlapped symbol if the total transmission power exceeds PCMAX on any overlapped portion of the symbol.

-	If the UE is configured with multiple TAGs, the UE shall, when requested by higher layers, to transmit PRACH in a secondary serving cell in parallel with SRS transmission in a symbol on a subframe of a different serving cell belonging to a different TAG, drop SRS if the total transmission power exceeds on any overlapped portion in the symbol.

-	If the UE is configured with multiple TAGs, the UE shall, when requested by higher layers, to transmit PRACH in a secondary serving cell in parallel with PUSCH/PUCCH in a different serving cell belonging to a different TAG, adjust the transmission power of PUSCH/PUCCH so that its total transmission power does not exceed on the overlapped portion.

If the UE is configured with a LAA SCell for uplink transmissions, the UE may compute the scaling factor  assuming that the UE performs a PUSCH transmission on the LAA SCell in subframe i irrespective of whether the UE can access the LAA SCell for the PUSCH transmission in subframe i according to the channel access procedures described in Clause 4.2.1 of [13].

For a BL/CE UE configured with CEModeA, if the PUSCH transmission, scheduled by one DCI, is transmitted in more than one subframe i0, i1, …, iN-1 where i0< i1< …< iN-1, the PUSCH transmit power in subframe ik , k=0, 1, …, N-1, is determined by

except in case the DCI contains an UL index set to ‘11’.

For a BL/CE UE configured with CEModeB, the PUSCH transmit power in subframe ik is determined by

<Unchanged parts are omitted>