3GPP TSG RAN WG1 #107-e														R1-211XXXX

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.12.3

Source: 			Moderator (BBC)

Title:	Feature lead summary #2 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states

Document for:	Discussion and Decision

# Introduction

During TSG RAN #86, 3GPP approved a Release-17 Work Item (WI) to introduce support for Multicast and Broadcast Services in NR (NR MBS) [1]. The NR MBS WI includes the following objective:

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states [RAN2, RAN1]: - Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception. [RAN2, RAN1]. Note: the possibility of receiving Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, without the need for those UEs to get the configuration of the PTM bearer carrying the Broadcast/Multicast service while in RRC CONNECTED state beforehand, is subject to verification of service subscription and authorization assumptions during the WI.

The agreements for AI 8.12.3 on Basic functions for broadcast/multicast for RRC_IDLE/ RRC_INACTIVE UEs in previous RAN1 and RAN Plenary meetings are listed in the Annex A of this document.

As announced by the Chair, the email discussion details with check points for agreements are as follows:

[107-e-NR-MBS-03] Email discussion/approval on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs with checkpoints for agreements on November 15 and 19 – David (BBC)

In this document, the Feature Lead (FL) presents a list of open Issues to enable reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states based on the technical documents (tdocs) submitted to RAN1#107-e. Each of the Issues has the following subsections: background, Tdoc analysis, FL assessment and a set of proposals that are updated based on rounds of discussion between companies. The final section of this document also contains the agreements reached at RAN1#107-e.

The reader can use the “Navigation Pane” utility of Word to quickly find the Issues and the rounds of discussion for the set of Proposals for this meeting.

# Issues

## [UPDATE] Issue 1: PDCCH: Design of DCI format for MCCH and MTCH channels

### Background

The following agreements at RAN1#105-e and RAN1#106-e on RRC idle/inactive UEs are relevant for this discussion:

Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, DCI format 1_0 is used as baseline for GC-PDCCH of MCCH and MTCH. - FFS details of FDRA. Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, study the following alternatives for MCCH change notification indication due to session start: - Alt 1: Define a dedicated RNTI to scramble the CRC of a DCI indicating a MCCH change notification; - Alt 2: Use of a field in a DCI format scheduling a MCCH without a dedicated RNTI for MCCH change notification; Other solutions are not precluded and it is also not precluded whether to support both Alt1 and Alt2. Conclusion: It is up to RAN2 to decide the specific contents of the MCCH change notification, e.g, whether notification only informs about session start, whether or not notification also informs about session modification/stop or whether or not the notification informs about any other information. Agreement: Study and reach an agreement by RAN1#106b-e on whether Alt1 and Alt2 for MCCH change notification indication can accommodate at least 2 bits for the notification of MCCH configuration changes due to a session start and the notification of MCCH configuration changes of an ongoing session (including session stop). Agreement: The DCI format for GC-PDCCH scheduling a GC-PDSCH carrying MCCH/MTCH at least includes the following fields for broadcast reception with UEs in RRC_IDLE/INACTIVE state: - FDRA field - TDRA field - Modulation and coding scheme - Redundancy version - FFS: - MCCH change notification (if supported and only for MCCH), - RB numbering starts from the lowest RB of the CFR and support of resource allocation with granularity of single or multiple RBs. - HARQ process number and New data indicator - VRB-to-PRB mapping - other fields if needed. Agreement: For broadcast reception with UEs in RRC_IDLE/INACTIVE state, the DCI size of GC-PDCCH scheduling a GC-PDSCH carrying MCCH/MTCH is aligned with DCI format 1_0 with CRC scrambled by C-RNTI in the CSS.

The following agreements at RAN1#106bis-e on RRC connected UEs are relevant for this discussion:

Agreement: For FDRA determination of the first DCI format for GC-PDCCH, Option 2 is supported. - Option 2: - is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; and - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell. - For resource indication value (RIV) of downlink resource allocation type 1, the similar scheme as for the case that the DCI size for DCI format 1_0 in USS is derived from the size of DCI format 1_0 in CSS but applied to an active BWP is used. - If the size of CFR (i.e. NCFR) is larger than the size of CORESET0/initial DL bandwidth part, the resource indication value (RIV) is defined as in section 5.1.2.2.2 in TS38.214, where K is the maximum value from set {1, 2, 4, 6, 8, 10, 12} which satisfies K≤NCFR/NBWPinitial;otherwise, K=1. Agreement: For GC-PDSCH scheduled with the first DCI format for multicast, RB numbering starts from the lowest RB of the CFR.

### Tdoc analysis

- In [R1-2110779, Huawei]

  - Discuss: Besides, the field for MCCH change notification has been reflected in the CR of TR38.212.

  - Discus: DCI format 1_0 scrambled by G-RNTI can be used for multicast scheduling and broadcast scheduling. However, the fields needed for multicast are not useful for broadcast, e.g., HARQ process number, New data indicator. VRB-to-PRB mapping can increase the frequency diversity gain for resource allocation type1, so that it can be included in the DCI format. TB scaling field is used for increasing robustness for the transmission of paging message or random access response, which does not seem useful for MTCH.

  - Proposal 5: For DCI format 1_0 scrambled by MCCH-RNTI/G-RNTI for MCCH/MTCH, at least the following field can be included in addition to those fields have been agreed:

    - VRB-to-PRB mapping

- In [R1-2110897, TD Tech]

  - Discuss: Because the interleaved VRB-to-PRB mapping will bring the SNR gain in UE, we suggest to add the field VRB-to-PRB in the DCI format for MCCH/MTCH.

  - Proposal 14: The following field is included in the DCI format for MCCH/MTCH:

    - VRB-to-PRB mapping

- In [R1-2110912, ZTE]

  - Discuss: Regarding TDRA, VRB-to-PRB mapping, MCS and RV field, they are the same as the legacy UE behaviour.
Regarding MCCH change notification, as agreed in RAN1#106b-e meeting, it should be included in the DCI scheduling MCCH. In order to share the same DCI fields design for DCI scheduling MCCH and DCI scheduling MTCH of broadcast, we can directly include MCCH change notification in the DCI format.
Regarding the bit size of FDRA, it is proposed to have the same handling as what we agreed in last meeting for the first DCI format for multicast. Also, if FDRA is determined by the CFR, it may end up with different FDRA bit sizes for MCCH scheduling and MTCH scheduling since the CFR size may be different for MCCH and MTCH. Thus, to have a unified solution, it is preferred to use the same mechanism for MCCH and MTCH of broadcast.

  - Proposal 7: The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by MCCH-RNTI/G-RNTI for broadcast:

    - Frequency domain resource assignment – bits

      - is the size of CORESET 0

      - If the size of CFR (i.e. NCFR) is larger than the size of CORESET0, the resource indication value (RIV) is defined as in section 5.1.2.2.2 in TS38.214, where K is the maximum value from set {1, 2, 4, 6, 8, 10, 12} which satisfies K≤NCFR/NBWPinitial;otherwise, K=1.

    - Time domain resource assignment – 4 bits

    - VRB-to-PRB mapping – 1 bit

    - Modulation and coding scheme – 5 bits

    - Redundancy version – 2 bits

    - MCCH change notification – 2 bits if the CRC of the DCI format 1_0 is scrambled by MCCH-RNTI. Otherwise, this bit field is reserved.

    - Reserved bits – 14 bits

- In [R1- 2111041, vivo]

  - Discuss: Regarding the DCI field for broadcast in RRC_IDLE/RRC_INACTIVE, besides FDRA, TDRA, MCS, and RV, HARQ process number and new data indicator should also be included as slot-level repetition for MTCH has been already supported.

  - Discuss: For UE in RRC_IDLE/RRC_INACTIVE, as no unicast and groupcast transmission is expected, HARQ processes defined for unicast and groupcast can be used for combing broadcast PDSCH repetition. And it is possible to leave it to RRC_IDLE/RRC_INACTIVE UE implementation to select one HPN without any indication in DCI. However, broadcast PDSCH with repetition can be also received by RRC_CONNECTED UE, if HPN and NDI is not indicated in DCI and RRC_CONNECTED UE randomly chooses a free HPN for combination, then it will cause chaos for further unicast and multicast reception. This is because network has no information about the HPN selected by UE for broadcast PDSCH combination and may indicate the same one for the subsequent unicast and multicast transmission.

  - Proposal 4: HARQ process number and new data indicator should be included in the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH carrying MTCH.

- In [R1-2111137, Nokia]

  - Proposal-14: Confirm DCI format 1_0 is sufficient for broadcast reception for RRC_IDLE/INACTIVE UEs.

    - FFS: If DCI format other than format 1_0 is agreed, further discuss the resource allocation type applied for Rel17 broadcast for RRC_IDLE/INACTIVE UEs.

  - Proposal-15: If DCI format 1_0 is the only DCI format to be supported, the VRB-to-PRB mapping can be fixed with interleaved, and this field is not needed.

  - Proposal-16: Considering of TB scaling field be included in the DCI.

  - Proposal-17: It is beneficial to support of HARQ combining for broadcast with slightly increase UE complexity, where a single additional dedicated HARQ process seems to be sufficient for all broadcast services associated with different G-RNTIs. And there is no need of including HARQ process number in the DCI field.

  - Proposal-18: It is beneficial to support NDI in the DCI field for broadcast to assist UE RV combining.

  - Proposal-19: Further discuss other fields to be included in the DCI, i.e. MCCH change notification field (if supported for MCCH), and TRS related field (if supported for MTCH).

- In [R1-2111232, CATT]

  - Discuss: It has been agreed that for RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, DCI format 1_0 is used as baseline for GC-PDCCH of MCCH and MTCH, so a fixed 1 bit VRB-to-PRB mapping field should be included in in DCI format for broadcast reception with UEs in RRC_IDLE/INACTIVE state. It implies that only the interleaved VRB-to-PRB mapping is supported for the group-common DCI.
Regarding HARQ process number (HPN) and New data indicator (NDI), to improve the reliability of UE in the edge of cell, blind retransmission should be applied. So, HPN and NDI fields should be included for the soft-combination.

  - Proposal 10: MCCH change notification, HARQ process number, new data indicator and VRB-to-PRB mapping (1 bit) fields can be included in the DCI format.

- In [R1-2111305, OPPO]

  - Proposal 9: The DCI format for GC-PDCCH scheduling a GC-PDSCH carrying MCCH/MTCH additionally includes the following fields for broadcast reception with UEs in RRC_IDLE/INACTIVE state:

    - Modulation and coding scheme

    - Reserve bits.

  - Proposal 10: The size of FDRA field in DCI for scheduling GC-PDSCH carrying MCCH/MTCH can be determined by the configuration of CFR used for broadcast MBS services transmission, i.e. CORESET#0 or SIB1 configured initial BWP.

- In [R1-2111518, Intel]

  - Discuss: UEs in RRC IDLE/INACTIVE mode can be configured to monitor for DCI format 1_0 which can be used to schedule MBS PDSCH. Based on the subsequent agreement that the CFR is assumed to have the same size of the CORESET#0 or initial BWP, the FDRA field in the for the DCI 1_0 will be dimensioned with respect to the CORESET#0 or initial BWP. This is assuming that Case E as proposed above is agreed. Otherwise special handling for Case E may be required.

  - Proposal 5: The FDRA field of DCI 1_0 is based on the starting PRB index and size of CORESET#0 or initial BWP.

- In [R1-2111551, Xiaomi]

  - Discuss: Regarding VRB-to-PRB mapping information field, we think it’s better to keep it in the DCI considering there are sufficient room for such 1 bit information. It can provide more flexibility for scheduling especially considering multiplexing between MBS PDSCH and SIB PDSCH is necessary in some cases. Furthermore, notification of MCCH configuration changes should be included in the DCI scheduling GC-PDSCH. All the other fields should be reserved.

  - Proposal 12: DCI format 1_0 with CRC scrambled by G-RNTI is used to schedule a GC-PDSCH carrying MCCH/MTCH for broadcast reception with UEs in RRC_IDLE/INACTIVE state, including the following information fields:

    - FDRA field

    - TDRA field

    - VRB-to-PRB mapping

    - Modulation and coding scheme

    - Redundancy version

    - MCCH configuration change notification

    - Reserved bits

  - Proposal 13: The FDRA should be determined by CORESET#0 or initial DL BWP if CORESET#0 is not configured.

- In [R1-2111629, CMCC]

  - Discuss: As for blind retransmission soft buffer combination, dedicated HARQ process is defined for system information in NR and similar mechanism can be used for broadcast service. In addition, as different G-RNTI are used to differentiate broadcast services, which means the HARQ buffer has one to one mapping with G-RNTI, it is up to UE’s implementation to buffer different services in different HARQ buffer without the necessary to be indicated the HARQ process number in the DCI.

  - Proposal 5. HARQ process number and New data indicator are not needed in the DCI format for GC-PDCCH scheduling a GC-PDSCH carrying MCCH/MTCH.

  - Proposal 6. The FDRA field bit length in DCI format for GC-PDCCH scheduling a GC-PDSCH carrying MCCH/MTCH is related to CFR size but not CORESET#0 size and the resource allocation granularity is single RB.

- In [R1-2112130, NTT DOCOMO]

  - Observation 1: If the existing RB numbering rule for PDSCH scheduled with DCI format 1_0 in CSS is reused for PDSCH scheduled with the DCI format scheduling MCCH/MTCH, there may be RBs that cannot be allocated GC-PDSCH.

  - Proposal 4: For GC-PDSCH carrying MCCH/MTCH, RB numbering starts from the lowest RB of the CFR.

  - Proposal 5: Include VRB-to-PRB mapping field in the DCI format scheduling MCCH/MTCH.

- In [R1-2112163, Lenovo]

  - Discuss: Consequently, the number of bits required for FDRA indicator is based on the bandwidth of CORESET 0 or SIB-1 configured initial DL BWP. Since RAN1 has already agreed that the group-common DCI format has same payload size as DCI format 1_0 with CRC scrambled by C-RNTI and monitored in CSS, and the FDRA field of DCI format 1_0 with CRC scrambled by C-RNTI and monitored in CSS is also given by the bandwidth of CORESET 0 or SIB-1 configured initial DL BWP, it is quite easy to align both DCI payload size.

  - Proposal 4: The number of bits for FDRA in the group-common DCI is determined based on the CFR in Case A and Case C with single RB granularity.

  - Proposal 5: RB numbering starts from the lowest RB of the CFR and the granularity of resource allocation only supports single RB.

  - Proposal 6: The number of bits in TDRA field in the group-common DCI format is determined by the number of entries in the time domain resource allocation table configured for MBS.

  - Discuss: Regarding VRB-to-PRB mapping, this field can be also configurable for group-common PDSCH transmission. The size is 0 bit if only resource allocation type 0 is configured or if interleaved VRB-to-PRB mapping is not configured for group-common PDSCH transmission, or 1 bit otherwise, only applicable to resource allocation type 1.

  - Proposal 7: VRB-to-PRB mapping in the group-common DCI format is 0 or 1 bit dependent on RRC configuration.

  - Proposal 8: For HARQ combining, 5 bits MCS, 1 bit NDI, 2 bits RV and 4 bits HARQ process number are included in the group-common DCI format.

  - Proposal 9: DAI/TPC/PRI/HARQ-timing indicator in the group-common DCI are removed.

  - Proposal 10: New field is introduced for indicating MCCH change notification.

  - Proposal 11: Support fields and sizes in Table 1 for the first DCI format.

  - Proposal 12: Zero bits are appended to the group-common DCI format in case its size prior to padding is smaller than the size of DCI format 1-0 with CRC scrambled by C-RNTI and monitored in CSS.

  - Discuss: Considering coverage is critic to broadcast transmission, the 2nd DCI format is not supported for RRC Idle/Inactive UEs.

  - Proposal 13: The second group-common DCI format is not supported for RRC Idle/Inactive UEs for broadcast reception.

- In [R1-2112314, MediaTek]

  - Discuss: Regarding the “HARQ process number” field, we think there is no clear motivation to add this. Even though the slot level repetition is supported for MTCH, the similar mechanism for SIB combining can be reused and it does not need to add a new “HARQ process number” field for broadcast reception. In addition, if more HARQ process are introduced for MBS broadcast services, it is not friendly to UE’s power saving and needs more device hardware affect, which is against the current WID description as copied following: Restrictions and assumptions in MBS WID: In order to facilitate implementation and deployment of the feature, the overall implementation impact should be limited, and the UE complexity should be minimized (e.g. device hardware impact should be avoided).

  - Proposal 7: “HARQ process number” field is not supported for MBS broadcast DCI.

- In [R1-2112348, Ericsson]

  - Observation 14: Multicast and broadcast can share the same DCI formats, with broadcast specific and multicast-specific fields made optional.

  - Observation 15: A broadcast DCI format limited to be based on legacy DCI format 1_0 would limit the potential of NR broadcast, by not allowing for cross-polar (two layer) MIMO or frequency diversity supported by Resource allocation Type 0.

  - Proposal 20: Support a first DCI format for broadcast, which is the same as the first DCI format for multicast, with broadcast specific and multicast-specific fields made optional.

  - Proposal 21: Support a second DCI format for broadcast, which is the same as the second DCI format for multicast, with broadcast specific and multicast-specific fields made optional.

### FL Assessment

This issue was discussed at the last meeting without reaching a conclusion.

On DCI format 1_0 fields for MCCH / MTCH

- FDRA

[OPPO, Intel, Xiaomi, CMCC, Lenovo] propose that the size of the FDRA field is determined by the size of the CFR. This aspect of the size of the FDRA was already discussed based on a proposal at the last meeting. However, it was discussed to wait until the conclusion of a related discussion in AI 8.12.1. However, as per the background section, an agreement has already been reached for AI 8.12.1. Another concern brought was whether there could be a mismatch between the CFR received by idle/inactive UEs and connected UEs. However, this has not been discussed in any of the submitted tdocs and it is also FL’s understanding that the FDRA parameters in connected state also depends on the CFR which would align both RRC states. Proposal 2.1-1 is put forward for discussion.

[Intel, NTT DOCOMO, Lenovo] propose that the starting PRB index is based on the CFR, rather than CORESET#0 as per existing numbering rule legacy with DCI format 1_0 in CSS. Proposal 2.1-2 is therefore put for discussion that also aligns with an agreement in AI 8.12.1.

[CMCC, Lenovo] propose that the resource allocation for broadcast is a single RB providing increased scheduling flexibility. However, [ZTE] proposes to have the same handling to what has been agreed for multicast in AI 8.12.1 to have a unified solution, which allows to have RB granularity larger than one. Although not many companies have discussed the resource allocation granularity for broadcast, the starting point for discussion is single RB resource allocation as per Proposal 2.1-3.

- HARQ Process Number (HPN)

[Huawei, Nokia, CMCC, MediaTek] discuss that HPN is not necessary for broadcast reception. However, [vivo, CATT, Lenovo] propose to include HPN. [vivo] discusses that it is necessary, based on the support of PDSCH repetition for broadcast reception. Although, this parameter may not be needed solely on the operation of RRC idle/inactive UEs, since broadcast can also be received in RRC connected UEs, these two parameters are needed to avoid collisions between HARQ processes used for unicast/multicast and broadcast.

This discussion is related to the progress on PDSCH repetition. However, companies have already discussed different options for the operation of HARQ process at the UE. Based on the discussion from [vivo], it seems this parameter is needed for the case when HARQ processes are shared between unicast/multicast and broadcast in RRC connected state. To promote discussion, the starting point is that HPN field is not included as per Proposal 2.1-4 (for conclusion) to collect companies’ views.

- New Data Indicator (NDI)

[Huawei, CMCC] discuss that NDI is not necessary for broadcast reception. However, [vivo, CATT, Lenovo] propose that NDI is necessary. [vivo] argues its introduction based on the support of PDSCH repetition for broadcast reception. Although, this parameter may not be needed solely on the operation of RRC idle/inactive UEs, since broadcast can also be received in RRC connected UEs, these two parameters are needed to avoid collisions between HARQ processes used for unicast/multicast and broadcast. [Nokia] also supports the introduction of NDI field to assist the UE RV combining. Although there is no clear position to include this field the starting point for the discussion is to include this field as per Proposal 2.1-5 to collect company views.

- VRB-to-PRB mapping

[Huawei, TD Tech, ZTE, Xiaomi, NTT DOCOMO] support to include this field where the main argument to included it is the increased diversity gain that can be achieved, which is especially useful for broadcast reception without knowledge at the gNB of the UEs’ channel frequency responses. [Nokia, CATT], proposes to fix this parameter to interleaved, where [Nokia] further argues that this field could therefore be saved. On the other hand, [Xiaomi] argues that is adequate to keep the field due to cases when MBS PDSCH and SIB PDSCH need to be multiplexed. [Lenovo] proposes that whether this field is included depends on RRC configuration and only for type 1 resource allocation. Given the different alternatives have been proposed for this field, Question 2.1-6 below discusses the 3 options in the table to collect companies’ views.

- TB scaling field

[Huawei] discusses that this field does not seem useful for MTCH. [Nokia] proposes a discussion on the potential inclusion of this field that could provide increased robustness for broadcast reception. However, most companies have included this field explicitly when listing the possible fields that are can be included in the DCI. Therefore, the starting point for the discussion is that TB scaling field is not included and Proposal 2.1-7 (for conclusion) is put forward to collect companies’ views.

- MCCH change notification and TRS related fields

The MCCH change notification was agreed at the last meeting as Working Assumption and has been included in the Draft CR of TS 38.212. The inclusion of TRS related fields depends on whether the respective functionality is supported pending the discussion other Issues in this summary.

On first and second DCI format for broadcast and unified solution with multicast

[Ericsson] proposes that in order to have an unified solution between multicast and broadcast, both can share the same DCI formats where broadcast-specific and multicast-specific-files are made optional. Building on this, they also propose to use a second DCI format for broadcast, which is the same as the second DCI for multicast where broadcast-specific and multicast-specific-files are made optional. Having a second DCI for broadcast would allow cross-polar MIMO and/or frequency diversity by resource allocation Type 0. On the other hand, [Lenovo] proposes that a second DCI format for broadcast is not supported arguing that a second DCI would not meet stringent coverage requirements required for broadcast reception.

Given that not many companies have discussed this aspect but has been first introduced in this meeting (for the second DCI aspect), Question 2.1-8 is included to collect companies’ views.

### 1st round FL proposals for Issue 1

#### Proposal 2.1-1

For GC-PDSCH scheduled with DCI format 1_0 for broadcast reception, the size of the FDRA field depends on the size of the CFR.

#### Proposal 2.1-2

For GC-PDSCH scheduled with DCI format 1_0 for broadcast reception, RB numbering starts from the lowest RB of the CFR.

#### Proposal 2.1-3

For GC-PDSCH scheduled with DCI format 1_0 for broadcast reception, the resource allocation granularity is single RB.

#### Proposal 2.1-4

(for conclusion)

for broadcast reception, the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH does not include the field HARQ Process Number.

#### Proposal 2.1-5

for broadcast reception, the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH includes the field New Data Indicator.

#### Question 2.1-6

for broadcast reception, which of the following options is supported for VRB-to-PRB mapping field in the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH?

- Opt-1: DCI includes the VRB-to-PRB mapping field with 1 bit according to Table 7.3.1.2.2-5 in TS 38.212

- Opt-2: VRB-to-PRB mapping is fixed to “interleaved” and no corresponding field is included in the DCI

- Opt-3: RRC signalling configures whether VRB-to-PRB mapping field in DCI is 0 bits or 1 bit according to Table 7.3.1.2.2-5 in TS 38.212

#### Proposal 2.1-7

(for conclusion)

for broadcast reception, the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH does not include the field TB scaling.

#### Question 2.1-8

Please provide your views on the following two bullets on the potential inclusion for broadcast reception

- Support a first DCI format for broadcast, which is the same as the first DCI format for multicast, with broadcast specific and multicast-specific fields made optional.

- Support a second DCI format for broadcast, which is the same as the second DCI format for multicast, with broadcast specific and multicast-specific fields made optional which would allow further functionality such as allow cross-polar MIMO and/or frequency diversity by resource allocation Type 0.

Please provide your answers in the table below. Considering the FL assessment above:

- do you agree with the Proposals 2.1-1, 2.1-2, 2.1-3, 2.1-4, 2.1-5 and 2.1-7? Please provide reasons and views in general.

- Please provide your views on Questions 2.1-6 and 2.1-8.

| company | Comments |
| --- | --- |
| LG Electronics | #### Proposal 2.1-4 We prefer to support HARQ Process Number with NDI. #### Proposal 2.1-5 We prefer to support New Data Indicator for combining. |
| NOKIA/NSB | #### Proposal 2.1-1: OK #### Proposal 2.1-2: OK #### Proposal 2.1-3: OK, and we understood that the support of single RB granularity is the starting point for discussion. #### Proposal 2.1-4: Does it mean that there will be a dedicated HARQ process for broadcast reception? Meaning that there will be no sharing HARQ process with unicast/multicast reception. If it is the case, we are fine with the proposal. #### Proposal 2.1-5: OK #### Question 2.1-6: Prefer Opt-2 #### Proposal 2.1-7: Fine #### Question 2.1-8: For Rel17 MBS, support only DCI format 1_0 for broadcast reception is enough, and there is no need to support the second DCI format |
| NTT DOCOMO | #### Proposal 2.1-1, 2.1-2, 2.1-3, 2.1-4: Support #### Proposal 2.1-5: We don’t think NDI is necessary. #### Question 2.1-6: We prefer Opt-1. DCI format 1_0 for broadcast has enough capacity to include the field, so we don’t see the need to fix to “interleaved”. Proposal 2.1-7: Support #### Question 2.1-8: We understand the motivation to support the second DCI format, but given the limited time, we don’t think it is necessary to support the second DCI format in Rel-17. |
| Lenovo, Motorola Mobility | #### Proposal 2.1-1: OK #### Proposal 2.1-2: OK #### Proposal 2.1-3: OK #### Proposal 2.1-4: Don’t support. For sake of HARQ soft combining at UE side, HARQ process number field is required to support blind HARQ retransmission. We don’t see the need to remove this field. #### Proposal 2.1-5: OK #### Question 2.1-6: Option 3 is supported. #### Proposal 2.1-7: OK #### Question 2.1-8: We think the first group-common DCI format for broadcast is sufficient. Even for the second group-common DCI format, the original motivation is to support MIMO as discussed in AI 8.12.1. However, companies can’t agree to include MCS/NDI/RV for 2nd TB. For broadcast, more important thing is to have wide coverage which requires a compact DCI payload size as DCI format 1-0 in CSS. In addition, considering the DCI blind detection for idle/inactive UEs, support the single DCI format can save UE power. #### Hence, we support only the first DCI format for broadcast in Rel-17. |
| Xiaomi | #### For Proposal 2.1-2/2.1-4/2.1-7, we are OK. For the other proposals, we don’t support and detail comments are provided as below. #### Proposal 2.1-1: Do not support. Our position is that the bit length of FDRA should be determined by the CORESET#0. This is the legacy mechanism, and we believe this is the only reasonable mechanism in order to align DCI payload size, for DCI format 1-0 in CSS which is used since Rel-15. On the other hand, 8.12.1 has already agreed that the FDRA bit length is determined by the CORESET#0 if configured. #### Proposal 2.1-3 Do not support. We should firstly achieve a consensus on proposal 2.1-1 and then come back to this proposal. #### Proposal 2.1-5 We don’t see the necessity to include NDI if HARQ-ACK is not supported for INACTIVE/IDLE UEs. Hence we are not ok with the proposal. #### Question 2.1-6 Option 1. The reason is provided in the background. With VRB-to-PRB mapping indicator, gNB has more flexibility on multiplexing PDSCHs. #### Question 2.1-8 First of all, we don’t see the necessity of introducing the second DCI format for broadcast. For the first subbullet, we are not sure what ‘with broadcast specific and multicast-specific fields made optional’ means. Does it mean the bit field in the DCI format 1-0 is configurable? |
| CATT | #### Proposal 2.1-1: OK #### Proposal 2.1-2: Support #### Proposal 2.1-3: OK #### Proposal 2.1-4: Not support. The HPN field should be included for blind retransmission soft buffer combination. And we believe that the dedicated HARQ process mechanism cannot be reused for broadcast. It has been agreed that multiple G-RNTI can be supported for UE. And each G-RNTI needs a dedicated HARQ process for the combination. In this way, the number of needed dedicated HARQ process may exceed than 16 with larger number of G-RNTI configured. This is not expected. Thus, the HPN field should be included in DCI format for broadcast. #### Proposal 2.1-5: Support. #### Proposal 2.1-6: Not support. It has been agreed that for RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, DCI format 1_0 is used as baseline for GC-PDCCH of MCCH and MTCH. Similarly with the DCI format 1_0, a fixed 1 bit VRB-to-PRB mapping field should be included in in DCI format for broadcast reception with UEs in RRC_IDLE/INACTIVE state. |
| OPPO | Proposal 2.1-2: OK. Proposal 2.1-4: OK. HARQ-ACK feedback is not supported in R-17 MBS, and this field seems not necessary. Proposal 2.1-5: If HARQ field in DCI is not included in the DCI, then NDI field is not needed. Unless this field is reserved/interpreted for other indication. Proposal 2.1-7: OK. #### Question 2.1-8: One DCI format is sufficient to schedule broadcast MBS services, which is the first DCI format. Whether/how to define the broadcast-specific and multicast-specific field in the DCI format should be further discussed. |
| Samsung | Proposal 2.1-1~5: Support #### Q 2.1-6) Option 1. #### Q 2.1-8) It is not clear the motivation of this question. We don’t need the second format in Rel-17. |
| ZTE | #### Proposal 2.1-1: FDRA field size has agreed to be based on CORESET#0/initial DL BWP instead of CFR for multicast scheduling with DCI format 1_0. Then for broadcast, it would be better if we can have consistent solution with multicast. Also, there are discussions on whether to share the same DCI fields between multicast and broadcast, if this is agreed, then the reserved bits for FDRA may not be sufficient if FDRA is based on CFR. Thus, we would propose to use the same approach as agreed in multicast for broadcast. #### Proposal 2.1-2: OK #### Proposal 2.1-3: This proposal is related to Proposal 2.1-1. We propose to discuss Proposal 2.1-1 first and then come back to this proposal later. #### Proposal 2.1-4: OK with this proposal. However, we should make it clear whether this means soft-combining is not supported for broadcast or a dedicated HARQ process is used for broadcast and thus HARQ process ID is not needed. #### Proposal 2.1-5: We this depends whether multiple different TBs can be transmitted in the MCCH/MTCH window. If only one TB can be transmitted in each MCCH/MTCH window, then this NDI is not needed. If multiple different TBs can be transmitted in the same MCCH/MTCH window, then the NDI is needed to differentiate initial transmission and retransmission. #### Question 2.1-6: Opt-1, which is the same behaviour with the existing SIB scheduling. #### Proposal 2.1-7: Support. The TB scaling field is mainly used for small TB. Thus, it is not needed for MBS scheduling since the MBS TB can be large. Question 2.1-8: We think the first DCI format is sufficient. We are ok to share the same DCI fields between multicast and broadcast, where some fields are specific to broadcast (reserved for multicast) and some other fields are specific to multicast (reserved for broadcast). |

| vivo | #### Proposal 2.1-4 we think HARQ Process Number field is necessary to support PDSCH repetition for MTCH, so that dynamic sharing of HARQ processes between broadcast and groupcast/unicast is possible. #### Question 2.1-6 #### We prefer option 1. #### Question 2.1-8 We support first DCI format only for broadcast. #### We are ok with other proposals |
| --- | --- |
| MediaTek | #### Proposal 2.1-1: Support. #### Proposal 2.1-2: Support. The similar behaviour as we agreed for multicast can be reused for broadcast reception since the CFR is limited. #### Proposal 2.1-3: Support. #### Proposal 2.1-4: Support. Introducing the extra HARQ process for RRC IDLE/INACTIVE state is not friendly from UE power saving perspective. For HARQ combining buffer, the similar behaviour for SIB can be reused, which is totally up to UE implementation. #### Proposal 2.1-5: Not support. Since the HARQ process number field is not used, why does DCI for broadcast need to introduce this field. #### Proposal 2.1-6: Opt 1 is preferred. #### Proposal 2.1-7: Support. #### Proposal 2.1-8: Not support. The motivation is not clear for introducing more DCI format for broadcast reception. For Rel-17 MBS broadcast reception, DCI format 1_0 is sufficient. |
| CMCC | Proposal 2.1-1~5: Support Question 2.1-6: Opt-1 Proposal 2.1-7: Support #### Question 2.1-8: We don’t support the second DCI format for broadcast |
| Ericsson | #### Proposal 2.1-1: OK #### Proposal 2.1-2: OK #### Proposal 2.1-3: OK #### Proposal 2.1-4: Not support. We prefer to support HARQ Process Number with NDI for broadcast, like it is supported for multicast. This is also enabled by having a common DCI format for multicast and broadcast. #### Proposal 2.1-5: Support. But the NDI field should be used in combination with the HARQ process number field. HPIDs are shared between unicast/multicast and broadcast. The gNB ensures by implementation that there are no collisions. #### Proposal 2.1-6: At least Option 3 is OK. #### Proposal 2.1-7: Support Proposal 2.1-8: Support. This provides maximum harmonization between multicast and broadcast and simplifies specification impact. With this, Broadcast also can also use advanced functions such as cross-polar MIMO, Type0 resource allocation and potentially HARQ combining. |
| Apple | #### Proposal 2.1-1/-2/-3: ok #### Proposal 2.1-7: ok Question 2.1-8: support first DCI format is enough for broadcast service. |
| Qualcomm | #### Proposal 2.1-1, Proposal 2.1-2, Proposal 2.1-3, Proposal 2.1-4 and Proposal 2.1-7 ok #### Proposal 2.1-5 no need to include NDI. we don’t prefer to define dedicated HARQ process(es) for broadcast. It can be up to UE implementation for soft-combining for broadcast MCCH/MTCH. #### Question 2.1-6 prefer Opt-1 #### Question 2.1-8 This question is dependent whether DCI format 1_0 can support some functionality of DCI format 1_1. For example, RAN1 agreed broadcast MTCH can support slot-level repetition. However, based on the current 38.214 CR, the pdschAggregationFactor and repetitionNumber in TDRA can only be applied to DCI format 1_1 for unicast/multicast. |
| Intel | #### Proposal 2.1-1, 2.1-2: OK Proposal 2.1-3: Why should we not support RBG based scheduling and only limit to single RB? Proposal 2.1-4, 2.1-5: Either both HPN and NDI are supported or neither are supported. #### Question 2.1-8: We don’t see any need to support 2nd DCI format in multicast. In DCI 1_1, some field lengths are configurable based on RRC and such functionality is not available in IDLE/INACTIVE mode and we do not see the need to discuss alternative approaches at this late stage. DCI 1_0 is enough to support broadcast. |
| Huawei, HiSilicon | #### 2.1-1: size needs to be aligned with 1_0/C-RNTI in CSS, so depends on the size of CORSET#0/initial BWP is the only way. 2.1-3, depends on 2.1-1, if FDRA field is determined by CORESET0/initial BWP, then granularity will depends on the size of CFR for broadcast. 2.1-5: both HPID and NDI are not needed. 2.1-6: opt-1 is better. #### 2.1-8, support first DCI would be sufficient for this release. |
| Moderator | Thank you for the comments. #### Proposal 2.1-1: Multiple companies were supporting of the above formulation of the proposal. However, [Xiaomi, ZTE, Huawei] do not support this and would like to align the procedure for broadcast to the procedure agreed for multicast. To hear company comments on this alternative Proposal 2.1-1is revised as below which reuses the procedure agreed for multicast. #### Proposal 2.1-2 This proposal is stable with wide support and is proposed for email approval. #### Proposal 2.1-3 This proposal is related to the discussion on Proposal 2.1-1, therefore this proposal is leave unchanged to wait for comments on Proposal 2.1-1rev1 #### Proposal 2.1-4 & Proposal 2.1-5 Here views are mixed. While [LG, Lenovo, CATT, vivo, Ericsson] want to support HPN, [NTT DOCOMO, Xiaomi, OPPO, Samsung, ZTE, MediaTek, CMCC, Huawei] are fine without including the field. While [LG, Nokia, Lenovo, CATT, Samsung, vivo, CMCC, Ericsson] want to support NDI, [NTT DOCOMO, Xiaomi, OPPO, MediaTek, Qualcomm, Huawei] do not support including the field. @LG, Lenovo, Nokia, ZTE, vivo: We do not have any agreement on whether HARQ process from unicast/multicast are reused for broadcast or whether a dedicated HARQ process is defined (we could have one we thought it would be useful). Agreeing to this does not automatically mean that there will be a dedicated HARQ for broadcast. My understanding from Qualcomm/MediaTek’s response is that this would be up to UE implementation to use the HARQ processes for MCCH/MTCH in idle/inactive UEs for PDSCH repetition. Any thoughts to make progress? It would be good to get comments from companies that do want to introduce HARQ Process Number whether the explanations from Qualcomm/MediaTek that in order to achieve soft-combining for PDSCH repetition the use of HARQ process is up to UE’s implementation. (Qualcomm, MediaTek, please correct me if I missed something!) Another discussion is the one under Issue 7 Question 2.7-3 where it is proposed that broadcast is able to receive HARQ retransmissions, that would require both HPN and NDI. I think therefore, that it would be better to first clarify the position of the group on Issue 7 Question 2.7-3 discussion before we conclude Proposals 2.1-4 & 2.1-5. #### Question 2.1-6 Based on the comments form companies the support for the different options is as follows: - Option 1: [NTT DOCOMO, Xiaomi, CATT, Samsung, ZTE, vivo, MediaTek. CMCC, Qualcomm, Huawei] - Option 2 [Nokia] - Option 3 [Lenovo, Ericsson] Most companies, prefer Option 1, therefore, FL proposes to agree Option 1 in Proposal 2.1-6 #### Proposal 2.1-7 (for conclusion) This proposal is stable and is placed for email approval. #### Question 2.1-8 Most companies have not replied to the first bullet on whether to support a first DCI format for broadcast, which is the same as the first DCI format for multicast, with broadcast specific and multicast-specific fields made optional. Some companies have requested clarification since this does not seem clear. ZTE has expressed support for such proposal. @Ericsson, could you please clarify your proposal? thank you. Most companies have replied to the second question on whether a second DCI should be supported for broadcast to enable functionality such as multi-layer MIMO and Type0 FDRA. [Nokia, NTT DOCOMO, Lenovo, OPPO, Samsung, ZTE, vivo, MediaTek, CMCC, Apple, Intel, huawei] do not support to include a second DCI format for broadcast. Some companies have also mentioned that this could be considered for Rel-18. Based on all the comments above, FL recommends not to pursue an agreement to support a second DCI format. Question 2.1-8 is revised to allow proponents to clarify on the first question. |

### 2nd round FL proposals for Issue 1

#### Proposal 2.1-1rev1

For FDRA determination of the DCI 1_0 format for GC-PDCCH for broadcast reception:

- is the size of CORESET 0

- If the size of CFR (i.e. NCFR) is larger than the size of CORESET0, the resource indication value (RIV) is defined as in section 5.1.2.2.2 in TS38.214, where K is the maximum value from set {1, 2, 4, 6, 8, 10, 12} which satisfies K≤NCFR/NBWPinitial;otherwise, K=1.

#### Proposal 2.1-2 [agreed]

For GC-PDSCH scheduled with DCI format 1_0 for broadcast reception, RB numbering starts from the lowest RB of the CFR.

#### Proposal 2.1-3 [waiting feedback P2.1-1rev1]

For GC-PDSCH scheduled with DCI format 1_0 for broadcast reception, the resource allocation granularity is single RB.

#### Proposal 2.1-7 [agreed]

(for conclusion)

for broadcast reception, the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH does not include the field TB scaling.

#### Proposal 2.1-4 [waiting feedback Issue 7]

(for conclusion)

for broadcast reception, the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH does not include the field HARQ Process Number.

#### Proposal 2.1-5 [waiting feedback Issue 7]

for broadcast reception, the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH includes the field New Data Indicator.

#### Proposal 2.1-6 [NEW]

for broadcast reception, the following options is supported for VRB-to-PRB mapping field in the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH

- Opt-1: DCI includes the VRB-to-PRB mapping field with 1 bit according to Table 7.3.1.2.2-5 in TS 38.212

#### Question 2.1-8rev1 [awaiting clarifications from proponents]

Please provide your views on the following bullet on the potential inclusion for broadcast reception

- Support a first DCI format for broadcast, which is the same as the first DCI format for multicast, with broadcast specific and multicast-specific fields made optional.

Please provide your answers in the table below considering the FL assessment above

- Please focus on Proposals 2.1-1rev1 and 2.1-6 that are revisions or new proposals.

- Please note that for Proposals 2.1-3, 2.1-4, 2.1-5 we are awaiting progress in other proposals/issues and Question 2.1-8rev1 we are awaiting clarifications from proponents.

| company | Comments |
| --- | --- |
| ZTE | #### Proposal 2.1-1rev1: Support. Same handling for FDRA interpretation for broadcast and multicast is preferred from our perspective. Proposal 2.1-4: We can accept that there is no HARQ Process Number in the DCI 1_0 for broadcast scheduling, we would prefer to have a concrete conclusion on whether a dedicated HARQ process will be used for broadcast like that for Rel-15 SIB broadcast or it is totally up to UE implementation. From our perspective, we would prefer to have a dedicated HARQ process for broadcast. Proposal 2.1-6 [NEW]: Support. Question 2.1-8rev1: We support this bullet. Our understanding of this bullet is to say, both DCI fields for broadcast and multicast will be included in the same DCI field. For example DCI format 1_0 for G-RNTI FDRA TDRA …… PUCCH resource indicator, 3 bits, reserved if the G-RNTI is for broadcast MCCH change notification, 2bits, reserved if the G-RNTI is for multicast |
| NOKIA/NSB | #### Proposal 2.1-1rev1: OK Proposal 2.1-4: It is important to discuss and align the understanding among companies on how the HARQ process should be handled for broadcast reception. - For legacy NR, a dedicated HARQ process is used for SIB, meaning there is no sharing or no occupying the total number of HARQ processes from unicast. For Rel17 MBS, we prefer to apply the same approach for broadcast, meaning that there should have a dedicated HARQ process used for broadcast reception, instead of sharing the total number of HARQ processed from unicast/multicast. - Moreover, a single dedicated HARQ process used for broadcast is sufficient for all broadcast services, which can be differentiated by broadcast reception UEs with different G-RNTIs. And for a certain G-RNTI, for UE performs the RV combing of the same TB (with different RVs at each (re-)transmissions), the toggling of the NDI field in the DCI can be used to indicate a new initial transmission to the UE, and by which the RV combining UE could know on whether the soft combing should be performed with the previously received data in the soft buffer or the soft buffer should be clearly with deleting of previously received data in the buffer. - Furthermore, for RRC_CONNECTED UEs, a dedicated HARQ process for broadcast reception can avoid limiting the number of HARQ processes used for unicast and multicast, and for unicast/multicast transmission with larger traffic payload required, the performance impact can be avoided. #### Proposal 2.1-5: As explained in above, the NDI field is needed for UE RV combining. Thus, we support to include it in the DCI format Proposal 2.1-6: If it is the majority view, then it is fine for us. #### Question 2.1-8rev1: Single DCI format 1_0 is enough for broadcast, and we don’t see the need of second DCI format for broadcast reception in Rel17 MBS. |
| Xiaomi | #### Proposal 2.1-1rev1: support Proposal 2.1-3: if proposal 2.1-1 rev 1 is supported, this proposal is not needed. Proposal 2.1-4. Support. From our understanding, a dedicated HARQ process should be allocated for broadcast. Proposal 2.1-6 [NEW]: support |
| OPPO | #### P2.1-1rev1: OK. P2.1-4: It is OK to have a dedicated HARQ process number allocated for broadcast. The proposal is also OK that there is no need to have HP field in DCI for always indicating the same value. P2.1-6[NEW]: OK. P2.1-8: One DCI format is sufficient. How to make some fields configurable/optional should be clarified. |
| Lenovo, Motorola Mobility | 2.1-1rev1: We don’t support this proposal since there are sufficient bits in the DCI format 1-0 for broadcast to indicate FDRA with single RB granularity. 2.1-3: Support as there are sufficient bits in the DCI format 1-0 for broadcast to indicate FDRA with single RB granularity. 2.1-4: we still support HPN in DCI for blind HARQ retransmission. 2.1-5: Agree. 2.1-6: before reaching agreement on this proposal, one question from my side is whether resource allocation Type 0 is supported or not? Whether non-interleaved mapping is supported or not? 2..1-8: OK. Maybe better to remove “first” as there is single DCI format for broadcast. - Support a DCI format for broadcast, which is the same as the first DCI format for multicast, with broadcast specific and multicast-specific fields made optional. |
| NTT DOCOMO | #### Proposal 2.1-1rev1: We have the similar view with Lenovo. We don’t think it is necessary to unify the FDRA interpretation for broadcast and multicast at the expense of scheduling flexibility. Proposal 2.1-6: Support |
| MeidaTek | #### Proposal 2.1-1rev1: we still slightly prefer the original version, but the current proposal is also ok for us #### Proposal 2.1-4: Support. As we explained in previous round, the similar behaviour for SIB can be reused for HARQ combining buffer, which is totally up to UE implementation and no need to make any agreement from Spec perspective. #### Proposal 2.1-4: Not support. #### Proposal 2.1-6: Support. #### Question 2.1-8rev1: we think there is no need to discuss the proposal. We only can focus on which field is used for broadcast DCI format (G-RNTI, MCCH-RNTI) or multicast DCI format (G-RNTI, G-CS-RNTI). |
| CMCC | #### Proposals 2.1-1rev1： Support for the progress Proposal 2.1-6: Support Question 2.1-8rev1: We don’t think there are any optional filed in the first DCI format, all the fields in the first DCI format are mandatory for UE. |
| CATT | #### Proposal 2.1-1rev1: OK. #### Proposal 2.1-6 [NEW]: Agree. |
| Ericsson | 2.1-1rev1: Support 2.1-3: Support 2.1-6: Support #### 2.1-8rev1: Support |
| Samsung | Proposal 2.1-1rev1, 2.1-2/3/7/4/6: Support Proposal 2.1-5: Not support. NDI is not needed since HARQ process ID is not used. Question 2.1-8rev1: This seems not essential discussion. We can discuss for broadcast, not for multicast here. |

## [UPDATE] Issue 2: PDCCH: MCCH change notification

### Background

RAN2 discussed the details of broadcast session delivery and the following agreements were made during RAN2#113-e meeting:

- Assume that MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start for delivery mode 2 of NR MBS (other cases FFS, if any).

During RAN2#113bis-e meeting, RAN2 discussed further aspects of MCCH scheduling and MCCH change notification leading to the following agreements with RAN1 impacts:

- The modification period is defined for NR MCCH and NR MCCH contents are only allowed to be modified at each modification period boundary. - The updated MCCH message should be sent in the same MCCH modification period where the change notification is sent. - It is up to RAN1 to decide about the RNTI and DCI format used for MCCH change notifications. - RAN2 will discuss and down-select from the following two options for the UE to get aware of session stop/modification: - Reading MCCH once per each MCCH modification period when receiving an ongoing broadcast session - DCI used for MCCH notification indicates the change of an ongoing broadcast session

At RAN1#105-e, RAN2 requests RAN1 [R1-2104165] to investigate and provide feedback, considering agreements made by RAN2 as indicated in the LS (cf. Annex B) where the following request is relevant for the discussion:

- Details of the RNTI and DCI design for carrying MCCH change notifications. - NOTE: RAN2 is still discussing some aspects that may have an impact on this issue, e.g. whether or not to support multiple MCCH or whether or not a notification about the modification/stop of an ongoing session is needed, as indicated above. RAN2 will update RAN1 as soon as further agreements are made on these items.

RAN2 discussed further the aspects related to MCCH design and made the following agreements during RAN2#114-e meeting:

- Indication of an MCCH change due to modification of an ongoing session’s configuration (including session stop) is provided with an explicit notification from the network (provided that RAN1 confirms a separate bit for this purpose can be accommodated in the MCCH change notification DCI, in addition to a bit for session start notification). FFS on whether this notification can be reused for modification of other information carried by MCCH, if any. - FFS whether the possibility of UE missing an MCCH change notification needs to be addressed or can be left to UE implementation. - At least in case RAN1 decides to utilize RNTI other than MCCH-RNTI for MCCH change notification, MCCH change notification is sent in the first MCCH monitoring occasion of each MCCH repetition period.

RAN2 discussed further the aspects related to MCCH design and made the following agreements during RAN2#115-e and RAN2#116-e meetings:

RAN2#115-e - RAN2 waits for RAN1’s final decision on which RNTI/DCI (i.e. Alt1 and/or Alt 2 as identified by RAN1) for MCCH change notification to be adopted. - Do not specify any mechanism to address the possibility of UE missing an MCCH change notification and it is left to UE implementation. - It is up to network implementation (e.g. paging repetitions) for addressing scenario of potential notification loss for UEs. RAN2#116-e - MCCH changes due to neighbouring cell information modification will be notified using the normal MCCH modification notification.

RAN1 discussed aspects related to RNTI and DCI design for carrying MCCH change notifications and made the following agreements during RAN1#105-e, RAN1#106-e and RAN1#106bis-e meetings:

Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, DCI format 1_0 is used as baseline for GC-PDCCH of MCCH and MTCH. - FFS details of FDRA. Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, study the following alternatives for MCCH change notification indication due to session start: - Alt 1: Define a dedicated RNTI to scramble the CRC of a DCI indicating a MCCH change notification; - Alt 2: Use of a field in a DCI format scheduling a MCCH without a dedicated RNTI for MCCH change notification; Other solutions are not precluded and it is also not precluded whether to support both Alt1 and Alt2. Conclusion: It is up to RAN2 to decide the specific contents of the MCCH change notification, e.g, whether notification only informs about session start, whether or not notification also informs about session modification/stop or whether or not the notification informs about any other information. Agreement: Study and reach an agreement by RAN1#106b-e on whether Alt1 and Alt2 for MCCH change notification indication can accommodate at least 2 bits for the notification of MCCH configuration changes due to a session start and the notification of MCCH configuration changes of an ongoing session (including session stop). Working assumption: Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes. - Send an LS to RAN2 with the mechanism agreed in RAN1

### Tdoc analysis

- In [R1-2110779, Huawei]

  - Discuss: As discussed in the last meeting, both alternatives can work. Since no fundamental problems have been discovered for Alt2 which was agreed as working assumption, this working assumption should be confirmed.

  - Proposal 4: Confirm the working assumption that Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.

- In [R1-2111551, Xiaomi]

  - Proposal 7: Confirm the following working assumption and send an LS to RAN2 accordingly.
Working assumption: Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.
Send an LS to RAN2 with the mechanism agreed in RAN1

- In [R1-2111629, CMCC]

  - Discuss: During the discussion in last RAN1 meetings, all companies acknowledge that both Alt 1 and Alt 2 are workable and Alt 2 doesn’t need the introduction of new RNTI. Some companies commented the bit for MCCH change notification is limited, but as the analysis above and as the example in Table 1, the reserved bits in DCI format for MCCH is much larger than 2 bits and is enough to be used as MCCH change notification and can also provide forward compatibility. Therefore, Alt 2 can be supported which is a simple and sufficient way without defining a new RNTI for MCCH change notification.

  - Proposal 7. Confirm the working assumption to support Alt 2 for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.

- In [R1-2111763, Samsung]

  - Proposal 4. Confirm the Working assumption for MCCH change notification.

- In [R1-2111899, Apple]

  - Proposal 2: Conform the following working assumption on MCCH change notification.

    - Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.

- In [R1- 2112082, Asustek]

  - Proposal 2: Confirm the working assumption: Alt 2 is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.

- In [R1-2112314, MediaTek]

  - Observation 1: UE needs more power consumption if Alt 2 is used for MCCH change notification.

  - Observation 2: The system latency is increased if Alt 2 is used for MCCH change notification.

  - Observation 3: The same DCI format used for MCCH/MTCH can be reused for MCCH change notification.

  - Proposal 8: MBS DCI format 1_0 used for MCCH and MTCH reception is reused for NR MBS MCCH change notification.

  - Observation 4: The Alt 1 with DCI 1_0 can keep the better decoding performance and backword compatibility.

  - Proposal 9: The Alt 1 is supported for MCCH change notification.

  - Proposal 10: MBS DCI format 1_0 used for MCCH and MTCH reception is reused for NR MBS MCCH change notification.

  - Proposal 11: A new RNTI (e.g., MCCH-N-RNTI) is defined for MCCH change notification.

- In [R1-2112348, Ericsson]

  - Discuss: Bit toggling- The two bits will signal MBS session activation and change of MCCH signaling data. We propose that bit toggling is used, which means that the information of the change indication lies in the change of the bit value rather than the absolute value.

The bit for MBS session activation is thus toggled each time a new MBS session is activated. If the initial value is e.g. ‘0’, this means that at the first MCCH PDCCH DCI in a Modification Period the UE will check whether the bit value has changed. If the UE detects that the value has changed to ‘1’ this implies that one MBS session is activated, and the UE will find out more by investigating the MCCH content. This bit value of ’1’ is retained in all MCCH PDCCH DCIs until there is a new MBS session activation to be notified to the UE, in which case the bit value changes back to ‘0’ and stays like that until a new change occurs etc.

If the UE misses the first MCCH PDCCH carrying the toggled bit, this is not catastrophic, since the UE can have one or more new attempts on the following MCCH PDCCH Monitoring Occasion.

The bit for change of MCCH content would work in the same way, i.e. the UE would check the first MCCH PDCCH DCI in each Modification Period whether the second bit has changed and, if so, would further investigate the MCCH content. If none of the bits are toggled, the UE can safely ignore the MCCH PDCCH until the start of the next Modification Period.

  - Proposal 26: Confirm the Alt2 WA from RAN1#106b-e

    - For each of the bits indicating MBS session activation and MCCH content change: the bit value is toggled each time there is a change and the bit retains that value until the next change occurs.

### FL Assessment

[Huawei, Xiaomi, CMCC, Samsung, Apple, AsusTek] propose to confirm the working assumption made at RAN1#106bis-e on supporting Alt 2 for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes. [Huawei] further clarifies that no fundamental issue has been found with Alt 2 and therefore this alternative works.

[MediaTek] proposes that Alt 1 is supported for MCCH change notification. They present drawbacks of Alt 2 and benefits of Alt 1. In particular, it is clarified that Alt 1 would not need a new DCI format and that the DCI 1_0 format already agreed for broadcast can also be used for Alt 1 – this aspect was already clarified at the last meeting by proponents of Alt 1 and included in the FL summary discussions of RAN1#106bis-e. [MediaTek] also discusses that compared to Alt 1, Alt 2 needs more power consumption, it has increased latency, it has worse decoding performance and it has less forward compatibility in case more bits need to be added for notification changes.

Most companies propose to confirm the working assumption (i.e., Alt 2), while one company proposes to support Alt 1 based on better performance compared with the working assumption. It is worth pointing out that no fundamental issue has been reported in the submitted tdocs demonstrating that Alt 2 does not work. Therefore, the FL proposes to confirm the working assumption in Proposal 2.2-1.

Another aspect on bit toggling has been presented by [Ericsson], which can increase the robustness of Alt 2 as per tdoc analysis above. This approach was already reported for the last meeting, although there was no time for discussion. Therefore, Proposal 2.2-2 to introduce this approach is put forward for discussion.

### 1st round FL proposals for Issue 2

#### Proposal 2.2-1 [agreed]

Confirm the working assumption made at RAN1#106bis-e:

Working assumption:

Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.

- Send an LS to RAN2 with the mechanism agreed in RAN1

#### Proposal 2.2-2

For the bits notifying MCCH configuration changes, each bit value is toggled each time there is a change and the bit retains that value until the next change occurs.

Please provide your answers in the table below. Considering the FL assessment above: do you agree with Proposals 2.2-1 and 2.2-2? Please provide reasons and views in general if you do not agree.

FL note: based on the discussion a potential LS to RAN2 will also be discussed.

| company | comments |
| --- | --- |
| NOKIA/NSB | #### Proposal 2.2-2: OK |
| NTT DOCOMO | #### Proposal 2.2-2: We think the same method as LTE SC-PTM (i.e., ‘1’ indicates the change of MCCH) will be sufficient. |
| Lenovo, Motorola Mobility | #### 2.2-1: Although we prefer Alt 1, we can live with Alt 2. #### 2.2-2: we think predetermined values for specific purpose is reliable enough. Using toggled or untoggled bits for indicating specific purpose may lead to problem when one DCI is missing. |
| Xiaomi | #### I am not sure how the toggling mechanism works if more than 1 bit is needed for MCCH notification. Hence we would like to see some clarifications from proponents. |
| CATT | #### Proposal 2.2-2: OK |
| OPPO | #### Proposal 2.2-2: The motivation/benefit of toggling multiple bits for MCCH change notification is not clear. The proponent should provide more details on how to use the “at least 2 bits” for the change notification. |
| Samsung | #### Proposal 2.2-2: OK |
| ZTE | #### Proposal 2.2-2: It seems we didn’t have the toggling mechanism in LTE. Our preference is to have the same LTE mechanism for NR here. |
| Spreadtrum | #### Proposal 2.2-2: reuse LTE solution is enough. The additional benefit for toggling is not clear to us. |
| vivo | Proposal 2.2-2 : Not support. Toggling bit value causes misunderstanding when UE miss-detects previous PDCCH, we think fixed value is better for reliability, i.e., the 1st bit’1’ indicates a session start, and the 2nd bit ‘1’ indicates session modification. |
| CMCC | Proposal 2.2-2 : Not support. Reuse LTE mechanism is enough. |
| Ericsson | #### Proposal 2.2-2: Support. #### Toggling is important to increase the robustness. Maybe there are different understandings of toggling. What we mean is that with toggling, the bit value is constant until there is a change indication and then the change indication lies in the change of bit value and not in the absolute value. After the change the new bit value is retained until there is a new change etc. There is therefore no issue with a lost DCI – rather the contrary. But without toggling the indication becomes very sensitive to a lost DCI, e.g. if the normal value is ’0’ and then the value ‘1’ is signalled only in one DCI, then loss of that DCI implies catastrophic consequences if the change is only signalled in one DCI. |
| Apple | #### Proposal 2.2-2: Reuse LTE mechanism is sufficient. |
| Qualcomm | #### Proposal 2.2-2: Similar view as DCM/ZTE/CMCC/Apple that toggling bit is not needed, which may result in unnecessary misunderstanding. |
| Intel | #### Proposal 2.2-1: OK to confirm WA. #### Proposal 2.2-2: Toggling is not needed. |
| Moderator | #### Proposal 2.2-1 was agreed at GTW3 on 11 Nov. Agreement Confirm the working assumption made at RAN1#106bis-e: Working assumption: Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes. - Send an LS to RAN2 with the mechanism agreed in RAN1 A Draft LS has been placed in the drafts 8.12.3 folder for your consideration. Please provide your views on the LS. On Proposal 2.2-2 - Support [Nokia, Samsung, Ericsson] - Not support/unnecessary [NTT DOCOMO, Lenovo, ZTE, Spreadtrum, vivo, CMCC, Apple, Qualcomm, Intel] - clarifications needed [Xiaomi, OPPO] Although this proposal has received some support, most companies do not see this functionality necessary. There have been some companies requesting more information from proponents. Ericsson has provided some clarifications, I also copy here for convenience the text from the tdoc that may be useful background from R1-2112348: “Bit toggling The two bits will signal MBS session activation and change of MCCH signaling data. We propose that bit toggling is used, which means that the information of the change indication lies in the change of the bit value rather than the absolute value. The bit for MBS session activation is thus toggled each time a new MBS session is activated. If the initial value is e.g. ‘0’, this means that at the first MCCH PDCCH DCI in a Modification Period the UE will check whether the bit value has changed. If the UE detects that the value has changed to ‘1’ this implies that one MBS session is activated, and the UE will find out more by investigating the MCCH content. This bit value of ’1’ is retained in all MCCH PDCCH DCIs until there is a new MBS session activation to be notified to the UE, in which case the bit value changes back to ‘0’ and stays like that until a new change occurs etc. If the UE misses the first MCCH PDCCH carrying the toggled bit, this is not catastrophic, since the UE can have one or more new attempts on the following MCCH PDCCH Monitoring Occasion. The bit for change of MCCH content would work in the same way, i.e. the UE would check the first MCCH PDCCH DCI in each Modification Period whether the second bit has changed and, if so, would further investigate the MCCH content. If none of the bits are toggled, the UE can safely ignore the MCCH PDCCH until the start of the next Modification Period. This toggling mechanism provides increased robustness thanks to the inherent repetition - one or even more missed MCCG PDCCHs would not be catastrophic – subsequent PDCCH DCIs carry the same change information.” Companies please check the clarifications and provide comments and whether this has addressed your concerns. |

### 2nd round FL proposals for Issue 2

#### Proposal 2.2-2 [comments needed]

For the bits notifying MCCH configuration changes, each bit value is toggled each time there is a change and the bit retains that value until the next change occurs.

#### DRAFT LS 2.2-3 [NEW]

Please provide your comments to the “DRAFT LS on MCCH change notification” in: Inbox/drafts/8.12.3/LS/

Please provide your answers in the table below. Considering the FL assessment above and clarifications provided:

- do you agree with Proposal 2.2-2? Please provide reasons and views in general if you do not agree.

- Please provide your comments to the “DRAFT LS on MCCH change notification” in: Inbox/drafts/8.12.3/LS/

| company | comments |
| --- | --- |
| ZTE | Proposal 2.2-2: One compromised way is to indicate this proposal in the LS to RAN2 and let RAN2 to dedicate whether it is needed or not. Because RAN2 will specify the detailed behaviours for UE on when to detect the notification change. |
| NOKIA/NSB | We also prefer the legacy method of LTE SC-PTM, i.e., ‘1’ indicates the change of MCCH |
| Xiaomi | Proposal 2.2-2 [comments needed]: Thanks for the further clarification. However, we tend to agree with majority views that the current mechanism is sufficient. |
| OPPO | P 2.2-2: Based on the clarification, we prefer to use existing mechanism for MCCH change notification rather than bits toggling mechanism. |
| Lenovo, Motorola Mobility | 2.2-2: We don’t think legacy mechanism can’t work for MCCH change notification. In legacy spec, there are several indicators in DCI format 1-0 with specific purpose which don’t adopt toggling mechanism. |
| MediaTek | #### Proposal 2.2-2 [comments needed]: No need. We prefer to reuse the legacy mechanism for MCCH change notification. |
| CMCC | #### P 2.2-2: No, reuse legacy LTE mechanism |
| Ericsson | P 2.2-2: Support. We agree with the LS approach proposed by ZTE #### Draft LS 2.2-3: Support, provided the bit toggling proposal is included as a possibility, for RAN2 to decide whether to adopt. |
| Samsung | We don’t see the necessity of toggling. The legacy method of LTE SC-PTM is enough, which is ‘1’ indicates the change of MCCH. |

## Issue 3: PDCCH: Details of CSS for MCCH/MTCH channels

### Background

The following agreement for RRC_IDLE/RRC_INACTIVE UEs at RAN1#103-e, RAN2#104-e, RAN1#105-e, RAN1#106-e and RAN1#106bis-e are relevant for this discussion:

Agreements: For RRC_IDLE/RRC_INACTIVE Ues, CSS is supported for group-common PDCCH. - FFS: reuse current CSS type, define a new CSS type, etc. - FFS other details. Agreement: For broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE Ues and RRC_CONNECTED Ues when UE-specific active BWP of RRC_CONNECTED UE contains the common frequency resource of RRC_IDLE/INACTIVE Ues and the SCS and CP are the same. - FFS: the case when UE-specific active BWP of RRC_CONNECTED UE does not contain the common frequency resource of RRC_IDLE/INACTIVE Ues. Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MCCH. Agreement: For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs support the same CSS type for MCCH and MTCH. - FFS support of different CSS types for MCCH and MTCH channels for broadcast reception. Conclusion: For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs, there is no specification support in Rel-17 of different CSS types for GC-PDCCH scheduling MCCH and MTCH. Agreement: Study whether the Type-x CSS supported for multicast in RRC_CONNECTED can be reused as baseline for broadcast in RRC_IDLE/RRC_INACTIVE for GC-PDCCH scheduling MCCH and MTCH. Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MTCH.

The following agreements for RRC_CONNECTED UEs at RAN1#105-e, RAN1#106-e and RAN1#106bis-e are also relevant for this discussion:

Agreement: For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, Alt 2 is supported: - Alt 2: support a Type-x CSS - The monitoring priority of Type-x CSS is determined based on the search space set indexes of the Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the Type-x CSS. - FFS: Whether the Type-x CSS is a Type-3 CSS Conclusion: The specification impact of having a new Type-x CSS for GC-PDCCH in RRC_CONNECTED state can be studied and discussed further. Agreement: The first and second DCI formats for multicast can be configured in the same or different search space sets belonging to type-x CSS.

### Tdoc analysis

- In [R1-2110897, TD tech]

  - Proposal 15: The CORESET/search spaces for GC-PDCCH carrying MCCH/MTCH can be configured as below.

    - If a CORESETs/search space not configured by initialDownlinkBWP is shared by MCCH and MBS sessions, configure the CORESETs/search space on the MCCH specific SIB with the type of the CORESET/search space set as “Shared”.

    - If a CORESETs/search space not configured by initialDownlinkBWP is only used by MCCH, configure the CORESETs/search space on the MCCH specific SIB with the type of the CORESET/search space set as “NOT Shared”.

    - If a CORESETs/search space not configured by initialDownlinkBWP is only used by MBS sessions, configure it on MCCH.

    - If at least one CORESET/search space configured by initialDownlinkBWP is used by MCCH, a CORESET/search space ID list is provided on the MCCH specific SIB to indicate which CORESETs/search spaces by initialDownlinkBWP are used by MCCH. For each CORESET/search space in the CORESET/search space ID list, if it’s shared by MBS sessions, its type is set as “Shared’. Otherwise its type is set as “NOT Shared”.

    - If at least one CORESET/search space configured by initialDownlinkBWP is used by MBS sessions but not used by MCCH, a CORESET/search space ID list is provided on MCCH to indicate which CORESETs/search spaces by initialDownlinkBWP are used by MBS sessions.

- In [R1- 2111041, vivo]

  - Proposal 3: The same type of CSS supported for multicast in RRC_CONNECTED can be reused for broadcast in RRC_IDLE/RRC_INACTIVE for GC-PDCCH scheduling MCCH and MTCH.

- In [R1-2111137, Nokia]

  - Proposal-12: From signalling configuration perspective, the Type-x CSS defined in RRC_CONNECTED cannot be directly reused, and there may need to define a new Type-y CSS specifically for RRC_IDLE/INACTIVE UE with MBS operation.

- In [R1-2111305, OPPO]

  - Proposal 5: One of the existing CSS types can be selected and reused for RRC_IDLE/RRC_CONNECTED UEs for broadcast reception.

  - Proposal 6: The Type-x CSS for multicast in RRC_CONNECTED is not reused for broadcast in RRC_IDLE/RRC_INACTIVE for GC-PDCCH scheduling MCCH and MTCH.

  - Proposal 7: Type-x CSS for RRC_IDLE is configured and the signaling is carried via SIB.

- In [R1-2111518, Intel]

  - Discuss: The PDCCH which schedules the MCCH carrying the MBS configuration can be monitored in a Type0-PDCCH CSS set configured by searchSpaceZero in PDCCH-ConfigBroadcast and associated with a CORESET#0 for both RRC_CONNECTED and IDLE mode UEs. Alternately it can be monitored in a new PDCCH CSS set e.g., searchSpaceBroadcast which is configured by the MBS specific PDCCH-ConfigBroadcast. The CSS set can be a Type-x CSS set similar to the case for RRC_CONNECTED UEs

  - Proposal 6: The PDCCH scheduling the MCCH can also be monitored in a Type-x CSS set configured by the MBS specific PDCCH-ConfigBroadcast

- In [R1-2111629, CMCC]

  - Discuss: The first is that RRC_CONNECTED UEs can both receive broadcast service and multicast service, and it is no sense to define different CSS types and different PDCCH monitoring priority rules between broadcast and multicast.
The second is that new Type-x CSS for MTCH can reduce unnecessary BD/CCE counting for RRC_CONNECTED UEs. For RRC_CONNECTED UEs, all configured CSS PDCCHs are counted into the monitored BD/CCEs and the left BD/CCEs capability are used for USS in Rel-15/16. However, it’s up to UE’s implementation to receive Rel-17 broadcast services or not, that is UE may not receive some configured broadcast service MTCH GC-PDCCHs. If current CSS type is reused for broadcast MTCH GC-PDCCH and the same PDCCH overbooking rule is reused for RRC_CONNECTED UEs, these non-received broadcast MTCH GC-PDCCHs will also be counted into monitored BD/CCEs, which causing the reduction of USS scheduling opportunity. If we take new Type-x CSS for MTCH, the monitoring priority is according to the search space index and the non-monitored broadcast MTCH GC-PDCCHs are not counted into the monitored BD/CCEs for RRC_CONNECTED UEs in order to not decrease USS scheduling opportunity.
During the discussion in last RAN1 meeting, some companies commented that the GC-PDCCH CSS configuration signalling is different from RRC_CONNECTED UEs and RRC_IDLE/INACTIVE UEs. From our point of view, the definition of CSS doesn’t have any relationship with the configuration signalling. For example, the paging search space can be configured in SIB for RRC_IDLE/INACTIVE UEs and can also be configured within UE’s active BWP by dedicated RRC signalling for RRC_CONNECTED UEs.

  - Proposal 4. For CSS of GC-PDCCH for broadcast, the same CSS type as multicast is supported, i.e., Type-x CSS.

- In [R1-2111763, Samsung]

  - Discuss: The suggested motivation for a new CSS is to avoid the default collision among PDCCH candidates that always start from CCE index 0. Therefore, whether or not there is any modification, is not applicable to that configuration of CSS sets (can remain as for Type-3 CSS sets with UE-common/SIB1 RRC instead of UE-specific RRC) but to the search space set equation where an initialization may not always be Y_(p,-1)=0. Such collision avoidance (also with PDCCH candidates for non-Type-3 CSS in case of CORESET#0) is necessary and should also apply for multicast for RRC_CONNECTED UEs (with respect to PDCCH monitoring, broadcast is only a particular realization of multicast and it has been agreed that the same GC-PDCCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs). The mechanism to avoid the collisions can be further discussed.

  - Observation 1: Configuration of SS sets for GC-PDCCH can be as for Type-3 PDCCH CSS sets in Rel-16 (via UE-common, instead of UE-specific, RRC signaling).

  - Proposal 2. Support avoidance of permanent collisions for PDCCH candidates of search space sets for GC-PDCCH for broadcast and multicast.

- In [R1-2112065, LGE]

  - Proposal 1: Idle/inactive UE monitors PDCCH for Type0A-PDCCH CSS set to detect a DCI with SI-RNTI and receive MBS specific SIB on the corresponding PDSCH on the initial DL BWP of a serving cell for broadcast.

- In [R1-2112130, NTT DOCOMO]

  - Proposal 3: For CSS for broadcast for RRC_IDLE/RRC_INACTIVE UEs, use the same CSS type as multicast (i.e., type-x CSS).

- In [R1-2112163, Lenovo]

  - Discuss: Correspondingly, an associated common search space is configured for the common CORESET, which can reuse current CSS type.

  - Proposal 14: New type-x CSS is configured for RRC IDLE/RRC INACTIVE UEs.

- In [R1-2112241, Qualcomm]

  - Discuss: If a CSS other than searchSpace#0 is configured in the broadcast CFR, RAN1 needs to discuss whether the searchSpaceBroadcast if configured in a CFR-Config-Broadcast is similar as legacy CSS or multicast Type-X CSS with configurable monitoring priority. If the broadcast DCI formats in the broadcast CSS is treated similar as SIB/paging in legacy CSS, it will always have higher monitoring priority than USS for unicast and CSS for multicast. Since a RRC_CONNECTED UE may receive broadcast/unicast/multicast in a BWP, we prefer to reuse the design of multicast Type-X CSS with configurable monitoring priority for the broadcast CSS.

  - Proposal 3: The searchSpaceBroadcast if configured in a CFR-Config-Broadcast is using same Type-X CSS as that of multicast CSS.

- In [R1-2112314, MediaTek]

  - Proposal 6: The CSS type defined in AI 8.12.1 (e.g., a new Type-x CSS) for MBS group scheduling can be used for both searchSpace#0 and search space other than searchSpace#0 for GC-PDCCH scheduling MCCH and MTCH.

- In [R1-2112348, Ericsson]

  - Discuss: It has been argued that broadcast cannot use the same CSS type as multicast, due to different way of configuration (RRC vs SIBx/MCCH), but how the configuration is conveyed is a totally different question from what is configured. There is nothing that prevents the same IEs to be conveyed via either RRC or SIBx/MCCH and be used for both multicast and broadcast. This means that the same CSS type can be used for both multicast and broadcast.

  - Proposal 23: The CSS type for broadcast should be the same as the CSS type for multicast.

### FL Assessment

On reusing Type-x CSS for multicast reception for broadcast reception in RRC idle/inactive UE states

[vivo, Intel, CMCC, NTT DOCOMO, Qualcomm, MediaTek, Ericsson] propose that the same type of CSS supported for multicast in connected RRC state is reused for broadcast reception in idle/inactive RRC states.

On whether the CSS type being discussed for multicast can be reused for broadcast, [Nokia] discusses that the Type-x CSS defined for connected RRC state cannot be reused for idle/inactive RRC states, since the configuration is different, i.e., RRC dedicated signalling vs. SIB/MCCH signalling for connected and idle/inactive states, respectively. However, [Ericsson, OPPO, CMCC] discuss that what is relevant is first to discuss whether the parameters for the CSS for multicast and broadcast are the same/different and second the way it is configured is a separate aspect that would not preclude reusing the same CSS type for multicast and broadcast.

While [OPPO, Samsung] propose to reuse one the existing CSS types for broadcast reception in idle/inactive RRC states, [Nokia, Lenovo] proposes to use a new CSS type rather than reusing the CSS type from multicast.

Finally, [TD Tech] discuss configuration options for the CSS.

On Draft CR TS 38.213 NR MBS [R1-2112445]

It is important to highlight the discussions related to the Draft CR for TS 38.213 on NR MBS [R1-2112445]. The relevant text if the draft CR is copied below:

Here, Type0-PDCCH CSS and Type0B-PDCCH CSS are configured for broadcast reception. On the other hand, for multicast Type3-PDCCH CSS is defined.

Regarding the CSS prioritization of CSS for broadcast in RRC connected state, the editor has the below note highlighting that RAN1 has not yet discussed this. Therefore, an agreement may be needed, but this may be better discussed at other AI on RRC connected UE state.

Based on the above, it seems that CSS types for broadcast reception in RRC idle/inactive states is concluded. However, to collect companies views on whether this is correct the FL puts forward for discussion Question 2.3-1.

Finally, as per the draft CR, Type3-PDCCH CSS is defined for multicast reception. Given [vivo, Intel, CMCC, NTT DOCOMO, Qualcomm, MediaTek, Ericsson] propose that the same type of CSS supported for multicast is reused for broadcast reception in idle/inactive RRC states, the FL puts forward Question 2.3-2 to confirm whether given the current draft CR there is still need for this.

### 1st round FL proposals for Issue 3

#### Question 2.3-1

Given the current draft CR on TS 38.213 [R1-2112445], are there any other critical aspect left for discussion on CSS types for broadcast reception with UEs in RRC idle/inactive states?

#### Question 2.3-2

Given the current draft CR on TS 38.213 [R1-2112445], should the multicast Type3-PDCCH CSS also be used for broadcast reception with UEs in RRC idle/inactive states?

Please provide your answers in the table below. Considering the FL assessment above, please provide your views on Questions 2.3-1 and 2.3-2:

| company | comments |
| --- | --- |
| NOKIA/NSB | #### Question 2.3-1: Thanks for the CR update information from the 38.213 editor. And specifically, the term Type0B-PDCCH CSS can be understood as the Type-X CSS as discussed, and if the general term “pdcch-Config-Broadcast” is defined to be received by both idle/inactive and connected mode UEs for broadcast reception, then the definition of Type0B-PDCCH CSS is fine for us. #### Question 2.3-2: NO, we don’t think Type-3-PDCCH CSS can be applied for idle/inactive UEs as legacy definition |
| NTT DOCOMO | #### Question 2.3-2: We don’t see any problem with using type-3 CSS for broadcast reception. |
| Xiaomi | For question 2.3-1, we think there is no other critical aspect left on CSS types for broadcast reception with UEs in RRC idle/inactive states. For question 2.3-2, we don’t think multi-cast Type3-PDCCH CSS is needed for broadcast reception considering Type0B CSS is already captured in the CR. |
| OPPO | Q 2.3-1: No other critical issue is left. Q 2.3-2: Type-3 CSS can be considered. |
| Samsung | We think Question 2.3-1 is not needed to discuss. For Q 2.3-2, the existing CSS types can be reused for broadcast reception. |
| ZTE | We are ok with the description for SS for broadcast in draft CR on TS 38.213 [R1-2112445], and we don’t think type-3 should be added for broadcast. |
| Spreadtrum | Q2.3-1: No Q2.3-2: No. Since we already have Type0B for broadcast, there is no need to reuse type-3 CSS for broadcast. |
| CMCC | Q 2.3-1: No Q 2.3-2: Yes |
| Ericsson | #### Question 2.3-1: No Question 2.3-2: No |
| CATT | #### Question 2.3-1: Per our understanding, the priorities between Type0B-PUCCH CSS for broadcast and the legacy CSS type/ USS type should be clarified. #### Question 2.3-2: We share same views with NOKIA/Xiaomi/ ZTE/Spreadtrum/Ericsson and don’t think multi-cast Type3-PDCCH CSS is needed for broadcast reception considering Type0B CSS is already captured in the CR. |
| Apple | #### Question 2.3-2: No, Type0B CSS for MBS broadcast is enough. |
| Qualcomm | No matter whether it is Type-0B or Type-3 or Type-x for broadcast DCI formats, we need to clarify: - whether the DCI formats of other RNTI can be configured in the same CSS as broadcast DCI formats? - whether the CSS for broadcast DCI formats can have configurable monitoring priority? Or will it be always higher than the unicast DCI format in USS or multicast in Type-x CSS for RRC_CONN UEs? - If a new Type-0B CSS is agreed for broadcast DCI formats, whether same Type-0B CSS can be used for multicast DCI formats? |
| Intel | Same view as Ericsson |

## [UPDATE] Issue 4: Parameters and configuration of the CFR for MCCH/MTCH

### Background

The following agreements for RRC_IDLE/RRC_INACTIVE UEs at RAN1#103-e, RAN1#106-e, and RAN1#106bis-e are relevant for this discussion:

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource(s) for group-common PDCCH/PDSCH. - the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: the relation of the common frequency resource(s) (if configured) and initial BWP. - FFS: whether to configure one/more common frequency resources - FFS: configuration and definition details of the common frequency resource Agreement: From RAN1 perspective, the CFR for broadcast reception of RRC_IDLE/INACTIVE UEs, includes at least the following configurations: - One set of parameters configured for PDSCH for broadcast reception with GC-PDSCH - One set of parameters configured for PDCCH for broadcast reception with GC-PDCCH - FFS: whether some parameters configured for PDSCH/PDCCH are optional/needed for the supported cases of CFR. - FFS: If necessary, depending on the cases supported, starting PRB and the number of PRBs - The reference for starting PRB is Point A. (Following the same approach to determine reference for starting PRB as that defined in AI8.12.1.) Agreement: For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use the same bandwidth configurations for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH. - FFS: use of different bandwidth configurations for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH Agreement: The PDCCH/PDSCH parameters for broadcast reception with GC-PDCCH/PDSCH, which are not configured, use as default the value of the PDCCH/PDSCH parameters for the configuration of the Rel-15/Rel-16 initial BWP for RRC_IDLE/RRC_INACTIVE UEs.

### Tdoc analysis

PDSCH TDRA table determination

- In [R1-2110912, ZTE]

  - Discuss: If search space 0 is used for MBS reception, it is viable to reuse the same default B table and C table for MBS PDSCH under the corresponding multiplexing patterns with the above restriction as shown in Figure-3. However, if search space other than 0 is used for MBS reception, the above restriction is invalid. And it will seriously affect the flexibility and capacity of MBS transmission, because most of the entities only have a length of 2 or 4 symbols for PDSCH allocation. This does not meet the requirement of mass data transmission, e.g., video service under MBS. The following approach is proposed to address this issue.

  - Proposal 6: Adding the following PDSCH TDRA table determination rule for broadcast to Table 5.1.2.1.1-1 of TS38.214.

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | pdsch-ConfigCommon includes pdsch-TimeDomainAllocationList | pdsch-Config includes pdsch-TimeDomainAllocationList | pdsch-Config-broadcast includes pdsch-TimeDomainAllocationList | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- | --- |
| MCCH_RNTI, G_RNTI for broadcast | Type-x Common for broadcast | 1 | No | - | No | Default A (if SearchSpaceZero is configured) |
|  |  | 2 | No | - | No | Default B (if SearchSpaceZero is configured) |
|  |  | 3 | No | - | No | Default C (if SearchSpaceZero is configured) |
|  |  | 1,2,3 | No | - | No | Default A (if SearchSpaceZero is NOT configured) |
|  |  | 1,2,3 | Yes | - | No | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | pdsch-TimeDomainAllocationList provided in pdsch-Config-broadcast |

Point A as reference for starting PRB

- In [R1-2111137, Nokia]

  - Proposal-5: To align the outcome agreement with RRC_CONNECTED, the Point A as reference point of starting PRB for CFR configuration of RRC_IDLE/INACTIVE UEs for all Case C, Case D and Case E.

- In [R1-2111232, CATT]

  - Discuss: Since CFR has the same size as the initial BWP (configured by SIB1) in Case C, so it is reasonable that the locationAndBandwidth of Case C is optional and can reuse the locationAndBandwidth of initial BWP. Regarding Case D and Case E, to configure the CFR, the indication of the MBS common frequency includes starting PRB and length of PRBs. Following the same approach to determine reference for starting PRB as that defined in AI8.12.1 for RRC-CONNECTED UEs, the Point A can be applied as the reference for starting PRB for RRC_IDLE/INACTIVE UEs. The indication method of starting point and length of PRBs can reuse the current RIV (resource indicator value) mechanism in Rel-15.

  - Proposal 3: The locationAndBandwidth parameter for PDSCH/PDCCH can be optional for Case C.

  - Proposal 4: For Case D (if supported) and Case E (if supported), the starting PRB is referenced to Point A. The current RIV mechanism can be applied for indicating the starting PRB and the length of PRB of CFR.

- In [R1-2112348, Ericsson]

  - Proposal 11: To define the broadcast BWP/CFR frequency resources, reuse the legacy definition of BWP frequency resources for unicast using the combination of Point A, offsetToCarrier and locationAndBandwidth to indicate the exact location of the BWP/CFR with respect to the carrier starting RB.

    - Note: With Case A, the frequency resources of BWP/CFR are – by definition – equal to CORESET#0 initial BWP, which is already defined by legacy, so no dedicated configuration of the frequency resources are required for the CFR.

Different BW configurations for MCCH and MTCH

- In [R1-2110779, Huawei]

  - Discuss: MTCH may require larger frequency resources than MCCH, so the CFR for MTCH can be configured in MCCH.

When CFR for at least MTCH can be configured with the same size as SIB1 configured initial BWP, the CORESET for MTCH scheduling can be configured to be larger than the bandwidth of CORESET#0. Hence, the CORESET for MTCH scheduling can be configured in MCCH which could be part of configuration of CFR but can be up to RAN2 for signaling design.

  - Proposal 3: The CFR, CORESET, and search space for MCCH and MTCH can be configured separately.

- In [R1-2110912, ZTE]

  - Discuss: MCCH only transmits some control information for MBS. However, MTCH needs to transmit MBS traffic, which may require large bandwidth. Considering the different requirements of MCCH and MTCH, it is worthwhile for network to have the flexibility of configuring different CFRs for MCCH and MTCH.

  - Proposal 3: Network supports configuring different CFRs for MCCH and MTCH.

- In [R1-2111137, Nokia]

  - Discuss: Practically the traffic payload size for MCCH and MTCH can be different a lot, where the control configuration payload carried via MCCH can be much smaller than the MBS traffic data payload carried via MTCH. Thus, the CFR for MCCH and MTCH can be also configured differently and controlled by network gNB based on traffic needs. An example is shown in Figure-1 with CFR Case C-1, where the MCCH CFR can be configured in CORESET#0 region and the MTCH CFR can be configured differently with larger CFR identical to initial BWP.

  - Proposal-2: CFR for MCCH and MTCH can be configured differently.

- In [R1-2111232, CATT]

  - Proposal 5: For RRC_IDLE/RRC_INACTIVE UEs, different bandwidth configurations for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH are not supported.

- In [R1-2112348, Ericsson]

  - Discuss: For the reception of MCCH data, the bandwidth is probably not that important, since the MCCH is cyclic, and the UE only needs to receive one cycle and can then monitor for changes. It is then more important that the monitoring of change notifications of the MCCH can be done in a power efficient way, which is possible with TDM due to the sparse way the change notifications are transmitted.

  - Discuss: Even with a single CFR, most part of the power saving is expected to come from the time domain DRX and change notification mechanism, which allows the UE to receive MCCH change notification using a very small percentage of all slots, once the cyclic MCCH as such as has been captured.

  - Proposal 9: Only a single common CFR for both MCCH and MTCH is supported in Rel-17.

- In [R1-2111305, OPPO]

  - Proposal 3: For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use the same bandwidth configuration by SIB for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH.

- In [R1-2111551, Xiaomi]

  - Proposal 4: For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use the same bandwidth configuration for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH.

- In [R1-2112163, Lenovo]

  - Discuss: In RAN1#106bis meeting, regarding CFR configuration for RRC connected mode UEs, RAN1 has agreed that no more than one CFR is configured per dedicated unicast BWP in Rel-17. Following this agreement, it is straightforward to extend it to RRC_IDLE/RRC_INACTIVE UEs. Since only one CFR is configured for RRC_IDLE/RRC_INACTIVE UEs, same CFR is used for receiving MCCH and MTCH.

  - Proposal 3: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, only same CFR for MCCH and MTCH is supported.

- In [R1-2112314, MediaTek]

  - Proposal 1: The unified CFR is defined/configured for GC-PDCCH/PDSCH carrying MCCH and GC-PDCCH/PDSCH carrying MTCH.

Different PDCCH-Config and PDSCH-Config for MCCH and MTCH

- In [R1-2110779, Huawei]

  - Discuss: As for the PDSCH configurations of MCCH and MTCH, some of them can be different. Similar to SIB, the GC-PDSCH carrying MCCH could fixed as QPSK and MCS Table 5.1.3.1-1 in TS38.214, and one layer is sufficient. While for GC-PDSCH carrying MTCH which may be with high data rate, the MCS for MTCH should be flexible, i.e., qam256 or qam64LowSE can be configured by high layer. In addition, due to only DCI format 1_0 was agreed for GC-PDCCH of MCCH and MTCH, there is no Antenna port(s) information can be configured. In order to minimize specification impact, GC-PDSCH carrying MTCH can also use one layer.

  - Proposal 6: GC-PDSCH carrying MCCH can be fixed as QPSK and single layer.

  - Proposal 7: GC-PDSCH carrying MTCH can be fixed as single layer and mcs-Table can be configured as qam256 or qam64LowSE by high layer.

- In [R1- 2112082, AsusTek]

  - Proposal 1: Only the basic parameters in the current PDSCH-Config are necessary for broadcast reception for RRC_IDLE/ INACTIVE UEs, e.g. pdsch-TimeDomainAllocationList, resourceAllocation, and rbg-Size, to simplify the implementation.

- In [R1-2112241, Qualcomm]

  - Proposal 4:

    - GC-PDSCH for broadcast MCCH can use QPSK and single layer.

    - GC-PDSCH for broadcast MTCH can be configured by MCCH to use flexible MCS.

- In [R1-2111629, CMCC]

  - Discuss: As the discussion above, we think the same CFR is used for MCCH and MTCH and the CFR is configured in SIBx. We also agreed that separate PDCCH-config/PDSCH-config can be configured for GC-PDCCH/PDSCH for MCCH/MTCH in CFR, but one issue is if the PDCCH-config/PDSCH-config for MCCH and MTCH are the same or not. From our point of view, considering the MCCH is similar to system information PDSCH and MTCH is similar to data PDSCH, and different broadcast service can also have different traffic parameters, the PDCCH/PDSCH configuration can be different. Thus, the PDCCH-config/PDSCH-config for MCCH is configured by SIB, and the PDCCH-config/PDSCH-config for MTCH is configured by MCCH.

  - Proposal 3. For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs:

    - The CFR used for MCCH and MTCH is configured by SIBx;

    - PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MCCH is configured by SIBx;

    - PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MTCH is configured by MCCH. If the PDCCH-config/PDSCH-config for MTCH is not configured, the PDCCH-config/PDSCH-config for GC-PDCCH/PDSCH carrying MCCH configured by SIBx is reused for GC-PDCCH/PDSCH carrying MTCH.

- In [R1-2112348, Ericsson]

  - Proposal x: For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs:

    - the set of parameters configured for PDCCH/PDSCH for broadcast reception with GC-PDCCH/PDSCH carrying MCCH can be configured by SIBx

    - the set of parameters configured for PDCCH/PDSCH for broadcast reception with GC-PDCCH/PDSCH carrying MTCH can be configured by MCCH

  - At RAN1#106b-e it was also discussed whether different configurations for MCCH and MTCH could be used on the same CFR. This could e.g. be different PDCCH-config, allowing for different Monitoring Occasions for MCCH and MTCH. We think such functionality could be useful to have, since the requirements to receive MCCH and MTCH are expected to be quite different.

  - Proposal 10: For UEs receiving broadcast in RRC IDLE/INACTIVE, support different configurations of PDCCH and PDSCH for MCCH and MTCH within a broadcast CFR.

- In [R1-2112241, Qualcomm]

  - Discuss: Even if using same BW size for MCCH and MTCH, it is also possible to use different pdsch-config, and/or pdcch-config. For example, the MCCH can use TDRA in the pdsch-Config-Common and SS#0 similar as SIB; while the MTCH can use flexible MCS, RM patterns, and CORESET/SS for broadcast data services. More detailed discussion is in Sect. 2.3 and 2.4.

  - Proposal 2: Different CFR-Config-Broadcast can be configured for MCCH and for MTCH.

- In [R1-2112314, MediaTek]

  - Proposal 2: The CFR for MCCH and MTCH is configured via MBS specific SIB (e.g., SIB-x).

- In [R1-2111899, Apple]

  - Discuss: According to RAN2 incoming LS in [4], MCCH related configurations will be carried by MSB specific SIB, it should include the CFR configuration to enable the reception of GC-PDCCH/PDSCH carrying the MCCH. The scheduling information for MTCH is included in MCCH contents, it’s natural CFR configurations for MTCH is delivered via the MCCH.

  - Proposal 3: For broadcast reception by RRC_IDLE/RRC_INACTIVE UEs,

    - the CFR of GC-PDCCH/PDSCH carrying MCCH is configured by MBS specific SIB

    - the CFR of GC-PDCCH/PDSCH carrying MTCH is configured by MCCH

RateMatchPattern

- In [R1-2111232, CATT]

  - Proposal 2: If RAN1 wants to configure RateMatchPattern/ RateMatchPatternLTE-CRS, the following issues should be discussed.

    - Issue1: Whether both RateMatchPattern and RateMatchPatternLTE-CRS are needed configured by SIBx/MCCH.

    - Issue2: The relationship between unicast and broadcast rate matching parameters when UE receiving both unicast and broadcast services.

### FL Assessment

PDSCH TDRA table determination

[ZTE] discusses limitations of the current PDSCH TDRA table determination for broadcast. The determination of the resource allocation table to be used for PDSCH has also been discussed as part of the draft CR to TS 38.214 [R1-2112485]. This issue has not been discussed yet for broadcast reception in idle/inactive UEs, therefore Proposal 2.4-1 uses table from ZTE as starting point for the discussion.

Point A as reference for starting PRB

[Nokia, Ericsson] propose to use The Point A as reference of the starting PRB for the CFR configuration of RRC idle/inactive UEs as it has been agreed for multicast. [CATT] proposes that for case C, this configuration can be optional.

This discussion is related to Issue 6 on definition of CFR and down-selection of case D/E. For Case A/C they may reuse the configuration from CORESET#0/SIB-1 initial BWP and may not require a dedicated configuration to indicate the frequency resources within the carrier. Proposal 2.4-2 is put for discussion and collect company views.

Different BW configurations for MCCH and MTCH

[Huawei, ZTE, Nokia] propose that MCCH and MTCH can have different bandwidth configurations between MCCH and MTCH, where the frequency resources of the MTCH can be larger and contain the frequency resources of MCCH.

[CATT, Ericsson, OPPO, Xiaomi, Lenovo, MediaTek] only support that both MCCH and MTCH have the same frequency resources for the CFR. [Ericsson] argues that if power saving is the criteria for configuring a frequency resource smaller for MCCH than for MTCH, power saving is better achieved by time domain DRX operation.

This aspect has been discussed at previous meetings without reaching a conclusion after various rounds of discussion. Based on the submitted tdocs, companies have not changed their position. Based on this, the FL proposes not to discuss this aspect and focus on other aspects that may require more discussion. Companies are welcome to provide their views in the table below.

Different PDCCH-Config and PDSCH-Config for MCCH and MTCH

[Huawei, Qualcomm, CMCC, Apple] discuss that the some of the PDSCH configurations of MCCH and MTCH can be different, for example the MCS Table. [Huawei] discusses that for PDSCH carrying MCCH table 5.1.3.1-1 in TS 38.214 could be used while for PDSCH carrying MTCH MCS should be more flexible, i.e., qam256 or qam64LowSE can be configured by higher layers. [Ericsson] also argues that the PDCCH-Config could also be different between MCCH and MTCH where different Monitoring Occasions are configured for MCCH and MTCH. [Apple] notes that MCCH related configurations will be carried by MSB specific SIB, it should include the CFR configuration to enable the reception of GC-PDCCH/PDSCH carrying the MCCH. The scheduling information for MTCH is included in MCCH contents, it’s natural CFR configurations for MTCH is delivered via the MCCH. [Asustek] discusses that only basic parameters should be configured.

[MediTek] proposes that both MCCH and MTCH are both configured via MBS specific SIB.

There is stronger support for having different PDCCH/PDSCH-Config parameters between MCCH and MTCH and justification on why some parameters could be different has been provide for this meeting. Based on this Proposal 2.4-3 puts forward an agreement to enable this.

RateMatchPattern

[CATT] proposes further discussion on RateMatchPattern / RateMatchPatternLTE-CRS. Given that this aspect was discussed at the past meeting without reaching an agreement and the limited time to close critical aspects in this meeting, the FL proposes to focus on the other aspects first. Companies are welcome to provide inputs in table below.

On the determination of modulation and target code-rate & TBS

The following agreements have been made in AI 8.12.1 for RRC connected UEs:

Agreement: For LBRM and TBS determination for GC-PDSCH: - The maximum number of layers can be provided by maxMIMO-Layers in PDSCH-Config for MBS in CFR; if not provided, a default value is defined. - FFS the default value. - The maximum modulation order can be determined from mcs-Table in PDSCH-Config for MBS in CFR; - FFS: if mcs-Table in PDSCH-Config for MBS is not configured in CFR, a value determined from mcs-Table in PDSCH-Config for unicast in the active DL BWP is used; if the mcs-Table in PDSCH-Config for unicast is not configured, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16). - xOverhead can be provided in PDSCH-Config for MBS in CFR; if not provided, a default value of zero is used. - The number of PRBs is determined based on the size of CFR. Agreement: For LBRM and TBS determination for GC-PDSCH, the default value of the maximum number of layers is 1 if maxMIMO-Layers in PDSCH-Config for MBS in CFR is not configured. Agreement: For determination of maximum modulation order for LBRM and TBS determination for GC-PDSCH, - if mcs-Table in PDSCH-Config for MBS is not configured in CFR, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16).

However, these have not been discussed for broadcast reception in idle/inactive UEs. It is not clear whether this agreement do also apply for broadcast reception. However, as per the draft CR on TS 38.214 [R1-2112485] below, the above agreements have only been implemented for multicast.

Therefore, the FL includes Proposal 2.4-4 to confirm whether these agreements can also be confirmed for broadcast reception with UEs in idle/inactive RRC states.

### 1st round FL proposals for Issue 4

#### Proposal 2.4-1

Adding the following PDSCH TDRA table determination rule for broadcast to Table 5.1.2.1.1-1 of TS38.214.

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | pdsch-ConfigCommon includes pdsch-TimeDomainAllocationList | pdsch-Config includes pdsch-TimeDomainAllocationList | pdsch-Config-broadcast includes pdsch-TimeDomainAllocationList | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- | --- |
| MCCH_RNTI, G_RNTI for broadcast | Type-x Common for broadcast | 1 | No | - | No | Default A (if SearchSpaceZero is configured) |
|  |  | 2 | No | - | No | Default B (if SearchSpaceZero is configured) |
|  |  | 3 | No | - | No | Default C (if SearchSpaceZero is configured) |
|  |  | 1,2,3 | No | - | No | Default A (if SearchSpaceZero is NOT configured) |
|  |  | 1,2,3 | Yes | - | No | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | pdsch-TimeDomainAllocationList provided in pdsch-Config-broadcast |

#### Proposal 2.4-2

For Case D/E (if supported), the definition of the broadcast BWP/CFR frequency resources reuses the legacy definition of BWP frequency resources for unicast using the combination of Point A, offsetToCarrier and locationAndBandwidth to indicate the exact location of the BWP/CFR with respect to the carrier starting RB.

#### Proposal 2.4-3

For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs:

- The CFR frequency resources used for MCCH and MTCH are configured by SIBx;

- PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MCCH is configured by SIBx

- PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MTCH is configured by MCCH. If the PDCCH-config/PDSCH-config for MTCH is not configured, the PDCCH-config/PDSCH-config for GC-PDCCH/PDSCH carrying MCCH configured by SIBx is reused for GC-PDCCH/PDSCH carrying MTCH.

#### Proposal 2.4-4

The following agreements for RRC_CONECTED UEs also apply for broadcast reception with UEs in RRC_IDLE/ RRC_INACTIVE states:

Agreement:

For LBRM and TBS determination for GC-PDSCH:

- The maximum number of layers can be provided by maxMIMO-Layers in PDSCH-Config for MBS in CFR; if not provided, a default value is defined.

  - FFS the default value.

- The maximum modulation order can be determined from mcs-Table in PDSCH-Config for MBS in CFR;

  - FFS: if mcs-Table in PDSCH-Config for MBS is not configured in CFR, a value determined from mcs-Table in PDSCH-Config for unicast in the active DL BWP is used; if the mcs-Table in PDSCH-Config for unicast is not configured, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16).

- xOverhead can be provided in PDSCH-Config for MBS in CFR; if not provided, a default value of zero is used.

- The number of PRBs is determined based on the size of CFR.

Agreement:

For LBRM and TBS determination for GC-PDSCH, the default value of the maximum number of layers is 1 if maxMIMO-Layers in PDSCH-Config for MBS in CFR is not configured.

Agreement:

For determination of maximum modulation order for LBRM and TBS determination for GC-PDSCH,

- if mcs-Table in PDSCH-Config for MBS is not configured in CFR, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16).

Please provide your answers in the table below. Considering the FL assessment above, do you support proposals above? Please provide reasons and views in general. Please provide any alternate proposals in case you don’t support the proposals.

| company | comments |
| --- | --- |
| LG Electronics | #### Proposal 2.4-3: We are fine with this proposal. |
| NOKIA/NSB | #### Proposal 2.4-1: Please find our proposal of the table in below RNTI; PDCCH search space; SS/PBCH block and CORESET multiplexing pattern; pdsch-ConfigCommon includes pdsch-TimeDomainAllocationList; pdsch-Config includes pdsch-TimeDomainAllocationList; pdsch-Config-broadcast includes pdsch-TimeDomainAllocationList; PDSCH time domain resource allocation to apply. MCCH_RNTI, G_RNTI for broadcast; Type-x Common for broadcast; 1; No; -; - No; Default A (if SearchSpaceZero is configured). 2; No; -; - No; Default B (if SearchSpaceZero is configured). 3; No; -; - No; Default C (if SearchSpaceZero is configured). 1,2,3; No; -; No; Default A (if SearchSpaceZero is NOT configured). 1,2,3; Yes; -; No; pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon. 1,2,3; No/Yes; -; Yes; pdsch-TimeDomainAllocationList provided in pdsch-Config-broadcast. #### Proposal 2.4-2: Not only for Case D/E, also for Case C, all 3 cases should be configured with the same manner. #### Proposal 2.4-3: Not OK #### For the 1st sub-bullet, to our view, the CFR for MCCH and MTCH can be different, and the MCCH CFR can be configured by SIBx. And the MTCH CFR can be the same as MCCH CFR, if MTCH CFR is not configured. Alternatively, the MTCH CFR can be also configured differently from MCCH CFR via MCCH configuration. - For the 2nd sub-bullet, we are OK with it - For the 3rd sub-bullet, OK #### Proposal 2.4-4: To our view, for Rel17 MBS with broadcast reception, single-MIMO layer with low MCS associated MCS table is enough for robust operation of broadcast reception based on SSB. |
| NTT DOCOMO | #### We are fine with the four proposals. |
| Lenovo, Motorola Mobility | #### 2.4.1: we prefer default TDRA table as legacy operation. 2.4.2: we don’t support as Case D or Case E is not agreed now. 2.4.3: we have some comments on the proposal. (1) for the first sub-bullet, is there a single CFR for both MCCH and MTCH? (2) for the third sub-bullet, we think same SIBx for configuring MCCH and MTCH should not be excluded. We suggest replacing “is” with “can be” and including “SIBx”. #### 2.4.4: OK. |
| Xiaomi | #### Proposal 2.4-1: We are not sure why search space#0 matters for TDRA table selection. #### Proposal 2.4-2: Considering there is no consensus on case E, we propose to remove ‘case E(if supported)’. Proposal 2.4-3: OK Proposal 2.4-4: OK |
| CATT | #### Proposal 2.4-2: OK #### Proposal 2.4-3: OK |
| OPPO | #### Proposal 2.4-2: It would be better not mentioning the specific cases on CFR which is still under discussion. For Case D/E (if supported), tThe definition of the broadcast BWP/CFR frequency resources reuses the legacy definition of BWP frequency resources for unicast using the combination of Point A, offsetToCarrier and locationAndBandwidth to indicate the exact location of the BWP/CFR with respect to the carrier starting RB. #### Proposal 2.4-3: OK. #### Proposal 2.4-4: OK |
| Samsung | #### Agree |
| ZTE | #### Proposal 2.4-1: We support this proposal. The motivation to have separate TDRA table for SS#0 and other SS#0 is as following. Most of SLIVs in default B table and default C table are short PDSCH length in order to align with the beam of SSBs, e.g., L=2 or L=4. This restriction is Ok for SS#0. However, if search space other than 0 is used for MBS reception, the above restriction is invalid and not needed. And it will seriously affect the flexibility and capacity of MBS transmission, because most of the entities only have a length of 2 or 4 symbols for PDSCH allocation. This does not meet the requirement of mass data transmission, e.g., video service under MBS. The following approach is proposed to address this issue. In case of other SS, default A is the better choice. Proposal 2.4-2: Similar view as Nokia, it should be the same for Case C, Case D and Case E. #### Proposal 2.4-3: We are fine with this proposal in general. We also see some benefits of supporting different CFRs for MCCH and MTCH. #### Proposal 2.4-4: For broadcast, maybe single-MIMO-layer is enough. Regarding the MCS table, maybe we can also define one default MCS table in the specification. |
| vivo | #### Proposal 2.4-3: Shall we allow a case that PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MCCH is not configured? |
| MediaTek | #### Proposal 2.4-2: Ok #### Proposal 2.4-3: Support |
| CMCC | #### Proposal 2.4-1: Prefer Nokia’s version #### Proposal 2.4-2: Don’t need to say Case D/E in the main bullet. Proposal 2.4-3: OK #### Proposal 2.4-4: OK |
| Ericsson | #### P2.4.2: Support P2.4.2: Support #### P2.4.3: Support |
| Apple | #### Proposal 2.4-2: OK Proposal 2.4-3: OK Proposal 2.4-4: OK |
| Qualcomm | #### Ok with the proposals #### Our understanding of the first subbullet of Proposal 2.4-3 is only talking about the frequency bandwidth size of the CFR for MCCH and MTCH. |
| Intel | #### OK with the proposals |
| Moderator | Thanks for comments. Proposal 2.4-1 Nokia has provided some changes that may address also comments from Xiaomi as well as preferred by other companies [CMCC]. The changes from Nokia are put forward for agreement. Proposal 2.4-2 [Nokia, OPPO, ZTE, CMCC] have proposed that the methodology to indicate the resources within the carrier should be common to all cases. This suggestion is put forward for agreement. Proposal 2.4-3 Some comments/clarifications: @Nokia, while I understand that some companies prefer that the frequency resources of the CFR of MCCH and MTCH can be different, I think that the majority of companies only support that both MCCH and MTCH have the same frequency resources. I think this is the most agreeable form. @Lenovo: on 1) yes, both MCCH and MTCH would be configured with the same frequency resources for the CFR, i.e., frequency range is not different. On 2) this is what the proposal is saying on the third sub-bullet. MCCH is configured by SIBx. MTCH is configured by MCCH. If MTCH is not configured in the MCCH, then the values that have been used to configure MCCH by SIBx are also used for MTCH. @ZTE: thanks for the compromise. @vivo: I am not sure I completely understand your point, can you please elaborate? thanks. @Qualcomm: yes. Besides the clarifications, there seems to be good support for this proposal, it remains unchanged and I would like to check whether it is agreeable after clarifications. Proposal 2.4-4: Multiple companies have shown support. Some companies have indicated that single layer MIMO may be enough. @Nokia, ZTE: single-layer MIMO is the only scheme supported so far, since there is no agreement on including a second DCI. Do you think we still need a clarification, or can you live with the current wording? |

### 2nd round FL proposals for Issue 4

#### Proposal 2.4-1rev1

Adding the following PDSCH TDRA table determination rule for broadcast to Table 5.1.2.1.1-1 of TS38.214.

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | pdsch-ConfigCommon includes pdsch-TimeDomainAllocationList | pdsch-Config includes pdsch-TimeDomainAllocationList | pdsch-Config-broadcast includes pdsch-TimeDomainAllocationList | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- | --- |
| MCCH_RNTI, G_RNTI for broadcast | Type-x Common for broadcast | 1 | No | - | - No | Default A (if SearchSpaceZero is configured) |
|  |  | 2 | No | - | - No | Default B (if SearchSpaceZero is configured) |
|  |  | 3 | No | - | - No | Default C (if SearchSpaceZero is configured) |
|  |  | 1,2,3 | No | - | No | Default A (if SearchSpaceZero is NOT configured) |
|  |  | 1,2,3 | Yes | - | No | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | pdsch-TimeDomainAllocationList provided in pdsch-Config-broadcast |

#### Proposal 2.4-2rev1

For Case D/E (if supported), the definition of the broadcast BWP/CFR frequency resources reuses the legacy definition of BWP frequency resources for unicast using the combination of Point A, offsetToCarrier and locationAndBandwidth to indicate the exact location of the BWP/CFR with respect to the carrier starting RB.

#### Proposal 2.4-3

For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs:

- The CFR frequency resources used for MCCH and MTCH are configured by SIBx;

- PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MCCH is configured by SIBx

- PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MTCH is configured by MCCH. If the PDCCH-config/PDSCH-config for MTCH is not configured, the PDCCH-config/PDSCH-config for GC-PDCCH/PDSCH carrying MCCH configured by SIBx is reused for GC-PDCCH/PDSCH carrying MTCH.

#### Proposal 2.4-4

The following agreements for RRC_CONECTED UEs also apply for broadcast reception with UEs in RRC_IDLE/ RRC_INACTIVE states:

Agreement:

For LBRM and TBS determination for GC-PDSCH:

- The maximum number of layers can be provided by maxMIMO-Layers in PDSCH-Config for MBS in CFR; if not provided, a default value is defined.

  - FFS the default value.

- The maximum modulation order can be determined from mcs-Table in PDSCH-Config for MBS in CFR;

  - FFS: if mcs-Table in PDSCH-Config for MBS is not configured in CFR, a value determined from mcs-Table in PDSCH-Config for unicast in the active DL BWP is used; if the mcs-Table in PDSCH-Config for unicast is not configured, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16).

- xOverhead can be provided in PDSCH-Config for MBS in CFR; if not provided, a default value of zero is used.

- The number of PRBs is determined based on the size of CFR.

Agreement:

For LBRM and TBS determination for GC-PDSCH, the default value of the maximum number of layers is 1 if maxMIMO-Layers in PDSCH-Config for MBS in CFR is not configured.

Agreement:

For determination of maximum modulation order for LBRM and TBS determination for GC-PDSCH,

- if mcs-Table in PDSCH-Config for MBS is not configured in CFR, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16).

Please provide your answers in the table below. Considering the FL comments above:

- do you support revised proposals 2.4-1rev1 and 2.4-2rev1?

- after the clarifications provided, do you support Proposals 2.4-3 and 2.4-4.

| company | comments |
| --- | --- |
| ZTE | #### We support the previous Proposal 2.4-1 rather than 2.4-1rev1. We have provided the reason for this in the previous round, let us further clarify it. The following is the default table B. Most of SLIVs in default B table and default C table are short PDSCH length in order to align with the beam of SSBs, e.g., L=2 or L=4. This restriction is Ok for SS#0 since the PDSCH has to use the same as the overlapped SSB. However, if search space other than 0 is used for MBS reception, the above restriction is invalid and not needed. The beam indication should be more flexible and the SLIV length should be more flexible. Thus, in case of SS other than SS#0, default A table is preferred. Table 5.1.2.1.1-4: Default PDSCH time domain resource allocation B Row index; dmrs-TypeA-Position; PDSCH mapping type; K0; S; L. 1; 2,3; Type B; 0; 2; 2. 2; 2,3; Type B; 0; 4; 2. 3; 2,3; Type B; 0; 6; 2. 4; 2,3; Type B; 0; 8; 2. 5; 2,3; Type B; 0; 10; 2. 6; 2,3; Type B; 1; 2; 2. 7; 2,3; Type B; 1; 4; 2. 8; 2,3; Type B; 0; 2; 4. 9; 2,3; Type B; 0; 4; 4. 10; 2,3; Type B; 0; 6; 4. 11; 2,3; Type B; 0; 8; 4. 12 (Note 1); 2,3; Type B; 0; 10; 4. 13 (Note 1); 2,3; Type B; 0; 2; 7. 14 (Note 1); 2; Type A; 0; 2; 12. 3; Type A; 0; 3; 11. 15; 2,3; Type B; 1; 2; 4. 16; Reserved. Note 1: If the PDSCH was scheduled with SI-RNTI in PDCCH Type0 common search space, the UE may assume that this PDSCH resource allocation is not applied. Proposal 2.4-2rev1: Prefer to add “For Case C, Case D (if supported) and Case E (if supported)” to make it clear. Proposal 2.4-4: It seems companies have the same understanding, it is just how to word it. We would suggest the following proposal to make it clear. Proposal: For LBRM and TBS determination for GC-PDSCH for broadcast, The maximum number of layers is 1 The maximum modulation order can be determined from mcs-Table in PDSCH-Config in CFR for broadcast, if mcs-Table in PDSCH-Config is not configured in CFR for broadcast, Table 5.1.3.1-1 in TS38.214 is used. |
| NOKIA/NSB | #### Proposal 2.4-1rev1 and Proposal 2.4-2rev1: Support BTW, the exact term of “Type-x” in Proposal 2.4-1rev1 may need to be updated later Proposal 2.4-3: Not supporting the first sub-bullet point, where to our view, the CFR for MTCH can be configured by MCCH, and the CFR for MCCH and MTCH can be different. Generally, as in legacy SC-PTM, the contents of MCCH carrying the scheduling information of MTCH. And the MTCH CFR is one kind of scheduling information of MTCH. Thus, it is proposed that - the CFR frequency resource used for MTCH is configured by MCCH, - and the CFR frequency resource used for MCCH is configured by SIBx, - if the CFR frequency resource used for MTCH is NOT configured, then the CFR frequency resources configured for MCCH is applied. #### Proposal 2.4-4: @FL, to our view, an agreement could be great to clarify if only single MIMO layer is supported for Rel17 MBS with broadcast reception. And in such case, the broadcast reception UE should not expect the configuration of “the maximum number of layers can be provided by maxMIMO-Layers in PDSCH-Config for MBS in CFR”, and if it is errorly configured by gNB, then the broadcast reception UE should ignore the configuration, and assume only the single MIMO layer applied for broadcast reception. |
| Xiaomi | #### We are OK with all the proposals. |
| OPPO | #### OK with the proposals by moderator. |
| Lenovo, Motorola Mobility | #### 2.4-1: OK. 2.4-2: For Case A, it is not necessary to define the BWP/CFR using combination of Point A, offsettocarrier and locationandbandwidth. For Case C, the BWP/CFR is already defined via SIB-1. Do we need to define it again? 2.4-3: OK. 2.4-4: OK. |
| NTT DOCOMO | #### Proposal 2.4-1rev1: Support #### Proposal 2.4-2rev1: Support #### Proposal 2.4-3: Support #### Proposal 2.4-4: Support |
| MeidaTek | #### Proposal 2.4-3: Support the proposal. #### Proposal 2.4-4: Support. |
| CMCC | #### Support all the proposals |
| CATT | #### We are OK with all the proposals. |
| Ericsson | #### 2.4-2rev1: Support #### 2.4-3: Support #### 2.4-4: Support |
| Samsung | #### OK |
| Moderator | Thanks for the comments. I will provide more updates for all the proposals after the GTW session, but regarding Proposal 2.4-4, based on the comments #### Proposal 2.4-4rev1 The following agreements for RRC_CONECTED UEs also apply for broadcast reception with UEs in RRC_IDLE/ RRC_INACTIVE states, with the following updates: Agreement: For LBRM and TBS determination for GC-PDSCH: - The maximum number of layers can be provided by maxMIMO-Layers in PDSCH-Config for MBS in CFR; if not provided, a default value is defined. - FFS the default value. - The maximum modulation order can be determined from mcs-Table in PDSCH-Config for MBS in CFR; - FFS: if mcs-Table in PDSCH-Config for MBS is not configured in CFR, a value determined from mcs-Table in PDSCH-Config for unicast in the active DL BWP is used; if the mcs-Table in PDSCH-Config for unicast is not configured, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16). - xOverhead can be provided in PDSCH-Config for MBS in CFR; if not provided, a default value of zero is used. - The number of PRBs is determined based on the size of CFR. Agreement: For LBRM and TBS determination for GC-PDSCH, the default value of the maximum number of layers is 1 if maxMIMO-Layers in PDSCH-Config for MBS in CFR is not configured. Agreement: For determination of maximum modulation order for LBRM and TBS determination for GC-PDSCH, - if mcs-Table in PDSCH-Config for MBS is not configured in CFR, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16). For LBRM and TBS determination for GC-PDSCH for broadcast reception: - the maximum number of layers is 1 - the maximum modulation order can be determined from mcs-Table in PDSCH-Config for broadcast. - If mcs-Table in PDSCH-Config is not configured in CFR for broadcast, Table 5.1.3.1-1 in TS38.214 is used. |

## Issue 5: Beam Sweeping for MCCH and MTCH channels

### Background

The following agreement for RRC_IDLE/RRC_INACTIVE UEs at RAN1#103-e, RAN2#104-e, RAN1#105-e, RAN1#106-e and RAN1#106bis-e are relevant for this discussion:

Agreements: - For RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH. - FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH. Agreement: For RRC_IDLE/RRC_INACTIVE Ues, for broadcast reception, the UE may assume that group-common PDCCH/PDSCH is QCL’d with SSB. - It is up to UE implementation whether UE monitors monitoring occasions corresponding to all SSB indexes or monitoring occasions corresponding to a subset of all SSB indexes. - FFS: association rules between SSB indexes and UE monitoring occasions. - FFS: group-common PDCCH/PDSCH is QCl’d with TRS if configured Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, RAN1 confirms the following assumptions made by RAN2 - RAN2 assumes, in case searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the mapping between PDCCH occasions and SSBs is the same as for SIB1. - RAN2 assumes that if common search space other than searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols are sequentially numbered from one in the MCCH transmission window and mapped to SSBs using the similar rule as defined for OSI in TS 38.331. Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, the same beam can be used for group-common PDCCH and the corresponding scheduled group-common PDSCH for carrying MCCH or MTCH. - UE may assume that DMRS ports of the group-common PDCCH/PDSCH for MCCH is QCL’d with SSB. - UE may assume that DMRS ports of the group-common PDCCH/PDSCH for MTCH is QCL’d with SSB. - FFS: group-common PDCCH/PDSCH for MTCH is QCL’d with periodic TRS if configured Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MCCH. Agreement: For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs support the same CSS type for MCCH and MTCH. - FFS support of different CSS types for MCCH and MTCH channels for broadcast reception. Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, if searchSpace#0 is configured for MTCH, the mapping between PDCCH occasions and SSBs is the same as for SIB1. Agreement: For RRC_IDLE/RRC_INACTIVE UEs with broadcast reception, if common search space other than searchSpace#0 is configured for MTCH, the mapping of PDCCH monitoring occasions to SSBs can be configured with a rule. - The existing rule defined for OSI in TS 38.331 is used as starting point to define the above rule. Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MTCH. Agreement: For RRC_IDLE/RRC_INACTIVE UEs for broadcast reception, MTCH scheduling is associated with a window defined by the MTCH monitoring periodicity and the starting of the periodicity - FFS: the window is associated to one or multiple or all G-RNTI. Agreement: For RRC_IDLE/RRC_INACTIVE UEs for broadcast reception, at least support that within the MTCH scheduling window, the association between the PDCCH monitoring occasions and SSB is defined as: - the [x×N+K]th PDCCH monitoring occasion(s) for MTCH in the scheduling window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in MTCH transmission window/N). - For the purpose of associating PDCCH monitoring occasion for MTCH and SSB, the UE assumes that, in the MTCH scheduling window, PDCCH for an MTCH scrambled by G-RNTI is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

The following agreements form RAN2#113bis-e meeting are relevant for this discussion:

- The concept of MCCH transmission window, similar to the one used for LTE SC-PTM, is used for NR MCCH scheduling. The exact parameters to define the window are FFS (discussed in the following proposals). - The MCCH transmission window is defined by MCCH repetition period, MCCH window duration and radio frame/slot offset. - R2 assumes PDCCH occasions for MCCH search space are associated with SSBs in a pre-defined manner so that the UE can receive MCCH scheduling on PDCCH occasions according to its detected SSB. - R2 assumes, In case searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the mapping between PDCCH occasions and SSBs is the same as for SIB1. - R2 assumes that If common search space other than searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols are sequentially numbered from one in the MCCH transmission window and mapped to SSBs using the similar rule as defined for OSI in TS 38.331.

The following agreements form RAN2#115-e meeting are relevant for this discussion:

- If Data Inactivity timer is configured, data monitoring is applied both for unicast and MBS multicast (i.e. both PTM and PTP data) (but not MBS broadcast) - The Multicast Long DRX operation has to support the following parameters which are similar to the UE-specific DRX for unicast, where the last two parameters are needed if the HARQ- feedback is enabled: - drx-onDurationTimerPTM - drx-InactivityTimerPTM - drx-LongCycleStartOffsetPTM - drx-SlotOffsetPTM - drx-HARQ-RTT-TimerDLPTM - drx-RetransmissionTimerDLPTM - For NR Broadcast, the DRX pattern is configured per G-RNTI. - For NR Broadcast, DRX configuration includes: drx-onDurationTimerPTM, drx-SlotOffsetPTM, drx-InactivityTimerPTM, drx-CycleStartOffsetPTM.

### Tdoc analysis

- In [R1-2110779, Huawei]

  - Discuss: In addition, an offset should be defined to determine the starting of the window, and it should not be replaced by the parameters defined for DRX because of the offset for beam sweeping window should be independent from DRX configurations.

  - Discuss: In our opinion, since the SSB is associated with the PDCCH monitoring occasions in search space, and the GC-PDCCHs scrambled by different G-RNTI may use different search spaces, the window should be associated to one G-RNTI.

  - Proposal 8: The MTCH transmission window is associated to one G-RNTI.

  - Proposal 9: An offset to the starting of the MTCH transmission window should be defined, e.g., OG-RNTI:

    - the PDCCH monitoring occasion(s) in slot nslot in the frame SFN is given by SFN∙Nslot+nslot-OG-RNTImod KG-RNTI=0, where Nslot is the number of slots in a radio frame.

- In [R1-2110897, TD Tech]

  - Proposal 12: For a CSS for MCCH other than search space 0, the mapping between GC-PDCCH occasions and SSB beams within each transmission window of MCCH is defined as:

    - The GC-PDCCH occasions within each transmission window are numbered in sequence with index 0 for the first GC-PDCCH occasion.

    - The GC-PDCCH occasion with index k=(N*x+n) is associated with SSB beam n, where n=0,…,N-1, N is the number of the SSB beams, SSB beam n is for SSB index n, x=0,…,INT[L/N]-1, and L is the number of the GC-PDCCH occasions in each transmission window.

  - Proposal 13: For a CSS for MTCH other than search space 0, the mapping between GC-PDCCH occasions and SSB beams within each monitoring period of the CSS is defined as:

    - The GC-PDCCH occasions within each monitoring period are numbered in sequence with index 0 for the first GC-PDCCH occasion.

    - The GC-PDCCH occasion with index k=(N*x+n) is associated with SSB beam n, where n=0,…,N-1, N is the number of the SSB beams, SSB beam n is for SSB index n, x=0,…,INT[L1/N]-1, and L1 is the number of the GC-PDCCH occasions in each monitoring period.

- In [R1-2111137, Nokia]

  - Proposal-22: It is preferred to keep the robust SSB-based beam sweeping operation as SIB for RRC_IDLE/INACTIVE UEs for both MCCH and MTCH in Rel17 MBS.

  - Discuss: To our understanding, the configured MTCH scheduling window corresponds to the broadcast DRX pattern, where the corresponding parameters, i.e. onDuration (periodicity) and SlotOffset (starting of the periodicity) have already been agreed and defined in RAN2-115-e meeting, as shown in below. And correspondingly, the MTCH scheduling window is configured per G-RNTI, and practically a MTCH scheduling window can be configured by gNB to be associated with multiple G-RNTI.

  - Proposal-23: The configured MTCH scheduling window corresponds to the broadcast DRX pattern, as already been agreed in RAN2.

  - Proposal-24: The MTCH scheduling window is configured per G-RNTI, and practically a MTCH scheduling window can be configured by gNB to be associated with multiple G-RNTI.

  - Proposal-25: It is proposed to consider additional association rules between SSB indexes and UE monitoring occasions other than the rule defined for OSI in TS 38.331.

  - Proposal-26: Consider the SSB index to PDCCH MO mapping across the MBS window can be “disabled” by network. Thus, the mapped number of mapped SSB beams can be evenly distributed among each MCCH window duration.

  - Proposal-27: Allow the network to control the number of repetition transmission for each SSB beam within the on-duration window.

- In [R1-2111232, CATT]

  - Proposal 7: In NR MBS system, PDCCH MOs in one MBS-window length are allocated to one SSB with consecutive MO should be considered.

  - Proposal 8: The MTCH scheduling window can be associated with one or multiple or all G-RNTI.

- In [R1-2111305, OPPO]

  - Discuss: First, whether multiple MTCH windows can be overlapped in time domain should be discussed. If multiple MTCH windows can be overlapped, and each window is associated with one G-RNTI, then it may work normally, but it increases the complexity. Within a MTCH window, if different G-RNTIs are associated with different MTCH TDM, it technically works. However, it still needs discussion that whether G-RNTI should be associated with MTCH window or they are two independent scheduling components. From the perspective of system and simple scheduling mechanism, a MTCH window associated with one G-RNTI is preferred for broadcast MBS services in this release.

  - Proposal 11: One MTCH window is associated with one G-RNTI.

- In [R1-2111551, Xiaomi]

  - Proposal 10: The association between the PDCCH monitoring occasions and SSB within the MCCH scheduling window is same as that of MTCH scheduling window.

  - Discuss: One open issue is how to define the association between the scheduling window and G-RNTI, considering more than one G-RNTI can be configured for a MBS UE. From UE complexity point of view, there is no additional burden for a UE tries to decode a DCI with different RNTI assumptions. On the other hand, gNB can configure multiple search spaces in a single monitoring occasion if different services may collide in time domain. Hence we have the following proposal:

  - Proposal 11: A MTCH scheduling window is associated with all G-RNTIs configured by gNB.

- In [R1-2112065, LGE]

  - Discuss: For scheduling of OSI in NR, different SI messages can be scheduled in different SI windows with different scheduling parameters e.g. different SI periodicities. Multiple SIBs having a same SI periodicity can be scheduled in a same SI message while different SIBs having different SI periodicities can be separately scheduled in different SI messages. With such basic principle of scheduling OSI, we think that group common transmissions for different G-RNTIs with different traffic patterns can be scheduled in different transmission windows, while Group common transmissions for different G-RNTIs with similar traffic pattern can be scheduled in same transmission windows.

  - Observation 3: Different SI messages can be scheduled in different SI windows with different scheduling parameters e.g. different SI periodicities.

  - Proposal 3A: Group common transmissions for different G-RNTIs with different traffic patterns or even for different SFN areas can be scheduled in different transmission windows. Different transmission windows can be configured with different window lengths as well as different periodicities of transmission windows, depending on MTCH traffic characteristics.

  - Proposal 3B: Group common transmissions for different G-RNTIs with similar traffic pattern can be scheduled in same transmission windows. If SFN is used, group common transmissions for different G-RNTIs in the same cell group i.e. the same SFN area can be scheduled in same transmission windows.

  - Observation 4: A certain broadcast service may be available only at a specific local service area within a cell. Besides, if a cell is at the boundary of SFN area, only a limited number of SSBs could participate in the SFN area for one or more services in the cell group in SFN.

  - Proposal 4: For a certain broadcast service, the number of actual transmitted SSBs used to determine PDCCH monitoring occasions within certain transmission windows can be smaller than the number of SSBs determined in SIB1. Different transmission windows can be configured with different number of actual transmitted SSBs, depending on actual broadcast service area.

  - Observation 5: RAN2 agreed that MCCH contents should include information about broadcast sessions such as G-RNTI, MBS session ID as well as scheduling information for MTCH (e.g. search space, DRX).

  - Proposal 5: PDCCH monitoring occasions are determined in DRX on-durations for MTCH of a broadcast service for idle/inactive UEs.

- In [R1-2112130, NTT DOCOMO]

  - Discuss: An MTCH scheduling window will be useful to limit the duration of the MTCH reception processing. Regarding the association between the scheduling window and G-RNTI, one window for all G-RNTI would be sufficient. Because RAN2 has already agreed that the DRX pattern for broadcast is configured per G-RNTI [5]. Even if there is only one MTCH scheduling window, the ‘on duration’ for each G-RNTI can be configured within the window. We don’t see a clear benefit in defining a different window for each G-RNTI.

  - Proposal 7: An MTCH scheduling window is associated with all G-RNTI.

- In [R1-2112348, Ericsson]

  - Discuss: We seek clarification how the MTCH scheduling window is related to the DRX for Multicast MBS that is part of the running CR 38.321 (R2-2108926). The DRX parameters are contained in the MCCH, therefore we assume it is relevant for reception of broadcast in RRC idle/inactive, not only for multicast in RRC connected. MTCH scheduling for broadcast must be done in accordance with the PTM-DRX scheme, which is inherited from the unicast scheme, i.e. using parameters like onDurationTimer, InactivityTimer and drx-LongCycle and drx-StartOffset. Any MTCH transmission window would have to coincide with the onDuration of the DRX cycle. We therefore think that parameters like a MTCH transmission window offset and periodicity are not needed.
We believe that different MBS services can have different latency requirements and different typical packet interarrival time. In order to avoid that UEs interested in a service with relaxed latency and long interarrivals has to monitor PDCCH as frequently as that may be necessary for other services, we propose the window can be different for different G-RNTI.

  - Proposal 14: The MTCH scheduling is associated with one G-RNTI.

  - Proposal 15: For the purpose of associating PDCCH monitoring occasion for MTCH and SSB, the UE assumes that, in the MTCH scheduling window, if any PDCCH for an MTCH scrambled by G-RNTI is transmitted, then such a PDCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

### FL Assessment

i) On configuration of MTCH transmission window

- Monitoring periodicity and the starting of the periodicity

[Huawei] discusses that the configuration of the parameters of the MTCH scheduling window (monitoring periodicity and the starting of the periodicity) should not be determined by the DRX parameters and further provide details on the definition of the parameters. On the other hand, [Nokia, Ericsson, LGE, NTT DOCOMO] discuss/question whether the MTCH scheduling window parameters directly correspond to the DRX parameters defined in RAN2.

This question, whether window parameters are already determined by DRX parameters, was presented for discussion at the last meeting in the FL summary in one of the rounds of discussion for this issue. However, not many companies commented on this aspect. Question 2.4-1 is put for discussion to collect company comments.

- Association of window & G-RNTI(s)

While [Huawei, Nokia, CATT, OPPO, LGE, Ericsson] propose that the window should be associated to one G-RNTI, [CATT, Xiaomi, NTT DOCOMO] proposes that the window should be associated all G-RNTI. [LGE] also proposes that transmissions with different G-RNTIs can be transmitted in the same window.

Based on majority view, Proposal 2.4-2 tries to agree that the window should be associated with one G-RNTI.

ii) On clarifications of mapping of PDCCH monitoring occasions to SSBs for MTCH

[Ericsson] proposes to clarify one of the sub-bullets in one of the previous agreements to avoid forcing the network to transmit PDCCH even if there is no MTCH traffic in a window. Question 2.4-3 seeks feedback from companies on this proposal.

iii) Other aspects

- On additional association rules between SSB indexes and UE monitoring occasions

[Nokia, CATT, LGE] proposes that additional association rules between SSB indexes and UE MO other than those defined for OSI are considered. These aspects have been discussed as well in previous meeting. In previous meetings multiple companies did not support such approaches as they were not considered essential for this release. The FL therefore proposes to first focus on finishing critical aspects open for beam sweeping. Companies are welcome to provide their views in the table below.

- Same association between PDCCH MO and SSBs for MCCH and MTCH

[Xiaomi] proposes that the association between the PDCCH monitoring occasions and SSB within the MCCH scheduling window is same as that of MTCH scheduling window. As per the comment above, the FL proposes to first focus on finishing the critical issues open for beam sweeping. Companies are welcome to provide their views in the table below.

### 1st round FL proposals for Issue 5

#### Question 2.5-1

regarding the parameters of MTCH scheduling window, i.e., monitoring periodicity and the starting of the periodicity.

- Option-1: there is no need to define these parameters since they are already determined by the RAN2 parameters agreed for DRX for NR broadcast.

- Option-2: the configuration of the DRX and the MTCH scheduling window are independent and therefore these parameters need to be defined.

#### Proposal 2.5-2

For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs, the MTCH scheduling window is associated to one G-RNTI.

#### Question 2.5-3

Provide your views on whether the second sub-bullet of the following agreement made at RAN1#106bis-e:

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs for broadcast reception, at least support that within the MTCH scheduling window, the association between the PDCCH monitoring occasions and SSB is defined as:

- the [x×N+K]th PDCCH monitoring occasion(s) for MTCH in the scheduling window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in MTCH transmission window/N).

- For the purpose of associating PDCCH monitoring occasion for MTCH and SSB, the UE assumes that, in the MTCH scheduling window, PDCCH for an MTCH scrambled by G-RNTI is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

should be updates as follows:

- For the purpose of associating PDCCH monitoring occasion for MTCH and SSB, the UE assumes that, in the MTCH scheduling window, if any PDCCH for an MTCH scrambled by G-RNTI is transmitted, then such a PDCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

Please provide your comments in the table below. Considering the FL assessment above:

- please provide your views on Questions 2.5-1 and 2.5-3.

- do you agree with Proposal 2.5-2? please provide your views and reasons.

| company | comments |
| --- | --- |
| LG Electronics | #### Question 2.5-1 RAN2 recently made agreements as follows: - Confirm that the same PTM DRX configuration parameters can be applied to multiple G-RNTIs. - Allow RRC signalling to configure the same DRX configuration instance to multiple G-RNTIs. - In case mtch-schedulingInfo is absent for a G-RNTI (i.e. no PTM DRX), the UE should monitor for PDCCH scrambled with G-RNTI in any slot according to the search space configured for MTCH. Meanwhile, RAN1 agreed that MTCH scheduling is associated with a window defined by the MTCH monitoring periodicity and the starting of the periodicity. RAN1 also agreed to support the association between the PDCCH monitoring occasions and SSB within the MTCH scheduling window. FFS: the window is associated to one or multiple or all G-RNTI. In our view, the MTCH window is mainly determined for the association between the PDCCH monitoring occasions and SSB. Thus, we would still need the MTCH window defined by the periodicity and the starting of the periodicity. Note that the MTCH scheduling window could be simply changed to ‘MTCH window’, like the existing term SI-window. Accordingly, both the configuration of the DRX and the MTCH window can be ‘separately’ configured and used for a UE to determine when UE actually receive MTCH. Considering RAN2 agreements, we think that one or more G-RNTIs can be scheduled in a same MTCH window according to DRX configuration(s) of the G-RNTI(s), and if DRX is not configured (i.e. no PTM DRX), UE monitors PDCCH monitoring occasions only based on the MTCH windows for interested G-RNTI(s). #### Proposal 2.5-2: We think that this proposal is not aligned with what RAN2 recently agreed. - Confirm that the same PTM DRX configuration parameters can be applied to multiple G-RNTIs. - Allow RRC signalling to configure the same DRX configuration instance to multiple G-RNTIs. So, this proposal can be changed to: Proposal 2.5-2: For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs, the MTCH scheduling window is associated to one or more G-RNTIs based on DRX configuration. #### Question 2.5-3: We are OK with this update “if any PDCCH for an MTCH scrambled by G-RNTI is transmitted, then such’ in order to avoid forcing the network to transmit PDCCH even if there is no MTCH traffic in a window. |
| NOKIA/NSB | #### Question 2.5-1: Option-1 is preferred #### Proposal 2.5-2: Support To our information and knowledge, the above latest RAN2 agreements quoted by LG is only targeted for multicast, but it is not related to broadcast. #### Question 2.5-3: OK for the re-wording to avoid forcing network to send GC-PDCCH if there is nothing to transmit. |
| NTT DOCOMO | Question 2.5-1: We have the similar view with LG. We think the MTCH window is necessary to determine the association between PDCCH MO and SSB. Proposal 2.5-2: We prefer the modified version by LG. #### Question 2.5-3: We are fine with the update. |
| Xiaomi | #### Question 2.5-1: same views as LGE. Proposal 2.5-2: There is no additional UE complexity for UE is MTCH scheduling window is associated to all the G-RNTIs. Even the DRX is configured per G-RNTI, one-to-many mapping between window and G-RNTI works well. Hence we don’t support the proposal. Question 2.5-3: It depends on what is the answer to question 2.5-1. From our perspective, we don’t see the need to update the previous agreement unless something is broken. We prefer to defer this discussion until we achieve a consensus on question 2.5-3. |
| CATT | Proposal 2.5-2: The window can be associated to one/multiple/all G-RNTI based on traffic pattern and/or repetition. There is no need to limit the associated number. #### Proposal 2.5-3: OK |
| OPPO | Question 2.5-1: Similar view with LG Proposal 2.5-2: OK #### Question 2.5-3: We prefer not to change agreements unless it is not working. |
| Samsung | Question 2.5-1) Option 1. Proposal 2.5-2) Agree #### Question 2.5-3) It seems not to be needed for the modification. |
| ZTE | #### Question 2.5-1: From our perspective, this issue is more related to RAN2, we suggest to leave it to RAN2. #### Proposal 2.5-2: Although we support this proposal, we still think it can be up to RAN2. Question 2.5-3: OK. |
| vivo | #### Question 2.5-1: option 2 Proposal 2.5-2: we don’t see the motivation to associate one MTCH scheduling window to one G-RNTI, as multiple G-RNTIs can use the same search space #### Question 2.5-3: ok for updates |
| CMCC | Question 2.5-1: Option 1 Proposal 2.5-2: OK #### Question 2.5-3: Don’t need the update. |
| Ericsson | Question 2.5-1: Neither Option 1, nor Option 2 are OK. More discussion on this is required, which we will provide in a later comment. Proposal 2.5-2: We agree with LG’s proposed update: For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs, the MTCH scheduling window is associated to one or more G-RNTIs based on DRX configuration. Question 2.5-3: Support |
| Qualcomm | We think the proposals should be up to RAN2 decision. |
| Intel | Question 2.5-1: Option 1 Proposal 2.5-2: OK |

## Issue 6: Definition and down-selection for CFR of MCCH/MTCH

### Background

During RAN2#113bis-e meeting, RAN2 discussed further aspects of MCCH scheduling with RAN1 impacts. Here, we reproduce relevant RAN2 agreements relevant to the discussion on the CFR:

- Request RAN1 to discuss the details of the configuration of the bandwidth for MCCH reception. - UE in RRC IDLE/INACTIVE should be able to monitor/read both MCCH channel and SI/Paging without BWP switch. It is up to RAN1 to decide how this is ensured.

RAN2 in [R1-2104165] requests RAN1 to investigate and provide feedback, considering agreements made by RAN2 as indicated in the LS where the following request is relevant for the discussion on CFR:

- Details of the allowed transmission bandwidth/BWP configurations for MCCH transmission.

RAN2 discussed further the aspects related to MCCH design and made the following agreements during RAN2#114 meeting relevant to the discussion on CFR for MCCH/MTCH:

- MBS specific SIB is defined to carry MCCH configuration. - Postpone the discussion on whether dedicated MCCH configuration is required until RAN1 makes progress on BWP/CFR for MCCH. - We support single MCCH (in this release)

The following agreements for RRC_IDLE/RRC_INACTIVE UEs at RAN1#103-e, RAN1#104-e, RAN1#105-e, RAN1#106-e and RAN#93-e are relevant for this discussion:

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource(s) for group-common PDCCH/PDSCH. - the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: the relation of the common frequency resource(s) (if configured) and initial BWP. - FFS: whether to configure one/more common frequency resources - FFS: configuration and definition details of the common frequency resource Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, further study the following cases of a configured/defined specific common frequency resource (CFR) for group-common PDCCH/PDSCH, and identify which case(s) will be supported: - [Case E] the case where a CFR is defined based on a configured BWP. - In particular, study the following: - whether a configured BWP for MBS is needed or not. - whether BWP switching is needed or not. - In this study, the configured BWP has the following properties: - The configured BWP is different than the initial BWP where the frequency resources of this initial BWP are configured smaller than the full carrier bandwidth. - The CFR has the frequency resources identical to the configured BWP. - The configured BWP needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP. - Note: The configured BWP is not larger than the carrier bandwidth - the case where the initial BWP fully contains the CFR in the frequency domain. - In this study the following sub-cases are considered: - [Case B] A CFR with smaller size than the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP. - [Case D] A CFR with smaller size than the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP. - In particular, study the following: - Whether the considered two options with a CFR with smaller size than the initial BWP are needed or not for MBS. - the case where the initial BWP has same size as the CFR in the frequency domain. - In this study the following two sub-cases are considered: - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP. - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP. - In particular, study the following: - Whether the considered two options with a CFR with the same size as the initial BWP are needed or not for MBS. Agreement: For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use a configured/defined CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case A), to receive GC-PDCCH/PDSCH carrying MCCH. - Note: GC-PDCCH/PDSCH transmission within a narrower portion of the Initial BWP (where the initial BWP has the same frequency resources as CORESET0) is possible by implementation via appropriate scheduling. Agreement: For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use a configured/defined CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case A), to receive GC-PDCCH/PDSCH carrying MTCH. - Note: GC-PDCCH/PDSCH transmission within a narrower portion of the Initial BWP (where the initial BWP has the same frequency resources as CORESET0) is possible by implementation via appropriate scheduling. Conclusion: There is no specification support in Rel-17 for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs with configured/defined CFRs for group-common PDCCH/PDSCH with smaller size than the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case B). Agreement (Updated proposal from RAN1#106e): For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state. - Support Case-C - Support at least one of Case D and Case E. - Down-selection to be made at RAN1#106b-e - Note: Case C, D and E are defined in previous agreements

At RAN1#106bis-e the was no agreement on the down-selection of Case D/E and the following was reported in the Chair’s notes:

Proposal: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state. - Support Case D - Working assumption: Support Case E - Note: Case D and E are defined in previous agreements Objection sustained by: Lenovo, CMCC, Xiaomi, Spreadtrum, Oppo Proposal: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state. - Support Case D - FFS: Support Case E - Note: Case D and E are defined in previous agreements Objection sustained by: NSB, ZTE, vivo, LG, Qualcomm

### Tdoc analysis

- In [R1-2110891, Futurewei]

  - Proposal 1: For Idle/Inactive UEs broadcast reception, the common frequency resource (CFR) for group-common PDCCH/PDSCH is fully contained within the initial BWP and is configured by SIB.  Furthermore, the frequency resources for the CFR does not need to be equal to CORESET0 (Case D).

- In [R1-2110897, TD Tech]

  - Proposal 1: The CFR for MCCH is the initial DL BWP.

  - Proposal 4: The CFR for MTCH for broadcast mode can be larger than the initial DL BWP, which means the CFR for MTCH can contain the initial DL BWP and has the same numerology as the initial DL BWP.

- In [R1-2110912, ZTE]

  - Proposal 1: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC_IDLE/INACTIVE states, support both Case D and Case E.

- In [R1- 2111041, vivo]

  - Discuss: During transition from RRC CONNECTED before RRC (re)-configuration to RRC CONNECTED after RRC (re)-configuration, it is observed that service continuity is kept for case C and D when firstActiveDownlinkBWP-Id is not configured, but not kept when firstActiveDownlinkBWP-Id is configured to indicate the first active downlink BWP. And for case E, service continuity is kept when firstActiveDownlinkBWP-Id is configured to indicate the first active downlink BWP to the broadcast CFR. but not kept when firstActiveDownlinkBWP-Id is not configured. Consequently, depending on conditions, whether service continuity could be kept or not is quite similar for the three cases.

  - Discuss: Another issue is how network identifies UEs having interest or not in broadcast services. One possible approach is via MBS interest indicator, even though it maybe optional from UE side, it helps network to ensure the broadcast performance of the UEs reporting their MBS interest. For the UEs not reporting, network may not have information for their interest and broadcast performance doesn’t have to be guaranteed in this case. When UE reports its MBS interest, network can configure it with first active DL BWP including the broadcast CFR, otherwise, network can configure it with first active DL BWP by considering unicast and multicast only. The network operation on this issue is common in case-C, -D, and -E.

  - Discuss: It is not wise to consider case E as an optimized solution over case C, on the contrary, case E is complementary to case C and is essential to be supported in Rel-17.

  - Proposal 1: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state:

    - Support Case-C

    - Support Case-E.

    - Note: Case C and E are defined in previous agreements.

- In [R1-2111115, Spreadtrum]

  - Discuss: Regarding Case E, firstly, we have not seen the specfic use cases, which must be delivered in idle sate, and are high data volume. In NR Rel-15/Rel-16, only small data, or even no traffice data is allowed to be transmitted in idle state. High traffic volume is always transmitted in connected state. One reason is that it is higher efficiency and reliablity in connected state. The necesarity of introducing CFR with large bandwidth.e g., case E in idle state, is not clear to us.

  - Discuss: In idle state, no matter case C or case E, there is no impact on legacy UE. This is because that SIB1 configured initial DL BWP can be active only in RRC connnected state, and legacy UE only camp in the bandwidth of CORESET#0.

  - In RRC connnected state, assuming all MBS UEs report MBS interest indication to gNB, then for case C, gNB can configure first active BWP and default BWP by UE RRC signalling to make SIB1 configured initial DL BWP invalid. There is no impact on legacy UE even if in the case where SIB1 configured inital DL BWP is enlarged due to MBS as the proponent of case E claimed.

  - In RRC connnected state, assuming MBS UEs not report MBS interest indication to gNB, then for both case C and case E, it is completely up to gNB’s implementation to configure unicast BWP by RRC signalling for each UE, which may be larger than the bandwidth of BWP configured by case E/ SIB1 configured initial DL BWP, or equal to the bandwidth of BWP configured by case E/ SIB1 configured initial DL BWP, or smaller than the bandwidth of BWP configured by case E/ SIB1 configured initial DL BWP.  For this case, there is no difference between case C and case E.

  - In RRC connnected state, assuming MBS UEs not report MBS interest indication to gNB, and first active BWP is not configured by gNB for each UE, some companies of proponent E claim that  for case E, legacy UE use SIB1 configured intial BWP as the first active BWP, and MBS UE uses the BWP configured by case E as the first active BWP by default. So, there will be no impact on legacy UEs for case E. While for case C, due to the enlarged SIB1 configured inital DL BWP as the proponent of case E claimed, there will be additional power cost on legacy UEs. But for this, we have different understanding.

    - For case E, it measn two initial DL BWPs are being maintained in the system.

    - For case E, in this case, gNB doesn’t know who is MBS UE, who is legacy UE. There is no common understanding between gNB and UE. There will be too much impact. For example, if gNB mistake one legacy UE as MBS UE, and scheudle it in the frequency resource not overlapping with SIB1 configured initial DL BWP, obviously the performance of legecy UE will be deteriorated, i.e., case E brought negative impact to legacy UEs.

    - For case C, there is no discrepancy between gNB and UE. There is no legacy bahivor change for legacy UE.

  - Proposal 1: For CFR configuration for RRC_IDLE/RRC_INACTIVE UEs, Case E is not supported.

- In [R1-2111137, Nokia]

  - To our view, the support of Case C, Case D, and Case E can be achieved in the same manner with a common signaling design approach, and all three cases should be treated with the same design priority. The difference among the Case D, Case E, as well as previously agreed Case C, is just the matter of configured value of CFR size, and the Case C and Case D can be seen as a subset of Case E.

  - Proposal-1: Support of CFR Case D and Case E.

- In [R1-2111232, CATT]

  - Discuss: When low data rate is required, Case D can be supported for power saving. When high data rate is required, both Case C and E can be generally supported. However, Case C will affect legacy UE due to initial BWP with increased bandwidth. Case E can solve the issue. For Case E, the first active BWP should contain the CFR, so that the MBS UEs can receive broadcast and SIB/paging/unicast without BWP switching. All in all, Case D and Case E can be supported for gNB scheduling flexibility.

  - Proposal 1: Support Case D and E for gNB scheduling flexibility.

- In [R1-2111305, OPPO]

  - Discuss: In order to keep the MBS reception continuous, the bandwidth of CFR should be maintained for unicast and MBS service reception even UE has transferred from RRC_IDLE to RRC_CONN state. This larger bandwidth is considered as the applicable “initial DL BWP” rather than the initial DL BWP which is configured by SIB1. UEs may have different initial DL BWPs because some of the UEs may use initial DL BWP configured by SIB1 while some other UEs are using a larger bandwidth equal to CFR.

  - Proposal 1: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state, Case D is selected.

  - Proposal 2: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state, Case E is not supported.

- In [R1-2111408, SONY]

  - Proposal 1: Support Case E.

- In [R1-2111518, Intel]

  - Discuss: This method of coupling the CFR with the initial BWP also means that the related configurations can then be re-used for the CFR which does not need an independent reference point, starting PRB and length in PRBs to be additionally configured.

For Case D, it assumes that CFR is smaller than the SIB1 configured initial BWP but potentially larger than and containing the resources of CORESET#0. In our understanding, this case can be resolved using FDRA within Case C. Case D should not provide any added advantage in terms of UE power consumption since the UE will move the wider SIB1 configured initial BWP when it transitions to CONNECTED mode.

  - Proposal 1: Case D can be implemented under Case C using appropriate FDRA since the resources required for broadcast reception under Case D are already included in Case C. Additional support for Case D is not required.

  - Discuss: . To our understanding, a configured BWP can be anything which is different from the legacy SIB1 configured initial BWP or CORESET#0. However, we also need to ensure that that configured BWP contains CORESET#0. Assuming that CFR in Case E is different (wider) from the initial BWP, there is an issue that the UE will need to transition to the smaller initial BWP when entering CONNECTED mode and in this case the CFR lies outside of the initial active BWP. Therefore, in this implementation of Case E, for CONNECTED mode UEs to continue receiving broadcast transmission within the broadcast CFR, the gNB needs to switch the relevant UEs to wider active BWP as part of the initial access procedure.

  - Observation 1: For Case E when the configured BWP is wider than legacy SIB1 configured initial BWP, when the UE transitions to CONNECTED mode, the CFR will be outside the initial active BWP, requiring the switching of the UE to a wider BWP (containing CFR) for continuous broadcast reception.

  - Discuss: On the other hand, if we define the “configured BWP” as another SIB-x configured initial BWP only for MBS UEs which supersedes the legacy initial BWP configuration, then the issue of CFR outside of initial active BWP for UEs transitioning to CONNECTED mode does not exist. In fact, it is reasonable also to assume that the UE which required a wider CFR would also require a wider initial BWP to continue receiving broadcast and it does not have any additional power consumption issues. The main difference here is that the configured BWP is now a new wider initial BWP for the MBS UEs while the legacy UEs still use the legacy initial BWP.

  - Proposal 2: Case E can be supported where the “configured BWP” is defined as a SIB-x configured wider initial BWP for MBS capable UEs which supersedes the legacy SIB1 configured initial BWP.

- In [R1-2111551, Xiaomi]

  - Discuss: Case C would be a possible solution to resolve the capacity issues for MBS. However, gNB still needs to consider the multiplexing issues between MBS and legacy broadcast channels. Case D can provide more tools for network to handle MBS scheduling and consequently the system can benefit from flexibility harvested via case D. Furthermore, as case D only requires a CFR which fully contained by the initial DL BWP, gNB can use the CFR seamlessly when it enters RRC CONNECTED state. It is pretty aligned with the spirit of ‘the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception’.

  - Proposal 1: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state, support case D.

  - Discuss: One concern on case C is that the larger initial DL BWP increase the power consumption for legacy UE. We don’t think this argument is reasonable as nothing new is introduced for the legacy UE in terms of initial DL BWP. Power saving is never the factor needs to be taken into account when gNB configure initial DL BWP. For a legacy UE, it can be configured with a first active DL BWP other than initial DL BWP. The first active DL BWP can be much smaller than the initial DL BWP as it doesn’t need to receive MBS.

  - Discuss: If a larger MBS-specific BWP is configured for MBS UE, additional BWP switching is required when it transfers to RRC CONNECTED state as it is larger than the initial DL BWP. It would also complicate the scheduling at gNB side as it has to maintain two ‘initial DL BWPs’ if the intention is to make legacy UE and MBS UE associate with separate initial DL BWP, i.e. SIB1-configured initial DL BWP and MBS-specific DL BWP.

  - Proposal 2: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state, do not support case E.

  - Proposal 3: The SIB-1 configured initial BWP for legacy Rel-15/Rel-16 UEs in RRC_CONNECTED state is applied as initial BWP for Rel-17 MBS capable UEs.

- In [R1-2111629, CMCC]

  - Discuss: Case D: RRC_IDLE/INACTIVE UE first receives SIB-1 and then receives the CFR configuration (Case D) in SIBx. If this UE wants to receive the broadcast service, it is UE’s implementation whether to always keep RF bandwidth same as Case D CFR to receive broadcast service and receive legacy behaviour, e.g., paging in the bandwidth of CORESET#0 or switch between CORESET#0 and Case D CFR according to the search space monitoring occasion.
And when UE transits into RRC_CONNECTED state, the SIB-1 configured initial DL BWP is used as first active BWP regardless UE whether sends MBS interest indication or not. There is no BWP switching/service interruption between the RRC state transition because UE can always set the RF bandwidth equals to Case D CFR or timely switch to Case D CFR before the complement of RRC connection establishment or re-establishment or resume.

Discuss: Case E: When in RRC_IDLE/INACTIVE states, the UE behaviour is similar as Case D, which it is UE’s implementation whether to always keep RF bandwidth same as Case E CFR or switch between CORESET#0 and Case E CFR.
But when UE transits into RRC_CONNECTED mode, UE will either take the SIB-1 configured initial DL BWP as the first active BWP when an active BWP is not configured or the configured new active BWP as first active BWP. In the first method which SIB-1 configured initial DL BWP used as the first active BWP, UE cannot receive the broadcast service considering the SIB1-configured initial DL BWP is smaller than Case E CFR.
In the second method, gNB can configure an active BWP to cover the frequency resources of Case E CFR, but the critical issue is that how gNB knows which UEs receive the broadcast service since the MBS interest indication is an optional feature. In addition, the broadcast service is interrupted during BWP re-configuration. There are two potential ways proposed by companies to relieve this problem, which the first one is all MBS-capable UE taking Case E MBS BWP as the first active BWP (there is additional RAN2 spec impact to define new first active BWP) or UE will not receive broadcast service if not report MBS interest indication (cannot guarantee all UEs’ broadcast reception).
Compared with Case D, Case E has these natural drawbacks and may introduce more spec impact. In addition, as the previous agreement, Case E is an MBS specific BWP, whether can or how to support two BWPs simultaneously by UE have never been widely discussed. Considering these, we don’t support Case E but Case D as the CFR for MCCH/MTCH.

  - Proposal 1. For RRC_IDLE/RRC_INACTIVE UEs, Case D can be supported as configured/defined specific CFR for MTCH/MCCH

- In [R1-2111763, Samsung]

  - Discuss: However, considering that RAN1#107-e is the last meeting for Rel-17, in order to complete this WI, supporting only Case D could be done because Case E needs many details related to BWP such as BWP switching and restrictions.

  - Proposal 1: Support Case D.

- In [R1-2111899, Apple]

  - Proposal 2: For MBS UE in RRC_IDLE/RRC_INACTIVE mode, the Case E is supported for broadcast reception.

- In [R1-2112065, LGE]

  - Proposal 2: PDCCH/PDSCH for MTCH transmission can be transmitted on a CFR defined based on a configured BW or a CFR with the same size as the initial BWP, depending on MCCH information.

- In [R1-2112130, NTT DOCOMO]

  - Discuss: The problem is that a UE receiving broadcast in RRC_IDLE/RRC_INACTIVE state may not be able to continue receiving the broadcast services after transitioning to RRC_CONNECTED state since the initial BWP applied after receiving Msg4 cannot include the CFR for broadcast in Case E. However, if gNB indicates a BWP that contains the CFR for broadcast as the first active downlink BWP, the UE will be able to continue receiving broadcast services without interruption. Also, broadcast services do not require high QoS, so interrupted reception may not be a problem. Both cases should be supported in the specification, and it should be up to network to choose which case to use.

  - Proposal 1: For a CFR for GC-PDCCH/PDSCH for broadcast, support both Case D and E.

- In [R1-2112163, Lenovo]

  - Observation 1: The motivation to support Case E is not justified.

  - Observation 2: Those UEs with small bandwidth capabilities can’t be supported in Case E.

  - Observation 3: Frequent BWP switching happens in Case E.

  - Discuss: RAN2 has already agreed that transmitting MBS interest indication to gNB for Idle/Inactive mode UE is not supported. Furthermore, the Idle/Inactive mode UE can’t transmit MBS interest indication to gNB due to lack of TA. Without such indication, gNB can’t know which Idle/Inactive mode UE is interested in the MBS with larger CFR and will not configure the first active BWP same as the MBS-specific BWP in Case E to the interested Idle/Inactive mode UE.

  - Observation 4: Idle/Inactive mode UE can’t send MBS interest indication to gNB.

  - Discuss: To support MBS-specific BWP with large CFR in Case E, standards should support Idle/Inactive mode UE to transmit MBS interest indication to gNB and support configuring first active BWP as MBS-specific BWP via SIBx or MCCH for Idle/Inactive mode UE.

In addition, how to configure the CFR with larger size than SIB-1 configured initial DL BWP is unknown and whether different parameters for CFR in Case E are configured for Case A or Case C has not been discussed.

Regarding group-common DCI format design for support of Case E, since CFR is larger than CORESET 0/SIB-1 configured initial DL BWP, solution is needed to determine the FDRA field size in case of Case E so as to align the DCI payload size of the group-common DCI with size of DCI format 1-0 with CRC scrambled by C-RNTI in CSS.

  - Observation 5: Significant standard impact is caused in Case E.

  - Observation 6: Case E is an optimization on top of Case C.

  - Proposal 1: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, for CFR configuration for group-common PDCCH/PDSCH, Case E is not supported.

- In [R1-2112241, Qualcomm]

  - Discuss: For RRC_IDLE/INACTIVE UEs, the CORESET#0 is still the “initial BWP to receive SIB/paging” but the broadcast CFR with bandwidth size of “SIB-indicated initial BWP” larger than CORESET#0 can be regarded as a new BWP, differentiated from the “initial BWP to receive SIB/paging”.

  - Discuss: If network does not configure SIB1-configured initial BWP, we should allow the network to configure a broadcast CFR/BWP larger than CORESET#0 for MBS UEs and it would be pointless to down-select Case C, D or Case E. The unified solution is preferred no matter whether it is Case E or Case C, and no matter whether there is SIB1-configured initial BWP or not. If Case C or Case E is configured, Case D can be implemented by using CORESET for GC-PDCCH and FDRA for GC-PDSCH by implementation. We don’t see the point to support the option of Case D only.

  - Proposal 1: Support Case E for a CFR-Config-Broadcast.

- In [R1-2112314, MediaTek]

  - Discuss: If the bandwidth of initial BWP is changed due to introducing the MBS services, it also will affect the legacy UEs’s capability. Therefore, we suggest to discuss the CFR configuration independently.

  - Proposal 3: CFR can be configured with any size as long as it covers CORESET#0.

- In [R1-2112348, Ericsson]

  - Observation 1: In NR, all data channels and reference signals used for transmission/reception between the UE and network occur in a DL/UL Bandwidth Part (BWP).

  - Observation 2: For Case A, this BWP is the CORESERT#0 initial BWP, which is configured for all UEs in RRC IDLE/INACTIVE and does therefore not need to be specifically configured for broadcast.

  - Observation 3: For Case C, D and E, since the CFR exceeds the CORESET#0 frequency resources, a specific BWP for broadcast needs to be configured.

  - Proposal 1: For UEs receiving broadcast in RRC IDLE/INACTIVE, the CFR is configured within a BWP.

    - Note1: For Case A this BWP is the CORESET#0 initial BWP (already agreed)

    - Note: Specific naming and configuration of the BWP is up to RAN2.

  - Proposal 2: For UEs receiving broadcast in RRC IDLE/INACTIVE according to other Cases than Case A, a specific BWP for broadcast, different from CORESET#0 initial BWP, is configured: “BWP-B”.

  - Proposal 3: The legacy CORESET#0 initial BWP is used to receive System Information and paging and for Random Access also for UEs receiving broadcast in RRC IDLE/INACTIVE.

  - Observation 4: UEs is RRC IDLE/INACTIVE receiving broadcast will need to receive in parallel legacy type of data, such as System Information, paging and RA signaling in the CORESET#0 initial BWP and broadcast data. For Case C, D and E, the broadcast is received in a specific broadcast BWP, wider than CORESET#0 initial BWP, but containing this.

  - Observation 5: With Case C, the configured broadcast BWP-B can naturally have identical frequency resources to the SIB1 initial BWP.

  - Observation 6: With Case D, the configured broadcast BWP-B can naturally have identical frequency resources to the configured CFR.

  - Observation 7: With Case E, the configured broadcast BWP-B can naturally have identical frequency resources to the configured CFR.

  - Observation 7: For all cases C, D and E, the configured broadcast BWP-B can naturally have identical frequency resources to the configured CFR

  - Proposal 4: For all cases, other than Case A, the configured broadcast CFR and BWP-B have identical frequency resources.

  - Discuss: With Case E as a starting point, it supports any CFR/BWP-B size within the carrier, independently of SIB1 initial BWP, Case D adds a constraint in that the CFR/BWP-B is required to be contained within the SIB1 initial BWP frequency resources. Case C adds a further constraint in that the CFR/BWP-B needs to have identical frequency resources to SIB1 initial BWP.

It is not clear what gains there could be of introducing such Case C/D constraints, when the solution could just as well be fully flexible with Case E.

  - Observation 9: When SIB1 does not configure the initial BWP, Case C and D are not applicable. Broadcast would then be limited to Case A, unless Case E is supported.

  - Proposal 5: Broadcast in a wider CFR/BWP than CORESET#0 initial BWP is supported when SIB1 does not configure the initial BWP.

  - Specification complexity: Case C allows for a simplification of the configuration of frequency resources for the CFR/BWP-B, in that the SIB1 initial BWP configuration can be reused for both CFR and BWP-B.

For both Case D and E there is a need to separately configure a CFR/BWP-B, which is different from the SIB1 initial BWP. This will require the same type of additional configuration in both cases, which is likely to be a locationAndBandwidth parameter in SIBx that will define the frequency resources of the CFR/BWP.

  - Observation 10: There is no significant difference in specification complexity between Case D and Case E.

  - Observation: There is no significant difference in UE complexity between Case D and Case E.

  - Discuss: In all cases C, D and E, without further information about whether the UE receives broadcast, the gNB would need to decide depending on what it finds most important, but there is no difference between the cases regarding the impact of this.

  - Observation 11: For UEs supporting broadcast, without additional RRC signaling support, the network cannot know whether a UE entering RRC Connected state is receiving broadcast or not, so cannot make an informed choice on which active BWP is optimum to use. It can either select an optimum active BWP, without broadcast considerations, or it can let the UE keep its current BW to allow for seamless transition of the broadcast service. This issue is however identical for all Cases C, D and E.

  - Observation 12: There are no significant differences in operational issues between Case D and Case E.

  - Proposal 6: For UEs in RRC INACTIVE/IDLE, broadcast can be received according to Case E.

    - Note: CFRs according to Case C and D can be supported by Case E.The BWP (”BWP-B”) to receive the broadcast CFR in RRC IDLE/INACTIVE has the same frequency resources as the CFR.

  - Proposal 7: When the UE state is changed from RRC INACTIVE/IDLE to RRC CONNECTED, the UE formally keeps its BWP-B used to receive Case E broadcast in RRC INACTIVE/IDLE until completion of RRC configuration. After RRC configuration, the UE releases the BWP-B but keeps the CFR, which is inherited by all configured BWPs in RRC CONNECTED, provided the CFR is contained within the respective BWP.

  - Discuss: Note: Only using a CFR (i.e. without a BWP) in RRC INACTIVE/IDLE is not possible since a BWP always needs to be used to receive data, for consistency with legacy NR. Only using a BWP is not possible either since the BWP needs to be released after completion of RRC configuration and UEs in all RRC states need to have something in common to receive the broadcast, which is the CFR. The CFR thus stays the same when a UE moves from RRC INACTIVE/IDLE to RRC CONNECTED, but the BWP changes.

  - The BWP_ID numbering used by UEs in RRC INACTIVE/IDLE is independent from the numbering used by RRC CONNECTED UEs. For UEs in RRC INACTIVE/IDLE, CORSESET#0 initial BWP is used by all UEs and BWP-B is used for UEs receiving broadcast with Case C, D or E. For UEs in RRC CONNECTED, the legacy numbering is unaffected.

  - Proposal 8: When the frequency range and numerology of the BWP to receive broadcast does not change with a change of RRC state, the UE is expected to receive the broadcast data without disruption.

### FL Assessment

On configuration of a BWP for the CFR (including agreed Case A/C)

In parallel to the discussion on down-selection of Case D/E, it is proposed to focus on critical aspects that may be open even for the already agreed Cases of CFR, i.e., Case A and Case C. The discussion should also consider the other two open cases if possible.

As per the background section, Cases A, C and D are defined as a CFR. However, whether the relationship of these cases within a BWP has not been concluded yet.

The contributions in [Ericsson, Qualcomm] detail, for Cases other than D/E, the relationship of the CFR and a BWP and proposes that the CFR is configured as a BWP. [Intel] also discusses potential implementation of Case E as an BWP, however, in this case as a specific initial BWP only MBS UEs. [Xiaomi] similarly proposes that for Case C, MBS UEs apply as initial BWP the one configured in the SIB-1 configured initial BWP. In previous meetings, some companies also proposed to leave this decision to RAN2. However, this was directly discussed at the last meeting and multiple companies expressed that it should be RAN1 that decides at least basic functionality such as whether it is based on BWP or not.

The common point in all the contributions above in this aspect is that the CFR is configured within a BWP since this is the basic operational mode to receive signals in NR. Therefore, Proposal 2.6-1 for discussion tries to agree this for all cases other than Case A. The proposal further details that the frequency resources of the CFR and the BWP are identical. The formulation is general to potentially accommodate Case D/E after down-selection. However, this agreement should also cover Case C that has already been agreed.

Down selection of Case D/E

This issue was debated at length at the past meetings without reaching a conclusion.

Multiple companies have contributed to this aspect of the discussion. Although some contributions have provided more detailed analysis based on the technical discussion from the last meeting, most companies provide similar arguments to support/not support the different Cases. Compared to the last meeting, most companies have not changed their views on the support Case D/E.

Please see FL summary#5 [R1-2110595] from RAN1#106bis-e for detailed discussions on this issue on down-selection.

- Support of Cases D/E

- Support of Case D

  - [Futurewei, Spreadtrum, Xiaomi, CMCC, Samsung] (5)

- Support of Case E

  - [TD Tech, vivo, SONY, Intel*, Apple, LGE, Qualcomm, Ericsson] (8)

    - Intel proposes Case E implemented as a new MBS initial BWP.

- Support of Case D/E

  - [ZTE, Nokia, CATT, NTT DOCOMO, MediaTek,] (5)

- Not support of Case E

  - [Spreadtrum, OPPO, Xiaomi, CMCC, Lenovo] (5)

- Technical issues

Based on the technical discussion on potential interruption due to UEs frequency range change and service continuity from previous meetings and tdocs to this meeting the following observations are done:

- potential interruption situations where were identified for Case C/D/E when the UE changes the frequency range from RRC idle/inactive to RRC connected. Some examples below:

  - for Case C this can happen for example when active BWP in RRC connected has a frequency resource larger than the frequency resources of Case C.

  - For Case D this can happen for example when UE has to change to the frequency resources to the frequency resources of the SIB-1 configured initial BWP (which are larger)

  - For Case E this can happen for example when active BWP in RRC connected has a frequency resource larger than the frequency resources of Case E

  - Note: it was also recognised that the potential interruption in all cases may be acceptable for broadcast reception.

- regarding service continuity when UE changes from RRC idle/inactive to RRC connected: situations were identified where service continuity cannot be guaranteed for cases C/D/E.

  - Since the gNB does not have any knowledge whether UEs are receiving the broadcast service or not, the gNB could configure an active BWP in RRC connected with frequency resources smaller than those of Case C/D/E, causing service loss.

    - For case C/D, in the case that gNB uses default active BWP (i.e., SIB-1 conf initial BWP) service continuity would be maintained but if the gNB configures an active BWP with frequency resources smaller than those of Case C/D service loss would occur.

  - To solve this potential service loss for all Cases, UE interest notification could be sent from UEs to gNB, however, this is a functionality that is not mandated in the current specifications.

- Motivation

Regarding motivation for Case E, there were multiple discussions.

Companies supporting Case E argue that using only Case D (and Case C) has an impact on legacy non-MBS UEs since configuring Case D and Case C both rely on changing the configuration of the SIB-1 configured initial BWP. These companies consider not having an impact on legacy UEs while being able to schedule broadcast services a basic function.

Companies that do not want Case E argue that they do not see the argument above as a limitation. Legacy UEs can use the SIB-1 configured initial BWP, therefore, nothing is broken. Case C already provides sufficient flexibility and therefore Case E is an optimisation, not a basic function and therefore should not be discussed/included.

After multiple exchanges of discussion, not consensus was reached.

- Specification Impact

One aspect that has been discussed in less detail is the potential specification impact of standardising case D and Case E and whether there is more/less/same specification impact for both cases. Contribution in [Ericsson] argues that specification impact of case D/E is similar, while [Lenovo] discusses aspects presenting significant more specification impact for Case E.

The proposal from the FL is that we also discuss potential specification impact of both case D/E. If there is a common understanding that the specification impact to introduce Case D and Case E are the same/similar both cases could be considered to be agreed. However, if there is a Case that has significant more impact than the other this would help the group reach a decision to down-select one Case over the other. Question 2.6-2 will be used to collect companies’ views on this.

### 1st round FL proposals for Issue 6

#### Proposal 2.6-1

For UEs receiving broadcast in RRC IDLE/INACTIVE, the CFR is configured within a BWP.

- for Case A (already agreed) this BWP is the CORESET#0 initial BWP

- for other Case(s) than Case A, a specific BWP for broadcast, different from CORESET#0 initial BWP, is configured

- the CFR and the specific BWP have identical frequency resources

- Specific naming and configuration of the specific BWP is up to RAN2.

#### Question 2.6-2

Regarding potential specification impact of Case D and E, please provide your views on whether the specification impact of Case D is the same/larger/smaller than the specification impact of case E.

Please provide your answers in the table below. Considering the FL assessment above:

- Do you support Proposal 2.6-1? If you don’t, please provide additional proposals or modifications to work towards a compromise.

- Please provide your views on Question 2.6-2.

- Please provide your views, if any, on the FL assessment above.

FL note: based on the discussion from these questions further proposals will be included for discussion and agreement.

| company | comments |
| --- | --- |
| NOKIA/NSB | #### Proposal 2.6-1: To our view, there is no need to have such restriction/limitation. All Case C/D/E can be applied with the same manner. Targeting and support only on Case C is no preferred. For UEs receiving broadcast in RRC IDLE/INACTIVE, the CFR is configured within a BWP. - for Case A (already agreed) this BWP is the CORESET#0 initial BWP - for other Case(s) than Case A, a specific BWP for broadcast, different from CORESET#0 initial BWP, is configured - the CFR and the specific BWP have identical frequency resources - Specific naming and configuration of the specific BWP is up to RAN2. #### Question 2.6-2: To our view, the specification impact of Case D and Case E are the same: - As stated in Proposal 2.6-1 by FL, the Case D and Case E together with Case C are commonly considered as new BWPs for idle/inactive UEs in addition to CORESET#0 initial BWP. - As stated in Proposal 2.4-2 by FL, the BWP/CFR configuration of Case D and E as well as Case C are commonly the same, by simply re-use the legacy definition of BWP frequency resources for unicast, i.e. with the parameter combination of Point A, offsetToCarrier and locationAndBandwidth to indicate the exact location of the BWP/CFR with respect to the carrier starting RB. - During the RRC-transition period, there may have the BWP switching impact as intensively discussed at last meeting, and the transmission “dis-continuity” issue is commonly exist for Case D and Case E, as well as to some cases of Case C. - Moreover, it is commonly that Case D and Case E have different bandwidth configuration than SIB1-configured initial BWP, with either narrower or larger than the bandwidth of SIB1-configured initial BWP. From network point of view, different broadcast services can be associated with different CFRs, i.e. one broadcast service G-RNTI-1 is with Case A CFR (CORESET#0), and other broadcast services G-RNTI-2/G-RNTI-3 is with Case D or Case E CFR. And different idle/inactive UEs may receive their own interested broadcast services respectively based on the configured CFR associated with the broadcast services. Meaning that from network point of view, there can be multiple CFR configured for different broadcast services, and from UE point of view, there can be either single CFR applied or multiple CFR utilized, depends on UE capability. And during the RRC-transition period, the idle/inactive UEs with different interests of broadcast services may need to indicate their MBS interests for assisting the gNB to make the proper BWP configuration, when entering the RRC connected mode. And in such case, it is the same for Case D and Case E, since it is anyway different from the bandwidth of SIB1-configured initial BWP |
| NTT DOCOMO | Proposal 2.6-1: Support #### Question 2.6-2: We think Case C, D and E can be achieved with a unified configuration framework. So we don’t think there is any difference in specification impact between Case D and Case E. |
| Lenovo, Motorola Mobility | #### 2.6.1: we agree with the first sub-bullet and have concern on the remaining three sub-bullets. For Case C, as defined, the BWP is SIB-1 configured BWP so the CFR has same frequency resource as it and same numerology. In that sense, we don’t need to define the CFR again. 2.6.2: if CFR is configured within the SIB-1 configured DL BWP, then Case D doesn’t bring additional complexity than Case C. However, for Case E, the motivation, use case, data rate requirements, BWP switching, first active BWP configuration, interest indication and RAN2 work need quite a lot discussion. Furthermore, only basic function for broadcast is needed in Rel-17 as stated in WID. We don’t want to repeat such discussion in the last Rel-17 meeting. But we are OK to enhance idle/inactive broadcast in Rel-18. |
| Xiaomi | a) we only support the main bullet. Even for the main bullet, it is obvious. b) Same views as Lenovo. We don’t support case E. Case E has more standard impacts as it introduces a larger BWP compared to what we have for now. We don’t see holes without case E. We should follow the objective captured in Rel-17 MBS WID, i.e. only focus on the basic functionality for broadcast for RRC_IDLE/INACTIVE UEs. |
| OPPO | - The main bullet is introducing a new terminology “BWP for RRC_IDLE/INACTIVE” as the container of CFR, if our understanding is correct. If the intention of the main bullet is to keep it aligned with that of CFR in multicast, it seems OK. But for the sub-bullets, we are not sure if they are necessary, and need further discussion. - With technical concerns on case E, we also think it has spec impact. With current agreement of case A and case C, the basic function of MBS can work normally. |
| Samsung | Proposal 2.6-1) Agree Q 2.6-2) Case D has less impact due to introduction of a new BWP. |
| ZTE | #### Proposal 2.6-1: We support the proposal with the understanding that both Case C, Case D and Case E are supported in a unified manner with this proposal. #### Question 2.6-2: After lengthy discussion in previous several meetings, we draw the following observations. 1) The potential interruption time may happen for all the three cases, i.e., Case C, Case D and Case E; 2) The MBS interest indication, if needed, is required for all the three cases, i.e., Case C, Case D and Case E; 3) Case C, Case D and Case E can be implemented via the same framework with even the same configuration; 4) Without supporting Case E would be too restrictive especially if network configures a small initial BWP in the legacy unicast network; Another point we want to make is, eventually the UE has to support two CFRs in the initial DL BWP, one for the broadcast and another one for multicast. Let’s say CFR for broadcast is 40MHz and CFR for multicast can be 100MHz. However if UE supports Case E for broadcast, probably network can configure the same CFR for both broadcast and multicast. |
| Spreadtrum | Proposal 2.6-1: The proposal is not needed. The definition of case A, case C, case D and case E have been clearly captured in the previous agreement. We should not waste time to re-discuss it again, especially in last RAN1 meeting. #### Q2.6-2: Share the same view with Samsung. |
| vivo | Question 2.6-2: we think specification impact of Case D is the same as the specification impact of case E. This is because the CFR in both cases is larger than CORESET0 and can’t be covered by valid initial BWP configured for UEs in RRC idle/inactive, frequency range of the CFR should be newly configured via SIB or MCCH. |
| MediaTek | Proposal 2.6-1: Not support. We should focus on whether the case A and case C is sufficient for broadcast reception. Regarding how to configure the CFR for multicast, we don’t support to define a specific MBS BWP for broadcast because the CFR need a container to configure. If it introduces a another BWP concept for MBS broadcast reception, it will exist two active BWP, and against the legacy rule that just only one activate BWP is supported for legacy UEs, it needs more RAN4’s work. In addition, it needs against the WID that “In order to facilitate implementation and deployment of te feature, the overall implementation impact should be limited, and the UE complexity should be minimized (e.g. device hardware impact should be avoided).” Regarding how to configure the CFR for broadcast reception, the Proposal 2.4-3 has gave the specific method that “The CFR frequency resources used for MCCH and MTCH are configured by SIBx”, and the IE configuration structure is RAN2’s work scope. To sum up, we suggest to defer the discussion and the proposal is not necessary. #### Proposal 2.4-3 For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs: - The CFR frequency resources used for MCCH and MTCH are configured by SIBx;. |
| CMCC | Proposal 2.6-1: Actually we don’t see this proposal can make progress on this issue, especially considering there are so many details need to be discussed, e.g., for Case C does gNB needs to configure another BWP with the same frequency resource as SIB-1 configured initial DL BWP or just reuse the current initial DL BWP configuration in SIB-1 and only needs to configure a CFR? Q2.6.2: Similar view as Lenovo/Xiaomi/OPPO/Samsung/Spreadtrum, Case D has less spec impact. |
| Ericsson | #### P2.6.1: Support P2.6.2: We think the specification impact of Case D and E is the same. Assuming the legacy SIB1 is reused, without modification, and a new SIBx is used for additional broadcast configurations, Case D and E both require the configuration of the CFR/BWP (assumed to be the same) in SIBx, to be used for broadcast. This configuration would use the same locationAndBandwidth mechanism for this. With Case D, the locationAndBandwidth would need to be constrained to be within the same frequency range as the initial BWP configured by SIB1, which is not necessary for Case E, which means that the specification impact is, if anything, larger for Case D than for Case E, since Case D adds an additional rule that is not needed in Case C. However, in both cases the UE would simply use the parameter provided in SIBx and would not need to involve SIB1 in the determination of the frequency resources to be used for the Case D/E CFR/BWP. In both cases, UEs that have just entered RRC Connected would receive RRC signaling on the SIB1 initial BWP. It would be a UE implementation functionality of broadcast UEs to be able to support parallel reception in that case, so no specification impact is required. @Lenovo: The Case D CFR cannot be configured as a part of the SIB1 configured BWP, since the latter only exists in RRC Connected. Instead, a logically separate BWP is required to receive broadcast in RRC IDLE/INACTIVE. The frequency resources of this are configured by SIBx. As a compromise, we are also fine with the alternative solution, as proposed by Intel, where broadcast UEs would use a separate SIBx-configured initial BWP, which would always have the same frequency resources as the CFR/BWP for broadcast transmission. With that solution, there is no difference at all between Case C, D and E – they could all be seen as a modified Case C, “Case Cx”, with SIBx initial BWP to be used by broadcast UEs instead of the legacy SIB1 initial BWP. Case C, D and E, as currently defined could then be supported without any difference in any way, since they would all be part of the same “Case Cx” solution. |
| Apple | #### Proposal 2.6-1: ok Question 2.6-2: if the proposal 2.6-1 is agreed, then the specification impact of Case D and Case E are the same. the exact size of CFR or size of the specific BWP is just configuration issue. |
| Qualcomm | #### P2.6.1: Support #### P2.6.2: Any CFR larger than CORESET#0 will require the configuration of the CFR/BWP in SIBx for broadcast reception of IDLE/INACTIVE UEs. So, the specification impact of Case C, D and E is the same. |
| Intel | #### Proposal 2.6-1: We support the general direction of this proposal i.e., that a common configuration framework is used to address Case C, D and E. However, as we have proposed before, we think the SIB-x configured BWP should be the new initial BWP of the MBS capable UEs. This SIB-x configured initial BWP should over-ride the SIB-1 configured initial BWP for MBS UEs which can decode this SIB-x. This way, the CFR is always within the initial BWP of the MBS UEs when transitioning to RRC_CONNECTED mode and legacy UEs are not impacted i.e., they continue using SIB-1 configured initial BWP or CORESET#0. In Case C, D, E, the only constraint required is for the CFR to contain the CORESET#0. We do not think we need to push this further to RAN2 since this is a RAN1 issue. We propose the following update to the Proposal: #### Proposal 2.6-1 For UEs receiving broadcast in RRC IDLE/INACTIVE, the CFR is configured within a has frequency resources identical to a new initial BWP (different from CORESET#0) which is configured by SIB-x - For MBS UEs which can decode the SIB-x, the configured initial BWP replaces the SIB-1 configured initial BWP - Note: For Case A (already agreed) this initial BWP is not configured, and the frequency resources of the CFR are identical to CORESET#0 - for other Case(s) than Case A, a specific BWP for broadcast, different from CORESET#0 initial BWP, is configured - the CFR and the specific BWP have identical frequency resources - Specific naming and configuration of the specific BWP is up to RAN2. With the above proposal, we do not need to differentiate Case C, D and E any more. #### Question 2.6-2: We do not think this is a relevant question to ask at this stage. Specification impact is secondary to making the feature work. The above proposal achieves that and should be considered as a compromise proposal that supports all Cases A-E. |

## [UPDATE] Issue 7: PDSCH repetition/HARQ combining

### Background

The following agreements at RAN1#102-e, RAN1#103-e, RAN1#104-e and RAN1#106bis-e for UEs in RRC_CONNECTED and RRC_IDLE/INACTIVE states are relevant for the discussion:

Agreements: For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH. - FFS: whether enhancement is needed Agreements: For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among: - Opt 1: by DCI - Opt 2: by RRC - Opt 3: by RRC+DCI - FFS: Opt 4: by MAC-CE - FFS: Opt 5: by RRC+MAC-CE - FFS details for each option. - FFS further enhancements for configuration of slot-level repetition Agreement: For slot-level repetition for group-common PDSCH for RRC_CONNECTED UEs receiving multicast, - (Config A) UE can be optionally configured with pdsch-AggregationFactor. - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table. - If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH. Conclusion: The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UE to support multicast reception. - How to allocate HARQ processes between unicast and multicast is up to gNB. Agreement: - If configured, the pdsch-AggregationFactor for multicast dynamic scheduling is configured per G-RNTI. - If configured, the pdsch-AggregationFactor for multicast SPS is configured per SPS-Config-Multicast. Agreement: For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs. - Config A or Config B can be configured to UE: - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast. - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast. If UE is configured with Config B, UE does not expect to be configured with Config A for the same SPS group-common PDSCH. - For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is - Alt1: equal to 1. Agreement: For broadcast reception with UEs in RRC_IDLE/INACTIVE states, support slot-level repetition for MTCH.

### Tdoc analysis

- In [R1-2110779, R1-2111917, Huawei]

  - Discuss: It was agreed in the last meeting to support slot-level repetition for MTCH for broadcast reception with UEs in RRC_IDLE/INACTIVE states. However, based on the discussion, it is unclear such configuration should be semi-static as aggregation as supported in Rel-15 or it is part of TDRA set configuration and can be dynamically changed by TDRA indication in DCI as supported in Rel-16. It is an open issue that can be discussed. Considering it is for broadcast scheduling and DCI format 1_0 with G-RNTI is the only format agreed to be supported, configured as slot aggregation for broadcast seems sufficient.

  - Proposal 10: Slot-level repetition is configured per G-RNTI as slot aggregation for broadcast.

  - Proposal 4: repetitionNumber-Broadcast is configured per G-RNTI and included in pdsch-Config-Broadcast for broadcast.

- In [R1-2110897, TD Tech]

  - Proposal 3: Support slot level repetition for MCCH

- In [R1-2110912, ZTE]

  - Discuss: The same mechanism as slot aggregation can be reused for broadcast repetition. The only remaining issue is how to configure the repetition number. The repetition number can be configured per G-RNTI via MCCH.

  - Proposal 4: For broadcast reception with UEs in RRC_IDLE/INACTIVE states, the repetition number of slot-level repetition for MTCH is configured per G-RNTI via MCCH.

- In [R1-2111137, Nokia]

  - Discuss: Proposal-7: For broadcast reception with UEs in RRC_IDLE/INACTIVE states, support slot-level repetition for GC-PDCCH/PDSCH carrying MCCH/MTCH.

  - Proposal-8: Further discussion on whether both Config A and Config B could be supported for broadcast reception, and whether it can be applied for both dynamic and semi-persistent scheduling.

  - Proposal-9: It is proposed that “when receiving group-common PDSCH scheduled by DCI format 1_0 in group-common PDCCH with CRC scrambled by G-RNTI or G-CS-CRNTI for broadcast reception, with NDI=1, if the UE is configured with pdsch-AggregationFactor in pdsch-config, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots”.

- In [R1-2112065, LGE]

  - Proposal 6: For slot-level repetition for group-common PDSCH for RRC_IDLE/INACTIVE UEs receiving broadcast,

    - (Config A) UE can be optionally configured with pdsch-AggregationFactor.

    - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table.

    - If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH.

- In [R1-2112163, Lenovo]

  - Discuss: Regarding slot level repetition, there are two types specified in standard in Rel-15 and Rel-16: Type A and Type B. Since both types have been supported for RRC_connected UEs, it is straightforward to extend both to RRC IDLE/RRC INACTIVE UEs. To support Type B, RRC configured TDRA table with number of repetitions in one or multiple entries should be supported.

  - Proposal 16: For RRC_IDLE/RRC_INACTIVE UEs, PDSCH repetition Type B is supported for MCCH and MTCH.

- In [R1-2112241, Qualcomm]

  - Proposal 5: For RRC_IDLE/INACTIVE UEs,

    - Support slot-level repetition for MCCH, using

      - (Config A) UE can be configured with pdsch-AggregationFactor, applied to DCI format 1_0 with MCCH-RNTI.

    - For slot-level repetition for MTCH, support

      - (Config A) UE can be configured with pdsch-AggregationFactor per G-RNTI, applied to DCI format 1_0 with the G-RNTI.

      - (Config B) UE can be configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Broadcast

      - If UE is configured with Config B, UE does not expect to be configured with Config A for the same GC-PDSCH.

- In [R1-2112348, Ericsson]

  - Discuss: Obviously, with broadcast the UE would not send any feedback to trigger HARQ retransmissions, so these would need to be scheduled by the network without such feedback. We may call this gNB-triggered HARQ retransmission to contrast with legacy HARQ retransmission, where the UE triggers the retransmission via NACK feedback.

  - Discuss: The main purpose of gNB-triggered HARQ retransmission would be to provide increased time diversity, similar to that provided by time interleaving in some legacy broadcast systems.

  - Discuss: The time diversity offered by slot-level repetition is very limited. With a maximum of 16 slots in a “repetition burst” the total duration would only be 16 ms with SCS 15 kHz and half of this with SCS 30 kHz. With a more realistic repetition over e.g. four slots the duration would be only 4 ms (15 kHz SCS) or 2 ms (30 kHz SCS), which is too small values to provide any significant time diversity gain in most scenarios.

  - Observation 16: With gNB-triggered HARQ retransmission for broadcast, the time diversity may be very significantly extended, and be significant also for low speeds such as walking speed.

  - Discuss: To increase time diversity, one could alternatively use HARQ retransmission, where the total time duration of a Transport Block (TB), considering all (gNB-triggered) HARQ retransmission may be much longer, which could allow for better time diversity also with low overhead. If the repetitions are e.g. spread over 100 ms, which may be feasible with broadcast applications that are not very sensitive to latency, significant time diversity gain could be gained also at walking speed.

It appears thus that both the required DCI signaling fields and the UE soft-combining capability will anyway be available for broadcast, so supporting also HARQ combining, based on gNB-triggered HARQ retransmissions, would not require any significant additional complexity, neither specification-wise, nor UE complexity-wise. Since such functionality could also provide significant gains in certain scenarios, we think this functionality should be supported in Rel-17, which can be done with almost no additional specification work.

  - Proposal 14: Support gNB-triggered (not feedback based) HARQ retransmissions for broadcast

    - Note: UE behavior is the same as with UEs receiving multicast, but with no feedback from the UE. The UE would soft-combine successive HARQ transmissions of the same G-RNTI and HARQ process. The total number of transmissions is pre-determined by the gNB.

### FL Assessment

While for the support of slot-level repetition for MTCH, [Huawei, ZTE, Lenovo] propose to use Config B, [Nokia, LGE, Qualcomm] propose/discuss to use both Config A and Config B. [TD Tec, Qualcomm, Nokia] also propose to support slot-level repetition for MCCH, but for this case only semi-static configuration (Config A) would be supported.

Based on this input, Proposal 2.7-1 is to agree for the support for slot-level repetition for MCCH and Config A, and Proposal 2.7-2 extends the agreements on RRC connected UEs to RRC idle/inactive UEs for broadcast reception.

In [Ericsson] it is also proposed to support for broadcast reception with idle/inactive UEs the reception of the HARQ retransmissions (initiated by the gNB only and not by direct request from idle/inactive UEs using UL feedback) to significantly increase the time interleaving depth (to hundreds of ms) compared to the time interleaving depth of slot level repetition (of only a few ms). It is discussed that to support such an approach, very limited specification impact should be expected since the required fields in DCI are already included as part of the support of slot-level repetition.

Question 2.7-3 collects companies’ views on this proposal that could be further extended for agreement based on company comments.

### 1st round FL proposals for Issue 7

#### Proposal 2.7-1

Proposal 5: For RRC_IDLE/INACTIVE UEs, support slot-level repetition for MCCH, using:

- (Config A) UE can be configured with pdsch-AggregationFactor, applied to DCI format 1_0 with MCCH-RNTI.

#### Proposal 2.7-2

For RRC_IDLE/INACTIVE UEs, for slot-level repetition for MTCH, support:

- (Config A) UE can be configured with pdsch-AggregationFactor per G-RNTI, applied to DCI format 1_0 with the G-RNTI.

- (Config B) UE can be configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Broadcast

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same GC-PDSCH.

#### Question 2.7-3

Provide your views on the support of gNB-triggered (not feedback based) HARQ retransmissions for broadcast

- Note: UE behavior is the same as with UEs receiving multicast, but with no feedback from the UE. The UE would soft-combine successive HARQ transmissions of the same G-RNTI and HARQ process. The total number of transmissions is pre-determined by the gNB.

Please provide your answers in the table below. Considering the FL assessment above:

- do you agree with the proposals 2.7-1 and 2.7-2? Please provide reasons and views in general.

- Please provide your views on Question 2.7-3.

| company | comments |
| --- | --- |
| LG Electronics | #### Proposal 2.7-1: This proposal seems not needed. MCCH is periodically transmitted anyway like system information. Thus, slot-level repetition for MCCH seems not needed. #### Proposal 2.7-2: We support this proposal. #### Question 2.7-3: We support blind HARQ retransmissions for broadcast. We think that blind HARQ retransmissions can be same as HARQ retransmissions with disabling HARQ-ACK for multicast. The total number of transmissions can be pre-determined by the gNB. |
| NOKIA/NSB | #### Proposal 2.7-1: It seems Not needed In previous RAN1 agreement at last meeting, it said support slot-level repetition only for MTCH, and not mentioning for MCCH at all. Proposal 2.7-2: Support #### Question 2.7-3: To our view, the NDI in the DCI can be used to dynamically indicate the new broadcast TB transmission as legacy, and also implicitly indicate the number of (re-)transmission for UE combining of the same G-RNTI with a HARQ process. And “the total number of transmissions is pre-determined by the gNB” seems not necessary. |
| NTT DOCOMO | #### Proposal 2.7-1: Support Proposal 2.7-2: Support #### Question 2.7-3: We think slot-level repetition will be sufficient to improve the reliability of broadcast PDSCH. |
| Lenovo, Motorola Mobility | #### Proposal 2.7-1: OK. #### Proposal 2.7-2: OK. #### Question 2.7-3: We support blind HARQ retransmissions for broadcast. There is no standard work to support it. |
| Xiaomi | #### Proposal 2.7-1: share the same view with LGE. #### Proposal 2.7-2: We believe one of the repetition scheme is sufficient and we prefer configB which has more flexibility. #### Question 2.7-3: Clarification is needed. What is the difference between gNB-triggered HARQ retransmission and repetition? It should be noted that different repetition uses different RV and UE can certainly combine them. |
| CATT | #### Question 2.7-3: Support blind HARQ retransmissions for broadcast |
| OPPO | #### Proposal 2.7-1: We share the similar view with LG that this proposal is not necessary. #### Proposal 2.7-2: For Rel-17 MBS RRC_IDLE, config B seems sufficient to support MTCH repetition. #### Question 2.7-3: The essential diffidence between this HARQ reTx and repetition should be clarified first, and then we can discuss whether gNB-trigger-based mechanism is needed or not. By now, we do not see the difference between these two mechanisms. |
| Samsung | Proposal 2.7-1/2) Support #### Question 2.7-3) No need for retransmission of broadcast. Repetition is enough. |
| ZTE | We are open to support slot-level repetition for MCCH. However, for both MCCH and MTCH, we think only Config A is needed. The motivation of supporting dynamic change of repetition number is not strong for broadcast considering that anyway conservative scheduling will be used for broadcast in most cases. Question 2.7-3) No need for retransmission of broadcast. Repetition is enough. |
| Spreadtrum | Proposal 2.7-1: share the same view with LG Proposal 2.7-2: One is enough, and prefer Config.A. Since it is broadcast, the flexibility is not needed. Question 2.7-3: Not need. Repetition is enough. |
| vivo | #### Proposal 2.7-1: slot-level repetition for MCCH is not necessary as it is sent periodically. Question 2.7-3: we are not quite sure about the gNB-triggered (not feedback based) HARQ retransmissions here, if it means slot-level repetition for MTCH, it has been supported already. If not, what’s the difference between them? |
| MediaTek | #### Question 2.7-3) No need for retransmission of broadcast. Repetition is enough. |
| CMCC | Proposal 2.7-1: Agree with LG #### Proposal 2.7-2: Ok #### Question 2.7-3: Not need. Repetition is enough. |
| Ericsson | #### Proposal 2.7-1: Support Proposal 2.7-2: Support Question 2.7-3: Support. We have the same understanding as Lenovo: “We support blind HARQ retransmissions for broadcast. There is no standard work to support it”. This assumes of course that the relevant DCI fields are available, i.e. HARQ process ID and NDI. With blind retransmission enabled, also HARQ process ID is needed to allow for large bit rates. @Nokia: About “The total number of transmissions can be pre-determined by the gNB” this should only mean that the number of transmissions is up to the gNB, not relying on UE feedback, but should not be interpreted in such a way that the number needs to be fixed. @NTT DOCOMO: As explained in our contribution, slot-level repetition will not provide any significant time diversity gain. However, with gNB-triggered (blind) HARQ retransmissions the time diversity gain could be quite large, since the time diversity can be far larger than with slot-level repetition. |
| Apple | #### Proposal 2.7-1: may not needed. Proposal 2.7-2: to make it clear, we support the repetition of PDSCH carrying MTCH, not for PDCCH carrying MTCH. We consider only supporting Config A is enough, don’t see strong motivation to support dynamic indication of repetition for inactive/idle UE. For config B, normally there are 64 entries in TDRA table for supporting full flexible resource allocation with dynamic repetition indication. There are 4bit (16 entries) TDRA table in first DCI format. The time domain resource allocation is restrictive to support dynamic repetition indication. |
| Qualcomm | #### Proposal 2.7-1: Support Proposal 2.7-2: Support Question 2.7-3: Need more clarification. Dynamic repetition by using TDRA seems can achieve the similar effect of soft-combining as gNB triggered HARQ retransmission. Does the gNB triggered HARQ retransmission indicate any RV instead of pre-defined RV order in the legacy slot-level repetition? |
| Intel | #### Proposals 2.7-1/2: OK #### Question 2.7-3: Our original proposal was to support HARQ for broadcast. Since that is not agreeable, we think repetition is the best we can do in this release and should strive to include full HARQ support in Rel-18. |
| Moderator | Thanks for discussion: Proposal 2.7-1: - Not needed for MCCH (8) [LG, Nokia, Xiaomi, OPPO, Spreadtrum, vivo, CMCC, Apple] (since MCCH is periodically transmitted) - Support for MCCH (4/5) [NTT DOCOMO, Lenovo, ZTE (open to discuss), Ericsson, Qualcomm, Intel] Although there are more companies that do not see the need for PDSCH repetition for MCCH, there are multiple companies that want to support it. It think it would be good that in the next round the companies supporting/proposing PDSCH repetition for MCCH provide arguments to convince companies. I will leave the proposal unmodified to seek comments. Proposal 2.7-2: - Support both Conf A & Conf B (8) [LG, Nokia, NTT DOCOMO, Lenovo, Samsung, CMCC, Ericsson, Qualcomm, Intel] - Only Conf A (3) [ZTE, Spreadtrum, Apple] - Only Conf B (2) [Xiaomi, OPPO] The majority of companies support both Conf A and Conf B. Comments to support only one conf are on the basis that one configuration is enough. However, no fundamental problems have been raised regarding supporting any of the options. Apple raises the point that with Conf B the full flexibility is not available due to limited bit-width of the TDRA. Given the stronger support, I still propose that both configurations are supported. Supporting both at least covers all the options proposed by proponents without fundamental issues by supporting them. Hope this an acceptable compromise. Question 2.7-3: Various companies have expressed support to support HARQ retransmissions for broadcast. However, multiple companies have also requested clarifications and details on the specific difference with PDSCH repetition. Introducing this would require including in the DCI (Issue 1) HPN and NDI fields so this discussion is relevant to DCI design. Regarding clarifications, besides the comments form Ericsson, I copy here background from R1-2112348 that may help clarify. Please check whether you have more comments/questions: “Assuming the unicast/multicast mechanism for slot-level repetition will be re-used also for broadcast, the implication is that the DCI format will need to carry the Redundancy Version (RV) and UE is expected to perform soft-combining each new RV of a TB with the HARQ buffer. The DCI format will also need to carry a HARQ process ID. This means that the DCI support for broadcast could also cover HARQ combining. Obviously, with broadcast the UE would not send any feedback to trigger HARQ retransmissions, so these would need to be scheduled by the network without such feedback. We may call this gNB-triggered HARQ retransmission to contrast with legacy HARQ retransmission, where the UE triggers the retransmission via NACK feedback. One purpose of gNB-triggered HARQ retransmission would be to provide sufficient coverage where beam sweeping of PDSCH using narrow beams is not desirable and wide beams do not provide enough coverage. The main purpose of gNB-triggered HARQ retransmission would be to provide increased time diversity, similar to that provided by time interleaving in some legacy broadcast systems. The time diversity offered by slot-level repetition is very limited. With a maximum of 16 slots in a “repetition burst” the total duration would only be 16 ms with SCS 15 kHz and half of this with SCS 30 kHz. With a more realistic repetition over e.g. four slots the duration would be only 4 ms (15 kHz SCS) or 2 ms (30 kHz SCS), which is too small values to provide any significant time diversity gain in most scenarios. Observation 16 With gNB-triggered HARQ retransmission for broadcast, the time diversity may be very significantly extended, and be significant also for low speeds such as walking speed. Having very large repetition factors would also be costly from the overhead point of view. To increase time diversity, one could alternatively use HARQ retransmission, where the total time duration of a Transport Block (TB), considering all (gNB-triggered) HARQ retransmission may be much longer, which could allow for better time diversity also with low overhead. If the repetitions are e.g. spread over 100 ms, which may be feasible with broadcast applications that are not very sensitive to latency, significant time diversity gain could be gained also at walking speed. It appears thus that both the required DCI signaling fields and the UE soft-combining capability will anyway be available for broadcast, so supporting also HARQ combining, based on gNB-triggered HARQ retransmissions, would not require any significant additional complexity, neither specification-wise, nor UE complexity-wise. Since such functionality could also provide significant gains in certain scenarios, we think this functionality should be supported in Rel-17, which can be done with almost no additional specification work. It should be noted that for the broadcast UE, the transmission and UE operation would look like multicast, for the case when no HARQ feedback is provided. For non-SPS, the proposed HARQ functionality should be straight-forward. One may also consider using the same type of gNB-triggered HARQ retransmission with SPS. This would however require significant specification work since, unlike regular SPS, retransmissions would need to be part of the SPS configuration, which is not currently the case for SPS for legacy unicast. The proposed HARQ functionality for broadcast therefore only applies to non-SPS in Rel-17. Proposal 24 Support gNB-triggered (not feedback based) HARQ retransmissions for broadcast - Note: UE behavior is the same as with UEs receiving multicast, but with no feedback from the UE. The UE would soft-combine successive HARQ transmissions of the same G-RNTI and HARQ process. The total number of transmissions is pre-determined by the gNB.” |

### 2nd round FL proposals for Issue 7

#### Proposal 2.7-1 [justification needed]

Proposal 5: For RRC_IDLE/INACTIVE UEs, support slot-level repetition for MCCH, using:

- (Config A) UE can be configured with pdsch-AggregationFactor, applied to DCI format 1_0 with MCCH-RNTI.

#### Proposal 2.7-2

For RRC_IDLE/INACTIVE UEs, for slot-level repetition for MTCH, support:

- (Config A) UE can be configured with pdsch-AggregationFactor per G-RNTI, applied to DCI format 1_0 with the G-RNTI.

- (Config B) UE can be configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Broadcast

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same GC-PDSCH.

#### Question 2.7-3 [motivation provided]

Provide your views on the support of gNB-triggered (not feedback based) HARQ retransmissions for broadcast

- Note: UE behavior is the same as with UEs receiving multicast, but with no feedback from the UE. The UE would soft-combine successive HARQ transmissions of the same G-RNTI and HARQ process. The total number of transmissions is pre-determined by the gNB.

Please provide your answers in the table below. Considering the FL comments above:

- do you agree with the proposals 2.7-2?

- Proponents of PDSCH repetition for MCCH, please provide the motivation

- After clarifications provided, provide your views on the support of gNB-triggered (not feedback based) HARQ retransmissions for broadcast.

| company | comments |
| --- | --- |
| ZTE | Proposal 2.7-1: We can support this proposal. Actually, both MCCH and MTCH are contained in PDSCH. The repetition is for PDSCH, it doesn’t matter whether it carriers MCCH or MTCH. We also didn’t see strong motivation to preclude this for MCCH. Proposal 2.7-2: Although we think Config A is sufficient, we can also accept the current proposal. |
| NOKIA/NSB | Question 2.7-3: @Ericsson: regarding ” @Nokia: About “The total number of transmissions can be pre-determined by the gNB” this should only mean that the number of transmissions is up to the gNB, not relying on UE feedback, but should not be interpreted in such a way that the number needs to be fixed.”, Yes, we agree, that is exactly our view as your explanation in above. @All: And the number of “pre-determined” transmissions is up-to gNB implementation, and it should NOT be higher-layer signalled to the broadcast reception UEs, where the toggling of the NDI field in the DCI (associated with a certain G-RNTI) can be used to indicate a new initial transmission to the RV combining UE, and by which the RV combining UE could know on whether the soft combing should be performed with the previously received data in the soft buffer or the soft buffer should be clearly with deleting of previously received data in the buffer. Hope that is align with yours. |
| LG Electronics | Proposal 2.7-1: This seems NOT needed. @ ZTE: It is not clear to us how PDCCH can schedule both MCCH and MTCH in one PDSCH considering different RNTIs are used for MCCH and MTCH. How can UE avoid receiving MTCH for NON-interested G-RNTI, if a DCI is transmitted based on MCCH-RNTI, not G-RNTI. Anyway, if both MCCH and MTCH can be multiplexed in one PDSCH, we assume that gNB does not perform slot-level repetition. It can be up to gNB whether to only transmit MTCH based on PDSCH repetitions or to transmit MTCH together with MCCH based on NO repetition. Proposal 2.7-2: OK Proposal 2.7-3: We support gNB-triggered (not feedback based) HARQ retransmissions for broadcast. |
| Lenovo, Motorola Mobility | 2.7-1: OK 2.7-2: Generally Ok with us. One question for clarification is: in case of Config B, how many entries are defined in TDRA table? What’s the max value? 2.7-3: We support gNB-triggered (not feedback based) HARQ retransmissions for broadcast. |
| NTT DOCOMO | Proposal 2.7-2: Support |
| MediaTek | Proposal 2.7-1: Not support. RAN2 has defined the corresponding MCCH behaviour, e.g., MCCH modification period, MCCH repetition period. Thus, the proposal is not needed. #### Proposal 2.7-2: prefer Config A for broadcast transmission. Question 2.7-3: Not support. Since we have agreed repetition is supported for broadcast transmission, there is no need to consider the gNB triggered HARQ retransmission for broadcast. In addition, if supporting the gNB triggered HARQ retransmission for broadcast with multiple HARQ process, it definitely have larger impact from UE side, especially for the UE HARQ buffer and UE power saving side, and it also against the WID that “In order to facilitate implementation and deployment of the feature, the overall implementation impact should be limited, and the UE complexity should be minimized (e.g. device hardware impact should be avoided)”. |
| CMCC | Proposal 2.7-2: Support Question 2.7-3: What’s the difference between “number of transmissions is pre-determined by the gNB” and pdsch-AggregationFactor or repetitionNumber, from UE’s behaviour, it can soft combine repetition PDSCHs. And what’s the spec impact of this proposal? |
| CATT | Proposal 2.7-3: We support gNB-triggered (not feedback based) HARQ retransmissions for broadcast. |
| Ericsson | 2.7-1: Support. The slot-level PDSCH repetition should be a general physical-layer functionality and not be linked to specific logical channels. It should be noted that, at the physical layer, the UE does not know which logical channel is received, so will in general support slot-level PDSCH repetition and will only discover at higher layers which logical channel was received. Any agreement not to support this functionality for MCCH would only mean a restriction for the gNB, which we see no reason for. Furthermore, if PDSCH repetition is used for MTCH then using it also for MCCH may provide matching coverage in challenging conditions. 2.7-2: Support 2.7-3: The motivation is significant performance increase without complexity increase. The reason for the performance increase (increased time diversity) should be clear from the FL’s quote from our contribution. On a channel without time variations, it does of course not matter whether repetitions come in subsequent slots or whether they are more time separated. In that case the resulting SINR improvement is the same, so there would not be any gain for broadcast in using HARQ retransmissions compared to PDSCH slot-level repetition (for the same amount of redundancy). However, when the channel is time varying, it is well known that time diversity is very valuable and may increase the reception performance a lot. As we showed in our contribution, the available time diversity to combat time variations, is negligible for slot-level repetition but can be very high for gNB-triggered HARQ retransmission. There does not seem to be any UE complexity issues either, since the soft-combining of PDSCH slot-level repetition is anyway supported. @CMCC: The spec impact is that at least the NDI field is necessary, so that the UE can detect when a new TB starts. To support high bit rates also multiple HARQ processes should be supported. The same HPID/NDI fields as for multicast could thus be supported and UE behaviour would be the same, except no feedback with broadcast. @MediaTek: About UE complexity etc: for a UE that already supports NR unicast/multicast, there is no increased complexity by the proposed functionality, i.e. it is fully in line with the WID. |
| Samsung | Proposal 2.7-1: Support |

## Issue 8: TRS as QLC source

### Background

The following agreement for RRC_IDLE/RRC_INACTIVE UEs at RAN1#103-e, RAN2#104-e, RAN1#105-e and RAN1#106-e are relevant for this discussion:

Agreements: - For RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH. - FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH. Agreement: For RRC_IDLE/RRC_INACTIVE Ues, for broadcast reception, the UE may assume that group-common PDCCH/PDSCH is QCL’d with SSB. - It is up to UE implementation whether UE monitors monitoring occasions corresponding to all SSB indexes or monitoring occasions corresponding to a subset of all SSB indexes. - FFS: association rules between SSB indexes and UE monitoring occasions. - FFS: group-common PDCCH/PDSCH is QCl’d with TRS if configured Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, the same beam can be used for group-common PDCCH and the corresponding scheduled group-common PDSCH for carrying MCCH or MTCH. - UE may assume that DMRS ports of the group-common PDCCH/PDSCH for MCCH is QCL’d with SSB. - UE may assume that DMRS ports of the group-common PDCCH/PDSCH for MTCH is QCL’d with SSB. - FFS: group-common PDCCH/PDSCH for MTCH is QCL’d with periodic TRS if configured

The following agreement at RAN#93-e is also relevant for this discussion:

Agreement: - The following aspects can be considered to be within the scope of the Rel-17 MBS WID and can be further discussed in the WGs with the aim of minimizing specification impacts: - Configurable scrambling sequence initialization for PDCCH/PDSCH and DMRS sequence generator initialization for PDCCH/PDSCH for broadcast transmission (as supported for RRC_CONNECTED UE). - Configuring TRS as QCL sources for broadcast transmission (as supported for RRC_CONNECTED UE). - Note: For broadcast transmission, the presence of TRS would be optional from a network perspective. - Note: Any SFN operation is transparent to the UE

### Tdoc analysis

- In [R1-2110779, Huawei]

  - Discuss: Depending on the expected SFN operation performance and the SFN area to be implemented, form network perspective, either SSB or TRS is configured as QCL source for broadcast transmission. Hence, the presence of TRS will be optional as agreed in RAN#93-e.
The UE assumes that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA' for Doppler shift, Doppler spread, average delay and delay spread. For intra-DU SFN operation, the delay spread of multiple SFN cells may be quite different, so SSB cannot be associated with for the delay spread.
As agreed in Rel-17 UE Power Saving Enhancements WI, for a RS resource configured for TRS/CSI-RS occasion(s) for idle/inactive UEs, a quasi co-location type can be determined as ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block and the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63. The broadcast deployment will dominate in low frequency range, e.g., 600MHz/700MHz, there is no beam selection problems in FR2. Hence, the UE can obtain cell timing and Doppler shift with “typeC” QCLed with SSB. Owing to the delay spread is not associated with SSB, the UE can have a more precise channel estimation for intra-DU SFN cells from period TRS.
In Rel-17 UE Power Saving Enhancements WI, configuration for TRS occasion(s) for idle/inactive UEs is based on periodic TRS only. For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a high layer parameter consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. On top of it, the additional specification impact for configuring TRS for broadcast is including such configurations into SIBx/MCCH for MTCH.

  - Proposal 1: Periodic TRS can be configured as QCL source for MTCH transmission especially for RRC_IDLE/INACTIVE UE. The configuration is included in SIBx/MCCH.

- In [R1-2111137, Nokia]

  - Observation-1: Scheme based on SSB with lower modulation scheme could be a better solution in practice from robustness perspective for RRC_IDLE/INACTIVE UE with broadcast transmission.

  - Proposal-20: For further discussion and supporting of TRS with higher modulation scheme, it is preferred having performance evaluation and justification from the proponents before the detailed specification work.

  - Observation-2: Based on the outcome of RAN#93e, there is no update of Rel17 MBS WID, meaning that there is no standardized support specifically for SFN is provided in Rel17 MBS WI. Any SFN operation should be transparent to the UE.

  - Observation-3: There is ongoing work on support of TRS for RRC_IDLE/INATIVE UEs in Rel17 UE power saving WI. How to align the two Rel17 Wis need to be carefully considered, so as to parallel duplicated work in Rel17 on supporting of TRS for RRC_IDLE/INATIVE UEs.

  - Proposal-21: If there is not enough time for specifying TRS for RRC_IDLE/INACTIVE UEs in Rel17 MBS, it can be further considered as a candidate in upcoming Rel18 MBS work.

- In [R1-2112065, LGE]

  - Observation 7: Assuming that low MCS is usually used for broadcast transmission, it is not clear how much we achieve better performance with TRS.

  - Proposal 7: If TRS is agreed to be supported, RAN1 is requested to agree the following proposals:

    - Proposal 7A: a list of NZP CSI-RS resource sets for TRS can be configured for the same cell group serving one or more G-RNTIs.

    - Proposal 7B: QCL-Info is associated with a NZP CSI-RS resource set for TRS and configured to be Type C QCLed with SSB (i.e. Doppler shift, average delay) via SIBx or MCCH.

    - Proposal 7C: The number of NZP CSI-RS resource sets in the list of NZP CSI-RS resource sets for TRS can be configurable for each cell group, similarly as specified in NZP-CSI-RS-ResourceSetList.

  - Proposal 8: For broadcast GC-PDCCH, UE assumes that a PDCCH Monitoring Occasion (MO) is associated with one NZP-CSI-RS-ResourceSet for TRS which is QCLed with the SSB-index mapped to the MO.

    - UE uses the TRS associated with the MO where GC-DCI scheduling GC-PDSCH is received for determining GC-PDSCH antenna port quasi co-location.

  - Proposal 9: If a same SSB index can be associated with more than one NZP CSI-RS resource sets for TRS e.g. in NZP-CSI-RS-ResourceSetPerSSB,

    - for the [x×N+K]th PDCCH monitoring occasion(s) for MTCH in the scheduling window, the number of PDCCH monitoring occasions in MTCH transmission window is greater than N i.e. the number of actual transmitted SSBs; and

    - the same SSB index can be mapped to multiple MOs of which each is associated with one NZP CSI-RS resource set for TRS e.g. in NZP-CSI-RS-ResourceSetPerSSB.

- In [R1-2112241, Qualcomm]

  - If broadcast is transmitted from SFNed multiple cells, GC-PDCCH/PDSCH should be QCL’d with periodic TRS with the multiple cells. The time delay spread of multi-cell transmission is different from that of serving cell’s SSB. The TRS can be configured in a broadcast CFR with transmission no larger than that of the CFR. The TRS can still be QCL-ed with SSB at least in terms of timing, doppler shift.

  - Even if the broadcast is transmission from single cell, the GC-PDSCH for MTCH may use high modulation and TRS is beneficial to link budget.

  - Proposal 7: TRS can be configured in a CFR-Config-Broadcast for RRC_IDLE/INACTIVE UEs.

    - UE may assume that the GC-PDCCH/PDSCH is QCL’d with periodic TRS if configured for broadcast.

    - The TRS can be QCL-ed with SSB at least in terms of timing, doppler.

- In [R1-2111552, Xiaomi]

  - Proposal: Introduce group-specific TRS for MBS capable UE in order to improve the accuracy of T/F synchronization.

    - MBS UE receives the group-specific TRS only when it is in Idle/Inactive state.

### FL Assessment

[Huawei, Qualcomm, Xiaomi] support the introduction of TRS for broadcast reception with UEs in idle/inactive RRC state. [Huawei, Qualcomm] clarify that TRS would be optional and transparent to the UEs. For intra-DU SFN the combined delay spread of the multi cell transmission would be different to that of a single cell and SSB cannot be associated in terms of delay spread while SSB can be used to obtain cell timing and Doppler shift. They clarify that since the broadcast deployment would dominate in the 600MHz/700MHz frequency range, beam selection problems are not in the scope.

[Nokia] discuss that the WID has not been updated so no standardised support of SFN is provided and highlight that ongoing work on TRS for power saving WI should be considered. They acknowledge that if there is not time in this release, it could be considered for Rel-18.

[LGE] although also prefers to delay the introduction to future releases, provides further proposals for the introduction of TRS. Two sets of proposals are made in [LGE], the first set address definition and configuration of NZP CSI-RS for TRS, secondly, it also proposes additional configurations targeted for beam sweeping operation. Since, the proposal to introduce TRS is targeting lower frequency bands, the first set of proposals will be put forward for discussion.

Proposal 2.8-1 addresses the potential agreement of introducing TRS for broadcast reception while Question 2.8-2 discusses aspects proposed on the configuration of NZP CSI-RS.

### 1st round FL proposals for Issue 8

#### Proposal 2.8-1

TRS can be configured in a CFR-Config-MCCH-MTCH for RRC_IDLE/INACTIVE UEs.

- UE may assume that the GC-PDCCH/PDSCH is QCL’d with periodic TRS if configured for broadcast.

- The TRS can be QCL-ed with SSB at least in terms of timing, doppler.

- The configuration is included in SIBx/MCCH

#### Question 2.8-2

Provide your views on the following items on configuration of TRS:

- a list of NZP CSI-RS resource sets for TRS can be configured for the same cell group serving one or more G-RNTIs.

- QCL-Info is associated with a NZP CSI-RS resource set for TRS and configured to be Type C QCLed with SSB (i.e. Doppler shift, average delay) via SIBx or MCCH.

- The number of NZP CSI-RS resource sets in the list of NZP CSI-RS resource sets for TRS can be configurable for each cell group, similarly as specified in NZP-CSI-RS-ResourceSetList.

Please provide your answers in the table below. Considering the FL assessment above,

- do you agree with the proposal 2.8-1? Please provide reasons, views in general if you do not agree.

- Please provide your views on Question 2.8-2.

| company | comments |
| --- | --- |
| LG Electronics | #### Proposal 2.8-1: We prefer to defer TRS to a later release. #### Question 2.8-2: If TRS is supported, the TRS can be configured as listed in this question. |
| NOKIA/NSB | #### Proposal 2.8-1: Not support, TRS should be handled in later release. |
| NTT DOCOMO | #### Proposal 2.8-1: Support |
| Xiaomi | #### Proposal 2.8-1: We agree with the intention of the proposal. However, some refinement is needed: TRS can be configured in a CFR-Config-MCCH-MTCH for RRC_IDLE/INACTIVE UEs. - UE may assume that the DMRS of GC-PDCCH/PDSCH is QCL’d with periodic TRS if configured for broadcast. - The TRS can be QCL-ed with SSB, i.e. QCL type C at least in terms of timing, doppler. - The configuration is included in SIBx/MCCH #### Question 2.8-2: we are fine with this proposal. |
| Samsung | Proposal 2.8-1: Agree #### Question 2.8-2: OK |
| ZTE | Proposal 2.8-1: we are open to support this proposal. |
| vivo | Regarding ‘UE may assume that the GC-PDCCH/PDSCH is QCL’d with periodic TRS if configured for broadcast’, does it mean when TRS is configured, all UEs in the cell shall use TRS as QCL source for broadcast reception instead of SSB? |
| Ericsson | #### P2.8.1: OK to defer TRS to a later Release P2.8.2: If TRS is supported, then this is OK. |
| Qualcomm | #### P2.8.1: support. #### P2.8.2: Some modifications are needed. For SFN scenarios, the TRS cannot be Type C QCLed with SSB. - a list of periodic NZP CSI-RS resource sets for TRS can be configured for the same cell group serving one or more G-RNTIs in a CFR-Config-Broadcast. - QCL-Info is associated with a NZP CSI-RS resource set for TRS and configured to be QCLed with SSB (i.e. timing, Doppler shift,) via SIBx or MCCH. - The number of NZP CSI-RS resource sets in the list of NZP CSI-RS resource sets for TRS can be configurable for each cell group, similarly as specified in NZP-CSI-RS-ResourceSetList. |
| Intel | #### Handle in Rel-18 |
| Huawei, HiSilicon | #### 2.8-1: support #### 2.8-2: ok with QC’s revision. |

## Issue 9: Multiplexing MCCH/MTCH and other PDCCH/PDSCH

### Background

As part of the discussion for the Draft CR on TS 38.202 [R1-2112515], companies discussed multiplexing of MCCH/MTCH and other PDCCH/PDSCH for RRC idle/inactive UE states. In the two figures below show the changes made to Table 6.2.1 on Downlink “Reception Types” where D5 and D6 corresponds to MCCH and MTCH, respectively.

Table 6.2-2 on Downlink “Reception Type” combinations of the Draft CR on TS 38.202 [R1-2112515] shows the comment from the Editor highlighting that more discussion is needed for the scenarios for which D5 and D6 are applicable.

### Tdoc analysis

- [R1-2112241, Qualcomm]

  - Discuss: For LTE SC-PTM

    - RRC_IDLE UEs are not required to receive FDMed SC-PTM and PBCH/SIB/Paging in PCell.

  - For NR broadcast MCCH/MTCH, RAN1 needs to discuss

    - For RRC_IDLE/INACTIVE UEs, whether the UE is required to support FDMed MCCH/MTCH and PBCH/SIB/Paging in PCell.

  - In our view, NR broadcast MCCH/MTCH can be treated similar as LTE SC-PTM, but in addition, we need to consider the multicast case for RRC_CONNECTED UEs.

RRC_IDLE/INACTIVE UEs are not required to support FDMed MCCH/MTCH and PBCH/SIB/Paging in PCell.

  - Proposal 8: For NR broadcast MCCH/MTCH

    - RRC_IDLE/INACTIVE UEs are not required to support FDMed MCCH/MTCH and PBCH/SIB/Paging in PCell.

    - RRC_CONNECTED UEs,

      - Shall be able to support FDMed one PDSCH (for MCCH/MTCH, multicast, or unicast) and PBCH/SIB in a DL CC.

      - Whether to support FDMed one PDSCH (for MCCH/MTCH) and one PDSCH for unicast in a DL CC is subject to UE capability

      - Whether to support FDMed one PDSCH (for MCCH/MTCH), one PDSCH for multicast and unicast in a DL CC is subject to UE capability.

### FL Assessment

Only the contribution in [Qualcomm] address the issue of multiplexing MCCH/MTCH and other PDCCH/PDSCH. However, given the discussions with the CRs to TS 38.202 this issue is included for discussion in this meeting. [Qualcomm] presents that for LTE SC-PTM, RRC idle UEs are not required to receive FDMed SC-PTM and PBCH/SIB/Paging in PCell and consider that this “Reception Type” combination should be applied to NR broadcast. They also discuss the case for RRC connected UEs, but in this AI we initially would only need to focus on RRC idle/inactive UEs.

Question 2.9-1 is put forward to collect company views. Based on rounds of discussion a further proposal for agreement could be included.

### 1st round FL proposals for Issue 9

#### Question 2.9-1

Are RRC_IDLE/INACTIVE UEs required to support FDMed MCCH/MTCH and PBCH/SIB/Paging in PCell?

Considering the FL assessment above, please provide your views on Question 2.9-1 in the table below.

| company | comments |
| --- | --- |
| NOKIA/NSB | To our view, it mainly depends on UE capability. If the idle/inactive UEs do not support simultaneous reception, it is straightforward that the high-priority PBCH/SIB/Paging PDSCH is received, with dropping of FDMed MCCH/MTCH GC-PDSCH. The network may try by implementation to avoid such dropping in reality, but if such dropping happened for idle/inactive UEs with broadcast reception, the performance of broadcast reception will be impacted. And for idle/inactive UE support simultaneous reception, then the performance impact can be avoided, but of course, with introducing the higher UE capability required. |
| Lenovo, Motorola Mobility | As discussed in AI8.12.1, for RRC connected UE, FDM between one multicast reception and another multicast reception is fully dependent on UE capability. For Idle mode UE, we think network can avoid such overlapping since network doesn’t have UE capability info. |
| ZTE | Based on the following spec, at least for FR1, UE has the UE capability to decode overlapping unicast PDSCH and SIB PDSCH. The broadcast PDSCH is similar to unicast PDSCH, thus we think UE is required to support FDMed MCCH/MTCH and PBCH/SIB/Paging in PCell at least in FR1. On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI. |
| MediaTek | Since UE cannot report capability, FDMed reception is not supported. We have the similar view with Lenovo that “For Idle mode UE, network can avoid such overlapping since network doesn’t have UE capability info.” |
| Ericsson | In the context of the Case C/D/E discussion, seamless transition of broadcast reception between RRC IDLE/INACTIVE and RRC CONNECTED has been considered important. When a UE is receiving broadcast in RRC IDLE/INACTIVE, it can be expected to be a common situation that the UE is paged to go to RRC Connected to receive some service there. It may be difficult for the network to ensure that paging occasions do not coincide with broadcast transmissions. Therefore, without the FDM support, UEs are likely to experience a broadcast service interruption when paged, which would be undesirable. To avoid this, UEs should support FDM, at least as a UE capability. |
| Apple | Share the similar view with Lenovo. FDM reception is optional UE feature for connected UE, it should not be mandate requirement for idle UE. |
| Qualcomm | We think RRC IDLE/INACTIVE UEs without UE capability indication are not required to receive FDMed MCCH/MTCH and PBCH/SIB/Paging in PCell. |
| Intel | Since FDM is UE capability, it’s not supported for broadcast. |

## Other Issues proposed for lower priority

The proposal for the Issues below is that are considered with lower priority to focus on the other issues above in this summary. If companies think that any of the issues below address a critical functionality that is missing for this AI, please provide also these comments to the table below to evaluate the situation based on comments.

### Other Issue 1: Number of CFRs for MTCH

This issue has been discussed at past meetings without reaching a conclusion. Although some companies did support specifying more than one CFR for MTCH, multiple companies did not support it. The motivation to introduce multiple CFRs for MTCH is to provide higher flexibility to the network to deploy services with different requirements. To this meeting contributions from [ZTE, vivo, Nokia] supported more than one CFR for MTCH, while [OPPO, Intel, Xiaomi, CMCC, NTT DOCMO, Lenovo, MediaTek, Ericsson] explicitly mentioned that only supported one CFR for MTCH.

The FL initial proposal is not to discuss further this Issue for this meeting.

### Other Issue 2: PDSCH: Semi Persistent Scheduling

This issue has been discussed at multiple meetings including the last one. There are companies that want to introduce SPS for broadcast reception with idle/inactive UEs with the motivation to reduce PDCCH overhead for services that have periodic transmissions. However, there were companies that had concerns with the complexity of such functionality at such at such late stage of the release, or concerns were raised on SPS without activation/deactivation mechanisms.

Contributions to this meeting from [ZTE, vivo, Nokia, Xiaomi, NTT DOCOMO, LGE, Ericsson] propose to support/discuss SPS for broadcast reception in idle/inactive UEs. On the other hand, [OPPO, Apple] propose to not to support or to consider for next releases.

The FL initial proposal is not to discuss further this Issue for this meeting.

### Other Issue 3: PDCCH: CORESET for MCCH and MTCH

There have been multiple agreements on the configuration of CORESET in past meetings. Based on the agreements so far, FL understands that the basic functionality is in place. Please comment in the table below if this understanding is not correct.

Aspects that were proposed at the last meeting were the possibility to configure separately different CORESET for MCCH and MTCH to provide higher flexibility on the configuration of services. However, there was no conclusion on the discussion.

Contributions to this meeting discussing configuration of CORESET are [Huawei, TD Tech, Nokia, OPPO, Xiaomi, Lenovo, Ericsson]. While [Huawei, Nokia] discusses different configuration for MCCH and MTCH, [OPPO, Xiaomi, Lenovo] propose to use the same coreset for both channels.

A note is that Issue 4 in this summary is discussing separate PDCCH/PDSCH-Configs for MCCH and MTCH.

The FL initial proposal is not to discuss further this Issue for this meeting.

### Other Issue 4: HARQ feedback for RRC_IDLE/RRC_INACTIVE UE states

This issue has been discussed at previous meetings but not the last one. Most companies did not support introducing such a functionality for idle/inactive UEs. To this meeting contributions form [vivo, Xiaomi, Samsung, Lenovo] propose that HARQ feedback is not supported, while [OPPO] proposes to support.

The FL’s proposal is not to discuss further this Issue for this meeting.

### Other Issue 5: Broadcast services supported for both RRC_CONNECTED and RRC_IDLE/RRC_INACTIVE UEs

This aspect was discussed at initial meetings of this release but has not been discussed at the last meetings. Inputs to this meeting [CATT, Intel, MediaTek, Ericsson] discuss aspects on this issue. However, most of the discussions and proposals are to clarify that network implementation makes sure that UEs in RRC connected state use a BWP that contains the CFR.

The FL’s proposal is not to discuss further this Issue for this meeting.

### Other Issue 6: Discontinuous Reception (DRX) and Wakeup Signals (WUS)

This issue has only been discussed at [CATT].

The FL’s proposal is not to discuss this Issue for this meeting.

### Other Issue 6: UE feedback for MBS Interest Indication for partial beam sweeping or MCS determination

This issue has been discussed at [Nokia, Sony].

The FL’s proposal is not to discuss this Issue for this meeting.

Provide your comments/vies if any in the following table.

| company | comments |
| --- | --- |
| Lenovo, Motorola Mobility | 2.10.1: we support only one CFR for MTCH. 2.10.2: we support SPS for broadcast for saving PDCCH overhead. 2.10.3: We support same CORESET for MTCH and MCCH. 2.10.4: No HARQ feedback for broadcast transmission. |

# Proposals for Discussion at GTW sessions

This section will include proposals for potential discussion at the different GTW scheduled for NR MBS at RAN1#107-e.

## GTW on 11 Nov

#### Proposal 2.2-1

Confirm the working assumption made at RAN1#106bis-e:

Working assumption:

Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.

- Send an LS to RAN2 with the mechanism agreed in RAN1

#### Proposal 2.1-1

For GC-PDSCH scheduled with DCI format 1_0 for broadcast reception, the size of the FDRA field depends on the size of the CFR.

#### Proposal 2.1-2

For GC-PDSCH scheduled with DCI format 1_0 for broadcast reception, RB numbering starts from the lowest RB of the CFR.

#### Proposal 2.1-3

For GC-PDSCH scheduled with DCI format 1_0 for broadcast reception, the resource allocation granularity is single RB.

## GTW on 15 Nov

#### Proposal 2.1-6

for broadcast reception, the following options is supported for VRB-to-PRB mapping field in the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH

- Opt-1: DCI includes the VRB-to-PRB mapping field with 1 bit according to Table 7.3.1.2.2-5 in TS 38.212

#### Proposal 2.1-1rev1

For FDRA determination of the DCI 1_0 format for GC-PDCCH for broadcast reception:

- is the size of CORESET 0

- If the size of CFR (i.e. NCFR) is larger than the size of CORESET0, the resource indication value (RIV) is defined as in section 5.1.2.2.2 in TS38.214, where K is the maximum value from set {1, 2, 4, 6, 8, 10, 12} which satisfies K≤NCFR/NBWPinitial;otherwise, K=1.

#### Proposal 2.4-3

For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs:

- The CFR frequency resources used for MCCH and MTCH are configured by SIBx;

- PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MCCH is configured by SIBx

- PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MTCH is configured by MCCH. If the PDCCH-config/PDSCH-config for MTCH is not configured, the PDCCH-config/PDSCH-config for GC-PDCCH/PDSCH carrying MCCH configured by SIBx is reused for GC-PDCCH/PDSCH carrying MTCH.

#### Proposal 2.7-2

For RRC_IDLE/INACTIVE UEs, for slot-level repetition for MTCH, support:

- (Config A) UE can be configured with pdsch-AggregationFactor per G-RNTI, applied to DCI format 1_0 with the G-RNTI.

- (Config B) UE can be configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Broadcast

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same GC-PDSCH.

#### Proposal 2.4-4rev1

The following agreements for RRC_CONECTED UEs also apply for broadcast reception with UEs in RRC_IDLE/ RRC_INACTIVE states, with the following updates:

Agreement:

For LBRM and TBS determination for GC-PDSCH:

- The maximum number of layers can be provided by maxMIMO-Layers in PDSCH-Config for MBS in CFR; if not provided, a default value is defined.

  - FFS the default value.

- The maximum modulation order can be determined from mcs-Table in PDSCH-Config for MBS in CFR;

  - FFS: if mcs-Table in PDSCH-Config for MBS is not configured in CFR, a value determined from mcs-Table in PDSCH-Config for unicast in the active DL BWP is used; if the mcs-Table in PDSCH-Config for unicast is not configured, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16).

- xOverhead can be provided in PDSCH-Config for MBS in CFR; if not provided, a default value of zero is used.

- The number of PRBs is determined based on the size of CFR.

Agreement:

For LBRM and TBS determination for GC-PDSCH, the default value of the maximum number of layers is 1 if maxMIMO-Layers in PDSCH-Config for MBS in CFR is not configured.

Agreement:

For determination of maximum modulation order for LBRM and TBS determination for GC-PDSCH,

- if mcs-Table in PDSCH-Config for MBS is not configured in CFR, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16).

For LBRM and TBS determination for GC-PDSCH for broadcast reception:

- the maximum number of layers is 1

- the maximum modulation order can be determined from mcs-Table in PDSCH-Config for broadcast.

- If mcs-Table in PDSCH-Config is not configured in CFR for broadcast, Table 5.1.3.1-1 in TS38.214 is used.

#### Proposal 2.4-1rev1

Adding the following PDSCH TDRA table determination rule for broadcast to Table 5.1.2.1.1-1 of TS38.214.

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | pdsch-ConfigCommon includes pdsch-TimeDomainAllocationList | pdsch-Config includes pdsch-TimeDomainAllocationList | pdsch-Config-broadcast includes pdsch-TimeDomainAllocationList | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- | --- |
| MCCH_RNTI, G_RNTI for broadcast | Type-x Common for broadcast | 1 | No | - | - No | Default A (if SearchSpaceZero is configured) |
|  |  | 2 | No | - | - No | Default B (if SearchSpaceZero is configured) |
|  |  | 3 | No | - | - No | Default C (if SearchSpaceZero is configured) |
|  |  | 1,2,3 | No | - | No | Default A (if SearchSpaceZero is NOT configured) |
|  |  | 1,2,3 | Yes | - | No | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | pdsch-TimeDomainAllocationList provided in pdsch-Config-broadcast |

#### Proposal 2.4-2rev1

For Case D/E (if supported), the definition of the broadcast BWP/CFR frequency resources reuses the legacy definition of BWP frequency resources for unicast using the combination of Point A, offsetToCarrier and locationAndBandwidth to indicate the exact location of the BWP/CFR with respect to the carrier starting RB.

# Stable Proposals

# Summary of Agreements

This section includes the agreements for RAN1#107-e.

Agreement

Confirm the working assumption made at RAN1#106bis-e:

Working assumption:

Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.

- Send an LS to RAN2 with the mechanism agreed in RAN1

Agreement

For GC-PDSCH scheduled with DCI format 1_0 for broadcast reception, RB numbering starts from the lowest RB of the CFR.

And we take the following as conclusion:

Conclusion

For broadcast reception, the DCI 1_0 format for GC-PDCCH scheduling a GC-PDSCH does not include the field TB scaling.

# References

- RP-201038	Revised Work Item on NR Multicast and Broadcast Services, Huawei, HiSilicon

Relevant tdoc from RAN#93-e

- RP-212559	Moderator's summary for email discussion [93e-19-MBS-WI]	3GPP RAN1 WG Vice-Chair

Relevant tdoc from AI 5

- R1-2112311	Discussion on RAN2 LS on broadcast session delivery about MCCH design	MediaTek Inc.

Relevant tdocs from AI 8.12.3

- R1-2110779	Discussion on UE receiving broadcast in RRC IDLE/INACTIVE state	Huawei, HiSilicon, CBN

- R1-2110891	MBS Support for RRC IDLE/INACTIVE UEs	FUTUREWEI

- R1-2110897	Discussion on basic functions for broadcast mode	TD Tech, Chengdu TD Tech

- R1-2110912	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	ZTE

- R1-2111041	Remaining issues on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE Ues	vivo

- R1-2111115	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	Spreadtrum Communications

- R1-2111137	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

- R1-2111232	Discussion on basic functions for broadcast multicast for RRC_IDLE RRC_INACTIVE UEs	CATT

- R1-2111305	Discussion on basic functions for RRC_IDLE/RRC_INACTIVE UEs	OPPO

- R1-2111408	Considerations on MBS functions for RRC_IDLE/INACTIVE UEs	Sony

- R1-2111518	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

- R1-2111551	Discussion on basic functions for broadcastmulticast for RRC_IDLERRC_INACTIVE UEs	Xiaomi

- R1-2111629	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states	CMCC

- R1-2111763	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

- R1-2111899	Discussion on MBS for RRC_IDLE and RRC_INACTIVE UEs	Apple

- R1-2112065	Basic function for broadcast/multicast	LG Electronics

- R1-2112082	Discussion on basic functions for broadcast or multicast for RRC_IDLE and RRC_INACTIVE UEs	ASUSTeK

- R1-2112130	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	NTT DOCOMO, INC.

- R1-2112163	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

- R1-2112241	Views on group scheduling for Broadcast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

- R1-2112314	Discussion on MBS for RRC_IDLE and INACTIVE UEs	MediaTek Inc.

- R1-2112348	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

Relevant tdocs from AI 8.12.4

- R1-2111552	Discussion on remaining issues for idle and inactive UE	Xiaomi

- R1-2111917	Discussion on RRC parameters for NR MBS	Huawei, HiSilicon, CBN

# Annex A: Agreements in previous RAN1 meetings

## RAN1#103-e agreements

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

- FFS details

Agreements:

- For RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH.

  - FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource(s) for group-common PDCCH/PDSCH.

- the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.

- FFS: the relation of the common frequency resource(s) (if configured) and initial BWP.

- FFS: whether to configure one/more common frequency resources

- FFS: configuration and definition details of the common frequency resource

Agreements: From physical layer perspective, for broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs.

- FFS details.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, CSS is supported for group-common PDCCH.

- FFS: reuse current CSS type, define a new CSS type, etc.

- FFS other details.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured within the common frequency resource for group-common PDCCH/PDSCH. CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP and the CORESET is not configured.

- FFS: configuration details of the CORESET for group-common PDCCH/PDSCH

## RAN1#104-e agreements

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, one common frequency resource for group-common PDCCH/PDSCH can be defined/configured.

- FFS: whether to define/configure more than one common frequency resources

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, the UE may assume that group-common PDCCH/PDSCH is QCL’d with SSB.

- It is up to UE implementation whether UE monitors monitoring occasions corresponding to all SSB indexes or monitoring occasions corresponding to a subset of all SSB indexes.

- FFS: association rules between SSB indexes and UE monitoring occasions.

- FFS: group-common PDCCH/PDSCH is QCl’d with TRS if configured

Agreement:

For broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs when UE-specific active BWP of RRC_CONNECTED UE contains the common frequency resource of RRC_IDLE/INACTIVE UEs and the SCS and CP are the same.

- FFS: the case when UE-specific active BWP of RRC_CONNECTED UE does not contain the common frequency resource of RRC_IDLE/INACTIVE UEs.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, further study the following cases of a configured/defined specific common frequency resource (CFR) for group-common PDCCH/PDSCH, and identify which case(s) will be supported:

- [Case E] the case where a CFR is defined based on a configured BWP.

  - In particular, study the following:

    - whether a configured BWP for MBS is needed or not.

    - whether BWP switching is needed or not.

  - In this study, the configured BWP has the following properties:

    - The configured BWP is different than the initial BWP where the frequency resources of this initial BWP are configured smaller than the full carrier bandwidth.

    - The CFR has the frequency resources identical to the configured BWP.

    - The configured BWP needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

  - Note: The configured BWP is not larger than the carrier bandwidth

- the case where the initial BWP fully contains the CFR in the frequency domain.

  - In this study the following sub-cases are considered:

    - [Case B] A CFR with smaller size than the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP.

    - [Case D] A CFR with smaller size than the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP.

  - In particular, study the following:

    - Whether the considered two options with a CFR with smaller size than the initial BWP are needed or not for MBS.

- the case where the initial BWP has same size as the CFR in the frequency domain.

  - In this study the following two sub-cases are considered:

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

  - In particular, study the following:

    - Whether the considered two options with a CFR with the same size as the initial BWP are needed or not for MBS.

## RAN1#105-e agreements

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MCCH.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, DCI format 1_0 is used as baseline for GC-PDCCH of MCCH and MTCH.

- FFS details of FDRA.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, RAN1 confirms the following assumptions made by RAN2

- RAN2 assumes, in case searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the mapping between PDCCH occasions and SSBs is the same as for SIB1.

- RAN2 assumes that if common search space other than searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols are sequentially numbered from one in the MCCH transmission window and mapped to SSBs using the similar rule as defined for OSI in TS 38.331.

Agreement:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs support the same CSS type for MCCH and MTCH.

- FFS support of different CSS types for MCCH and MTCH channels for broadcast reception.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, study the following alternatives for MCCH change notification indication due to session start:

- Alt 1: Define a dedicated RNTI to scramble the CRC of a DCI indicating a MCCH change notification;

- Alt 2: Use of a field in a DCI format scheduling a MCCH without a dedicated RNTI for MCCH change notification;

Other solutions are not precluded and it is also not precluded whether to support both Alt1 and Alt2.

Conclusion:

It is up to RAN2 to decide the specific contents of the MCCH change notification, e.g, whether notification only informs about session start, whether or not notification also informs about session modification/stop or whether or not the notification informs about any other information.

Agreement:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use a configured/defined CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case A), to receive GC-PDCCH/PDSCH carrying MCCH.

- Note: GC-PDCCH/PDSCH transmission within a narrower portion of the Initial BWP (where the initial BWP has the same frequency resources as CORESET0) is possible by implementation via appropriate scheduling.

Agreement:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use a configured/defined CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case A), to receive GC-PDCCH/PDSCH carrying MTCH.

- Note: GC-PDCCH/PDSCH transmission within a narrower portion of the Initial BWP (where the initial BWP has the same frequency resources as CORESET0) is possible by implementation via appropriate scheduling.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, the CORESET index can be the same for GC-PDCCH of MCCH and MTCH.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, the same beam can be used for group-common PDCCH and the corresponding scheduled group-common PDSCH for carrying MCCH or MTCH.

- UE may assume that DMRS ports of the group-common PDCCH/PDSCH for MCCH is QCL’d with SSB.

- UE may assume that DMRS ports of the group-common PDCCH/PDSCH for MTCH is QCL’d with SSB.

- FFS: group-common PDCCH/PDSCH for MTCH is QCL’d with periodic TRS if configured

Agreement:

For Rel-17, for broadcast reception, RRC_IDLE/RRC_INACTIVE UEs do not exceed the maximum number of CORESETs mandatorily (in the minimum capability) supported for Rel-15/Rel-16 UEs, i.e., 2 CORESETs.

- If the CFR has the same frequency range as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 or where the initial BWP has the frequency resources configured by SIB1, RRC_IDLE/RRC_INACTIVE UEs can be configured with the following options:

  - CORESET#0 (default option if CFR is the initial BWP and CORESET is not configured); or

  - CORESET configured by commonControlResourceSet; or

  - CORESET#0 and CORESET configured by commonControlResourceSet.

## RAN1#106-e agreements

Agreement:

From RAN1 perspective, the CFR for broadcast reception of RRC_IDLE/INACTIVE UEs, includes at least the following configurations:

- One set of parameters configured for PDSCH for broadcast reception with GC-PDSCH

- One set of parameters configured for PDCCH for broadcast reception with GC-PDCCH

- FFS: whether some parameters configured for PDSCH/PDCCH are optional/needed for the supported cases of CFR.

- FFS: If necessary, depending on the cases supported, starting PRB and the number of PRBs

  - The reference for starting PRB is Point A. (Following the same approach to determine reference for starting PRB as that defined in AI8.12.1.)

Conclusion:

There is no specification support in Rel-17 for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs with configured/defined CFRs for group-common PDCCH/PDSCH with smaller size than the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case B).

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, if searchSpace#0 is configured for MTCH, the mapping between PDCCH occasions and SSBs is the same as for SIB1.

Agreement:

Study and reach an agreement by RAN1#106b-e on whether Alt1 and Alt2 for MCCH change notification indication can accommodate at least 2 bits for the notification of MCCH configuration changes due to a session start and the notification of MCCH configuration changes of an ongoing session (including session stop).

Agreement:

The DCI format for GC-PDCCH scheduling a GC-PDSCH carrying MCCH/MTCH at least includes the following fields for broadcast reception with UEs in RRC_IDLE/INACTIVE state:

- FDRA field

- TDRA field

- Modulation and coding scheme

- Redundancy version

- FFS:

  - MCCH change notification (if supported and only for MCCH),

  - RB numbering starts from the lowest RB of the CFR and support of resource allocation with granularity of single or multiple RBs.

  - HARQ process number and New data indicator

  - VRB-to-PRB mapping

  - other fields if needed.

Agreement:

Only one CFR can be configured for group-common PDCCH/PDSCH carrying MCCH for broadcast reception with UEs in RRC_IDLE/INACTIVE state.

Agreement:

For broadcast reception with UEs in RRC_IDLE/INACTIVE state, the DCI size of GC-PDCCH scheduling a GC-PDSCH carrying MCCH/MTCH is aligned with DCI format 1_0 with CRC scrambled by C-RNTI in the CSS.

Agreement:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use the same bandwidth configurations for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH.

- FFS: use of different bandwidth configurations for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH

Conclusion:

For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs, there is no specification support in Rel-17 of different CSS types for GC-PDCCH scheduling MCCH and MTCH.

Agreement:

Study whether the Type-x CSS supported for multicast in RRC_CONNECTED can be reused as baseline for broadcast in RRC_IDLE/RRC_INACTIVE for GC-PDCCH scheduling MCCH and MTCH.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs with broadcast reception, if common search space other than searchSpace#0 is configured for MTCH, the mapping of PDCCH monitoring occasions to SSBs can be configured with a rule.

- The existing rule defined for OSI in TS 38.331 is used as starting point to define the above rule.

## RAN#93-e agreements

Agreement:

- The following aspects can be considered to be within the scope of the Rel-17 MBS WID and can be further discussed in the WGs with the aim of minimizing specification impacts:

  - Configurable scrambling sequence initialization for PDCCH/PDSCH and DMRS sequence generator initialization for PDCCH/PDSCH for broadcast transmission (as supported for RRC_CONNECTED UE).

  - Configuring TRS as QCL sources for broadcast transmission (as supported for RRC_CONNECTED UE).

- Note: For broadcast transmission, the presence of TRS would be optional from a network perspective.

- Note: Any SFN operation is transparent to the UE

Agreement (Updated proposal from RAN1#106e):

For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state.

- Support Case-C

- Support at least one of Case D and Case E.

  - Down-selection to be made at RAN1#106b-e

- Note: Case C, D and E are defined in previous agreements

## RAN1#106bis-e agreements

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MTCH.

Agreement:

The PDCCH/PDSCH parameters for broadcast reception with GC-PDCCH/PDSCH, which are not configured, use as default the value of the PDCCH/PDSCH parameters for the configuration of the Rel-15/Rel-16 initial BWP for RRC_IDLE/RRC_INACTIVE UEs.

Agreement:

For initializing scrambling sequence generator for GC-PDCCH for MCCH/MTCH for broadcast,

- nID equals the higher layer parameter pdcch-DMRS-ScramblingID if it is configured in a CFR used for the GC-PDCCH for MCCH/MTCH; nID=NIDcell otherwise.

- nRNTI=0.

Agreement:

For broadcast reception with UEs in RRC_IDLE/INACTIVE states, support slot-level repetition for MTCH.

Agreement:

For initializing scrambling sequence generator for GC-PDSCH for MCCH/MTCH for broadcast,

- nID equals the higher layer parameter dataScramblingIdentityPDSCH if it is configured in a CFR used for GC-PDSCH for MCCH/MTCH and the RNTI equals the G-RNTI or MCCH-RNTI; nID=NIDcell otherwise.

- nRNTI corresponds to the RNTI associated with the GC-PDSCH transmission.

Agreement:

For initializing sequence generator for DMRS of GC-PDCCH for MCCH/MTCH for broadcast,

- NID equals the higher layer parameter pdcch-DMRS-ScramblingID if it is configured in a CFR used for the GC-PDCCH for MCCH/MTCH; NID=NIDcell otherwise.

Agreement:

For initializing sequence generator for DMRS of GC-PDSCH for MCCH/MTCH for broadcast,

- NID0  equals the higher-layer parameters scramblingID0 if it is configured in the DMRS-DownlinkConfig IE in a CFR used for GC-PDSCH for MCCH/MTCH; NIDnSCIDλ=NIDcell  otherwise.

Working assumption:

Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.

- Send an LS to RAN2 with the mechanism agreed in RAN1

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs for broadcast reception, MTCH scheduling is associated with a window defined by the MTCH monitoring periodicity and the starting of the periodicity

- FFS: the window is associated to one or multiple or all G-RNTI.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs for broadcast reception, at least support that within the MTCH scheduling window, the association between the PDCCH monitoring occasions and SSB is defined as:

- the [x×N+K]th PDCCH monitoring occasion(s) for MTCH in the scheduling window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in MTCH transmission window/N).

- For the purpose of associating PDCCH monitoring occasion for MTCH and SSB, the UE assumes that, in the MTCH scheduling window, PDCCH for an MTCH scrambled by G-RNTI is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

# Annex B: [R1-2104165] RAN2 LS on broadcast session delivery and MCCH design

R1-2104165 submitted to RAN1#105-e reproduced here for convenience:

3GPP TSG RAN WG1 #105-e R1-2104165 e-Meeting, May 10th – 27th, 2021 3GPP TSG-RAN WG2 Meeting #113bis-e R2-2104639 E-meeting, 12th – 20th April 2021 Title: LS on broadcast session delivery and MCCH design Response to: Release: Release 17 Work Item: NR_MBS-Core Source: RAN2 To: RAN1 Contact person: Dawid Koziol dawid.koziol@huawei.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: N/A # 1 Overall description RAN2 discussed the details of broadcast session delivery and the following agreements were made during RAN2#113-e meeting: - Both idle/inactive UEs and connected mode UEs can receive MBS services transmitted by NR MBS delivery mode 2 (Broadcast service as already agreed, TBD other). The ability for connected mode UEs to receive this may depend on the network provisioning of the service (e.g. which freq), UE connected mode configuration and UE capabilities. - The two-step based approach (i.e. BCCH and MCCH) as adopted by LTE SC-PTM is reused for the transmission of PTM configuration for NR MBS delivery mode 2. - Assume it is possible to reuse LTE SC-PTM mechanism for the CONNECTED UEs to receive the PTM configuration for NR MBS delivery mode 2, i.e. broadcast based manner. - Assume that MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start for delivery mode 2 of NR MBS (other cases FFS, if any).. For RAN1 to better understand the above agreements, RAN2 would like to clarify that RAN2 is working on two MBS delivery modes (DM1 and DM2), summarized as follows: - DM1 is used for multicast session delivery and is applicable to UEs in RRC Connected state (FFS UEs in RRC Inactive, but this scenario is down-prioritized). The UE is provided with MBS configuration e.g. G-RNTI using dedicated RRC signalling when the UE is in RRC Connected state. DM1 can use both Point-to-Point and Point-to-Multipoint transmissions and can take advantage of UL UE feedback (e.g. HARQ) when the UE is in RRC Connected. - DM2 is used for broadcast session (FFS for multicast session for UEs in RRC Inactive, but this scenario is down-prioritized) delivery and is applicable to UEs in all RRC states. The UE is provided with MBS configuration using common RRC signalling in a two-step based approach, i.e. SIB will be used to provide the transmission configuration of MCCH. Based on the MCCH configuration received via SIB, UE reads MCCH, which carries transmission configuration of MTCH(s), e.g. G-RNTI. The MTCH configuration acquired from MCCH is applied by the UE for MTCH reception regardless of UE’s RRC state (for RRC_CONNECTED state, the possibility to receive MTCH can be further subject to UE’s configuration and capabilities). It was also agreed that RAN2 will prioritize multicast session reception in RRC Connected mode in Rel-17. If time permits multicast support for RRC Inactive can be considered later, once connected mode Multicast solution and Broadcast solution become more mature. Furthermore, RAN2 defines two types of logical channels used at least for broadcast session delivery using DM2: - MTCH: A point-to-multipoint downlink channel for transmitting traffic data from the network to the UE. - MCCH: A point-to-multipoint downlink channel used for transmitting MBS control information from the network to the UE, for one or several MTCH(s). - In RAN2, some companies think it should be allowed to configure multiple MCCH(s) for different services, but other companies disagree with the need for multiple MCCH and RAN2 has not made a decision on this issue yet. During RAN2#113bis-e meeting, RAN2 discussed further aspects of MCCH scheduling and MCCH change notification leading to the following agreements with RAN1 impacts: - The concept of MCCH transmission window, similar to the one used for LTE SC-PTM, is used for NR MCCH scheduling. The exact parameters to define the window are FFS (discussed in the following proposals). - The MCCH transmission window is defined by MCCH repetition period, MCCH window duration and radio frame/slot offset. - New RNTI is defined for scheduling MCCH. - Common search space is needed for MCCH scheduling. RAN2 should request RAN1 to discuss the details of CSS for MCCH. - R2 assumes PDCCH occasions for MCCH search space are associated with SSBs in a pre-defined manner so that the UE can receive MCCH scheduling on PDCCH occasions according to its detected SSB. - R2 assumes, In case searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the mapping between PDCCH occasions and SSBs is the same as for SIB1. - R2 assumes that If common search space other than searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols are sequentially numbered from one in the MCCH transmission window and mapped to SSBs using the similar rule as defined for OSI in TS 38.331. - Request RAN1 to discuss the details of the configuration of the bandwidth for MCCH reception. - The modification period is defined for NR MCCH and NR MCCH contents are only allowed to be modified at each modification period boundary. - The updated MCCH message should be sent in the same MCCH modification period where the change notification is sent. - UE in RRC IDLE/INACTIVE should be able to monitor/read both MCCH channel and SI/Paging without BWP switch. It is up to RAN1 to decide how this is ensured. - It is up to RAN1 to to decide about the RNTI and DCI format used for MCCH change notifications. - FFS whether to support multiple MCCH, e.g. to support different service types. - RAN2 will discuss and down-select from the following two options for the UE to get aware of session stop/modification: - Reading MCCH once per each MCCH modification period when receiving an ongoing broadcast session - DCI used for MCCH notification indicates the change of an ongoing broadcast session. The agreements made by RAN2 require further discussions in RAN1. In particular, RAN2 would like to request RAN1 to investigate and provide feedback on the following aspects, considering the above agreements made by RAN2: - Details of Common Search Space design for MCCH channel, e.g. is SS#0 allowed to be configured as a search space for MCCH, is search space other than SS#0 allowed to be configured as a search space for MCCH. - Details of the allowed transmission bandwidth/BWP configurations for MCCH transmission. - Details of the RNTI and DCI design for carrying MCCH change notifications. - NOTE: RAN2 is still discussing some aspects that may have an impact on this issue, e.g. whether or not to support multiple MCCH or whether or not a notification about the modification/stop of an ongoing session is needed, as indicated above. RAN2 will update RAN1 as soon as further agreements are made on these items. # 2 Actions To RAN1 group: ACTION: RAN2 respectfully asks RAN1 to take RAN2 agreements into account in their work on MBS and discuss RAN1 aspects of MCCH as requested above. # 3 Dates of next RAN2 meetings TSG-RAN2 Meeting #114-e May 19 – May 27, 2021 E-Meeting

# Annex C: [R1-2106410] RAN2 LS on update for MCCH design

R1-2106410 submitted to RAN1#106-e reproduced here for convenience.

3GPP TSG RAN WG1 #106-e R1-2106410 e-Meeting, August 16th – 27th, 2021 3GPP TSG-RAN WG2 Meeting #114-e R2-2106544 E-meeting, 19th – 27th May 2021 Title: LS on update for MCCH design Response to: Release: Release 17 Work Item: NR_MBS-Core Source: RAN2 To: RAN1 Contact person: Dawid Koziol dawid.koziol@huawei.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: N/A # 1 Overall description RAN2 discussed further the aspects related to MCCH design and made the following agreements during RAN2#114 meeting: - MBS specific SIB is defined to carry MCCH configuration. - MCCH contents should include information about broadcast sessions such as G-RNTI, MBS session ID as well as scheduling information for MTCH (e.g. search space, DRX). L1 parameters that need to be included in MCCH are pending further RAN1 progress and input. - Postpone the discussion on whether dedicated MCCH configuration is required until RAN1 makes progress on BWP/CFR for MCCH. - Indication of an MCCH change due to modification of an ongoing session’s configuration (including session stop) is provided with an explicit notification from the network (provided that RAN1 confirms a separate bit for this purpose can be accommodated in the MCCH change notification DCI, in addition to a bit for session start notification). FFS on whether this notification can be reused for modification of other information carried by MCCH, if any. - FFS whether the possibility of UE missing an MCCH change notification needs to be addressed or can be left to UE implementation. - At least in case RAN1 decides to utilize RNTI other than MCCH-RNTI for MCCH change notification, MCCH change notification is sent in the first MCCH monitoring occasion of each MCCH repetition period. - We support single MCCH (in this release) - MCCH is mapped to the DL-SCH for NR MBS delivery mode 2.. RAN2 would like RAN1 to take these agreements into account when discussing PHY layer aspects of MCCH design (in particular for RNTI and DCI design for carrying the MCCH change notifications), in addition to the agreements RAN2 informed earlier in R2-2104639. # 2 Actions To RAN1 group: ACTION: RAN2 respectfully asks RAN1 to take RAN2 agreements into account when discussing PHY layer aspects of MCCH. # 3 Dates of next RAN2 meetings TSG-RAN2 Meeting #115-e August 16 – August 27, 2021 Online TSG-RAN2 Meeting #116-e November 01 – November 12, 2021 Online