3GPP TSG RAN WG1 Meeting #107-e							R1-211xxxx

e-Meeting, November 11 – 19, 2021

Title:	[Draft] LS on RA-RNTI and MSGB-RNTI for 480 and 960 kHz

Response to:

Release:	Rel-17

Work Item:	NR_ext_to_71GHz

Source:	Intel Corporation [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Daewon Lee

E-mail Address:	daewon.lee (at) intel (dot) com,

Attachments:

1. Overall Description:

RAN1 would like to let RAN2 be aware that the RAN1 has made the following agreements regarding updates required for RA-RNTI and MSGB-RNTI for 480 kHz and 960 kHz based Random Access procedure.

- For 480kHz and 960kHz PRACH, reuse the RA-RNTI and MSGB-RNTI formula as FR2 and express the slot indexes t_id based on 120kHz SCS:

  - RA-RNTI =1+s_id+14×t_id+14×80×f_id +14×80×8×ul_carrier_id

  - MSGB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2

    - where the subcarrier spacing to determine t_id is based on the value of µ specified in clause 5.3.2 in TS 38.211 [8] for µ = {0, 1, 2, 3}

    - and for µ = {5, 6}, t_id is the index of the first 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80)

  - Note: As per previous RAN1 agreement, there is only one 480 or 960 kHz PRACH slot in a 120kHz slot, such that RA-RNTI and MSGB-RNTI does not result in ID collision.

RAN1 would like to kindly ask RAN2 to update appropriate RAN2 specifications based on RAN1 agreements listed above.

2. Actions:

To RAN4: 	RAN1 would like to kindly ask RAN2 to update appropriate RAN2 specifications based on RAN1 agreements listed above.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #107-bis-e 	17 – 25 Jan 2022			Online

TSG-RAN WG1 Meeting #108-e 	21 Feb – 04 Mar 2022		Online