3GPP TSG RAN WG1 Meeting #107	R1-21xxxxx

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.9

Source:	Moderator (Huawei)

Title:	Feature lead summary on 107-e-R17-RRC-NB-IoT-eMTC

Document for:	Discussion and Decision

Introduction

In email discussion of RAN1#106bis-e, there were discussion on RRC parameters for the WI [1]. And the list of stable rows was endorsed in [2].

This documents provides the proposals and summary of discussions of the corresponding email discussion on RRC parameters.

[106bis-e-R17-RRC-NB-IoT-eMTC] Email discussion on Rel-17 RRC parameters for Rel-17 NB-IoT and eMTC – Yubo (Huawei)

The RRC parameter list is also attached: RAN1#107-e_Rel-17_RRC_NB-IoT_eMTC_v2.xlsx.

Discussion

Support of 16-QAM for unicast in UL and DL for NB-IoT

### Issue 1-1: Configuration of PUR

For the configuration of NPUSCH for PUR, the two rows below are marked as stable.

| NB_IOTenh4_LTE_eMTC6 | 16QAM for NB-IoT | 36.211, 36.213 |  |  |  | enable16QAM-ul in PUR-config-NB | New | enable 16QAM for NPUSCH in PUR procedure | Enable/disable the use of 16QAM for NPUSCH in PUR procedure | Enable | disable | Per UE | UE specific | 36.331 | According to the following agreement, the configurations are up to RAN2, including the MCS indices, RU indices and UL power control parameter: Agreement To support 16-QAM for NPDSCH and NPUSCH in PUR procedure, • 16-QAM can be enabled/disabled by UE specific RRC signaling for NPDSCH and NPUSCH separately • The corresponding configurations and signaling details are up to RAN2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NB_IOTenh4_LTE_eMTC6 | 16QAM for NB-IoT | 36.211, 36.213 |  |  |  | enable16QAM-dl in PUR-config-NB | New | enable 16QAM for NPDSCH in PUR procedure | Enable/disable the use of 16QAM for NPDSCH in PUR procedure | Enable | disable | Per UE | UE specific | 36.331 | According to the following agreement, the configurations are up to RAN2, including the configuration of power allocation: Agreement To support 16-QAM for NPDSCH and NPUSCH in PUR procedure, • 16-QAM can be enabled/disabled by UE specific RRC signaling for NPDSCH and NPUSCH separately • The corresponding configurations and signaling details are up to RAN2 |

Please input your comments for the above proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | We are Ok with it. |
| Qualcomm | OK |
| Nokia, NSB | OK |
| Lenovo, MotoM | OK |
| ZTE, Sanechips | OK |

### Issue 1-2: Configuration for downlink power allocation

For the following rows for configuration of power allocation:

| NB_IOTenh4_LTE_eMTC6 | 16QAM for NB-IoT | 36.213 |  |  |  | NPDSCH-NRS-PowerRatio | New | Power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS | the Power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS for standalone and guard-band deployments, or in symbols without NRS nor CRS for in-band deployments | FFS |  | Per UE | UE specific | 36.331 | Confirm working assumption: Working Assumption For downlink power allocation to support 16QAM: • For standalone and guard-band deployments: o One power ratio is signaled optionally § NPDSCH EPRE to NRS EPRE in symbols without NRS o The same transmit power is assumed across different symbols. o If the signalling is not indicated, the legacy power allocation is used. § i.e., the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports • UE specific signalling is used |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NB_IOTenh4_LTE_eMTC6 | 16QAM for NB-IoT | 36.213 |  |  |  | NPDSCH-NRS-PowerRatio-withCRS | New | Power ratio of NPDSCH EPRE to NRS EPRE in symbols with CRS | the Power ratio of NPDSCH EPRE to NRS EPRE in symbols with CRS for inband deployments | FFS |  | Per UE | UE specific | 36.331 | Agreement Confirm the following working assumption: For downlink power allocation to support 16QAM: • For inband deployments, a power ratio is signaled in addition to the signalling for standalone and guard-band deployments which in this case applies to “symbols with NRS” and “symbols without NRS nor CRS”. o the power ratio between NPDSCH EPRE and NRS EPRE in symbols with CRS is signalled o the signalling is UE specific Note: “symbols with NRS” and “symbols without NRS nor CRS” have the same power. |

There is following proposal

| Sourcing | Proposals |
| --- | --- |
| [3] | Proposal 6: The value range for RRC parameter power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS should be {-6, -4.77, -3, -1.77, 0, 1, 2, 3} dB. |

Please input your comments on the value range of the higher layer parameters for power allocation:

| Companies | Comments |
| --- | --- |
| Ericsson | Proposal 6 refers to the power ratio of “NPDSCH EPRE to NRS EPRE in symbols without NRS”, is the right understanding that the same set of values is being proposed to be used for “the Power ratio of NPDSCH EPRE to NRS EPRE in symbols with CRS”? |
| Qualcomm | We agree with Ericsson that it would be good to clarify the ranges for both types of symbols. |
| Nokia, NSB | We are fine with the proposed range for NPDSCH EPRE to NRS EPRE in symbols without NRS |
| Lenovo, MotoM | We are fine with the proposed range for both types of symbols. (e.g., NPDSCH EPRE to NRS EPRE in all symbols without NRS) |
| ZTE, Sanechips | We are OK with the range clarification. |
| Moderator (Huawei) | Based on the comments, the proposal is updated as: Proposal: The value range for following RRC parameters is {-6, -4.77, -3, -1.77, 0, 1, 2, 3} dB - power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS - Power ratio of NPDSCH EPRE to NRS EPRE in symbols with CRS |
| Ericsson v009 | We are fine with the updated proposal, for which we suggest the following updates to make it consistent with the list in Excel: Proposal: The value range for the following RRC parameters is {-6, -4.77, -3, -1.77, 0, 1, 2, 3} dB - Power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS for standalone and guard-band deployments, or in symbols without NRS nor CRS for in-band deployments. - Power ratio of NPDSCH EPRE to NRS EPRE in symbols with CRS for in-band deployments. |
| Moderator (Huawei) | The update from Ericsson seems reasonable to me, therefore, the following is proposed: Proposal: The value range for the following RRC parameters is {-6, -4.77, -3, -1.77, 0, 1, 2, 3} dB - Power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS for standalone and guard-band deployments, or in symbols without NRS nor CRS for in-band deployments. - Power ratio of NPDSCH EPRE to NRS EPRE in symbols with CRS for in-band deployments. In addition, the value ranges in the excel file for the corresponding parameters are also updated accordingly. |

### Issue 1-3: Configuration for uplink power control

It has been agreed as below:

Agreement

The following working assumption is confirmed.

For the new term  introduced for power control of NPUSCH,

- Reuse the LTE definition simplified for NB-IoT:  for  and  for , where  is given by higher layer parameter deltaMCS-Enabled, and  where K is the code block size.

- FFS: whether the new term applies to QPSK when configured with 16QAM, if it does not, whether an additional term is introduced to avoid jump between QPSK and 16QAM

Then the row for the configuration of uplink power control can be marked as stable, as below:

| NB_IOTenh4_LTE_eMTC6 | 16QAM for NB-IoT | 36.213 |  |  |  | deltaMCS-Enabled | New | Ks for uplink power control of NPUSCH | the parameter Ks of uplink power control of NPUSCH | {en0, en1}, where en0 corresponds to value 0 corresponding to state "disabled", en1 correponds to value 1.25 corresponding to "enabled" | en0 | Per UE | UE specific | 36.331 | Agreement The following working assumption is confirmed. For the new term ∆_(TF,c) introduced for power control of NPUSCH, Reuse the LTE definition simplified for NB-IoT: ∆_(TF,c) (i)=10log_10 ((2^(BPRE∙K_s )-1)) for K_s=1.25 and ∆_(TF,c) (i)=0 for K_s=0, where K_s is given by higher layer parameter deltaMCS-Enabled, and BPRE=K/N_RE where K is the code block size. FFS: whether the new term applies to QPSK when configured with 16QAM, if it does not, whether an additional term is introduced to avoid jump between QPSK and 16QAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Please input your comments on the above row:

| Companies | Comments |
| --- | --- |

### Issue 1-4: Others

Please input your comments on issues other than the above ones:

| Companies | Comments |
| --- | --- |
| Ericsson | None |

Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC

Please input your comments in below table:

| Companies | Comments |
| --- | --- |
| Ericsson | None |

Support a maximum DL TBS of 1736 bits as a Rel-17 optional UE capability

Please input your comments in below table:

| Companies | Comments |
| --- | --- |
| Ericsson | Due that RAN2 has agreed to support PUR for this feature, we will need to add a parameter to Enable/Disable it (Similar to what has been done for 16-QAM). |
| Moderator (Huawei) | There was discussion a bit in last meeting (R1-2108684). The understanding was that if eNB wants to enable max DL TBS, it just schedules TBS larger than 1000 bits as long as UE indicates support it. The support of max DL TBS of 1736 bits has no impact on DCI, therefore, there’s no ambiguity between eNB and UE without the configuration. |
| Ericsson v009 | Thanks for the clarification, we are ok with the approach described above. |

Summary

- References

- R1-2110650 Feature lead summary on 106bis-e-R17-RRC-NB-IoT-eMTC, Moderator (Huawei), RAN1#106bis-e, Oct., 11th-19th, 2021.

- R1-2110572	Consolidated higher layers parameter list for Rel-17 LTE, Moderator (Ericsson), RAN1#106bis-e, Oct., 11th-19th, 2021.

- R1-2110857	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2111070	Discussion on 16QAM for NB-IoT	ZTE, Sanechips

- R1-2111133	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2111449	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2112001	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2112300	Discussion on CQI table and NPUSCH power control parameter for 16QAM	MediaTek Inc.

- R1-2112361	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2110858	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

- R1-2111071	Remaining issues on 14-HARQ processes in DL for eMTC	ZTE, Sanechips

- R1-2111134	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

- R1-2111450	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

- R1-2112362	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson

- R1-2111939	Further considerations on Rel-17 NB-IoT and eMTC enhancements	Huawei, HiSilicon

- R1-2112363	On the support of 16-QAM for unicast in UL and DL in TDD NB-IoT	Ericsson