3GPP TSG-RAN WG1 Meeting #107bis-e	Tdoc R1-2200400

e-Meeting, 17th – 25th January, 2022

Agenda Item:	8.2.1

Source:	Ericsson

Title:	Initial Access Aspects

Document for:	Discussion, Decision

# Introduction

In this contribution, initial access aspects related to SS/PBCH block and PRACH are discussed.

# DBTW applicability to licensed operation

During the email discussion post RAN1#107-e on the CR to 38.213, it was debated whether or not the following paragraph from Section 13 [1] should be applicable for operation without shared spectrum channel access.

For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. - Table 4.1-2: Mapping between the combination of subCarrierSpacingCommon and spare to NSSBQCL for operation with shared spectrum channel access in FR2-2 subCarrierSpacingCommon; spare; NSSBQCL. scs15or60; 0; 16. scs15or60; 1; 32. scs30or120; 0; 64. scs30or120; 1; reserved.

In RAN1#107e the following agreement was made:

Agreement - Same NSSBQCL values using the same set of signaling bits are supported for 120, 480, and 960 kHz. - Supported values of NSSBQCL: {16, 32, 64} - Note: - For operation with shared spectrum channel access, any supported value of NSSBQCL can be indicated and value < 64 indicates DBTW enabled - UE is expected to be configured with NSSBQCL=64 in licensed operations - For operation with and without shared spectrum channel access, NSSBQCL=64 indicates that the SS/PBCH block index and the candidate SS/PBCH block index have a one-to-one mapping relationship.

For operation without shared spectrum channel access, the UE is expected to be configured with NSSBQCL=64, which effectively disables the DBTW mechanism. Thus, in our view it does not make sense to have the above paragraph related to DBTW procedures apply to operation without shared spectrum channel access. TS 38.213 already contains a paragraph covering the case of operation without shared spectrum channel access as follows:

For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index.

Based on this we make the following proposal:

RAN1 to conclude that the DBTW procedure in 38.213 is not applicable to operation without shared spectrum channel access and no further spec changes are needed for that case.

# 3	ANR procedure in FR2-2

In 38.213 Section 13 [1], there is an open issue regarding if the special ANR procedure introduced for shared spectrum channel access in Rel-16 is also applicable to FR2-2. There is no RAN1 agreement to support this special ANR procedure for FR2-2. In addition, in our view, the Rel-16 solution is not feasible for the FR2-2 range due to that there is no fixed channel bandwidth defined. Thus, we make the following proposal and associated text proposal

Adopt TP#1 to correct Section 13 in 38.213 so that the Rel-16 ANR procedure for shared spectrum channel access is not applicable to FR2-2.

>>> Text Proposal (TP#1) for 38.213, Section 13 >>>

*** Unchanged text omitted ***

For operation without shared spectrum channel access and in FR2-2, a UE assumes that the offset in Tables 13-1 through 13-10C is defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set from the smallest RB index of the CORESET for Type0-PDCCH CSS set to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block. The SCS of the CORESET for Type0-PDCCH CSS set is provided by subCarrierSpacingCommon for FR1 and FR2-1 and same as the SCS of the corresponding SS/PBCH block for FR2-2. In Tables 13-7, 13-8, and 13-10 kSSB is defined in [4, TS 38.211].

For operation with shared spectrum channel access in FR1, a UE determines an offset from a smallest RB index of the CORESET for Type0-PDCCH CSS set to a smallest RB index of the common RB overlapping with a first RB of the corresponding SS/PBCH block

-	according to the offset in Table 13-1A or Table 13-4A, if the frequency position of the SS/PBCH block corresponds to the GSCN of a synchronization raster entry as defined in [8-1, TS 38.101-1], and

-	according to a sum of a first offset and a second offset if the frequency position of the SS/PBCH block is provided by ssbFrequency in a measurement configuration associated with a reporting configuration providing reportCGI and does not correspond to the GSCN of a synchronization raster entry as defined in [8-1, TS 38.101-1], where

-	the first offset is provided in Table 13-1A or Table 13-4A, and

-	the second offset is determined as the offset from a smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block indicated in the measurement configuration to a smallest RB index of the common RB overlapping with the first RB of a SS/PBCH block hypothetically located at the GSCN of a synchronization raster entry, where the single synchronization raster entry is located in the same channel as the SS/PBCH block used for the shared spectrum channel access procedure, as described in [15, TS 37.213]

where the offsets are defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set that is same as the SCS of the corresponding SS/PBCH block.

*** Unchanged text omitted ***

>>> End Text Proposal >>>

# 3	Correction for PRACH

In 38.211 [2], Section 5.3.2 describes the procedure for OFDM baseband signal generation for PRACH for the case of the long sequence lengths L = 571 and 1151. However, the procedure is tied to the definition of RB sets defined in Clause 7 of 38.214 [3] which is only relevant for operation with shared spectrum channel access in FR1. RB sets are not relevant for FR2-2.

Adopt TP#2 and TP#3 to correct Section 5.3.2 of 38.211 and Section 7 of 38.214, respectively, so that the OFDM baseband signal generation for PRACH for sequence length L = 571 and 1151 is not dependent on the definition of RB sets which are not relevant for FR2-2.

>>> Text Proposal (TP#2) for 38.211, Section 5.3.2 >>>

*** Unchanged text omitted ***

5.3.2	OFDM baseband signal generation for PRACH

The time-continuous signal  on antenna port p for PRACH is defined by

sl(p,μ)t=k=0LRA-1ak(p,RA)ej2πk+Kk1+kΔfRAt-NCP,lRATc-tstartRA
K=ΔfΔfRA
k1=k0μ+NBWP,istart-Ngridstart,μNscRB-Ngridsize,μNscRB2+nRAstartNscRB+nRANRBRANscRBif LRA∈139, 839 or LRA∈571, 1151 in FR2-2NRB,UL,n0+nRAstart,μ-NRB,UL,n0start,μNscRBif LRA∈571, 1151 in FR1
k0μ=Ngridstart,μ+Ngridsize,μ2NscRB-Ngridstart,μ0+Ngridsize,μ02NscRB2μ0-μ

where  and

-	 is given by clause 6.3.3;

-	 is the subcarrier spacing of the initial uplink bandwidth part during initial access. Otherwise,  is the subcarrier spacing of the active uplink bandwidth part;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList;

-	 is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP during initial access. Otherwise,  is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

-	nRAstart is the frequency offset of the lowest PRACH transmission occasion in frequency domain with respect to physical resource block 0 of the active uplink bandwidth part. The quantity nRAstart is given by the higher-layer parameter msgA-RO-FrequencyStart if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213], otherwise by msg1-FrequencyStart as described in clause 8.1 of [5 TS 38.213];

-	 is the PRACH transmission occasion index in frequency domain for a given PRACH transmission occasion in one time instance as given by clause 6.3.3.2;

-	 is the number of resource blocks occupied and is given by the parameter allocation expressed in number of RBs for PUSCH in Table 6.3.3.2-1.

-	NRB,UL,nstart,μ is the start CRB index of uplink RB set n corresponding to the quantity RBn,ULstart,μ. The UE assumes that the RB set is defined as when the UE is not provided IntraCellGuardBandsPerSCS for an UL carrier as described in Clause 7 of [6, TS 38.214]

-	n0 is the index of the RB set which contains the lowest PRACH transmission occasion in frequency domain indicated by nRAstart. The UE may assume that nRAstart is configured such that each PRACH transmission occasion is fully contained within an RB set.

-	 and  are given by clause 6.3.3

-	NCP,lRA=NCPRA+n∙16κ where

-	for , n=0

-	for ΔfRA∈15,30,60,120,480,960kHz, n is the number of times the interval tstartRA,tstartRA+NuRA+NCPRATc overlaps with either time instance 0 or time instance  in a subframe

The starting position tstartRA of the PRACH preamble in a subframe (for ) or in a 60 kHz slot (for ΔfRA∈60,120,480,960kHz) is given by

where

-	the subframe or 60 kHz slot is assumed to start at t=0;

-	a timing advance value NTA=0 shall be assumed;

-	Nuμ and NCP,l-1μ are given by clause 5.3.1;

-	 shall be assumed for ∆fRA∈1.25, 5 kHz, otherwise the value of μ corresponds to ∆fRA∈15, 30, 60, 120, 480, 960 kHz and the symbol position  is given by

l=l0+ntRANdurRA+14nslotRA

where

-	 is given by the parameter "starting symbol" in Tables 6.3.3.2-2 to 6.3.3.2-4;

-	 is the PRACH transmission occasion within the PRACH slot, numbered in increasing order from 0 to  within a RACH slot where  is given Tables 6.3.3.2-2 to 6.3.3.2-4 for LRA∈139,571,1151 and fixed to 1 for ;

-	 is given by Tables 6.3.3.2-2 to 6.3.3.2-4;

-	 is given by

-	if ∆fRA∈1.25, 5, 15, 60 kHz, then

-	if ∆fRA∈30, 120 kHz and either of "Number of PRACH slots within a subframe" in Tables 6.3.3.2-2 to 6.3.3.2-3 or "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then nslotRA=1, otherwise nslotRA∈0,1

-	if ∆fRA∈480, 960 and -	the "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then nslotRA=7 for ∆fRA=480 kHz and nslotRA=15 for ∆fRA=960 kHz

-	the "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 2, then nslotRA∈3,7 for ∆fRA=480 kHz and nslotRA∈7,15 for ∆fRA=960 kHz.

*** Unchanged text omitted ***

>>> End Text Proposal >>>

>>> Text Proposal (TP#3) for 38.214, Section 7 >>>

*** Unchanged text omitted ***

7	UE procedures for transmitting and receiving on a carrier with intra-cell guard bands

For operation with shared spectrum channel access in FR1, when the UE is configured with any of IntraCellGuardBandsPerSCS for UL carrier and for DL carrier with SCS configuration μ, the UE is provided with  NRB-set,x-1 intra-cell guard bands on a carrier with μ, each defined by start CRB and size in number of CRBs, GB s,xstart,μ  and GB s,xsize,μ , provided by higher layer parameters startCRB and nrofCRBs, respectively, where s∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. The UE does not expect that nrofCRBs is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. The UE determines the start and end CRB indices for s∈0,1,…,NRB-set,x-1 as

RB s,xstart,μ=Ngrid,xstart,μ+0s=0GB s-1,xstart,μ+GB s-1,xsize,μotherwise

and

RB s,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1s=NRB-set,x-1GB s,xstart,μ-1otherwise

*** Unchanged text omitted ***

>>> End Text Proposal >>>

# 5	Higher Layer Parameters Related to Initial Access

## 5.1	PRACH Root Sequence Index and Sequence Length for 2-Step RACH

The following parameter signaled via SIB1 is captured in the RRC parameter spreadsheet sent to RAN2 after RAN1#107-e [4]. This parameter is used to configure the PRACH root sequence index and the PRACH sequence length (139, 571, or 1151).

| Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- |
| SSB and RACH | prach-RootSequenceIndex-r16 | existing | May not need to change the IE, but need to add in the note on the limitation to be used with SCS. Field description requires updating to capture that L = 1151 is not supported for SCS 480 and 960 kHz and L = 571 is not supported for 960 kHz. | CHOICE { l571 INTEGER {0..569}, l1151 INTEGER {0..1149}} | RACH-ConfigCommon | Cell-specific |

In our understanding, the RAN1 agreements related to the above parameter apply to both 4-step and 2-step RACH; however, the spreadsheet contains a parameter only related to 4-step RACH.

Add the parameter msgA-PRACH-RootSequeceIndex for configuring the root sequence index and sequence length for 2-step RACH to the higher layer parameter spreadsheet and send update to RAN2.

| Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- |
| SSB and RACH | msgA-PRACH-RootSequeceIndex-r16 | existing | May not need to change the IE, but need to add in the note on the limitation to be used with SCS. Field description requires updating to capture that L = 1151 is not supported for SCS 480 and 960 kHz and L = 571 is not supported for 960 kHz. | CHOICE { l571 INTEGER {0..569}, l1151 INTEGER {0..1149}} | RACH-ConfigCommonTwoStepRA-r16 | Cell-specific |

## 5.2	Values for SSB-PositionQCL-Relation

In Rel-16, the information element SSB-PositionQCL-Relation is defined in 38.331 as follows:

SSB-PositionQCL-Relation

The IE SSB-PositionQCL-Relation is used to indicate the QCL relationship between SSB positions on the frequency indicated by ssbFrequency (see TS 38.213 [13], clause 4.1) for operation with shared spectrum channel access. Value n1 corresponds to 1, value n2 corresponds to 2 and so on.

SSB-PositionQCL-Relation information element

-- ASN1START

-- TAG-SSB-POSITIONQCL-RELATION-START

SSB-PositionQCL-Relation-r16 ::=  ENUMERATED {n1,n2,n4,n8}

-- TAG-SSB-POSITIONQCL-RELATION-STOP

-- ASN1STOP

This information element is used to configure Q for RRM measurements within the following IEs:

- SIB2

- SIB3

- SIB4

- MeasObjectNR

- ServingCellConfigCommon

As can be seen above, the supported values for SSB-PositionQCL-Relation are only {1,2,4,8} which are applicable for operation with shared spectrum channel access in FR1. However, for operation in FR2-2, a different set of values are applicable: specifically {16, 32, 64} according to the following RAN1 agreement:

Agreement

- Same NSSBQCL values using the same set of signaling bits are supported for 120, 480, and 960 kHz.

- Supported values of NSSBQCL: {16, 32, 64}

  - Note:

    - For operation with shared spectrum channel access, any supported value of NSSBQCL can be indicated and value < 64 indicates DBTW enabled

    - UE is expected to be configured with NSSBQCL=64 in licensed operations

    - For operation with and without shared spectrum channel access, NSSBQCL=64 indicates that the SS/PBCH block index and the candidate SS/PBCH block index have a one-to-one mapping relationship.

Clearly, the value range needs to be extended, or a new information element needs to be defined, to allow configuration of values 16, 32, or 64 for operation with shared spectrum channel access in FR2-2.

Inform RAN2 that the either the value range of the information element SSB-PositionQCL-Relation-r16 needs to be extended, or a new Rel-17 IE need to be defined to allow configuration of Q = 16, 32, or 64 in SIB2, SIB3, SIB4, MeasObjectNR, and ServingCellConfigCommon for RRM measurements when operating with shared spectrum channel access in FR2-2.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to conclude that the DBTW procedure in 38.213 is not applicable to operation without shared spectrum channel access and no further spec changes are needed for that case.

Proposal 2	Adopt TP#1 to correct Section 13 in 38.213 so that the Rel-16 ANR procedure for shared spectrum channel access is not applicable to FR2-2.

Proposal 3	Adopt TP#2 and TP#3 to correct Section 5.3.2 of 38.211 and Section 7 of 38.214, respectively, so that the OFDM baseband signal generation for PRACH for sequence length L = 571 and 1151 is not dependent on the definition of RB sets which are not relevant for FR2-2.

Proposal 4	Add the parameter msgA-PRACH-RootSequeceIndex for configuring the root sequence index and sequence length for 2-step RACH to the higher layer parameter spreadsheet and send update to RAN2.

Proposal 5	Inform RAN2 that the either the value range of the information element SSB-PositionQCL-Relation-r16 needs to be extended, or a new Rel-17 IE need to be defined to allow configuration of Q = 16, 32, or 64 in SIB2, SIB3, SIB4, MeasObjectNR, and ServingCellConfigCommon for RRM measurements when operating with shared spectrum channel access in FR2-2.

# References

3GPP TS 38.213, "Physical layer procedures for control," v17.0.0, December 2021.

3GPP TS 38.211, "Physical channels and modulation," v17.0.0, December 2021.

3GPP TS 38.214, "Physical layer procedures for data," v17.0.0, December 2021.

R1-2112976, "Consolidated higher layers parameter list for Rel-17 NR," Ericsson, RAN1#107-e, November 2021.