3GPP TSG RAN WG1 #107bis-e	    	     					R1-2200483

e-Meeting, January 17th -25th, 2022

Agenda Item:	8.15.7

Source:	Ericsson

Title:	UE features for UEPS

Document for:	Discussion/Decision

# 1	Introduction

This document discusses UE features related to Rel-17 UE power savings work item.

# 2	Discussion and proposals

Below are some of the main changes proposed for the FGs (using track changes in below table).

- For FG 29-1 (PEI),

  - It was agreed in last RAN1 meeting that it is left to RAN2 to decide whether capability signalling is introduced or not for FG 29-1. It may be helpful if RAN1 can also indicate the reporting granularity (as per UE or per Band) in case ‘optional with capability signalling’ is identified as essential by RAN2. The granularity can be per UE or at most per Band, latter is captured in proposal 1.

- For FG 29-2 (TRS occasions),

  - ‘Consequence column’: The current sentence (Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode) should be removed. OK to add “UE does not support TRS occasions for idle/inactive UEs” or it can be left empty.

  - Allowing optional UE capability signalling can be useful for NW to know when to turn on these features, but it is not essential to have capability signalling for this or any additional separate capabilities (for reception of L1 signalling). TRS occasion configuration and L1 availability configuration is not UE-specific. Idle/Inactive UEs can ignore any TRS occasion-related information they are not interested in/capable of receiving. If ‘optional with capability’ signalling is identified as essential, it should be per UE granularity or at most per Band, latter is captured in proposal 1.

- For FG29-3 (PDCCH monitoring adaptation)

  - ‘Consequence column’ can be left empty – there is no need to say that the feature is not supported as a consequence.

  - We are OK to add FGs 29-3c and FG 29-3d.

  - The FGs should be per UE or at most per Band, latter is captured in proposal 1.

Based on above discussion, below are proposed updates to the UEPS FG table.

Proposal 1: Update the UE feature list for UEPS as shown in below Table.

# 29. NR_UE_pow_sav_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  |  |  | UE does not support paging enhancement | Per Band | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability |  | N |  | UE does not support TRS occasions for idle/inactive UEs | Per Band | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI for up to M= 3 skipping durations if SSSG is not configured |  | Y |  |  | Per Band | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per Band | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based SSSG switching | 29-3b | Y |  |  | Per Band | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based SSSG switching | 29-3a, 29-3b | Y |  |  | Per Band | N | N | N |  | Optional with capability signaling |

# References

[1] R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e	Moderators (AT&T, NTT DOCOMO, INC.) RAN WG1 #107-e, Nov 2021.

[2] R1-2109713	Summary on UE features for UE power saving enhancements	Moderator (NTT DOCOMO, INC.), RAN WG1#106bis-e, Oct 2021.

[3] R1-2112139	Summary on UE features for UE power saving enhancements	Moderator (NTT DOCOMO, INC.) , RAN WG1#107-e, Nov 2021.