3GPP TSG RAN WG1 #109-e		R1-2203320

e-Meeting, May 9th – 20th, 2022

Agenda Item:     9.1.1.1

Source:	Spreadtrum Communications

Title:	Discussion on unified TCI framework extension for multi-TRP

Document for:	Discussion

Introduction

In RAN #94 meeting，the following objectives on unified TCI framework for m-TRP have been agreed [1].

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we provide some discussion on the unified TCI framework extension for indication of multiple DL/UL TCI states and power control for UL single DCI.

Discussion

Extension for indication of multiple DL/UL TCI state

In Rel-17, the unified TCI framework has been defined, and beams for downlink and uplink transmission are both based on TCI state indication. DL TCI state and UL TCI state can be used for DL and UL transmission, respectively. Besides, with the assumption of beam correspondence, a joint TCI state can be used for DL reception and UL transmission. Specifically, whether a UE is to operate with joint TCI or separate DL/UL TCI is based on RRC signaling.

In order to achieve the unified framework for TCI state indication of data and control channel for DL and UL, a TCI state indication mechanism is introduced in Rel-17 as shown in Figure 1. The TCI state indication mechanism can be summarized as: up to 2 TCI state lists are configured by RRC signaling first, and then MAC CE is used to activate some of the TCI states and map each of them with a codepoint of TCI field in DCI. Finally, DCI is utilized to indicate the TCI states corresponding to one codepoint. The target channel/RS in unified TCI state is given as follows.

- For DL transmission: UE dedicated reception on all CORESET, PDSCH and aperiodic CSI-RS (for beam management and CSI);

- For UL transmission: UE dedicated transmission on all PUCCH, PUSCH, and periodic/semi-periodic/aperiodic SRS (for beam management, codebook based transmission, non-codebook based transmission and antenna switching).

Figure 1. Structure for unified TCI framework

In Rel-18, the objective of unified TCI is to perform the indication of multiple DL/UL TCI states in m-TRP operation mode. Regarding the extension of functionality of Rel-17 TCI framework, there comes two questions that need to be solved:

- Question 1: How to perform the multiple DL and UL TCI indication in m-TRP operation mode?

- Question 2: Which TRP is the activated TCI state applied for?

Firstly, for the reply to Question 1, the scenarios of S-DCI based and m-DCI based TCI state indication are discussed, respectively.

For s-DCI m-TRP transmission, activation of two different TCI states has been supported in Rel-16, in addition, these two TCI states can be simultaneously indicated by one codepoint in MAC CE. To support the multiple simultaneously activate UL and DL TCI states in one instance of DCI in m-TRP scenario, the existing MAC CE format could be enhanced. So we propose that:

Proposal 1: Multiple DL and UL TCI states for multi-TRP can be simultaneously indicated by one TCI codepoint in MAC CE. And the value set of (M, N)∈{(1, 1), (2, 0), (0, 2), (1, 2), (2, 1), (2, 2)}.

Figure 2. Example of enhanced codepoint in MAC CE for TCI state indication

Figure 2 shows an example of the enhanced MAC CE, including both separate TCI mode and joint TCI mode. In separate TCI mode,  MAC CE activate one codepoint containing two pairs of UL&DL TCI state, each of the TCI state pairs corresponds to a TRP.  Note that the number of the TCI states that mapping to a codepoint could be set to 1, 2, 3 or 4, and satisfying the constraint of M≤2, N≤2. In joint TCI mode, similar to the mechanism in Rel-16, up to two joint TCI states can be mapped to one codepoint, one for each TRP.

Similarly, for m-DCI m-TRP transmission, one or two UL and/or DL TCI state(s) in a codepoint can be selected by DCI, so that simultaneous multiple TCI states indication is achieved.

With respect to Question 2, in m-DCI based m-TRP transmission, m-DCI based m-TRP PDSCH transmission is supported in Rel-16 and a higher layer parameter CORESETPoolIndex is configured for each CORESET. CORESETPoolIndex value associated with TCI states can be used to determine which TRP the TCI state should be applied for, and illustration is presented in Figure 3.From our view, CORESETPoolIndex can also be used for TRP identification in Rel-18 with minimal specification impact.

Figure 3. Example of TCI state indication for m-DCI based m-TRP transmission

Observation 1：For m-DCI based m-TRP transmission, CORESETPoolIndex can be reused to determine the association between TCI state and TRP in Rel-18.

Furthermore, we provide our discussion on a general TCI indication scheme. The general scheme can be used in both s-DCI scenario and m-DCI scenario, two possible methods are discussed as follows:

In a direct way, a RRC parameter which is used to identify different TRPs explicitly, e.g., “TRP-like ID”, can be introduced in RRC signaling, so that the association between TCI state and TRP is determined. Note that the RRC parameters could be associated with or included in TCI state.

Figure 4. Illustration of the association among TCI state, channel/RS group and TRP

Alternatively, the association between TCI state and TRP can also be determined implicitly. In our understanding, the core of the scheme is to introduce two kinds of association, i.e., the association between TCI state and channel/RS group and the association between TCI state and TRP. Figure 4 shows an illustration of the association among TCI state, channel/RS group and TRP. For the first association, multiple channels/RSs can be grouped flexibly, and each channel/RS in the same group shares the same TCI state. The grouping of channels/RSs is reasonable because the channels/RSs probably are propagated through almost the same path, and the same TCI state can be shared.  For the second association, the association between TRP and channel/RS group could be determined implicitly. In this way, different channel/RS groups associated with different TRPs, and activated TCI states are associated with corresponding channel/RS groups according to the target channel/RS in TCI state. It is an effective way to indicate the target TRP for activated TCI state, since the association among TRP, TCI state and channel/RS group is established implicitly. In summary, we propose that:

Proposal 2: To enable m-TRP based beam indication, the following two candidate functionalities should be considered:

- Explicit indicator like “TRP ID” could be introduced in RRC configuration, the parameter is associated with (or included in) TCI state.

- Multiple channels/RSs can be grouped flexibly and each channel/RS group is associated with a TRP.

In Rel-18, we also support simultaneous multi-panel UL transmission, e.g., PUSCH+PUSCH or PUCCH+PUCCH. These channels’ beam indication associated with different TRPs should be clarified. From our perspective, UE can find the corresponding beam information in the m-TRP based unified TCI. There is no strong motivation of doing further optimization.

Observation 2:  Very small specification impact is expected to support simultaneous multi-panel UL transmission for UL beam indication.

Power control for UL single DCI

In Rel-18, power control enhancement is necessary for the support of simultaneous transmission of multiple UL channels. Within legacy power control framework, UE has individual maximum power limitation on CC-level, e.g., PCMAX,f,c(i) and the overall power limitation PCMAX(i). For UEs supporting simultaneous transmission of 2 UL channels, they can be classified into two categories as below:

- Type-1: All simul-trans UL channels share same PCMAX,f,c(i) and same PCMAX(i).

- Type-2: All simul-trans UL channels share individual PCMAX,f,c(i) and individual PCMAX(i).

Type-1 has the higher complexity of power control.  From our view, both types should be thoroughly investigated in Rel-18. Accordingly, the PHR concerning simultaneous transmission of 2 UL channels should also be enhanced.

Proposal 3: The following UE types should be studied in Rel-18:

- Type-1: All simul-trans UL channels share same PCMAX,f,c(i) and same PCMAX(i).

- Type-2: All simul-trans UL channels share individual PCMAX,f,c(i) and individual PCMAX(i).

Proposal 4: The PHR enhancement for simultaneous transmission of 2 UL channels should also be studied in Rel-18.

Conclusion

In this contribution, we gave our views on unified TCI framework for m-TRP and power control for UL single DCI. The following observations and proposals are achieved:

Observation 1：For m-DCI based m-TRP transmission, CORESETPoolIndex can be reused to determine the association between TCI state and TRP in Rel-18.

Observation 2:  Very small specification impact is expected to support simultaneous multi-panel UL transmission for UL beam indication.

Proposal 1: Multiple DL and UL TCI states for multi-TRP can be simultaneously indicated by one TCI codepoint in MAC CE. And the value set of (M, N)∈{(1, 1), (2, 0), (0, 2), (1, 2), (2, 1), (2, 2)}.

Proposal 2: To enable m-TRP based beam indication, the following two candidate functionalities should be considered:

- Explicit indicator like “TRP ID” could be introduced in RRC configuration, the parameter is associated with (or included in) TCI state.

- Multiple channels/RSs can be grouped flexibly and each channel/RS group is associated with a TRP.

Proposal 3: The following UE types should be studied in Rel-18:

- Type-1: All simul-trans UL channels share same PCMAX,f,c(i) and same PCMAX(i).

- Type-2: All simul-trans UL channels share individual PCMAX,f,c(i) and individual PCMAX(i).

Proposal 4: The PHR enhancement for simultaneous transmission of 2 UL channels should also be studied in Rel-18.

Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94, Samsung (Moderator)