3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2204107

e-Meeting, 9th – 20th May, 2022

Agenda Item:	9.3.2

Source:	Ericsson

Title:	Sub-band non-overlapping full duplex

Document for:	Discussion, Decision

# 1	Introduction

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

The detailed objectives are as follows:

- Identify applicable and relevant deployment scenarios (RAN1).

- Develop evaluation methodology for duplex enhancement (RAN1).

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

  - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

- Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

- Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

The Rel-18 study item on evolution of NR duplex operation contains the following list of objectives, with the RAN1 objectives highlighted in yellow.

One of the important aspects of the study is impact on legacy operation. It is expected that when subband full duplex (SBFD) is deployed, both legacy UEs and SBFD capable UEs would co-exist in the same cell for the foreseeable future. Hence, it is important to study how SBFD can be deployed in a way that would satisfy both of the following goals jointly:

- Enable benefits for SBFD capable UEs in terms of improved UL coverage and/or improved UL latency

  - For convenience of presentation, we refer to "SBFD capable UEs" as "Rel-18 UEs" despite that no normative work on SBFD is anticipated in Rel-18.

- Allow legacy UEs (Rel-17 and prior) to operate in a cell where SBFD is deployed without mandating that new Rel-18 features related to SBFD are implemented.

In this paper, we discuss various aspects of a baseline solution that satisfies these goals jointly. Here we do not focus on performance; that is treated in our companion contribution [1]. Instead, we focus on configuration and operation of a serving cell with SBFD from both a Rel-18 and legacy UE perspective.

# 2	Assumptions for Baseline Solution

Here we discuss some assumptions for a baseline SBFD solution for a serving cell

- SBFD operation (simultaneous gNB transmission/reception) is within a carrier

  - While it is within scope of the SI, for now, we do not focus on simultaneous transmission/reception on different (co-sited) carriers

- A baseline time domain TDD UL/DL pattern is indicated cell-specifically with TDD-UL-DL-ConfigCommon. Thus, the baseline time domain pattern is the same for all UEs in the cell, e.g., D-D-D-D-U

  - We assume that legacy UEs do not support flexible 'F' symbols in the cell-specific pattern, since initial deployments of NR employ static TDD. Hence all symbols in the time domain TDD UL/DL pattern would be configured either as 'D' or 'U'.

  - The U slot(s)/symbols in the time domain pattern is/are preserved for UL-only operation for both Rel-18 and legacy UEs. This is beneficial from the following perspectives:

    - Allows robust UL performance that is not impacted by CLI, assuming co-ordinated time domain TDD patterns amongst different gNBs

    - Allows legacy procedures to be reused for UL transmissions during initial access, i.e., RACH, PUSCH, PUCCH where UEs are likely unaware of the frequency domain pattern used for SBFD operation

    - Depending on the hardware/antenna architecture, facilitates reciprocity-based DL beamforming allowing for SRS reception on the same set of sub-carriers and antenna panel as will be used for DL transmission

  - Only symbols within 'D' slots can be configured for SBFD operation with frequency domain pattern D-U-D

    - The rationale that has been provided for SBFD is that it enables improved UL coverage and/or latency by enabling more UL opportunities compared to the baseline time domain TDD pattern. Hence, it is not motivated to configure a time domain U slot /symbol for SBFD operation, e.g., with frequency domain pattern U-D-U. Furthermore, we assume that all 'D' slots are configured for SBFD operation (no DL-only slots) in order to have persistent availability of UL resources to maximize coverage/latency improvements.

    - Rel-18 UEs are expected to receive either DL or transmit UL in the 'D' slots/symbols, but not simultaneously.

    - In contrast, legacy UEs are only expected to receive DL in one or both of the 'D' subbands since they do not understand that a 'D' slot/symbol can also be used for UL by Rel-18 UEs.

Figure 1 illustrates a carrier used for SBFD operation from a system perspective for the example of a time domain pattern D-D-D-D-U indicated by TDD-UL-DL-ConfigCommon.

Figure 1: Time/frequency domain pattern with SBFD from a system perspective.

From a UE perspective, a 'D' slot in the above time domain pattern would appear as shown  in Figure 2. A legacy UE would see the slot as configured for DL reception only, but the gNB would need to avoid scheduling/configuring DL transmission in the middle RBs for this UE. A legacy UE would be unaware that these RBs can be used for UL by Rel-18 UEs. This puts constraints on the configuration and scheduling of DL for legacy UEs, e.g., some signals/channels need to be scheduled/configured in only one of the 'D' subbands.

For Rel-18 UEs, the UE understands that a 'D' slot in the above pattern can be used for either DL reception or UL transmission (but not simultaneously). While the gNB would still need to avoid scheduling/configuring DL in the middle RBs like for legacy UEs, new behavior for certain signals/channels can be specified for making use of both 'D' subbands.

Figure 2: Time/frequency domain pattern from a UE perspective (legacy and Rel-18 UEs) within one of the 'D' slots.

A baseline SBFD time/frequency domain pattern should contain one or more UL-only slots. The remainder of the slots (configured as 'D' by TDD-UL-DL-ConfigCommon) can be configured for SBFD operation for new (Rel-18) UEs. A legacy UE sees slots/symbols configured for SBFD operation as DL-only slots.

# 3	Carrier and BWP Configuration

## 3.2	Carrier Configuration

In the baseline solution, we assume that legacy UEs expect to be configured with a cell-specific time domain TDD UL/DL pattern in which all slots/symbols are indicated with a direction ('D' or 'U'), but not flexible ('F'). As discussed in the previous section, only the 'D' slots/symbols are configured for SBFD operation with a frequency domain pattern, i.e., D-U-D. Such configuration is only needed for Rel-18 UEs and can be handled with dedicated signaling, for example, by extending TDD-DL-ULConfigDedicated to indicate frequency domain (FD) information. The FD info should include the subband configuration and potentially guardbands between the subbands to allow for filter rolloffs in the gNB and/or UE. Unlike prior releases where only 'F' symbols can be overridden, the dedicated signaling for Rel-18 UEs would be allowed to override a 'D' slot/symbol, i.e., converting it to D-U-D. With the baseline solution, dedicated signaling is not needed for legacy UEs. Those UEs would obtain the time domain pattern from TDD-UL-DL-ConfigCommon as usual.

One approach for extending TDD-DL-ULConfigDedicated is to introduce the notion of RB sets, potentially with guard bands in between. Such RB sets and guardbands are very similar to what was specified in Rel-16 for NR-U (see 38.214 Clause 7); however, probably more flexibility is needed to adjust the size of the RB sets for SBFD operation. Figure 3 shows two options for achieving such configuration. In Option 1, the starting RB and number of RBs is provided for each RB set and the guardband sizes are implicitly determined. The direction 'D' or 'U' is indicated for each RB set. In Option 2, it is recognized that since the dedicated signaling overrides a 'D' slot/symbol, then it is only necessary to indicate the size and position of the 'U' RB set and the size of the guardbands. The remaining RBs are then implicitly determined to be 'D' RB sets.

Figure 3: Options for RB set and guard band configuration provided by dedicated signaling that overrides a 'D' slot/symbol in TDD-UL-DLConfigCommon.

Figure 4 shows an example of the baseline solution where TDD-UL-DL-ConfigCommon is used to configure the time domain pattern for all UEs with only 'D' and 'U' slots/symbols. An extension of TDD-UL-DL-ConfigDedicated is used to configure the frequency domain pattern (RB sets) for Rel-18 UEs.

Figure 4: Example time domain pattern configured for all UEs; frequency domain pattern configured for Rel-18 UEs

For new (Rel-18) UEs, dedicated signaling should configure slots/symbols for SBFD operation. The frequency domain pattern can be configured in terms of RB sets with 'D' and 'U' direction (e.g., D-U-D) and guardbands between the RB sets.

## 3.1	BWP Configuration

### 3.1.1	Legacy UEs

In the baseline solution, a legacy UE sees the whole carrier bandwidth as either 'D' or 'U' based on the cell-specifically indicated time domain TDD pattern. As per current specifications, such a UE can be configured with a DL and UL BWP that spans a part or the whole of the carrier bandwidth, and the center frequencies of the UL and DL BWPs are aligned.

As will be discussed in a later section, if the DL BWP spans the whole carrier, both 'D' subbands can be used for DL signals/channels that support non-contiguous frequency domain resource allocation (FDRA) such as PDSCH scheduled by DCI 1_1 or DCI 2_1 and a PDCCH in a search space associated with a CORESET with non-zero index. For other signals that support only contiguous FDRA, e.g., SSB, CSI-RS, PDCCH in a search space associated with CORESET0, then only one of the two 'D' subbands within the BWP can be used.

In the baseline solution, legacy UEs are configured/scheduled UL transmission only in slots/symbols that are configured as 'U' by TDD-UL-DL-ConfigCommon, hence the UL BWP will always include contiguous RBs used only for UL. Hence, no special handling is needed to avoid a particular subband.

### 3.1.2	Rel-18 UEs

In contrast to legacy UEs, if a Rel-18 UE is configured with a DL BWP that spans the whole carrier, then it is within scope to consider enhancements for certain signals/channels to support non-contiguous FDRA in order to make better use of both 'D' subbands. In a later section, we discuss potential enhancements for CSI-RS which in current specifications is limited to only contiguous FDRA.

Also in contrast to legacy UEs, a Rel-18 UE can be configured/scheduled with UL transmission in either or both of a D-U-D slot or an UL-only slot, and the number of RBs available for UL transmissions is different in each case. Figure 4 illustrates the situation. In this diagram, we assume that a single UL BWP is configured spanning the whole carrier. During D-U-D slots, this BWP includes RBs which cannot be configured/scheduled for UL transmissions. However, these RBs can be avoided by configuration/scheduling based on existing specifications, at least in CONNECTED mode.

Figure 5: Available RBs for UL transmission within the UL BWP for a Rel-18 UE

According to the discussion in the previous section, a Rel-18 UE is unaware of D-U-D slots prior to entering CONNECTED mode, since the RB sets are configured via dedicated signaling only once the UE enters CONNECTED mode. For UL signals/channels transmitted during initial access (i.e., in IDLE mode), we assume that such UL transmissions are restricted to UL-only slots that are indicated by TDD-DL-UL-ConfigCommon in SIB1 (as in current specifications). This means that in the baseline solution, all UEs (legacy and Rel-18) enter the system using the UL-only slots for UL transmissions.

# 4	DL Signals and Channels

In this section we discuss configuration/scheduling of DL signals and channels for both legacy and Rel-18 UEs. We discuss CONNECTED and IDLE modes separately.

## 4.1	IDLE Mode

In the baseline SBFD solution, we assume that in IDLE mode, all UEs (legacy and Rel-18) are not aware that certain slots/symbols are configured as D-U-D. Such information is only made available to Rel-18 UES by dedicated signaling once the UE enters CONNECTED mode.

During initial access, the UE receives the following signals/channels, all of which are restricted in current specifications to have contiguous frequency domain resource allocation (FDRA):

- SS/PBCH Block

- PDCCH in Type0-CSS

- RMSI PDSCH

Hence, in an SBFD system, the gNB must ensure that these signals "fit" within one of the 'D' subbands in D-U-D slots/symbols. This puts a constraint on the maximum size of the 'U' RB for a given carrier bandwidth. The constraint is determined by investigating the bandwidth requirements of CORESET0 for various configurations. Here we analyse the maximum size of the 'U' RB set under the following assumptions:

- Symmetric D-U-D allocation ('D' subbands have equal bandwidth)

- Same subcarrier spacing for SS/PBCH block and CORESET0

  - 30 kHz SCS for FR1

  - 120 kHz SCS for FR2

- No guard bands between RB sets

- Minimum value of SSB-CORESET0 offset for each CORESET0 size according to tables in 38.213 Section 13 (allows for maximization of the size of the 'U' RB set)

Table 1 shows the maximum size of the UL RB set as a percentage of the total number of RBs in the transmission bandwidth under the constraint that both SSB and CORESET0 must fit into one of the 'D' subbands. The table can be summarized as follows, where "feasible" means that the maximum UL allocation corresponds to greater than 20% of the total number RBs:

- FR1 with 15 kHz SCS:

  - 40 MHz channel bandwidth is sufficient for feasibility of 24 and 48 RB CORESET0 (but not 96 RB)

- FR1 with 30 kHz SCS:

  - 100 MHz channel bandwidth is required for feasibility of 48 RB CORESET0

  - 40 MHz bandwidth is sufficient for feasibility of 24 RB CORESET0

- FR2 with 120 kHz SCS:

  - 400 MHz channel bandwidth is required for feasibility of:

    - 48 RB CORESET0 + Mux Pattern1

    - 24 RB CORESET0 + Mux Pattern 1 or 2

  - 200 MHz channel bandwidth is sufficient for feasibility of 24 RB CORESET0 + Mux Pattern 1 (but not Mux Pattern 2)

Table 1: Maximum % of RBs that can be configured for 'U' RB set for FR1 and FR2

| Frequency Range | SCS | Channel BW | Transmission BW (RBs) | Mux Pattern | CORESET0 Size (RBs) | Maximum UL Allocation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 kHz | 40 MHz | 216 | 1 | 24 | 69% |
|  |  |  |  |  | 48 | 36% |
|  |  |  |  |  | 96 | 0% |
|  | 30 kHz | 40 MHz | 106 | 1 | 24 | 45% |
|  |  |  |  |  | 48 | 0% |
|  |  | 100 MHz | 273 | 1 | 24 | 79% |
|  |  |  |  |  | 48 | 61% |
| 2 | 120 kHz | 100 MHz | 66 | 1 | 24 | 0% |
|  |  |  |  |  | 48 | 0% |
|  |  |  |  | 2 | 24 | 0% |
|  |  |  |  |  | 48 | 0% |
|  |  | 200 MHz | 132 | 1 | 24 | 26% |
|  |  |  |  |  | 48 | 0% |
|  |  |  |  | 2 | 24 | 0% |
|  |  |  |  |  | 48 | 0% |
|  |  | 400 MHz | 264 | 1 | 24 | 63% |
|  |  |  |  |  | 48 | 34% |
|  |  |  |  | 2 | 24 | 45% |
|  |  |  |  |  | 48 | 8% |

For both legacy and new (Rel-18) UEs in IDLE mode, DL reception (i.e., during initial access) in SBFD slots/symbols (configured as D-U-D) occurs only in one of the 'D' subbands. Assuming a certain minimum channel bandwidth with a symmetric D-U-D configuration in either FR1 or FR2, it is feasible to configure slots/symbols for SBFD operation such that CORESET0 "fits" within a single 'D' subband. For 48 RB CORESET0, 40/100/400 MHz channel bandwidth is required for 15/30/120 kHz SCS.

## 4.2	CONNECTED Mode

### 4.2.1	Legacy UEs

A legacy UE sees the whole carrier bandwidth as either 'D' or 'U' based on the cell-specifically indicated time domain TDD pattern TDD-UL-DL-ConfigCommon. In CONNECTED mode, if the active DL BWP spans the whole carrier, both 'D' subbands can be used for certain DL signals/channels that support non-contiguous frequency domain resource allocation (FDRA). These include the following:

- PDSCH scheduled by DCI 1_1 and DCI 2_1

  - DCI 1_1 and DCI 2_1 support Type0 FDRA in which a bitmap indicates the scheduled RBGs which can be non-contigous

- PDCCH in a search space associated with a CORESET other than CORESET0

  - The RRC parameter frequencyDomainResources within a CORESET configuration is a bitmap used to indicate groups of 6 contiguous RBs, and the groups can be non-contigous

For other signals/channels that support only contiguous FDRA then only one of the two 'D' subbands within the DL BWP can be used for legacy UEs.  These include the following

- SS/PBCH block

- PDSCH scheduled by DCI 1_0

  - DCI 1_0 only supports Type1 FDRA in which the start PRB and number of contiguous RBs are indicated

- CSI-RS

For legacy UEs in CONNECTED mode, DL reception of some channels may occur within both 'D' RB sets, while reception of others is restricted to a single 'D' RB set.

### 4.2.2	Rel-18 UEs

In contrast to legacy UEs, if a Rel-18 UE has an active DL BWP in CONNECTED mode than spans the whole carrier, then it is within scope to consider enhancements for certain signals/channels to support non-contiguous FDRA in order to make better use of both 'D' subbands. In current specifications, CSI-RS is limited to only contiguous FDRA. To enable non-contiguous FDRA, several possible enhancements exist.

One solution is to make use of the RRC configuration of RB sets as an implicit indication of RBs that are not available for DL transmission. This is illustrated in Figure 3 where based on either option, the 'U' RB set and guardbands are not available. Thus, a Rel-18 UE can be configured with a CSI-RS with contiguous FDRA (as in current specifications) that spans all RB sets of the carrier. The UE would understand that the RBs that intersect with the 'U' RB set and the guardbands are not available. This effectively results in a CSI-RS resource  with non-contiguous allocation in both of the 'D' RB sets. Similar handling is specified for Rel-16 NR-U with respect to RB set availability, but the Rel-16 behavior is to drop the whole CSI-RS if any RB set is unavailable (due to failed LBT). For SBFD, the UE would only drop the RBs that overlap the unavailable RBs, rather than the whole resource.

Another solution which allows for more configuration flexibility is to configure two CSI-RS resources, one per 'D' RB set, each with contiguous FDRA (as per current specications). The two CSI-RS resources can be treated as one effective CSI-RS resource by defining a separate ID to refer to the effective CSI-RS resource. This ID can be associated with the two legacy IDs by configuration. Alternatively, one could extend the current CSI-RS resource definition to explicitly configure non-contiguous FDRA in the two non-contigious 'D' RB sets.

For new (Rel-18) UEs in CONNECTED mode, DL reception of some channels may occur within both 'D' RB sets, while reception of others is restricted to a single 'D' RB set unless enhancements are made.

For new (Rel-18) UEs, CSI-RS should be enhanced to allow non-contiguous frequency domain resource allocation, i.e., in both 'D' RB sets.

# 5	UL Signals and Channels

## 5.1	IDLE Mode

In the baseline SBFD solution, we assume that in IDLE mode, all UEs (legacy and Rel-18) are not aware that certain slots/symbols are configured as D-U-D. Such information is only made available to Rel-18 UES by dedicated signaling once the UE enters CONNECTED mode. Hence during initial access, UL transmissions by all UEs (legacy and Rel-18) would occur within the UL-only slot(s) indicated by TDD-UL-DL-ConfigCommon. These transmissions include

- PRACH

- Msg3 PUSCH

- Cell-specifically configured PUCCH

For both legacy and new (Rel-18) UEs in IDLE mode, UL transmissions (i.e., during initial access) should occur only within UL-only slots.

## 5.2	CONNECTED Mode

### 5.2.1	Legacy UEs

A legacy UE in CONNECTED mode is not aware that certain slots/symbols are configured as D-U-D, same as for IDLE mode. Such a UE sees the whole carrier bandwidth as either 'D' or 'U'. Hence legacy UEs will be configured/scheduled only in the UL-only slot(s) indicated by TDD-UL-DL-ConfigCommon. Such UL transmissions can be configured/scheduled as per current specifications without changes. These signals include

- CG- and DG-PUSCH

- PUCCH resources configured by dedicated signaling

- SRS

- PRACH

In contrast to the DL where there is a need to restrict certain signals to only a single subband, no such constraints exist for legacy signals/channels in UL-only slots where all UL resources are inherently contiguous.

For legacy UEs in CONNECTED mode, UL transmissions should occur only within UL-only slots.

### 5.2.2	Rel-18 UEs

In CONNECTED mode, a Rel-18 UE can be configured/scheduled UL transmissions in either or both D-U-D slots or UL-only slots as shown in Figure 5. In either case, the UL RBs are contiguous, hence there is not a need to make enhancements to support non-contiguous FDRA, like for the DL. Hence current specifications can be reused for configuration/scheduling of the same set of signals/channels as listed in the previous section for legacy UEs.

For new (Rel-18) UEs in CONNECTED mode, UL transmissions should occur in either or both SBFD slots/symbols (configured as D-U-D) and UL-only slots/symbols. Current specifications can be reused since non-contiguous frequency domain resource allocation (FDRA) is not required.

# Conclusion

In this contribution we make the following observations:

Observation 1	A baseline SBFD time/frequency domain pattern should contain one or more UL-only slots. The remainder of the slots (configured as 'D' by TDD-UL-DL-ConfigCommon) can be configured for SBFD operation for new (Rel-18) UEs. A legacy UE sees slots/symbols configured for SBFD operation as DL-only slots.

Observation 2	For new (Rel-18) UEs, dedicated signaling should configure slots/symbols for SBFD operation. The frequency domain pattern can be configured in terms of RB sets with 'D' and 'U' direction (e.g., D-U-D) and guardbands between the RB sets.

Observation 3	For both legacy and new (Rel-18) UEs in IDLE mode, DL reception (i.e., during initial access) in SBFD slots/symbols (configured as D-U-D) occurs only in one of the 'D' subbands. Assuming a certain minimum channel bandwidth with a symmetric D-U-D configuration in either FR1 or FR2, it is feasible to configure slots/symbols for SBFD operation such that CORESET0 "fits" within a single 'D' subband. For 48 RB CORESET0, 40/100/400 MHz channel bandwidth is required for 15/30/120 kHz SCS.

Observation 4	For legacy UEs in CONNECTED mode, DL reception of some channels may occur within both 'D' RB sets, while reception of others is restricted to a single 'D' RB set.

Observation 5	For new (Rel-18) UEs in CONNECTED mode, DL reception of some channels may occur within both 'D' RB sets, while reception of others is restricted to a single 'D' RB set unless enhancements are made.

Observation 6	For new (Rel-18) UEs, CSI-RS should be enhanced to allow non-contiguous frequency domain resource allocation, i.e., in both 'D' RB sets.

Observation 7	For both legacy and new (Rel-18) UEs in IDLE mode, UL transmissions (i.e., during initial access) should occur only within UL-only slots.

Observation 8	For legacy UEs in CONNECTED mode, UL transmissions should occur only within UL-only slots.

Observation 9	For new (Rel-18) UEs in CONNECTED mode, UL transmissions should occur in either or both SBFD slots/symbols (configured as D-U-D) and UL-only slots/symbols. Current specifications can be reused since non-contiguous frequency domain resource allocation (FDRA) is not required.

# References

R1-2204106, "Evaluation of NR duplex evolution," Ericsson, RAN1#109-e, May 2022.