3GPP TSG RAN WG1 #109-e			R1-22xxxxx

e-Meeting, May 9th – 20th, 2022

Title:	Reply LS on power control for NR-DC

Response to:	(R1-2103030/R4-2206566) Further Reply LS on power control for NR-DC

Release:	Rel-16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	RAN1

To:	RAN2, RAN4

Contact Person:

Name: Karri Ranta-aho

E-mail Address: Karri.Ranta-aho@Nokia.com

Attachments:	-

1. Overall Description:

RAN1 would like to thank RAN4 for the Further Reply LS on power control for NR-DC. RAN1 discussed the issue in RAN1#109e meeting in May 2022 and does not see the need to update the RAN1 specifications.

RAN1 identified the need to update the descriptions of the NR dual-connectivity related UE capabilities as well as the 38.331 field description of p-NR-FR1 to correctly reflect the specification support for NR dual connectivity.

RAN1 requests RAN2 to modify the NR-DC power sharing mode related capabilities in TS38.306 (and potentially in TR38.822) as follows:

- For capabilities intraFR-NR-DC-PwrSharingMode1-r16, intraFR-NR-DC-PwrSharingMode2-r16 and intraFR-NR-DC-DynamicPwrSharing-r16 (FGs 18-1/1a/1b):

  - In case MCG and/or SCG have cells in different frequency ranges, this FG indicates the capability of the power sharing only between those MCG and SCG cells with UL in FR1.

- Note: above clarification for FG18-1/1a/1b does not mean that Rel-16 Ues are mandated to support power sharing mechanisms like FG18-1/1a/1b for FR2-FR2 DC.

RAN1 also requests RAN2 to add a note to the p-NR-FR2 field description as follows:

p-NR-FR2 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 2 (FR2). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR2 (configured total for all serving cells operating on FR2). This field is only used in NR-DC. UE does not expect to be configured with this parameter in this release of the specification.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account and update the specifications accordingly.

3. Date of Next TSG WG RAN1 Meetings:

TSG-RAN WG1 Meeting #110	22  -  26 August 2022         Location TBC