3GPP TSG-RAN WG1 Meeting #109-e		R1-2205333

e-Meeting, May 9th – 20th, 2022

Title:	[DRAFT] Draft reply LS on the inter-UE coordination mechanism

Response to:	R2-2203695

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	vivo [RAN1]

To:	RAN2

Cc:

Contact person:	Zichao Ji

jizichao@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN1 thanks RAN2 for the LS asking the definitions of higher layer parameters priorityScheme1CoordInfoExplicit, priorityScheme1Request, and priorityScheme1CoordInfoCondition.

RAN1 introduced these higher layer parameters (priorityScheme1CoordInfoExplicit, priorityScheme1Request, and priorityScheme1CoordInfoCondition) for various purposes relating to priority, including the following:

- Higher layer parameters (priorityScheme1CoordInfoExplicit, priorityScheme1CoordInfoCondition) are introduced to determine the priority value for sensing and candidate resource (re-)selection for transmitting the TB carrying the IUC MAC CE, and the priority value in the SCI Format 1-A corresponding to the TB carrying the IUC MAC CE.

  - Note that the above priority value also depends on other data multiplexed within the TB if any.

- Higher layer parameter (priorityScheme1Request) is introduced to determine the priority value for sensing and candidate resource (re-)selection for transmitting the TB carrying the IUC request MAC CE, and the priority value in the SCI Format 1-A corresponding to the TB carrying the IUC request MAC CE.

  - Note that the above priority value also depends on other data multiplexed within the TB if any.

In detail, the UE follows the behaviour in the RAN1 agreements below:

Agreement - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data Agreement - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B. - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data Agreement - For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. - FFS: Otherwise, the priority value is determined by UE-A’s implementation. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

Note: It is up to RAN2 to decide whether to fix the priorities of IUC MAC CE and IUC request MAC CE to ‘1’, as well as whether/how to update the related RAN2 specifications.

# 2	Actions

To RAN2

RAN1 respectfully asks RAN2 to take the above information into account in related work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG RAN WG1 Meeting #110			August 22nd – 26th, 2022   			Toulouse

TSG RAN WG1 Meeting #110bis-e			October 10th – 19th, 2022 			E-meeting