3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2205466

e-Meeting, 9th – 20th May, 2022

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Correction to BD/CCE budget allocation over multiple serving cells for multi-DCI multi-TRP for FR2-2

Document for:	Discussion, Decision

# 1	Introduction

This document contains a proposed correction to BD/CCE budget allocation over multiple serving cells for multi-DCI multi-TRP for FR2-2.

# 2	Discussion

One aspect that has not been fully captured in specifications yet is multi-DCI multi-TRP support for the case of per-slot group monitoring with multiple serving cells. Support for multi-TRP for FR2-2 has already been agreed in AI 8.2.4 (Beam Management), and for multi-TRP to be fully functional there is an impact on per-slot group PDCCH monitoring that is analogous to the impact on per-slot monitoring. For the case of per-slot monitoring with multi-DCI multi-TRP in Rel-16, 38.213 contains the following text:

38.213 Section 10

If a UE can support

-	a first set of Ncells,0DL serving cells where the UE is either not provided coresetPoolIndex or is provided coresetPoolIndex with a single value for all CORESETs on all DL BWPs of each scheduling cell from the first set of serving cells, and

-	a second set of Ncells,1DL serving cells where the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with a value 0 for a first CORESET, and with a value 1 for a second CORESET on any DL BWP of each scheduling cell from the second set of serving cells

the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is a value reported by the UE.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than 4 serving cells and the UE is not provided monitoringCapabilityConfig for any downlink cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and for a maximum number of non-overlapped CCEs the UE can monitor per slot when the UE is configured for carrier aggregation operation over more than 4 cells. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to Ncellscap downlink cells, where

-	Ncellscap is Ncells,0DL+R⋅Ncells,1DL if the UE does not provide pdcch-BlindDetectionCA where Ncells,0DL+Ncells,1DL is the number of configured downlink serving cells

-	otherwise, Ncellscap is the value of pdcch-BlindDetectionCA

According to this text, serving cells are partitioned into two mutually exclusive sets with Ncells,0DL in the first set and Ncells,1DL in the second set. The  first set corresponds to serving cells that can schedule PDSCH from only a single TRP (single value of coresetPoolIndex), and the 2nd set corresponds to serving cells that can schedule PDSCH from two TRPs (two values of coresetPoolIndex). Then for the purposes of reporting the blind decode capabilities (pdcch-BlindDetectionCA) the UE computes a number of serving cells as Ncellscap=Ncells,0DL+R⋅Ncells,1DL where R∈1,2 is a value reported by the UE capability parameter blindDetectFactor-r16 [4] (corresponds to FG 16-2a-10 [5]). Reporting R = 2 allows the UE to use its full BD/CCE budget for monitoring DCIs from two TRPs.

Then, if the total number of configured serving cells over all SCS configurations μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ exceeds the computed value Ncellscap, the aggregated BD/CCE budget over Ncellscap is divided amongst the configured serving cells as follows (see 38.213 Section 10.1):

MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j

CPDCCHtotal,slot,μ=Ncellscap⋅CPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j

where the parameter γ is configured as 1 or R. This applies to serving cells configured with Rel-15 per-slot monitoring.

In the spec review process after RAN1#108-e, it was pointed out that the BD/CCE budget allocation to serving cells did not capture this multi-TRP aspect for the case of Rel-17 per slot group monitoring, i.e., for 480/960 kHz SCS. The spec editor acknowledged this, and provided the following response. In the end there was not time to finish, and the RAN1 chairman recommended to do this in RAN1#109-e.

| Comment #5 | If possible, with the remaining time this week, we think it would be good to capture mTRP impact on PDCCH monitoring as commented by the editor: [Aris] I am fine to assume applicability of M-TRP for PDCCH in 480/960 kHz – no issue with capturing it. The only question is whether to do so now or leave for May. Should be possible to do now as the other draft CRs are getting close to stable. [Aris2]: I will request the chairman to extend the approval of this draft CR. Can then update for M-TRP and consider the update for approval together with the URLLC/SL draft CRs. |  |
| --- | --- | --- |

The issue is that the expressions for computing the BD/CCE budget per serving cell for per-slot group monitoring for 480/960 kHz are currently not written as a function of γ like above; the existing expressions effectively assume R = 1.  This means that a UE reporting R = 2 cannot take advantage of its full BD/CCE budget for PDCCH monitoring for multi-DCI mTRP.

To correct this we propose that the following TP is recommended to the spec editor.

Update the formulas in 38.213 Section 10.1 that provide the allocation of the BD/CCE budget over multiple-serving cells (MPDCCHtotal,Xs,μ and CPDCCHtotal,Xs,μ) for the case of per-slot group monitoring to account for the reported capability parameter blindDetectFactor-r16  for multi-DCI-based multi-TRP (variable R in 38.213). Note: The candidate values of blindDetectFactor-r16 are R = {1,2} and correspond to FG 16-2a-10.

  - Recommend TP#1 to the spec editor

>>> Begin TP#1 for Section 10 of 38.213 >>>

10		UE procedure for receiving control information

*** Unchanged text is omitted ***

If a UE can support

-	a first set of Ncells,0DL serving cells where the UE is either not provided coresetPoolIndex or is provided coresetPoolIndex with a single value for all CORESETs on all DL BWPs of each scheduling cell from the first set of serving cells, and

-	a second set of Ncells,1DL serving cells where the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with a value 0 for a first CORESET, and with a value 1 for a second CORESET on any DL BWP of each scheduling cell from the second set of serving cells

the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, pdcch-BlindDetectionCA-r17, pdcch-BlindDetectionCAr15, and pdcch-BlindDetectionCAr15 a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is a value reported by the UE.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than 4 serving cells and the UE is not provided monitoringCapabilityConfig for any downlink cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and for a maximum number of non-overlapped CCEs the UE can monitor per slot when the UE is configured for carrier aggregation operation over more than 4 cells. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to Ncellscap downlink cells, where

-	Ncellscap is Ncells,0DL+R⋅Ncells,1DL if the UE does not provide pdcch-BlindDetectionCA where Ncells,0DL+Ncells,1DL is the number of configured downlink serving cells

-	otherwise, Ncellscap is the value of pdcch-BlindDetectionCA

*** Unchanged text is omitted ***

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than four downlink cells, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that the UE can monitor per group of Xs slots when the UE is configured for carrier aggregation operation over more than four downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapabilityμ∈5, 6. When a UE is not configured for NR-DC operation for all downlink cells where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per group of Xs slots that corresponds to Ncellscap-r17 downlink cells, where

-	Ncellscap-r17 is Ncells,0DL+R⋅Ncells,1DL if the UE does not provide pdcch-BlindDetectionCA-r17, where Ncells,0DL+Ncells,1DL is the number of configured downlink serving cells

-	otherwise, Ncellscap-r17 is the value of pdcch-BlindDetectionCA-r17

When the UE is configured for carrier aggregation operation over more than 4 cells, the UE does not expect to monitor per group of Xs slots a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r17.

*** Unchanged text is omitted ***

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than one downlink cell with monitoringCapabilityConfig = r15monitoringcapability or larger than one downlink cell with monitoringCapabilityConfig = r17monitoringcapability, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs the UE can monitor for downlink cells with monitoringCapabilityConfig = r15monitoringcapability or for downlink cells with monitoringCapabilityConfig = r17monitoringcapability when the UE is configured for carrier aggregation operation over more than two downlink cells with at least one downlink cell with monitoringCapabilityConfig = r15monitoringcapability and at least one downlink cell with monitoringCapabilityConfig = r17monitoringcapabilityμ∈5, 6. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot or per group of Xs slots that corresponds to Ncells,r15/r17cap-r17 downlink cells or to Ncells,r17/r15cap-r17 downlink cells, respectively, where

-	Ncells,r15/r17cap-r17 is Ncells,0DL+R⋅Ncells,1DL if the UE does not provide pdcch-BlindDetectionCAr15, where Ncells,0DL+Ncells,1DL is the number of configured downlink serving cells

-	otherwise,

-	if the UE reports only one combination of (pdcch-BlindDetectionCAr15, pdcch-BlindDetectionCAr17), Ncells,r15/r17cap-r17 is the value of pdcch-BlindDetectionCAr15

-	else, Ncells,r15/r17cap-r17 is the value of pdcch-BlindDetectionCAr15 from a combination of (pdcch-BlindDetectionCAr15, pdcch-BlindDetectionCAr17) that is provided by pdcch-BlindDetectionCA-CombIndicator-r17

and

-	Ncells,r17/r15cap-r17 is Ncells,0DL+R⋅Ncells,1DL if the UE does not provide pdcch-BlindDetectionCAr17, where Ncells,0DL+Ncells,1DL is the number of configured downlink serving cells

-	otherwise,

-	if the UE reports only one combination of (pdcch-BlindDetectionCAr15, pdcch-BlindDetectionCAr17), Ncells,r17/r15cap-r17 is the value of pdcch-BlindDetectionCAr17

-	else, Ncells,r17/r15cap-r17 is the value of pdcch-BlindDetectionCAr17 from a combination of (pdcch-BlindDetectionCAr15, pdcch-BlindDetectionCAr17) that is provided by pdcch-BlindDetectionCA-CombIndicator-r17

*** Unchanged text is omitted ***

When the UE is configured for carrier aggregation operation over more than two downlink cells with at least one downlink cell with monitoringCapabilityConfig = r15monitoringcapability, at least one downlink cell with monitoringCapabilityConfig = r17monitoringcapability, and no downlink cell with monitoringCapabilityConfig = r16monitoringcapability, the UE does not expect to

-	monitor per slot a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncells,r15/r17cap-r17, and

-	monitor per group of Xs slots a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncells,r17/r15cap-r17

*** Unchanged text is omitted ***

10.1	UE procedure for determining physical downlink control channel assignment

*** Unchanged text is omitted ***

If a UE

-	does not report pdcch-BlindDetectionCA, pdcch-BlindDetectionCA-r17, pdcch-BlindDetectionCAr15, or pdcch-BlindDetectionCAr15 or is not provided BDFactorR, γ=R

-	reports pdcch-BlindDetectionCA, pdcch-BlindDetectionCA-r17, pdcch-BlindDetectionCAr15, or pdcch-BlindDetectionCAr15, the UE can be indicated by BDFactorR either γ=1 or γ=R

If a UE is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig, or is provided monitoringCapabilityConfig = r15monitoringcapability and is not provided CORESETPoolIndex, with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor, on the active DL BWPs of the scheduling cells,

-	more than MPDCCHtotal,slot,μ=MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,0DL,μ downlink cells, or

-	more than MPDCCHtotal,slot,μ=γ∙MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=γ∙CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells

-	more than MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHmax,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells

Ncellscap is replaced by Ncells,r15cap-r16, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability.

If a UE

-	is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig, or is provided monitoringCapabilityConfig = r15monitoringcapability and is not provided coresetPoolIndex,

-	with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cell(s) using SCS configuration μ, where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, and

-	a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell,

the UE is not required to monitor more than MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j  PDCCH candidates or more than CPDCCHtotal,slot,μ=Ncellscap⋅CPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the Ncells,0DL,μ+Ncells,1DL,μ downlink cells. Ncellscap is replaced by Ncells,r15cap-r16 if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability.

For each scheduled cell from the Ncells,0DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot.

For each scheduled cell from the Ncells,1DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell

-	more than minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot

-	more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value

*** Unchanged text is omitted ***

If a UE is configured with Ncells,r17DL,μ Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for μ∈5, 6the active DL BWPs of the scheduling cells, and with Ncells,r17DL,Xs,μ Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ of the Ncells,r17DL,μ Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells using any combination Xs,Ys for a group of Xs slots for PDCCH monitoring, where μ=56Ncells,r17,0DL,μ+γ∙Ncells,r17,1DL,μ≤Ncellscap-r17, μ=56Ncells,r17DL,μ≤Ncellscap-r17the UE is not required to monitor, on the active DL BWP of the scheduling cell

- more than MPDCCHtotal,Xs,μ=MPDCCHmax,Xs,μ PDCCH candidates or more than CPDCCHtotal,Xs,μ=CPDCCHmax,Xs,μ non-overlapped CCEs per group of Xs slots for each scheduled cell when the scheduling cell is from the Ncells,r17,0DL,Xs,μ downlink cells, or

- more than MPDCCHtotal,Xs,μ=γ∙MPDCCHmax,Xs,μ PDCCH candidates or more than CPDCCHtotal,Xs,μ=γ∙CPDCCHmax,Xs,μ non-overlapped CCEs per group of Xs slots for each scheduled cell when the scheduling cell is from the Ncells,r17,1DL,Xs,μ downlink cells, or

- more than MPDCCHmax,Xs,μ PDCCH candidates or more than CPDCCHmax,Xs,μ non-overlapped CCEs per group of Xs slots for CORESETs with same coresetPoolIndex for each scheduled cell when the scheduling cell is from the Ncells,r17,1DL,Xs,μ downlink cells.

If the UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs, Ncellscap-r17 is replaced by Ncells,r17/r15cap-r17. If the UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs, Ncellscap-r17 is replaced by Ncells,r17/r16cap-r17. If the UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapabilityμ∈5, 6 for the active DL BWPs, Ncellscap-r17 is replaced by Ncells,r17/{r15,r16}cap-r17. If, for one or more of the cells, the UE is provided with monitoringCapabilityConfig = r16monitoringcapability, the UE assumes γ=1.

If a UE is configured Ncells,r17DL,μ Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs of the scheduling cells, and with Ncells,r17DL,Xs,μ Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ of the Ncells,r17DL,μ Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells using any combination Xs,Ys for a group of Xs slots for PDCCH monitoring, where μ=56Ncells,r17,0DL,μ+γ∙Ncells,r17,1DL,μ>Ncellscap-r17μ=56Ncells,r17DL,μ>Ncellscap-r17, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than

MPDCCHtotal,Xs,μ=Ncellscap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17DL,Xs,μj=56Ncells,r17DL,jMPDCCHtotal,Xs,μ=Ncellscap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j

PDCCH candidates, or more than

CPDCCHtotal,Xs,μ=Ncellscap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17DL,Xs,μj=56Ncells,r17DL,jCPDCCHtotal,Xs,μ=Ncellscap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j

non-overlapped CCEs, per group of Xs slots on the active DL BWP(s) of scheduling cell(s) from the Ncells,r17DL,Xs,μ  Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ downlink cells where Ncells,r17DL,j Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j Ncells,r17,0DL,j+Ncells,r17,1DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. If the UE is configured Ncells,r17DL,μ Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j Ncells,r17,0DL,j+Ncells,r17,1DL,j downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability or monitoringCapabilityConfig = r16monitoringcapability, and monitoringCapabilityConfig = r17monitoringcapability for the active DL BWP, Ncellscap-r17 is replaced by Ncells,r17/r15cap-r17, or by Ncells,r17/r16cap-r17, or by Ncells,r17/{r15,r16}cap-r17, respectively, and

MPDCCHtotal,Xs,μ=Ncells, refcap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17DL,Xs,μj=06Ncells,r17DL,j,MPDCCHtotal,Xs,μ=Ncellscap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=06Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j

CPDCCHtotal,Xs,μ=Ncells,refcap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17DL,Xs,μj=06Ncells,r17DL,j,CPDCCHtotal,Xs,μ=Ncellscap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=06Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j

and Ncells,refcap-r17 is one of Ncells,r17/r15cap-r17, Ncells,r17/r16cap-r17, or Ncells,r17/{r15,r16}cap-r17, respectively. If, for one or more of the cells, the UE is provided with monitoringCapabilityConfig = r16monitoringcapability, the UE assumes γ=1.

For each scheduled cell from the Ncells,r17,0DL,Xs,μ downlink cells using any combination Xs,Ys for a group of Xs slots, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell, more than minMPDCCHmax,Xs,μ,MPDCCHtotal,Xs,μ PDCCH candidates or more than minCPDCCHmax,Xs,μ,CPDCCHtotal,Xs,μ non-overlapped CCEs per group of Xs slots.

For each scheduled cell from the Ncells,r17,1DL,Xs,μ downlink cells using any combination Xs,Ys for a group of Xs slots, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell,

- more than minγ∙MPDCCHmax,Xs,μ,MPDCCHtotal,Xs,μ PDCCH candidates or more than minγ∙CPDCCHmax,Xs,μ,CPDCCHtotal,Xs,μ non-overlapped CCEs per group of Xs slots

- more than more than minMPDCCHmax,Xs,μ,MPDCCHtotal,Xs,μ PDCCH candidates or more than minCPDCCHmax,Xs,μ,CPDCCHtotal,Xs,μ non-overlapped CCEs per group of Xs slots for CORESETs with the same coresetPoolIndex value.

*** Unchanged text is omitted ***

>>> End TP >>>