3GPP TSG RAN WG1 #109-e	R1-22xxxxx

e-Meeting, May 9th – 20th, 2022

Agenda item:		8.8

Source:	Moderator (China Telecom)

Title:	Summary of preparation phase for Rel-17 NR coverage enhancements

Document for:		Discussion and Decision

Introduction

This contribution is a summary of the discussion to identify maintenance issues for Rel-17 NR coverage enhancements in RAN1#109-e.

Discussion

AI 8.8.1

- Enhancements on PUSCH repetition type A

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue#1: Frequency hopping for PUSCH scheduled by RAR UL grant or DCI format 0_0 | R1-2203095, R1-2203191, R1-2203439, R1-2203610, R1-2203994, R1-2204212, R1-2204278, R1-2204664, R1-2204871 | [Medium Priority] Suggest to be discussed in RAN1#109-e. This issue has been discussed for a couple of meetings without achieving consensus on any particular TP. Recommend firstly checking if FL’s proposed conclusion from the last meeting is acceptable to everyone. |
| Issue#2: DL throughput degradation for PUSCH repetitions due to limited range of RRC dL-DataToUL-ACK and UCI multiplexing restriction | R1-2203095 | [Suggest NOT to be discussed in RAN1#109-e.] The issue is valid. Companies are invited their views on whether this issue should be discussed in RAN1#109-e or not. |
| Issue#3: Available slot counting for Inter-cell multi-TRPs | R1-2203191, R1-2203521, R1-2203869, R1-2204089, R1-2204278, R1-2204527, R1-2204548, R1-2204664, R1-2204775, R1-2204871 | [High Priority] Suggest to be discussed in RAN1#109-e. This discussion has been deferred, and now is the time to re-open since there was some related progress in MIMO session in the last meeting. |
| Issue#4: SSB for Available slot counting in CA | R1-2203521 | Suggest NOT to be discussed in RAN1#109-e. The current spec seems clear enough. |
| Issue#5: Restriction on K2 offset for DG-PUSCH with K>1 and Out-of-order handling when AvailableSlotCounting is enabled | R1-2203610, R1-2203994, R1-2204657, R1-2204664 | [High Priority] Suggest to be discussed in RAN1#109-e. At least, it is important to confirm the common understanding on this issue in order to avoid different assumptions between UE and gNB. |
| Issue#6: Editorial change on TB repetition across N*K slots | R1-2203791 | Suggest NOT to be discussed in RAN1#109-e. The current spec seems clear enough. |
| Issue#7: Editorial correction on available slot counting for RedCap HD UE | R1-2204664, R1-2204775, R1-2204871, R1-2204990 | [Low Priority] Suggest to be discussed in RAN1#109-e. This correction does not require much discussion/effort. |

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | Regarding issue#2, it is surely in scope because the Rel-17 feature PUSCH repetition with up to 32 repetitions in current spec has severe degradation of DL throughput which is unacceptable to practical network. For example, as shown in figure 2 of R1-2203095, 11 DL slots cannot be scheduled if the UE is scheduled for PUSCH repetitions with only 6 available UL slots (equivalently scheduled 22 physical slots). The larger repetition number, the more degradation of DL throughput. It is a severe issue to Rel-17 PUSCH repetition feature. Therefore, a discussion is necessary. Since larger range of RRC dL-DataToUL-ACK is not a complete solution, nor the only potential solution, we suggest to rename the issue as “DL throughput degradation for PUSCH repetitions due to limited range of RRC dL-DataToUL-ACK and UCI multiplexing restriction” Additionally, since TBoMS relies on slot counting based on available slots, the same issue also impacts on TBoMS severely. We suggest to discuss it for PUSCH repetition first, then determine whether it can fully be reused to TBoMS or any additional impact is necessary. |
| FL(Sharp) | @Huawei: Thanks for the explanation. The issue you raised seems valid. So, I updated the initial assessment on Issue#2, accordingly. At the same time, I’m still wondering if the extension of dL-DataToUL-ACK can mitigate DL throughput degradation, because the number for DL HARQ processes is anyway limited. Let’s see other’s views. |

- TB processing over multi-slot PUSCH

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue #1: Removing the condition that AvailableSlotCounting is enabled for TBoMS | R1-2203191, R1-2203610, R1-2203791, R1-2204871, R1-2203095, R1-2204775 | [High priority] Suggest to be discussed in RAN1#109-e. In Rel-17, the number of slots for TBoMS is always counted based on the available slots without any dependency on a RRC parameter. TP may be needed for capturing this correctly. |
| Issue #2: MAC layer may not generate MAC PDU for CG PUSCH with TBoMS when there is overlapping with DG PUSCH. | R1-2203191 | Suggest NOT to be discussed in RAN1#109-e. The agreements in RAN1 are clear. This should be part of normative works in RAN2. |
| Issue #3: Multiplexing of HARQ-ACK on TBoMS following UL T-DAI in case without overlapping PUCCH since DL DCI is missed. | R1-2203191 | [Low priority] Suggest to be discussed in RAN1#109-e. |
| Issue #4: Support of Rel-17 coverage enhancement techniques in unlicensed band. | R1-2203837, R1-2203869 | Suggest NOT to be discussed in RAN1#109-e. Whether CovEnh techniques are applicable for unlisenced band or not should not be discussed in the maintenance of CovEnh itself. In addition, TBoMS is a PUSCH transmission with a single TB and single HARQ process number, the use of this NR-U feature for TBoMS seems to be irrelevant. |
| Issue #5: TP for available slot determination for TBoMS in CG-PUSCH | R1-2204664, R1-2204775 | [High priority] Suggest to be discussed in RAN1#109-e. |
| Issue #6: Out-of-order handling for TBoMS | R1-2204775 | Suggest NOT to be discussed in RAN1#109-e. The same issue, if any, seems to exist in Rel-15/16 as well. Therefore, it should be discussed in Rel-15/16 maintenance. Rel-17 can simply follow the existing handling. |
| Issue #7: CSI report on TBoMS | R1-2204775 | [High priority] Suggest to be discussed in RAN1#109-e. |

| Company | Views |
| --- | --- |

- Type A PUSCH repetitions for Msg3

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue#1: Handling of the working assumption on CFRA PUSCH repetition | - R1-2203610, R1-2203791, R1-2204089, R1-2204278, R1-2204349, R1-2204775, R1-2204871 | [High Priority] Suggest to be discussed in RAN1#109-e. |
| Issue#2: TP for Msg3 PUSCH repetition in section 8.3 of TS38.213 | R1-2203791 | Suggest NOT to be discussed in RAN1#109-e. The current specification texts are technically correct, and the proposed TP is not essential. |
| Issue#3: Collision handling between SSB and Msg3 PUSCH transmission for HD-FDD UE | R1-2204278 | Suggest NOT to be discussed in RAN1#109-e. RAN1 has agreed to reuse the decision from RedCap WI, i.e., SSB is prioritized in case of the collision. No specification impact is expected. |
| Issue#4: Available slot counting for Msg3 repetition with K=1 | R1-2204990 | Suggest NOT to be discussed in RAN1#109-e. If a UE requests Msg3 repetition, it means it can support Msg3 repetition and gNB can indicate a repetition factor equal to or larger than 1 subject to RRC configuration. If a UE does not request Msg3 repetition, gNB can only schedule Msg3 without repetition. There is no ambiguity on whether a Msg3 transmission would be transmitted or not. The proposed TP is not needed. |

| Company | Views |
| --- | --- |

AI 8.8.2

- Joint channel estimation for PUSCH

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue#1: Group common TPC commands handling for DMRS bundling for PUSCH and PUCCH. | R1-2203192, R1-2203611, R1-2204350, R1-2204991, R1-2203652, R1-2204090, R1-2204213, R1-2204279, R1-2204513, R1-2204872, R1-2203096, R1-2203309, R1-2203402, R1-2203440, R1-2203522, R1-2203870, R1-2204776 | [High priority] Suggest to be discussed in RAN1#109-e. |
| Issue#2: Clarification on UE behavior of restarting DMRS bundling with respect to multiple semi-static and dynamic events within one nominal TDW. | R1-2203611, R1-2203652, R1-2204090, R1-2204279, R1-2204872, R1-2203402, R1-2203522 | [High priority] Suggest to be discussed in RAN1#109-e. |
| Issue#3: Clarification on the description of maximum duration in RAN1 and RAN2 specifications. | R1-2203611 | [Low priority] Suggest to discuss related issue only in RAN1 specifications in RAN1#109-e. |
| Issue#4: Clarification on candidate values for maximum duration. | R1-2203611, R1-2203612 | Suggest to be discussed in UE feature session AI 8.16.8. |
| Issue#5: Clarification on DMRS bundling in unlicensed band. | R1-2203870 | Suggest to be discussed in UE feature session AI 8.2.5. |
| Issue#6: Clarification on the interpretation of KPUSCH(i)KPUSCH(i) for DG-PUSCH repetitions for accumulated TPC command and clarification on the timeline KPUSCH(i) for absolute TPC mode. | R1-2204513 | Suggest NOT to be discussed in RAN1#109-e. This issue was extensively discussed in Rel-15 maintenance in RAN1 #108-e, no consensus has been reached and no further discussion is expected. |
| Issue#7: Clarification on UE behaviour about performing the same precoder of precoding cycling within an actual TDW. | R1-2203402 | Suggest NOT to be discussed in RAN1#109-e. This issues was proposed and discussed during the past meetings, it seems to be a common understanding that precoder cannot be changed within one actual TDW. |
| Issue#8: Other minor clarification/correction issues. Issue#8-1: Editorial issue on “power control parameters” in TS 38.214. Issue#8-2: Alignment on RRC parameter ‘[maxDMRS-BundlingDuration]’ in TS 38.214 with RAN2 spec. | R1-2203096, R1-2203192 | [Low priority] Suggest to be discussed in RAN1#109-e. |

| Company | Views |
| --- | --- |

- PUCCH enhancements

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue#1: Correction/Clarification on frequency hopping for DMRS bundling for PUCCH and PUSCH. | R1-2204991, R1-2204872, R1-2203096, R1-2203522, R1-2204776, R1-2203440, R1-2203193 | [High priority] Suggest to be discussed in RAN1#109-e. |
| Issue#2: Clarification on inter-slot frequency hopping for Msg3 when DMRS bundling is enabled. | R1-2204513 | [Low priority] Suggest to be discussed in RAN1#109-e. |
| Issue#3: Clarification on Rel-17 inter-slot frequency hopping when frequency hopping interval is configured but DMRS bundling is not enabled. | R1-2204872, R1-2204873 | [Low priority] Suggest to be discussed in RAN1#109-e. |
| Issue#4: Clarification on the interrelationship between the length of hopping interval and that of nominal TDW. | R1-2204549 | Suggest NOT to be discussed in RAN1#109-e. Based on the agreement in RAN1 #107, the hopping interval and nominal TDW are separately configured, while the nominal TDW was determined followed by the determination of hopping interval. It seems this issue is clear. |
| Issue#5: Clarification on dynamic PUCCH repetition factor indication for HARQ-ACK of first SPS PDSCH associated with the activation DCI and SPS release DCI. | R1-2203309, R1-2204957, R1-2203193 | [Medium priority] Suggest to be discussed in RAN1#109-e. |

| Company | Views |
| --- | --- |

Conclusion

References

- R1-2203095	Discussion on PUSCH enhancements	Huawei, HiSilicon

- R1-2203191	Discussion on remaining issues for PUSCH enhancements	ZTE

- R1-2203439	Remaining issues on PUSCH enhancements in Rel-17	CATT

- R1-2203521	Remaining issues on PUSCH enhancements	vivo

- R1-2203610	Remaining issues on PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2203791	Maintenance on PUSCH enhancements	xiaomi

- R1-2203837	Remaining issues on TB processing over multi-slot PUSCH	Langbo

- R1-2203869	PUSCH enhancements	Samsung

- R1-2203994	Enhancements on PUSCH repetition type A	OPPO

- R1-2204089	Remaining issues on PUSCH enhancements	InterDigital, Inc.

- R1-2204212	Remaining issues on PUSCH repetition type A enhancement	Apple

- R1-2204278	Discussion on the remaining issues of CE PUSCH enhancement	CMCC

- R1-2204349	Remaining issues on PUSCH enhancements for coverage enhancement	NTT DOCOMO, INC.

- R1-2204527	Remaining details on PUSCH enhancements	LG Electronics

- R1-2204548	Remaining issues on enhancements for PUSCH repetition Type A	WILUS Inc.

- R1-2204657	Discussion on remaining issues on PUSCH repetition Type A enhancements	Panasonic

- R1-2204664	PUSCH enhancements for Rel-17 CovEnh	Sharp

- R1-2204726	Discussion on PUSCH enhancements	MediaTek Inc.

- R1-2204728	Discussion on Joint channel estimation over multi-slot	MediaTek Inc.

- R1-2204775	Remaining issues on PUSCH enhancements	Intel Corporation

- R1-2204871	Maintenance for PUSCH Repetition and TBoMS	Ericsson

- R1-2204990	PUSCH Enhancements	Qualcomm Incorporated

- R1-2203096	Discussion on joint channel estimation for PUSCH and PUCCH	Huawei, HiSilicon

- R1-2203192	Discussion on remaining issues for joint channel estimation	ZTE

- R1-2203309	Discussion on joint channel estimation for PUSCH&PUCCH	Spreadtrum Communications

- R1-2203402	Discussion on joint channel estimation for PUSCH and PUCCH	Panasonic

- R1-2203440	Remaining issues on joint channel estimation in Rel-17	CATT

- R1-2203522	Remaining issues on joint channel estimation	vivo

- R1-2203611	Remaining issues on joint channel estimation for PUSCH and PUCCH	Nokia, Nokia Shanghai Bell

- R1-2203652	Remaining issues on joint channel estimation for PUSCH and PUCCH	China Telecom

- R1-2203870	Joint channel estimation for PUSCH and PUCCH	Samsung

- R1-2204090	Joint channel estimation for PUSCH and PUCCH	InterDigital, Inc.

- R1-2204213	Remaining issues on cross-slot channel estimation for PUSCH	Apple

- R1-2204279	Discussion on the remaining issues of joint channel estimation for PUSCH and PUCCH	CMCC

- R1-2204350	Remaining issues on joint channel estimation for PUSCH and PUCCH for coverage enhancement	NTT DOCOMO, INC.

- R1-2204455	Discussion on joint channel estimation for PUSCH&PUCCH	Spreadtrum Communications

- R1-2204513	Joint channel estimation for PUSCH and PUCCH	Sharp

- R1-2204549	Remaining issues on Joint channel estimation for PUCCH and  PUSCH	WILUS Inc.

- R1-2204776	Remaining issues on joint channel estimation for PUSCH and PUCCH	Intel Corporation

- R1-2204872	Maintenance of Joint Channel Estimation for PUSCH and PUCCH	Ericsson

- R1-2204991	Joint channel estimation for PUSCH and PUCCH	Qualcomm Incorporated

- R1-2203193	Discussion on remaining issues for coverage enhancements for PUCCH	ZTE

- R1-2203612	Draft LS on description of RRC parameters for nominal time domain window length for PUSCH and PUCCH DMRS bundling	Nokia, Nokia Shanghai Bell

- R1-2203792	Other considerations for TB processing over multi-slot PUSCH	xiaomi

- R1-2204873	Rel-17 Multi-Slot Frequency Hopping and Further Enhancements	Ericsson

- R1-2204902	Further consideration on PUSCH coverage enhancment	Huawei, HiSilicon

- R1-2204957	Remaining issues for PUCCH coverage enhancements	InterDigital, Inc.