Proposal 1.E.1: On the Type-II codebook refinement for CJT mTRP, the resulting codebook(s) are associated with at least the following parameters:

  - Parameters for basis reporting, including

    - The number of basis vectors: gNB-configured via higher-layer signaling

      - FFS: Whether it is layer-common or layer-specific, whether it is per TRP/TRP-group or common for all TRPs

    - Basis selection indicator(s): a part of CSI report

      - FFS: Whether it is layer-common or layer-specific, whether it is per TRP/TRP-group or common for all TRPs

  - Quantized combining coefficients (W2): a part of CSI report

    - FFS: details of quantization scheme

  - Number of non-zero coefficients and bitmap to indicate non-zero coefficients, including whether it is per TRP/TRP-group (separate) or across all TRPs/TRP-groups (joint): a part of CSI report

  - Strongest coefficient indicator(s) (SCI(s)): a part of CSI report

    - FFS: One per TRP/TRP-group or common for all TRPs

    - FFS: Additional need for strongest TRP indicator

Proposal 1.E.2: For the Type-II codebook refinement for CJT mTRP, further study the following issues:

- The need for the following additional parameters:

  - Receiver side information by per RX reporting or per layer, e.g. information related to the left singular matrix U of the channel

  - Indication of relative offset of reference FD basis per TRP with respect to a reference TRP

  - Information related to the windows for FD basis

  - Delay/frequency difference(s) across TRPs

- Specification entity corresponding to a TRP (e.g. port-group, NZP CSI-RS resource)

- For codebooks with per-TRP/TRP-group SD/FD basis (structure Alt1A/1B), whether to support co-amplitude/phase as a part of CSI report (explicit) or not (implicit)

- Design details of reference amplitudes and differential amplitudes in W2:

- Whether/how supported parameter combinations are refined from Rel-16/17

Proposal 1.F: On the Type-II codebook refinement for CJT mTRP, down-select from the following TRP selection/determination schemes (where N is the number of cooperating TRPs assumed in PMI reporting):

  - Alt1. N is gNB-configured via higher-layer (RRC) signaling

    - The N configured TRPs are gNB-configured via higher-layer (RRC) signaling

    - Note: only one transmission hypothesis is reported

  - Alt2. N is UE-selected and reported as a part of CSI report where N∈{1,..., NTRP}

    - N is the number of cooperating TRPs, while NTRP is the maximum number of cooperating TRPs configured by gNB

    - In this case, the selection of N out of NTRP TRPs is also reported (FFS: exact reporting scheme)

    - FFS: Configuration of NTRP TRPs and the value of NTRP, whether explicit or implicit

    - FFS: In addition to one transmission hypothesis, whether reporting multiple transmission hypotheses (with the same N value or possibly different N values) is supported

  - Alt3. The UE reports CSI corresponding to K transmission hypotheses

    - The N configured TRPs are gNB-configured via higher-layer (RRC) signaling

    - FFS: supported value(s) of K, and whether the K transmission hypotheses are gNB-configured or UE-reported

Proposal 2.E.1: On the Type-II codebook refinement for high/medium velocities, for codebook structures with TD or DD basis (Alt1 or Alt2 from codebook structure agreement), the codebook(s) include at least the following additional codebook parameters:

- Doppler-/time-domain (DD/TD) basis vector length

- Parameters for DD/TD basis vector selection, including

  - The number of DD/TD basis vectors

  - If applicable, Basis selection indicator(s)

    - FFS: restrictions on the basis vector selection

  - If applicable, the total number of available DD/TD basis vectors (not needed for orthogonal DFT basis set), whether explicitly or implied from another parameter (e.g. oversampling factor)

Proposal 2.E.2: For the Type-II codebook refinement for high/medium velocities, further study the following issues:

- The need for basis type indicator, if both a trivial basis (e.g. identity) and a non-trivial (e.g. DFT) basis are supported, and if so, whether implicit or explicit

- The need for DD/TD (compression) unit (analogous to PMI sub-band for Rel-16 codebook)

Proposal 2.F: On potential refinement of Resource setting configuration associated with Type-II codebook refinement for high/medium velocities, study the following options to assess whether/how the legacy Resource setting configuration needs to be enhanced for “burst” measurement:

- Periodic (P) CSI-RS: periodicity and offset

- Semi-persistent (SP) CSI-RS: activation/deactivation, periodicity, and offset

- Aperiodic (AP) CSI-RS: triggering, offset of a group of AP CSI-RS resources

FFS: Support for K>1 NZP CSI-RS resources association with Type-II codebook refinement for high/medium velocities

FFS: Whether specification support for jointly utilizing two types of CSI-RS time-domain behaviors is needed

Proposal 3.D:  The TRS-based TDCP reporting is down selected from the following alternatives:

- Alt1 (stand-alone): TDCP reporting comprises auxiliary feedback information to enable refinement of CSI reporting configuration, and/or codebook configuration parameters, and/or (to be confirmed in RAN1#110) gNB-side CSI prediction

  - Aperiodic reporting is supported

  - FFS: Whether periodic, semi-persistent and/or event-triggered (UE-initiated) reporting are supported

- Alt2 (non-stand-alone): TDCP reporting corresponds to a subset of the UCI parameters associated with a codebook/PMI for high/medium velocities, reported by the UE and measured via TRS

  - FFS: The associated codebook(s)/PMI(s)