Proposal 1.G: On the spatial-domain (SD) and frequency-domain (FD) basis design for the Rel-16 Type-II codebook refinement for CJT mTRP, down-select from the following alternatives:

- Alt1 (separate, legacy DFT): SD basis and FD basis are separate, each fully reusing the legacy Rel-16 DFT-based design

- Alt2 (joint, DFT): joint SD-FD DFT-based basis

  - FFS: Details on DFT parameters, e.g. length, oversampling (if any), rotation (if any)

- Alt3 (joint, eigenvector): joint SD-FD eigenvector-based basis

  - FFS: eigenvector codebook design, parametrization

- Alt4 (separate, eigenvector): SD basis and FD basis are separate, using eigenvector-based basis

  - FFS: eigenvector codebook design, parameterization

Proposal 1.H: On the W2 coefficient quantization scheme for the Type-II codebook refinement for CJT mTRP:

- At least for N=2, reuse the following components of the legacy Rel-16/17 per-coefficient quantization scheme:

  - Alphabets for amplitude and phase

  - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients

- Further study the following:

  - For larger N values, if supported, whether/how to improve throughout-overhead trade-off using, e.g. lower-resolution alphabets for amplitude and/or phase than legacy, or higher/same resolution alphabets but smaller number of coefficients than legacy

  - What constitutes a “group” (e.g. per polarization across TRPs/TRP-groups, per polarization per TRP/TRP-group, per TRP/TRP-group, combination of the two), the number of “groups” per layer for phase and amplitude (1 ≤Cgroup,phase ≤ 2N, 1 ≤ Cgroup,amp ≤ 2N), and how to indicate/configure “grouping”

Proposal 2.G: On the CSI reporting and measurement for the Type-II codebook refinement for high/medium velocities, at least for discussion purposes, define the following:

- Assume a CSI report in slot n, and let the length of the DD/TD basis vector be N4

  - Note that basis vector has no span/window in time-domain, only length

- CSI-RS measurement window of [k,k+Wmeas –1], representing the window in which CSI-RS occasion(s) are measured for calculating a CSI report

  - k is a slot index and Wmeas is the measurement window length (in slots)

  - Note: In the legacy Rel-16/17 CSI, the CSI-RS occasion(s) are configured in CSI-ReportConfig

- CSI reporting window of [l,l+WCSI –1], associated to the CSI report in slot n

  - l is a slot index and WCSI is the reporting window length (in slots)

- CSI reference resource(s) in time-domain

  - The location of a CSI reference resource is denoted as nref (slot index)

Proposal 2.H: On the CSI reporting and measurement for the Type-II codebook refinement for high/medium velocities, consider at least the following alternatives for potential down-selection:

- Alt1: nref (CSI reference resource slot) as boundary

  - Alt1.A:  l + WCSI –1 ≤ nref

  - Alt1.B:  l ≥ nref

  - Alt1.C: l < nref and l + WCSI –1 > nref

- Alt2: n (report slot) as boundary

  - Alt2.A: l + WCSI –1 ≤ n

  - Alt2.B: l ≥ n

  - Alt2.C: l < n and l + WCSI –1 > n

- Alt3: End slot of Wmeas (k + Wmeas –1) as boundary

  - Alt3.A: l + WCSI –1 ≤ k + Wmeas –1 with the following as a special case: l=k, WCSI = Wmeas

  - Alt3.B: l ≥ k + Wmeas –1

  - Alt3.C: l < k + Wmeas –1 and l + WCSI –1 > k + Wmeas –1 with the following as special cases:

    - l=k, l + WCSI = n

    - l=k, l + WCSI = nf > n

FFS: whether nref represents the slot index of Rel-15 CSI reference resource or a newly defined CSI reference resource

FFS: whether/how the CSI measurement window and reporting window are configured