3GPP TSG RAN WG1 #109e                                                        R1-2205526

e-Meeting, May 9th – 20th, 2022

Source: 	Moderator (Ericsson)

Agenda item: 	9.5.3

Title: 	Feature Lead Summary#1 for [109-e-R18-Pos-08] Positioning for RedCap Ues

Document for: 	Discussion and Decision

# Introduction

This document summarizes the proposals in [1]-[22] for the agenda item 9.5.3, and documents the discussion for the email discussion [109-e-R18-Pos-08] as per the following chairman decision

[109-e-R18-Pos-08] Email discussion on positioning for RedCap UEs by May 20 – Florent (Ericsson)

- Check points: May 16, May 20

# Summary of proposed discussion topics

- The received contributions provide proposals on the following aspects:

- Use cases and target requirements for positioning for redcap UEs

- Scenarios, evaluation assumptions and simulation Parameters  for redcap UEs

- Evaluation results for redcap UEs

- Enhancements for redcap UEs

# Use cases and target requirements for Redcap UEs

## Summary of proposals

Several companies have proposed to generally discuss use cases and performance targets [2][7][10][18][20] during this meeting. [3][9][11][12] proposes to re-use the use case and requirements from rel-17 support  for commercial /IIOT use cases, and [9] [11]  proposes to consider relaxing the IIOT requirement for redcap devices. [10] proposed to re-use rel16 commercial requirements as a starting point. The use case of wearables is mentioned by [4][11][14], and [14] proposes to target < 1 m horizontal, <3m vertical accuracy. [19] mentions IWSN for IIOT

Latency requirement are discussed in [18][10]. [22] proposes to involve RAN4 regarding the requirements for 1Rx branch.

| - Source | - Proposal |
| --- | --- |
| - [1] | - Proposal 1: Set the following target requirements for RedCap positioning: - Horizontal positioning accuracy: <1m - Vertical positioning accuracy: <3m |
| - [2] | Proposal 1: RAN1 to discuss the performance targets for RedCap UEs. |
| - [3] | Proposal 1: The Rel-18 positioning accuracy requirement for RedCap UEs should be no higher than the Rel-17 positioning accuracy requirement for commercial and IIoT use cases, e.g., 1~3 meter (90% of UEs). |
| - [4] | Proposal 1 - Select the use case of wearables as a single representative use case for RedCap positioning. - The following accuracy requirement can be considered for RedCap positioning. - Horizontal position accuracy of 2m for 90% of UEs. - Vertical position accuracy of 3m for 90% of UEs. |
| - [7] | Proposal 3: RAN1 needs to define the target positioning accuracy for RedCap UE. |
| - [9] | Proposal 2: - In commercial scenario, reusing the Rel-16/Rel-17 requirements for commercial scenario for RedCap positioning - In industrial scenario, consider relaxing the requirements for RedCap positioning compared with the R17 IIoT requirement |
| - [10] | Observation 1: the positioning accuracy (or latency) requirements are not clear. Proposal 1: RAN1 is to study the requirements on positioning accuracy for RedCap UEs. Proposal 2: using positioning accuracy requirement for commercial use cases defined in R16 as starting point. |
| - [11] | Proposal 1: Regarding the positioning support for RedCap UEs, support the following two categories of use cases: - Commercial use cases mainly for wearables - IIoT use cases mainly for industrial wireless sensors Proposal 4: The requirements of positioning performance for RedCap UE should be lower than that of normal UEs. - FFS: the exact values of requirements |
| - [12] | Proposal 3: For horizontal and vertical accuracy, reuse Rel. 17 targets (less than 1m and 0.2m for commercial and IIoT applications for horizontal accuracy and less than 3m and 1m for commercial and IIoT applications for vertical accuracy) |
| - [14] | Proposal 1: The requirements of the RedCap UE positioning is defined as: - Horizontal position accuracy (< 1 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs |
| - [18] | Proposal 1: RAN1 needs to define the potential indoor and outdoor use cases and positioning requirements for RedCap positioning. Proposal 2: RAN1 needs to study at least the vertical and horizontal positioning accuracy and availability requirements for the RedCap devices considering indoor and outdoor deployments. Proposal 3: Study the end-to-end latency requirements for the RedCap devices. |
| - [19] | Proposal 1: Among three Rel-17 RedCap use cases, the target for positioning should be IWSN for IIoT. Proposal 2: The use case of the RedCap positioning should be selected among the use cases categorized in TS22.104. Observation: Given that the positioning is applied for RedCap devices with IWSN use case, the positioning use case that overlaps with the RedCap IWSN would be use case #4 (Process automation: Sequence container (Intralogistics) in 22.104. Proposal 3: For RedCap IWSN, how the clarified requirements for positioning use case can be achievable should be studied. |
| - [20] | - RAN1 to decide on target positioning accuracy for RedCap use-cases for NR-based positioning, potentially considering availability of multiple complementary positioning methods to ascertain the need for enhancements for RedCap UEs with max UE BW of 20 MHz. |
| - [22] | Proposal 1: Send LS to RAN4 to ask them to include positioning requirements derived using simulation assumptions wherein 1 Rx is assumed at the UE. |

## First round of discussion (closed)

Considering the work plan, the ambition for this meeting is to  agree on target requirements and use case for RedCap positioning. Based on the exisiting proposals and observation,  it seem possible to begin the discussion from  the rel17 targets for commercial and IIOT use cases as a starting point for evaluation. Further discussion is needed to consider additional, potentially more relaxed, requirements.

For convenience, rel17 requirements for commercial and IIOT is copied below:

Agreement: - In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - End-to-end latency for position estimation of UE (< 100 ms) - Physical layer latency for position estimation of UE (< 10 ms) - In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< 0.2 m) for 90% of UEs - Vertical position accuracy (< 1 m) for 90% of UEs - End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired) - Physical layer latency for position estimation of UE (<10ms) - Note 1: Target positioning requirements may not necessarily be reached for all scenarios and deployments - Note 2: For some scenarios the requirement for Horizontal position accuracy can be relaxed to < 0.5 m in IIoT use cases. - Note 3: All positioning techniques may not achieve the target positioning requirements over all scenarios

- Proposal 3.2 For the study of positioning performance of  RedCap UEs, the following accuracy requirements are used for performance evaluations:

  - IIOT: Horizontal position accuracy (< 0.2m) ,  vertical position accuracy (< 1 m) for [90%] of UEs

  - Commercial: Horizontal position accuracy (< 1m), vertical position accuracy (<  3m) for 90% of UEs

  - FFS: other requirements and use case

  - FFS:  whether latency requirements should be considered.

Companies are encouraged to comment on how to evolve the proposal in the table below, especially if further use cases and requirements should be considered.

Proposal 3.2:

| - Company | - comment |
| --- | --- |
| - CATT | - We prefer to use 1m as the requirement for IIoT horizontal positioning accuracy and remove the latency requirements. The updated proposal as follows, - Proposal 3.2 For the study of positioning performance of RedCap UEs, the following accuracy requirements are used for performance evaluations: - IIOT: Horizontal position accuracy (< 10.2m) , vertical position accuracy (< 1 m) for [90%] of UEs - Commercial: Horizontal position accuracy (< 1m), vertical position accuracy (< 3m) for 90% of UEs - FFS: other requirements and use case - FFS: whether latency requirements should be considered. |
| - Huawei, HiSilicon | - We think 0.2m may be too aggressive for a RedCap UE, while <1m horizontal provides a good starting point. - We prefer to have <1m for both IIoT and commercial requirement for a RedCap device. |
| - OPPO | - Since RedCap UE is low cost, it is no need to achieve the same positioning accuracy as normal UE. Thus, we suggest R16 positiong requirments for commercial use cases [TR38.855]: - Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios - Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios - Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios - Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios - For IIoT scenarios, we prefer to have 1m rather than 0.2m |
| - vivo | - The same view as oppo, it is no need to achieve the same positioning accuracy as normal UE. - In addition, for commercial use cases, wearables can be selected as a representative use case. The corresponding positioning requirements for wearables are defined in TR22.872, captured as follows. Therefore, horizontal position accuracy (< 2m), vertical position accuracy (< 3m) for 90% of UEs can be supported. Table 6.1-1 – Use cases synthesis Use cases; Potential requirements per use cases. Environment of Use; Position Accuracy; Velocity; Avail.; Update rate or interval; TTFF; Latency; Other KPI. 5.2.3; Wearables; 5G positioning service area - -Outdoor/Indoor; 2m Horizontal 1-3m Vertical; 90 %; 30s - 300s; 10s; Power saving mode. 5G positioning service area - -Outdoor/Indoor; 2m Horizontal 1-3m Vertical; 99 %; 1s - 30s; 10s; 1s; Normal mode. - So, we are okay to reuse the requirement of wearable or Rel-16 Requirement. In addition, we prefer to support LPHAP with high accuracy with 100MHZ other than RedCap UE. |
| - Qualcomm | - We prefer to not include the latency requirements. With regards to accuracy, we prefer to have <1m for both IIoT and commercial requirement for a RedCap device. |
| - ZTE | - We are open to study commercial use cases and IIOT use cases. A RedCap UE’s data processing capability (including the maximum bandwidth and the number of antenna and etc.) is limited and this has a strong impact on positioning accuracy. Therefore in our understanding, we prefer more relaxed requirements for RedCap UE as follows. - - IIOT: Horizontal position accuracy (< 0.21 m) , vertical position accuracy (< 1 m) for [90%] of UEs - - Commercial: Horizontal position accuracy (< 1 2m), vertical position accuracy (< 3m) for 90% of UEs |
| - Spreadtrum | - The centimeter level positioning accuracy is difficult for redcap. In our view, for IIOT, the requirement of positioning accuracy is set to be less than 1m. |
| - NEC | In our understanding, for RedCap UE with limited capability, sub-meter accuracy target might not realistic. We tend to agree with a more reasonable target, i.e., 1-3 m horizontal and vertical accuracy requirement for both use cases. |
| - LGE | - Regarding IIOT case, we have a similar view to majority companies who has concern about aggressive requirement. But, we are generally fine with 1m for both cases. Considering the different other companies’ views and the first meeting to discuss the issue, we prefer to use the ‘X’ and ‘Y’ instead of the specific value for horizontal accuracy for each cases as shown below, and then we prefer to define 'X' and ‘Y’ in the next meeting. - Regarding first FFS, does it means that RAN1 needs to introduce additional use case and define related requirements? If it is right, we are not supportive of the point since both two use cases are sufficient. - Regarding second FFS, we prefer to keep the FFS point considering that battery life is also critical point that needs to be discussed and it directly depends on latency. - So, we suggest modified proposal as follows - Proposal 3.2 For the study of positioning performance of RedCap UEs, the following accuracy requirements are used for performance evaluations: - IIOT: Horizontal position accuracy (< X0.2m) , vertical position accuracy (< 1 m) for [90%] of UEs - FFS: X - Commercial: Horizontal position accuracy (< Y1m), vertical position accuracy (< 3m) for 90% of UEs - FFS: Y - FFS: other requirements and use case - FFS: whether latency requirements should be considered. |
| - InterDigital | - We agree with the proposed target accuracy requirements for both use cases. In addition, we propose to evaluate whether the reduced UE BW affects the latency (For example ,RedCap UE needs to measure PRS longer than the Normal UE to achieve the same target accuracy). |
| - Nokia/NSB | - Agree with others than .2 m may be too tight of a requirement. < 1 m seems reasonable to start with. - Question for clarification: for commercial use cases do we have a particular scenario in mind (e.g., IOO or UMi) or is it a general requirement? Outdoors may be a challenging place to achieve sub-meter accuracy for RedCap UEs. |
| - Samsung | - We don’t see the need to set the requirement for redcap UE to be that high as IIOT in rel17. We’d like to see if there is evidence to support such case before supporting it. So far, we think commercial use case is enough. and we agree with CATT that latency seems not that critical for redcap UEs. - Proposal 3.2 For the study of positioning performance of RedCap UEs, the following accuracy requirements are used for performance evaluations: - IIOT: Horizontal position accuracy (< 0.2m) , vertical position accuracy (< 1 m) for [90%] of UEs - Commercial: Horizontal position accuracy (< [1]m), vertical position accuracy (< 3m) for 90% of UEs - FFS: other requirements and use case - FFS: whether latency requirements should be considered. |
| - Lenovo | - We prefer a uniform requirement for both IIoT and commercial devices for indoor scenario and for that <1m accuracy can be defined. Then <10m accuracy for outdoor as suggested by OPPO can be defined. |
| - Sony | - There should be separate requirements for IIoT and commercial as they are targeting different use-cases. 0.2 cm may be too tight for RedCap UE. It may be OK to relax the requirements. However, it should be reasonable for the supported use-cases. (e.g., [<1m] horizontal and vertical accuracy). |

## second round of discussion (stable)

all the companies commenting propose to relax the requirements to at least 1m instead of the proposed 0.2m. for commercial use cases, some companies want to increase the target for horizontal accuracy from 1m to at least 2, up to 3 m. LGE propose to keep the accuracy values open until the next meeting. Considering the workload of the SI, from the FL perspective it is preferred to close issues that can be closed during this meeting, since we have to run evaluations which will take time.

for this round, we can ask companies there preference for the accuracy of commercial cases, and revise the accuracy of IIOT to 1m. the latency FFS is also removed based on the comments from CATT and Qualcomm:

- Proposal 3.3 For the study of positioning performance of  RedCap UEs, the following accuracy requirements are used for performance evaluations:

  - IIOT: Horizontal position accuracy (< 1 m) ,  vertical position accuracy (< 1 m) for [90%] of UEs

  - Commercial: Horizontal position accuracy (< Y m), vertical position accuracy (<  3m) for 90% of UEs

      - FFS: Y

Companies are encouraged to comment on the revised proposal, and on what value should used for the horizontal accuracy for commercial use cases:

Proposal 3.3:

| - Company | - comment |
| --- | --- |
|  | Candidate value for Y |
| Samsung | Y could be 3m |
| Sharp | We are OK with the revised proposal. Y can be 1 – 3 m. |
| vivo | We still have some concerns about IIoT requirements, especially some companies concerned about whether Music can be used for redcap UE. As we know, the <1m accuracy can be achieved in Rel-17 is because of the super-resolution algorithm So, we think more evaluation and study are needed for determining the target, can we also put 1m for IIoT to FFS in this stage |
| CATT | Support. For the value of Y, we can live with Y=1 or Y=3. |
| OPPO | We are fine with the proposal and Y=3 |
| Qualcomm | Fine with the proposal. We can live with Y=1 or Y=3, even though we still prefer it should be Y=1. We are OK to leave Y FFS also for now. |
| ZTE | Maybe Y=2 or Y=3. As LGE proposed in the first round discussion, we also prefer to leave the value horizontal position accuracy of both IIOT and commertial use case as FFS. MediaTek questioned whether MUSIC can be used for RedCap UE during the GTW session, however, we also use MUSIC in our simulations and we need time to further study the reasonable requirements. - IIOT: Horizontal position accuracy (< 1 X m) , vertical position accuracy (< 1 m) for [90%] of UEs - FFS: X - Commercial: Horizontal position accuracy (< Y m), vertical position accuracy (< 3m) for 90% of UEs - FFS: Y |
| InterDigital | We support the proposal from the FL and Y= 1 or 3 is acceptable for us. |
| Huawei, HiSilicon | We are OK with proposal and do not think that 1m for IIoT should be reverted to X by ZTE. For Y values, we prefer 1, and we can accept to leave Y FFS. On the using MUSIC algorithm on RedCap UEs, which further impact the target requirement, we do not believe that should be the case. Requirement should not be dependent on the positioning method. For example, RedCap UE positioning may still use UL methods, where the computation is at network side. |
| Intel | OK; also, fine with either Y = 1 or 3. |
| NTT DOCOMO | OK with either Y = 1 or 3. We slightly prefer Y = 1. |
| LGE | We prefer to leave both X and Y for details as we mentioned in first round and want to discuss it in the next meeting. But, concerning the FL’s comment about closing the issue in the consideration of workload, if majorities are okay to discuss/decide the detail values, we will follow the majorities views. If we have to decide the value of Y, we slightly prefer Y=1. In addition, regarding following the second FFS in the first round, since we believe that the battery life is highly dependent on latency and there are no strong concerns about it, we prefer to keep it. - FFS: whether latency requirements should be considered. |
| Sequans | We support the proposal. Y between 1-3 seems reasonable. |
| Nokia/NSB | Support. |
| Ericsson | Support. For Y we’re ok with 1 meter as in rel17 |
| NEC | We are fine with the proposal, and Y could be 1-3m. |
| FL | Since almost companies are in agreement with the proposal, let’s try and bring it to an email agreement. We can settle Y value later. |

## Summary before May 19 GTW

the issue of horizontal accuracy for IIOT is still controversial, with concern due to computational complexity at the UE side to reach high accuracy.  According to the work plan, we are supposed to discuss and decide on evaluation assumptions during RAN1#109e, and the following 3 meetings are meant to be used to review evaluation results and discuss potential enhancements. Therefore, in order not to already deviate from the plan it would be beneficial to settle the issue this meeting. That being said, it is understandable that companies want to assess the feasibility of a requirement. Maybe one way forward is to have more than one requirement target for IIOT.

Since it does not look like we will agree to values for IIOT horizontal accuracy, we propose to start with ZTE’s proposal and have the following proposal discuss at the GTW.

- Proposal 3.4 For the study of positioning performance of  RedCap UEs, the following accuracy requirements are used for performance evaluations:

  - IIOT: Horizontal position accuracy (< X m) ,  vertical position accuracy (< 1 m) for [90%] of UEs

  - FFS: X

  - Commercial: Horizontal position accuracy (< Y m), vertical position accuracy (<  3m) for 90% of UEs

      - FFS: Y

## Status after GTW

Many companies have had concerns on setting a target accuracy for redcap UEs, especially for IIOT use cases. Checking the SID, the RedCap part of the SI is described as follow:

Justification: Release-17 has specified support for RedCap UEs with reduced bandwidth support and reduced complexity including reduced number of receive chains. Such UEs could support NR positioning functionality but there is a gap in that the core and performance requirements have not been specified for the positioning related measurements performed by RedCap UEs, and no evaluation was performed to see how the reduced capabilities of RedCap UEs might impact eventual position accuracy. This gap is to be investigated by the present SI. Objective: - Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

As can be seen , the SID does not define any requirements to be met. During this meeting, we have tried to set a list of target accuracies, i.e. what is the ambition for RedCap UEs positioning performance. The reasoning was that once we have the target accuracy we are aiming for, we can check the existing performance and if the target is not met, look at possible enhancements and decide whether these enhancements are feasible.

Since it looks like it is not possible to agree on target requirements, another approach would be to first evaluate the RedCap UEs in the agreed scenarios, and look at the gap in accuracy compared to rel16 and rel17 scenarios. We can then discuss which requirements should be met, i.e. for which requirements  it is worth closing the performance gaps with enhancments.

From the FL perspective, we will have to look at target requirements either now or after initial evaluation. Maybe knowing the basic performance of the redcap UE before any enhancements will help frame the discussion better.  My proposal is thus to close the performance target discussion for this meeting and open it again together with the base evaluation discussion. If companies think a different approach is required, please leave a comment below:

| - Company | - comment |
| --- | --- |

# Evaluation assumptions for Redcap UEs

## Summary of proposals

For evaluation assumptions,  several companies proposed different baselines to be used indoor and outdoor. For indoor, reusing the methodology for indoor factory scenario from rel17 (iiot scenario) and indoor office from rel16 (commercial scenarios) was proposed in  [3][9][11][12][13]. [22] propose to consider InH for indoor. For outdoor Commercial scenario baseline (using Umi as channel model) is discussed in [9][11][12][22]

In addition to rel16 and rel17 simulation scenarios and methodology,  [21][22] propose to introduce evaluation of 700MHz macro scenarios (Uma for [22], Rma for [21])

Several proposal mention specific parameters to be modified for redcap:

Bandwidth:

  - 20MHz in FR1 [1] [7][10][12][13][18][21][22]

  - 5MHz in FR1: [2]

  - 100 MHz in FR2 [7][12][18][22]

  - Use of redcap initial BWP for PRS/ SRS [4]

Number of Rx branches for redcap devices:

  - 1RX branch [1][4][6][2][7][9][10][11][13]

  - 2Rx branch[1][11]

  - Note: redcap Ues only have 1 Tx branch

Different positioning  methods have been mentioned, including DL AOD[12], UTDOA[1], DL TDOA[2] [3] [4] [7] [10] [11], E-CID[16] and multi-RTT.

Half duplex FDD is discussed in [4]

DRX impact is discussed in [4][15]

Recap reduced processing time[12]

Reference signals and their parameterization (e.g. PRS duration) are discussed in [16][17] [18]. From the FL perspective, it is proposed to leave it to proponent to detail the reference signal parameters in the evaluation, and leave RS details out of the methodology, as it was done during previous releases.

Based on the proposals, it is proposed to discuss the following:

- General consideration for the  UE and gNB simulation parameters

- Channel models and evaluation scenarios

- Bandwidths to be evaluated

- Number of Tx and Rx branches to be evaluated

- Positioning methods

- Reference signals

- DRX/power consumption

- Rel17 reduced redcap processing time

| - Source | - Proposal |
| --- | --- |
| - [1] | - Observation 1: The use cases of RedCap Ues (e.g., sensors, wearables) would require a sub-meter (< 1m in horizontal) positioning accuracy considering the reduced capabilities: - Bandwidth: 20MHz at FR1 - Antenna setting: 1Tx&1Rx, 1Tx&2Rx |
| - [2] | Proposal 2: RAN1 to consider evaluation assumption parameters of Table 1 as a baseline for accuracy performance evaluation of RedCap UE. |
| - [3] | Proposal 2: Suggest reusing Rel-17 Indoor Factory scenarios for the evaluation of the positioning performance of RedCap Ues. |
| - [4] | Proposal 3 - For power saving, positioning impacted by CDRX should be considered for RedCap positioning, including: - PRS measurement ehavior inside/outside drx-onDurationTimer or DRX active time. - LMF awareness of DRX configurations and DRX state change(e.g., short-long DRX cycle transition due to drx-ShortCycleTimer, etc.). - Related signaling and procedure. Proposal 4 - The following aspects can be considered for RedCap positioning, including: - Separated initial BWP support for PRS measurement and SRS transmission. - Priority/collison rules for DL PRS processing and SRS transmission when Half-duplex FDD is supported. - The impact of UE not supporting CA/DC. |
| - [6] | Proposal 1: Study the impact on the accuracy of the positioning with a bandwidth reduction and reduced Rx branches. |
| - [7] | Proposal 1: Study the impact of RedCap UE maximum bandwidth and minimum number of Rx-branch to the positioning accuracy. Proposal 2: RAN1 needs to define the simulation assumptions suitable for RedCap UE. |
| - [9] | Proposal 1: - The baseline application scenario for RedCap positioning is the commercial scenario - The industrial scenario can be considered for RedCap positioning as well Proposal 3: - Reuse the evaluation methodology defined in 38.855 and 38.857 for the evaluation of RedCap positioning - For assumption of RedCap devices - Reduced bandwidth is assumed - Both reduced Rx branch and normal Rx branch is assumed |
| - [10] | Proposal 3: RAN1 to study the impacts to measurements caused by reduced BW and antenna number. |
| - [11] | Proposal 2: To evaluate positioning performance of RedCap Ues, support the following scenarios and evaluation assumptions: - For commercial use cases, select one or two of the following cases defined in Section 6.1 of TR 38.855 - Case 1: Indoor Office - Case 2: Umi street canyon - For IioT use cases, select one or two of the following cases defined in Section 6 of TR 38.857 - Case 3: InF-SH - Case 4: InF-DH - FR1 is the first priority - For the detailed information of evaluation assumptions for each case, refer to TR 38.855 and TR 38.857. |
| - [12] | Observation 1: The effect of reduced bandwidth for RedCap Ues should be investigated for both commercial and IioT applications Proposal 1: For system parameters for SLS evaluation of positioning accuracy, use the evaluation assumptions in TR 38.875 as the baseline Proposal 2: For DL-based positioning methods, use 20MHz for FR1 and 50MHz or 100MHz for FR2 for PRS bandwidth. Proposal 4: Study the effect of reduced UE processing time on the operation of the PRS processing window Proposal 5: Study the effect of reduced UE bandwidth on latency for positioning |
| - [13] | Observation 1: Of the device capability differences between Rel-16 baseline Ues and RedCap Ues, only the maximum bandwidth and the number of Rx antenna branches affect the positioning performance. Proposal 1: For the RedCap UE evaluation, set the bandwidth of the Redcap devices to 20 MHz and the Number of Rx antennas to 1. Proposal 2: RAN1 to set a common evaluation methodology and common evaluation parameters based on the parameters in 38.855 [4] and 38.857 [3]. - it is necessary to agree on the simulation assumptions for the configuration of the positioning reference signals |
| - [16] | Proposal 1: - The accuracy of DL positioning should be evaluated first for RedCap Ues in Rel-18. Proposal 2: - The followings are proposed for the evaluation of ECID of RedCap Ues: - The number of gNBs for the position calculation of an outdoor UE should be evaluated in the urban scenario. - The performance enhancement for NloS scenario should be considered with ECID. Proposal 3: - When the requirements of positioning accuracy and latency are not high, PSS, SSS, PBCH DMRS should be used for positioning. The performance of positioning with PSS, SSS, PBCH DMRS should be evaluated. |
| - [17] | Proposal #1: - Discuss which of the reduced feature among RedCap UE impacts the performance of positioning in terms of accuracy / latency / resource efficiency. Proposal #2: - Discuss whether and how to configure PRS resources for RedCap Ues. |
| - [18] | Proposal 4: Evaluate RedCap positioning with reduced bandwidths e.g., 20MHz for FR1 and 100MHz for FR2 with reduced Rx antenna/RF chain of 1 for link budget evaluation. Proposal 5: Evaluate and study the positioning performance of RedCap devices with longer PRS symbol lengths, e.g., 12 to support RedCap devices. |
| - [21] | Proposal 1 Evaluation scenarios for RedCap positioning include InF, IOO, Umi, Uma and Rma. Proposal 2 Evaluate DL-TDOA and multi-RTT for RedCap positioning, with high priority. Proposal 3 For performance evaluation of RedCap positioning, • maximum UE bandwidth is 20MHz in FR1 and 100MHz in FR2, • minimum antenna configurations are one Rx branch and one Tx branch in FR1 and (Mg, Ng, M, N, P) = (1, 1, 1, 2, 2) with PC7 in FR2, • maximum antenna configurations are two Rx branches and two Tx branches in FR1 and (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) with PC3 in FR2, and • minimum EIRP in FR2 is 17dBm for RedCap Ues with PC7 and 22.4dBm for RedCap Ues with PC3. • Note: performance evaluation for RedCap Ues with PC3 in FR2 is optional. Proposal 4 Carrier frequencies are 700MHz for Rma and 3.5GHz for other scenarios in FR1, and 28GHz in FR2. Proposal 5 SCS can be 15kHz for Rma and 30kHz for other scenarios in FR1, and 120kHz in FR2. Proposal 6 UE distribution in Umi and Uma is 80% indoor with the speed of 3km/h and 20% outdoor in cars with the speed of 30km/h. Proposal 7 UE distribution in Rma is 50% indoor with the speed of 3km/h and 50% outdoor in cars with the speed of 120km/h. Proposal 8 Use the simulation assumptions listed in Table 2 for performance evaluation in Rma. |
| - [22] | Proposal 2: For TDD scenarios, - We propose to reuse NR Rel-16/17 Evaluation assumptions for InH, and UMI FR1 scenarios as agreed in RAN1 #94-Bis for 4 GHz and 2 GHz. - Support adding optionally to evaluate the UMI FR1 cases with DeltaTau modeling similar to the one introduced for InF scenarios. - For FR1 FDD scenario introduce a new Urban Macro at 700 MHz with 500m ISD according to the simulation assumptions shown below. - For FR2 TDD Redcap, reuse the NR Rel-16/17 FR2 simulation assumptions with 100 MHz PRS bandwidth. |

## First round of discussion (closed)

### Channel models and evaluation scenarios

We propose to start the discussion by identifying which channel model and scenarios should be evaluated. Once the channels are agreed, we can proceed with the details regarding the deployment. Since a majority of companies mentioned that the previous releases scenarios can be reused for evaluation of RedCap Ues, we can start from these scenarios and see if modifications are needed.

Question 4.2.1: which of the following scenarios can be evaluated for positioning performance of Redcap Ues (please also mark which case could be optional and which case should be mandatory/baseline). Please also state which modifications should be made to the scenarios, if any.

Outdoor:

- Case 1: Umi street canyon , as described in Table 6.1-1-4 of 38.855

- Case 2: Uma , as described in Table 6.1-1-6 of 38.855

- Case 3: Rma (FFS details of the scenario)

Indoor:

- Case 4: InF-SH and DH as described in Table 6.1-1 of 38.857

- Case 5: Indoor Open  Office, as described in Table 6.1-1-3 of 38.855

Companies are encouraged to provide their view on which channel model should be evaluated

Question 4.2.1

| - Company | - comment |
| --- | --- |
| - CATT | - We prefer to use Case 4 for the baseline. - Other cases can be optional. |
| - Huawei, HiSilicon | - We think that InF and IOO could be priorized assuming no GNSS is available. |
| - OPPO | - We pefer Case 4 as the baseline and also support Case 1 and Case 5 as optional. |
| - vivo | - We support Case 4 as baseline for evaluation. We can focus on InF scenarios for RedCap positioning, since there are relatively complete assumptions (e.g., the excess delay ∆τ modeling for NLOS, Rx/Tx timing error modeling, etc.) and the majority of companies have provided evaluation results with larger bandwidth in this scenario. It is helpful to avoid taking too much effort on too many scenarios. |
| - Qualcomm | - We think that there should be one baseline for indoor and one for outdoor redcap devices. There are many outdoor cases, that GNSS may not be available, (UMI) is a typical scenario. - For outdoor, we support Case 1 as baseline, and case 2 as optional. We don’t think that Rma is needed to be evaluated. - For indoor, we also support case 4 as baseline and case 5 as optional. |
| - ZTE | - Prioritize Case 4. |
| - Apple | - Case 1 and Case 4 |
| - Spreadtrum | - We prefer Case 4. |
| - NTT DOCOMO | - Prioritize Case 4. |
| - LGE | - We have similar view with Qualcomm. Currently, we have been discussed two use cases and related requirements. So, we think RAN1 needs to select each case for scenarios respectively. For out door, we support case 2 (e.g. Uma) as base line since the channel model is normally used in both 6GHz and 30 GHz, but not for Umi. For indoor, we support case 4 as base line in the consideration of Rel-17. |
| - InterDigital | - For baseline evaluations, we propose to evaluate Case 4: InF-SH and DH scenarios. |
| - Nokia/NSB | - We support case 4 as the baseline. We are also supportive of UMi as an outdoor use case. Do we only want to investigate 100% outdoor UEs in UMi? We would also like to clarify if the <1 m requirement applies to UMi or not. |
| - Samsung | - If Redcap UE is considered more like the wearable device, we think case 1 or case 2 should be the baseline instead of indoor factory. |
| - Lenovo | - We prefer to define one outdoor and one indoor for evaluation such as case 1 and case 4 |
| - Sony | - Prioritize Case 4 |
| - Ericsson | - Indoor: case 1 baseline, other can be optional. - Outdoor: case 4 baseline, other can be optional. |

### General consideration for the  UE and gNB simulation parameters

Based on the received proposal, we can re-use some of the common scenario parameters from rel17 as proposed by many companies.  The proposal below is based on table 6.1 in 38.857, with the UE antenna model removed from the table in order to discuss it separately. Some of the parameters are marked as TBD due to the dependency on agreed simulation bandwidth. 700MHz carrier is added as an FR1 scenario.

- Proposal 4.2.2 For evaluation of positioning performance of redcap UEs, adopt the general parameters are detailed in the table below

  - TBD  parameters are discussed separately

Table 6-1: Common scenario parameters applicable for all scenarios for Redcap Ues evaluations

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz, 700MHz Note 1 | 28GHz Note 1 |
| Bandwidth, MHz | TBD | TBD |
| Subcarrier spacing, kHz | 30KHz, 15KHz (for 700MHz carriers) | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| UE/gNB RX and TX timing error | (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: X ns for gNB and Y ns for UE - X and Y are up to sources - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. |  |
| Note 1: According to TR 38.802 Note 2: According to TR 38.901 |  |  |

Companies are encouraged to provide their view  the common scenario parameters , and if any, any additional common parameters to be added:

Proposal 4.2.2

| - Company | - Comment |
| --- | --- |
| - CATT | - Support |
| - Huawei, HiSilicon | - We do not consider high priority for 700MHz under Uma/Rma. |
| - OPPO | - Support. One value for carrier frequency in FR1 is sufficient and we prefer 3.5GHz |
| - vivo | - In FR1, we prefer 3.5GHz as the baseline, which aligns with our assumptions in R16/17. |
| - Qualcomm | - We are OK to have 3.5 Ghz as baseline, but 700 MHz outdoor is a very useful scenario also. Redcap devices are not only TDD, there is a good market for FDD type of devices, and it should be added in this release, and evaluated. - So, the outdoor scenarios (e.g.., UMI/Uma) the 700 MHz scenario would also need to be baseline. Note a different gNB antenna configuration may be needed though compared to the 3.5 GHz Uma/Uma. We propose to use: - (M, N, P, Mg, Ng) = (8, 4, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, 8 TXRUs |
| - ZTE | - Support. |
| - Apple | - Support this proposal |
| - Spreadtrum | - Support. |
| - NTT DOCOMO | - Support. |
| - LGE | - Support. |
| - Nokia/NSB | - We are not sure that we need to evaluate Tx/Rx timing errors in Rel-18. We already have a solution for that in Rel-17 and not sure there are specific issues related to RedCap for that (especially given reduced Tx/Rx chains). Unless companies can clarify why this is important to evaluate for RedCap UEs we suggest to remove it entirely. Otherwise we are supportive. |
| - Samsung | - ok |
| - Lenovo | - We support 3.5GHz as baseline for evaluation as it is aligned with NR positioning baseline simulation |
| - Sony | - Support |

### Bandwidths to be evaluated

Contributions have mentioned 5MHz and 20MHz bandwidth to be evaluated for FR1 and 100MHz for FR2. Companies are asked which of these bandwidth should be, if any, optional and which should be a mandatory evaluation:

Question 4.2.3: which of the UE bandwidth configuration should be evaluated?  (please also mark which case could be optional and which case should be mandatory/baseline)

  - FR1, 20MHz

  - FR1 5MHz

  - FR2 100MHz

  - other

Companies are encouraged to provide their view on which bandwidth should be evaluated, and if so,  whether it should be mandatory/optional:

Question 4.2.3

| - Company | - comment |
| --- | --- |
| - CATT | - Support the following two options as the baseline, considering the capability of RedCap UEs: - FR1, 20MHz - FR2 100MHz |
| - Huawei, HiSilicon | - FR1 20MHz should first priority. |
| - OPPO | - FR1: 20MHz - FR2: 100MHz - We prefer FR1 as the first priority |
| - vivo | - In FR1, we prefer 20MHz as baseline. |
| - Qualcomm | - FR1: 20 Mhz as baseline - FR2: 100 MHz baseline - We don’t agree on prioritization between FR1 and FR2. Each FR, could have a baseline bandwidth, but it is not good for the discussion and the Redcap devices ecosystem to explicitly write that FR2 is lower priority. |
| - ZTE | - Prioritize 20MHz in FR1. |
| - Apple | - FR1: 20 MHz - FR2: 100 MHz. |
| - Spreatrum | - We prefer FR1 with 20MHz as the first priority. |
| - NTT DOCOMO | - FR1: 20 MHz - FR2: 100 MHz. |
| - NEC | - Support FR1 20MHz with first priority. |
| - LGE | - We prefer FR1 with 20MHz as the first priority. Considering dedicated spectrum less than 5MHz for FR1 in Rel-18, we are okay with 5Mhz as second priority. |
| - InterDigital | - For baseline UE bandwidth configuration, use 20MHz for FR1 and 100MHz for FR2 for PRS bandwidth. |
| - Nokia/NSB | - We agree that for FR1 20 Mhz should be top priority but feel it is worthwhile to optionally evaluate 5 Mhz as well. For FR2 we also support 100 Mhz as top priority. |
| - Samsung | FR1, 20MHz - FR2 100MHz |
| - Lenovo | We prefer to simulate FR1 first with 20Mhz bandwidth, FR2 100Mhz is lower priority |
| - Sony | Support FR1 20MHz and FR2 100MHz as the baseline. |

### UE antenna configuration and Number of Tx and Rx branches to be evaluated

Based on the proposals, it is proposed to update the UE antenna configuration table from 38.855 to adapt it to Redcap UEs. The majority of proposals suggest  using 1 Rx branch as a baseline, but some companies also propose to use 2Rx which we could have as optional.  For the antenna configuration, some companies have mentioned either (Mg, Ng, M, N, P)= (1, 1, 1, 2, 2) (i.e. minimum antenna configuration), other also included (1, 1, 2, 2, 2). We propose to have the minimum configuration as baseline as a starting point.

- Proposal 4.2.4-1 Adopt the following table for th UE model parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| UE model parameters |  |  |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | See proposal 4.2.4-2 below |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| Number of UE branches | Baseline: 1Rx 1Tx Optional: 2Rx 1 Tx |  |

Companies are encouraged to provide their view on the proposed UE model parameters below:

Proposal 4.2.4-1

| - Company | - Comment |
| --- | --- |
| - Huawei, HiSilicon | - (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) for 1Rx - (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) for 2Rx - Only one Tx is used for 2Rx UE. |
| - OPPO | - Support Huawei’s revision |
| - Qualcomm | - OK with HW revision |
| - ZTE | - Agree with Huawei’s modification. |
| - Apple | - Fine with Huawei’s revision |
| - Spreadtrum | - Agree with HW revision. |
| - NEC | - Fine with Huawei’s revision |
| - LGE | - Agree with HW’s suggestion. |
| - Samsung | - Fine, but even the 2rx optional is needed for redcap UE? But it’s optional anyway. |
| - Lenovo | - Fine with HW revision |
| - Sony | - Okay with HW’s revision |

- Proposal 4.2.4-2 For the FR2 UE antenna configuration, select between the following options:

- Option1: as in 38.855:

Multi-panel Configuration 1 and Panel Configuration a – Note 1

-	Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0)

-	Panel Configuration a:

-	Each antenna array has shape dH=dV=0.5λ

-	Config a: (M, N, P) = (2, 4, 2),

-	the polarization angles are 0° and 90°

-	The antenna elements of the same polarization of the same panel is virtualized into one TXRU

- Option 2: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) as minimum antenna configuration (baseline) and (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1) as optional configuration.

Companies are encouraged to provide their view on the proposed FR2 UE configuration below:

Proposal 4.2.4-2:

| - Company | - Comment |
| --- | --- |
| - vivo | - Option 1. A typo, ‘Option 1: as in 38.3855’. - FL: corrected. Thanks for noticing. |
| - Qualcomm | - Option 1 |
| - Apple | - Option 2 |
| - Nokia/NSB | - Option 2. |
| - Lenovo | - Option 2 |
| - Sony | - Option 2 |

### Positioning methods (closed - stable)

All positioning methods used in previous releases have been mentioned in the contributions. It is proposed to leave to proponents to detail the positioning method used to evaluate RedCap UEs performances in their evaluations.

- Proposal 4.2.5: For evaluation of RedCap UE positioning performances, all RAT based positioning methods can be considered. Sources should detail the chosen method(s) when presenting performance evaluations.

Companies are encouraged to provide their view on the proposal below:

Proposal 4.2.5:

| - Company | - Comment |
| --- | --- |
| - Huawei, HiSilicon | - Support. |
| - OPPO | - OK |
| - vivo | - OK |
| - Qualcomm | - OK |
| - ZTE | - Support. |
| - Apple | - Support |
| - Spreadtrum | - Support. |
| - NTT DOCOMO | - Support. |
| - NEC | - Support |
| - LGE | - Okay. |
| - Nokia/NSB | - Okay. |
| - Samsung | - Fine. Reported by companies. |
| - Lenovo | - support |
| - Sony | - Support |
| - FL | - Since the proposal is agreeable, we can close the discussion and bring it up to become an agreement. |

### Reference signals

Some companies have proposed parameters for the reference signals. As in previous release, we propose to agree  that the evaluation methodology does not define any baseline reference signals, and sources can detail the used reference signal parameters in their evaluation.

- Proposal 4.2.6 	The evaluation methodology for RedCap UEs positioning performance does not define any baseline reference signals. Sources should detail the chosen reference signal(s) when presenting performance evaluations.

Companies are encouraged to provide their view on the proposal below:

- Proposal 4.2.6

| - Company | - Comment |
| --- | --- |
| - Huawei, HiSilicon | - Support. |
| - OPPO | - OK |
| - vivo | - OK |
| - Qualcomm | - OK |
| - ZTE | - Support. |
| - Apple | - Okay |
| - Spreadtrum | - Support. |
| - NTT DOCOMO | - Support. |
| - NEC | - Support |
| - LGE | - Support. |
| - Nokia/NSB | - We are not opposed to evaluating other reference signals but what is the advantage of not using SRS/PRS as the baseline? We should aim at minimum specification impact in our view and those are the reference signals currently specified for RAT-dependent techniques. |
| - Samsung | - Actually both UL/DL RS should be encouraged to check the impact on the reduced capabililty. |
| - Lenovo | - support |
| - Sony | - We have similar views as NOKIA. We think, we should consider the legacy reference signals as the baseline |
| - FL | - To Nokia, Sony. From the FL perspective, the proposal is only to simplify the discussion on the evaluation assumptions, and just leave it to sources to provide their RS configuration. using new reference signals (i.e. not SRS or PRS) should fall under enhancements. |

### DRX/power consumption(closed)

A few companies have discussed power consumption and the impact of DRX on the performance.  Since there are only a few contribution mentioning the issue, it is propose to first ask whether the issue should be discussed in RAN1 for this meeting.

Question 4.2.7: should the evaluation of RedCap UE positioning performance include DRX and power consumption aspects?

Question 4.2.7

| - Company | - Comment |
| --- | --- |
| - Huawei, HiSilicon | - We think that power related evaluation can be done in LPHAP AI. |
| - OPPO | - No |
| - Qualcomm | - No |
| - ZTE | - Deprioritize the DRX and power consumption evaluation. |
| - Apple | - No |
| - Spreadtrum | - Low priority. |
| - NTT DOCOMO | - Our preference is to discuss DRX and power consumption evaluation because study in LPHAP AI is limited to the enhancements in RRC_INACTIVE and/or RRC_IDLE state. However, we are OK to deprioritize it if many companies think it is enough to discuss in 9.5.2.3. |
| - LGE | - We have a similar view with DOCOMO. Even though power consumption is dealt with in LPHAP AI., it would not be for RRC connected state. We think that discussion on positioning measurement with DRX for connected state can be discussed in this agenda. But, if majorities are not agreeable, we are okay to let the issue as low priority. |
| - Nokia/NSB | - Low priority. LPHAP can investigate this angle. |
| - Samsung | - Deprioritized. |
| - Lenovo | - Not needed |
| - Sony | - No (Beyond the scope) |
| - FL | - Based on the comment, we can de-prioritize the issue and close it in this AI. |

### Rel17 reduced redcap processing time(closed)

One company mentioned the impact of redcap UE processing time in [12]. Since no other companies have raised the issue, we can start by asking whether to pursue the issue during the study:

Question 4.2.8: should the evaluation of RedCap UE positioning performance include UE processing time aspects?

| - Company | - Comment |
| --- | --- |
| - Huawei, HiSilicon | - We think it should be low priority. |
| - vivo | - No |
| - Qualcomm | - No |
| - ZTE | - Deprioritize. |
| - Apple | - No |
| - Spreadtrum | - Low priority. |
| - NTT DOCOMO | - Low priority. |
| - LGE | - No. |
| - Nokia/NSB | - Low priority. |
| - Samsung | - Deprioritized. |
| - Lenovo | - Not needed |
| - Sony | - Low priority |
| - FL | - Based on the comment, we can de-prioritize the issue and close it in this AI. |

## Second round  of discussion (closed)

### Channel models and evaluation scenarios

Based on the first round of comments, the following cases are preferred, for outdoor and indoor:

Outdoor:

- Case 1: Umi street canyon , as described in Table 6.1-1-4 of 38.855

    - Qualcomm, Apple, Nokia, Samsung, Lenovo, Ericsson

- Case 2: Uma , as described in Table 6.1-1-6 of 38.855

  - LGE, Samsung

  - Optional: Qualcomm , ericsson

- Case 3: Rma (FFS details of the scenario)

  - Optional: Ericsson

Indoor:

- Case 4: InF-SH and DH as described in Table 6.1-1 of 38.857

  - CATT, Huawei, OPPO, vivo, Qualcomm, ZTE, Spreadtrum, NTT DOCOMO, LGE, InterDigital, Nokia/NSB, Lenovo, Sony, Ericsson

- Case 5: Indoor Open  Office, as described in Table 6.1-1-3 of 38.855

  - Huawei

  - Optional: OPPO, Qualcomm, Ericsson

Based on the expressed preferences, case 4 can be chosen  as baseline indoor case, and case 1 as baseline outdoor case. Other use cases can be made optional.

- Proposal 4.3.1 the following scenarios are evaluated for positioning performance of Redcap

  - Baseline outdoor: (Case 1): Umi street canyon , as described in Table 6.1-1-4 of 38.855

  - Optional outdoor:

    - (Case 2): Uma , as described in Table 6.1-1-6 of 38.855

    - (Case 3): Rma (FFS details of the scenario)

  - Baseline indoor : (Case 4): InF-SH and DH as described in Table 6.1-1 of 38.857

  - Optional indoor: (Case 5) Indoor Open  Office, as described in Table 6.1-1-3 of 38.855

Companies are encouraged to provide  their views on the revised proposal below:

Proposal 4.3.1

| - Company | - comment |
| --- | --- |
| - Samsung | - Generally fine. - Just one question, is it necessary to put both SH and DH, to ease the work load, we could put SH as baseline and DH to be optional. |
| - Sharp | - Support |
| - vivo | - If Los and Nlos identification isn’t the critical issue for Redcap UE and it can be assumed that is resolved by Rel-17 enhancement, can we take InF-SH only as a baseline, and InF-DH can be optional for workload reduction in the InF scenario? |
| - CATT | - Support - We can live with only InF-SH is selected as baseline indoor to reduce the workload. |
| - OPPO | - Support. Samsung’s suggestion is also fine to us. |
| - Qualcomm | - We also think that only one of SH and DH is needed. OK to keep only InF-SH as baseline. |
| - ZTE | - Support in general and treat InF-SH as baseline. |
| - InterDigital | - We support the FL’s proposal. A suggestion from Samsung is acceptable for us. |
| - Huawei, HiSilicon | - Why is the baseline selected from indoor/outdoor, instead of from IIoT/commercial when the corresponding requirements are defined? - From our side, we only accept InF-SH as the baseline. Outdoor should not consider any baseline evaluation. |
| - Intel | - Support the proposal and the suggestion from Samsung. |
| - Apple | - Fine with the proposal. Support Samsung’s suggestion. |
| - NTT DOCOMO | - Support the proposal with Samsung’s suggestion. |
| - LGE | - Fine with Samsung’s suggestion. |
| - Nokia/NSB | - Agree with Samsung’s suggestion. |
| - Ericsson | - Support. Agree with Samsung that SH would be enough for indoor baseline. |

### General consideration for the UE and gNB simulation parameters

Based on the received comments, we can keep the common scenario parameter table, but put the 700MHz carrier case as optional since some companies want to down prioritize it. For the 700MHz gNB antenna parameters, we have a separate proposal.  regarding the use of timing error models, there is only one company wanting to remove the model. However, considering we also want to relax the accuracy based on the comments received on the target accuracy for RedCap UEs discussion, we could perhaps remove the model. Since only one company has commented on removing timing error models so far, we can leave them on and wait for more feedback.

- Proposal 4.3.2-1 For evaluation of positioning performance of redcap UEs, adopt the general parameters are detailed in the table below

  - TBD  parameters are discussed separately

Table 6-1: Common scenario parameters applicable for all scenarios for Redcap Ues evaluations

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz, 700MHz (optional) Note 1 | 28GHz Note 1 |
| Bandwidth, MHz | TBD | TBD |
| Subcarrier spacing, kHz | 30KHz, 15KHz (for 700MHz carriers) | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| UE/gNB RX and TX timing error | (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: X ns for gNB and Y ns for UE - X and Y are up to sources - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. |  |
| Note 1: According to TR 38.802 Note 2: According to TR 38.901 |  |  |

Companies are encouraged to provide their view  on the parameter table above, and particularly on the use of the timing error model in the proposed parameter table.

Proposal 4.3.2-1

| - Company | - Comment |
| --- | --- |
| - Samsung | - Fine for us. |
| - Sharp | - OK |
| - OPPO | - Support |
| - Qualcomm | - We would like to have a baseline for TDD and a baseline for FDD. So we don’t think that 700 MHz should be optional. |
| - ZTE | - OK. |
| - Huawei, HiSilicon | - OK. 700MHz should be optional. |
| - Intel | - OK |
| - Apple | - Support |
| - LGE | - Agree. |
| - Sequans | - Support in general, but agree with Qualcomm about usefulness to evaluate 700MHz case |
| - CATT | - OK |
| - Nokia/NSB | - Support. |
| - Ericsson | - Support |
| - FL | - Since there is a lot of support let’s bring the agreement to email to see if a compromise is possible. |
| - FL | - Proposal agreed in email discussion. |

- Proposal 4.3.2-2 For evaluation of positioning performance of redcap UEs in 700MHz band, the gNB antenna is modeled as (M, N, P, Mg, Ng) = (8, 4, 2, 1, 1),  (dH, dV) = (0.5, 0.8)λ, 8 TXRUs

Proposal 4.3.2-2

| - Company | - Comment |
| --- | --- |
| - vivo | - Since 700 M is suggested as optional and no related channel model has been provided in NR, can we leave companies to provide the detailed simulation parameter？ |
| - Qualcomm | - OK |
| - Huawei, HiSilicon | - Is there any reference to have such a deployment? It appears that a 700MHz gNB has 64 Tx and 8 TXRU? We do not think that it is realistic. How to virtualize the 64 Tx to 8 TXRU is up to each company? |
| - CATT | - We prefer to list a reference for such 700MHz configuration, which will benefit the assessment for companies. |
| - Qualcomm | - To Huawei, HiSilicon: IMT-2020 config-A is the one above: Config. A. Carrier frequency for evaluation; 700 MHz. BS antenna height; 35 m. Total transmit power per TRxP; 46 dBm for 10 MHz bandwidth. UE power class; 23 dBm. Percentage of high loss and low loss building type; 100% low loss (applies to Channel model B). Inter-site distance; 1732 m. Number of antenna elements per TRxP; 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization. Number of TXRU per TRxP; 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1). |
| - ZTE | - For the evaluation methodology of Rel-17 redcap UE, the Rel-17 Coverage Enhancement SI assumptions on gNB antenna configuration are reused according to TR 38.875. TR 38.875 The assumptions in the Rel-17 Coverage Enhancement SI regarding link budget template and antenna array gain are reused [5]. Furthermore, the Rel-17 Coverage Enhancement SI assumptions on gNB antenna configuration, # gNB Tx and Rx chains, channel model and delay spread are reused, with the revision or addition shown in Table 6.3-1. [5] 3GPP TR 38.830: Study on NR coverage enhancements. - Therefore, we prefer to reuse the gNB antenna configuration from TR38.830 rather than IMT-2020 for redcap UE. - When carrier frequency is equal to 700MHz, the gNB antenna model for RedCap UE is as follows: Number of antenna elements for BS; - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1). Number of TxRUs for BS; gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled.. |

### Bandwidths to be evaluated (closed)

It seems all companies agree that the 20MHz in FR1 and 100MHz in FR2 bandwidth should be evaluated as baseline. Some companies have also voiced interest in the 5MHz bandwidth as a second priority. It is therefore proposed to have it as optional.

- Proposal 4.3.3 For the evaluation of RedCap positioning,  the following bandwidth can be evaluated:

  - FR1: 20MHz baseline, 5MHz optional

  - FR2: 100MHz

Companies are encouraged to provide their view on the proposal in the table below

Proposal  4.3.3

| - Company | - comment |
| --- | --- |
| - Samsung | - Fine. |
| - Sharp | - Support. |
| - CATT | - Support |
| - OPPO | - Support |
| - Qualcomm | - Support |
| - ZTE | - Fine with Proposal 4.3.3 if giving priority to FR1 which we still prefer is not possible. Normally for FR1, the positioning performance is quite insufficient because of limited bandwidth and we should take time to evaluate it and maybe also study its potential enhancement. The problem is not that urgent for FR2. But we understand that evaluating positioning performance in FR2 may also be needed for RedCap positioning. |
| - InterDigital | - We support the FL’s proposal. |
| - Huawei, HiSilicon | - We share similar view to ZTE. - We think that FR2 should be in general optional. |
| - Intel | - OK |
| - Apple | - Fine with the proposal |
| - NTT DOCOMO | - Fine with the proposal. |
| - LGE | - support the FL’s current proposal. |
| - Sequans | - Support 20 MHz as priority for FR1. 5MHz FR1 case could also be more interesting to evaluate from 100MHz FR2 |
| - Qualcomm2 | - We don’t agree on adding the tag “Optional” in the FR2. |
| - Nokia/NSB | - Support. |
| - Ericsson | - Support |
| - NEC | - Support |
| - FL | - Based on the support, we can check if the proposal can be agreed. For companies wanting to have evaluation in FR2 optional, hopefully a compromise is possible. |
| - FL | - Proposal agreed in email discussion. |

### UE antenna configuration and Number of Tx and Rx branches to be evaluated

For the FR1 UE antenna configuration, the table is revised according to Huawei’s comment. For the FR2 antenna configuration, the opnions are split, with few companies commenting. The proposal is repeated to collect more views, with the typo mentioned by vivo corrected.

- Proposal 4.3.4-1 Adopt the following table for th UE model parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| UE model parameters |  |  |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, for 1Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) for 2Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) | See proposal 4.3.4-2 below |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| Number of UE branches | Baseline: 1Rx 1Tx Optional: 2Rx 1 Tx |  |

Companies are encouraged to provide their view on the proposed UE model parameters below:

Proposal 4.3.4-1

| - Company | - Comment |
| --- | --- |
| - Samsung | - Fine for us. |
| - Sharp | - OK |
| - vivo | - OK |
| - OPPO | - Support |
| - Qualcomm | - OK |
| - ZTE | - OK |
| - Huawei, HiSilicon | - OK. |
| - Intel | - OK |
| - Apple | - OK |
| - LGE | - OK. |
| - Sequans | - OK |
| - CATT | - Support the proposal in principle with the following revision: - Regarding the last row: “Number of UE branches”, we prefer to list the number in two column as similar to the first and the second rows, i.e., the first column for FR1 and the second column for FR2. |
| - Nokia/NSB | - Okay |
| - Ericsson | - Support |
| - NEC | - Support |
| - FL | - Since there is a lot of support, let’s see if it can br brought to agreemnent by email, with the suggestion by CATT: - Proposal 4.3.4-1a Adopt the following table for th UE model parameters FR1 Specific Values; FR2 Specific Values. UE model parameters. UE antenna configuration; Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, for 1Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) for 2Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1); See proposal 4.3.4-2 below. UE antenna radiation pattern; Omni, 0dBi; Antenna model according to Table 6.1.1-2 in TR 38.855. Number of UE branches; Baseline: 1Rx 1Tx Optional: 2Rx 1 Tx; TBD. |
| - FL | - Proposal agreed in email discussion. |

- Proposal 4.3.4-2 For the FR2 UE antenna configuration, select between the following options:

- Option1: as in 38.855:

Multi-panel Configuration 1 and Panel Configuration a – Note 1

-	Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0)

-	Panel Configuration a:

-	Each antenna array has shape dH=dV=0.5λ

-	Config a: (M, N, P) = (2, 4, 2),

-	the polarization angles are 0° and 90°

-	The antenna elements of the same polarization of the same panel is virtualized into one TXRU

- Option 2: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) as minimum antenna configuration (baseline) and (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1) as optional configuration.

Companies are encouraged to provide their view on the proposed FR2 UE configuration below:

Proposal 4.2.4-2:

| - Company | - Comment |
| --- | --- |
| - Samsung | - To clarify, is the proposal to ask companies to select one now or select one during the simulation and reported which one is selected? - In which case, we have slight preference on option 2. |
| - OPPO | - Does it mean which option used in the evaluation is up to each companies? |
| - Huawei, HiSilicon | - We think FR2 RedCap antenna deployment can be up to each company to report. |
| - Intel | - Perhaps we are missing something, but why are considering multi-panel antenna configurations for RedCap UEs as in Option 1? |
| - LGE | - If our understading is right, we have to select one of both options. Regarding option #1, we have a similar question with Intel. If it is supported for RedCap UEs, we need to check it out first. |
| - CATT | - We prefer Option 2, since Option 2 aligns with the R17 RedCap UE minimum/maximum antenna configurations. - For Option 1, it seems that “Multi-panel Configuration 1 or Panel Configuration a” cannot be supported by R17 RedCap UEs. |
| - Qualcomm | - From our understanding, in Rel-17, a device could have Multi-panels, even though indeed, in FR2 RAN4 there is a dedicated Redcap 'low power' UE which is nominally one panel. We are OK to use the 38.855 asumptions (Option 1) if it simpler for companies, but we would accept Option 2 also. |
| - Ericsson | - Preference with option 2 , but if needed to get an agreement we can include option 1 as another optional configuration. |
| - ZTE | - We prefer Option 2. |
| - FL | - Regarding the choice of options, it would be best to simplify the evaluation and select one of the options now for future evaulations, if that can be agreed. |

### Reference signals

The discussion is almost stable, with two companies commenting on using legacy reference signals. From the FL perspective, the proposal is only to simplify the discussion on the evaluation assumptions, and just leave it to sources to provide their RS configuration.   Using new reference signals (i.e. not SRS or PRS) should fall under enhancements.

In the second round, we can check if companies with concerns can agree to the proposal as it is in section 4.2.6:

- Proposal 4.2.6

| - Company | - Comment |
| --- | --- |
| - Huawei, HiSilicon | - We think that the intention of proposal 4.2.6 is not to report what a new RS is used, but rather report the configuration aspects (comb size, number of symbols) for the existing DL/UL positioning RS. |
| - Apple | - Agree with Huawei |
| - LGE | - We have a similar view to Huawei. Considering concerns from some companies, if we stick to a proposal, we prefer to leave a note for that or prefer to modify the original proposal to prevent the concerns. |
| - CATT | - We prefer to use the R16 DL-PRS/SRS-Pos as the baseline for performance evaluation assumptions. The enhancements to R16 DL-PRS/SRS-Pos or new reference siganls will be considered, only if the performance cannot meet the requirements. |
| - Ericsson | - Our understanding is that sources will provide the RS configuration they used for the evaluation. We agree with Huawei it does not mean a new RS. |
| - Qualcomm | - Agree with Huawei |

## Third round  of discussion (closed)

### Channel models and evaluation scenarios (closed)

For the indoor baseline, we can remove InF-DH and focuse on InF-SH. Huawei suggest to split the cases along use cases. From the FL view, IIOT use case and commercial use cases have the same baseline outdoor (Umi) and indoor (Inf SH).  For optional scenarios, we could identify which scenario should be considered for commercial or IIOT.

- Proposal 4.4.1 the following scenarios are evaluated for positioning performance of Redcap

  - Commercial and IIOT use cases baseline outdoor : (Case 1): Umi street canyon , as described in Table 6.1-1-4 of 38.855

  - Optional outdoor:

    - Commercial use cases:

      - Uma , as described in Table 6.1-1-6 of 38.855

      - Rma (FFS details of the scenario)

  - Commercial and IIOT use cases baseline indoor : (Case 4): InF-SH  as described in Table 6.1-1 of 38.857

  - Optional indoor:

    - Commercial use cases:  Indoor Open  Office, as described in Table 6.1-1-3 of 38.855

    - InF-DH  as described in Table 6.1-1 of 38.857

Companies are encouraged to provide  their views on the revised proposal below:

Proposal 4.4.1

| - Company | - comment |
| --- | --- |
| - CATT | - Support the proposal in principle, and we prefer to remove the IIoT use cases from the 1st sub-bullet as follows, since IIoT seems to be a indoor scenario, there is no need to define the baseline outdoor for IIoT use cases: - Commercial and IIOT use cases baseline outdoor : (Case 1): Umi street canyon , as described in Table 6.1-1-4 of 38.855 |
| - ZTE | - The structure of this proposal is not very neat and we prefer Proposal 4.3.1 from the second round discussion and keep InF-SH as baseline. - Moreover, we share the similar view with CATT. Industrial wireless sensors for redcap UE can be considered as IIOT use case and we tend to take industrial wireless sensors are mostly located indoors. Also wearable devices for redcap UE can be considered as commercial use case and wearable devices can distributed indoors or outdoors. |
| - Nokia/NSB | - Support with the update suggested by CATT |
| - Samsung | - We are fine with the proposal. - Regarding CATT’s change, we are not so sure if redcap IIoT device are treated as always indoor. There could sensors these are equipped in open-groud not only for factory. |
| - LGE | - We agree with FL’s proposal that is removed “and IIOT” in the first main-bullet. As we all know, only two scenarios (e..g. InF-SH an InF-DH) are described for IIOT. But, the case in the first main-bullet is about Umi street canyon. So, deleting the “and IIOT” seems appropriate. |
| - Sharp | - Support |
| - Qualcomm | - Agree with CATT |

### General consideration for the UE and gNB simulation parameters

Proposal 4.3.2.1 is stable brought for email agreement. We can continue the discussion for the gNB antenna model in 700MHz band. Qualcomm and ZTE proposed different references and vivo proposed to leave the anteanna model description to sources in evaluations.

- Proposal 4.4.2-2 For evaluation of positioning performance of redcap UEs in 700MHz band, the gNB antenna model  is (down-select):

  - IMT-2020 config-A, with (M, N, P, Mg, Ng) = (8, 4, 2, 1, 1),  (dH, dV) = (0.5, 0.8)λ, 8 TXRUs

  - gNB antenna configuration from TR38.830,  (M,N,P,Mg,Ng) = (4,2,2,1,1), 2 or 4 TXRUs

  - Up to companies to detail in their evaluations

Proposal 4.4.2-2

| - Company | - Comment |
| --- | --- |
| - CATT | - Support - We prefer that up to companies to select the gNB antenna model and report the used model in their simulation of 700MHz band. |
| - ZTE | - We prefer the gNB antenna configuration reused from TR 38.830 but we are open to discuss other options as long as persuasive reasons are provided. It’s better to use a unified gNB antenna model in order to fairly compare the simulation results among companies. |
| - Nokia/NSB | - Better to select one and not leave it up to companies as ZTE says. |
| - Samsung | - Fine to discuss. |
| - LGE | - We are open to which antenna model is used for gNB. But, we also prefer to decide unified model as ZTE and Nokia say. |
| - Sharp |  |
| - Qualcomm | - We are OK to go with the TR38.830 option shown above proposed by ZTE, if that is the preference. |

### Bandwidths to be evaluated (under email approval)

### UE antenna configuration and Number of Tx and Rx branches to be evaluated

The proposal discussion can continue below. To answer the question from the previous round, we hope to be able to select one option for the evaluations. Since one company proposed to leave it to sources to describe in their evaluations, it is proposed as option 3:

- Proposal 4.4.4-2 For the FR2 UE antenna configuration, select between the following options:

- Option1: as in 38.855:

Multi-panel Configuration 1 and Panel Configuration a – Note 1

-	Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0)

-	Panel Configuration a:

-	Each antenna array has shape dH=dV=0.5λ

-	Config a: (M, N, P) = (2, 4, 2),

-	the polarization angles are 0° and 90°

-	The antenna elements of the same polarization of the same panel is virtualized into one TXRU

- Option 2: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) as minimum antenna configuration (baseline) and (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1) as optional configuration.

- Option 3: up to sources to describe in their evaluations.

Companies are encouraged to provide their view on the proposed FR2 UE configuration below:

Proposal 4.4.4-2:

| - Company | - Comment |
| --- | --- |
| - CATT | - We prefer Option 2, since Option 2 aligns with the R17 RedCap UE minimum/maximum antenna configurations. |
| - ZTE | - We prefer Option 2. |
| - Nokia/NSB | - Slightly prefer option 2. We think option 3 is not a good way to go. |
| - Samsung | - As we commented before, option2 is preferred |
| - LGE | - We prefer option 2 based on the discussion about multiple panels (option 1). |
| - Sharp | - We are OK with option 2. |
| - Qualcomm | - We can accept Option 2 |

### Reference signals

Companies have commented that it should be clarified that the reference signal to be used in evaluation should be an exisiting reference signal. We propose the following rewording for clarification:

- Proposal 4.4.5 	The evaluation methodology for RedCap UEs positioning performance uses DL PRS and UL SRS for positioning.

    - The methodology does not define any baseline reference signal configuration. Sources should detail the chosen configuration of reference signal(s) when presenting performance evaluations.

| - Company | - Comment |
| --- | --- |
| - CATT | - Support. |
| - ZTE | - OK. |
| - Nokia/NSB | - Support. |
| - Samsung | - Small wording change: - uses DL PRS and/or UL SRS for positioning. - Not necessary to always use both of them. |
| - LGE | - Agree. |
| - Sharp | - Support |

## Summary before May 19 GTW

### Channel models and evaluation scenarios (closed)

Based on the received comments,  one issue to discuss is whether there is a use case for IIOT outdoor. From the FL perspective, industrial scenarios in rel17 considered both indoor and outdoor. The outdoor case was to capture devices in the factory lot (as commented by Samsung).  We can try as a  way forward  to make the outdoor case for IIOT optional:

- Proposal 4.5.1 the following scenarios are evaluated for positioning performance of Redcap

  - Commercial and IIOT use cases baseline outdoor : (Case 1): Umi street canyon , as described in Table 6.1-1-4 of 38.855

  - Optional outdoor:

    - IIOT: (Case 1): Umi street canyon , as described in Table 6.1-1-4 of 38.855

    - Commercial use cases:

      - Uma , as described in Table 6.1-1-6 of 38.855

      - Rma (FFS details of the scenario)

  - Commercial and IIOT use cases baseline indoor : (Case 4): InF-SH  as described in Table 6.1-1 of 38.857

  - Optional indoor:

    - Commercial use cases:  Indoor Open  Office, as described in Table 6.1-1-3 of 38.855

    - InF-DH  as described in Table 6.1-1 of 38.857

### General consideration for the UE and gNB simulation parameters  (closed)

During the email discussion,  proposal 4-3-3 and 4.3.2-1 were agreed.

Proposal 4.4.2-2 is also stable and can be agreed. However, it would be beneficial to progress further during the meeting and finalize the gNB andtenna model for 700MHz. the proposal is unchanged and copied below to be discussed during the GTW.

- Proposal 4.4.2-2 For evaluation of positioning performance of redcap UEs in 700MHz band, the gNB antenna model  is (down-select):

  - IMT-2020 config-A, with (M, N, P, Mg, Ng) = (8, 4, 2, 1, 1),  (dH, dV) = (0.5, 0.8)λ, 8 TXRUs

  - gNB antenna configuration from TR38.830,  (M,N,P,Mg,Ng) = (4,2,2,1,1), 2 or 4 TXRUs

  - Up to companies to detail in their evaluations

### UE antenna configuration and Number of Tx and Rx branches to be evaluated (closed)

The commenting companies are ok with option 2. The proposal is therefore updated as follow:

- Proposal 4.5.3 The FR2 UE antenna configuration is as follow:

- (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) as minimum antenna configuration (baseline)

- (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1) as optional configuration.

### Reference signals (closed)

We can bring the proposal for agreement at the GTW, including Samsung small change of “and/or”:

- Proposal 4.4.5 	The evaluation methodology for RedCap UEs positioning performance uses DL PRS and/or UL SRS for positioning.

    - The methodology does not define any baseline reference signal configuration. Sources should detail the chosen configuration of reference signal(s) when presenting performance evaluations.

## Remaining issues for UE antenna configuration and Number of Tx and Rx branches to be evaluated

The table for UE antenna configuration as agreed during the GTW. The remaining TBD point was the number of UE branches for FR2. The minimum number of Rx branches for FR2 is 2, according to the LS from RAN4 (R4-2206545)

RAN4 decides to specify FR2 RedCap UE with below considerations: - Define new power class: Power class 7 - Not reduce the number of Rx branches, i.e. 2 Rx branches assumed for FR2 RedCap UE - Equip with dual polarization receiver - Reduce to half of the number of array elements of PC3 NR FR2 UE with below key RF parameter - Min Peak EIRP: 16.4 dBm which is 6 dB below the min Peak EIRP of the PC3 for n257, n258 and n261 - REFSENS: 3 dB relaxation compared to the PC3 REFSENS - Note: Existing power classes are not precluded for RedCap. RAN4 also agree the # of DL layers is not mandated for FR2 RedCap UE

Based on the LS,  my proposal is to use 2Rx 1Tx as a baseline for FR2.

Proposal 4.6: use 2Rx and 1Tx  for number of UE branches in FR2 in the UE antenna configuration table for RedCap UEs evaluation

Companies are encouraged to provide their view on the proposed  number of UE branches in FR2 below:

Proposal 4.6:

| - Company | - Comment |
| --- | --- |

# Evaluations and Enhancements for Redcap UEs

## Summary of proposals

Many observations have been raised by the contributions for the different positioning techniques.  While it is too early to propose to capture any observation since the simulation assumptions have not been aligned, we can make progress by agreeing on a format to capture the results of the future evaluation.

Several enhancements have been proposed. Many contributions have proposed to handle the limited bandwidth of RedCap Ues by applying some form of frequency hopping [2][5][6][7][8][12][14][22]. In [8][22] phase difference AOD is discussed. [13] mention evaluating if changes to reference signal parameters can be enough to compensate the performance loss. [14] proposes to evaluate carrier phase positioning for redcap Ues. TRS and SSB based measurements are mentioned in [22].

| - Source | - Proposal |
| --- | --- |
| - [1] | Observation 2: Compared with eMBB UE, the positioning accuracy for RedCap UE is seriously degraded due to the reduced bandwidth. - Observation 3: The performance for RedCap UE with existing positioning methods (>3m@90% with NLOS detection) cannot meet the sub-meter positioning accuracy requirement. - Proposal 2: Identify positioning solutions suitable for RedCap Ues to achieve high accuracy positioning, i.e., sub-meter positioning accuracy. |
| - [2] | Observation 1: The horizatonal positioning accuracy performance of RedCap Ues is significantly degraded compare with normal NR Ues. Proposal 3: RAN1 to study more dynamic SRS transmissions for RedCap Ues. Proposal 4: RAN1 to investigate group based positioning schemes for RedCap Ues. Proposal 5: RAN1 to study the ability to receive wideband PRS signals with a narrow band receiver (e.g., over multiple 20 MHz chunks) and transmit wideband signal on a narrow band transmitter (e.g., frequency hopping over multiple 20 MHz chunks). Identifying any specification impact should be part of the study. Observation 2: RedCap Ues may have positioning measurement performance degradation due to power saving/reduced capability features it is implementing. Proposal 6: RAN1 to study methods for reducing the impact of reduced capability features (e.g., RRM measurement relaxation) on the positioning measurement accuracy of RedCap Ues. |
| - [3] | Observation 1: Simulation result shows that in InF-SH, the horizontal positioning error is 2.39 meter for 20MHz bandwidth for 90% of Ues, and 0.27 meter for 100MHz bandwidth for 90% of Ues. |
| - [4] | Observation 1 - For DL-TDOA positioning, the potential performance target [2m 90%] can be achieved in InF-SH and InF-DH with bandwidth of 20MHz. - For UL-TDOA positioning, the potential performance target [2m 90%] can be achieved in InF-SH and InF-DH with bandwidth of 20MHz. Observation 2 - For UL-AOA positioning, the performance of 20MHz bandwidth in SH is close to the potential performance target [2m 90%]. - For DL-AOD positioning, the performance of 20MHz bandwidth in SH is close to the performance target [2m 90%]. Observation 4 - Accuracy requirement for RedCap positioning can be achieved under the bandwidth of 20MHz, based on current positioning techniques. - Rel-18 techniques of AI-based positioning and carrier phase positioning can also be applied to scenarios where bandwidth is limited for better accuracy. Proposal 2 - For accuracy enhancement of RedCap positioning, support reusing existing positioning method and measurements with RedCap Ues as much as possible. |
| - [5] | Observation 1: For FR1, the positioning performance is insufficient because of limited bandwidth. Observation 2: The random phase between hops will damage the positioning performance if it was not adjusted. Observation 3: PRS frequency hopping can improve positioning performance if the random phase between hops can be adjusted. Proposal 1: Consider at least SRS frequency hopping for positioning. Proposal 2: To eliminate phase difference between hops, some methods should be researched. |
| - [6] | Proposal 2: For NR REDCAP Ues, study the PRS frequency hopping scheme and consider the time and frequency allocation for each hopping subband Proposal 3: For NR REDCAP Ues, if frequency hopping is enable, study the muting mechanism for subbands. |
| - [7] | Observation 2: Reducing the bandwidth for DL-PRS transmission also reduces the positioning accuracy. Proposal 4: Further study the positioning techniques to improve positioning accuracy of RedCap UE. |
| - [8] | Proposal 2-1: Support DL-PRS transmission hopping Proposal 2-2: For DL-PRS transmission hopping, the partial overlapping in frequency domain between the BW before and after the hopping is preferred Proposal 2-3: The SRS transmission outside UL BWP may also be considered, especially for the positioning techniques of requiring DL and UL measurements Proposal 3-1: The phase difference based AOD needs to be justified to be better than the RSRP based AOD. Otherwise there is no need to define another solution for the angle based measurement, since there are quite a lot similarity between the RSRP based AOD and the phase difference based AOD Proposal 3-2: Move the discussion of the phase difference based AOD to AI 9.5.2.2 for carrier phase measurement |
| - [10] | Observation 2: the reduced FFT size (in translated to reduced sampling rate) could obviously reduce the performance. Observation 3: the reduced sequence length could obviously reduce performance at low SNR region. |
| - [11] | Proposal 3: The support of positioning for RedCap Ues and any potential enhancement should not have large impact on the cost of RedCap Ues. |
| - [12] | Observation 2: In FR2, for DL-AoD, reduced BW from 400MHz to 100MHz leads to nearly 1m accuracy loss at CDF of 90% ile. Proposal 6 : Study enhancements related to frequency hopping for DL-PRS, at least for DL-AoD, to improve positioning accuracy for RedCap Ues |
| - [13] | Proposal 3: RAN1 to investigate if modification of the reference signal parameters assumptions can compensate for performance loss based on a common set of evaluation parameters. |
| - [14] | Proposal 2: Further study enhancements of RedCap UE positioning: - SRS frequency hopping - Carrier phase positioning |
| - [15] | Observation 1: - Applying existing timing-based positioning procedures and measurements to RedCap Ues will lead to degraded positioning performance compared with non-RedCap Ues due to reduced bandwidth. Observation 2: - It may be better to evaluate each method and consider which methods are suitable for RedCap Ues with considering complexity of the method in addition to performance. Proposal 1: - Once the evaluation results reveal performance degradation, RAN1 should work for the enhancement for positioning for RedCap Ues and determine the target performance level. Proposal 2: - RAN1 should consider PRS frequency hopping and joint estimation as potential solutions to overcome the performance concern due to the bandwidth reduction. Proposal 3: - For Rel-18 positioning for RedCap Ues with lower power consumption, it should be discussed the optimization feasibility of positioning in DRX state. |
| - [16] | Proposal 4: - To reduce the UE complexity, it is proposed for RedCap UE to report channel estimation to its serving gNB and the gNB implements the position estimation with the reported channel estimation. |
| - [17] | Proposal #3: - Discuss time/frequency domain enhancement (e.g., BWP hopping/switching and burst transmission of SRS for positioning) to compensate performance degradation of RedCap UE |
| - [18] | Observation 1: Redcap Ues are designed with the aim of lowering device cost through reduced complexity. Observation 2: Significant degradation in the accuracy of the positioning is seen for 20 MHz PRS bandwidth in outdoor scenarios. |
| - [20] | - Study frequency hopping (FH) techniques for the UL-SRS and DL-PRS signal transmission to enhance the timing-based estimates of the DL-TDOA, UL-TDOA, and Multi-RTT positioning methods for the RedCap Ues. - If enhancements are determined as necessary for 20 MHz RedCap Ues, study super resolution MUSIC-like methods for performance improvement of the DL-TDOA, UL-TDOA, and Multi-RTT positioning methods for RedCap Ues and the support of frequency hopping-based schemes to enable such algorithms. - If enhancements are determined as necessary for 20 MHz RedCap Ues, study carrier phase measurements-based positioning techniques for positioning performance improvement for RedCap Ues. |
| - [22] | Observation 3: At the 80% percentile, for DL-TDOA, without network synchronization error, in 4 GHz UMI TDD, without Deltautau modelling, ,and 20 MHz Ues, the horizontal accuracy degrades from 11.3 meters to 12.2 meters due to reducing the number of Rx antennas from 2 to 1. Observation 4: At the 80% percentile, for DL-TDOA, without network synchronization error, in 28 GHz InF-SH TDD, and 100 MHz Ues, the horizontal accuracy degrades from 0.07 meters to 0.20 meters (case 4) due to reducing the number of Rx antennas from 2 to 1. Observation 5: In the 3GPP UMI 4 GHz Scenario, without DeltaTau odelling, Horizontal accuracy <10m is achievable with 20 MHz, 1 Rx with M-RTT and outlier rejection positioning engine for outdoor Ues. - Reducing the number of antennas from 2 Rx to 1 Rx, has smaller impact in the performance compared to reducing the bandwidth - M-RTT seems to be demonstrating better performance compared to DL-TDOA on the same scenario. Observation 6: Reducing from (100 MHz, 2 Rx) to (20 MHz, 1 Rx), results in a horizontal performance degradation from the [1-3]m range to the [7-12]m range, with the reduction in Bandwidth playing the main role in this performance reduction Observation 7: Reducing from (400 MHz, 2 Rx) to (100 MHz, 1 Rx), results in a horizontal performance degradation from the 0.0x m range to the 0.x0 m range, with the reduction in Bandwidth playing the main role in this performance reduction. Proposal 3: Support Enhancements for Redcap devices, in both FR1 and FR2, to enable reaching a horizontal accuracy performance close to the NR Rel-17 positioning requirements at least in a subset of evaluation scenarios. Observation 8: Enabling receiver’s PRS hopping would allow sharing the legacy PRS across eMBB and Redcap devices. Proposal 4: Study inter & intra-slot repetition of a DL PRS resource for the purpose of enabling receive PRS hopping for both FR1 and FR2. Proposal 5: Study inter & intra-slot repetition of a SRS resource for positioning for the purpose of enabling transmit SRS hopping for both FR1 and FR2. Observation 9: The phase discontinuity introduced due to PRS frequency hopping results a in performance degradation which could be mitigated by using frequency hopping with overlapping tones. Proposal 6: Study PRS/SRS overlapping configuration for the purpose of enabling phase estimation across PRS hops both FR1 and FR2. Observation 10: Enabling transmitter’s PRS hopping could improve the Tx power, and further improve the positioning accuracy. Proposal 7: Study DL-PRS/ SRS resource configuration for the purpose of enabling transmitter PRS hopping. Observation 11: Phase-Difference-based AoD may be a useful additional method for scenarios with unprecoded transmissions from gNB side (e.g. FDD cases). Proposal 8: For the purpose of Redcap positioning enhancements, study supporting Phase-Difference AoD. Proposal 9: For the purpose of Redcap positioning enhancements, study supporting Positioning measurements (RSTD, UE Rx-Tx, RSRPP) derived on SSB, TRS. Proposal 10: For the purpose of Redcap positioning enhancements, study supporting M-RTT using SRS-MIMO. |

## First round of discussion

### Format for capture of the evaluations

Several companies have already provided results in their contributions. To capture the results in the SI TR, a common template should be used. It is proposed to re-use TR 38.857 template as a starting point and discuss what field in the template should be removed or added. The template is copied in appendix of this FL summary for convenience.

Question 5.2.1: To capture the evaluation parameters and  results of rel18 evaluations of positioning for RedCap Ues, the template used in TR 38.857 for the inclusion of simulation results can be  reused.  Please comment whether the template should be used as is or if any field should be removed or added

| - Company | - Comment |
| --- | --- |
| - Huawei, HiSilicon | - One question for clarification. For the Rel-17 template, we should have separate sections for Rel-17 baseline and enhancements, respectively, right? |
| - Nokia/NSB | - If possible we prefer to see one Section that shows both baseline and enhancements. The Rel-17 TR was highly unreadable due to the very high number of papers. If we can avoid making that same mistake again we should. |
| - FL | - Regarding the template, I agree, there will be different sections in the TR for baseline and enhancements. However the tables for each of the results can have a similar format. |

### Candidates for enhancements of RedCap UE positioning performance (closed)

Since bandwidth hopping for PRS and SRS has been already mentioned by many companies, it is proposed to capture that it will be studied during the evaluation

- Proposal 5.2.2 PRS and SRS Bandwidth hopping will be investigated in Rel-18 NR positioning for RedCap Ues.

| - Company | - Comment |
| --- | --- |
| - CATT | - We prefer to firstly assess whether positioning performance of existing positioning procedures and measurements with RedCap Ues can meet the requirements, if not, further consider study the enhancements, such as PRS and SRS Bandwidth hopping. |
| - Huawei, HiSilicon | - Support. |
| - OPPO | - We share similar view as CATT. R18 new mechniam studied in other AIs (e.g., carrier phase based method) can also be considered as a candidate solution. |
| - vivo | - It is too early to make such conclusion. Based on the objectives, we should focus on evaluating positioning performance of existing positioning procedures and measurements first, and if enhancement is needed, study potential solutions. In this meeting, there is no conclusion that existing positioning procedures and measurements cannot meet the requirement. Therefore, PRS/SRS hopping may only be studied up to companies at this stage. |
| - Qualcomm | - Support |
| - ZTE | - We support investigating PRS and SRS frequency hopping for RedCap UE positioning. |
| - Apple | - Need to establish that there is a gap based on an agreed methodology and assumptions. Decide on improvements after this has been done |
| - NEC | - Support to investigate PRS and SRS frequency hopping for RedCap UE positioning. |
| - LGE | - Support. |
| - InterDigital | - We support the proposal to investigate bandwidth hopping for DL-PRS to improve positioning accuracy for RedCap UEs. |
| - Nokia/NSB | - We support studying PRS and SRS frequency hopping. |
| - Samsung | - Seems premature to having this proposal. |
| - Lenovo | - We prefer to start evaluation with the Rel16/Rel17 as baseline |
| - Sony | - Support and other enhancements should not be precluded. |
| - FL | - Since many company think it is too early to make such agreements, we can close the discussion. |

Conclusions

The following agreements have been reached during RAN1#109e:

References

- R1-2203168, Discussion on RedCap positioning, Huawei, HiSilicon

- R1-2203180, Initial Views on Positioning for RedCap UEs, Nokia, Nokia Shanghai Bell

- R1-2203471, Discussion on positioning for RedCap UEs, CATT

- R1-2203570, Discussion on positionig for RedCap Ues, vivo

- R1-2203628, Discussion on Positioning for RedCap UE, ZTE

- R1-2203696, Discussion on positioning support for RedCap UEs, NEC

- R1-2203740, Discussion on positioning for RedCap UEs, Sony

- R1-2203754, The potential solutions for RedCap UEs for positioning, MediaTek Inc.

- R1-2203826, Initial views on the positioning for RedCap UEs, xiaomi

- R1-2203915, Discussion on Positioning for RedCap Ues, Samsung

- R1-2203968, Discussion on Positioning for RedCap Ues, OPPO

- R1-2204157, Evaluation assumptions and potential solutions for positioning for RedCap UEs, InterDigital, Inc.

- R1-2204254, Discussions on Positioning for RedCap UEs, Apple

- R1-2204314, Discussion on RedCap positioning, CMCC

- R1-2204388, Discussion on positioning for RedCap UEs, NTT DOCOMO, INC.

- R1-2204425, Discussion on Positioning for RedCap UEs, Quectel

- R1-2204526, Discussion on positioning support for RedCap Ues, LG Electronics

- R1-2204563, Positioning for RedCap devices, Lenovo

- R1-2204671, Views on positioning for RedCap UEs, Sharp

- R1-2204808, On enhancements for NR positioning support of RedCap UEs, Intel Corporation

- R1-2204954, Positioning for RedCap Ues, Ericsson

- R1-2205042, Positioning for Reduced Capabilities UEs, Qualcomm Incorporated

Appendix: template for the capture of RedCap UE positioning performance evaluation.

Table 1: template for capture of the parameters used for performance evaluation

| Parameter | Source X, scenario, FRx] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) |  |
| Carrier frequency |  |
| Subcarrier spacing |  |
| Reference Signal Transmission Bandwidth |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  |
| Reference signal (type of sequence, number of ports, …) |  |
| Number of sites |  |
| Number of symbols used per occasion |  |
| number of occasions used per positioning estimate |  |
| Power-boosting level |  |
| Uplink power control (applied/not applied) |  |
| interference modelling (ideal muting, or other) |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  |
| Network synchronization assumptions |  |
| UE/gNB Tx/Rx Calibration Error |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) |  |
| Additional notes, if any |  |

Table 2: template for capture of the evaluation results

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Evaluation #,scenario, FR#, technique | Convex UEs or all UEs |  |  |  |  |