3GPP TSG RAN WG1 #110bis-e                                         R1- 2208528

e-Meeting, October 10th – 19th, 2022

Source:	New H3C

Title:	Discussion for Evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for:	Discussion and Decision

- Introduction

In the RAN1 #110 meeting, evaluation on NR duplex evolution in Rel.18 was discussed. Some agreements were made as below [1]

Agreement

Two types of RU (Resource utilization) are defined for SBFD evaluation.

- Type-1 RU: DL/UL Type-1 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of all the RBs per cell including DL, UL and guard bands over observation time.

- Type-2 RU (Follow TR 36.814): DL/UL Type-2 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of RBs per cell available for traffic for the given link direction over observation time

- Note: In case of MU-MIMO, one RB allocated to N users within a cell is only counted as used once.

- Companies are to submit results for both RU definitions

- FFS: RU definition for dynamic TDD evaluations

Agreement

For UE distribution of Urban Macro and Dense Urban Macro layer,

- Baseline: (UE clustering)

  - 10 users per macro TRP

    - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster

    - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped throughout the macro geographical area

    - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

    - FFS the values of X, Dmacro-to-cluster, Dinter-cluster, R, Y%

- Optional:

  - 10 users per macro TRP, and all users are randomly and uniformly dropped within the macro cell

  - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

    - FFS: FR2 details

Agreement

For Dense Urban with 2-layer for FR1, consider micro cell TRPs are deployed as following

- Step 1: Randomly drop [3] micro TRP centers within one macro cell geographical area considering the minimum distance between micro TRP centers (Dinter-micro-center) and the minimum distance between macro TRP and micro TRP center (Dmacro-to-micro-center).

- Step 2: Randomly deploy one micro TRP on the area circle around each micro TRP center with the radius of half of Dinter-micro-center

- Step 3: Determine the horizontal angle of the micro TRPs with the planer facing to the micro TRP center.

- Dinter-micro-center =[57.9 m], Dmacro-to-micro-center = [105 m]

Agreement

Update the previous agreement as below:

For UE distribution of Urban Macro and Dense Urban Macro layer,

- Baseline: (UE clustering at least for FR1)

  - M users per macro TRP

    - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster

    - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped in the macro geographical area outside the clusters

    - Note: UEs dropped within the UE cluster(s) are indoor with 3km/h; UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h

    - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

      - Outdoor UEs: 1.5 m;

      - FFS: Indoor UEs height

    - Y%=80%

    - FFS the values of M, X, Dmacro-to-cluster, Dinter-cluster, R

- Optional:

  - 10 users per macro TRP (per direction), and all users are randomly and uniformly dropped within the macro cell

  - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

    - Outdoor UEs: 1.5 m;

    - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) [refer to TR 36.873 Table 6-1]

    - FFS: FR2 details

In this contribution, we provide our considerations on evaluation on NR duplex evolution.

- Discussion

  - Simulation assumptions

For UE distribution of Urban Macro and Dense Urban Macro layer, in term of chair notes of RAN1#110 [1], we have the following agreement.

Agreement Update the previous agreement as below: For UE distribution of Urban Macro and Dense Urban Macro layer, - Baseline: (UE clustering at least for FR1) - M users per macro TRP - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped in the macro geographical area outside the clusters - Note: UEs dropped within the UE cluster(s) are indoor with 3km/h; UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - FFS: Indoor UEs height - Y%=80% - FFS the values of M, X, Dmacro-to-cluster, Dinter-cluster, R - Optional: - 10 users per macro TRP (per direction), and all users are randomly and uniformly dropped within the macro cell - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) [refer to TR 36.873 Table 6-1] - FFS: FR2 details

For FFS values of baseline in above agreement, we suggest using the set of values including X=[3], Y%=[2/3]; for Urban Macro for FR1: Dmacro-to-cluster = [262.5 m], Dinter-cluster = [114.8 m], R = [72.3 m]; 	For Dense Urban Macro layer for FR1 and FR2-1: Dmacro-to-cluster = [105 m], Dinter-cluster = [57.9 m], R = [28.9 m] because this set of parameters values are close to practical scenario.

Proposal 1: For UE distribution of Urban Macro and Dense Urban Macro layer, the set of values ] can be used for FFS parameters value of the baseline as follows:

X=[3], Y%=[2/3];

For Urban Macro for FR1: Dmacro-to-cluster = [262.5 m], Dinter-cluster = [114.8 m], R = [72.3 m]; 

For Dense Urban Macro layer for FR1 and FR2-1: Dmacro-to-cluster = [105 m], Dinter-cluster = [57.9 m], R = [28.9 m.

For work assumption for SBFD and dynamic/flexible TDD evaluation, we have the following agreement in RAN1#110 [1].

| Parameters | Indoor office | Urban macro / Dense Urban Macro layer | Dense Urban with 2-layer |
| --- | --- | --- | --- |
| Layout | Single layer Indoor floor: (12BSs per 120m x 50m) | Single layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. | Two layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. Micro layer: According to previous agreement - Baseline: 3 Micro BSs per Macro BS - Optional: 6, or 9 Micro BSs per Macro BS |
| Inter-BS (2D) distance | 20m [TR 38.802 Table A.2.1-11] | 500m for Urban Macro [TR 38.802 Table A.2.1-11] 200m for Dense Urban Macro layer [TR 38.802 Table A.2.1-1] | Macro-to-macro: 200m Minimum Macro-to-micro-center distance: 105m Minimum Micro-center-to-micro-center distance: 57.9m |
| Minimum BS-UE (2D) distance | 0m [TR 38.802 Table A.2.1-11] | 35m [TR 38.802 Table A.2.1-11] | Macro-to-UE: 35m Micro-to-UE: 10m [TR 38.802 Table A.2.1-11] |
| Minimum UE-UE (2D) distance | FFS | FFS :3m [TR 38.802 Table A.2.1-11] | FFS: 3m [TR 38.802 Table A.2.1-11] |
| BS antenna height | 3 m [TR 38.802 Table A.2.1-1] | 25 m [TR 38.802 Table A.2.1-1] | 25m for macro cells and 10m for micro cells [TR 38.802 Table A.2.1-1] |

Regarding the above-mentioned minimum BS-UE (2D) distance, considering the workload of evaluation, it is better to reuse minimum BS-UE (2D) distance value (3m) defined in TR 38.802 for coverage evaluation of SBFD.

Proposal 2: For minimum BS-UE (2D) distance for SBFD and dynamic/flexible TDD evaluation, minimum BS-UE (2D) distance value (3m) defined in TR 38.802 for flexible duplex should be reused.

Conclusion

In this contribution, we discuss about evaluation on NR duplex evolution with the following proposal.

Proposal 1: For UE distribution of Urban Macro and Dense Urban Macro layer, the set of values ] can be used for FFS parameters value of the baseline as follows:

X=[3], Y%=[2/3];

For Urban Macro for FR1: Dmacro-to-cluster = [262.5 m], Dinter-cluster = [114.8 m], R = [72.3 m]; 

For Dense Urban Macro layer for FR1 and FR2-1: Dmacro-to-cluster = [105 m], Dinter-cluster = [57.9 m], R = [28.9 m.

Proposal 2: For minimum BS-UE (2D) distance for SBFD and dynamic/flexible TDD evaluation, minimum BS-UE (2D) distance value (3m) defined in TR 38.802 for flexible duplex should be reused.

- Reference

- Chairman’s Notes, RAN1#110, final, August 22th – 26th, 2022