3GPP TSG-RAN WG1 Meeting #110bis-e 	R1-2208611

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | x | rev | x | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for inter-UE coordination scheme-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022/9/30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. It has been agreed that the priority value of PSFCH TX for a resource conflict indication is the smallest priority value of the conflicting TBs. However, the rule is applied only for the case when the resource conflict is detected based on Condition 2-A-1. It has not discussed how to determine the PSFCH TX priority when the resource conflict is determined based on Condition 2-A-2, e.g., when UE-A does not expect to perform SL reception from UE-B due to its UL transmission. Since there is only a single SL TB transmission when determining the resource conflict based on condition 2-A-2, it is straightforward to assume that priority value of PSFCH TX for a resource conflict indication is the priority value associated with the TB transmission. 2. In current spec, when UE determines a number of PSFCH resources available for multiplexing conflict information in a PSFCH transmission as RPRB, CSPSFCH=Ntype PSFCH⋅Msubch, slotPSFCH⋅NCSPSFCH, the PRBs are associated with the starting subchannel or the number of subchannels of the corresponding PSSCH, where the corresponding PSSCH is determined based on PSFCHOccasionScheme2. However, this parameter is introduced to indicate the time domain of PSFCH occasion. Hence, the corresponding PSSCH cannot be deduced from this parameter according to the current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the following corrections for Clause 16.2.4.2 and Clause 16.3.0: - Clarification on the priority value of PSFCH for conflict indication when resource conflict is determined based on Condition 2-A-2, i.e., when there is only a single SCI format 1-A associated with the PSFCH. - Clarification on PSSCH resource determination based on PSFCHOccasionScheme2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear on how to determine the PSFCH TX priority when the resource conflict is determined based on Condition 2-A-2. It is not clear on how to determine the corresponding PSSCH based on PSFCHOccasionScheme2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.4.2, 16.3.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

16.2.4.2	Simultaneous PSFCH transmission/reception

For a PSFCH transmission or reception with HARQ-ACK information, a priority value for the PSFCH is equal to the priority value indicated by an SCI format 1-A associated with the PSFCH, or equal to the priority value determined by the corresponding SCI format 1-A for the conflicting resource if there is a single SCI format 1-A associated with the resource conflict determination.

For PSFCH transmission with conflict information, a priority value for the PSFCH is equal to the smallest priority value determined by the corresponding SCI formats 1-A for the conflicting resources.

===omitted===

16.3.0	UE procedure for transmitting PSFCH with control information

===omitted===

A UE determines a number of PSFCH resources available for multiplexing HARQ-ACK or conflict information in a PSFCH transmission as RPRB, CSPSFCH=Ntype PSFCH⋅Msubch, slotPSFCH⋅NCSPSFCH where NCSPSFCH is a number of cyclic shift pairs for the resource pool provided by sl-NumMuxCS-Pair and, based on an indication by sl-PSFCH-CandidateResourceType,

-	if sl-PSFCH-CandidateResourceType is configured as startSubCH, Ntype PSFCH=1 and the Msubch, slotPSFCH PRBs are associated with the starting sub-channel of the corresponding PSSCH

-	if sl-PSFCH-CandidateResourceType is configured as allocSubCH, Ntype PSFCH=Nsubch PSSCH and the Nsubch PSSCH⋅Msubch, slotPSFCH PRBs are associated with the Nsubch PSSCH sub-channels of the corresponding PSSCH

-	for conflict information, the corresponding PSSCH is determined based on PSFCHOccasionScheme2. If PSFCHOccasionScheme2 = 'followSCI', the corresponding PSSCH is the PSSCH transmitted with the SCI format 1-A indicating the conflict reserved resource. If PSFCHOccasionScheme2 = 'followReservedResource', the corresponding PSSCH is the PSSCH of the conflict reserved resource.

===omitted===