3GPP TSG RAN WG1 #110bis-e                                                          R1-2208639

e-Meeting, October 10th  – 19th, 2022

Source:	vivo

Title:	Other aspects on AI/ML for positioning accuracy enhancement

Agenda Item:	9.2.4.2

Document for:	Discussion and Decision

Introduction

Many positioning methods have been specified in Rel-16 and Rel-17 NR positioning, to obtain position estimation with target horizontal positioning accuracies of <0.2 m (90%) for IIoT use cases and <1 m (90%) for commercial use cases. However, the performance of these positioning methods highly relies on the existence of multiple LOS (line-of-sight) paths between the target terminal and multiple TRPs (Transmission-Reception Points). In the scenarios with extremely low LOS probability, positioning accuracy would decrease dramatically, which may be not able to satisfy the high-accuracy positioning requirements coming from new applications and industry verticals.

AI/ML technology has powerful abilities in feature extraction, environment awareness, complex problem modeling and processing. In recent years, applying AI/ML into air-interface has attracted great attentions from academics to industries, and a lot of meaningful exploration has been made to verify the performance gain compared to conventional non-AL/ML schemes. Related research has also verified that AI/ML technology has the potential to significantly improve the performance of wireless communications.

Under this background, a new SI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface has been agreed at RAN #94e [1], including three use cases to assess the applications of AI/ML in air-interface. Among them, AI/ML based positioning accuracy enhancement is included, with the target to improve the positioning accuracy for different scenarios, especially for some challenging scenarios with heavy NLOS (non-line-of-sight) conditions.

The objective of the new SI for RAN1 AI/ML based positioning includes the following:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

At the RAN1 #110 meeting, some agreements on sub use cases and potential specification impact have been reached, which are listed as follows:

Agreement For characterization and performance evaluations of AI/ML based positioning accuracy enhancement, the following two AI/ML based positioning methods are selected. - Direct AI/ML positioning - AI/ML assisted positioning - Note 1: the selection does not intend to provide any indication of the prospects of any future normative project. - Note 2: further discussion (including selection of other sub use cases and/or down selection of selected sub use cases) are not precluded based on performance evaluation and potential specification impact study results Agreement Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report) - Partial and/or noisy ground truth label - Signaling for data collection - Other aspects are not precluded Agreement Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report - Other aspects are not precluded Agreement Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least - UE-side or Network-side training - UE-side or Network-side inference - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches Note: companies are encouraged to clarify aspects of their proposed AI/ML approaches for positioning when AI/ML model training and inference are not performed at the same entity Conclusion To use the following terminology defined in TS 38.305 when describe their proposed positioning methods - UE-based - UE-assisted/LMF-based - NG-RAN node assisted Note: companies are required to clarify their positioning method(s) when their approaches do not fall in one of the above

In this contribution, we present our views on sub use cases and potential specification impact for AI/ML based positioning accuracy enhancement.

Sub use case

During RAN1#109-e, there’re discussions on what is(are) sub use case(s) for AI/ML positioning enhancement. In our view, we think the SID [1] with relevant part copied below clearly defines the scope of work for us to look for as sub use case(s).

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

The SID says “positioning accuracy enhancements for different scenarios” as the “set of use cases” for AI/ML positioning. So for the “use case” of positioning, “sub use cases” are referring to AI/ML positioning for different scenarios where “AI/ML based positioning in heavy NLOS scenario” is an explicit example of “sub use case” given by the SID.

For this sub use case, depending on the role where AI/ML technology plays in positioning, there are two AI/ML approaches, including direct AI/ML positioning and AI/ML assisted positioning, as shown in Figure 1.

The first is to utilize AI/ML to estimate UE location with reference to some measurements and/or measurement reports directly, such as estimating location according to multiple TRPs’ CIRs.  This type of method may achieve relatively high positioning accuracy, while suffering from poor generalization capability since the AI model is strongly related to TRP’s geographical distribution.

The second is to utilize AI to extract intermediate features from some measurements and/or measurement reports, and then estimate position according to the intermediate feature. For positioning accuracy enhancement, multiple intermediate features can be considered, such as time of arrival (TOA), Angle of Arrival (AoA), Line-of-Sight (LoS)/NLoS condition, and so on. This type of method has the advantages of better generalization capability, deployment flexibility, compatibility with the existing positioning protocol framework, and positioning accuracy enhancement. We consider TOA as the intermediate feature, and showed that the positioning accuracy and generalization capability are better than the direct AI/ML based positioning method. The positioning accuracy of AI/ML based positioning are shown in Table 1 where Rel-17 positioning methods are also shown as baseline for comparison. Note that more detailed evaluation results for both direct AI/ML positioning and AI/ML assisted positioning are shown in our companion contribution [2].

Two AI/ML approaches for AI/ML based positioning accuracy enhancement

CDF of positioning accuracy (m) of different positioning methods

| Scenario | Methods | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| InF-DH {0.6,6,2} | DL-TDOA | 8.38 | 11.09 | 15.95 | 32.12 |
|  | UL-TDOA | 8.60 | 11.52 | 16.33 | 32.81 |
|  | RTT | 8.32 | 11.42 | 15.72 | 32.41 |
|  | AOA | 8.13 | 10.36 | 14.09 | 20.16 |
|  | Direct AI/ML pos. (CIR-pos) training dataset and testing dataset from the same drop | 0.35 | 0.49 | 0.70 | 0.99 |
|  | AI/ML assisted pos. (CIR-TOA-pos) training dataset and testing dataset from the same drop | 0.19 | 0.28 | 0.40 | 0.60 |
|  | Direct AI/ML pos. (CIR-pos) training dataset and testing dataset from different drops | 2.76 | 3.68 | 4.58 | 6.00 |
|  | AI/ML assisted pos. (CIR-TOA-pos) training dataset and testing dataset from different drops | 0.93 | 1.27 | 1.73 | 2.51 |

Both direct AI/ML positioning and AI/ML assisted positioning show significant accuracy improvement compared to conventional RAT-dependent positioning methods in a heavy NLOS scenario.

As agreed in RAN1#110, we think both direct AI/ML positioning and AI/ML assisted positioning are necessary and sufficient sub use cases for study of AI/ML based positioning performance and specification impact. Hence, there’s no need to consider other sub use case(s) as representative or to down select between those two during this SI.

There’s no need to consider other sub use case(s) as representative or to down select between direct AI/ML positioning and AI/ML assisted positioning during this SI for performance and potential specification impact study.

Potential specification impact

In this section, we analyze the potential specification impact of AI/ML based positioning accuracy enhancement from various perspectives, including model training, model indication/configuration, model monitoring and update, model inference and required UE capability report.

Common framework

According to the common functional framework for RAN intelligence illustrated in Figure 2 [3], we analyze the general specification impact from the perspective of lifecycle management for AI/ML based positioning accuracy enhancement.

Data Collection is a function that provides a training dataset to model training and input data to model inference functions. An example of a training data format is {CIR, location}, and accordingly, the input data to model inference functions is CIR. Especially, a data pre-processing procedure is necessary after data collection, such as CIR truncation and translation. Data normalization has the potential to decrease the overhead for data collection and improve the model performance.

Model Training is a function that performs the AI/ML model training, validation, and testing which may generate model performance metrics as part of the model testing procedure. AI/ML model training can be further divided into two phases, i.e., model pre-training based on collected data and model update based on collected field data if the current model does not work well.

Model Inference is a function that provides AI/ML model inference output (e.g. predictions or decisions). In different sub use cases, the output of model inference could be different, such as UE’s location and an intermediate feature.

Actor is a function that receives the output from the model inference function and triggers or performs corresponding actions. For AI/ML assisted positioning, the output from the Model Inference function is the intermediate feature, and then Actor performs position estimation according to the intermediate feature.

A common functional framework for RAN intelligence [3]

Model training

The high-accuracy positioning relies on a well-trained AI/ML model deployed at UE side or network side. Enabling such high-quality model training usually requires some assistance information interaction between UE side and network side. In particular, training data is the most important part for model training. In this section, we mainly discuss the potential specification impacts from the perspective of data collection.

AI/ML is data-driven, and data collection is a necessary but challenging process for AI/ML based positioning. As shown in our companion contribution [2], time domain channel CIR as the input of AI/ML model can obtain the best positioning accuracy compared to other inputs, such as power, delay and angle of the first path. Therefore, we propose

Support time domain CIR as one model input for training of AI/ML model for positioning.

At the RAN1#110 meeting, it was agreed that:

Agreement

Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement

- Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report)

  - Partial and/or noisy ground truth label

- Signaling for data collection

- Other aspects are not precluded

In general, for downlink positioning,  the input for an AI/ML model training is CIR estimated from downlink PRS measurement plus UE ground truth location data. Such training data including training input and labels, could be collected by UE side, including PRUs and/or regular UEs. Among them, PRUs deployed in advance are more convenient to collect data with ground truth labels, and regular UEs may be also able to collect some data with noisy labels or without labels. For uplink positioning, the input for an AI/ML model training is CIR estimated from uplink SRS measurement plus UE ground truth location data. Such training data could be collected by the collaboration of network side and UE side. For example, training labels should be collected by UE side, and training input, such as CIR, should be collected by network side. Moreover, some dedicated reference signal configuration may be also required to support data collection. In particular, in the process of data collection, some non-ideal factors should be considered, including synchronization error, timing error, CIR estimation error, and labeling error, and some dedicated RSs may be configured to mitigate or eliminate the impact of these factors.

Here, we will discuss the specification impacts of these two data collection schemes in detail.

The procedure of data collection for data measurement at network side

A possible procedure is shown in Figure 3 when data measurement is performed at UE side, and the details are clarified as follows:

Network side sends PRS configuration signaling to the target UEs when a dedicated PRS configuration is required to support data collection.

Network side sends a data collection request to the target UEs, which may consist of training data size and type, data preprocessing, and so on.

The target UEs perform data measurement, and then report collected data to network side, where dedicated resources for data report should be allocated in advance.

The procedure of data collection for data measurement at network side

As shown in Figure 4,  when data measurement is performed at network side, the detailed specification impacts are listed as follows:

Network side sends SRS configuration signaling to the target UEs when a dedicated SRS configuration is required to support data collection.

Network side sends data collection request to the target UEs.

Network side performs measurement for training input, such as CIR.

The target UEs report related training labels to network side, where dedicated resources for training labels report should be allocated in advance.

Study signaling, procedures and assistance information for data collection for both cases where measurement is conducted at UE side and at the network.

Model training usually requires large amounts of training data and computational resources. Considering limited battery capacity and available training data of mobile terminals, we think that online model training with a large-scale dataset should be avoided at UE side. It is worth noted that AI/ML model updating with a small dataset at the UE side should be supported and should not be excluded as type of UE online training.

Real-time on-device model training with a large-scale dataset should be avoided at UE side.

Model indication/configuration

- To support flexible model deployment, some necessary assistance information related to model indication is required. In general, model indication consists of signaling for model transfer, model activation/deactivation and model selection.

Model transfer

At the RAN1#110 meeting, it was agreed that:

Agreement

Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least

- UE-side or Network-side training

- UE-side or Network-side inference

  - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches

Following the above agreement, there are five possible cases for model training and inference:

Case 1: Model training at network side, model inference at UE side.

Case 2: Model training at UE side, model inference at network side.

Case 3: Model training at some UEs, model inference at other UEs (requires model transfer, e.g., different UE manufacturers).

Case 4: Model training and inference at network side.

Case 5: Model training and inference at UE side (requires no model transfer).

As shown in Table 2, we further analyze the advantages and disadvantages of these cases in detail from the perspectives of overhead and performance.

Analysis of advantages and disadvantages for different cases of model training/inference

| - Case 1: Model training at network side, model inference at UE side. |  |
| --- | --- |
| - Need model transfer? | - Yes |
| - Advantages | - Disadvantages |
| Less concern on training complexity due to the availability of enough training hardware resource at NW. Model life cycle management (e.g., monitoring and update) can be better supported by NW since the full knowledge of the model is available at NW. | The overhead of model delivery cannot be neglected. A unified AI/ML model representation format is required to align the understanding of the model across network and UEs. The overhead of training data reporting cannot be neglected. |
| - Case 2: Model training at UE side, model inference at network side |  |
| - Need model transfer? | - Yes |
| - Advantages | - Disadvantages |
| The computational resource of UE side can be leveraged to train an AI model jointly through distributed learning frameworks, such as federated learning. The overhead of training data reporting can be avoided. | The AI/ML model trained by UE side may not be always reliable or suitable, and applying distributed learning into communication is premature and still has a long way to go. The overhead of model delivery cannot be neglected. A unified AI/ML model representation format is required to align the understanding of the model across network and UEs. |
| - Case 3: Model training at some UEs, model inference at other UEs (requires model transfer) |  |
| - Need model transfer? | - Yes |
| - Advantages | - Disadvantages |
| The computational resource of some UEs can be leveraged to train an AI model jointly through distributed learning frameworks, such as federated learning. A wide range of other UEs (e.g., those not capable of training due to resource constraints and/or from different UE manufactures) can still utilize that trained model for system performance benefit. The overhead of training data reporting can be avoided. | The overhead of model delivery can be not neglected. A unified AI/ML model representation format is required to align the understanding of the model across network and UEs. |
| - Case 4: Model training and inference at network side |  |
| - Need model transfer? | - No |
| - Advantages | - Disadvantages |
| The overhead of model delivery is negligible A unified AI/ML model representation format across network and UEs is not required. Less concern on training complexity due to the availability of enough training hardware resource at NW. Model life cycle management (e.g., monitoring and update) can be better supported by NW since the full knowledge of the model is available at NW. | The overhead of training data collection cannot be neglected. The overhead of training data reporting cannot be neglected if AI/ML model relies on UE measurement/report as the input |
| - Case 5: Model training and inference at UE side (requires no model transfer) |  |
| - Need model transfer? | - No |
| - Advantages | - Disadvantages |
| The overhead of model delivery is negligible A unified AI/ML model representation format across platforms is not required. | Only limited UEs (e.g., those with training capability and/or from the same UE manufacture as those training capable UEs) can utilize the AI/ML model The AI/ML model trained by UE side may not be always reliable and/or suitable for deployment scenario due to the limited training data and hardware resource. It is hard for NW to help UEs with model monitoring and update due to the lack of model meta-information at NW. |

As we discussed above, online model training with a large scale dataset should be avoided at UE side. Then at least for downlink positioning, pre-trained AI/ML model should be indicated to UE side such that a UE can conduct inference to get the UE location.

When AI/ML model is deployed at network side, there is no specification impart on air-interface for model transfer. When AI/ML model is deployed at UE side, network side should transfer the model information to the target UE. The model information may consist of the input and output of model, the architecture of the model,  the weight of the model, the configuration of the model, the state of the optimizer, and so on. Moreover, UE side should report the device capability to network side in advance, so that network side can verify whether the target UE supports AI/ML based positioning or what kind of  model should be configured for the target UEs. The detailed specification impacts are listed as follows.

UE side reports the UE capability to network side. This process may also be triggered by network side, such as sending a device capability request to UE side.

Network side sends the model information to the target UE if the target UE can support AI/ML based positioning.

Information exchange procedure for model configuration

In particular, model information should contain meta-information indicating the physical and network environment or condition under which the model is suitable for operation. For example, the physical and network environment or condition consists of PRS configuration, SNR range, TRP IDs, and so on.

When AI/ML model is deployed at UE side, network side should transfer the model information to the target UE.

Model information should contain meta-information indicating the physical and network environment or condition under which the model is suitable for operation.

Model activation/deactivation

An AI/ML model may not always work well due to user mobility and environmental changes. For example, the user is out of the current model's service area or the surroundings have changed significantly. In such case, a new AI/ML model is required to continue the high-accuracy positioning service for target UEs. The process of model activation and deactivation is needed to flexibly control the model's lifecycle, so as to ensure positioning performance.

When AI/ML model is deployed at UE side and the current model does not work well, network side should send a model deactivation signaling to invalidate the current model. Then, network side may transfer a new model to UE side or instruct UE side to fine-tune the current model. Optionally, falling back to non-AI methods should be also supported. Finally, network side should activate the new model to provide AI/ML based positioning service for UEs. In addition to network-side triggering, UE side can also trigger model activation or deactivation by proactively transferring the model activation or deactivation request to network side. The detailed specification impacts are listed as follows.

UE side sends model deactivation request to network side when model deactivation is triggered by UE side.

Network side should send a model deactivation signaling to invalidate the current model.

Network side may transfer a new model to UE side or instruct UE side to fine-tune the current model.

UE side sends a model activation request to network side when model activation is triggered by UE side.

Network side should send a model activation signaling to activate the new model.

Information exchange procedure for model activation and deactivation

The process of model activation and deactivation is needed to flexibly control the model's lifecycle, so as to ensure positioning performance.

Model selection

Model selection is the process of selecting a suitable model from a pre-deployed model pool. In practice, considering the dynamics and complexity of the environment, a model pool may be deployed in advance at UE side to enable seamless model switching. When the current model does not work well, network side can indicate the target UE to conduct model selection immediately, so as to adapt to the new environment, which is different from either the aforementioned model transfer or fine-tuning (which is described in detail in section 3.4 as part of model monitoring/updating). In our view, model selection may enable seamless model switching to achieve better service continuity as compared to model transfer or fine-tuning, since the time consumption of parameter transmission over the air-interface and model finetuning can be saved. The detailed specification impacts are listed as follows:

Network side sends a model pool to UE side in advance, the models in which are used for positioning.

Network side sends a model selection instruction to the target UE to perform model selection. This instruction may consist of an ID set of candidate models or other assistance information to support UE side perform model selection.

UE side selects a suitable model from the model pool with reference to the model selection instruction.

Information exchange procedure for model selection

In particular, to reduce the overhead of model transfer, the deployed model pool can consist of multiple AI/ML models with same structure but different parameters to adapt to varying environments. Meanwhile, each of AI/ML models within the model pool could be associated with a meta-information to assist model selection. For example, each of AI/ML models may be associated with one PRS configuration.

Network side could send a model selection instruction to instruct the target UE to select a suitable model from the model pool, when the current model does not work well.

Model monitoring and updating

At the RAN1#110 meeting, it was agreed that:

Agreement

Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement

- AI/ML model monitoring performance metrics

- Condition of AI/ML model update

- Reference signals and measurement feedback/report

- Other aspects are not precluded

Offline-trained AI/ML model may not always work well due to the change of wireless propagation environments, such as the change of cabinet location in the factory. In such case, we can fine-tune the model with some newly collected field data. But the first step is to make the network (network side and UE side) aware that the current model is no longer valid. Therefore, model monitoring is a necessary procedure to maintain model performance in the process of model inference.

There are two granularities of model monitoring:

Fine granularity: The current model only fails at certain locations or at certain times due to some unexpected factors, such as the shading from moving vehicles, but works well at other locations or at other times. In such case, it is not necessary to collect filed data and further fine-tune the current model. The performance of fine granularity can be monitored according to the immediate output of the model.

Coarse granularity: The current model fails at many locations or at many times due to the change of wireless environments, such as the internal structure of the factory. In such case, model fine-tuning is required to adapt the model to the new environment. The performance of coarse granularity can be monitored according to the long-term statistics of positioning error or channel condition.

Model monitoring

When the AI/ML model is deployed at UE side, model monitoring can be performed at UE side. In such case, network side should send a model monitoring instruction to the target UE, and then, the results of model monitoring should be fed back to network side. The detailed specification impacts are listed as follows.

Network side should send a model monitoring instruction to inform the target UE to perform model monitoring. This instruction may also include an assistance information request containing the specific model monitoring performance metrics which UE side should measure. Moreover, the process of such model monitoring can also be triggered by UE side.

UE side performs model monitoring, and then feeds back a model validity identification to network side. Optionally, the reason for model invalidation can be also attached when UE side confirms the current model does not work well.

Information exchange procedure when model monitoring is conducted at UE side

The assistance information from network side is required to support model monitoring at UE side.

When the AI/ML model is deployed at UE or network side, model monitoring can be performed at network side.

When model inference is performed at UE side and model monitoring is performed at network side, network side should firstly send an assistance request to inform UE to measure and report related performance metrics for model monitoring. Then, UE side should send requested assistance information for model monitoring to network side if available. In the end, the results of model monitoring should be fed back to UE side. It is noted that the procedure of model monitoring can be also triggered by network side. In such case, the model monitoring request from UE side is not needed. The detailed specification impacts are listed as follows.

Network side should send a model monitoring instruction to inform the target UE to measure and report related performance metrics for model monitoring at network side. Moreover, the process of such model monitoring can also be triggered by UE side.

UE side reports the assistance information for model monitoring to network side. This assistance information contains the required performance metrics of model monitoring.

Network side performs model monitoring, and then feeds back a model validity identification to UE side. Optionally, the reason for model invalidation can be also attached when then network side confirms the current model does not work well.

Information exchange procedure when model inference is performed at UE side and model monitoring is performed at network side

When model inference and model monitoring are performed at network side, network side should send assistance information request to UE side for model monitoring firstly. Then, UE side should send the requested assistance information to network side if available. The detailed specification impacts are listed as follows.

Network side sends an assistance information request to UE side for model monitoring. This assistance information request may also contain the specific model monitoring performance metrics which UE side should measure and report to network side.

UE side reports the requested assistance information to network side if available.

Information exchange procedure when model inference and model monitoring are performed at network side

The assistance information from UE side is required to support model monitoring at network side.

Before touching the specific model monitoring performance metrics, we firstly analyze the causes of model failure or invalidation, which are listed as follows:

Physical environment changes, such as the internal structure of the factory.

Network environment changes, such as SINR range, synchronization error range, PRS configuration.

Other unexpected factors, such as obstacles caused by moving vehicles.

Specifically, the physical and network environments change can be detected by some sensors or long-term statistic information, while the impact of other unexpected factors is temporary and negligible which can be eliminated with statistic information.

Based on above analysis, we list possible AI/ML model monitoring performance metrics as follows：

Forward inference

  - Environment monitoring, such as by visual sensors deployed at factories.

  - Long-term CSI statistic

  - Long-term SNR statistic

  - Long-term synchronization error detection

  - PRS configuration matching

  - Other possible features

Backward inference

  - Long-term model inference error (location or other intermediate features)

A model monitoring framework

As show in Figure 11, forward inference performs model monitoring based on the feature statistic related to model input, while backward inference performs model monitoring according to the error statistic related to model output.

Moreover,  dedicated reference signals may be required to perform model monitoring, such as configuring more dense PRS to obtain near-truth inference for backward extrapolation.

It is noted that when AI/ML based positioning methods are out of work, a fallback mechanism should be also considered as an option.

The possible AI/ML model monitoring performance metrics are listed as follows：

Forward extrapolation

  - Environment monitoring, such as by visual sensors deployed at factories.

  - Long-term CSI statistic

  - Long-term SNR statistic

  - Long-term synchronization error detection

  - PRS configuration matching

  - Other possible features

Backward extrapolation

  - Long-term inference error (location or other intermediate features)

Dedicated reference signals may be required to obtain performance metrics so as to support model monitoring.

Model updating/fine-tuning

In our companion contribution [2], we have shown that model fine-tuning can significantly improve model generalization performance. To adapt to the dynamic wireless environment quickly, model fine-tuning is necessary.

In general, model fine-tuning can be regarded as model updating with prior model information and a small scale field data. In this section, we further analyze the detailed specification impacts when model fine-tuning is included.

For the case in which AI model inference is at UE side, fine-tuning can be conducted at UE side based on the pre-trained model. The detailed specification impacts are listed as follows.

UE side needs to inform the network of device capability in advance. The device capabilities may consist of computation capability, available computation resource, available storage resource, battery status, data collection, and other capabilities related to model fine-tuning.

Based on the received device capability from the target UE, if network side identifies that the device can support model fine-tuning, network side delivers assistance information including pre-trained model information and training configuration to the target UE to support model fine-tuning. The model information may consist of the input and output of the model, the architecture of the model,  the weight of the model, the configuration of the model, the state of the optimizer, and so on. The training configuration consists of the learning rate, training epoch,  batch size,  end condition, and so on.

Information exchange procedure when fine-tuning is conducted at UE side

When fine-tuning is conducted at UE side, UE capability corresponding to fine-tuning is required.

To enable model fine-tuning when AI/ML model inference is at UE side, support assistance information to the target UE about pre-trained model and training configuration.

- For the case that AI model inference is at UE or network side, fine-tuning can be conducted at network side based on the pre-trained model. In this case, UE side does not need to inform network side of device capability, and network side does not need to transfer model information and training configuration to the target UE. To support model fine-tuning at network side, UE should feed back some requested training data to network side. The detailed specification impacts are listed as follows.

Network side should send data collection requests to the target UEs in advance. The data collection request may consist of data format, privacy permission, training data collection configuration, training data reporting configuration, and so on.

UE should provide training data to network side to support model fine-tuning if available.

Information exchange procedure when fine-tuning is conducted at network side

Training data collection request for model fine-tuning and feedback from the target UE is required to support model fine-tuning at network side.

Regarding the occasion or condition for triggering a model updating process, there are two points to consider:

The current model cannot work well. For example, the result of model monitoring indicates that the current model is out of work.

Model updating operation can be supported in terms of available training data and hardware resource.

The result of model monitoring and the achievability of model updating should be jointly considered as the condition of model updating.

Model inference

From the perspective of model deployment, this section discusses the detailed specification impact of AI/ML model inference for direct AI/ML positioning and AI/ML assisted positioning respectively. As we discussed in section 3.2 and showed the performance results in our companion contribution [2], we think at least time domain CIR should be selected as one model input.

Support time domain CIR as one model input for AI/ML based positioning.

For direct AI/ML positioning

When the AI/ML model has already been deployed and activated at UE side, UE side can perform positioning using this model. For direct AI/ML positioning, there is no extra specification impart when model inference is performed at UE side.

In the case of AI/ML model inference at the network side, UE side does not need to inform network side of device capability, and network side does not need to deliver model information to the target UE. To enable direct AI/ML positioning, information exchange is still necessary. The detailed specification impacts are listed as follows.

Network side should inform UE side of the required measurement type (e.g., CIR as input to AI/ML model for inference).

UE side reports requested measurement feedback.

Network side performs position estimation and then transfers positioning results to the target UE if the positioning application is originated from UE side.

Information exchange procedure for direct AI/ML positioning when model inference is at network side

For direct AI/ML positioning, when model inference is at network side, request to and feedback from the target UE of the necessary measurement (e.g., as the input to the AI/ML model) for model inference is needed.

For AI/ML assisted positioning

When the AI/ML model has already been deployed and activated at UE side, UE side can utilize this AI/ML model to extract the intermediate feature. The detailed specification impacts are listed as follows.

UE side reports the intermediate feature to network side where the intermediate feature is extracted by the AI/ML model.

Network side acting as an Actor performs position estimation according to the received intermediate feature, and then feeds back the positioning results to the target UE if the positioning application originates from UE side.

Information exchange procedure for AI/ML assisted positioning when model inference is at UE side

For AI/ML assisted & UE assisted positioning, support the target UE to report the output of AI/ML model inference (intermediate feature for positioning) when model inference is at UE side.

In the case of AI/ML assisted positioning and AI/ML model inference is at the network side, UE side does not need to inform network side of device capability, and network side does not need to deliver model information to the target UE. To enable AI/ML assisted positioning, the interaction of assistance information is still necessary. The detailed specification impacts are listed as follows.

Network side should inform UE side of the required measurement type (e.g., CIR as input to AI/ML model for inference).

UE side reports requested measurement.

Network side utilizes the AI/ML model to extract intermediate features from received measurement, and performs position estimation by conventional positioning methods. Then, networks side sends the positioning results to the target UE if the positioning application originates from UE side.

Information exchange procedure for AI/ML assisted positioning when model inference is at network side

For AI/ML assisted positioning, when model inference is at network side, request to and feedback from the target UE of the necessary measurement (e.g., as the input to the AI/ML model) for model inference is needed.

A general model management procedure

In the above sections, we have listed common model operations and analyzed their potential specification impacts, respectively. Here, a general model management procedure is presented by integrating all the above model operations. For convenience, only UE based direct AI/ML positioning is given, and similar procedures can be easily extended to other cases.

A general model management procedure for UE based direct AI/ML positioning

A general model management procedure should be specially studied for AI/ML based positioning accuracy enhancement.

A detailed list of assistance signaling

From section 3.2 to section 3.6, we have analyzed the potential specification impacts of each aspect of model operation in high level, and presented a general model management procedure. There are some requirements for assistance signaling involved in this procedure to support model management. In this section, we further analyze the possible assistance signaling configuration and present a detailed list of assistance signaling.

A detailed list of assistance signaling

| Procedure of model management |  | Assistance signaling |  |
| --- | --- | --- | --- |
|  |  | From network to UE | From UE to network |
| UE capability |  | / | - Device capability to support model management, e.g., data collection, model training, model inference |
| Model training | Data collection | - PRS/SRS configuration (such as RS pattern, RS resource set) - Data requirement (such as format, type, size, preprocessing, labeling) | - Data collection report, including model input and label |
| Model indication/configuration | Model transfer | - Model information (such as structure, parameters, input type and shape, output type and shape) - Meta information | / |
|  | Model activation/ deactivation | - Dedicated signaling for model activation/deactivation - Model ID to be activated/deactivated | / |
|  | Model selection | - A model pool for positioning - A set of selected model ID | / |
| Model monitoring and updating | Model monitoring at network | - Performance metrics fed back from UE (such as positioning error, channel condition, movement status) - Model validity identification | - Required Performance metrics - Other information useful for model monitoring (such as PRS quality, sensor information, movement status, and positioning results from other positioning methods) |
|  | Model monitoring at UE | - Performance metrics monitored at UE (such as positioning error, channel condition, movement status) | - Model validity identification |
|  | Model updating/ fine-tuning at network side | / | / |
|  | Model updating/ fine-tuning at UE | - Training configuration (such as training method, learning rate, optimizer, epochs) - Pre-trained Model information (such as structure, parameters, input type and shape, output type and shape) - Training data (e.g., collected by UEs) | Model training termination instruction |
| Model inference at UE | Direct AI/ML positioning | / | / |
|  | AI/ML assisted positioning | Required measurement result type(such as ToA, LOS/NLOS identification) | - Intermediate features (such as TOA, LOS/NLOS identification) |
| Model inference at network | Direct AI/ML positioning | - Required measurement type (such as CIR) | - Measurement (such as CIR) |
|  | AI/ML assisted positioning | - Required measurement type (such as CIR) | - Measurement (such as CIR) |

Support to study detailed assistance signaling configuration when the model management procedure for AI/ML based positioning is agreed.

Conclusions

In this contribution, we discuss AI/ML based positioning accuracy enhancement with the following observations and proposals.

- Both direct AI/ML positioning and AI/ML assisted positioning show significant accuracy improvement compared to conventional RAT-dependent positioning methods in a heavy NLOS scenario.

- There’s no need to consider other sub use case(s) as representative or to down select between direct AI/ML positioning and AI/ML assisted positioning during this SI for performance and potential specification impact study.

- Support time domain CIR as one model input for training of AI/ML model for positioning.

- Study signaling, procedures and assistance information for data collection for both cases where measurement is conducted at UE side and at the network.

- Real-time on-device model training with a large-scale dataset should be avoided at UE side.

- When AI/ML model is deployed at UE side, network side should transfer the model information to the target UE.

- Model information should contain meta-information indicating the physical and network environment or condition under which the model is suitable for operation.

- The process of model activation and deactivation is needed to flexibly control the model's lifecycle, so as to ensure positioning performance.

- Network side could send a model selection instruction to instruct the target UE to select a suitable model from the model pool, when the current model does not work well.

- The assistance information from network side is required to support model monitoring at UE side.

- The assistance information from UE side is required to support model monitoring at network side.

The possible AI/ML model monitoring performance metrics are listed as follows：

Forward extrapolation

  - Environment monitoring, such as by visual sensors deployed at factories.

  - Long-term CSI statistic

  - Long-term SNR statistic

  - Long-term synchronization error detection

  - PRS configuration matching

  - Other possible features

Backward extrapolation

  - Long-term inference error (location or other intermediate features)

Dedicated reference signals may be required to obtain performance metrics so as to support model monitoring.

- When fine-tuning is conducted at UE side, UE capability corresponding to fine-tuning is required.

- To enable model fine-tuning when AI/ML model inference is at UE side, support assistance information to the target UE about pre-trained model and training configuration.

- Training data collection request for model fine-tuning and feedback from the target UE is required to support model fine-tuning at network side.

- The result of model monitoring and the achievability of model updating should be jointly considered as the condition of model updating.

- Support time domain CIR as one model input for AI/ML based positioning.

- For direct AI/ML positioning, when model inference is at network side, request to and feedback from the target UE of the necessary measurement (e.g., as the input to the AI/ML model) for model inference is needed.

- For AI/ML assisted & UE assisted positioning, support the target UE to report the output of AI/ML model inference (intermediate feature for positioning) when model inference is at UE side.

- For AI/ML assisted positioning, when model inference is at network side, request to and feedback from the target UE of the necessary measurement (e.g., as the input to the AI/ML model) for model inference is needed.

- A general model management procedure should be specially studied for AI/ML based positioning accuracy enhancement.

- Support to study the detailed assistance signaling configuration when the model management procedure for AI/ML based positioning is agreed.

References

3GPP RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”,  #94e, Dec.6-17, 2021.

vivo, R1-2208638 “Evaluation on AI/ML for positioning accuracy enhancement”,  RAN1 #110bis-e, October 10-19, 2022.

Nokia, R3-222653, “(TR for TR 37.817)  General Framework”, #115-e, Feb.21– Mar.3, 2021.