3GPP TSG RAN WG1 #110bis-e	                                                                          R1-2208668

e-Meeting, October 10th – 19th, 2022

Source:	vivo

Title:	Evaluation methodologies for R18 LP-WUS/WUR

Agenda Item:	9.13.1

Document for:	Discussion and Decision

- Introduction

In RAN plenary #94e meeting, the SID for R18 low-power Wake-up Signal and Receiver for NR was agreed [1], and the objectives are given below:

The study item includes the following objectives: - Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms and their coverage availability, as well as latency impact. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In this contribution, the target use cases, design targets/KPIs, evaluation aspects for R18 LP-WUS/WUR study are firstly discussed. Then we give our views on the power evaluation methodologies for both RRC idle/inactive and connected modes, followed by the initial simulation results. After that, LLS and coverage evaluation methodology for LP-WUS is discussed as well.

- Target use case and applicable RRC states

As per the R18 LP-WUS/WUR SID [1], the study on Low Power Wake-up Signal or receiver (LP-WUS/WUR) should target for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables. Also, other use cases are not precluded, e.g. XR/smart glasses, smart phones. Table 1 summarizes the use cases for LP-WUR/WUS. For the devices that are out of reach, small form-factor or rechargeable, power saving will become a very crucial requirement for them to extend battery life or standby time. For example, some industrial sensors used for infrastructure, medical implants or security systems are inaccessible and vital, so the battery life of them need to be extended as longer as possible. In addition, mobility of these use case is another design aspect. For IoT devices, the mobility requirement of them should be stationary or nomadic. For wearables, XR devices or smart phones, the mobility requirement for LP-WUS/WUR is 3~100km/h.

Table 1. Use cases for LP-WUR/WUS

| Use case | Power | Latency | Mobility |
| --- | --- | --- | --- |
| Unreachable, small form-factor or rechargeable sensors | Last for years | Within tens of seconds | Stationary, nomadic |
| Fire alarms sensors | Last for years | Within 1 or 2 seconds | Stationary, nomadic |
| Controllers | Last for years | Within several seconds | Stationary, nomadic |
| Wearable | Last for a few weeks | Within several seconds | low/medium speed |
| XR glasses | Last for days | Within several milliseconds | low/medium speed |
| Smart phone | Last for days | Within tens or hundreds of milliseconds | low/medium speed |

Besides, latency sensitive use cases should also be oriented in the study on R18 LP-WUS/WUR. For example, fire shutters and fire sprinklers shall be triggered by the actuators within 1 to 2 seconds. Additionally, XR service has the stringent requirements for both power consumption and latency (e.g., 10ms packet delay budget for XR DL video traffic).

Proposal 1: Study the following use cases for LP-WUS/WUR:

- For IoT devices (e.g., sensors and controllers): The battery should last at least few years; Latency would be within several or tens of seconds; For latency sensitive sensors/actuators, the latency requirement is 1 or 2 seconds; The mobility would be stationary or nomadic.

- For wearable devices: The battery should last a few weeks; Latency need to be within several seconds; Support of low/medium speed is required.

- For XR devices or smart phones: power saving in CONNECTED mode is desirable; Latency which is critical for RRC CONNECTED state need to be in the order of milliseconds; Support of low/medium speed is required.

In RRC idle/inactive mode, LP-WUS/WUR is applicable for UEs to avoid periodically wake up once per paging cycle. By using LP-WUS which is detected via a separate receiver (termed as LP-WUR) operating in ultra-low power consumption level (similar to power off), the UE can save the unnecessary wake-up power and be triggered on demand. By the assistance of LP-WUS detection, the UE can wake up in time so that the large delay caused by long eDRX cycle can be significantly reduced as well.

Apart from RRC idle/inactive mode, the applicability of LP-WUS/WUR in RRC connected mode is also worth studying. For XR use case, to guarantee 10ms packet delay budget (PDB) requirement, main radio can be kept in micro or light sleep state (as modelled in TS38.840 [2]) instead of an ultra-deep sleep state. As such, the additional latency by adopting LP-WUS/WUR will only be 0ms or 3ms. And the total power consumption including both main radio and LP-WUR for LP-WUS monitoring can be less than half of the power consumption of PDCCH monitoring.

Observation 1: For RRC connected mode, the additional latency by adopting LP-WUS/WUR is up to 3ms.

Proposal 2: Study the application of LP-WUS/WUR in both RRC idle/inactive mode and RRC connected mode.

- Design targets and KPIs

As analysed above, we further discuss the design targets and KPIs of these target use cases from the perspectives of power consumption, latency, etc.

- KPI 1: Power consumption of LP-WUR

The overall target for the power consumption of LP-WUR is to strive for a longer battery life or standby time of device. For RRC idle/inactive UEs, the average UE power consumption is currently on several or tens of mW. To achieve substantial power saving gain, a reasonable target power consumption of LP-WUR need to be reduced by a factor of 100 to 1000, which is tens or hundreds of μW.

Observation 2: For RRC idle/inactive UEs, the average UE power consumption is currently on several to tens of mW.

Observation 3: To achieve substantial power saving gain, a reasonable target power consumption of LP-WUR need to be at the level of 1/100~1/1000 of the main receiver, corresponding to tens to hundreds of μW.

In addition, based on the power consumption target set for LP-WUR, we roughly evaluate how much power saving gain can be obtained by using LP-WUS/WUR in actual data transmission compared with the existing paging scheme. For wearable case, as shown in Figure 1, when the LP-WUR power consumption is 30μW~500μW, around 67%~87% power saving gain can be achieved by using LP-WUS/WUR compared to paging scheme (that is, UE power consumption can be reduced by 3 to 7 times). Thus, targeting the power consumption of LP-WUR to be tens or hundreds of μW, substantial power saving gain can be achieved.

Figure 1. Estimated power saving gain under different power consumption of LP-WUR

Observation 4: When the LP-WUR power consumption is 30μW~500μW, UE battery life can be increased by 3~7 times compared with legacy UEs.

For IoT case such as industrial sensors, the approximate standby time (without considering any data transmission process) of sensors under different target power consumption of LP-WUR can also be calculated by simple numerical estimation. As shown in Table 2, the standby time can be extended to up to 10 years with the use of a button battery for power supply.

Table 2. The standby time by adopting LP-WUR/WUS with the use of a button battery for power supply

| Supported standby time |  | Button battery |  |
| --- | --- | --- | --- |
|  |  | 210mAh (3V) | 950mAh (3V) |
| The power consumption of LP-WUR | 30μW | 2.1 years | 9.3 years |
|  | 100μW | 0.7 years | 3.3 years |
|  | 200μW | 0.4 years | 1.6 years |
|  | 500μW | 0.1 years | 0.7 years |

Observation 5: When the LP-WUR power consumption is 30μW~500μW, the standby time of IoT devices can be extended to up to 10 years with the use of a button battery for power supply.

According to the analysis above, tens or hundreds of μW is verified to be a reasonable target KPI for LP-WUR power consumption.

Proposal 3: The target KPI for LP-WUR power consumption should be tens to hundreds of μW.

- KPI 2: Latency

The latency caused by adopting LP-WUS/WUR is from receiving LP-WUS to being ready for NR signals/channels reception by main radio, which consists of two parts as follows:

- Wake-up delay:

- LP-WUS length

- LP-WUS monitoring cycle

- Transition time:

- The sleep transition time of main radio

Proposal 4: The latency introduced by LP-WUR/WUS needs to consider two parts: wake-up delay and transition time of main radio.

The LP-WUS length depends on the specific LP-WUS design. And the expected LP-WUS length is expected to be no more than several slots or milliseconds. In this sense, the main contribution of the latency comes from the LP-WUS monitoring cycle and the transition time of main radio.

Observation 6: The main contribution of latency comes from the LP-WUS monitoring cycle and the transition time of main radio.

For RRC idle/inactive mode, since there are variety of use cases with different latency requirements, it will be difficult to set very certain latency targets. The transition time of main radio is relatively fixed. So, the latency will highly depend on the LP-WUS monitoring cycle. To reduce the latency caused by eDRX cycle, the length of LP-WUS monitoring cycle may need to be less than that of eDRX cycle.

For RRC connected mode, since XR is the target use case for RRC connected mode, the target latency should orient to XR service which requires much stringent latency requirements. Considering the PDB of XR DL video traffic is 10ms, the total latency by adopting LP-WUS/WUR should be no more than several milliseconds. In addition, during LP-WUS monitoring, main radio can enter micro or light sleep state, so the additional transition time can up to 3ms (i.e., half of transition time of light sleep).

Proposal 5: Target KPI of latency should be as follows：

- Depends on the length of LP-WUS monitoring cycle for RRC idle/inactive mode

- Wake up delay: depends on the length of LP-WUS monitoring cycle, e.g. hundreds of milliseconds

- Transition time: hundreds of milliseconds

- Several milliseconds for RRC connected mode

- Wake up delay: 0 (assuming continuous monitoring of LP-WUS)

- Transition time: up to 3ms

- Design target 1: Coverage

There are multiple physical channels, while different channels have different coverage. The coverage of gNB depends on the bottleneck channel. The coverage of the LP-WUS should be at least comparable to the NR bottleneck channel for corresponding deployment scenarios.

Proposal 6: Coverage of the LP-WUS should be comparable to the NR bottleneck channel.

- Design target 2: Data rate

For idle UEs, the maximum data rate for paging can be calculated as R = 48 bits (UE ID length) * 32 (UEs per PO) * 4(POs per radio frame)/10ms = 614kbps. While for LP-WUS, highest data rate may not necessary, e.g., full UE ID delivery may be not required. Hence, around 100kbps data rate design target can be considered.

Proposal 7: Around 100kbps data rate can be considered as design target for LP-WUS.

- Design target 3: coexistence

LP-WUS need to support at least in band operating with legacy NR signals/channels. And the overall design target of coexistence is to support multiplexing between LP-WUS with legacy NR signals/channels and to allow reuse of unused LP-WUS resources for other downlink transmissions.

Proposal 8: The overall design target of coexistence is to support multiplexing between LP-WUS with legacy NR signals/channels and to allow reuse of unused LP-WUS resources for other DL transmissions.

- General consideration for evaluation

In this section, we summarize the evaluation aspects for R18 LP-WUS/WUR study.

- Power consumption

As per the SID [1], the most important aspect of the evaluation is to study the potential UE power saving gain compared to the existing R15/16/17 UE power saving mechanisms. For RRC idle/inactive mode, the numerical analysis method which is adopted for the evaluation of R17 paging enhancement can be reused for the evaluation of R18 LP-WUS/WUR power consumption. Besides, for R18 LP-WUS/WUR evaluation in RRC connected mode, sysmtem level simulation is needed. Besides, the evulation methodologies and assumptions agreed in R17/18 XR can be reused.

Proposal 9: Adopt the following power consumption evaluation methods for R18 LP-WUS/WUR study:

- For RRC idle/inactive mode, the simulation assumptions for R17 Power saving paging enhancement evaluation can be reused.

- For RRC connected mode, the simulation assumptions for R17/18 XR power evaluation can be reused.

- Latency

The latency by adopting LP-WUS/WUR is approximately equal to 1/2 LP-WUS monitoring cycle length + transition time of main radio, since the latency caused by LP-WUS length is negligible.

Proposal 10: The latency can be roughly caculated as 1/2 LP-WUS monitoring cycle length + transition time of main radio.

- WUS detection performance

For LP-WUS channel, the FAR and MDR should be considered in evaluation. The required SINR for miss detection should be read at 1% BLER as that the PDCCH in R17 CovEnh SI. The required FAR alarm rate should be limited, otherwise the false wake up may diminish the power saving gain brought by the LP-WUS. 0.1% FAR can be assume in evaluation. Note that, the FAR should be regarded as the overall false alarm rate of the LP-WUS channel. If the LP-WUS channel is composed of preamble and data with CRC attachment, the target FAR can be achieved by taking both the preamble sequence and the CRC bits into consideration, rather than required for sequence part only or for CRC part only.

Proposal 11: FAR and MDR is used as the performance metric for LP-WUS in link level simulation,

- {FAR, MDR}: {0.1%, 1%} can be assumed as starting point.

- WUS Coverage

MIL is a simple metric defined in Coverage Enh SI, which can well reflect the coverage of physical channels. Compared with MCL, more antenna gain components are considered in MIL calculation, coverage of physical channels can be better reflected by MIL metric. Hence, MIL can be reused as the metric for relative comparison between coverage of LP-WUS and NR channels.

Proposal 12:  MIL metric defined in Coverage Enh SI can be reused as the metric for relative comparison between coverage of LP-WUS and NR channels.

- Other performance metrics

  - Resource overhead

We need to strive for a minimized resource overhead of LP-WUS, e.g. to minimize the guard band between LP-WUS and NR signals/channels. And numerical analysis method adopted in R17 paging enhancement or R18 NW energy saving study can be reused.

  - Network power consumption

The network power conumption brought by LP-WUS/WUR depends on the resource overhead of LP-WUS, and the impact on it needs to be minimized. And the evaluation method used in R18 NW energy saving study can be considered.

  - Capacity

For capacity impact, capacity need to be evaluated by system level simualtion and the evaluation methodologies of capacity can reuse that of XR for RRC connected mode. For example, if the latency is impacted by WUS, with the delay bound set by XR traffic assumption, the capacity may be impacted.

Proposal 13: Resource overhead and network power consumption are evaluated by numerical analysis, using the outcome of NW energy saving study. Capacity impact is evaluated by system level simualtion using XR evaluation methodlogy for RRC CONNECTED.

- Power consumption evaluation

  - Evaluation methodologies

Separate wake-up receiver can largely reduce the power consumption. Upon reception of low-power WUS, low-power WUR triggers main radio to switch on, otherwise, the main radio is OFF or keep in an extreme deep sleep mode (this is for IDLE/INACTIVE mode) or light sleep/microsleep (this is for CONNECTED mode).

Proposal 14: Adopt the following terminology for future discussion,

- Main radio：the Tx/Rx module operating for legacy system

- LP-WUR: The Rx module operating for receiving/processing LP-WUS

In this section, the following aspects for R18 LP-WUS/WUR evaluation methodologies are proposed/clarified,

- A) Extend the UE power consumption model for main radio based on TR38.840 [2].

- B) Define a new UE power consumption model for LP-WUR.

    - Extend the UE power model for Main Radio

The detailed UE power consumption model, including the defined power states and the relative power consumption values under the reference configuration etc., has been approved for the study of UE power saving as per the TR 38.840 [2]. After that, in R17 power saving and RedCap, the UE power consumption model has been extended according to different bandwidth and UE capability [3][4].

For the power evaluation of R18 LP-WUS/WUR, the UE power consumption model needs to be extended further. Since with the assistance of a separate receiver, i.e. LP-WUR, UE’s main radio has the opportunity to enter an ultra-deep sleep state (which is different from the existing sleep states), and UE will only transition out of the ultra-deep sleep state when LP-WUS is detected by LP-WUR. In the ultra-deep sleep state, almost all the components of UE main radio can be turned off, so that UE will save plenty of power from this. Meanwhile, longer transition time and more energy will be costed to convert out from the ultra-deep sleep state, compared to that of deep sleep state.

Observation 7: With the assistance of a separate receiver i.e., LP-WUR, the main radio has the opportunity to enter an ultra-deep sleep state (i.e., deeper than the existing deep sleep).

Observation 8: Compared with existing deep sleep mode, the main radio in ultra-deep sleep state consumes longer transition time and more energy for the transition from the ultra-deep sleep state to normal operation.

In the last RAN1 meeting, an ultra-deep sleep state has been agreed in R18 positioning SI as below for LPHAP (Low Power High Accuracy Positioning) evaluation. There are two options of the ultra-deep sleep state, wherein Option 1 is a universal assumption for normal terminal devices, while for Option 2, it is considered for the devices that only have positioning functionality. In this sense, for the power evaluation of R18 LP-WUS/WUR, the option 1 of the power consumption model of the ultra-deep sleep state can be reused. Besides, for the additional transition energy provided in Option 1, it can be further confirmed after there is any agreement formed in R18 positioning SI.

Agreement For the purpose of LPHAP evaluation, an ultra-deep sleep state is considered. The following options of the power consumption model of the ultra-deep sleep state can be further discussed: - Option 1: - The relative power unit: 0.015 - Additional transition energy: 2000 - Total transition time: 400ms - Option 2: - The relative power unit: 0.01 - Additional transition energy: 450; - Total transition time: 25ms - FFS: restrictions in processing associated with option 2 after the UE comes out of ultra-deep sleep state - Notes: the values above can be further discussed Agreement For option 1 in the agreement above, the value of additional transition energy is changed to “a value between 2000 and 20000”. FFS which value.

Proposal 15: For R18 LP-WUS/WUR power evaluation, the following power model of the ultra-deep sleep state agreed in R18 positioning SI for LPHAP is reused for main radio.

  - Note: the value of additional transition energy can be updated based on further agreement made in R18 positioning SI.

| Power State | Relative Power | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- | --- |
| Ultra-deep sleep | 0.015 | [2000-20000] | 400ms |

    - New power model for LP-WUR

A new power consumption model for UE LP-WUR is also needed as shown in Table 3. Similar as the power model for UE main radio, there will be two power states for LP-WUR, i.e. LP-WUR monitoring state and LP-WUR sleep state. During the LP-WUR monitoring state, UE will do LP-WUS monitoring via LP-WUR, while the UE will stop the monitoring and go to sleep during the LP-WUR sleep state. Regarding the power values of these power states, in accordance with the setting target KPI in section 3, a range of power value (including both absolute power and relative power) of each state are provisionally shown in the 5~6 columns of Table 3. And these values in brackets need to be further confirmed according to LP-WUR architecture. Moreover, the transition energy/time from LP-WUR sleep state to LP-WUR monitoring state can be assumed as 0 because of the quite small power difference between the two states.

Table 3. New power model for LP-WUR.

| Power State | Absolute Power | Relative Power | Additional transition energy | Total transition time |
| --- | --- | --- | --- | --- |
| LP-WUR sleep | [2] μW | [0.002] [1] | 0 | 0 |
| LP-WUR monitoring | [30~500] μW | [0.03~0.5] [1] | - | - |
| [1] Assume 1mW= 1units/ms, hence, 2μW = 0.002units/ms. |  |  |  |  |

Proposal 16: For R18 LP-WUS/WUR power evaluation, the following power model of LP-WUR is considered.

- Note: these value in brackets can be further confirmed based on the progress of LP-WUR architecture.

| Power State | Absolute Power | Relative Power | Additional transition energy | Total transition time |
| --- | --- | --- | --- | --- |
| LP-WUR sleep | [2] μW | [0.002] | 0 | 0 |
| LP-WUR monitoring | [30~500] μW | [0.03~0.5] | - | - |

  - Power evaluation for RRC idle/inactive mode

In this section, power evaluation assumptions have been discussed, and the initial simulation results for different power saving schemes in RRC idle/inactive mode are provided based on the proposed assumptions and the power models mentioned above.

    - Evaluation assumptions

To compare the existing R15/ 16/17 power saving schemes (including I-DRX paging, PEI and eDRX) with R18 LP-WUS/WUR, Table 4 summarizes the power saving schemes to be evaluated for RRC idle/inactive mode.

Table 4. The evaluation power saving schemes for RRC idle/inactive mode.

| Schemes | Description |
| --- | --- |
| Baseline 1-1: I-DRX Paging without PEI | Assume low, medium, high SINR cases |
| Baseline 1-2: I-DRX Paging with PEI | Assume low, medium, high SINR cases |
| Baseline 2-1: eDRX without PEI | Assume low, medium, high SINR cases |
| Baseline 2-2: eDRX with PEI | Assume low, medium, high SINR cases |
| LP-WUS/WUR | After receiving the LP-WUS, UE’s main radio will be turned on and the UE is required to monitoring its PO. |

Proposal 17: For comparison with R18 LP-WUS/WUR, both I-DRX paging and eDRX can be taken as baseline schemes.

In R17 power saving, for the study of paging enhancement, the evaluation assumptions for I-DRX paging and PEI have been discussed[2], which can be reused for R18 LP-WUS/WUR power evaluation. And the key assumptions of I-DRX paging and PEI are shown in Table 5.

Table 5. Key evaluation assumptions for I-DRX paging and PEI

| Parameters | Value |
| --- | --- |
| Paging cycle length | 1.28s |
| Number of SSB before PO | 1, 2 or 3, (used for AGC adjustment, T/F tracking, serving cell and intra-F measurement) |
| Number of SSB after PO | 1, (used for inter-F measurement); 0, (for High SINR case). |
| Number of SSB before PEI | 1, (used for PEI PDCCH receiving) |

Table 6 gives two options, the Heart beat traffic model agreed in TR 38.875 [4] and per PO/UE paging rate. And both options can be considered in R18 LP-WUS/WUR power evaluation. Considering the real traffic in RRC idle/inactive mode, we adopt option 1 in the following simulation.

Table 6. Evaluation assumptions for traffic model or paging rate

| Option 1: Traffic model (based on TR 38.875[4]) | Heart beat traffic model Model; FTP3. Packet size; 100 Bytes. Mean inter-arrival time; 60s (per UE paging rate≈2%). |
| --- | --- |
| Option 2: Per PO/UE paging rate | 10%; 1% |

Besides, eDRX scheme can also be evaluated as one of the baseline schemes, because it is expected to be adopted for power saving purpose in some cases. Table 7 further provides the key configurations of eDRX according to the Heart beat traffic model.

Table 7. eDRX configuration

| Parameters | Value |
| --- | --- |
| PTW length | 8 paging cycles, i.e. 10.24 s |
| eDRX cycle length | 48 paging cycles, i.e. 61.44s |

Proposal 18: The evaluation assumptions given in Table 4~7 should be considered for R18 LP-WUS/WUR power evaluation in RRC idle/inactive mode.

    - Initial simulation results for LP-WUS scheme

- Initial simulation results for different schemes

As per the proposed power models and evaluation assumptions mentioned above, we perform the power evaluation for different power saving schemes in RRC idle/inactive mode. To facilitate comparison and analysis, Figure 2 elaborates the simulation results based on one of the evaluation assumptions listed as follows. Note that the latency is calculated from the data arrival time to the start time of paged PO, which can be regarded as paging latency.

- Paging Latency is roughly equal to the sum of transition time+ 1/2 I-DRX or eDRX cycle length;

- No DRX configuration for LP-WUR i.e., always do LP-WUS detection;

- Assume in medium SINR case;

- Adopt the Heart beat traffic model.

- Assume the power consumption of LP-WUR is 30μW.

Figure 2. Initial simulation results of power consumption and latency for different schemes

From these statistical charts, it can be observed that LP-WUR/WUS scheme will achieve the lowest UE power consumption compared to I-DRX paging and eDRX schemes. In addition, LP-WUS/WUR can reduce UE power consumption by about 5 or 7 times compared to I-DRX paging. This is because the introduction of LP-WUR enables the UE to enter the ultra-deep sleep state, thereby saving power. Meanwhile, in terms of latency, I-DRX paging scheme will cause the lowest paging latency, closely followed by LP-WUS/WUR, and finally eDRX. Despite eDRX can achieve the similar power saving effect as LP-WUS/WUR, the causing latency of it is relatively huge, which is not suitable for the services with requirements of both long battery life and low latency. One thing to emphasize is due to the transition time of UE main radio (from ultra-deep sleep state to PDCCH monitoring state), there is additional 400ms latency by adopting LP-WUR/WUS compared to that of I-DRX paging.

Observation 9: Comparing with I-DRX paging, LP-WUR/WUS can largely reduce the UE power consumption (5x or 7x), with similar latency performance.

Observation 10: Comparing with eDRX, LP-WUR/WUS can largely reduce the latency (26x), with even lower UE power consumption.

- The trend of power consumption and latency for different schemes

In order to obviously elaborate the trend of power consumption and latency for different schemes, we further provide the following curves in Figure 3 under multiple DRX configurations for I-DRX paging, eDRX and LP-WUS/WUR schemes, respectively. Note that I-DRX paging and eDRX schemes are not configured with PEI, and the trend results actually will not be impacted by PEI configuration. Besides, for LP-WUS/WUR scheme, the duty cycle of it is assumed as 10%. From these curves, it is clear that LP-WUR/WUS can do optimized trade-off between power consumption and latency, and achieve both lowest latency and power consumption effects.

Observation 11: Comparing with I-DRX paging and eDRX, LP-WUR/WUS provides a much better trade-off between latency and power consumption.

Figure 3. The trend of power consumption and latency for different schemes

Figure 4. The results of power consumption for LP-WUS/WUR under different duty cycle configurations

Figure 4 further shows the potential power saving benefits brought by the DRX cycle configuration of LP-WUS monitoring. And these results are given by assuming the mean packet arrival is equal to 600s. Obviously, LP-WUR in AlwaysOn operation would consume 88.37 unit/s power consumption, and duty-cycled LP-WUS monitoring (i.e., LP-WUR with DRX configuration) will further cut the power demand into 56.06 unit/s. Thus, by using duty-cycled LP-WUS, around 36% power saving gain can be achieved at the expense of some latency compared to AlwaysOn LP-WUR.

Observation 12: Comparing with AlwaysOn LP-WUS monitoring, duty-cycled LP-WUS monitoring can further reduce power consumption while sacrificing latency performance.

  - Power evaluation in RRC connected mode

    - Motivation to adopt LP-WUS/WUR in RRC connected mode

As mentioned earlier, XR service with stringent requirements on both power consumption and latency is the target use case for RRC connected mode. Per the discussion in R17/18 XR, one of the identified pain point issues is XR data packets having a wide range of unpredictable jitter. UE has to perform intensive PDCCH monitoring in the entire jitter range in order to receive the scheduling of XR traffic timely and avoid packet loss caused by exceeding the PDB (e.g., 10ms for XR video traffic). However, the existing R15/16/17 power saving schemes (including R17 PDCCH monitoring adaptation) can only reduce partial unnecessary PDCCH monitoring by switching the periodicity of PDCCH monitoring, but cannot completely avoid it. In this regard, the motivation to study LP-WUS/WUR in RRC connected mode is to further reduce the unnecessary power consumption caused by uncertain jitter.

Observation 13: The motivation to study LP-WUS/WUR in RRC connected mode for XR service is to reduce the excessive PDCCH monitoring due to unpredictable jitter.

As shown in Figure 5 (b), enhanced C-DRX, i.e. aligned C-DRX cycle to XR traffic periodicity is configured for all the schemes. To further reduce the unnecessary PDCCH monitoring during the jitter range (covered by DRX on duration), LP-WUS monitoring occasions (MOs) can be configured within DRX active time. And UE will firstly detect LP-WUS on the LP-WUS MOs from the beginning of the DRX on duration. When XR traffics arrive at the network side, LP-WUS will be sent to wake up the UE. After detecting the LP-WUS, the UE just start to monitor PDCCH. Compared to the existing R17 search space set group (SSSG) switching depicted in Figure 5 (a), by replacing the PDCCH monitoring on the sparse SSSG with LP-WUS detection, UE will have the opportunity to maintain longer sleep therefore achieve power savings.

Figure 5. Working procedure for LP-WUS/WUR based jitter handling

    - Evaluation methodologies for RRC connected mode

R17 XR power evaluation methodologies and assumptions captured in TR 38.838 [5] should be reused for R18 LP-WUS/WUR power evaluation in RRC connected mode unless otherwise stated. Besides, as analyzed in our contribution [6], the approved R17 XR assumptions for jitter model were derived based on the earlier trace and configuration files (S4aV200634 [7]) provided by SA4. After that, however, SA4 has updated these trace and configuration files. Hence, it is reasonable to consider the additional jitter model deriving according to the updated trace files. In the following, Table 8 shows the jitter distribution based on the statistical analysis of the updated trace fields. It can be observed that based on the updated trace files, the maximum jitter is approximately ranging from -18ms to +18ms. Besides, considering the range truncated by 5%-tile and 95%-tile in CDF, the stable jitter range is around [-8, +8]ms, which is larger than the jitter range (i.e. [-4, 4] ms) agreed in R17 XR.

Table 8. Jitter distribution deriving from the updated trace files provided by SA4

| Parameter | VR 2-1 | VR 2-2 | VR 2-3 | VR 2-4 | VR 2-5 | VR 2-6 | VR 2-7 | VR 2-8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean (ms) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| STD (ms) | 4.96 | 5.00 | 4.85 | 4.89 | 5.31 | 4.02 | 5.19 | 5.18 |
| Min (ms) | -15.89 | -16.00 | -11.87 | -11.96 | -18.95 | -14.40 | -16.48 | -16.18 |
| Max (ms) | +11.27 | +10.30 | +10.10 | +10.51 | +18.69 | +8.08 | +11.59 | +11.13 |
| Jitter range (ms) | [-8.39, +7.89] | [-8.42, +7.91] | [-8.26, +7.69] | [-8.30, +7.77] | [-8.74, +8.42] | [-7.05, +6.24] | [-8.82, +8.19] | [-8.79, +8.20] |
| Jitter range = [5%-tile in CDF, 95%-tile in CDF] ms |  |  |  |  |  |  |  |  |

Proposal 19: For R18 LP-WUS/WUR power evaluation in RRC connected mode, R17 evaluation methodologies and assumptions captured in TR 38.838 (for XR) should be reused as baseline. And the jitter model (jitter range is [-8, +8]ms and jitter STD is 5ms) should be considered.

In addition, the power model of LP-WUR proposed in section 5.1.2 can also be adopted for power evaluation in RRC connected mode. Moreover, to guarantee the latency requirement of XR traffic, main radio need to enter micro or light sleep state rather than the ultra-deep sleep state during LP-WUS monitoring. Accordingly, the latency caused by LP-WUS detection will be only 0 or 3ms. And the total power consumption of LP-WUR and main radio will be close to that of micro or light sleep state i.e., 45 or 20 power units, which is less than half of the power consumption of PDCCH monitoring (i.e., 100 power units).

Proposal 20: For R18 LP-WUS/WUR power evaluation in RRC connected mode, during the LP-WUS monitoring by separate receiver, the power state of main radio can be micro or light sleep.

    - Initial simulation results for LP-WUS scheme

Based on the R17 XR power evaluation methodologies and assumptions, we further evaluate power consumption and system capacity of different cases. And the detailed configurations can be founded in Appendix B. Note that for capacity results of LP-WUS/WUR scheme (combined with main receiver light sleep) corresponding to the UE satisfaction metric with both 95% and 99% packet successful rate are shown. Evaluations for both low load and high loads are given, corresponding to 5 and 10 UEs per cell, respectively.

Figure 6. Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

Figure 7. Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

According to the results shown in Figure 7, compared to always-On scheme (no DRX configuration), R17 PDCCH monitoring adaptation scheme can obtain 10~19% power saving gain, while LP-WUS monitoring schemes can achieve about 25% or 30% power saving gain, meaning up to 15% additional power saving gain.

For both low load ang high load cases, comparing to R17 PDCCH monitoring adaptation scheme, LP-WUS/WUR schemes with main radio enters micro sleep during LP-WUS monitoring can bring {6%~15%} additional UE power saving gain with no capacity loss.

In addition, for low load case, comparing to R17 PDCCH monitoring adaptation scheme, LP-WUS/WUR schemes with main radio enters light sleep during LP-WUS monitoring can bring {10%~22%} additional UE power saving gain with 10% (UE satisfaction metric as 99% packet successful rate) or no (UE satisfaction metric as 95% packet successful rate) capacity loss. But, for high load case, LP-WUS/WUR schemes with main radio enters light sleep during LP-WUS monitoring can achieve similar additional UE power saving gain with as that in low load, but the capacity loss is significant.

In this sense, the LP-WUS monitoring combined with main receiver micro sleep can be applied to all the cases, while the combination with main receiver light sleep are mainly useful for low load scenarios. Moreover, it can be observed that the larger jitter range, the more additional power saving gain can be obtained by LP-WUS/WUR.

Observation 14: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {6%~15%} additional UE power saving gain with no capacity loss in both low load and high load cases.

Observation 15: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {10%~22%} additional UE power saving gain, with acceptable capacity loss at least in low load case.

- Link performance and coverage evaluation

  - Evaluation methodology

As discussed in section 3, the coverage of LP-WUS should be comparable to existing NR channels. In R17 CovEnh SI, the coverage of existing NR signals is evaluated extensively, and PUSCH is identified as the bottleneck channel with the poorest coverage. Hence, the coverage of LP-WUS should be comparable with PUSCH coverage in evaluation, rather than compared with other channels, e.g., DL channels which can achieve coverage far better than real deployments.

For coverage evaluation, R17 CovEnh SI have provide complete evaluation methodology, including definition of performance metric, assumptions and parameters. To avoid repeat discussion, the evaluation methodology provided by R17 CovEnh SI can be reused in coverage comparison between LP-WUS and NR PUSCH. Note that, the PUSCH coverage is evaluated with certain data rates, e.g., 1Mbps for urban, and the same assumptions should be reused.

MIL metric is a simple metric defined in Coverage Enh SI, it can be reused as the metric for relative comparison. LP-WUS specific evaluation assumption and parameters, it can be further discussed in the study.

Proposal 21: For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI should be reused.

- MIL is used as the metric for coverage evaluation;

- LP-WUS should be compared with NR bottle neck channel, i.e., PUSCH.

- PUSCH coverage is evaluated with the certain data rates as in CovEnh SI, e.g., 1Mbps for urban;

- LP-WUS specific evaluation assumptions and parameter, can be further discussed in the study.

For legacy NR DL channels, 7dB noise figure is assumed for the main receiver. While for WUR, for reduced power consumption, separated components are used which are inferior to those in main radio. 15dB noise figure can be assumed for MIL calculation, which means 8dB worse than the main radio.

Proposal 22: 15dB noise figure can be assumed for MIL calculation for LP-WUS.

To achieve the same MIL as PUSCH, certain target receiver sensitivity should be achieved. In [8], The MIL is defined as:

- MIL = Total transmit power – Receiver sensitivity – Tx loss – Rx loss + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain.

To achieve comparable coverage between LP WUS and PUSCH, MIL of LP-WUS should be no less than MIL of PUSCH. As shown in Appendix C, the Receiver sensitivity for LP-WUS should be lower than ~-80dBm to achieve same coverage as PUSCH with 1Mbps data rates.

Observation 16: About -80 dBm Receiver sensitivity is required for LP-WUS, to achieve same coverage as PUSCH with 1Mbps data rates in Urban scenario.

  - Evaluation Assumptions/Parameters and Initial Evaluation Results

The coverage evaluation of LP-WUS could be based on MIL, in which the receiver sensitivity should be calculated based on the required SINR from LLS. Besides, in signal design stage, the schemes should be studied based on LLS.

Figure 8. Signal processing diagram for link level simulation

As shown in Figure 8, the signal generation and transmission are similar as that for NR signals and channels. As discussed in [10], a waveform, e.g., OOK/ASK based waveform, which can be generated by existing OFDM signal is preferred, an OFDM transmitter may still need to be modeled in LLS simulation. For receiver, there would be differences compared with legacy NR signals. The receiver model highly depends on the receiver structure discussed in AI 9.13.2. As shown in [9], non-coherent envelop detector which detects the information from amplitude, is widely reused in ultra-low power receiver, instead of detector for OFDM based NR signals and channels.

In this section, we will discuss the parameters and assumptions should be considered for LP-WUS evaluation.

- Waveform

For waveform of LP-WUS, the waveform should be reported. Typically, the OOK, ASK, etc., based waveform which can be decoded with low power components, e.g., envelop detector, is used for ultra-low power receivers.

Proposal 23: OOK/ASK based waveform, which are widely used for ultra-lower power receiver, can be assumed in link level simulation.

- Coding

For OOK based wave form, some line coding, e.g., Manchester coding are usually used to avoid consecutive zeros. Hence, one information 0/1 symbol is composed of two chips. Furthermore, repetition of the symbol waveform may be considered within a WUS channel to ensure the reliability of LP-WUS.

- Data rate

Similar to legacy channels, LP-WUS with different data rates means different detection performance and coverage. Hence, the data rate should be one important parameter in the evaluation.

Note that, data rate and symbol rate are different concepts. If the LP-WUS is composed of multiple parts, e.g., preamble, information bits/symbols, and CRC bits/symbols, the data rate should be calculated counting all the overhead for a WUS transmission. As shown in Figure 9, the LP-WUS is composed of preamble, information bits/symbols and CRC symbols, where the preamble length is 32 chips, and followed by Manchester coded 32 formation bits and 8 CRC bits. Assuming each OFDM symbol with 30kHz SCS contains 2 chips, (32+ (32+8) x2)/2= 56 OFDM symbols, i.e., 4 slots or 2ms duration is needed to deliver such WUS format, in this example, the symbol rate can be considered as 28ksps, while the effective data rate is 32 information bits *1000/2ms =16 kbps. Hence, the effective data rate should be provided for evaluation LP-WUS with channel structure with different overhead.

Figure 9. Example of a LP-WUS channels structure

Proposal 24: Channel structure/coding/data rate should be reported in link level simulation,

- Channel Structure like {[Sync field] + information bits + [CRC]} can be assumed;

- Simple coding scheme, e.g., Manchester coding, can be used as starting point;

- ~10 to ~100kbps raw data rate can be evaluated in the simulation.

- Signal bandwidth/guard band

OOK signal with different bandwidth may mean different power allocated for LP-WUS channel. Hence, the channel bandwidth should be reported. To ensure reliability of LP-WUS channel, sufficient signal bandwidth should be supported, while excessive overhead should be avoided. Around 1.4MHz ~ 4MHz Signal bandwidth can be considered as baseline for LP-WUS evaluation. To avoid interference between LP-WUS and FDMed DL channels, guard band should be reserved on each side of LP-WUS bandwidth, [1-2] RB can be considered as starting point.

Proposal 25: For frequency domain resources for LP-WUS, following can be assumed in link level simulation

- Around 1.4MHz ~ 4MHz channel bandwidth can be assumed as stating point;

- [1~2] RB Guard band reserved on each side of LP-WUS bandwidth can be assumed as starting point.

- Filter/ACI/Frequency error

Since the LP-WUS channel may not occupied whole channel bandwidth, FDM multiplexing between LP-WUS and existing DL channels may be required for better coexistence. While the WUR radio typically demodulate the LP-WUS in time domain, the FDMed DL signals would be present as adjacent interference to LP-WUR if adjacent frequency is not rejected. Typically, an IF LPF/BPF can be included in WUR in some receiver structure to suppress the ACI. Legacy DL channel, e.g., PDSCH, can be FDMed with the LP-WUS channel to model the adjacent channel interference. For example, the PDSCH can be mapped on RBs not used for LP-WUS and guard band, and the EPRE ratio between LP-WUS and that of PDSCH can be further evaluated.

Figure 10. Illustration of WUS channel multiplexing with NR DL channels

The lower power receiver is typically equipped with low power consumption and low accuracy oscillator, e.g., ring oscillator, the accuracy is far worse than the high-power consumption oscillators used in main receiver. Typically, the frequency offset of a ring oscillator is about 200ppm. As we discussed previously, BPF or LPF should be implemented in the receiver to reject ACI. Considering the frequency uncertainty at WUR, the bandwidth of the filter needs to be wider than the signal bandwidth to make sure the desired LP-WUS signal is maintained, while adjacent channel interference is blocked. For example, if the channel bandwidth of LP-WUS channel is 4MHz, and the frequency error of ring oscillator is ~200ppm, and carrier frequency is 2GHz, the frequency error would be ~400kHz. Thus, the bandwidth of the filter for suppressing ACI may be wider than the signal bandwidth to accommodate the frequency error at WUR.

Proposal 26: For modeling adjacent interference, PDSCH can be mapped on RBs not used for LP-WUS and guard band.

Observation 17: The bandwidth of the filter for suppressing ACI may be wider than the signal bandwidth to accommodate the frequency error at WUR.

- Sampling rate

The OOK/ASK based waveform is detected based on amplitude of samples in time domain, the number of samples used for symbol determination directly impact the performance of LP-WUS. The sampling rate of digital unit in WUR is lower compared with legacy NR receivers which support tens or hundreds MHz channel bandwidth. Around 4/2/1/0.5MHz sampling rate can be assumed in evaluation.

Proposal 27: Around [4/2/1/0.5] MHz sampling rate can be considered in evaluation.

- Rx numbers

For NR DL channels, 2 or 4 antennas are required for different bands. For LP-WUS channel, how many antennas are used can be further discussed. Since most of the LP-WUS/WUR use cases includes power sensitive and small form factor devices, 1 Rx can be considered as baseline in evaluation.

Proposal 28: [1] receiving antennas can be assumed for WUR in evaluation.

- ADC bits

For LP-WUR, one candidate waveform is OOK, and the information is delivered simply through signal amplitude, i.e., on/off. Theoretically, the on/off state can be easily detected by a comparator followed by envelop detector. However, if the interference is strong enough, the comparator cannot work well, and the detection performance will be greatly degraded in high SINR region. If ADC is used instead of comparator, more information can be subtracted, and better performance can be achieved. Bearing in mind that, due to limitation by complexity and low power consumption, ADC with long bit length may not applicable. Hence, a 1-bit comparator or 2/4-bits ADC can be assumed in the evaluation.

Proposal 29: 1-bit comparator or 2/4-bits ADC can be assumed for WUR in LLS.

Based on the discussion above, the attributes and preliminary assumptions should be considered in LLS are provided in Table 9.

Table 9. Assumptions for link level simulation for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 0.7GHz, 0.9GHz, 2GHz or 4GHz |
| Channel structure | Preamble +data +CRC: 32 chips+ 32 bits +8 CRC bits |
| Coding | Manchester coding (For information bits and CRC bits) |
| Date rate/ Raw Data rate* | 28kbps (16kbps) 112kbps (64kbps) 224kbps (128kbps) |
| SCS | 30kHz |
| gNB Channel BW | 20MHz (50 RB) 50MHz (133 RBs) |
| WUS BW | 12RB ~4.32MHz 5RB ~1.8MHz |
| Guard band | 1RB on each side of LP-WUS bandwidth |
| Filter | 5th Order Butterworth filter with bandwidth around WUS BW |
| Frequency offset | ~200 PPM |
| ACI | PDSCH mapped on RBs not used for LP-WUS and guard band; EPRE of LP-WUS vs EPRE of PDSCH = 1:1. |
| Sampling Rate | ~[4/2/1/0.5]MHz |
| ADC | 1bit (comparator) 2bits 4bits ADC |
| Channel Model | TDL-C 300/AWGN |
| Performance metric | {FAR, MDR}: {0.1%, 1%} |
| * The raw data rates are calculated considering overhead (preamble length and CRC length) together with data part. |  |

Some preliminary simulation results are provided in Appendix C.

- Conclusion

In this contribution, the target use cases, applicable RRC states, design targets/KPIs and evaluation methodologies for R18 LP-WUS/WUR study are discussed with the following observations and proposals:

Observation 1: For RRC connected mode, the additional latency by adopting LP-WUS/WUR is up to 3ms

Observation 2: For RRC idle/inactive UEs, the average UE power consumption is currently on several to tens of mW.

Observation 3: To achieve substantial power saving gain, a reasonable target power consumption of LP-WUR need to be at the level of 1/100~1/1000 of the main receiver, corresponding to tens to hundreds of μW.

Observation 4: When the LP-WUR power consumption is 30μW~500μW, UE battery life can be increased by 3~7 times compared with legacy UEs.

Observation 5: When the LP-WUR power consumption is 30μW~500μW, the standby time of IoT devices can be extended to up to 10 years with the use of a button battery for power supply.

Observation 6: The main contribution of latency comes from the LP-WUS monitoring cycle and the transition time of main radio.

Observation 7: With the assistance of a separate receiver i.e., LP-WUR, the main radio has the opportunity to enter an ultra-deep sleep state (i.e., deeper than the existing deep sleep).

Observation 8: Compared with existing deep sleep mode, the main radio in ultra-deep sleep state consumes longer transition time and more energy for the transition from the ultra-deep sleep state to normal operation.

Observation 9: Comparing with I-DRX paging, LP-WUR/WUS can largely reduce the UE power consumption (5x or 7x), with similar latency performance.

Observation 10: Comparing with eDRX, LP-WUR/WUS can largely reduce the latency (26x), with even lower UE power consumption.

Observation 11: Comparing with I-DRX paging and eDRX, LP-WUR/WUS provides a much better trade-off between latency and power consumption.

Observation 12: Comparing with Always-On LP-WUS monitoring, duty-cycled LP-WUS monitoring can further reduce power consumption while sacrificing latency performance.

Observation 13: The motivation to study LP-WUS/WUR in RRC connected mode for XR service is to reduce the excessive PDCCH monitoring due to unpredictable jitter.

Observation 14: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring  combined with main receiver micro sleep can bring {6%~15%} additional UE power saving gain with no capacity loss in both low load and high load cases.

Observation 15: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {10%~22%} additional UE power saving gain, with acceptable capacity loss at least in low load case.

Observation 16: About -80 dBm Receiver sensitivity is required for LP-WUS, to achieve same coverage as PUSCH with 1Mbps data rates in Urban scenario.

Observation 17: The bandwidth of the filter for suppressing ACI may be wider than the signal bandwidth to accommodate the frequency error at WUR.

Proposal 1: Study the following use cases for LP-WUS/WUR:

- For IoT devices (e.g., sensors and controllers): The battery should last at least few years; Latency would be within several or tens of seconds; For latency sensitive sensors/actuators, the latency requirement is 1 or 2 seconds; The mobility would be stationary or nomadic.

- For wearable devices: The battery should last a few weeks; Latency need to be within several seconds; Support of low/medium speed is required.

- For XR devices or smart phones: power saving in CONNECTED mode is desirable; Latency which is critical for RRC CONNECTED state need to be in the order of milliseconds; Support of low/medium speed is required.

Proposal 2: Study the application of LP-WUS/WUR in both RRC idle/inactive mode and RRC connected mode.

Proposal 3: The target KPI for LP-WUR power consumption should be tens to hundreds of μW.

Proposal 4: The latency introduced by LP-WUR/WUS needs to consider two parts: wake-up delay and transition time of main radio.

Proposal 5: Target KPI of latency should be as follows.

- Depends on the length of LP-WUS monitoring cycle for RRC idle/inactive mode

- Wake up delay: depends on the length of LP-WUS monitoring cycle, e.g. hundreds of milliseconds

- Transition time: hundreds of milliseconds

- Several milliseconds for RRC connected mode

- Wake up delay: 0 (assuming continuous monitoring of LP-WUS)

- Transition time: up to 3ms

Proposal 6: Coverage of the LP-WUS should be comparable to the NR bottleneck channel.

Proposal 7: Around 100kbps data rate can be considered as design target for LP-WUS.

Proposal 8: The overall design target of coexistence is to support multiplexing between LP-WUS with legacy NR signals/channels and to allow reuse of unused LP-WUS resources for other DL transmissions.

Proposal 9: Adopt the following power consumption evaluation methods for R18 LP-WUS/WUR study:

- For RRC idle/inactive mode, the simulation assumptions for R17 Power saving paging enhancement evaluation can be reused.

- For RRC connected mode, the simulation assumptions for R17/18 XR power evaluation can be reused.

Proposal 10: The latency can be roughly caculated as 1/2 LP-WUS monitoring cycle length + transition time of main radio.

Proposal 11: FAR and MDR is used as the performance metric for LP-WUS in link level simulation,

- {FAR, MDR}: {0.1%, 1%} can be assumed as starting point.

Proposal 12:  MIL metric defined in Coverage Enh SI can be reused as the metric for relative comparison between coverage of LP-WUS and NR channels.

Proposal 13: Resource overhead and network power consumption are evaluated by numerical analysis, using the outcome of NW energy saving study. Capacity impact is evaluated by system level simualtion using XR evaluation methodlogy for RRC CONNECTED.

Proposal 14: Adopt the following terminology for future discussion,

- Main radio：the Tx/Rx module operating for legacy system

- LP-WUR: The Rx module operating for receiving/processing LP-WUS

Proposal 15: For R18 LP-WUS/WUR power evaluation, the following power model of the ultra-deep sleep state agreed in R18 positioning SI for LPHAP is reused for main radio.

  - Note: the value of additional transition energy can be updated based on further agreement made in R18 positioning SI.

| Power State | Relative Power | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- | --- |
| Ultra-deep sleep | 0.015 | [2000-20000] | 400ms |

Proposal 16: For R18 LP-WUS/WUR power evaluation, the following power model of LP-WUR is considered.

- Note: these value in brackets can be further confirmed based on the progress of LP-WUR architecture.

| Power State | Absolute Power | Relative Power | Additional transition energy | Total transition time |
| --- | --- | --- | --- | --- |
| LP-WUR sleep | [2] μW | [0.002] | 0 | 0 |
| LP-WUR monitoring | [30~500] μW | [0.03~0.5] | - | - |

Proposal 17: For comparison with R18 LP-WUS/WUR, both I-DRX paging and eDRX can be taken as baseline schemes.

Proposal 18: The evaluation assumptions given in Table 4~7 should be considered for R18 LP-WUS/WUR power evaluation in RRC idle/inactive mode.

Proposal 19: For R18 LP-WUS/WUR power evaluation in RRC connected mode, R17 evaluation methodologies and assumptions captured in TR 38.838 (for XR) should be reused as baseline. And the jitter model (jitter range is [-8, +8]ms and jitter STD is 5ms) should be considered.

Proposal 20: For R18 LP-WUS/WUR power evaluation in RRC connected mode, during the LP-WUS monitoring by separate receiver, the power state of main radio can be micro or light sleep.

Proposal 21: For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI should be reused.

- MIL is used as the metric for coverage evaluation;

- LP-WUS should be compared with NR bottle neck channel, i.e., PUSCH.

- PUSCH coverage is evaluated with the certain data rates as in CovEnh SI, e.g., 1Mbps for urban;

- LP-WUS specific evaluation assumptions and parameter, can be further discussed in the study.

Proposal 22: 15dB noise figure can be assumed for MIL calculation for LP-WUS.

Proposal 23: OOK/ASK based waveform, which are widely used for ultra-lower power receiver, can be assumed in link level simulation.

Proposal 24: Channel structure/coding/data rate should be reported in link level simulation,

- Channel Structure like {[Sync field] + information bits + [CRC]} can be assumed;

- Simple coding scheme, e.g., Manchester coding, can be used as starting point;

- ~10 to ~100kbps raw data rate can be evaluated in the simulation.

Proposal 25: For frequency domain resources for LP-WUS, following can be assumed in link level simulation

- Around 1.4MHz ~ 4MHz channel bandwidth can be assumed as stating point;

- [1~2] RB Guard band reserved on each side of LP-WUS bandwidth can be assumed as starting point.

Proposal 26: For modeling adjacent interference, PDSCH can be mapped on RBs not used for LP-WUS and guard band.

Proposal 27: Around [4/2/1/0.5] MHz sampling rate can be considered in evaluation.

Proposal 28: [1] receiving antennas can be assumed for WUR in evaluation.

Proposal 29: 1-bit comparator or 2/4-bits ADC can be assumed for WUR in LLS.

# References

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, RAN#97e, vivo.

- TR 38.840, Study on User Equipment (UE) power saving in NR, V16.0.0.

- Chairman’s Notes of RAN1 #104bis-e final (Online meeting, April 12th – 20th, 2021).

- TR 38.875, Study on support of reduced capability NR devices, V17.0.0.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR, v17.0.0.

- R1-2208660, Discussion on XR specific power saving enhancements, vivo.

- S4aV200634, 	[FS_XRTraffic] Traces and Configurations for VR2, CG and AR2, Qualcomm Incorporated, Mediatek.

- TR 38.830, Study on NR coverage enhancements, v17.0.0.

- https://www.eecs.umich.edu/wics/low_power_radio_survey.html

- R1-2208670, Discussion on physical signal and procedure for low power WUS, vivo.

# Appendix A – Processing timeline for the schemes in RRC idle/inactive mode

The assumed processing timelines of I-DRX paging, eDRX and LP-WUS/WUR schemes.

# Appendix B – Evaluation assumption for RRC connected mode

The detail configurations for R17 PDCCH monitoring adaptation and LP-WUS/WUR schemes are illustrated as below.

R17 PDCCH monitoring adaptation scheme includes both PDCCH skipping indication and SSSG switching indication, and its key assumptions are listed below:

- Default/sparse SSSG: do PDCCH monitoring in every two slots;

- Dense SSSG: do PDCCH monitoring in every slot.

- Two candidate PDCCH skipping durations: 4ms and 6ms.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

For LP-WUS/WUR scheme, the key assumptions are introduced below:

- The LP-WUS monitoring is performed within DRX onduration.

- The main receiver enters into the micro sleep when doing LP-WUS monitoring.

- The total relative power for LP-WUS monitoring is assumed as 45 or 20 power units (i.e., the main radio enters micro or light sleep).

- And the latency from receiving LP-WUS to be ready for PDCCH monitoring is set to 1 slot.

- R17 PDCCH skipping indication is adopted. And three candidate PDCCH skipping durations are 2ms, 4ms and 6ms respectively.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

- System capacity is assumed as the maximum number of users per cell with at least 90% of UEs being satisfied. And a UE can be regarded as a satisfied UE if more than 99% or 95% of packets are successfully transmitted within a given air interface PDB.

Table I. Simulation assumption for FR1 Indoor Hotspot scenario

| Parameter | value |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

Table II.  The DL video traffic models with 60 FPS adopted in R17 XR SI

| Traffic model | VR/AR | VR/AR |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |
| Mean packet size (Bytes) | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 | 16.67 |
| PDB (ms) | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |
| Jitter Mean (ms) | 0 |  |
| Jitter STD (ms) | 2 or 5 |  |
| Jitter Range (ms) | [-4, +4], [-6, +6] or [-8, +8] |  |

# Appendix C – Evaluation Results on Coverage for LP-WUS

In the evaluation results shown in Appendix C, some common parameters are assumed, provided as follows

- Carrier frequency: 2GHz

- Channel model: TDL-C 300ns

- Channel structure: Preamble (32 chips) + data (32 bits) + CRC (8 bits), where data bits and CRC bits are Manchester coded.

- Data rates: 28kbps/112kbps/224kbps*

- Channel bandwidth: 20MHz (50RB with 30kHZ SCS)

- LP-WUS channel bandwidth: 4.32MHz (12 RB with 30kHz SCS)

- Guard band: 1RB with 30kHz SCS

- Filter bandwidth: WUS BW

- ACI (if exist): mapped on RBs not used for LP-WUS and guard band; EPRE of LP-WUS vs EPRE of PDSCH = 1:1.

- Sampling rate: 3.84MHz

- ADC bits: 1(comparator)/2/4

* Cases in this section legend with 28kbps/112kbps/224kbps are not raw data rates for LP-WUS. If overhead for preamble, CRC, and the raw data rates are 16kbps/64kbps/128kbps, respectively.

- Performance of WUR with different ADC bitwidth

As shown in above figures, obvious performance loss is observed if comparator (1-bit ADC) is assumed for WUR, especially when ACI is also present. At least ADC with bit width of 2 bits are required to achieve reliable LP-WUS detection performance. ADC with bit width of 4 bits can achieve nearly optimal (no quantization) performance.

- Performance of WUR with/or without filter with co-current PDSCH interference

This simulation assumes As shown in above figure, when co-current PDSCH interference is present, the WUR can not work without filter. If BPF/LPF which can reject ACI is implemented in WUR, FDMed allocation of DL channels with LP-WUS can be allowed, while the WUR receiver performance can still be guaranteed.

- Performance of LP-WUS with different data rates

As shown in above figure, different data rates lead to different detection performance and coverage. Hence, LP-WUS with a certain target data rates, which can achieve the comparable coverage as NR PUSCH with selected data rates (e.g., 1Mbps for urban scenario), can be determined in evaluation.

- Comparison of MIL between LP-WUS and NR PUSCH

BAR graph which shows MIL comparison between NR PUSCH and LP-WUS are provided as follows. The MIL of LP-WUS reuse the same calculation defined in Rel-17 CovEnh SI. As shown in following figure, the WUS with 112kbps data rate (64kbps raw data rate) can achieve higher coverage than existing NR PUSCH with 1Mbps data rate. While the WUS with 224kbps data rate (128kbps raw data rate) can achieve coverage very close to coverage of NR PUSCH with 1Mbps data rate.

Detailed MIL calculation for NR PUSCH and LP-WUS with different data rates, are provided in the following attachment which is revised from the template provided in [8].