3GPP TSG RAN WG1 #110bis-e		                                                                 R1-2208715

e-Meeting, October 10th – 19th, 2022

Title:	Draft reply LS on IUC with non-preferred resource set

Response to:	R1-2205728/R2-2206314

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	ZTE, Sanechips [to be RAN1]

To:	RAN2

Cc:

Attachment:	-

Contact Person:

Name:	Yuzhou Hu

E-mail Address:	hu.yuzhou@zte.com.cn

Overall description

RAN1 thanks RAN2 for the LS (R1-2205728/R2-2206314) on IUC with non-preferred resource set. In the LS, RAN2 addressed an inter-UE coordination scenario in which UE B receives IUC Scheme 1 non-preferred resource set from UE A, but UE B does not perform sensing in the resource pool associated with the non-preferred resource set, e.g. UE B performs mode 2 random resource selection, etc. For this case, RAN2 raised the following questions, and RAN1 would provide response as follows:

Q1: Is the scenario described above a valid scenario or not? A1: This scenario may occur, but there is no RAN1 agreement to handle this scenario. Q2: If the answer to Q1 is yes, does resource exclusion based on non-preferred resource set needs to be performed by UE B or not? A2: In the scenario described above, it is NOT necessary to perform resource exclusion based on non-preferred resource set, instead, UE can perform normal random resource selection. Following RAN1’s conclusion, the exclusion of non-preferred resource is performed in PHY layer during sensing procedure, and the UE procedure for using a received non-preferred resource set has been captured in clause 8.1.4c of 38.214 as follows: - 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement.. Q3: If the answer to Q2 is yes, then, in RAN1’s view, which specification (PHY or MAC) should capture the resource exclusion behavior?

Actions:

To RAN2:

RAN1 respectfully requests RAN2 to take above responses into account for the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#111			14th -18th Nov 2022				Toulouse, France

RAN1#112			27 February to 3 March 2023			Athens, Greece